3GPP TSG-RAN2 Meeting # 115-e	R2-2108852

# Electronic Meeting,  16th - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4712 | rev | 1 | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On T330 resetting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | T330 (logged MDT DurationTimer) is used to indicate the effectiveness period of a logged MDT configuration which shall keep running irrespective of state transition or RAT type. However, according current specs T330 will be stopped when UE enters RRC_INACTIVE or when UE HO to another RAT, as a consequence, UE cannot continue logging MDT results when transiting to idle or connected (if configured to log MBSFN measurements) at a later poin in time or coming back to LTE from NR since T330 is stopped and not restarted. UE behavior as per current specification: - The UE is configured with logged MDT configuration by an LTE cell connected to EPC (Cell-A). - The UE is released to RRC Idle by Cell-A. - The UE performs cell reselection to another LTE cell connected to 5GC (Cell-B) and comes to RRC connected in Cell-B. - The UE is released to RRC Inacitve by Cell-B. - The UE stops T330 timer and does not log any logged MDT measurements in the RRC Inactive as T330 is not running and as the section 5.6.8 is applicable only for RRC Idle and RRC Connected mode. - The UE comes to connected in Cell-X. - The UE is released to RRC Idle by Cell-X. - The UE does NOT log MDT measurement as T330 is already stopped. This is not the intended behavior and the UE should continue to log the MDT measurements upon returning to RRC Idle mode. The correct UE behavior: - The UE is configured with logged MDT configuration by an LTE cell connected to EPC (Cell-A). - The UE is released to RRC Idle by Cell-A. - The UE performs cell reselection to another LTE cell connected to 5GC (Cell-B) and comes to RRC connected in Cell-B. - The UE is released to RRC Inacitve by Cell-B. - The UE does not log any logged MDT measurements as the section 5.6.8 is applicable only for RRC Idle and RRC Connected mode. - The UE comes to connected in Cell-X. - The UE is released to RRC Idle by Cell-X. - The UE logs MDT measurement as T330 is still running. Similar issue exists for inter-RAT handover from LTE to NR for a Rel-15 UE. UE behavior as per current specification: - The UE is configured with logged MDT configuration by an LTE cell connected to EPC (Cell-A). - The UE is released to RRC Idle by Cell-A. - The UE performs cell reselection to another LTE cell connected to 5GC (Cell-B) and comes to RRC connected in Cell-B. - The UE is handed over to NR cell, Cell-C. - The UE stops T330 timer. - The UE is handed over to Cell-A by NR Cell-C. - The UE is released to RRC Idle by Cell-A. - The UE does NOT log MDT measurement as T330 is already stopped. This is not the intended behavior and the UE should continue to log the MDT measurements upon returning to RRC Idle mode in LTE. The correct UE behavior: - The UE is configured with logged MDT configuration by an LTE cell connected to EPC (Cell-A). - The UE is released to RRC Idle by Cell-A. - The UE performs cell reselection to another LTE cell connected to 5GC (Cell-B) and comes to RRC connected in Cell-B. - The UE is handed over to NR cell, Cell-C. - The UE is handed over to Cell-A (LTE cell conneted to EPC) by NR Cell-C. - The UE is released to RRC Idle by Cell-A. - The UE logs MDT measurement as T330 is still running. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in subclause 5.3.8.7 and 5.4.3.4 that T330 is not stopped upon the transition to RRC Inactive state and upon handing over the UE to NR. It is to be noted that the UE behaviour of not logging the MDT measurements while being in RRC Inactive is not changed. Impact analysis Impacted functionality: Logged MDT Inter-operability analysis: No inter-operability issues foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Logged MDT is terminated which is not with stage2 since T330 will be stopped when UE enters RRC_INACTIVE, therefore UE cannot log MDT results when it enters RRC_IDLE or log MBSFN measurements if configured if it enters RRC_CONNECTED afterwards. Also, there will be misalignment between UE and NW about the running time of durationTimer, which could lead to misunderstanding at NW’s side on the MDT results stored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.8.7, 5.4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No Rel-16 shadow CR is needed since the changes were already specified in Rel-16. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-0: R2-2108312 Rev-1: Cover sheet is updated to capture better explanation for reason for change and the summary of change. The WI code is changed to TEI15. |  |  |  |  |  |  |  |  |

/* Start of first change */

#### 5.3.8.7	UE actions upon entering RRC_INACTIVE

Upon entering RRC_INACTIVE, the UE shall:

1>	reset MAC and release the default MAC configuration if any;

1>	stop all timers that are running except T302, T309, T320, T325 and T330;

1>	re-establish RLC entities for all SRBs and DRBs;

1>	if the RRCConnectionRelease message is including the waitTime:

2>	start timer T302, with the timer value set according to the waitTime;

2>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2';

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	apply the received rrc-InactiveConfig;

1>	if the RRCConnectionRelease message was received in response to an RRCConnectionResumeRequest:

2>	in the stored UE Inactive AS context:

3>	replace the KeNB and KRRCint keys with the current KeNB and KRRCint keys;

3>	replace the C-RNTI with the temporary C-RNTI which the UE has used to receive the RRCConnectionRelease message;

3>	replace the cellIdentity with the cellIdentity of the PCell at the time the UE has received the RRCConnectionRelease message;

3>	replace the previously stored physical cell identity with the physical cell identity of the PCell at the time the UE has received the RRCConnectionRelease message;

1>	else:

2>	store in the UE Inactive AS Context, the current KeNB and KRRCint keys, the ROHC state, the stored QoS flow to DRB mapping rules, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, and all other parameters configured;

1>	if the periodic-RNAU-timer is included:

2>	start timer T380, with the timer value set to the periodic-RNAU-timer;

1>	suspend all SRB(s) and DRB(s), except SRB0;

1>	indicate PDCP suspend to lower layers of all DRBs;

1>	indicate the suspension of the RRC connection to upper layers;

1>	enter RRC_INACTIVE and perform procedures as specified in TS 36.304 [4], clause 5.2.7;

Upon selecting to an inter-RAT cell or switching to another CN type, the UE shall:

1>	perform the actions upon leaving RRC_INACTIVE as specified in 5.3.12, with release cause 'other';

/* End of first change */

/* Start of second change */

#### 5.4.3.4	Successful completion of the mobility from E-UTRA

Upon successfully completing the handover, the cell change order or enhanced 1xRTT CS fallback, the UE shall:

1>	if the targetRAT-Type in the received MobilityFromEUTRACommand is set to eutra (intra-E-UTRA inter-system HO):

2>	indicate to the upper layers associated to the source system the release of the RRC connection together with the release cause 'other';

2>	the procedure ends;

1>	else if the UE was connected to 5GC prior to the reception of the MobilityFromEUTRACommand and the targetRAT-Type in the received MobilityFromEUTRACommand is set to nr:

2>	reset MAC;

2>	stop all timers that are running except T325 and T330;

2>	release ran-NotificationAreaInfo, if stored;

2>	release the AS security context including the KRRCenc key, the KRRCint, the KUPint key and the KUPenc key, if stored;

2>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity and SDAP entity for all established RBs;

NOTE 1:	PDCP and SDAP configured by the source configurations RAT prior to the handover that are reconfigured and re-used by target RAT when delta signalling (i.e., during inter-RAT intra-sytem handover when fullConfig is not present) is used, are not released as part of this procedure.

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

NOTE 2:	If the UE performs enhanced 1xRTT CS fallback along with concurrent mobility to CDMA2000 HRPD and the connection to either CDMA2000 1xRTT or CDMA2000 HRPD succeeds, then the mobility from E-UTRA is considered successful.

/* End of second change */