3GPP TSG-RAN #93	Tdoc RP-21XXXX

Electronic meeting, 2021-09-13 - 2021-09-17

Agenda Item:	X

Source:	Ericsson (Moderator)

Title:	Moderator Summary for [93e-30-band-n77]

Document for:	Discussion, Decision

# 1	Introduction

This is a summary of the email discussion [93e-30-band-n77].

# 2	Discussion

### 2.1	Background

RAN2 and RAN4 were tasked to extend the n77 band in the US to cover the 3450-3550 MHz region, in addition to the 3700-3980 MHz region.

At RAN2#115, RAN2 discussed two solutions for this:

-	A new cap signalling + new NS value

-	B new frequency band replace n77 in the US including the DoD part.

RAN4 agreed that new capability signalling shall be defined, i.e., not the new frequency band solution.

For Solution A, RAN2 clarified in their LS RP-211671 that:

RAN2 has agreed that UE’s that don’t support the DoD band need to be barred from accessing the DoD band in the US. RAN2 thinks that a new NS-value can be defined to prevent legacy UEs supporting n77 from camping on the DoD bands and as legacy UEs cannot identify the new value, the UE would not camp on that cell.

RAN2 provided technically endorsed CRs for Solution A which are adding the capability bit for Solution A, see RP-212445. RAN4 provided CRs for Solution A in RP-211887, but these CRs are lacking the NS-value.

Nokia (RP-212169), Ericsson (RP-212204) and OPPO (RP-211815) propose to approve CRs as per Solution A defined by RAN2 (capability bit + NS-value). Company contributions for this approach from Ericsson, Nokia, Verizon, Qualcomm can be found in RP-212513, RP-212514, RP-212515, RP-212516, RP-212517, RP-212518.

Apple, MediaTek and Skyworks Solution Inc. (RP-212305) proposes to approve CRs without the NS-value.

### 2.2	Initial round

#### 2.2.1	Discussion initial round

Do you agree to approve Solution A as per RAN2's agreement, i.e. having both UE capability bit and an NS value? If no, please clarify how do you propose to ensure that UE’s that don’t support the DoD band need to be barred from accessing the DoD band in the US.

| Company | Input |
| --- | --- |

If Solution A, as defined in the RAN2 LS (with both capability bit and NS value) should be introduced, do you agree to introduce them as per the CRs in RP-212513, RP-212514, RP-212515, RP-212516, RP-212517, RP-212518?

| Company | Input |
| --- | --- |

#### 2.2.2	Conclusion initial round

# 3	Conclusion

TODO