3GPP TSG RAN Meeting #94-e	RP-212714

E-meeting, December 6th – 17th 2021

# Source:	Moderator (RAN VC | Deutsche Telekom)

# Title:	New WID: Enhancements of NR Multicast and Broadcast Services

# Document for:	Approval

Agenda Item:	8A.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Enhancement of NR Multicast and Broadcast Services

## Acronym: E_NR_MBS

## Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

{Not applicable for Feature nor for a Study Item}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title.

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 860048 | Work Item on NR Multicast and Broadcast Services | Rel-17 NR Multicast and Broadcast Services Support WI |
| 900038 | Architectural enhancements for 5G multicast-broadcast services | Rel-17 SA2 WI on 5G multicast-broadcast services. |
| 850040 | Broadcast / Multicast requirements supporting Mission Critical Services in 5G | Rel-17 SA6 WI on 5G MBS support of MCS |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

## 3	Justification

[Note by the moderator: this draft justification is based on multiple proposals I received offline and some which have been shared after my invite in the initial round. Update is needed based on the final decision which objectives will be part of the WI]

3GPP has developed NR broadcast/multicast in Rel-17, aiming to enable general MBS services over 5GS. Further enhancements for NR MBS are needed in several aspects.

To enable resource-efficient delivery of multicast/broadcast services, 3GPP has developed NR broadcast/multicast in Rel-17 according to the WID in RP-193248, aiming to enable general MBS services over 5GS. The use cases identified that could benefit from this feature include public safety and mission critical, V2X applications, IPTV, live video, software delivery over wireless and IoT applications, etc. Two delivery modes have been agreed for Rel-17 MBS with delivery mode 1 (only for multicast) capable of addressing higher QoS services and delivery mode 2 (only for broadcast) focusing on lower QoS services. Given that Rel-17 MBS already provide the basic function to support MBS services, the general main goal for Rel-18 should be to enable better deployment of MBS, such as improvement of spectrum efficiency/capacity/reliability based on Rel-17 MBS and to enable better support of broadcast services required by operators such as FTA.

In Rel-17, RAN only specifies multicast for UEs in RRC_CONNECTED state, which cannot fulfil the requirements of, e.g., Mission Critical Services, especially for cells with a large number of UEs according to TR 23.774. Also, to always keep UEs in RRC_CONNECTED state is not power efficient. It is ‎therefore important to support multicast for UEs in RRC_INACTIVE. Furthermore, it is also meaningful to support multicast for UEs in RRC_IDLE

The Rel-17 MBS only supports transparent SFN mechanism within one gNB-DU which would limit the number of cells configured for SFN and thus imposes severe restrictions on the expected gain of SFN especially for the use cases requiring large area SFN. Enhanced SFN support for ‎the [intra-CU/inter-DU] case is needed to handle the interference and to improve the cell edge performance. This is ‎particularly important for services (e.g., video) that require very high data rate.

Free-to-air (FTA) is a type of broadcast service where a UE accesses broadcast services without subscription and UE is not required to support uplink transmission for FTA reception. The Rel-17 NR MBS Broadcast solution allows that the UE receives broadcast service in a downlink only manner i.e. performing broadcast reception without a need to access the network beforehand. However, in the typical use case for FTA, the UE is able to receive FTA service and access the network of same or another operator for unicast service, but different receive/transmit modules in some UEs would have impact on each other due to constraint of cost, interference etc.. Therefore, the unicast connection might be impacted by the reception of FTA frequency for this kind of UEs. If the enhancement can be made so that network could be aware of the UE capability reduction in unicast side and the carrier info where it receives FTA service, better performance for unicast and FTA reception can be achieved. There is also strong market requirement to support MBS reception by UEs with no ‎subscription or with only 3rd party content provider ‎subscription. Based on Rel-17 broadcast mode, further enhancements are needed in ‎both RAN and CN for complete and improved support of FTA, including the case when UE simultaneously receive unicast and ‎FTA services from different serving cells or bands‎.‎

Network sharing is a common practice to reduce network CAPEX. With RAN sharing deployment, if the same Multicast/Broadcast service is provided by two (or more) operators separately, this service would be recognized as separate TMGIs resulting in duplicated PTM radio resources consumption in the same cell for transmission of the same content. This justifies resource efficiency improvement in the RAN sharing scenario.

Note that public safety services benefit from the Rel-17 NR MBS functions, as well as from Rel-18 enhancements that follow the above ‎justifications. ‎

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

This Work Item is to further enhance the NR Multicast/Broadcast functions based on Rel-17 MBS. The objectives for Rel-18 include:

- Specify the enhancements of SFN for [inter-DU and inter-CU] scenario based on existing numerology and Cyclic Prefix [RAN3]

  - [Study and specify mechanism to support coordination and synchronization across gNBs/DUs for SFN [RAN3]]

- Specify support of multicast reception by UEs in RRC_INACTIVE [and RRC_IDLE] states [RAN2, RAN3, RAN1?]

  - PTM configuration for UEs receiving multicast in RRC_INACTIVE [and RRC_IDLE] states [RAN2, RAN1?]

  - Study this impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE [and RRC_IDLE states] [(Seamless/lossless mobility is not required)] [RAN2, RAN3]

- Study and if necessary, specify enhancements to improve the resource efficiency for MBS reception in RAN sharing scenarios [RAN3]

- [Specify signalling enhancements for improved support of Free-To-Air (FTA) / Receive Only Mode (ROM), i.e. ‎including UE capability related assistant information report regarding simultaneous reception of ‎FTA / ROM and unicast services provided by the same or different operator [RAN2]]

- [(Low priority) Specify enhancements of power saving mechanism for MBS [RAN2, RAN1]]

Note: collaboration with SA2 is expected in due course for the above objectives.

Rel-17 left-overs might also be included, but final decision to be taken after finalisation of Rel-17:

- x

- y

- z

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {E.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {E.g. "TSG#87"} | {E.g. "TSG#89"} | {e.g.: rapporteur: <FamilyName>, <GivenName>, <Company>, <email address>} |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.300 | NR and NG-RAN Overall Description | RAN#9x | Core part |
| 38.331 | NG-RAN; Radio Resource Control (RRC) Protocol Specification | RAN#9x | Core part |
| 38.304 | NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state | RAN#9x | Core part |
| 38.321 | NG RAN; Medium Access Control (MAC) protocol specification | RAN#9x | Core Part |
| 38.322 | NG RAN: Radio Link Control (RLC) protocol specification | RAN#9x | Core Part |
| 38.323 | NG RAN: Packet Data Convergence Protocol (PDCP) specification | RAN#9x | Core Part |
| 38.423 | NG-RAN; Xn Application Protocol (XnAP) | RAN#9x | Core part |
| 38.413 | NG-RAN; NG Application Protocol (NGAP) | RAN#9x | Core part |
| 38.211 | NG RAN: Physical channels and modulation | RAN#9x | Core part |
| 38.212 | Multiplexing and channel coding | RAN#9x | Core part |
| 38.213 | Physical layer procedures for control | RAN#9x | Core part |
| 38.214 | Physical layer procedures for data | RAN#9x | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Tbd.

## 7	Work item leadership

Primary: RAN2

Secondary: RAN3, RAN1 ?

## 8	Aspects that involve other WGs

Coordination with SA2 is needed

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name