3GPP TSG RAN #94-e	RP-213120

e-Meeting, December 6th – 17th, 2021

Source:	Sharp

Title:	Views on Rel-18 DL MIMO

Agenda Item:	8A.1

Document for:	Discussion and Decision

# Background

The topic of MIMO evolution for Downlink and Uplink was set as one of the email threads as a candidate of Rel-18 work items based on the guidance after RAN#93-e, and the email discussion was conducted during October 20th to 29th. The moderator’s summary and the draft WID were prepared as RP-212661 and RP-212701, respectively. In this contribution, we show our views on downlink MIMO evolution in Rel-18.

# Discussion

In the current proposed WID, the following objectives for the DL MIMO enhancements are captured:

- Enhancements on DL MIMO: - Study, and if justified, specify CSI enhancement in high/medium velocities for exploiting time-domain correlation/Doppler-domain information, including potential enhancement to feedback-based CSI acquisition mechanisms for DL precoding - Specify extension of Rel-17 Unified TCI framework, e.g., - for indication of multiple DL and UL TCI states (e.g., M>1 and/or N>1) for multi-TRP schemes - for multi-TRP, it includes multi-TRP schemes supported in Rel-16/Rel-17 - Specify to support larger number of orthogonal DMRS ports - Study, and if justified, specify enhancements on CSI acquisition for Coherent-JT for both FDD and TDD targeting FR1 - Study, and if justified, specify overhead and/or Latency reduction with UE-initiated beam management/beam acquisition procedures

# CSI enhancements

Up to Rel-17, CSI enhancements were specified for SU-MIMO and MU-MIMO, and frequency domain compression has bee specified. However, as discussed in the email discussion, the current CSI feedback is not sufficient for supporting MIMO for UEs with medium/high mobility. In our view, time-domain correlation/Doppler domain correlation should be included in the objectives in this WID.

Proposal 1: The CSI enhancements by exploiting time-domain correlation/Doppler domain correlation should be specified on the WID.

# Multi-beam/Multi-TRP enhancements

### Unified TCI framework

In Rel-17, only M=1 and N=1 is supported for indication of the unified TCI for beam management. In our view, indication of multiple TCI states (i.e. M>1 and/or N>1) should be specified in Rel-18 MIMO. In addition, UE-initiated beam management should be studied in this framework.

Proposal 2: Indication of multiple TCI states (i.e. M>1 and/or N>1) should be specified in Rel-18 DL MIMO

### Large number of DMRS ports

During email discussion, the necessity of large number of DMRS ports more than 12 ports is identified especially for multi-TRP transmission. In our view, more than 12 orthogonal DMRS ports should be studied in Rel-18

Proposal 3: more than 12 orthogonal DMRS ports should be studied.

### Asynchronous MTRP operation

In FR2, the CP length is relatively smaller and synchronization requirements would be tight. In our view, asynchronous MTRP operation should be included in the objectives in the WID.

Proposal 4: Asynchronous MTRP operation should be studied in the WID.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: The CSI enhancements by exploiting time-domain correlation/Doppler domain correlation should be specified on the WID.

Proposal 2: Indication of multiple TCI states (i.e. M>1 and/or N>1) should be specified in Rel-18 DL MIMO

Proposal 3: more than 12 orthogonal DMRS ports should be studied.

Proposal 4: Asynchronous MTRP operation should be studied in the WID.

# References

- RP-212657, “Additional Guidance on RAN Rel-18 Email Discussion during October 20th to 29th”, RAN#94-e

- RP-212661, “Moderator’s summary of discussion for [RAN94e-R18Prep-01] MIMO Evolution for Downlink and Uplink” , RAN#94-e

- RP-212701, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e