3GPP TSG RAN meeting #94e										RP-213365

Electronic Meeting, Dec. 6-17, 2021

## Status Report to TSG

Agenda item:			10.4.1

| WI / SI Name | Additional enhancements for NB-IoT and LTE-MTC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NB_IOTenh4_LTE_eMTC6 |  |  |  |  |
| Unique ID | 860044 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-211340 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2022 | Performance part: 09/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 85% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG 1 |
| --- | --- | --- |
| Rapporteur | Name | Yubo YANG Emre YAVUZ |
|  | Company | Huawei Ericsson |
|  | Email | yangyubo1@huawei.com emre.yavuz@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#106bis-e meeting, 20 contributions [1-20] were submitted, and the following agreements were achieved:

For NB-IoT 16-QAM:

Working Assumption

For the new term ∆TF,c introduced for power control of NPUSCH,

- Reuse the LTE definition simplified for NB-IoT: ∆TF,ci=10log102BPRE∙Ks-1 for Ks=1.25 and ∆TF,ci=0 for Ks=0, where Ks is given by higher layer parameter deltaMCS-Enabled, and BPRE=KNRE where K is the code block size.

- FFS: whether the new term applies to QPSK when configured with 16QAM, if it does not, whether an additional term is introduced to avoid jump between QPSK and 16QAM

Agreement

Support 16-QAM for NPDSCH in PUR procedure

- CSI report is not supported/expected during PUR procedure.

Agreement

Support 16-QAM for NPDSCH and NPUSCH in PUR procedure,

- 16-QAM can be enabled/disabled by UE specific RRC signaling for NPDSCH and NPUSCH separately

  - The corresponding configurations and signaling details are up to RAN2

Agreement

The reserved state to indicate the use of 16QAM in DCI format N0 and DCI format N1 should be “1111”.

Agreement

Confirm the following working assumption:

Working Assumption

For downlink power allocation to support 16QAM:

- For inband deployments, a power ratio is signaled in addition to the signaling for standalone and guard-band deployments which in this case applies to “symbols with NRS” and “symbols without NRS nor CRS”.

  - the power ratio between NPDSCH EPRE and NRS EPRE in symbols with CRS is signaled

  - the signaling is UE specific

Note: “symbols with NRS” and “symbols without NRS nor CRS” have the same power.

Agreement

For the UE configured with 16-QAM for NPDSCH, the deployment of the carrier is signaled by operationModeInfo in MIB or inbandCarrierInfo in SIB/UE specific signaling.

Note: Existing agreement from RAN1#106e is "For the UE configured with 16-QAM for NPDSCH, the deployment of the carrier is signaled by operationModeInfo in MIB or inbandCarrierInfo in SIB", which is replaced by the updated agreement above from RAN1#106bis-e.

For eMTC 14-HARQ processes:

Working Assumption

For the joint encoding of “PDSCH Scheduling delay” and “HARQ-ACK delay” when Alt-2e is configured, the HARQ-ACK delay set has a size of:

- Alt-C:

  - 12 elements: HARQ-ACK delay set = {a, b, c, d, e, f, g, h, i, j, k, l} for the PDSCH Scheduling delay expression associated to the delay of 2.

  - 10 elements: HARQ-ACK delay set = {o, p, q, r, s, t, u, v, x, w} for the two PDSCH Scheduling delay expressions associated to the delay of 7.

  - FFS: The values of {a, b, c, d, e, f, g, h, i, j, k, l}, {o, p, q, r, s, t, u, v, x, w} where some of these elements may share the same value.

Conclusion:

How to implement/describe the states, e.g., table, resulting from the joint encoding solution of Alt-2e is left up to the Editor, based on the agreements for the PDSCH scheduling delay, HARQ-ACK delay and the WA confirmed for Alt-2e.

Agreement

The Rel-17 14 HARQ processes feature only applies to User Specific Search Space (USS).

Agreement (completed and confirmed on Oct 22nd)

In Rel-17, for the 14 HARQ processes feature the “HARQ-ACK process number” field uses 4-bits.

- The mapping associated to the 4-bits of this field is updated to include the newly added HARQ processes (i.e., 11th, 12th, 13th, and 14th HARQ processes).

Agreement

In Rel-17, for the 14 HARQ processes feature the following updates on the technical specification are to be performed.

- The maximum number of received PDSCH receptions pending HARQ-ACK is set to W = 12 (in Sect. 7.3.1 of TS 36.213) when the UE is configured with 14 HARQ processes.

Agreement

In Rel-17, one option will be down selected from Opt-2 and Opt-3 for the 14 HARQ processes feature the “Repetition number” field in RAN1#107e:

- Opt-2: 0-bits when the 14 HARQ processes feature is configured (i.e., 2-bits from this field become available for jointly-encoding purposes).

- Opt-3: 2-bits as in legacy.

In RAN1#107-e meeting, 19 contributions [21-39] were submitted, and the following agreements were achieved:

For NB-IoT 16-QAM:

Agreement

- The value range for the following RRC parameters is {-6, -4.77, -3, -1.77, 0, 1, 2, 3} dB

- Power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS for standalone and guard-band deployments, or in symbols without NRS nor CRS for in-band deployments.

- Power ratio of NPDSCH EPRE to NRS EPRE in symbols with CRS for in-band deployments.

Agreement

- Variant of option 1 is agreed in principle, detailed content in following table will be revisited.

  - A new table is defined for the combination of NPDCCH repetitions and NPDSCH MCS

- FFS: larger number NPDCCH repetition level

| Reported value | NPDCCH repetition level | NPDSCH transport block error probability not exceeding 0.1 |  |  | SNR |
| --- | --- | --- | --- | --- | --- |
|  |  | Modulation | Code rate x 1024 | Efficiency |  |
| noMeasurement | No measurement reporting | Out of range |  |  |  |
| candidateRep-A | 1 | QPSK (TBS index 4) | 221 | 0.4316 | -0.6 dB ([2]) |
| candidateRep-B | 2 | QPSK (TBS index 2) | 280 | 0.2737 | -3.6 |
| candidateRep-C | 4 | BPSK (TBS index 0) | 162 | 0.1579 | -6.6 |
| candidateRep-D | 8 | BPSK (TBS index 0, repetition 2) | 162 | 0.0789 | -9.6 |
| candidateRep-E | 16 | BPSK (TBS index 0, repetition 4) | 162 | 0.0395 | -12.6 |
| candidateRep-F | 32 | BPSK (TBS index 0, repetition 8) | 162 | 0.0198 | -15.6 |
| candidateRep-G | 1 | QPSK (TBS index 6) | 336.8 | 0.6579 | 1.0 dB ([3]) |
| candidateRep-H | 1 | QPSK (TBS index 8) | 453.6 | 0.8860 | 2.6 dB ([3]) |
| candidateRep-I | 1 | QPSK (TBS index 10) | 579.4 | 1.1316 | 4.1 dB ([3]) |
| candidateRep-J | 1 | QPSK (TBS index 12) | 759 | 1.4825 | 6.3 dB ([3]) |
| candidateRep-K | 1 | 16QAM (TBS index 14) | 487.3 | 1.9035 | 8.9 dB ([3]) |
| candidateRep-L | 1 | 16QAM (TBS index 16) | 541.2 | 2.1140 | 9.7 dB ([3]) |
| candidateRep-M | 1 | 16QAM (TBS index 18) | 658 | 2.5702 | 11.7 dB ([3]) |
| candidateRep-N | 1 | 16QAM (TBS index 20) | 783.7 | 3.0614 | 13.0 dB ([3]) |
| candidateRep-O | 1 | 16QAM (TBS index 21) | 837.6 | 3.2719 | 14.1 dB ([3]) |

Note: The (TBS index X) and SNR are just for information, based on standalone deployment. They will be removed once it’s agreed.

Agreement

- The table is taken as working assumption.

| Reported value | NPDCCH repetition level | NPDSCH transport block error probability not exceeding 0.1 |  |  |  | SNR |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Modulation | Code rate x 1024 | Repetition | Efficiency |  |
| noMeasurement | No measurement reporting | Out of range |  |  |  |  |
| candidateRep-A | 1 | QPSK (TBS index 4) | 221 | 1 | 0.4316 | -0.6 dB ([2]) |
| candidateRep-B | 2 | QPSK (TBS index 2) | 280 | 1 | 0.2737 | -3.6 |
| candidateRep-C | 4 | QPSK (TBS index 0) | 81 | 1 | 0.1579 | -6.6 |
| candidateRep-D | 8 | QPSK (TBS index 0) | 81 | 2 | 0.0789 | -9.6 |
| candidateRep-E | 16 | QPSK (TBS index 0) | 81 | 4 | 0.0395 | -12.6 |
| Working assumption candidateRep-F | 32 | QPSK (TBS index 0) | 81 | 8 | 0.0198 | -15.6 |
| candidateRep-G | 1 | QPSK (TBS index 6) | 336.8 | 1 | 0.6579 | 1.0 dB ([3]) |
| candidateRep-H | 1 | QPSK (TBS index 8) | 453.6 | 1 | 0.8860 | 2.6 dB ([3]) |
| candidateRep-I | 1 | QPSK (TBS index 10) | 579.4 | 1 | 1.1316 | 4.1 dB ([3]) |
| candidateRep-J | 1 | QPSK (TBS index 12) | 759 | 1 | 1.4825 | 6.3 dB ([3]) |
| candidateRep-K | 1 | 16QAM (TBS index 14) | 487.3 | 1 | 1.9035 | 8.9 dB ([3]) |
| candidateRep-L | 1 | 16QAM (TBS index 16) | 541.2 | 1 | 2.1140 | 9.7 dB ([3]) |
| candidateRep-M | 1 | 16QAM (TBS index 18) | 658 | 1 | 2.5702 | 11.7 dB ([3]) |
| candidateRep-N | 1 | 16QAM (TBS index 20) | 783.7 | 1 | 3.0614 | 13.0 dB ([3]) |
| candidateRep-O | 1 | 16QAM (TBS index 21) | 837.6 | 1 | 3.2719 | 14.1 dB ([3]) |

Note: The (TBS index X) and SNR are just for information, based on standalone deployment. They will be removed once it’s agreed.

Agreement:

The following working assumption is confirmed with following modification

- The table is endorsed.

| Reported value | NPDCCH repetition level | NPDSCH transport block error probability not exceeding 0.1 |  |  |  | SNR |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Modulation | Code rate x 1024 | Repetition | Efficiency |  |
| noMeasurement | No measurement reporting | Out of range |  |  |  |  |
| candidateRep-A | 1 | QPSK (TBS index 4) | 221 | 1 | 0.4316 | -0.6 dB ([2]) |
| candidateRep-B | 2 | QPSK (TBS index 2) | 280 | 1 | 0.2737 | -3.6 |
| candidateRep-C | 4 | QPSK (TBS index 0) | 81 | 1 | 0.1579 | -6.6 |
| candidateRep-D | 8 | QPSK (TBS index 0) | 81 | 2 | 0.0789 | -9.6 |
| candidateRep-E | 16 | QPSK (TBS index 0) | 81 | 4 | 0.0395 | -12.6 |
| Working assumption candidateRep-F | 32 | QPSK (TBS index 0) | 81 | 8 | 0.0198 | -15.6 |
| candidateRep-G | 1 | QPSK (TBS index 6) | 336.8 | 1 | 0.6579 | 1.0 dB ([3]) |
| candidateRep-H | 1 | QPSK (TBS index 8) | 453.6 | 1 | 0.8860 | 2.6 dB ([3]) |
| candidateRep-I | 1 | QPSK (TBS index 10) | 579.4 | 1 | 1.1316 | 4.1 dB ([3]) |
| candidateRep-J | 1 | QPSK (TBS index 12) | 759 | 1 | 1.4825 | 6.3 dB ([3]) |
| candidateRep-K | 1 | 16QAM (TBS index 14) | 487.3 | 1 | 1.9035 | 8.9 dB ([3]) |
| candidateRep-L | 1 | 16QAM (TBS index 16) | 541.2 | 1 | 2.1140 | 9.7 dB ([3]) |
| candidateRep-M | 1 | 16QAM (TBS index 18) | 658 | 1 | 2.5702 | 11.7 dB ([3]) |
| candidateRep-N | 1 | 16QAM (TBS index 20) | 783.7 | 1 | 3.0614 | 13.0 dB ([3]) |
| candidateRep-O | 1 | 16QAM (TBS index 21) | 837.6 | 1 | 3.2719 | 14.1 dB ([3]) |

Conclusion

There’s no consensus on the introduction of CSI reference resource in NB-IoT in Rel-17, whether/how to introduce CSI reference resource is up to RAN4.

R1-2112971 LS on channel quality reporting for NB-IoT From: RAN1, To: RAN2, RAN4

Agreement

The following working assumption is confirmed.

For the new term ∆TF,c introduced for power control of NPUSCH,

- Reuse the LTE definition simplified for NB-IoT: ∆TF,ci=10log102BPRE∙Ks-1 for Ks=1.25 and ∆TF,ci=0 for Ks=0, where Ks is given by higher layer parameter deltaMCS-Enabled, and BPRE=KNRE where K is the code block size.

- FFS: whether the new term applies to QPSK when configured with 16QAM, if it does not, whether an additional term is introduced to avoid jump between QPSK and 16QAM

For eMTC 14 HARQ processes

Agreement

Confirm the following Working Assumption:

Working Assumption

For the joint encoding of “PDSCH Scheduling delay” and “HARQ-ACK delay” when Alt-2e is configured, the HARQ-ACK delay set has a size of:

- Alt-C:

    - 12 elements: HARQ-ACK delay set = {a, b, c, d, e, f, g, h, i, j, k, l} for the PDSCH Scheduling delay expression associated to the delay of 2.

    - 10 elements: HARQ-ACK delay set = {o, p, q, r, s, t, u, v, x, w} for the two PDSCH Scheduling delay expressions associated to the delay of 7.

FFS: The values of {a, b, c, d, e, f, g, h, i, j, k, l}, {o, p, q, r, s, t, u, v, x, w} where some of these elements may share the same value.

Agreement

For the joint encoding of “PDSCH Scheduling delay” and “HARQ-ACK delay” when Alt-2e is configured, the HARQ-ACK delay sets consist of the following elements:

- Alt-C2:

    - 12 elements: HARQ-ACK delay set = {4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 15, 17} for the PDSCH Scheduling delay expression associated to the delay of 2.

    - 10 elements: HARQ-ACK delay set = {4, 5, 10, 12, 13, 14, 15, 16, 17, 18} for the two PDSCH Scheduling delay expressions associated to the delay of 7.

Agreement

In Rel-17, for the 14 HARQ processes feature the “Repetition number” field is:

         Opt-3: 2-bits as in legacy

Note: Further optimization for using Repetition number” field is not pursued

Agreement

In Rel-17, for the 14 HARQ processes feature the “HARQ-ACK delay” field is:

         0-bits when the 14 HARQ processes feature is configured (i.e., 3-bits from this field become available for jointly-encoding purposes).

Rel-17 NB-IoT and eMTC related MAC CE impact is captured in the following LS:

Agreement

LS to RAN2 on Rel-17 MAC-CE is endorsed in R1-2112842.

#### 2.1.2	Remaining Open issues

None. The RAN1 core part is complete.

## 2.2	RAN2

#### 2.2.1	Agreements

Contributions [40]– [65] were submitted to RAN2#116-e meeting.

NB-IoT neighbour cell measurements and corresponding measurement triggering before RLF

RAN2 discussed NB-IoT neighbour cell measurements and corresponding measurement triggering before RLF and made the following agreements:

RAN2#116-e agreements: - NW signals two separate thresholds for intra- and inter-frequency measurements. - The values of s-SearchDeltaP and TSearchDeltaP may be different in RRC_CONNECTED and RRC_IDLE, they are signalled in a separate set of parameters. - s-SearchDeltaP has the same value range as the existing RRC_IDLE parameter - FFS how to specify the state change - [FFS] An indication that the UE starts measurement is not introduced. - No enhancement is introduced to have a shorter T310 timer for mobile UEs supporting connected mode measurement. - For RRC_CONNECTED state, TSearchDeltaP is configured via SIB. - Working assumption: For RRC_CONNECTED state, TSearchDeltaP range is 10 – 60 seconds. - For RRC_CONNECTED state, no default value for TSearchDeltaP. - No limit for how long UE can remain in relaxed neighbour cell monitoring state while it is in RRC_CONNECTED state. - For RRC_CONNECTED state, the RRC_IDLE state SSearchDeltaP is not used if the RRC_CONNECTED state SSearchDeltaP is not provided. - Relaxed neighbour cell monitoring is enabled in RRC_CONNECTED state if TSearchDeltaP and SSearchDeltaP for RRC_CONNECTED state are provided.

RAN2 has agreed on an email discussion until next meeting:

[Post116-e][310][NBIOT/eMTC R17] RLF measurements (Qualcomm)

Scope: Remaining details of relaxed monitoring

Intended outcome: report to the next meeting

Deadline: long

NB-IoT carrier selection based on the coverage level and associated carrier specific configuration

RAN2 discussed NB-IoT carrier selection based on the coverage level and associated carrier specific configuration and made the following agreements.

RAN2#116-e agreements: - DRX is not used a criterion that needs to be explicitly considered for paging carrier selection. - Option 1c with Alt2 (fallback when cell change) is supported

RAN2 has agreed on an email discussion until next meeting:

[Post116-e][311][NBIOT/eMTC R17] NB-IoT carrier selection (ZTE)

Scope: open issues and solution details

Intended outcome: report to the next meeting

Deadline: long

NB-IoT: 16-QAM for unicast in UL and DL

RAN2 discussed 16-QAM for unicast in UL and DL and made the following agreements.

RAN2#116-e agreements: - Confirm the working assumption of 12000 bytes for DL 16QAM for NB-IoT

eMTC: 14-HARQ processes in DL, for HD-FDD Cat M1 UEs

RAN2 did not discuss 14-HARQ processes in DL, for HD-FDD Cat M1 UEs.

eMTC: maximum DL TBS of 1736 bits

RAN2 discussed maximum DL TBS of 1736 bits and made the following agreements.

RAN2#116-e agreements: - No change to existing L2 buffer requirements for supporting 1736bits TBS for eMTC

#### 2.2.2	Remaining Open issues

- For NB-IoT, support of NB-IoT neighbour cell measurements and corresponding measurement triggering before RLF, details of relaxed monitoring criteria and whether to have a UE indication that it starts/stops measurements.

- For NB-IoT, support of NB-IoT carrier selection based on the coverage level and associated carrier specific configuration, details of the solution and associated signaling.

- For NB-IoT, 16-QAM for unicast in DL and UL, CQI reporting pending on RAN1/ RAN4 inputs.

- For all topics, UE capabilities and FDD/TDD differentiation pending on RAN1/ RAN4 inputs.

## 2.3	RAN3

In RAN3#114-e meeting, 10 contributions were submitted in [40] ~ [49], and the following progress was achieved:

#### 2.3.1	Agreements

Contributions [66-75] were submitted to RAN3#114-e meeting.

Support for Carrier Selection and Carrier Specific Configuration

- Agree to wait RAN2 decision between RAN2 option 1 or option 2 to decide the RAN3 solution.

#### 2.3.2	Remaining Open issues

- Support of Carrier Selection and Carrier Specific Configuration.

## 2.4	RAN4

#### 2.4.1	Agreements

For RF core part:

In RAN4#101-e meeting, 9 contributions [82-90] were submitted, and the following agreements were achieved:

- CR IBE mask and MPR for NB-IoT 16QAM (R4-2119952) is agreed, and the RF requirements for UE in support of 16QAM are completed.

- WF on remaining issues for NB-IoT 16QAM (BS side) is agreed in R4-2119948.

For RRM core part:

In RAN4#101-e meeting, 6 contributions [76-81] were submitted, and the following agreements were achieved:

Deprioritize defining CONNECTED mode neighbour cell measurement requirements when the target cell is in enhanced coverage

#### 2.4.2	Remaining Open issues

For RF core part:

NB-IoT:

FFS rated output power for 16QAM, down-select from two options: one declaration or up to 2 declarations.

LTE-MTC:

For UEs supporting PUSCH sub-PRB resource allocation, study and if found feasible for the concluded options, specify support power reduction for PRACH, PUCCH, and full-PRB PUSCH, with a maximum reduction of e.g. 3 dB below sub-PRB PUSCH power.

For RRM core part:

Discuss the relation between carriers for connected mode neighbour cell measurement and carriers detected in IDLE mode. [NB-IoT]

Discuss the minimum interval between two measurements/samples for inter-frequency measurement [NB-IoT]

Discuss the minimum length of single measurement occasions for inter-frequency measurement [NB-IoT]

Discuss the detailed requirements for intra-frequency/inter-frequency measurement and multiple carriers operations. [NB-IoT]

For performance part:

Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above-mentioned enhancements and core requirements. [NB-IoT][LTE-MTC]

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2108777	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

- R1-2109174	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

- R1-2109314	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-2109320	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

- R1-2109337	Discussion on UL and DL 16QAM for NB-IoT	ZTE, Sanechips

- R1-2109559	Remaining Issues on supporting 16QAM in NB-IOT R17	MediaTek Inc.

- R1-2110316	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2110459	Feature lead summary #1 on 106bis-e-LTE-Rel17-NB-IoT-eMTC-01	Moderator (Huawei)

- R1-2110555	Feature lead summary #2 on 106bis-e-LTE-Rel17-NB-IoT-eMTC-01	Moderator (Huawei)

- R1-2108778	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

- R1-2109175	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

- R1-2109315	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

- R1-2109338	Remaining issues on 14-HARQ processes in DL for eMTC	ZTE, Sanechips

- R1-2110317	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson

- R1-2110413	Feature Lead Summary [106bis-e-LTE-Rel17-NB-IoT-eMTC-02] - first checkpoint	Moderator (Ericsson)

- R1-2110414	Feature Lead Summary [106bis-e-LTE-Rel17-NB-IoT-eMTC-02] - final checkpoint	Moderator (Ericsson)

- R1-2110318	On the support of 16-QAM for unicast in UL and DL for TDD NB-IoT	Ericsson

- R1-2110372	Discussion on RRC parameters for max DL TBS of 1736 bits	Huawei, HiSilicon

- R1-2110650	Feature lead summary on 106bis-e-R17-RRC-NB-IoT-eMTC	Moderator (Huawei)

- R1-2110691	RAN1 agreements of Additional enhancements for NB-IoT and LTE-MTC	WI rapporteur (Huawei)

- R1-2112747	Feature lead summary #3 on 107-e-LTE-Rel17-NB-IoT-eMTC-01	Moderator (Huawei)

- R1-2112651	Feature lead summary #2 on 107-e-LTE-Rel17-NB-IoT-eMTC-01	Moderator (Huawei)

- R1-2112576	Feature lead summary #1 on 107-e-LTE-Rel17-NB-IoT-eMTC-01	Moderator (Huawei)

- R1-2110857	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

- R1-2111070	Discussion on 16QAM for NB-IoT	ZTE, Sanechips

- R1-2111133	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-2111449	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

- R1-2112001	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

- R1-2112300	Discussion on CQI table and NPUSCH power control parameter for 16QAM	MediaTek Inc.

- R1-2112361	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2112517	Summary#1st check point [107-e-LTE-Rel17-NB-IoT-eMTC-02] on support additional PDSCH scheduling	Moderator (Ericsson)

- R1-2110858	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

- R1-2111071	Remaining issues on 14-HARQ processes in DL for eMTC	ZTE, Sanechips

- R1-2111134	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

- R1-2111450	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

- R1-2112362	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson

- R1-2111939	Further considerations on Rel-17 NB-IoT and eMTC enhancements	Huawei, HiSilicon

- R1-2112363	On the support of 16-QAM for unicast in UL and DL in TDD NB-IoT	Ericsson

- R1-2112877	Feature lead summary on 107-e-R17-RRC-NB-IoT-eMTC	Moderator (Huawei)

- R2-2109911	Report of email discussion [302] [NBIOT/eMTC R17] Carrier Selection	Ericsson

- R2-2109912	Analysis of Rmax based solution and carrier-based solution	Ericsson

- R2-2109913	Discussion on connected mode measurement in NB-IoT	Ericsson

- R2-2109914	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R2-2110109	Remaining FFSs on RLF measurements	ZTE Corporation, Sanechips

- R2-2110110	Option1c for CEL-based paging carrier selection	ZTE Corporation, Sanechips

- R2-2110111	Remaining FFSs on 16QAM for NB-IoT	ZTE Corporation, Sanechips

- R2-2110112	Remaining FFSs on 1736bits TBS for eMTC	ZTE Corporation, Sanechips

- R2-2110147	Network assistance for Re-establishment enhancement	Nokia, Nokia Shanghai Bells

- R2-2110148	Paging strategy impacts for coverage based paging carrier selection	Nokia, Nokia Shanghai Bells

- R2-2110149	Network configuration for paging carrier selection based on coverage level	Nokia, Nokia Shanghai Bells

- R2-2110191	Further discussion on enhanced paging carrier selection	NEC Corporation

- R2-2110473	L2 buffer size calculations for eMTC and NB-IoT enhancements	Huawei, HiSilicon

- R2-2110474	Relaxed monitoring in RRC connected mode	Huawei, HiSilicon

- R2-2110475	Discussion on coverage based paging carrier	Huawei, HiSilicon

- R2-2110476	Summary of [301] RLF measurements (Huawei)	Huawei

- R2-2110477	Running CR: Introduction of Rel-17 enhancements for NB-IoT and eMTC	Huawei

- R2-2110692	[Running CR] Introduction of NB-IoT/eMTC Enhancements	Qualcomm Incorporated

- R2-2110693	Consideration on open issues for neighbour cell measurement in RRC connected state	Qualcomm Incorporated

- R2-2110694	Further consideration on open issues for coverage-based paging carrier selection	Qualcomm Incorporated

- R2-2110695	Signalling for coverage-based paging carrier selection	Qualcomm Incorporated

- R2-2110800	On remaining issues of 16QAM	Nokia Solutions & Networks (I)

- R2-2111113	Discussion on details of paging carrier selection options	MediaTek Inc.

- R2-2111393	Report of [AT116e][303][NBIOT/eMTC] RLF measurements (Qualcomm)	Qualcomm Incorporated

- R2-2111394	Report of [AT116-e][304][NBIOT/eMTC] NB-IoT carrier selection (ZTE)	ZTE (email discussion rapporteur)

- R2-2111396	RAN2 agreements for Rel-17 NB-IoT and LTE-MTC	Ericsson

- R3-214738	Discussion on Carrier Selection and Carrier Specific Configuration	ZTE

- R3-214739	Introduction of CEL based paging carrier selection	ZTE

- R3-214783	Support of Carrier Selection based on coverage level	Nokia, Nokia Shanghai Bell

- R3-214784	Support of Carrier Selection based on coverage level	Nokia, Nokia Shanghai Bell

- R3-215004	(TP to TS36.413) Consideration on Carrier Selection and Carrier Specific Configuration	Huawei

- R3-215005	Support of CE based carrier selection	Huawei

- R3-215419	Discussion on Rel-17 NB-IoT Carrier selection	Ericsson

- R3-215420	Enhancement on Intra eNB CP Relocation for NB-IoT	Ericsson

- R3-215897	CB: # NBIoTMTC1_CarrierSelect - Summary of email discussion	Nokia - moderator

- R3-216219	CB: # NBIoTMTC2_Others - Summary of email discussion	Ericsson – moderator

- R4-2120383      Email discussion summary for [101-e][241] NB_IOTenh4_LTE_eMTC6_RRM_NWM moderator (Huawei)

- R4-2120340	WF on RRM requirements for Rel-17 NB-IoT and LTE-MTC Huawei, HiSilicon

- R4-2117496	On NB-IoT neighbor cell measurements in RRC_CONNECTED Qualcomm Incorporated

- R4-2118853	Discussion on RRM requirements for NB Huawei, Hisilicon

- R4-2119064	Discussions on RRM requirements for release 17 NB-IoT Ericsson

- R4-2119594	Discussion on neighbour cell measurements in NB-IoT RRC_CONNECTED state Nokia, Nokia Shanghai Bell

- R4-2117192	CR IBE mask and MPR for NB-IoT 16-QAM	Nokia, Nokia Shanghai Bell

- R4-2117250	Proposals on BS RF requirements for support of 16QAM in NB-IoT	Nokia, Nokia Shanghai Bell

- R4-2117620	On max power reduction for PRACH, PUCCH, and full-PRB PUSCH	Sony

- R4-2118544	Remaining issues for NB-IoT 16QAM BS RF requirements	Huawei,HiSilicon

- R4-2118545	PC6 MPR for NB-IoT 16QAM	Huawei,HiSilicon

- R4-2118996	BS RF impact analysis on R17 NB_IoT	Ericsson

- R4-2118997	RF impact analysis on R17 eMTC WID	Ericsson

- R4-2119952	CR IBE mask and MPR for NB-IoT 16-QAM	Nokia, Nokia Shanghai Bell

- R4-2119948	WF on remaining issues for NB-IoT 16QAM	Huawei, HiSilicon

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template