3GPP TSG RAN meeting #94e										       RP-21xxxx

Electronic Meeting, December 6 - 17, 2021						  (revision of RP-213075)

Status Report to TSG

Agenda item:			9.3.1.5

| WI / SI Name | NR coverage enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_cov_enh |  |  |  |  |
| Unique ID | 900061 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-211566 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 03/2022 | Performance part: 09/2022 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 85% (RAN1 99%, RAN2 75%, RAN4 65%) | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Jianchi Zhu |
|  | Company | China Telecom |
|  | Email | zhujc@chinatelecom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

RAN1 #106b-e

Final LS R1-2110642 is endorsed.

Agreement

The higher layer parameters for Rel-17 NR in R1-2110573 is endorsed.

Enhancements on PUSCH repetition type A:

Working Assumption is confirmed

Working Assumption

The maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32

Conclusion:

For CG-PUSCH repetitions counted on the basis of available slots, all the K transmission occasions including the 1st transmission occasion are determined on the basis of available slots.

Agreement

For CG-PUSCH repetition Type A with the counting based on available slots, the R16 existing restrictions as defined in Clause 6.1.2.3.1 of TS38.214 at least on the initial transmission of a transport block are applied, assuming the K repetitions of R17 determined based the rule of counting available slots.

Observation

- Whether or not the counting based on available slots is applicable only to unpaired spectrum is not discussed under AI 8.8.1.1 in RAN1#106bis-e. Discussions on how HD-FDD RedCap UEs support the available slot counting may take place in AI 8.8.1.1 in RAN1#107-e, depending on the progress of RedCap WI discussions.

Agreement

- For the K repetitions of DG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the slot which is determined by the slot offset K2.

  - No RAN1 spec impact is expected in terms of the relation with the slot which is determined by the slot offset K2.

  - Note: The available slot determination is to be specified.

- For the K repetitions of CG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the first slot which is determined by at least ConfiguredGrantConfig.

  - No RAN1 spec impact is expected in terms of the relation with the first slot which is determined by at least ConfiguredGrantConfig.

  - Note: The available slot determination is to be specified.

Agreement

- Only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst are considered for the determination of available slots.

  - Any other RRC configuration is not considered for the determination of available slots.

Agreement

- The existing restriction “The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P” applies to both the counting based on physical slots and the counting based on available slots.

- The above “the time duration for the transmission of K repetitions” means the time duration between the start of the 1st slot of the K repetitions and the end of the last slot of the K repetitions for any instance of a CG period.

Agreement

A single RRC paramter AvailableSlotCounting that applies to both DG-PUSCH and CG-PUSCH is introduced.

Agreement

Two enhancements are configured separately (simultaneous configurations allowed).

- If the new Rel-17 RRC parameter “AvailableSlotCounting” set to “enabled” is configured, numberofrepetitions-r17 may or may not be configured and the counting based on available slots is used.

- Otherwise, numberofrepetitions-r17 may or may not be configured and the counting based on physical slots is used.

TB processing over multi-slot PUSCH:

Agreement

- For transmission power determination of TBoMS transmission in Rel-17, RAN1 to down-select one of the following two options:

- Option 1: The transmission power determination of TBoMS should be based on all the REs allocated in one available slot for the TBoMS transmission, excluding the overhead of reference signals

- Option 2: The transmission power determination of TBoMS should be based on all the REs allocated in the N available slots for the TBoMS transmission, excluding the overhead of reference signals.

- FFS: details on BPRE

Agreement

The number of MIMO layers (rank) for TBoMS transmission in Rel-17 is limited to 1.

Agreement

For a single TBoMS transmission and TBoMS repetitions in Rel-17, at least the legacy Rel-15/16 inter-slot frequency hopping framework used in PUSCH repetition Type A is supported.

- FFS: other frequency hopping schemes.

Agreement

- The number N of allocated slots for TBoMS is indicated via a new column added to the TDRA table configured via PUSCH-TimeDomainAllocationList. The column for configuring the number of repetitions in the TDRA for Rel-17 PUSCH repetition Type A, i.e., numberOfRepetitions, is used for indicating the number of repetitions M of a single TBoMS, when TBoMS transmission is enabled.

- FFS: supported values of N and M.

- FFS: how to enable the TBoMS transmission

- FFS: details of retransmission of TBoMS

Agreement

For the repetition of a single TBoMS transmission, redundancy versions (RVs) are cycled across the TBoMS repetitions. The legacy Rel-15/16 RV sequences and RV index indication are reused.

Conclusion

Values 1<K<N for the scaling factor to calculate N_info for TBS determination for TBoMS transmission in Rel-17 are not supported.

Agreement

At least the following values are supported in Rel-17 for the number N of allocated slots for the single TBoMS:

FFS: whether N=1 is also supported depends on how TBoMS transmission feature is enabled (or disabled)

FFS: other values, if any.

FFS: further constraints on N*M

Agreement

The following values are supported in Rel-17 for the number M of repetitions of the single TBoMS:

FFS: further constraints on N*M, e.g., N*M is a valid value according to agreements in AI 8.8.1.1

Agreement

BPRE for TBOMS is calculated as  where N is the number of slots allocated for a single TBOMS and  is the number of allocated REs in one allocated slot of a single TBOMS.

Note: How this equation or its equivalent is captured in the specification is left to the editor

Agreement

For a single TBoMS transmission and TBoMS repetitions in Rel-17, the legacy Rel-15/16 intra-slot frequency hopping framework used in PUSCH repetition Type A is supported.

- FFS: other frequency hopping schemes.

Working Assumption

For TBoMS in Rel-17, the following is supported:

- Bit interleaving is performed per slot.

·       The index of the starting coded bit for each transmitted slot is predetermined prior to the start of the TBoMS transmission.

- Transmission is limited to one CB only.

- FFS: whether UCI multiplexing bits or cancellation/dropping of coded bits, if any, have to be known prior to the determination of the index of the starting coded bit for each transmitted slot or not

- FFS: Performance with UCI multiplexing on single and multiple slots of a single TBoMS

Note: How UCI multiplexing and cancellation/dropping of coded bits influence the sequence of coded bits transmitted in each slot of a single TBOMS is to be further discussed. Some knowledge on UCI to be multiplexed or cancellation/dropping of coded bits in each slot of a single TBOMS may be known prior to the start of a single TBOMS transmission. How this is to be handled is to be discussed further.

Agreement

For the bit selection for each transmitted slot for TBoMS, one of the following is to be down selected in RAN1 #107-e for determining the index of the starting coded bit in the circular buffer:

- Option B: the index of the starting coded bit in the circular buffer is the index continuous from the position of the last bit selected in the previous allocated slot.

- Option C: the index of the starting coded bit in the circular buffer is the index continuous from the position of the last bit selected in the previous allocated slot, regardless of whether UCI multiplexing occurred in the previous allocated slot or not.

FFS: whether the index of the starting coded bit for each transmitted slot is expressed as a multiple integer of the lifting size Zc

Note: Dropping/cancellation rules are not considered for the starting bit position determination in both Option B and Option C.

Agreement

For TBoMS transmission in Rel-17:

- TBoMS feature is enabled (or disabled) by configuring (or not) the number of allocated slots for a single TBoMS (N) in a row of the TDRA table.

  - TBoMS transmission is enabled when N>1, where N is the number of allocated slots for a single TBoMS.

  - Single-slot PUSCH transmission is enabled when N=1.

  - Supported combinations of N and M that can be configured in the TDRA table, these combinations are constrained by retransmission are to be further discussed

Joint channel estimation for PUSCH:

Agreement:

- For PUSCH repetition type A counting based on physical slots

  - The start of the first configured TDW is the first physical slot for the first PUSCH transmission.

  - The end of the last configured TDW is the last physical slot for the last PUSCH transmission.

- For PUSCH repetition type A counting based on available slots

  - The start of the first configured TDW is the first available slot for the first PUSCH transmission.

  - The end of the last configured TDW is the last available slot for the last PUSCH transmission.

  - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Conclusion:

- Joint channel estimation over PUSCH transmissions across non-consecutive slots is not supported in Rel-17.

Agreement:

Down-select one of the following options in this meeting:

Option 1:

- The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

Option 1’:

- The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

    - If L is not configured, default behavior should be defined, e.g., the configured TDW length is equal to all repetitions

Option 3’:

- Whether the window length L of the configured TDW can be longer than maximum duration is subject to UE capability.

  - If UE is capable of L being longer than maximum duration,

    - The maximum value of the window length L of the configured TDW is the duration of all repetitions.

      - FFS: whether L cannot be other values other than the duration of all repetitions, if it is longer than the maximum duration.

    - If L is longer than the maximum duration, UE does not expect dynamic events.

      - FFS: details of dynamic events

Agreement

- For DG-PUSCH, Type1 CG-PUSCH and Type2 CG-PUSCH, the window length L of the configured TDW is at least configured by RRC.

- FFS: For DG-PUSCH and Type2 CG-PUSCH, whether the window length L of the configured TDW can be indicated by DCI or indicated by TDRA table with one additional entry.

Agreement

- The window length L of the RRC configured TDW is configured separately for PUSCH and PUCCH.

  - For PUSCH, L is configured per BWP.

- FFS whether the window length L can be configured with each row in the TDRA table

Agreement

- For PUSCH repetition type A counting based on physical slots

  - The configured TDWs are consecutive, where the start of other configured TDWs is the first physical slot right after the last physical slot of a previous configured TDW.

- For PUSCH repetition type A counting based on available slots

  - The configured TDWs are determined based on available slots, where start of a configured TDWs is the first available slot after the last available slot of a previous configured TDW.

  - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Working assumption:

- The start of the first actual TDW is the first symbol (at least determined by TDRA table) for the first PUSCH transmission in an available slot within the configured TDW.

- The end of the actual TDW is

  - the last symbol (at least determined by TDRA table) for the last PUSCH transmission in an available slot within the configured TDW if the actual TDW reaches the end of the last PUSCH transmission within the configured TDW.

  - the last symbol (at least determined by TDRA table) of the PUSCH transmission right before the event if an event occurs that violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

- For UE capable of restarting DM-RS bundling, the start of the new actual TDW is the first symbol (at least determined by TDRA table) for PUSCH transmission after the event violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

Agreement

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over back-to-back PUSCH transmissions for one TB processed over multiple slots

    - It’s subject to UE capability

    - if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A

Agreement

- For non-back-to-back PUSCH transmissions across consecutive slots (no uplink transmission in the middle of two PUSCH transmissions), support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over non-back-to-back PUSCH transmissions for one TB processed over multiple slots

    - It’s subject to UE capability

    - if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A

Agreement

Down-select one of the following options:

- Option 1: If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events.

- Option 2: UE capability of restarting DMRS bundling is applied to both semi-static events and dynamic events.

Agreement

- Support at least the following events that violate power consistency and phase continuity.

  - Dropping/cancellation based on Rel-15/16 collision rules.

  - FFS: Rel-17 collision rules.

  - DL slot or DL reception/monitoring based on semi-static DL/UL configuration for unpaired spectrum.

  - FFS: Other UL transmission in between PUSCH/PUCCH transmissions.

  - Gap between two PUSCH/PUCCH transmissions exceeds 13 symbols.

  - FFS: Transmission parameters need to be changed due to network-indicated operations, including: Tx power, UL beam/TPMI, and RB allocation.

  - FFS: TPC command.

  - FFS: TA adjustment.

  - FFS: The actual TDW reaches the maximum duration.

  - FFS: Frequency hopping.

  - FFS: Precoder cycling.

  - FFS: other events.

  - FFS: whether events are semi-static events or dynamic events.

  - FFS: the time duration of an event.

Agreement

Introduce two RRC parameters to indicate enabling of DM-RS bundling and the window length of the configured TDW respectively.

Agreement

Introduce a new RRC parameter for when UE restarts a PUSCH bundling window.

PUCCH enhancements:

Agreement

Dynamic signaling to enable/disable DMRS bundling for PUCCH or PUSCH repetitions is not supported in Rel-17.

Agreement:

For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, a UE perform the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering. One option of the following options is to be selected.

- Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination”

- Option 2: “configured TDW determination” -> “hopping intervals determination” -> “actual TDW determination”

- Option 4: “configured TDW determination” -> “actual TDW determination” and “hopping intervals determination”

Note: option 1, and 2, and 3 assume a hopping interval can be different than an actual TDW. Option 4 assumes a hopping interval is the same as an actual TDW.

Agreement

Support dynamic PUCCH repetition factor indication for all PUCCH formats including format 0, 1, 2, 3, 4 with a unified mechanism as agreed in RAN1#106e under agenda 8.8.2.

Note: it does not impact the discussion of slot level or sub-slot level repetition

Type A PUSCH repetitions for Msg3:

LS R1-2110585 is endorsed.

Working Assumption

Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission.

- Alt 1: If TDRA information field is chosen, Option 2 is supported.

  - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]}

- Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows.

  - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values.

    - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]}

Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field.

Agreement

Include the following into the reply LS to R1-2108712(R2-2109195).

RAN1 thinks at least the number of preambles per SSB per RO for request of Msg3 repetition, is needed. It’s up to RAN2 whether to indicate the start of preamble index for request of Msg3 repetition with shared RO.

Agreement

Include the following into the reply LS to R1-2108712(R2-2109195).

- From RAN1 perspective, there is no need to separately configure the following legacy RACH parameters configured in RACH-ConfigCommon for requesting Msg3 PUSCH repetition with shared RO on a given UL carrier.

- prach-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- totalNumberOfRA-Preambles

- ssb-perRACH-OccasionAndCB-PreamblesPerSSB

- FFS: rsrp-ThresholdSSB

- rsrp-ThresholdSSB-SUL

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

Conclusion:

There is no consensus to additionally support intra-slot frequency hopping for Msg3 PUSCH with repetition in Rel-17.

Note: intra-slot FH is supported when a UE is scheduled Msg3 PUSCH without repetition.

Agreement

Include the following into the reply LS to R1-2108712(R2-2109195)

- From RAN1 perspective, it can be beneficial to separately configure rsrp-ThreBsholdSS for requesting Msg3 PUSCH repetition with shared RO on a given UL carrier.

Agreement

If UE is indicated with Msg3 PUSCH with repetition, the frequency hopping flag information field in UL RAR grant or DCI format 0_0 with CRC scrambled by TC-RNTI is reused to enable/disable inter-slot frequency hopping.

Agreement

The Rel-15/16 Msg3 PUSCH collision handling rules are reused for transmission of Msg3 PUSCH repetition in an available slot.

- FFS whether collision with downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated is an exceptional case, i.e., Msg3 PUSCH repetition cannot be canceled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated in Rel-17.

- FFS: Rel-17 Msg3 PUSCH collision rules are also applied if introduced in other WI(s)

RAN1 #107-e

Enhancements on PUSCH repetition type A:

Agreement

- The counting based on available slots is applicable to unpaired spectrum, paired spectrum and SUL

  - For paired spectrum and SUL except HD-FDD, all slots are considered as available slots in the first step of determining the available slots.

Agreement

- For HD-FDD RedCap Ues supporting the counting based on available slots.

  - For CG-PUSCH, ssb-PositionsInBurst is used in the first step of determining of available slots.

    - A slot is not counted in the number of available slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionInBurst.

  - FFS: For DG-PUSCH

  - Note: Neither tdd-UL-DL-ConfigurationCommon nor tdd-UL-DL-ConfigurationDedicated is configured for FDD.

Agreement

- Rel-17 does not support numberOfRepetitions-r17 for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0.

- repK-r17 supporting up-to-32 repetitions is introduced and is applicable to Type 1 CG-PUSCH and Type 2 CG-PUSCH (irrespective of the activating DCI format).

  - Note: No RAN1 spec impact is expected.

  - The possible values of repK-r17 includes 16 and 32. FFS: other values.

- numberOfRepetitions-r17 is not applicable to Type 1 CG-PUSCH repetition type A.

Agreement

- All the following combinations support the counting based on available slots.

  - DG-PUSCH with Rel-15 repetition factor

  - Type-1 CG-PUSCH with Rel-15 repetition factor

  - Type-2 CG-PUSCH with Rel-15 repetition factor

  - DG-PUSCH with Rel-16 repetition factor

  - Type-2 CG-PUSCH with Rel-16 repetition factor

  - DG-PUSCH with Rel-17 repetition factor

  - Type-1 CG-PUSCH with Rel-17 repetition factor, if supported in Issue#1-1

  - Type-2 CG-PUSCH with Rel-17 repetition factor

Conclusion:

- Rel-17 PUSCH repetition Type A with K>1 does not support PUSCH transmission without UL-SCH.

Agreement

- For repK-r17,

  - The value range of repK-17 is {1, 2, 4, 8, 12, 16, 24, 32}.

  - repK-r17 is included in ConfiguredGrantConfig.

  - When repK-r17 is provided, the legacy repK is not provided.

Agreement

- For HD-FDD RedCap Ues supporting the counting based on available slots.

- For DG-PUSCH, ssb-PositionsInBurst is used in the first step of determining of available slots.

  - A slot is not counted in the number of available slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionInBurst.

- Note: Neither tdd-UL-DL-ConfigurationCommon nor tdd-UL-DL-ConfigurationDedicated is configured for FDD.

TB processing over multi-slot PUSCH:

Agreement

A single RV is used to transmit a single TBoMS.

Note: It is common assumption for option B and option C for “Starting bit in each slot for the single TBoMS”

Note: below working assumption does not need confirm.

Working Assumption

Single TBoMS structure of Option 3 is selected

- Option 3: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using a single RV.

FFS: how the single RV is rate matched across single or multiple TOTs, e.g., rate matched for each TOT, rate matched for all the TOTs, rate matched for each slot and so on.

Agreement

The working assumption is confirmed.

Working Assumption

For TBoMS in Rel-17, the following is supported:

- Bit interleaving is performed per slot.

·       The index of the starting coded bit for each transmitted slot is predetermined prior to the start of the TBoMS transmission.

- Transmission is limited to one CB only.

- FFS: whether UCI multiplexing bits or cancellation/dropping of coded bits, if any, have to be known prior to the determination of the index of the starting coded bit for each transmitted slot or not

- FFS: Performance with UCI multiplexing on single and multiple slots of a single TBoMS

Note: How UCI multiplexing and cancellation/dropping of coded bits influence the sequence of coded bits transmitted in each slot of a single TBOMS is to be further discussed. Some knowledge on UCI to be multiplexed or cancellation/dropping of coded bits in each slot of a single TBOMS may be known prior to the start of a single TBOMS transmission. How this is to be handled is to be discussed further.

Conclusion:

There is no consensus in RAN1 to introduce any restriction on the combinations of N and M that can be configured in the TDRA table, other than the already agreed N*M <= 32 restriction.

Agreement

- For TBoMS, UCI is multiplexed on the individual overlapping slot for UL transmission in one carrier

- FFS: timeline requirements

- FFS: details on the calculation of the number of coded modulation symbols per layer for UCI multiplexing on a single TBoMS.

- Note: no new UCI multiplexing mechanism other than existing puncturing or rate-matching is introduced for TBoMS in Rel-17.

Agreement

For TBoMS repetitions, if the parameter numberOfRepetitions is not configured in the TDRA table, then the number of repetitions M of a single TBoMS is equal to 1.

Agreement

For a configured grant type 2, if M=1, or if M>1 and the configured grant is configured with startingFromRV0 set to 'off', the initial transmission of the transport block may only start at the first slot of the N*M slots determined as available for PUSCH transmission of TBoMS. Otherwise, the initial transmission of the transport block may start at

- The first slot of the N*M slots determined as available for PUSCH transmission of TBoMS if the configured RV sequence is {0,2,3,1},

- The first slot of any of the M groups of N slots determined as available for PUSCH transmission of TBoMS associated with RV=0, if the configured RV sequence is {0,3,0,3} or {0,0,0,0}.

Note: It is up to Editor to decide how to capture these rules.

Agreement

For UCI multiplexing on an available slot for TBoMS, the following are supported in Rel-17 for calculating , , ,  and :

- is the number of symbols in an available slot for TBoMS in which UCI is multiplexed.

- The CB size is scaled by , where N is the number of slots allocated for TBoMS, i.e.,  becomes .

Note: It is up to the Editor to decide how to capture the scaling in the specification.

Agreement

The UE does not expect NW to indicate a TBoMS configuration which results in a TBS which exceeds the maximum TBS for single CB transmission.

Agreement

For the retransmission of a single TBoMS with or without repetition in Rel-17:

- The gNB schedules only complete retransmissions of TBs.

- How the retransmission of the entire TB is done is up to gNB, e.g., could be single slot PUSCH retransmission or TBoMS retransmission, etc.

Note: this has no specification impact.

Joint channel estimation for PUSCH:

Agreement:

Support Option 1’-a

Option 1’-a:

- If L is configured, the maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

- If L is not configured, the default value of L = min (maximum duration, duration of all PUSCH repetitions)

Agreement:

- For non-back-to-back PUSCH/PUCCH transmissions across consecutive slots, the other uplink transmission in the middle of two PUSCH/PUCCH transmissions constitutes an event that violates power consistency and phase continuity.

Conclusion:

- Dynamic indication of the window length L of the configured TDW by DCI or indicated by TDRA table with one additional entry is not supported.

Agreement:

This working Assumption is confirmed.

Working assumption: - The start of the first actual TDW is the first symbol (at least determined by TDRA table) for the first PUSCH transmission in an available slot within the configured TDW. - The end of the actual TDW is - the last symbol (at least determined by TDRA table) for the last PUSCH transmission in an available slot within the configured TDW if the actual TDW reaches the end of the last PUSCH transmission within the configured TDW. - the last symbol (at least determined by TDRA table) of the PUSCH transmission right before the event if an event occurs that violates power consistency and phase continuity, and the PUSCH transmission is in an available slot. - For UE capable of restarting DM-RS bundling, the start of the new actual TDW is the first symbol (at least determined by TDRA table) for PUSCH transmission after the event violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

Agreement:

- The action of gNB indicated TA commands constitutes an event that violates power consistency and phase continuity.

Agreement:

- If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events.

  - An event is regarded as a dynamic event if it is triggered by a DCI or MAC-CE, otherwise it is regarded as a semi-static event.

  - Note: At least frequency hopping event is considered as semi-static event.

Working assumption:

- The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity.

  - If UE is configured to accumulate TPC commands,

    - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW.

  - If UE is not configured to accumulate TPC commands

    - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW.

      - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

Agreement:

The following working Assumption is confirmed.

Working assumption: For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, all the repetitions are covered by one or multiple consecutive/non-consecutive configured TDWs. - Each configured TDW consists of one or multiple consecutive physical slots. - The window length L of the configured TDW(s) can be explicitly configured with a single value. - FFS: The maximum value of L - FFS: Solutions to error propagation issue if L is longer than the maximum duration is to be discussed further. - FFS: The window length L is configured per UL BWP - The start of the first configured TDW is the first PUSCH transmission - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission. - The start of other configured TDWs can be implicitly determined prior to first repetition. - FFS: The configured TDWs are consecutive for paired spectrum/SUL band - FFS: The start of the configured TDWs for unpaired spectrum is implicitly determined based on semi-static DL/UL configuration. - The end of the last configured TDW is the end of the last PUSCH transmission. - FFS: The end of the configured TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission. - Within one configured TDW, one or multiple actual TDWs can be implicitly determined: - The start of the first actual TDW is the first PUSCH transmission within the configured TDW. - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission. - After one actual TDW starts, UE is expected to maintain the power consistency and phase continuity until one of the following conditions is met, then the actual TDW is ended. - The actual TDW reaches the end of the last PUSCH transmission within the configured TDW. - FFS: The end of the actual TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission. - An event occurs that violates power consistency and phase continuity - FFS: The events may include e.g., a DL slot based on DL/UL configuration for unpaired spectrum, the actual TDW reaches the maximum duration, DL reception/monitoring occasion for unpaired spectrum, high priority transmission, frequency hopping, precoder cycling. - FFS: The end of the actual TDW is the last available slot/symbol of the PUSCH transmission right before an event such that the power consistency and phase continuity are violated. - If the power consistency and phase continuity are violated due to an event, whether a new actual TDW is created is subject to UE capability of supporting restarting DMRS bundling. - If UE is capable of restarting DM-RS bundling, one new actual TDW is created after the event, - FFS: The start of the new actual TDW is the first available slot/symbol for PUSCH transmission after the event. - If UE is not capable of restarting DM-RS bundling, no new actual TDW is created until the end of the configured TDW. - FFS: UE capability of restarting DMRS bundling is applied only to dynamic event or not Note 1: A ‘configured TDW’ refers to a time domain window whose length can be configured to ‘L’ and whose start and end is determined as described above. Note 2: An ‘actual TDW’ refers to a time domain window during whose entire duration the DM-RS bundling is actually applied. An ‘actual TDW’ duration is always less than or equal to the ‘configure TDW’ duration. Note 3: Whether the terms ‘configured TDW’ and ‘actual TDW’ are revised to other terms and if such terminology is used in specifications is to be further discussed.

Agreement:

- The candidate values of the window length L of the configured TDW can be any integer value that is larger than 1 and no larger than the maximum duration.

Agreement: The following agreement is clarified as follows.

- For PUSCH repetition type A counting based on available slots,

  - “The configured TDWs are determined based on available slots” in the agreement means “The start of the configured TDWs is determined based on available slots”

Agreement - For PUSCH repetition type A counting based on physical slots - The configured TDWs are consecutive, where the start of other configured TDWs is the first physical slot right after the last physical slot of a previous configured TDW. - For PUSCH repetition type A counting based on available slots - The configured TDWs are determined based on available slots, where start of a configured TDWs is the first available slot after the last available slot of a previous configured TDW. - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Agreement:

- UE should not perform UE autonomous TA adjustment during the actual time domain window.

Agreement:

- The TDW determination procedure agreed for PUSCH repetition type A is reused, when applicable, for PUSCH repetition type B and TBoMS with or without repetition.

- No additional specification enhancements for PUSCH repetition type B and TBoMS.

Agreement:

- If DMRS bundling and UL beam switching for multi-TRP operation are configured simultaneously, UL beam switching for multi-TRP operation constitutes an event that violates power consistency and phase continuity.

  - FFS: UL beam switching for multi-TRP operation is regarded as a semi-static event.

PUCCH enhancements:

Agreement

For a PUCCH resource to transmit a PUCCH without an associated scheduling DCI (e.g. P/SP-CSI or SR), if the PUCCH resource is configured with RRC parameter “nrofSlots-r17”, “nrofSlots-r17” is ignored and the RRC parameter “nrofSlots” is used for determining the repetition factor of the specific PUCCH resource.

Agreement

The following use case 5 of PUCCH DMRS bundling is not supported in Rel-17

- Use case 5: PUCCH repetitions across non-consecutive slots.

  - Use case 5a: no uplink transmission in the middle of two PUCCH repetitions

  - Use case 5b: other uplink transmissions in the middle of two PUCCH repetitions

Agreement

For PUCCH DMRS bundling, when appliable, reuse the procedure developed for PUSCH DMRS bundling to determine configured TDW(s) and actual TDW(s).

- FFS: events for PUCCH actual TDW(s)

Agreement

For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, a UE perform the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering.  One option of the following options is to be selected.

- Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination”

  - FFS: DMRS bundling should be restarted in case of frequency hopping event

  - FFS: whether same or separate RRC configuration(s) for hopping interval and configured TDW.

- Option 2: “configured TDW determination” -> “hopping intervals determination” -> “actual TDW determination”

Agreement

For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, a UE performs the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering, based on the following option 1.

- Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination”

  - DMRS bundling shall be restarted at the beginning of each frequency hop

  - DMRS bunding is per actual TDW

  - FFS: Frequency hopping pattern is determined by physical slot indices.

    - FFS: different FH pattern determination for PUCCH and PUSCH

    - FFS: details of FH pattern design

  - Support separate RRC configuration(s) for hopping interval and configured TDW length.

    - if hopping interval is not configured, the default hopping interval is the same as the configured TDW length

      - FFS: if both hopping interval and TDW length are not configured

    - Note: hopping interval is only determined by the configuration of hopping interval if hopping interval is configured

Type A PUSCH repetitions for Msg3:

Agreement

- Flexible symbol indicated by tdd-UL-DL-ConfigurationCommon and not overlapped with SSB symbols indicated by ssb-PositionsInBurst can be regarded as available symbols for Msg3 PUSCH repetition.

- Note: whether and how to introduce other potential mechanisms to use the flexible symbols are separately discussed.

- Note: The Rel-15/16 rules are reused for collision handling between Msg3 PUSCH transmission and a CORESET for Type0-PDCCH CSS set indicated to a UE by pdcch-ConfigSIB1 in MIB in a set of flexible symbols indicated by tdd-UL-DL-ConfigurationCommon.

Conclusion

- There is no consensus to additionally introduce explicit indication to indicate whether or not flexible slots/symbols configured via TDD-UL-DL-Configcommon are available for Msg3 repetition.

Agreement

- RV cycling for Msg3 PUSCH repetition is based on transmission occasions on available slots.

Agreement

For inter-slot FH for Msg3 PUSCH repetition, adopt the following legacy rules.

- The Rel-16 RB offset determination mechanism defined in Table 8.3-1 of TS 38.213 for intra-slot FH for Msg3 PUSCH is reused.

- The Rel-16 additional DMRS configuration defined in Clause 6.2.2 of TS 38.214 for Msg3 PUSCH in case intra-slot FH is disabled is reused.

- The Rel-16 inter-slot FH pattern defined in Clause 6.3.1 of TS 38.214 for PUSCH repetition type A is reused.

Agreement

- For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted.

  - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent.

  - If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}.

Agreement

- For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted.

  - FFS: [8] MCS index to be used for Msg3 re-transmission

Agreement

- Reuse legacy collision handling rule between Msg3 PUSCH transmission and downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated.

- Note: there is no specification impact.

Working assumption

- Support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH.

  - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions.

    - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced.

    - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition.

  - No additional RAN1 specification impact.

Note: UE reports Msg3 repetition capability after initial access.

Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH.

2.1.2	Remaining Open issues

- Completion of CR(s) for TBoMS in case of UCI multiplexing

2.2	RAN2

2.2.1	Agreements

RAN2#116e

- Confirm Msg3 repetition is supported on both NUL and SUL, and network can configure different RSRP thresholds for requesting Msg3 repetition on NUL and SUL.

- Group B preambles with Msg3 repetition is supported, it is up to network to decide whether to configure Group B together with Msg3 repetition.

- If Group B preambles with Msg3 repetition is configured, network can configure separate parameters for requesting Msg3 repetition, including ra-Msg3SizeGroupA, messagePowerOffsetGroupB and numberOfRA-PreamblesGroupA (ASN.1 details can be discussed in session on RACH partitioning)

- ra-ContentionResolutionTimer is started or restarted in the first symbol after all Msg3 repetitions

- In shared RO case, it is not supported to configure a separate set of RACH parameters (preambleReceivedTargetPower, powerRampingStep, preambleTransMax) for requesting Msg3 repetition.

- In shared RO case, it is not supported to separately configure following parameters for requesting Msg3 repetition:

prach-ConfigurationIndex

msg1-FDM

msg1-FrequencyStart

zeroCorrelationZoneConfig

totalNumberOfRA-Preambles

ssb-perRACH-OccasionAndCB-PreamblesPerSSB

rsrp-ThresholdSSB-SUL

prach-RootSequenceIndex

msg1-SubcarrierSpacing

restrictedSetConfig

msg3-transformPrecoder

- In shared RO case, it is up to the common RACH session to decide how to configure the number of preamble per SSB per RO, and how to indicate the start of preamble index for requesting Msg3 repetition.

- A separate rsrp-ThresholdSSB threshold is introduced for requesting Msg3 repetition.

- From CE perspective, carrier selection and BWP selection are performed ahead of CE selection during RACH procedure.

- From CE perspective, UE compares the RSRP of DL path-loss reference with the Msg3 repetition threshold [rsrp-Threshold-Msg3Rep] during the RACH initialization procedure and decides whether to use CE or non-CE RA.

- From CE perspective, if CE RA is selected, then the decision doesn’t change during the entire RACH procedure (i.e. until RACH failure).

2.2.2	Remaining Open issues

- RA procedure for the UE capable of Msg3 repetition, except the common aspects related to RACH indication and partitioning.

- RRC signaling to include physical parameters introduced by RAN1 and UE capability.

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

The progress in RAN4 #101e meeting is summarized below:

- The email discussion summary for [101-e][131] NR_cov_enh was provided in R4-2119931.

- The reply LS on PUCCH and PUSCH transmissions was approved in R4-2120002.

- The WF on phase continuity and power consistency for PUCCH and PUSCH transmissions was approved in R4-2120003.

- The RAN4 RF work plan for NR coverage enhancements WI was updated in R4-2120004.

2.4.2	Remaining Open issues

- RF requirements for the non-scheduled gap and tolerance for power consistency/phase continuity to be investigated and specified if necessary.

- Length(s) of maximum duration for joint channel estimation.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

RAN1 #106b-e:

- R1-2108738	Discussion on coverage enhancements for PUSCH repetition type A	Huawei, HiSilicon

- R1-2108739	Discussion on TB processing over multi-slot PUSCH	Huawei, HiSilicon

- R1-2108740	Discussion on joint channel estimation for PUSCH	Huawei, HiSilicon

- R1-2108741	Discussion on PUCCH coverage enhancement	Huawei, HiSilicon

- R1-2108742	Discussion on Msg3 repetition for coverage enhancement	Huawei, HiSilicon

- R1-2108845	Discussion on enhanced PUSCH repetition type A	ZTE

- R1-2108846	Discussion on TB processing over multi-slot PUSCH	ZTE

- R1-2108847	Discussion on joint channel estimation for PUSCH	ZTE

- R1-2108848	Discussion on coverage enhancements for PUCCH	ZTE

- R1-2108849	Discussion on support of Type A PUSCH repetitions for Msg3	ZTE

- R1-2108850	Performance impacts of residual frequency error for joint channel estimation of PUSCH and PUCCH transmissions	ZTE

- R1-2108919	Discussion on enhancements for PUSCH repetition Type A	Spreadtrum Communications

- R1-2108920	Discussion on TB processing over multi-slot PUSCH	Spreadtrum Communications

- R1-2108921	Discussion on joint channel estimation for PUSCH	Spreadtrum Communications

- R1-2108922	Discussion on PUCCH enhancements	Spreadtrum Communications

- R1-2108923	Discussion on Type A PUSCH repetitions for Msg3	Spreadtrum Communications

- R1-2108989	Discussion on enhancement for PUSCH repetition type A	vivo

- R1-2108990	Discussion on PUSCH TB processing over multiple slots	vivo

- R1-2108991	Discussion on joint channel estimation for PUSCH	vivo

- R1-2108992	Discussion on PUCCH enhancements	vivo

- R1-2108993	Discussion on Type A PUSCH repetitions for Msg3	vivo

- R1-2108994	Enhanced contention resolution mechanism for CBRA procedure with MSG3 PUSCH repetition	vivo

- R1-2109035	Views on TB processing over multi-slot PUSCH	Fujitsu

- R1-2109088	Enhancements on PUSCH repetition type A	OPPO

- R1-2109089	Further considerations for TB over multi-slot PUSCH	OPPO

- R1-2109090	Consideration on Joint channel estimation for PUSCH	OPPO

- R1-2109091	PUCCH enhancements for coverage	OPPO

- R1-2109092	Type A PUSCH repetitions for Msg3 coverage	OPPO

- R1-2109133	Discussion on TB processing over multi-slot PUSCH	NEC

- R1-2109134	Discussion on PUSCH repetition for Msg3	NEC

- R1-2109141	On TB processing over multiple slots for PUSCH	Indian Institute of Tech (H)

- R1-2109240	Discussion on enhancements on PUSCH repetition type A	CATT

- R1-2109241	Discussion on TB processing over multi-slot PUSCH	CATT

- R1-2109242	Discussion on joint channel estimation for PUSCH	CATT

- R1-2109243	Discussion on PUCCH enhancement	CATT

- R1-2109244	Discussion on Type A PUSCH repetitions for Msg3	CATT

- R1-2109245	Views on reusing PUSCH enhancements for Msg3	CATT

- R1-2109247	Remaining issues on PUSCH repetition type A enhancements	China Telecom

- R1-2109248	Remaining issues on TB processing over multi-slot PUSCH	China Telecom

- R1-2109249	Remaining issues on joint channel estimation for PUSCH	China Telecom

- R1-2109250	FL Summary of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2109251	Remaining issues on PUCCH enhancements	China Telecom

- R1-2109252	Remaining issues on type A PUSCH repetitions for Msg3	China Telecom

- R1-2109295	Discussion on enhancements on PUSCH repetition type A	CMCC

- R1-2109296	Discussion on TB processing over multi-slot PUSCH	CMCC

- R1-2109297	Discussion on joint channel estimation for PUSCH	CMCC

- R1-2109298	Discussion on PUCCH enhancements	CMCC

- R1-2109299	Discussion on type A PUSCH repetitions for Msg3	CMCC

- R1-2109329	Discussion on TBoMS PUSCH	TCL Communication Ltd.

- R1-2109330	Discussion on joint channel estimation for PUSCH	TCL Communication Ltd.

- R1-2109424	Enhancements on PUSCH repetition type A	Xiaomi

- R1-2109425	Discussion on TB processing over multi-slot PUSCH	Xiaomi

- R1-2109426	Discussion on joint channel estimation for PUSCH	Xiaomi

- R1-2109427	Discussion on PUCCH enhancements	Xiaomi

- R1-2109428	Discussion on Type A PUSCH repetition for Msg3	Xiaomi

- R1-2109429	Other considerations for coverage enhancement	Xiaomi

- R1-2109452	Discussion on enhancements on PUSCH repetition type A	Rakuten Mobile, Inc

- R1-2109455	Discussion on enhancements on PUSCH repetition Type A	Panasonic Corporation

- R1-2109456	Discussion on TB processing over multi-slot PUSCH	Panasonic Corporation

- R1-2109457	Discussion on PUCCH enhancement for NR coverage enhancement	Panasonic Corporation

- R1-2109458	Discussion on Type A PUSCH repetitions for Msg.3	Panasonic Corporation

- R1-2109459	Discussion on joint channel estimation for PUSCH	Panasonic Corporation

- R1-2109504	Enhancements on PUSCH repetition type A	Samsung

- R1-2109505	TB processing over multi-slot PUSCH	Samsung

- R1-2109506	Joint channel estimation for PUSCH	Samsung

- R1-2109507	PUCCH enhancements	Samsung

- R1-2109508	Type A PUSCH repetitions for Msg3	Samsung

- R1-2109509	Considerations on Rel-17 RRC parameters for Coverage Enhancement	Samsung

- R1-2109571	Discussion on TB processing over multi-slot	MediaTek Inc.

- R1-2109572	Discussion on Joint channel estimation over multi-slot	MediaTek Inc.

- R1-2109624	Enhancements on PUSCH repetition type A	Intel Corporation

- R1-2109625	Discussion on TB processing over multi-slot PUSCH	Intel Corporation

- R1-2109626	Discussion on joint channel estimation for PUSCH	Intel Corporation

- R1-2109627	Discussion on PUCCH enhancements	Intel Corporation

- R1-2109628	On Msg3 PUSCH repetition	Intel Corporation

- R1-2109692	Enhancements on PUSCH repetition type A	NTT DOCOMO, INC.

- R1-2109693	TB processing over multi-slot PUSCH	NTT DOCOMO, INC.

- R1-2109694	Joint channel estimation for PUSCH	NTT DOCOMO, INC.

- R1-2109695	PUCCH enhancements for coverage enhancement	NTT DOCOMO, INC.

- R1-2109696	Type A PUSCH repetitions for Msg3	NTT DOCOMO, INC.

- R1-2109746	On further potential coverage enhancements	Huawei, HiSilicon

- R1-2109799	Views on Joint Channel Estimation for PUSCH	Sony

- R1-2109814	PUCCH enhancements	ETRI

- R1-2109815	Type A PUSCH repetitions for Msg3	ETRI

- R1-2109886	Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

- R1-2109887	Transport block processing for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2109888	Joint channel estimation for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2109889	PUCCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2109890	Approaches and solutions for Type A PUSCH repetitions for Msg3	Nokia, Nokia Shanghai Bell

- R1-2109891	On the support of DMRS bundling for the scenario of other UL transmission in-between PUSCH/PUCCH repetitions	Nokia, Nokia Shanghai Bell

- R1-2109990	Design considerations for PUSCH repetition Type A Enhancements	Sierra Wireless. S.A.

- R1-2109991	Design Considerations for Joint channel estimation for PUSCH	Sierra Wireless. S.A.

- R1-2110000	Enhancements on PUSCH repetition type A	Sharp

- R1-2110001	Transport block processing over multi-slot PUSCH	Sharp

- R1-2110002	Joint channel estimation for multiple PUSCH transmission	Sharp

- R1-2110003	PUCCH coverage enhancement	Sharp

- R1-2110004	Type-A PUSCH repetition for msg3	Sharp

- R1-2110046	Discussion on PUSCH repetition type A enhancement	Apple

- R1-2110047	Discussion on TB processing over multi-slot PUSCH	Apple

- R1-2110048	Discussion on joint channel estimation for PUSCH	Apple

- R1-2110049	PUCCH coverage enhancement	Apple

- R1-2110050	Discussion on Msg3 Coverage Enhancement	Apple

- R1-2110096	Discussions on PUSCH repetition type A enhancements	LG Electronics

- R1-2110097	Discussions on TB processing over multi-slot PUSCH	LG Electronics

- R1-2110098	Discussions on joint channel estimation for PUSCH	LG Electronics

- R1-2110099	Discussions on coverage enhancement for PUCCH	LG Electronics

- R1-2110100	Discussion on coverage enhancement for Msg3 PUSCH	LG Electronics

- R1-2110122	PUSCH Repetition Type A Enhancement	Ericsson

- R1-2110123	TB Processing over Multi-Slot PUSCH	Ericsson

- R1-2110124	Joint Channel Estimation for PUSCH	Ericsson

- R1-2110125	PUCCH Dynamic Repetition and DMRS Bundling	Ericsson

- R1-2110126	Type A PUSCH Repetition for Msg3	Ericsson

- R1-2110127	Frequency Hopping for TBoMS	Ericsson

- R1-2110152	Type-A PUSCH repetition for coverage enhancement	InterDigital, Inc.

- R1-2110153	TB processing over multiple slots	InterDigital, Inc.

- R1-2110154	Joint channel estimation for PUSCH	InterDigital, Inc.

- R1-2110155	Discussions on PUCCH enhancements	InterDigital, Inc.

- R1-2110201	Enhancements on PUSCH Repetition Type A	Qualcomm Incorporated

- R1-2110202	TB processing over multi-slot PUSCH	Qualcomm Incorporated

- R1-2110203	Joint channel estimation for PUSCH	Qualcomm Incorporated

- R1-2110204	PUCCH enhancements	Qualcomm Incorporated

- R1-2110205	Type A PUSCH repetition for Msg3	Qualcomm Incorporated

- R1-2110236	Type A PUSCH repetitions for Msg3	InterDigital, Inc.

- R1-2110237	Enhancements on PUSCH repetition type A	Lenovo, Motorola Mobility

- R1-2110238	Enhancements for TB processing over multi-slot PUSCH	Lenovo, Motorola Mobility

- R1-2110239	Enhancements for joint channel estimation for multiple PUSCH	Lenovo, Motorola Mobility

- R1-2110240	Enhancements for PUCCH repetition	Lenovo, Motorola Mobility

- R1-2110327	Discussion on enhancements on PUSCH repetition type A	WILUS Inc.

- R1-2110328	Discussion on TB processing over multi-slot PUSCH	WILUS Inc.

- R1-2110329	Discussion on joint channel estimation for PUSCH	WILUS Inc.

- R1-2110330	Discussion on Type A PUSCH repetitions for Msg3	WILUS Inc.

- R1-2110417	Feature lead summary #1 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

- R1-2110418	Feature lead summary #2 on support of Type A PUSCH repetitions for Msg4	Moderator (ZTE)

- R1-2110419	Feature lead summary #3 on support of Type A PUSCH repetitions for Msg5	Moderator (ZTE)

- R1-2110428	FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia/Nokia Shanghai Bell)

- R1-2110441	FL summary#1 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2110448	FL Summary #1 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2110449	FL Summary #2 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2110450	FL Summary #3 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2110502	FL Summary#2 of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2110503	FL Summary#3 of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2110513	FL summary#2 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2110528	FL summary #2 of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia/Nokia Shanghai Bell)

- R1-2110529	FL summary #3 of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia/Nokia Shanghai Bell)

- R1-2110530	Final FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia/Nokia Shanghai Bell)

- R1-2110568	FL Summary#4 of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2110569	[106bis-e-NR-R17-CovEnh-03] Summary of email discussion on joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2110585	Reply LS on Msg3 repetition in coverage enhancement	RAN1, ZTE

- R1-2110589	Summary of discussion on reply LS for Msg3 repetition in coverage enhancement	Moderator (ZTE)

- R1-2110590	FL summary #3 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2110596	FL Summary #4 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2110688 RAN1 agreements for Rel-17 NR coverage enhancements, WI rapporteur (China Telecom)

RAN1 #107-e:

- R1-2110789	Discussion on coverage enhancements for PUSCH repetition type A	Huawei, HiSilicon

- R1-2110790	Discussion on TB processing over multi-slot PUSCH	Huawei, HiSilicon

- R1-2110791	Discussion on joint channel estimation for PUSCH	Huawei, HiSilicon

- R1-2110792	Discussion on PUCCH coverage enhancement	Huawei, HiSilicon

- R1-2110793	Discussion on Msg3 repetition for coverage enhancement	Huawei, HiSilicon

- R1-2110863	Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

- R1-2110864	Transport block processing for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2110865	Joint channel estimation for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2110866	PUCCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2110867	Approaches and solutions for Type A PUSCH repetitions for Msg3	Nokia, Nokia Shanghai Bell

- R1-2110868	Discussion on the dependency of RRC parameters for counting on available slots and TBoMS	Nokia, Nokia Shanghai Bell

- R1-2110918	Discussion on enhanced PUSCH repetition type A	ZTE

- R1-2110919	Discussion on TB processing over multi-slot PUSCH	ZTE

- R1-2110920	Discussion on joint channel estimation for PUSCH	ZTE

- R1-2110921	Discussion on coverage enhancements for PUCCH	ZTE

- R1-2110922	Discussion on support of Type A PUSCH repetitions for Msg3	ZTE

- R1-2110923	Discussion on RRC parameters for coverage enhancement	ZTE

- R1-2111027	Remaining issues on enhancement for PUSCH repetition type A	vivo

- R1-2111028	Remaining issues on PUSCH TB processing over multiple slots	vivo

- R1-2111029	Remaining issues on joint channel estimation for PUSCH	vivo

- R1-2111030	Remaining issues on PUCCH enhancements	vivo

- R1-2111031	Remaining issues on Type A PUSCH repetitions for Msg3	vivo

- R1-2111032	Enhanced contention resolution mechanism for CBRA procedure with MSG3 PUSCH repetition	vivo

- R1-2111106	Discussion on enhancements for PUSCH repetition Type A	Spreadtrum Communications

- R1-2111107	Discussion on TB processing over multi-slot PUSCH	Spreadtrum Communications

- R1-2111108	Discussion on joint channel estimation for PUSCH	Spreadtrum Communications

- R1-2111109	Discussion on PUCCH enhancements	Spreadtrum Communications

- R1-2111110	Discussion on Type A PUSCH repetitions for Msg3	Spreadtrum Communications

- R1-2111149	Views on TB processing over multi-slot PUSCH	Fujitsu

- R1-2111203	Discussion on PUSCH repetition type A enhancements	TCL Communication Ltd.

- R1-2111204	Discussion on TB processing over multi-slot PUSCH	TCL Communication Ltd.

- R1-2111205	Discussion on joint channel estimation for PUSCH	TCL Communication Ltd.

- R1-2111271	Discussion on enhancements on PUSCH repetition type A	CATT

- R1-2111272	Discussion on TB processing over multi-slot PUSCH	CATT

- R1-2111273	Discussion on joint channel estimation for PUSCH	CATT

- R1-2111274	Discussion on PUCCH enhancement	CATT

- R1-2111275	Discussion on Type A PUSCH repetitions for Msg3	CATT

- R1-2111328	Enhancements on PUSCH repetition type A	OPPO

- R1-2111329	Further considerations for TB over multi-slot PUSCH	OPPO

- R1-2111330	Consideration on Joint channel estimation for PUSCH	OPPO

- R1-2111331	PUCCH enhancements for coverage	OPPO

- R1-2111332	Type A PUSCH repetitions for Msg3 coverage	OPPO

- R1-2111425	Discussion on joint channel estimation for PUSCH	Panasonic Corporation

- R1-2111426	Remaining issues on PUSCH repetition type A enhancements	China Telecom

- R1-2111427	Remaining issues on TB processing over multi-slot PUSCH	China Telecom

- R1-2111428	Remaining issues on joint channel estimation for PUSCH	China Telecom

- R1-2111429	FL Summary of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2111430	Remaining issues on PUCCH enhancements	China Telecom

- R1-2111431	Remaining issues on type A PUSCH repetitions for Msg3	China Telecom

- R1-2111437	Discussion on enhancements on PUSCH repetition Type A	Panasonic Corporation

- R1-2111438	Discussion on TB processing over multi-slot PUSCH	Panasonic Corporation

- R1-2111439	Discussion on PUCCH enhancement for NR coverage enhancement	Panasonic Corporation

- R1-2111440	Discussion on Type A PUSCH repetitions for Msg.3	Panasonic Corporation

- R1-2111507	Enhancements on PUSCH repetition type A	Intel Corporation

- R1-2111508	Discussion on TB processing over multi-slot PUSCH	Intel Corporation

- R1-2111509	Discussion on joint channel estimation for PUSCH	Intel Corporation

- R1-2111510	Discussion on PUCCH enhancements	Intel Corporation

- R1-2111511	On Msg3 PUSCH repetition	Intel Corporation

- R1-2111584	Enhancements on PUSCH repetition type A	Xiaomi

- R1-2111585	Discussion on TB processing over multi-slot PUSCH	Xiaomi

- R1-2111586	Discussion on joint channel estimation for PUSCH	Xiaomi

- R1-2111587	Discussion on PUCCH enhancements	Xiaomi

- R1-2111588	Discussion on Type A PUSCH repetition for Msg3	Xiaomi

- R1-2111589	Other considerations for coverage enhancement	xiaomi

- R1-2111590	Discussion on enhancements on PUSCH repetition type A	Rakuten Mobile, Inc

- R1-2111620	Discussion on enhancements on PUSCH repetition type A	CMCC

- R1-2111621	Discussion on TB processing over multi-slot PUSCH	CMCC

- R1-2111622	Discussion on joint channel estimation for PUSCH	CMCC

- R1-2111623	Discussion on PUCCH enhancements	CMCC

- R1-2111624	Discussion on type A PUSCH repetitions for Msg3	CMCC

- R1-2111681	On events for Joint Channel Estimation for PUSCH	Sony

- R1-2111693	Discussion on TB processing over multi-slot PUSCH	NEC

- R1-2111694	Discussion on dynamic PUCCH repetition factor	NEC

- R1-2111751	Enhancements on PUSCH repetition type A	Samsung

- R1-2111752	TB processing over multi-slot PUSCH	Samsung

- R1-2111753	Joint channel estimation for PUSCH	Samsung

- R1-2111754	PUCCH enhancements	Samsung

- R1-2111755	Type A PUSCH repetitions for Msg3	Samsung

- R1-2111792	Type-A PUSCH repetition for coverage enhancement	InterDigital, Inc.

- R1-2111793	TB processing over multiple slots	InterDigital, Inc.

- R1-2111794	Joint channel estimation for PUSCH	InterDigital, Inc.

- R1-2111795	Discussions on PUCCH enhancements	InterDigital, Inc.

- R1-2111796	Type A PUSCH repetitions for Msg3	InterDigital, Inc.

- R1-2111841	Design Considerations for Joint channel estimation for PUSCH	Sierra Wireless. S.A.

- R1-2111842	Design considerations for PUSCH repetition Type A Enhancements	Sierra Wireless. S.A.

- R1-2111887	Discussion on PUSCH repetition type A enhancement	Apple

- R1-2111888	Discussion on TB processing over multi-slot PUSCH	Apple

- R1-2111889	Discussion on joint channel estimation for PUSCH	Apple

- R1-2111890	PUCCH coverage enhancement	Apple

- R1-2111891	Discussion on Msg3 Coverage Enhancement	Apple

- R1-2111948	Enhancements on PUSCH repetition type A	Lenovo, Motorola Mobility

- R1-2111949	Enhancements for TB processing over multi-slot PUSCH	Lenovo, Motorola Mobility

- R1-2111950	Enhancements for joint channel estimation for multiple PUSCH	Lenovo, Motorola Mobility

- R1-2111951	Enhancements for PUCCH repetition	Lenovo, Motorola Mobility

- R1-2111979	Discussions on TB processing over multi-slot PUSCH	LG Electronics

- R1-2111980	Discussions on joint channel estimation for PUSCH	LG Electronics

- R1-2111981	Discussions on coverage enhancement for PUCCH	LG Electronics

- R1-2111982	Discussion on coverage enhancement for Msg3 PUSCH	LG Electronics

- R1-2111993	PUCCH enhancements	ETRI

- R1-2111994	Type A PUSCH repetitions for Msg3	ETRI

- R1-2112019	Enhancements on PUSCH repetition type A	Sharp

- R1-2112020	Transport block processing over multi-slot PUSCH	Sharp

- R1-2112021	Joint channel estimation for multiple PUSCH transmission	Sharp

- R1-2112022	PUCCH coverage enhancement	Sharp

- R1-2112023	Type A PUSCH repetitions for Msg3	Sharp

- R1-2112035	Remaining Issues for PUSCH Repetition Type A Enhancement	Ericsson

- R1-2112036	Remaining Issues for TB Processing over Multi-Slot PUSCH	Ericsson

- R1-2112037	Remaining Issues for Joint Channel Estimation for PUSCH	Ericsson

- R1-2112038	Remaining Issues for PUCCH Dynamic Repetition and DMRS Bundling	Ericsson

- R1-2112039	Remaining Issues for Type A PUSCH Repetition for Msg3	Ericsson

- R1-2112040	Frequency Hopping for TBoMS	Ericsson

- R1-2112119	Enhancements on PUSCH repetition type A	NTT DOCOMO, INC.

- R1-2112120	TB processing over multi-slot PUSCH	NTT DOCOMO, INC.

- R1-2112121	Joint channel estimation for PUSCH	NTT DOCOMO, INC.

- R1-2112122	PUCCH enhancements for coverage enhancement	NTT DOCOMO, INC.

- R1-2112123	Type A PUSCH repetitions for Msg3	NTT DOCOMO, INC.

- R1-2112230	Enhancements on PUSCH Repetition Type A	Qualcomm Incorporated

- R1-2112231	TB processing over multi-slot PUSCH	Qualcomm Incorporated

- R1-2112232	Joint channel estimation for PUSCH	Qualcomm Incorporated

- R1-2112233	PUCCH enhancements	Qualcomm Incorporated

- R1-2112234	Type A PUSCH repetition for Msg3	Qualcomm Incorporated

- R1-2112316	Discussion on TB processing over multi-slot	MediaTek Inc.

- R1-2112317	Discussion on Joint channel estimation over multi-slot	MediaTek Inc.

- R1-2112390	Discussion on TB processing over multi-slot PUSCH	WILUS Inc.

- R1-2112391	Discussion on joint channel estimation for PUSCH	WILUS Inc.

- R1-2112392	Discussion on PUCCH enhancements for coverage enhancement	WILUS Inc.

- R1-2112393	Discussion on Type A PUSCH repetitions for Msg3	WILUS Inc.

- R1-2112401	On futher potential coverage enhancements	Huawei, HiSilicon

- R1-2112462	FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia/Nokia Shanghai Bell)

- R1-2112519	FL Summary #1 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2112520	FL Summary #2 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2112521	FL Summary #3 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2112543	Discussion on Msg3 repetition for coverage enhancement	Huawei, HiSilicon

- R1-2112556	Feature lead summary #1 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

- R1-2112557	Feature lead summary #2 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

- R1-2112558	Feature lead summary #3 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

- R1-2112561	FL Summary#2 of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2112562	FL Summary#3 of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2112563	FL Summary#4 of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2112582	FL summary #1 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2112611	Remaining Issues for TB Processing over Multi-Slot PUSCH	Ericsson

- R1-2112622	FL summary #2 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2112686	FL summary #2 of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia/Nokia Shanghai Bell)

- R1-2112687	FL summary #3 of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia/Nokia Shanghai Bell)

- R1-2112688	Final FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia/Nokia Shanghai Bell)

- R1-2112700	FL summary #3 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2112708	FL summary #4 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2112828	[107-e-NR-R17-CovEnh-03] Summary of email discussion on joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2112836	Feature lead summary #4 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

RAN2 #116-e:

- R2-2109443  Further Discussion on RAN2 Impacts of Msg3 Repetition	  vivo

- R2-2109456  RAN2 aspects of coverage enhancements	Qualcomm Incorporated

- R2-2109503  Discussion on CE’s impact on the start of ra-ContentionResolutionTimer  OPPO

- R2-2109530  MAC Aspects of UL Coverage Enhancements  Samsung Electronics Co., Ltd

- R2-2109877  RAN2 aspects of Msg3 PUSCH repetition  Intel Corporation

- R2-2109894  Consideration on Msg3 repetition in CE  ZTE Corporation, Sanechips

- R2-2110038  RAN2 impact of coverage enhancements  Apple

- R2-2110192  Considerations on requesting Msg3 repetition  NEC Corporation

- R2-2110440  Analysis on Type A PUSCH repetitions for Msg3  CATT

- R2-2110814  RAN2 aspects for Coverage Enhancement	Nokia, Nokia Shanghai Bell

- R2-2110833  On Type A PUSCH repetitions for Msg3  Ericsson

- R2-2111026  Further discussions on RAN2 support of Msg3 PUSCH repetition  Huawei, HiSilicon

- R2-2111160  Discussion on Msg3 PUSCH repletion  LG Electronics Inc.

- R2-2111210  Reply LS on Msg3 repetition in coverage enhancement (R1-2110585; contact: ZTE)  RAN1

- R2-2111346  [offline-112] Coverage Enhancements aspects  ZTE

RAN4 #101e:

- R4-2117184	Updated RAN4 RF work plan for NR coverage enhancements WI	China Telecom

- R4-2117185	On phase continuity and power consistency tolerance	China Telecom

- R4-2117186	On maximum duration for joint channel estimation	China Telecom

- R4-2117187	On UE autonomous adjustment and DL symbol(s) in-between transmission	China Telecom

- R4-2117475	Discussion on the definition of downlink reception in RAN1 LS	Nokia, Nokia Shanghai Bell

- R4-2117476	Discussion on RF requirements for non-scheduled gap	Nokia, Nokia Shanghai Bell

- R4-2117477	Discussion on maximum duration for joint channel estimation	Nokia, Nokia Shanghai Bell

- R4-2117478	Discussion on UE autonomous timing advance (TA) adjustment	Nokia, Nokia Shanghai Bell

- R4-2117617	Views on phase continuity and power consistency for PUSCH and PUCCH repetition	Sony

- R4-2118981	On measurement of the TX coherent transmission	Ericsson

- R4-2118982	On JCE phase continuity and power consistency tolerance for PUCCH and PUSCH	Ericsson

- R4-2118983	RF impact on non-scheduled gap	Ericsson

- R4-2118984	simulation updated results for phase tolerance for PUSCH  repetition	Ericsson

- R4-2118985	LS reply On maximum duration of phase continuity and power consistency for PUCCH and PUSCH repetition	Ericsson

- R4-2118986	LS reply on PUCCH and PUSCH repetition	Ericsson

- R4-2119089	On phase continuity for multiple transmissions	Huawei Technologies France

- R4-2119193	Discussion on phase discontinuity and power inconsistency tolerance across different repetitions	ZTE Corporation

- R4-2119494	UE requirement for phase continuity	Qualcomm Incorporated

- R4-2119495	Simulation results and criterias for setting requriements for UE	Qualcomm Incorporated

- R4-2119504	Remaining open issues on coverage enhancements	MediaTek (Chengdu) Inc.

- R4-2119731	Email discussion summary for [101-e][131] NR_cov_enh	Moderator (China Telecom)

- R4-2119931	Email discussion summary for [101-e][131] NR_cov_enh	Moderator (China Telecom)

- R4-2120002	Reply LS on PUCCH and PUSCH transmissions	 Qualcomm

- R4-2120003	WF on phase continuity and power consistency for PUCCH and PUSCH transmissions	Huawei, HiSilicon

- R4-2120004	Updated RAN4 RF work plan for NR coverage enhancements WI	China Telecom

- R4-2120005	On phase continuity and power consistency tolerance	China Telecom