3GPP TSG-RAN WG1 Meeting #108-e	R1-2202899

# e-meeting, 21 February - 03 March, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0287 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the SPS PDSCH activation and HARQ-ACK transmission for the 1st SPS PDSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2022-03-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During RAN1#107 and RAN1#107bis in the Rel-17 PUCCH coverage enhancements discussion the question of whether the 1st SPS PDSCH transmitted in response of the SPS PDSCH activation should be considered as an SPS-PDSCH (PDSCH without a corresponding PDCCH), or a “regular” dynamic-grant-scheduled PDSCH that has a corresponding PDCCH. RAN1#108 discussed the issue, identified that two possible interpretations exist and proceeded to clarify the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in 9.1.3.1 that - DAI field of the DL SPS activation DCI is to be ignored - The SPS-PDSCH “associated with a corresponding activation DCI” is considered as SPS-PDSCH in HARQ-ACK information multiplexing to the OACK HARQ-ACK information bits. Clarify in 9.2.3 that - The PUCCH resource determination procedure excludes the PUCCH Resource Indication field in the SPS PDSCH activation DCI - The PUCCH resource determination for the SPS-PDSCH “associated with the corresponding activation DCI” follows the RRC-configured n1PUCCH-AN, or SPS-PUCCH-AN-List like all the subsequent SPS-PDSCHs do. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether the 1st PDSCH the SPS-PDSCH activation DCI points to is considered an SPS-PDSCH or dynamically granted PDSCH. This leads to different understandings in how the HARQ-ACK for the 1st PDSCH is to be transmitted. Isolated impact analysis: The CR impacts the HARQ-ACK transmission associated with the 1st SPS PDSCH that is triggered by the DL SPS activation DCI. - If the gNB is implemented according to the CR, and the UE is not, or vice versa, the two nodes have a different understanding of the PUCCH resource to transmit the HARQ-ACK for the 1st PDSCH leading to loss of the HARQ-ACK of the activation DCI and failure for the gNB to detect that the UE has DL SPS activated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.1, 9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is for Rel-16. Rel-15 UE/gNB may also implement the same change RAN1 discussion summary in R1-2202833 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Unaffected subclauses omitted *****

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

If a UE is configured to monitor PDCCH for multicast DCI formats with CRC scrambled by one or more G-RNTIs that the UE generates a Type-2 HARQ-ACK codebook, the UE separately applies the procedures in this clause per G-RNTI and determines the Type-2 HARQ-ACK codebook by concatenating the Type-2 HARQ-ACK codebook for unicast DCI formats followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-RNTI values.

A UE determines monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions, or having associated HARQ-ACK information without scheduling PDSCH reception, on an active DL BWP of a serving cell c, as described in clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot n based on

-	PDSCH-to-HARQ_feedback timing indicator field values, or a dl-DataToUL-ACK, dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-ForDCI-Format1-2 value if the PDSCH-to-HARQ_feedback timing indicator field is not present in a DCI format, for PUCCH transmission with HARQ-ACK information in slot n, as described in clause 9.2.3, in response to PDSCH receptions, or in response to a DCI format having associated HARQ-ACK information without scheduling PDSCH reception

-	slot offsets K0 [6, TS 38.214] provided by time domain resource assignment field in a DCI format scheduling PDSCH receptions and by pdsch-AggregationFactor, or pdsch-AggregationFactor-r16, or repetitionNumber, when provided.

The set of PDCCH monitoring occasions for DCI formats scheduling PDSCH receptions, or having associated HARQ-ACK information without scheduling PDSCH reception, is defined as the union of PDCCH monitoring occasions across active DL BWPs of configured serving cells. PDCCH monitoring occasions are indexed in an ascending order of their start times. The cardinality of the set of PDCCH monitoring occasions defines a total number M of PDCCH monitoring occasions.

A value of the counter downlink assignment indicator (DAI) field in DCI formats denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pairs in which PDSCH receptions, or HARQ-ACK information bits that are not in response for PDSCH receptions, associated with the DCI formats, excluding the SPS activation DCI, is present up to the current serving cell and current PDCCH monitoring occasion,

-	first, if the UE indicates by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion, in increasing order of the PDSCH reception starting time for the same {serving cell, PDCCH monitoring occasion} pair,

-	second in ascending order of serving cell index, and

-	third in ascending order of PDCCH monitoring occasion index m, where 0≤m<M.

If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the value of the counter DAI is in the order of the first CORESETs and then the second CORESETs for a same serving cell index and a same PDCCH monitoring occasion index.

The value of the total DAI, when present [5, TS 38.212], in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s), or HARQ-ACK information that does not correspond to PDSCH receptions, associated with DCI formats, excluding the SPS activation DCI, is present, up to the current PDCCH monitoring occasion m and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion. If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the total DAI value counts the {serving cell, PDCCH monitoring occasion}-pair(s) for both the first CORESETs and the second CORESETs.

Denote by NC-DAIDL the number of bits for the counter DAI and set TD=2NC-DAIDL. Denote by VC-DAI,c,mDL the value of the counter DAI in a DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, on serving cell c in PDCCH monitoring occasion m according to Table 9.1.3-1 or Table 9.1.3-1A. Denote by VT-DAI,mDL the value of the total DAI in a DCI format in PDCCH monitoring occasion m according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats that include a total DAI field in PDCCH monitoring occasion m. A UE does not expect to multiplex, in a same Type-2 HARQ-ACK codebook, HARQ-ACK information that is in response to detection of DCI formats with different number of bits for the counter DAI field.

If the UE transmits HARQ-ACK information in a PUCCH in slot n and for any PUCCH format, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK, for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code:

Set m=0 – PDCCH, with DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling a PDSCH reception, monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion

Set j=0

Set Vtemp=0

Set Vtemp2=0

Set Vs=∅

Set NcellsDL to the number of serving cells configured by higher layers for the UE

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs

-	if the UE indicates type2-HARQ-ACK-Codebook, a serving cell is counted NPDSCHMO times where NPDSCHMO is the number of PDSCH receptions that can be scheduled for the serving cell by DCI formats in PDCCH receptions at a same PDCCH monitoring occasion based on the reported value of type2-HARQ-ACK-Codebook

Set M to the number of PDCCH monitoring occasion(s)

while m<M

Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

while c<NcellsDL

if PDCCH monitoring occasion m is before an active DL BWP change on serving cell c or an active UL BWP change on the PCell and an active DL BWP change is not triggered in PDCCH monitoring occasion m

c=c+1;

else

if there is a PDSCH providing a transport block for a HARQ process with enabled HARQ-ACK information on serving cell c associated with PDCCH in PDCCH monitoring occasion m, or there is a PDCCH providing a DCI format associated with HARQ-ACK information without scheduling PDSCH reception on serving cell c

if VC-DAI,c,mDL≤Vtemp

j=j+1

end if

Vtemp=VC-DAI,c,mDL

if VT-DAI,mDL=∅

Vtemp,2=VC-DAI,c,mDL

else

Vtemp,2=VT-DAI,mDL

end if

if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell,

o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell

o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell

Vs=Vs∪2⋅TD⋅j+2VC-DAI,c,mDL-1, 2⋅TD⋅j+2VC-DAI,c,mDL-1+1

elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and m is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell,

oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell

Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1

else

oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell

Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1

end if

end if

c=c+1

end if

end while

m=m+1

end while

Vtemp=j mod4TD×4TD+Vtemp

if UE does not set Vtemp2=VT-DAIUL and TD=2

Vtemp2=Vtemp

end if

j=j×TD4

if Vtemp2<Vtemp

j=j+1

end if

if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell,

OACK=2⋅4⋅j+Vtemp2

else

OACK=4⋅j+Vtemp2

end if

oiACK=NACK for any i∈0,1,⋯,OACK-1\Vs

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACK HARQ-ACK information bits.

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions, including the ones associated with the corresponding activation DCI, in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACK HARQ-ACK information bits.

The UE generates HARQ-ACK information with ACK value in response to a detection of a DCI format that does not trigger a Type-3 HARQ-ACK codebook report and has associated HARQ-ACK information without scheduling a PDSCH reception.

For a PDCCH monitoring occasion with DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling a PDSCH reception, in the active DL BWP of a serving cell, when a UE receives a PDSCH with one transport block, or detects a DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and the value of maxNrofCodeWordsScheduledByDCI is 2, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

If a UE is not provided PDSCH-CodeBlockGroupTransmission for any serving cells, or for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions, or for SPS PDSCH reception, or for a DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as

nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c+g=0G-1VDAI,mlast,gDL-c=0Ncells,gDL-1UDAI,c,gmodTD,gNTB,max,gDL+c=0Ncells,gDL-1m=0Mg-1Nm,c,greceived+NSPS,c,g

where

-	NcellsDL is a number of serving cells where the UE is configured to receive unicast PDSCHs

-	Ncells,gDL is a number of serving cells where the UE is configured to receive multicast PDSCHs for a G-RNTI g or a G-CS-RNTI g

-	G is a total number of G-RNTIs or G-CS-RNTIs configured to the UE

-	M is the number of PDCCH monitoring occasions for unicast DCI formats

-	Mg is the number of PDCCH monitoring occasions for multicast DCI formats with CRC scrambled by G-RNTI g or G-CS-RNTI g

-	TD=2NC-DAIDL where NC-DAIDL the number of bits for the counter DAI field in unicast DCI formats

-	TD,g=2NC-DAI,gDL where NC-DAI,gDL the number of bits for the counter DAI field in multicast DCI formats with CRC scrambled by G-RNTI g or G-CS-RNTI g

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling PDSCH reception or having associated HARQ-ACK information without scheduling PDSCH reception, that the UE detects within the M PDCCH monitoring occasions.

-	if Ncells,gDL=1, VDAI,mlast,gDL is the value of the counter DAI in the last multicast DCI format with G-RNTI g, or G-CS-RNTI g, scheduling PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, that the UE detects within the Mg PDCCH monitoring occasions

-	if NcellsDL>1 or if Ncells,gDL>1

-	if the UE does not detect any DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M or Mg PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c, VDAI,mlastDL or VDAI,mlast,gDL, respectively, is the value of the counter DAI in a last DCI format the UE detects in the last PDCCH monitoring occasion

-	if the UE detects at least one DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M or Mg, for G-RNTI g or G-CS-RNTI g, PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c, VDAI,mlastDL or VDAI,mlast,gDL, respectively, is the value of the total DAI in the at least one DCI format that includes a total DAI field

-	VDAI,mlastDL=0 or VDAI,mlast,gDL=0 if the UE does not detect any DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception,  for any serving cell c in any of the M or Mg PDCCH monitoring occasions, respectively.

-	UDAI,c or UDAI,c,g, for G-RNTI g or G-CS-RNTI g, is the total number of DCI formats scheduling PDSCH receptions, or having associated HARQ-ACK information without scheduling a PDSCH reception, that the UE detects within the M or Mg PDCCH monitoring occasions, respectively, for serving cell c. UDAI,c=0 or UDAI,c,g=0 if the UE does not detect any DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for serving cell c in any of the M or Mg, respectively,PDCCH monitoring occasions.

-	NTB,maxDL=2 if the value of maxNrofCodeWordsScheduledByDCI is 2 for any serving cell c and harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, NTB,maxDL=1.

-	NTB,max,gDL=1.

-	Nm,creceived or Nm,c,greceived, for G-RNTI g or G-CS-RNTI g, is the number of transport blocks the UE receives in a PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided, or the number of DCI formats that the UE detects and have associated a HARQ-ACK information without scheduling PDSCH reception in PDCCH monitoring occasion m for serving cell c.

-	NSPS,c or NSPS,c,g, for G-RNTI g or G-CS-RNTI g, is the number of SPS PDSCH receptions by the UE on serving cell c for which the UE transmits corresponding HARQ-ACK information in the same PUCCH as for HARQ-ACK information corresponding to PDSCH receptions within the M or Mg PDCCH monitoring occasions, respectively.

If a UE

-	is provided PDSCH-CodeBlockGroupTransmission for NcellsDL,CBG serving cells; and

-	is not provided PDSCH-CodeBlockGroupTransmission, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,CBG=NcellsDL

the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for

-	SPS PDSCH reception,

-	a DCI format having associated HARQ-ACK information without scheduling PDSCH reception,

-	TCI state update, and

-	TB-based PDSCH receptions on the NcellsDL,CBG serving cells and on the NcellsDL,TB serving cells,

-	NcellsDL is replaced by NcellsDL,CBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,CBG serving cells for CBG-based PDSCH receptions, and

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and

-	instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,CBG serving cells, the UE generates NHARQ-ACK,maxCBG/TB,max HARQ-ACK information bits, where NHARQ-ACK,maxCBG/TB,max is the maximum value of NTB,cDL⋅NHARQ-ACK,cCBG/TB,max across all NcellsDL,CBG serving cells and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c it is NTB,cDL⋅NHARQ-ACK,cCBG/TB,max<NHARQ-ACK,maxCBG/TB,max, the UE generates NACK for the last NHARQ-ACK,maxCBG/TB,max-NTB,cDL⋅NHARQ-ACK,cCBG/TB,max HARQ-ACK information bits for serving cell c

-	the pseudo-code operation when harq-ACK-SpatialBundlingPUCCH is provided is not applicable

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook

If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,CBG for obtaining a PUCCH transmission power, as described in clause 7.2.1, with

nHARQ-ACK,CBG=VDAI,mlastDL-c=0NcellsDL,CBG-1UDAI,cCBGmodTDNHARQ-ACK,maxCBG/TB,max+c=0NcellsDL-1m=0M-1Nm,creceived,CBG

where

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling CBG-based PDSCH reception that the UE detects within the M PDCCH monitoring occasions

-	if NcellsDL>1, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling CBG-based PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions

-	VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for any serving cell c in any of the M PDCCH monitoring occasions

-	UDAI,cCBG is the total number of DCI formats scheduling CBG-based PDSCH receptions that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cCBG=0 if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for serving cell c in any of the M PDCCH monitoring occasions

-	Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH scheduled by a DCI format that supports CBG-based PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH

If a UE is not provided numberOfHARQ-BundlingGroups, detects a first DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, if any, and a second DCI format scheduling more than one PDSCH reception on a serving cell from the NcellsDL serving cells, if any, and the UE would provide corresponding HARQ-ACK information in a same PUCCH, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	the UE determines a first HARQ-ACK sub-codebook based on each detected DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, or SPS PDSCH receptions, if any, and

-	the UE determines a second HARQ-ACK sub-codebook based on each detected DCI format scheduling more than one PDSCH reception, and

-	instead of generating one HARQ-ACK information bit per transport block for serving cell c, the UE generates NHARQ-ACKmax HARQ-ACK information bits where NHARQ-ACKmax is the maximum value of NTB,cDL⋅NPDSCH,cmax across all NcellsDL serving cells, NPDSCH,cmax is a maximum number of PDSCH receptions that can be scheduled by a DCI format on serving cell c as described in [6, TS 38.214], and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. The UE generates the NHARQ-ACKmax HARQ-ACK information bits in ascending order of the NPDSCH,cmax PDSCHs, including any PDSCH that the UE does not receive in a slot as described in clause 11.1. If, for serving cell c, the UE detects a DCI format that schedules NPDSCH,c PDSCH receptions and NTB,cDL⋅NPDSCH,c<NHARQ-ACKmax , the UE generates NACK for the last NHARQ-ACKmax -NTB,cDL⋅NPDSCH,c HARQ-ACK information bits

-	The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable.

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook.

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook.

If a UE is provided numberOfHARQ-BundlingGroups for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c.

If a UE

-	is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and

-	is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL

the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for

-	SPS PDSCH reception,

-	any DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and

-	PDSCH reception scheduled by a DCI format scheduling one PDSCH

-	PDSCH reception with NHARQ-ACKTBG,max=1 for TBG-based HARQ-ACK information on the NcellsDL,TB serving cells,

-	NcellsDL is replaced by NcellsDL,TBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,TBG serving cells for TBG-based HARQ-ACK information, or for TB-based HARQ-ACK information corresponding to multiple PDSCH receptions scheduled by a single DCI format, and

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and

-	instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits, where NHARQ-ACK,maxTBG,max is the maximum value between NTB,cDL⋅NHARQ-ACK,cTBG,max across all NcellsDL,TBG serving cells if the UE is provided numberOfHARQ-BundlingGroups, and NTB,cDL⋅NPDSCH,cmax across all NcellsDL,TBG serving cells where the UE is not provided numberOfHARQ-BundlingGroups, and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c where the UE is provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NHARQ-ACK,cTBG,max<NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK information bits for serving cell c. If for a serving cell c where the UE is not provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NPDSCH,cmax <NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NPDSCH,cmax  HARQ-ACK information bits for serving cell c.

-	The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable.

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook.

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook.

Table 9.1.3-1: Value of counter DAI for NC-DAIDL=2 and of total DAI

| DAI MSB, LSB | VC-DAIDL or VT-DAIDL | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH generating a HARQ-ACK information bit without scheduling a PDSCH reception or providing TCI state update is present, denoted as Y and Y≥1 |
| --- | --- | --- |
| 0,0 | 1 | Y-1modTD+1=1 |
| 0,1 | 2 | Y-1modTD+1=2 |
| 1,0 | 3 | Y-1modTD+1=3 |
| 1,1 | 4 | Y-1modTD+1=4 |

Table 9.1.3-1A: Value of counter DAI for NC-DAIDL=1

| DAI | VC-DAIDL | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH generating a HARQ-ACK information bit without scheduling a PDSCH reception or providing TCI state update is present, denoted as Y and Y≥1 |
| --- | --- | --- |
| 0 | 1 | Y-1modTD+1=1 |
| 1 | 2 | Y-1modTD+1=2 |

***** Unaffected subclauses omitted *****

### 9.2.3	UE procedure for reporting HARQ-ACK

In this clause, for the purpose of determining a PUCCH resource for a PUCCH transmission in a slot using a PUCCH resource indicator field in a DCI format that schedules a PDSCH reception, and for the purpose of determining the slot for the PUCCH transmission, a UE is assumed to generate HARQ-ACK information regardless of whether or not the PDSCH reception provides a transport block for a HARQ process with disabled HARQ-ACK information as indicated by HARQ-feedbackEnabling-disablingperHARQprocess, if provided. The UE determines a number of HARQ-ACK information bits OACK as described in clauses 9.1 through 9.1.5 and a corresponding set of PUCCH resources as described in clause 9.2.1.

A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot per priority index, if the UE is not provided ackNackFeedbackMode = separate.

For DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8} for SCS configuration of PUCCH transmission μ≤3, to {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and to {13, 16, 24, 32, 40, 48, 56, 64} for μ=6. For a unicast DCI format, other than DCI format 1_0 or requesting Type-3 HARQ-ACK codebook report without scheduling a PDSCH reception as described in clause 9.1.4, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17 as defined in Table 9.2.3-1. If the DCI format indicates a cell for the PUCCH transmission, as described in clause 9.A, the PDSCH-to-HARQ_feedback timing indicator field value maps to slots of the active UL BWP of the cell; otherwise, the PDSCH-to-HARQ_feedback timing indicator field value maps to slots of the active UL BWP of the Pcell. For multicast DCI formats, the PDSCH-to-HARQ_feedback timing indicator field values are provided by dl-DataToUL-ACK-MulticastDciFormat4_1 or, if dl-DataToUL-ACK-MulticastDciFormat4_1 is not provided, by {1, 2, 3, 4, 5, 6, 7, 8}.

If the UE is provided subslotLengthForPUCCH, n is the last UL slot for PUCCH transmission that overlaps with a PDSCH reception or with a PDCCH reception providing a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception; otherwise, n is the last UL slot for PUCCH transmission that overlaps with the DL slot nD for the PDSCH reception or with the DL slot nD for the PDCCH reception in case of a DCI format that triggers a HARQ-ACK information report and does not schedule a PDSCH reception.

For a SPS PDSCH reception ending in DL slot nD, the UE transmits the PUCCH in UL slot n+k where n is the last UL slot for PUCCH transmission that overlaps with slot nD and k is provided by the PDSCH-to-HARQ_feedback timing indicator field, if present, in a DCI format activating the SPS PDSCH reception.

If the UE detects a DCI format that does not include a PDSCH-to-HARQ_feedback timing indicator field and schedules a PDSCH reception or activates a SPS PDSCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k where n is the last UL slot for PUCCH transmission that overlaps with slot nD and k is provided by dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1.

If the UE detects a DCI format scheduling a number of PDSCH receptions ending in DL slot nD or if the UE detects a DCI format generating a HARQ-ACK information bit and does not schedule a PDSCH reception through a PDCCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k, where n is the last UL slot for PUCCH transmission that overlaps with slot nD and k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1.

A PUCCH transmission with HARQ-ACK information is subject to the limitations for UE transmissions described in clause 11.1 and clause 11.1.1.

Table 9.2.3-1: Mapping of PDSCH-to-HARQ_feedback timing indicator field values to numbers of slots

| PDSCH-to-HARQ_feedback timing indicator |  |  | Number of slots k |
| --- | --- | --- | --- |
| 1 bit | 2 bits | 3 bits |  |  |
| '0' | '00' | '000' | 1st value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1 |  |
| '1' | '01' | '001' | 2nd value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1 |  |
|  | '10' | '010' | 3rd value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1 |  |
|  | '11' | '011' | 4th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, dl-DataToUL-ACK-MulticastDciFormat4_1 |  |
|  |  | '100' | 5th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1 |  |
|  |  | '101' | 6th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1 |  |
|  |  | '110' | 7th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1 |  |
|  |  | '111' | 8th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1 |  |

For a PUCCH transmission with HARQ-ACK information, a UE determines a PUCCH resource on the cell of the PUCCH transmission, as described in clause 9.A, after determining a set of PUCCH resources for OUCI HARQ-ACK information bits, as described in clause 9.2.1. The PUCCH resource determination is based on a PUCCH resource indicator field [5, TS 38.212], if present, in a last DCI format, excluding the SPS activation DCI, among the DCI formats that have a value of a PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1, indicating a same slot for the PUCCH transmission, that the UE detects and for which the UE transmits corresponding HARQ-ACK information in the PUCCH. For PUCCH resource determination, detected DCI formats are first indexed in an ascending order across serving cells indexes for a same PDCCH monitoring occasion and are then indexed in an ascending order across PDCCH monitoring occasion indexes. For indexing DCI formats within a serving cell for a same PDCCH monitoring occasion, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs on an active DL BWP of a serving cell, and with ackNackFeedbackMode = joint for the active UL BWP, detected DCI formats from PDCCH receptions in the first CORESETs are indexed prior to detected DCI formats from PDCCH receptions in the second CORESETs.

The PUCCH resource indicator field values map to values of a set of PUCCH resource indexes, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, provided by resourceList for PUCCH resources from a set of PUCCH resources provided by PUCCH-ResourceSet with a maximum of eight PUCCH resources. If the PUCCH resource indicator field includes 1 bit or 2 bits, the values map to the first two values or the first four values, respectively, of Table 9.2.3-2. If the last DCI format does not include a PUCCH resource indicator field, the first value of Table 9.2.3-2 is used.

For the first set of PUCCH resources and when the size RPUCCH of resourceList is larger than eight, when a UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a last DCI format in a PDCCH reception, among DCI formats with a value of the PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1, indicating a same slot for the PUCCH transmission, the UE determines a PUCCH resource with index rPUCCH, 0≤rPUCCH≤RPUCCH-1, as

where NCCE,p is a number of CCEs in CORESET p of the PDCCH reception for the DCI format as described in clause 10.1, nCCE,p is the index of a first CCE for the PDCCH reception, and ∆PRI is a value of the PUCCH resource indicator field in the DCI format. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1, the CORESET is associated with the search space set having the smaller index. If the DCI format does not include a PUCCH resource indicator field, ∆PRI=0.

Table 9.2.3-2: Mapping of PUCCH resource indication field values to a PUCCH resource in a PUCCH resource set with maximum 8 PUCCH resources

| PUCCH resource indicator |  |  | PUCCH resource |
| --- | --- | --- | --- |
| 1 bit | 2 bits | 3 bits |  |  |
| '0' | '00' | '000' | 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList |  |
| '1' | '01' | '001' | 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList |  |
|  | '10' | '010' | 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList |  |
|  | '11' | '011' | 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList |  |
|  |  | '100' | 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList |  |
|  |  | '101' | 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList |  |
|  |  | '110' | 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList |  |
|  |  | '111' | 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList |  |

If a UE determines a first resource for a PUCCH transmission with HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH or detects a first DCI format indicating a first resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot and also detects at a later time a second DCI format indicating a second resource for a PUCCH transmission with corresponding HARQ-ACK information in the slot, the UE does not expect to multiplex HARQ-ACK information corresponding to the second DCI format in a PUCCH resource in the slot if the PDCCH reception that includes the second DCI format is not earlier than N3∙2048+144∙κ∙2-μ∙Tc from the beginning of a first symbol of the first resource for PUCCH transmission in the slot where, κ and Tc are defined in clause 4.1 of [4, TS 38.211] and μ corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs providing the DCI formats and the SCS configuration of the PUCCH. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the second DCI format and for all serving cells with corresponding HARQ-ACK information multiplexed in the PUCCH transmission in the slot, N3=3 for μ=0, N3=4.5 for μ=1, N3=9 for μ=2; otherwise, N3=8 for μ=0, N3=10 for μ=1, N3=17 for μ=2, N3=20 for μ=3, N3=80 for μ=5, and N3=160 for μ=6.

If a UE is not provided SPS-PUCCH-AN-List and transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, which includes the first SPS PDSCH reception associated with the corresponding activation DCI, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN.

If a UE transmits a PUCCH with HARQ-ACK information using PUCCH format 0, the UE determines values m0 and mCS for computing a value of cyclic shift α [4, TS 38.211] where m0 is provided by initialCyclicShift of PUCCH-format0 or, if initialCyclicShift is not provided, by the initial cyclic shift index as described in clause 9.2.1 and mCS is determined from the value of one HARQ-ACK information bit or from the values of two HARQ-ACK information bits as in Table 9.2.3-3 and Table 9.2.3-4, respectively.

Table 9.2.3-3: Mapping of values for one HARQ-ACK information bit to sequences for PUCCH format 0

| HARQ-ACK Value | 0 | 1 |
| --- | --- | --- |
| Sequence cyclic shift | mCS=0 | mCS=6 |

Table 9.2.3-4: Mapping of values for two HARQ-ACK information bits to sequences for PUCCH format 0

| HARQ-ACK Value | {0, 0} | {0, 1} | {1, 1} | {1, 0} |
| --- | --- | --- | --- | --- |
| Sequence cyclic shift | mCS=0 | mCS=3 | mCS=6 | mCS=9 |

If a UE transmits a PUCCH with HARQ-ACK information using PUCCH format 1, the UE is provided a value for  by initialCyclicShift of PUCCH-format1 or, if initialCyclicShift is not provided, by the initial cyclic shift index as described in clause 9.2.1.

If a UE transmits a PUCCH with OACK HARQ-ACK information bits and OCRC bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes MRBPUCCH PRBs, the UE determines a number of PRBs MRB,minPUCCH for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs MRBPUCCH provided respectively by nrofPRBs of PUCCH-format2 or nrofPRBs of PUCCH-format3 and start from the first PRB from the number of PRBs, that results to OACK+OCRC≤MRB,minPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r and, if MRBPUCCH>1, OACK+OCRC>MRB,minPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, where Nsc,ctrlRB, Nsymb-UCIPUCCH, Qm, and r are defined in clause 9.2.5.2. For PUCCH format 3, if MRB,minPUCCH is not equal 2α2⋅3α3⋅5α5 according to [4, TS 38.211], MRB,minPUCCH is increased to the nearest allowed value of nrofPRBs [12, TS 38.331]. If OACK+OCRC>MRBPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, the UE transmits the PUCCH over MRBPUCCH PRBs.

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation and transmits a PUCCH with OACK HARQ-ACK information bits and OCRC bits using PUCCH format 2 or PUCCH format 3, the UE transmits the PUCCH over the first interlace if OACK+OCRC≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r; otherwise, if the UE is provided a second interlace by interlace1 in PUCCH-format2 or PUCCH-format3, the UE transmits the PUCCH over the first and second interlaces.

***** Unaffected subclauses omitted *****