3GPP TR 36.779 V1.0.1 (2022-03)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Introduction of upper 700MHz A block E-UTRA band for the US;

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	6

4.1	Justification	6

4.2	Objectives	8

5	Frequency band arrangements and regulatory background	8

5.1	Frequency band and channel bandwidth	8

5.2	Adjacent 3GPP bands	9

5.3	Nearby non-3GPP Services	9

5.4	FCC regulatory requirements	9

6	Operating band, channel bandwidths, channel numbering	10

6.1	Operating bands	10

6.2	Operating bands for category NB1 and NB2	10

6.3	Channel bandwidth	11

7	Study of E-UTRA specific issues	11

7.1	UE aspect issues	11

7.1.1 	UE maximum output power for category NB1 and NB2	11

7.1.2	UE maximum output power with additional requirements for category NB1 and NB2 UE	11

7.1.3	Spurious emission band UE co-existence	12

7.2	BS aspect issues	12

7.2.1 	Transmitter characteristics	12

7.2.1.1 	Base station output power	12

7.2.1.2 	Operating band unwanted emission	12

7.2.1.3 	Spurious emission	13

7.2.2 	Receiver characteristics	13

7.2.2.1 	Blocking requirement	13

Annex A: Change history	14

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for the work item of Introduction of upper 700MHz A block E-UTRA band for the US.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-212618, “New WID: Introduction of upper 700MHz A block E-UTRA band for the US”

[3]	RP-212917, “Revised WID: Introduction of upper 700MHz A block E-UTRA band for the US”

[4]	Reallocation of TV Channels 60-69, the 746-806 MHz Band, ET Docket No. 97-157.

[5]	https://www.fcc.gov/wireless/bureau-divisions/mobility-division/lower-700-mhz-service.

[6]	Middle Class Tax Relief and Job Creation Act of 2012, Pub. L. No. 112-96, 126 Stat. 156 (2012).

[7]	Implementing Public Safety Broadband Provisions of the Middle Class Tax Relief and Job Creation Act of 2012, Docket No. DA 12-1462

[8]	3GPP TS 36.101 V15.3.0 (2018-07), Table 5.5-1 E-UTRA frequency bands.

[9]	Reallocation and Service Rules for the 698-746 MHz Spectrum Band, GN Docket No. 01-74.

[10]	“700 MHz Public Safety Spectrum” (https://www.fcc.gov/700-mhz-public-safety-narrowband-spectrum).

[11]	PART 27 — MISCELLANEOUS WIRELESS COMMUNICATIONS SERVICES, Electronic Code of Federal Regulations.

[12]	R4-2119425, UE coexistence and emission requirements for 700 MHz NB-IoT, RAN4#101-e.

[13]	3GPP TS 36.104 V17.4.0 “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

# 3	Definitions, symbols and abbreviations

Delete from the above heading those words which are not applicable.

Clause numbering depends on applicability and should be renumbered accordingly.

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ERP	Effective Radiated Power

HAAT	Height Above Average Terrain

NAM	North America

PSNB	Public Safety Narrow Band

<ACRONYM>	<Explanation>

# 4	Background

## 4.1	Justification

During the RAN#93-e and RAN#94-e, the WI to introduce Upper 700MHz A Block as an E-UTRA band for the US was approved [2][3]. The A Block in the Upper 700 MHz Band Plan [4], is shown in Figure 4.1-1. In 1998, the FCC reallocated the 746-806 MHz spectrum band (Upper 700 MHz Band as in Figure 4.1-1) that had been allocated from television Channels 60-69 for public safety use and the fixed, mobile services for commercial use [5].

Figure 4.1-1 – FCC Upper 700 MHz Band Plan

The FCC combined the D Block with the Broadband portion of the Public Safety band [6] [7] to provide broadband public safety services; which were standardized by 3GPP as Band 14.

The existing 3GPP Band Classes in the whole 700 MHz band for the North America (NAM) is shown in Figure 4.1-2. In addition to Band 14, Upper 700 MHz C Block became Band 13 [8]. Figure 4.1-2 also includes the Lower 700 MHz Band [9] with the following 3GPP bands [8] defined: Band 12, Band 17, Band 85 that work in FDD mode; as well as Band 29 that is used as a FDD supplemental downlink (SDL) for Carrier Aggregation only.

Figure 4.1-2 – 700 MHz Band Plan and 3GPP Band Classes

From regulatory point of view, the Upper 700 MHz A Block and Band 13 are subject to the same requirements (FCC 47 CFR § 27.53 part (c)); which are very similar to those for the other adjacent band, that is, Band 14 (FCC 47 CFR §90R).

Recently, Band 85 was created by adding a 1+1 MHz guard band to Band 12 [8].  This was facilitated by the vacating of the adjoining spectrum by the TV broadcasters [8].  The guard bands are intended to be allocated for stand-alone NB-IoT carriers.  The successful standardization of Band 85 demonstrated that NB-IoT can coexist with other services in the 700MHz Band.

The proposed Upper 700 MHz A Block as a new stand-alone band for NB-IoT is shown in Figure 4.1-3. To be specific, the uplink band for this new band is 1 MHz, 787-788 MHz; the downlink band is 1 MHz, 757-758 MHz; the duplex spacing is 30 MHz.

Figure 4.1-3 – New Band Proposed for Upper 700 MHz A Block

The 700 MHz band is a premium spectrum that exhibits excellent propagation characteristics, which makes it perfect for NB-IoT applications. The Upper 700 MHz A Block is currently used by utility companies and other critical infrastructure industries in the US to provide critical communications services. They have expressed a strong desire for standardizing this band to ensure the interoperability and leverage the existing 3GPP eco-system (see the accompanying Liaison Statements). Industrial IoT is an important vertical for 5G mMTC deployments. Standardizing this band will encourage the utility industry to use standards-based 3GPP solutions instead of proprietary ones and will co-exist better with the neighboring 3GPP bands.

The operation of NB-IoT in the Upper 700 MHz A Block has been successfully demonstrated in field commercial networks in North Florida region using FCC certified equipment for the past two years [9].   The required changes to implement NB-IoT on this band using existing 700 MHz capable devices and equipment were minimal.

## 4.2	Objectives

The following are the core objectives of the Work Item:

-	Study the need for any applicable co-location and/or co-existence requirements for the 700MHz NB-IoT band, considering regional regulations, and if needed specify the co-existence requirements

-	Study any identified unique aspects specific to the proposed new band.

-	Standardize a new FDD E-UTRA operating band based on the 2 MHz paired spectrum consisting of Upper 700 MHz A Block. To be specific, the uplink band for this new band is 1 MHz, 787-788 MHz; the downlink band is 1 MHz, 757-758 MHz; the duplex spacing is 30 MHz. Refer to Figure 4.1-3.

-	Support for 200 kHz bandwidth

-	Specify band numbering and RF characteristics of the new band.

-	Ensure the new band supports only NB-IoT operation.

-	Update the related 3GPP E-UTRA technical specifications for the core part to include support for the new band

-	Update the related 3GPP E-UTRA technical specifications for the performance part to include support for the new band, if necessary.

# 5	Frequency band arrangements and regulatory background

## 5.1	Frequency band and channel bandwidth

The upper 700MHz block A to be introduced as a new E-UTRA band for standalone NB-IoT operation is summarized as below:

- Uplink Frequency Range: 787-788 MHz

- Downlink Frequency Range: 757-758 MHz

- Channel Bandwidth: 1 MHz

- Duplex Spacing: 30 MHz

## 5.2	Adjacent 3GPP bands

The Region 2 3GPP bands nearby to operating band 103, were identified as the following: 5, 12, 13, 14, 17, 26, 29, 71 and 85; as shown in Figure 5.2-1.

17

17

12

12

13

14

14

29

71

71

13

85

85

Uplink

Downlink

5

26

814

849

5

26

859

894

New band

Figure 5.2-1. 3GPP Bands operating nearby to Upper 700MHz Block A

Operating Band 103 resides between E-UTRA Band 13 and Band 14.

## 5.3	Nearby non-3GPP Services

FCC assigned a pair of 6+6 MHz FDD spectrum for public safety narrow band (PSNB) services [10]. The downlink is in 769 – 775 MHz and uplink is in 799 – 805 MHz.

In order to protect the PSNB services, FCC requires that the emission limits to this band (both the uplink and downlink) be limited to -46 dBm / 6.25 kHz from base stations operated in the Upper 700 MHz A block and -35 dBm / 6.25 kHz from mobile devices [11]; respectively.

## 5.4	FCC regulatory requirements

The relevant FCC rules governing the Upper 700 MHz Band (746 – 806 MHz) can be found in the 47 CFR Part 27 [2]. They are summarized as follows:

1.	Power Limits

The power limits are defined in the FCC 47 CFR Part 27 [2], §27.50 - Power limits and duty cycle:

Fixed and base stations transmitting a signal must not exceed an effective radiated power (ERP) of 1000 watts and an antenna height of 305 m height above average terrain (HAAT).

Control stations and mobile stations transmitting in the 776-788 MHz band and fixed stations transmitting in the 787-788 MHz band are limited to 30 watts ERP.

Portable stations (hand-held devices) transmitting in the band are limited to 3 watts ERP.

For transmissions in the 757-758, 775-776, 787-788, and 805-806 MHz bands, maximum composite transmit power shall be measured over any interval of continuous transmission using instrumentation calibrated in terms of RMS-equivalent voltage. The measurement results shall be properly adjusted for any instrument limitations, such as detector response times, limited resolution bandwidth capability when compared to the emission bandwidth, etc., so as to obtain a true maximum composite measurement for the emission in question over the full bandwidth of the channel.

2. Emission Limits

The emission limits are defined in the FCC 47 CFR Part 27 [11], §27.53 - Power limits and duty cycle:

The power of any emission outside a licensee´s frequency band/bands of operations shall be attenuated below the transmitter power P, measured in watts, by at least 43+10log10(P) dB, considering a resolution bandwidth of 100 kHz or greater, except within 0-100 kHz immediately outside and adjacent a licensee´s frequency block where it is required to be 30 kHz.   That corresponds to a single absolute emission power spectral density limit of -13 dBm (Emission limit = PdBW-43-PdB= - 43dBW = -13dBm) over the specified resolution bandwidth for all transmitters (including base stations and portable devices) regardless of actual ERP or frequency offset.

The emission power for all frequencies between 763-775 MHz and 793-805 MHz shall be attenuated below the transmitter power P, measured in watts, by a factor not less than 76 + 10 log (P) dB in a 6.25 kHz band segment, for base and fixed stations; and by a factor not less than 65 + 10 log (P) dB in a 6.25 kHz band segment, for mobile and portable stations

The emissions in the band 1559-1610 MHz shall be limited to −70 dBW/MHz equivalent isotropically radiated power (EIRP) for wideband signals, and −80 dBW EIRP for discrete emissions of less than 700 Hz bandwidth. For the purpose of equipment authorization, a transmitter shall be tested with an antenna that is representative of the type that will be used with the equipment in normal operation.

It should be noted from the above that the regulatory requirements regarding emissions are the same for both Band 13 and upper 700 MHz block A.

# 6	Operating band, channel bandwidths, channel numbering

The core objective of the WI is to standardize the upper 700MHz A block defined by the FCC in the US to support standalone NB-IoT operation [2][3].

## 6.1	Operating bands

Band number 103 is assigned to the Upper 700 MHz A Block.

Table 6.1-1 E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 103 | 787 MHz | – | 788 MHz | 757 MHz | – | 758 MHz | FDD |

## 6.2	Operating bands for category NB1 and NB2

Category NB1 and NB2 are designed to operate in the E-UTRA operating band 103 which is defined in Table 6.2-1.

Band 103 is subject to the same emission requirements as Band 13, therefore NS_04 is applicable.

In case UE receives network signaling value NS_04 on any of the operating bands listed in Table 6.2-1 then the lower and upper limit of those bands are shown in Table 6.2-1 to account for the USA emission requirements.

Table 6.2-1 E-UTRA operating bands for NB-IoT in the USA

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 13 | 777 MHz | – | 786.9 MHz | 746 MHz | – | 755.9 MHz | FDD |
| 85 | 698.1 MHz | – | 715.9 MHz | 728.1 MHz | – | 745.9 MHz | FDD |
| 103 | 787.1 MHz | – | 787.9 MHz | 757.1 MHz | – | 757.9 MHz | FDD |

## 6.3	Channel bandwidth

Band 103 is the first band that supports NB-IoT only operation. Thus far, none of the E-UTRA channel bandwidths is supported, as the whole band is only of 1+1 MHz paired spectrum.

# 7	Study of E-UTRA specific issues

## 7.1	UE aspect issues

The upper 700MHz A block resides between Band 13 and Band 14. Hence, this new band and Band 13 / 14 exhibit the same propagation characteristics. Moreover the regulatory requirements for this band are exactly the same as those for Band 13. Therefore, it was decided to define the requirements based on the work done for bands 13 with the support for NB-IoT operation only.

### 7.1.1 	UE maximum output power for category NB1 and NB2

Following the above motivation to reuse the Band 13 work, Band 103 was agreed to use the following NB1 and NB2 power classes, as listed in table 7.1.1-1.

Table 7.1.1-1: UE Power Class for Band 103

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 103 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |

### 7.1.2	UE maximum output power with additional requirements for category NB1 and NB2 UE

To address the additional spurious emission requirement of -13 dBm/30 kHz outside of the band (787 - 788 MHz), as described in Section 5.2, a 100 kHz guard band at both band edges is introduced by making use of the NS_04 signaling for US NB-IoT bands [12]. For Band 103, the result is effectively truncating its bandwidth from 1 MHz to 800 kHz. Band 103 is added to the row of NS_04 in the following table.

Table 7.1.2-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_04 | 5.5F | 2, 4, 5, 12, 13, 17, 25, 26, 66, 71, 85, 103 | N/A |

### 7.1.3	Spurious emission band UE co-existence

UE to UE co-existence is specified the same as for Band 13, i.e., adopting the -50 dBm/MHz emission requirement to/from other 3GPP bands.

Table 7.1.3-1: Requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 103 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 |  |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 7.1.3-1 … |  |  |  |  |  |  |  |

## 7.2	BS aspect issues

To enable on the upper 700MHz A Block new E-UTRA band for NB-IoT standalone operation, the following specific changes on the section 6 and 7 for BS requirements in TS 36.104 [13], are expected due to introduction of Band 103. In TS36.104 there is already a full set of requirements to enable NB-IoT standalone operation. The introduction of E-UTRA Band 103 will have impact on those requirements applicable for NB-IoT standalone operation except for those requirements defined with band agnostic way. Particularly, the spurious requirements and blocking requirements for co-existence purpose should be updated to add the new band.

### 7.2.1 	Transmitter characteristics

#### 7.2.1.1 	Base station output power

For Base station output power defined in 6.2 of TS36.104, similar to Band 85, dedicated paragraph should be added for Band 103 NB-IoT standalone operation as the BS rated output power applies only within the range of the DL operation band with assumption of equal PSD.

#### 7.2.1.2 	Operating band unwanted emission

For minimum requirement of unwanted emission for NB-IoT standalone, the reuiqrements are defined in clause 6.6.3.2E, 6.6.3.2F, 6.6.3.2G and 6.6.3.2H of TS36.104 with band agnostic way. Hence no upldate to those clauses is needed.

In the section of  ”6.6.3.3 additional requirements” for unwanted emissions of TS36.104 [13], the additonal emission limits defined in table 6.6.3.3-2 , which are applicalbe to the newly introduced Band 103, should be updated with band 103 accordingly.

#### 7.2.1.3 	Spurious emission

For the section of ” 6.6.4.3 Additional spurious emissions requirements” and ” 6.6.4.4 Co-location with other base stations” for co-existence and co-location requirements with the newly introduced band, changes are expected to table 6.6.4.3.1-1, table 6.6.4.3.1-1A,  Table 6.6.4.3.1-3, table Table 6.6.4.4.1-1, Table 6.6.4.4.1-2 and Table 6.6.4.4.1-3 to guarantee protection to related system in applicable co-existence and co-location scenarios.

### 7.2.2 	Receiver characteristics

#### 7.2.2.1 	Blocking requirement

For the section of ” 7.6.1 General blocking requirement” in TS 36.104, the new band 103 shall be introduced in the NB-IoT standalone operation tables (for different types of BS) as Table 7.6.1.1-3, Table 7.6.1.1-3a, Table 7.6.1.1-3b, Table 7.6.1.1-3c to guarantee the receiver requirement for general blocking.

For the section of ” 7.6.2 Co-location with other base stations” in TS 36.104, the new band 103 shall be introduced in the tables (for different types of BS) as Table 7.6.2.1-1,  Table 7.6.2.1-2 and Table 7.6.2.1-3 to guarantee co-located deployment with the BS in the newly introduced band.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting # | Doc. | CR | Rev | Subject/Comment | Old | New |
| 2021-10 | RAN4#101-e | R4-2117054 |  |  | Skeleton for TR 36.779 | 0.0.0 | 0.0.1 |
| 2022-01 | RAN4#101-bis-e | R4-2200766 |  |  | Revised based on the comments received during RAN4#101-e | 0.0.1 | 0.0.3 |
| 2022-01 | RAN4#101-bis-e | R4-2204346 |  |  | Merged the following TPs approved at RAN4#101-e: - R4-2202391: TP to TR 36.779 on Frequency band arrangements and regulatory background - R4-2202392: TP to TR 36.779 on Operating band, channel bandwidths, channel numbering - R4-2202393: TP to TR 36.779 on Analysis on UE requirements for upper 700MHz A block - R4-2202398: TP to TR 36.779 on Analysis on BS requirements for upper 700MHz A block Corrected copyright year to 2022 from 2021 | 0.0.3 | 0.1.0 |
| 2022-03 | RAN#95-e | RP-220406 |  |  | Corrected Note 15 in Table 7.1.3-1 & editorial changes | 0.1.0 | 1.0.0 |
| 2022-03 | RAN#95-e | RP-220860 |  |  | Editorial corrections to pass the TR Checker | 1.0.0 | 1.0.1 |