3GPP TSG RAN Meeting #96 		RP-221265

Budapest, Hungary. June 6-9, 2022

Title:	[DRAFT] LS to SA2 on handling legacy UE in UAM

Response to:	-

Release:	Release-18

Work Item:	TEI18

Source:	RAN

To:	SA2

Cc:	SA

Contact person:	Chungwoo Hwang

cwhwang@kt.com

Jaehyun Ahn

Jaehyun80@lguplus.co.kr

Soonki Jo

sk.jo@sktelecom.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

Korean government plans to launch UAM (Urban Aerial Mobility) service at the end of 2024. In order to support the government plan, we would like to make sure that legacy 5G NR devices (e.g. Rel-15/16/17) on board maintains at sufficient level. As NR UAV work has been initiated in Release-18, legacy 5G NR device does not have capabilities to send a measurement report message triggered by altitude-based measurement event. For this reason, legacy 5G NR devices likely to experience DL/UL performance degradation due to interference.

Also, aerial channel characteristic with higher LOS probability can induce neighbour relationship among cells to alter comparing to one on the ground. Especially, a cell which to be temporarily seen only by an aerial device should not be included into the NRT (Neighbour Relationship Table) by ANR (Autonomous Neighbour Relationship). However, since a legacy device will not be able to indicate flying situation and could report measurement result for the temporal strong cell, network would insert the wrong relationship information into the NRT, which would waste NRT storage

To solve this problem we would like to consult SA2 whether Application-server based solution can be supported within Release-18 timeframe. Legacy device would send the information about flying to AF (UTM) by application. AF delivers the information to AMF/MME through UDR/PCF. With the corresponding N2 (S1) signalling from AMF/MME, gNB/eNB could cope with dedicated radio configuration for the legacy device.

Figure 1. Application-server based solution for handling legacy 5G NR UE in UAM

# 2	Actions

To SA2

ACTION: RAN respectfully ask SA2 to take the above information into account and provide RAN with reply whether above work can be done as TEI18 within Release-18 timeframe.

# 3	Dates of next TSG RAN meetings

RAN#97e	12 – 16 September	Electronic Meeting

RAN#98	12 – 15 December	Sevilla, Spain