3GPP TSG-RAN#96	Tdoc RP-221433

Budapest, Hungary, 2022-06-06 - 2022-06-09

Agenda Item:	9.3.2.1

Source:	Ericsson

Title:	Rel-18 MobEnh scope

Document for:	Discussion, Decision

# 1	Introduction

In the Rel-18 WID for Mobility enhancements, we have these objectives:

- To specify CHO including target MCG and target SCG [RAN3, RAN2]. Note 5: This is already being targeted for Rel-17, so this objective will be reviewed at RAN#95-e.

At least objective 3 needs updating. We present our view of how to do this in this paper.

# 2	Discussion

Objective 3 is intended to cover that a Conditional HO can include an SCG-configuration. The UE will apply this SCG-configuration when triggering the CHO. There is no conditions particularly for the SCG-configuration.

This was specified in Rel-17 in RAN2, and partially in RAN3.

The RAN2 specs allows that the UE executes a CHO containing an SCG config. However, in case the UE already has an SCG-config, which the SN updates, the SCG config within the CHO that the UE has would need to be updated in some cases. In case the (source) SCG config is updated the normal way, i.e. that the MN provides the config to the UE, it is not a problem. The MN will be aware and can take action. However, the SN can do an intra-SN change, e.g. change from one PSCell to another. The MN will not be aware of these types of changes. If the UE later applies the CHO (i.e. a CHO as defined by objective 3), the UE would apply that CHO on top of the current configuration. That config is not based on the latest config the UE has, because the MN was not aware of the change. When the UE later executed the (outdated) CHO the UE may fail the configuration and reestablish the RRC connection.

To address this, it is important that the MN is made aware of an intra-SN-change. This requires some additional RAN3 signalling. We want to update objective 3 like in the following:

- To specify SN->MN signalling supporting CHO including target MCG and target SCG [RAN3, RAN2]. Note 5: This is already being targeted for Rel-17, so this objective will be reviewed at RAN#95-e.

Update the Rel-18 MobEnh WID as above.

# 3	Conclusion

Based on the discussion above we propose:

Proposal 1	Update the Rel-18 MobEnh WID as above.