3GPP TSG RAN WG1 #109-e	        		   			R1-2205670

# e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0325 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on eIAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Availability of soft resources in one or multiple slots for all RB sets of a DU cell cannot be indicated by only one resourceAvailability in AvailabilityCombination if an RB set group is not provided. - Missing procedure when determining symbols equivalent to being configured as hard when RB set is provided - Ambiguous descriptions for implicit determination of availability of a soft resource. - Missing descriptions for association of DL Tx power adjustment MAC CE with multiplexing mode, slot index, and applicability conditions - Misaligned parameter names in 38.473/331 and 38.213. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture that an IAB-MT may receive a DCI format 2_5 where the availability combination includes one resourceAvailability indicating availability of soft resources in all RB sets in one or more slots for the IAB-DU cell. - Capture that a symbol of a slot is equivalent to being configured as hard if the IAB-DU would transmit cell-specific/semi-static signals and channels also applies to an RB set. - Capture definition of implicit determination of availability for SCG in dual connectivity. - Capture the association of DL Tx power adjustment MAC CE with multiplexing mode, slot index, and applicability conditions. - Align parameter names in 38.473/331 and 38.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for for IAB enhancements in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS 38.473 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged parts are omitted ***

# 14	Integrated access-backhaul operation

Throughout this specification, unless otherwise noted, statements using the term "UE" in clauses 4 through 13 are equally applicable to the IAB-MT of an IAB node.

A procedure for an IAB-MT to perform cell search, system information acquisition, or random access procedure is same as a corresponding one for a UE except for the following.

For initial cell selection, an IAB-MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames.

For PRACH transmission, an IAB-MT determines frames and subframes/slots within the frames containing PRACH occasions as described in [4, TS 38.211].

The IAB-MT determines an association period for mapping SS/PBCH blocks to PRACH occasions based on a PRACH configuration period as described in clause 8.1 and according to Table 14-1 instead of Table 8.1-1. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 640 msec. A PRACH occasion in a PRACH slot is valid according to the conditions in clause 8.1.

Table 14-1: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period for an IAB-MT

| PRACH configuration period (msec) | Association period (number of PRACH configuration periods) |
| --- | --- |
| 10 | {1, 2, 4, 8, 16, 32, 64} |
| 20 | {1, 2, 4, 8, 16, 32} |
| 40 | {1, 2, 4, 8, 16} |
| 80 | {1, 2, 4, 8} |
| 160 | {1, 2, 4} |
| 320 | {1, 2} |
| 640 | {1} |

If an IAB-node is provided an index Tdelta in a Timing Delta MAC CE [11, TS 38.321] from a serving cell, the IAB-node may assume that TTA/2+Ndelta+Tdelta⋅Gstep-NTA,Offset/2⋅Tc is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-MT when TTA/2+Ndelta+Tdelta⋅Gstep-NTA,Offset/2⋅Tc>0, where

-	TTA is the difference between the IAB-MT reception time and the IAB-MT transmission time for IAB-MT transmission timing mode 'Case6', and is defined in clause 4.3.1 of [4, TS 38.211] for IAB-MT transmission timing mode 'Case1' and parent node reception mode 'Case7'

-	Ndelta and Gstep are determined as

-	Ndelta=-70528 and Gstep=64, if the serving cell providing the Timing Delta MAC CE operates in FR1

-	Ndelta=-17664 and Gstep=32, if the serving cell providing the Timing Delta MAC CE operates in FR2

The IAB node may assume that a same value of index Tdelta is provided from a serving cell for the IAB-MT transmission timing modes 'Case7' and 'Case1'

The IAB-node may use the time difference to determine a DU transmission time.

For a serving cell of an IAB-MT, the IAB-MT can be provided by Timing Case Indication MAC CE [11, TS 38.321] an indication of the IAB-MT transmission timing mode in a slot. Upon reception of the Timing Case Indication for a serving cell in a TAG, the IAB-MT applies a same IAB-MT transmission timing mode in a slot on all serving cells in the TAG.

If the indicated IAB-MT transmission timing mode in a slot is set to 'Case1', the IAB-MT transmission time is determined as for a "UE" in clause 4.2.

If the indicated IAB-MT transmission timing mode in a slot is set to 'Case6', the IAB-node sets the IAB-MT transmission time to the transmission time of the IAB-DU.

If the indicated IAB-MT transmission timing mode in a slot is set to 'Case7', the IAB-MT is provided a timing advance offset value NTA,offset,2 for a serving cell by 'Case7' Timing Advance Offset MAC CE [11, TS 38.321]. The IAB-MT determines its uplink transmission timing as NTA+ NTA,offset+ NTA,offset,2 ⋅Tc where NTA and NTA,offset are obtained as for a "UE" in clause 4.2 and NTA,offset,2=Toffset,2⋅16⋅642μ where Toffset,2 is provided by the 'Case7' Timing Advance Offset MAC CE [11, TS 38.321].

A slot format for an IAB-DU or an IAB-MT includes downlink symbols, uplink symbols, and flexible symbols.

For each cell of an IAB-DU, the IAB-DU can be provided an indication for a slot format over a number of slots by gNB-DU Cell Resource Configuration [16, TS 38.473].

For each serving cell, an IAB-MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigurationDedicated-IAB-MT. If the IAB-MT is provided tdd-UL-DL-ConfigurationDedicated-IAB-MT, the statements in clause 11.1 that include "tdd-UL-DL-ConfigurationDedicated" apply to the IAB-MT of an IAB node by replacing "tdd-UL-DL-ConfigurationDedicated" with "tdd-UL-DL-ConfigurationDedicated-IAB-MT" for the IAB-MT, except that the tdd-UL-DL-ConfigurationDedicated-IAB-MT provides

-	a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT

-	for each slot configuration from the set of slot configurations

-	a slot index for a slot provided by slotIndex

-	a set of symbols for a slot by symbols-IAB-MT where

-	if symbols-IAB-MT = allDownlink, all symbols in the slot are downlink

-	if symbols-IAB-MT = allUplink, all symbols in the slot are uplink

-	if symbols-IAB-MT = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible.

-	if symbols-IAB-MT = explicit-IAB-MT, nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible.

If an IAB-MT is configured with an MCG and an SCG, is not capable of simultaneous transmission and reception, and would simultaneously transmit and receive on the MCG and the SCG,

-	if flexible symbols are configured by both parent nodes for operation with inter-donor NR-DC, the IAB-MT operates according to the scheduling from the MCG

-	otherwise, if the IAB-MT is configured with multiple serving cells, is provided directionalCollisionHandling-r17 = 'enabled' for a set of serving cell(s) from the multiple serving cells, and indicates half-DuplexTDD-CA-SameSCS capability across MCG and SCG for NR-DC operation, the IAB-MT applies the procedures for resolving directional collisions as described in clause 11.1 for resolving directional collisions across serving cells.

An IAB-MT can be provided, by SlotFormatCombinationsPerCell, a list of slot format combinations applicable for one serving cell and, by SlotFormatIndicator, a configuration for monitor a DCI format 2_0 indicating a slot format combination, from the list of slot format combinations, over a number of slots as described in clause 11.1.1. In addition to the slot formats in Table 11.1.1-1, an SFI field for an IAB-MT in DCI format 2_0 can indicate to the IAB-MT a slot format from the slot formats in Table 14-2.

Table 14-2: Slot formats for normal cyclic prefix

| Slot Format | Symbol number in a slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| 56 | U | U | U | U | U | U | U | U | U | U | U | U | U | F |
| 57 | U | U | U | U | U | U | U | U | U | U | U | U | F | F |
| 58 | U | U | U | U | U | U | U | U | U | U | U | F | F | F |
| 59 | U | U | U | U | U | U | U | U | U | U | F | F | F | F |
| 60 | U | U | U | U | U | U | U | U | U | F | F | F | F | F |
| 61 | U | U | U | U | U | U | U | U | F | F | F | F | F | F |
| 62 | U | U | U | U | U | U | U | F | F | F | F | F | F | F |
| 63 | U | U | U | U | U | U | F | F | F | F | F | F | F | F |
| 64 | U | U | U | U | U | F | F | F | F | F | F | F | F | F |
| 65 | U | U | U | U | F | F | F | F | F | F | F | F | F | F |
| 66 | U | U | U | F | F | F | F | F | F | F | F | F | F | F |
| 67 | U | U | F | F | F | F | F | F | F | F | F | F | F | F |
| 68 | U | F | F | F | F | F | F | F | F | F | F | F | F | F |
| 69 | U | F | F | F | F | F | F | F | F | F | F | F | F | D |
| 70 | U | U | F | F | F | F | F | F | F | F | F | F | F | D |
| 71 | U | U | U | F | F | F | F | F | F | F | F | F | F | D |
| 72 | U | F | F | F | F | F | F | F | F | F | F | F | D | D |
| 73 | U | U | F | F | F | F | F | F | F | F | F | F | D | D |
| 74 | U | U | U | F | F | F | F | F | F | F | F | F | D | D |
| 75 | U | F | F | F | F | F | F | F | F | F | F | D | D | D |
| 76 | U | U | F | F | F | F | F | F | F | F | F | D | D | D |
| 77 | U | U | U | F | F | F | F | F | F | F | F | D | D | D |
| 78 | U | U | U | U | U | U | U | U | U | U | U | U | F | D |
| 79 | U | U | U | U | U | U | U | U | U | U | U | F | F | D |
| 80 | U | U | U | U | U | U | U | U | U | U | F | F | F | D |
| 81 | U | U | U | U | U | U | U | U | U | U | U | F | D | D |
| 82 | U | U | U | U | U | U | U | U | U | U | F | F | D | D |
| 83 | U | U | U | U | U | U | U | U | U | F | F | F | D | D |
| 84 | U | F | D | D | D | D | D | D | D | D | D | D | D | D |
| 85 | U | U | F | D | D | D | D | D | D | D | D | D | D | D |
| 86 | U | U | U | F | D | D | D | D | D | D | D | D | D | D |
| 87 | U | F | F | D | D | D | D | D | D | D | D | D | D | D |
| 88 | U | U | F | F | D | D | D | D | D | D | D | D | D | D |
| 89 | U | U | U | F | F | D | D | D | D | D | D | D | D | D |
| 90 | U | F | F | F | D | D | D | D | D | D | D | D | D | D |
| 91 | U | U | F | F | F | D | D | D | D | D | D | D | D | D |
| 92 | U | U | U | F | F | F | D | D | D | D | D | D | D | D |
| 93 | U | U | U | U | U | U | U | U | U | F | F | F | F | D |
| 94 | U | U | U | U | U | U | F | F | F | F | F | F | D | D |
| 95 | U | U | U | U | U | U | F | F | D | D | D | D | D | D |
| 96 | U | U | U | U | U | U | U | D | D | D | D | D | D | D |

For a serving cell of an IAB-MT, the IAB-MT can be provided by Provided Guard Symbols MAC CE a number of symbols that will not be used for the IAB-MT in slots where the IAB-node transitions between IAB-MT and IAB-node DU and a SCS configuration for the number of symbols [11, TS 38.321].

With reference to slots of an IAB-DU cell, a symbol in a slot of an IAB-DU cell can be configured to be of hard, soft, or unavailable type by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473]. When a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive in the symbol.

When a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive in the symbol only if

-	the IAB-MT does not transmit or receive during the symbol of the IAB-DU cell, or

-	with respect to all serving cells, the IAB-MT would transmit or receive during the symbol of the IAB-DU cell, and the transmission or reception during the symbol of the IAB-DU cell is not changed due to a use of the symbol by the IAB-DU, or

-	the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available if the IAB-MT is not configured with an SCG, or

-	the IAB-MT detects two DCI formats 2_5 with an AI index field indicating the soft symbol as available from the MCG and SCG, respectively, or

-	the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available from one cell group and with respect to all serving cells of the other cell group

-	the IAB-MT does not transmit or receive during the symbol of the IAB-DU cell, or

-	the IAB-MT would transmit or receive during the symbol of the IAB-DU cell, and the transmission or reception during the symbol of the IAB-DU cell does not change due to a use of the symbol by the IAB-DU.

When the IAB-MT receives a DCI format 2_5 from a serving cell in a cell group, the IAB-MT applies the information of the DCI format 2_5 to all serving cells of the cell group.

When a symbol is configured as unavailable, the IAB-DU neither transmits nor receives in the symbol.

A symbol of a slot is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS in the symbol of the slot, or would receive a PRACH or a SR in the symbol of the slot. An RB set of a symbol is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS in the RB set of the symbol, or would receive a PRACH or a SR in the RB set of the symbol.

With reference to slots of an IAB-DU cell, the IAB-DU can be provided an indication of hard, soft or unavailable type per RB set for symbols configured as downlink, uplink or flexible in a slot by Frequency-Domain HSNA Configuration List [16, TS 38.473]. The RB set size and the number of RB sets are configured by RB-Set-Configuration [16, TS 38.473]. If an indication of hard, soft or unavailable type is not provided for an RB set of a symbol in a slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot. If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Frequency-Domain HSNA Configuration List [16, TS 38.473] when the IAB-node uses simultaneous transmission and reception in the slot.

When an RB set of a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive on the RB set in the symbol.

When an RB set of a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive on the RB set in the symbol only if

-	the IAB-MT does not transmit or receive on the RB set during the symbol of the IAB-DU cell, or

-	with respect to all serving cells, the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set or any RB set that is configured as unavailable or configured as soft and not indicated as available during the symbol of the IAB-DU cell is not changed due to a use of the RB set in the symbol by the IAB-DU, or

-	the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available if the IAB-MT is not configured with an SCG, or

-	the IAB-MT detects two DCI formats 2_5 with an AI index field value indicating the soft RB set as available from the MCG and SCG, respectively, or

-	the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available from one cell group and with respect to all serving cells of the other cell group, the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set during the symbol of the IAB-DU cell does not change due to a use of the RB set in the symbol by the IAB-DU.

When an RB set of a downlink, uplink, or flexible symbol is configured as unavailable, the IAB-DU neither transmits nor receives in the RB set in the symbol.

If an IAB-node is provided an AvailabilityIndicator, the IAB-node is provided an AI-RNTI by ai-RNTI and a payload size of a DCI format 2_5 by dci-PayloadSizeAI. The IAB-node is also provided a search space set configuration, by SearchSpace, for monitoring PDCCH.

For each cell of an IAB-DU in a set of cells of the IAB-DU, the IAB-DU can be provided:

-	an identity of the IAB-DU cell by iab-DU-CellIdentity

-	a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI or by positionInDCI-AI-r17

-	a set of availability combinations by availabilityCombinations or by availabilityCombinationsRBGroups, where each availability combination in the set of availability combinations includes

-	resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-DU cell, or one resourceAvailability indicating availability of soft resources in all RB sets in one or more slots for the IAB-DU cell, or one or multiple RbSetGroups with each RbSetGroup indicating resourceAvailability for soft resources in one or more slots for the associated rbSets, and

-	a mapping for the soft symbol, and/or for soft resources, availability combinations provided by resourceAvailability to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId

The IAB-DU can assume a same SCS configuration for availabilityCombinations for a cell as an SCS configuration provided by gNB-DU Cell Resource Configuration for the cell.

An AI index field value in a DCI format 2_5 indicates to an IAB-DU a soft symbol or a soft RB set in an RB set group availability in each slot for a number of slots starting from the earliest slot of the IAB-DU which overlaps in time with the slot of the IAB-MT where the IAB-MT detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace. The AI index field includes maxlog2maxAIindex+1,1 bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An availability for a soft symbol or a soft RB set in an RB set group in a slot is identified by a corresponding value resourceAvailability as provided in Table 14-3.

Table 14-3: Mapping between values of resourceAvailability elements and types of soft symbol or soft RB set availability in a slot

| Value | Indication |
| --- | --- |
| 0 | No indication of availability for soft symbols or soft RB sets in an RB set group |
| 1 | DL soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for UL and Flexible soft symbols or RB set group |
| 2 | UL soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for DL and Flexible soft symbols or RB set group |
| 3 | DL and UL soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for Flexible soft symbols or RB set group |
| 4 | Flexible soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for DL and UL soft symbols or RB set group |
| 5 | DL and Flexible soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for UL soft symbols or RB set group |
| 6 | UL and Flexible soft symbols or soft RB sets in an RB set group are indicated available No indication of availability for DL soft symbols or RB set group |
| 7 | DL, UL, and Flexible soft symbols or soft RB sets in an RB set group are indicated available |

If a PDCCH monitoring periodicity for DCI format 2_5 is smaller than a duration of an availability combination of soft symbols over a number of slots that the IAB-MT obtains at a PDCCH monitoring occasion for DCI format 2_5 by a corresponding AI index field value, and the IAB-MT detects more than one DCI formats 2_5 indicating an availability combination of soft symbols or of soft RB sets in RB set groups in a slot, the IAB-MT expects that each of the more than one DCI formats 2_5 indicates a same value for the availability combination of the soft symbols or of soft RB sets in an RB set group in the slot. An IAB-MT monitors PDCCH candidates for a DCI format 2_5 with CRC scrambled by AI-RNTI in one or both of the following search space sets:

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common;

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific.

The IAB-node can be provided by the parent node a set of RS resource indexes that indicate quasi co-location properties of an IAB-DU cell where simultaneous transmission/reception from the IAB-MT and transmission from the IAB-DU cells is restricted by Child IAB-DU Restricted Beam Indication MAC CE as described in [11, TS 38.321]. The IAB-DU does not transmit on a cell if the IAB node is operating in a non-TDM multiplexing mode using an indicated RS resource index on a symbol or RB set configured as soft in an IAB-DU cell

-	when it is not indicated as available by resourceAvailability

-	when the IAB-MT is operating on an associated carrier, if that indication is provided

-	when the current IAB-DU transmission mode corresponds to an associated multiplexing mode, if that indication is provided

-	when one of the associated TCI states, RS resource indexes, or SRI of the IAB-MT, if provided, is simultaneously used for reception or transmission of the IAB-MT

-	when simultaneous transmission/reception by the IAB-MT and transmission from the IAB-DU cell occur in non-overlapping frequency resources, if such indication is provided, or when simultaneous transmission/reception by the IAB-MT and transmission from the IAB-DU cell occur in overlapping frequency resources

-	in a given slot, if that indication is provided

For a serving cell of an IAB-MT, the IAB-MT can be provided a set of TCI states or a set of RS resource indexes corresponding to a SS/PBCH block or to a CSI-RS resource index for a slot where a PDSCH EPRE adjustment is indicated by DL Tx Power Adjustment MAC CE as described in [11, TS 38.321]. The PDSCH EPRE can be derived from a downlink CSI-RS EPRE as described in [6, TS 38.214] and a PDSCH power offset provided by powerControlOffsetIAB as described in [11, TS 38.321]. For a downlink DM-RS and/or PT-RS associated with a PDSCH, the IAB-MT may assume that the ratio of PDSCH EPRE to DM-RS EPRE, and/or PT-RS EPRE to PDSCH EPRE, is obtained as for a "UE" in [6, TS 38.214]. If no TCI state or RS resource index is provided to the IAB-MT, the IAB-MT may assume that a same PDSCH EPRE adjustment applies to all TCI states or RS resource indexes configured for the IAB-MT. A PDSCH EPRE adjustment provided by DL Tx Power Adjustment MAC CE may be associated with

- multiplexing mode of the IAB-node, if provided, and/or

- when simultaneous reception by the IAB-MT and transmission/reception by an IAB-DU cell occur in non-overlapping frequency resources, if provided, or when simultaneous reception by the IAB-MT and transmission/reception by an IAB-DU cell occur in overlapping frequency resources, if provided, and/or

- slots indicated by slot index, if provided