3GPP TSG RAN meeting #97e										RP-221947

Online, 12 – 16 September, 2022

Status Report to TSG

Agenda item:			9.3.2.4

| WI / SI Name | NR sidelink relay enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_SL_relay_enh |  |  |  |  |
| Unique ID | 941002 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221262 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 12/2023 | Performance part: 06/2024 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: 10% | Performance Part: 0% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Youngdae LEE |
|  | Company | LG Electronics Inc. |
|  | Email | youngdae.lee@lge.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#119-e (August 2022)

71 contributions ([1] ~ [71]) were submitted in this meeting. The following agreements were made:

UE-to-UE relay

- RAN2 confirm that the Scenario, Assumption and Requirement in section 5.1 of TR 38.836 apply for UE-to-UE relay support, with below clarifications:

  - For cast type on UE-to-UE communication, only unicast is considered

  - FFS if coverage and RRC state aspects need to be revisited in light of the existing U2N support.

  - RAN2 will follow SA2 decision on the discovery model including cast type.

- gNB will not configure a Uu RSRP threshold to be used by U2U Relay or Remote UE to determine whether to transmit U2U discovery signalling.  FFS what conditions would govern transmission of the discovery signalling.

Service continuity enhancements for L2 UE-to-network relay

- For inter-gNB d2i path switching and intra-/inter-gNB i2i path switching in Rel-18, the network can select a target U2N relay UE in any RRC state, i.e., RRC_CONNECTED/IDLE/INACTIVE.

- For the target U2N relay UE in any RRC state, the Rel-17 procedures for intra-gNB d2i path switching are used as a baseline for inter-gNB d2i path switching with the addition of inter-gNB signaling over the Xn interface.

- The Rel-17 remote UE oriented solution to trigger the target U2N relay UE to the CONNECTED state should also be applicable to the Rel-18 inter/intra-gNB scenarios as a baseline for single-path relay.  Other mechanisms are not excluded if an issue is found with the baseline.

- When indirect-to-indirect path switch is initiated, the Remote UE can inform upper layers to release the PC5 unicast link with the source relay UE. The timing to execute link release is up to UE implementation.

- Introduce a new measurement event that considers both the PC5 link quality with the serving Relay UE and that with candidate Relay UE for the indirect-to-indirect path switch purpose.  FFS if there would be more than one event type.

- For the signalling and procedures in Uu and PC5, intra-gNB indirect-to-direct path switch is used as the baseline for inter-gNB i2d path switch.

Multi-path relaying

- RAN2 anticipate benefits from multi-path in the following areas:

  - Relay and direct multi-path operation (including both scenarios 1 and 2) can provide efficient path switching between direct path and indirect path

  - The remote UE in multi-path operation can provide enhanced user data throughput and reliability compared to a single link

  - gNB can offload the direct connection of the remote UE in congestion to indirect connection via the relay UE (e.g. at different intra/inter-frequency cells)

- RAN2 can confirm the justifiable benefits that multi-path with relay and UE aggregation can improve the throughput and reliability/robustness, e.g., for UE at the edge of a cell, and UE with limited UL transmission power.

- The terms “relay UE” and “remote UE” are used for scenarios 1 and 2.  FFS if we would use additional terms specific to scenario 2.

- Confirm the remote UE in Scenario 1 and the remote UE in Scenario 2 as follows:

  - Scenario 1: the remote UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay,

  - Scenario 2: the remote UE is connected to the same gNB using one direct path and one indirect path via 2) via another UE (where the UE-UE inter-connection is assumed to be ideal).

- RAN2 assumes that the relation between remote UE and relay UE in scenario 2 is pre-configured or static and how the relation is pre-configured or static is out of the 3GPP scope.

- RAN2 deprioritizes discussion on authorization and association mechanism between remote UE and relay UE in scenario 2.

- Support the following cell deployment scenarios for multi-path relaying in Rel-18:

  - Scenario C1: The relay UE and remote UE are served by a same cell.

  - Scenario C2: The relay UE and remote UE are served by different intra-frequency cells of a same gNB

  - Scenario C3: The relay UE and remote UE are served by different inter-frequency cells of a same gNB

- Support the following sidelink scenarios for multi-path:

  - Scenario S1: SL TX/RX and Uu share the same carrier at the remote UE.

  - Scenario S2: SL TX/RX and Uu use different carriers at the remote UE.

  - Scenario S3: SL TX/RX and Uu share the same carrier at the relay UE.

  - Scenario S4: SL TX/RX and Uu use different carriers at the relay UE.

- Support direct bearer (bearer mapped to direct path on Uu), indirect bearer (bearer mapped to indirect path via relay UE), and MP split bearer (bearer mapped to both paths, based on the existing split bearer framework).

- For a MP split bearer in scenario 1, one PDCP entity at the remote UE is configured with one direct Uu RLC channel and one indirect PC5 RLC channel.

  - For upstream, a PDCP entity delivers to a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side.

  - For downstream, a PDCP entity receives from a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side.

- FFS if we need to take decisions on the mapping of protocol entities in scenario 2.

2.2.2	Remaining Open issues

- Mechanisms to support single-hop Layer-2 and Layer-3 UE-to-UE relay (i.e., source UE -> relay UE -> destination UE) for unicast

- Mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay

- Study on potential solutions for multi-path support in Scenario 1 and 2

- Support of sidelink DRX for Layer-2 UE-to-Network sidelink relay operation if not done in Rel-17

2.3	RAN3

2.3.1	Agreements

RAN3#117-e (August 2022)

36 contributions ([72] ~ [107]) were submitted in this meeting. The following agreements were made:

UE-to-UE relay

- RAN3 waits for RAN2 progress on E2E PC5 QoS split for U2U relay.

- WA: NG-RAN receives the multi-path authorization from the AMF.

- WA: Support U2U relay in CU-DU split architecture, FFS on the enhancements.

Service continuity enhancements for L2 UE-to-network relay

- Reuse the existing network procedures to support single-hop L2 U2N Relay in Rel-18.

- Source gNB decides to trigger path switching for the L2N remote UE.

- Current signaling can support Scenario C, i.e., intra-gNB indirect to indirect path switch.

- RAN3 focuses on the XnAP and possible F1AP impacts to support the basic scenarios.

- FFS on which node should decide the new path type, i.e., either indirect or direct.

- Regarding the support of lossless data delivery during path switch, RAN3 would wait for RAN2’s progress first.

- RAN3 continues analyzing the following options for selection of target Relay UE.

  - Option 1: source gNB selects one target Relay UE and sends the ID related information to the target gNB

  - Option 2: source gNB sends a list of candidate target Relay UE information to the target gNB for selection

  - Option 3: source gNB provides also the measurement information of Remote UE to the target gNB for selection of target Relay UE

Multi-path support

- From RAN3 perspective, multi-path scenario should be supported in Rel-18.

- Both intra-DU and inter-DU cases will be supported under the same gNB.

- RAN3 waits for the RAN2 progress on how to define control plane and user plane scenarios for multi-path support.

- RAN3 waits for the RAN2 progress on whether and how to define the Primary path in multi-path support.

- Addition of direct/indirect path are supported as follows:

  - Add direct path, after the establishment of the indirect path.

  - Add indirect path, after the establishment of the direct path.

  - This does not imply the exclusion of any other path addition possibility.

- RAN3 will study the signaling impact on the direct or indirect path change under the same gNB for a UE connected via multi-path. The other mobility scenarios can be further considered based on RAN2 decision.

- The following use cases are not supported in Rel-18.

  - Configure two indirect paths

  - More than two paths

  - Inter-gNB multi-path support

2.3.2	Remaining Open issues

- Signalling support for Relay and remote UE authorization if SA2 concludes it is needed

- Mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay

- Study on potential solutions for multi-path support in Scenario 1 and 2

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

- RRM core requirements for relay discovery and (re)selection in UE-to-UE relay

- RRM performance requirements for relay discovery and (re)selection in UE-to-UE relay

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#119-e

- R2-2207015	Discussion on multi-path SL relay	OPPO

- R2-2207077	Discussion on NR sidelink UE-to-UE relay	OPPO

- R2-2207078	Discussion on further enhancement of U2N service continuity	OPPO

- R2-2207126	Discovery and Relay (re-)selection for UE-to-UE relay	Qualcomm Incorporated

- R2-2207133	Service continuity for UE-to-Network relay	Qualcomm Incorporated

- R2-2207137	Initial discussion on multi-path operation for UE-to-Network relay	Qualcomm Incorporated

- R2-2207169	Service Continuity Enhancement for Sidelink Relay	MediaTek Inc.

- R2-2207170	Connection management and procedures for L2 UE-to-UE relay	MediaTek Inc.

- R2-2207180	Discussion on multi-path	Xiaomi

- R2-2207181	Discussion on service continuity enhancement	Xiaomi

- R2-2207182	Discussion on U2U relay discovery and relay selection	Xiaomi

- R2-2207187	Initial consideration on the multi-path relaying	ZTE, Sanechips

- R2-2207198	Discussion on U2U relay discovery and (re)selection	ZTE

- R2-2207199	Discussion on Service continuity enhancement for U2N relay	ZTE

- R2-2207220	Service Continuity Enhancements for Layer-2 UE-to-Network Relays	Ericsson España S.A.

- R2-2207221	Multipath Support with Direct path and Indirect path	Ericsson España S.A.

- R2-2207239	Discussion on Sidelink U2U Relay Discovery and (Re-)Selection	Fraunhofer IIS, Fraunhofer HHI

- R2-2207252	Design aspects of relay selection and reselection for U2U relay	Ericsson

- R2-2207278	Discovery and reselection with UE-to-UE relaying	Intel Corporation

- R2-2207279	Service continuity enhancements for L2 U2N relaying	Intel Corporation

- R2-2207280	Discussion on Multi-path Relaying	Intel Corporation

- R2-2207336	Basic aspects for U2U Relay work	Lenovo

- R2-2207361	Multipath support for remote UE	MediaTek Beijing Inc.

- R2-2207420	Discussion on Service continuity enhancement of L2 U2N relay	Apple

- R2-2207457	Discussion on U2U Relay Discovery and Relay (Re)-selection	Apple

- R2-2207458	Discussion on multi-path support	Apple

- R2-2207520	Discussion on U2U Relay Discovery and (Re)selection	CATT

- R2-2207521	Service Continuity Enhancements for L2 U2N Relay	CATT

- R2-2207522	Discussion on Multi-path	CATT

- R2-2207642	Discussion on service continuity enhancements for L2 U2N relay	China Telecom

- R2-2207643	Discussion on multi-path support to enhance reliability and throughput	China Telecom

- R2-2207644	Discussion on mechanisms to support UE-to-UE relay	China Telecom

- R2-2207652	Service continuity enhancements for L2 U2N relay	LG Electronics France

- R2-2207653	Consideration for UE-to-UE relay operation	LG Electronics France

- R2-2207686	Discussion on relay discovery and (re)selection for U2U relay	Spreadtrum Communications

- R2-2207687	Service continuity enhancements support for L2 U2N relay	Spreadtrum Communications

- R2-2207688	Discussion on multi-path relaying	Spreadtrum Communications

- R2-2207700	Discussion on Service continuity in U2N relay case	Lenovo

- R2-2207701	Discussion on Multi-path relaying	Lenovo

- R2-2207729	Overall views on U2U sidelink relay	Samsung R&D Institute UK

- R2-2207838	UE-to-UE relay cell reselection and Relay UE DRX	Sony

- R2-2207839	Service continuity enhancements for UE sidelink relay	Sony

- R2-2207840	Multi-path relaying discussion	Sony

- R2-2207847	Discussion on multipath for sidelink relay enhancement	Samsung

- R2-2207860	Scenarios that require UE-to-UE relay (re)selection	Sharp

- R2-2207861	UE-to-UE relay discovery and (re)selection	Sharp

- R2-2207862	benefit of multi-path relay	Sharp

- R2-2207963	Considerations on Service Continuity Enhancement	NEC Corporation

- R2-2207964	Considerations on Multipath of Sidelink Relay	NEC Corporation

- R2-2208005	Clarifications on the scope of SL based U2U Relay	Nokia, Nokia Shanghai Bell

- R2-2208006	Discussion on service continuity enhancement for L2 U2N relay	Nokia, Nokia Shanghai Bell

- R2-2208039	Initial considerations for U2U L2 relay CP operations	Kyocera

- R2-2208041	Initial considerations for U2U relay discovery and (re)selection 	Kyocera

- R2-2208081	Multi-path UE aggregation on PC5 and Ideal-link	vivo

- R2-2208082	On service continuity enhancement for L2 U2N relay	vivo

- R2-2208083	Discussion on L2 and L3 U2U relay	vivo

- R2-2208151	Discovery and Relay Selection for UE-to-UE Relays	InterDigital

- R2-2208152	Scenarios, Use Cases, and Protocol Architecture for Multi-path	InterDigital

- R2-2208153	Design Aspects for Multi-path	InterDigital

- R2-2208154	Considerations on reliability and throughput for multi-path	Nokia, Nokia Shanghai Bell

- R2-2208158	U2N Relay UE operation Threshold Conditions: Impact of UE Mobility	Philips International B.V.

- R2-2208229	Discussion on Service Continuity	Huawei, HiSilicon

- R2-2208260	Service continuity enhancement for L2 U2N relay	Samsung

- R2-2208345	Work plan for NR sidelink relay enhancements	LG Electronics France

- R2-2208349	Multi-path relaying for NR sidelink relay enhancements	LG Electronics France

- R2-2208427	Consideration on U2U relay	CMCC

- R2-2208428	Service continuity on U2N relay	CMCC

- R2-2208429	Multi-path and UE aggregation	CMCC

- R2-2208488	Discussion on Rel-18 multi-path via SL relay and UE aggregation	Huawei, HiSilicon

- R2-2208489	Discussion on UE-to-UE relay	Huawei, HiSilicon

- R2-2208809	[AT119-e][427][Relay] Handling of scenarios 1 and 2	LG Electronics France

RAN3#117-e

- R3-224749	Work plan for NR sidelink relay enhancements		LG Electronics

- R3-224370	Discussion on UE authorization for U2U relay		ZTE

- R3-224380	Authorization information for multi-path			Nokia, Nokia Shanghai Bell

- R3-224551	SL relay: U2U relay						Huawei

- R3-224600	Authorization for multi-path support and U2U relays	Qualcomm Incorporated

- R3-224750	Discussion on authorization for UE-to-UE relay operation	LG Electronics

- R3-224757	Authorization Enhancement to support Rel.18 Sidelink Relay	Samsung

- R3-224904	Consideration on authorization for U2U relay			CMCC

- R3-225020	Summary of offline discussion on CB# SLRelay1_Authorization	ZTE

- R3-224371	Discussion on Service continuity enhancement for U2N relay	ZTE

- R3-224381	Discussion on Support Service Continuity Enhancements		Nokia, Nokia Shanghai Bell

- R3-224398	Service continuity for L2 U2N relay				China Telecommunication

- R3-224522	Service Continuity Enhancements for Layer-2 UE-to-Network Relays	Ericsson

- R3-224523	RAN decision on selection of the target Relay UE		Ericsson

- R3-224552	SL relay: Inter-gNB mobility					Huawei

- R3-224601	Service continuity enhancements for L2 relays		Qualcomm Incorporated

- R3-224739	Support Service Continuity Enhancements for SL relay	CATT

- R3-224740	Correction to 38.423 for SL relay (R18)			CATT

- R3-224751	Service continuity enhancements for L2 U2N relay		LG Electronics

- R3-224832	(TP to TS 38.401) Initial discussion on service continuity enhancement		Samsung

- R3-224905	Service continuity for U2N relay					CMCC

- R3-225021	Summary of Offline Discussion on SL Relay Service Continuity		Ericsson

- R3-225084	Summary of Offline Discussion on SL Relay Service Continuity		Ericsson

- R3-224345	Authorization and mobility for Multi-path UE aggregation	vivo

- R3-224372	Initial considerations on multi-path support			ZTE

- R3-224382	Discussion on multi-path support				Nokia, Nokia Shanghai Bell

- R3-224392	On multi-path support for sidelink relay			China Telecommunication

- R3-224524	Multi-path support with Direct path and Indirect path	Ericsson

- R3-224553	SL relay: Multi-path relay and UE aggregation		Huawei

- R3-224602	Multi-path support for sidelink relays				Qualcomm Incorporated

- R3-224741	Multi-path Support for SL relay					CATT

- R3-224752	Multi-path relaying for NR sidelink relay enhancements	LG Electronics

- R3-224831	(TP to TS38.401 on SL Relay) Discussion on multipath for sidelink relay	Samsung

- R3-224906	Multi-path and UE aggregation					CMCC

- R3-225022	SoD of CB: # SLRelay3_Others					LG Electronics

- R3-225086	SoD of CB: # SLRelay3_Others 				LG Electronics

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template