3GPP TSG RAN meeting #97-e									RP-222152

Electronic Meeting, September 12-16, 2022

Status Report to TSG

Agenda item:		9.5.4.2

| WI / SI Name | Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FR1_TRP_TRS |  |  |  |  |
| Unique ID | 911010 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221785.zip |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Mar/2022 | Performance part: Sep/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: 100%* | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

*Note: Completion level percentage is quantified based on RAN4 decision of further prioritized requirement objectives. A revised WID is provided in RP-222269.

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Wang, Ruixin; Liu, Qifei; Shaozhe |
|  | Company | vivo, OPPO, CMCC |
|  | Email | ruixin.wang@vivo.com liuqifei@oppo.com shaozhe@CHINAMOBILE.COM |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.4	RAN4

2.4.1	Agreements

RAN4 #104-e (Aug 2022, Electronic Meeting)

- General

- The email discussions have been summarized in [39] [40]

- WF on concluding FR1 TRP TRS was approved [42]

- WF on FR1 TRP TRS for UE with multi-antenna and test time reduction [41]

- TP to TS 38.161 on general aspects was agreed [44]

- TP to TS 38.161 on TRP TRS requirements was agreed [45]

- Reply LS to RAN5 on TT and requirement framework was approved [46]

- CR to TR 38.834 on TAA configuration was agreed [43]

- Outcome of 3GPP TRP TRS lab alignment activity was discussed and concluded [25]

- Analysis of TRP TRS Performance Test Campaign and Proposals for requirements was discussed and concluded [26]

- 3GPP TS 38.161 v0.4.0 was approved [18]

•	Agreements on concluding FR1 TRP TRS

Issue 1-2: Proposals on concluding Rel-17 TRP TRS WI

Agreements:

- Proposal 1: RAN4 conclude the successful Rel-17 FR1 TRP TRS lab alignment activity, all the 8 test labs are well aligned.

- Proposal 2: RAN4 should conclude the Test Tolerance for FR1 TRP TRS and provide reply LS to RAN5.

- Proposal 3: RAN4 should finalize Rel-17 FR1 TRP TRS requirement for n41 and n78 in RAN4#104-e meeting.

- Proposal 4: With concluding the TRP TRS requirements, an LS to other OTA groups is needed to ensure industry coordination on this topic.

- Proposal 5: TRP TRS test methods study for 2Tx chain, reverb-chamber system, RedCap UE, and other deprioritized aspects can be further discussed and organized in a new Rel-18 TRP TRS WI.

Issue 2-1: EN-DC decision tree

Agreements:

- Proposal 1: It is proposed to define the decision tree to select the EN-DC band combination for TRP/TRS testing as outlined in Figure 1. The refined decision tree will be added in the revision of TP R4-2212813.

- Proposal 2: As a working principle in future TRP/TRS efforts, it will be necessary to continue to add an example EN-DC band combination for each new band that is added to the scope of TRP/TRS requirements.

Issue 3-1-1: Outcome of lab alignment activity

Agreement:

3GPP Rel-17 FR1 TRP TRS lab alignment activity can be successfully concluded and all the 8 labs (vivo, CAICT, CMCC, SRTC, OPPO, Sporton, Huawei, Element) with anechoic chamber system are well aligned.

Issue 3-1-2: Whether a new pass/fail limits is needed for lab alignment activity

Agreement:

Final pass/fail limits for Rel-17 FR1 TRP TRS lab alignment activity: 1.2dB for TRP and 1.5dB for TRS;

Issue 3-2-1: Recommended TT values for FR1 TRP TRS

Agreement:

Agree 0.62 * preliminary MU as TT recommendation (1.1dB TT for TRP and 1.4dB TT for TRS) in line with actual RAN4 3GPP lab alignment framework and measurements summarized in [R4-2212817].

Optimization of the MU assessment could be done in RAN5, but RAN4 suggests not to change the TT values for FR1 TRP TRS.

Issue 3-3-1: Requirements for FR1 TRP TRS

Agreement: TRP applies to PC2, TRS applies to both PC2 and PC3

- n41 TRP/TRS:12.5 dBm/-80dBm;

- n78 TRP/TRS:13 dBm/-81.2dBm;

•	Agreements on multi-antenna and test time reduction

Issue 1-2-1: How to evaluate multiple antenna system on UE?

Agreements:

- For multi-antenna system, the target is to develop the test methodologies with multi-antenna active if feasible.

- Test mode is a back-up solution for multi-antenna system measurement, and can be considered when no other valid test method identified at a specific time point in work plan.

Issue 1-2-2: View on dedicated TxD/TAS features algorithms and triggers

Agreement:

- Dedicated UE algorithms and triggers are not required to be declared.

Issue 1-2-3: Is a test campaign needed to correlate between test methodology and real environment expectations?

Agreement:

- The correlation between test methodology and real environment expectations for TxD and/or TAS is valuable.

- Further study and inputs in new WI of future release, if approved by RAN-P, are required before conducting the correlation test campaign, e.g. how to quantify “the real environment expectations”.

Issue 1-2-6: Proposal to test methodology for TAS ON

Agreement:

- Further study and validation in new WI of future release, if approved by RAN-P, on the TAS methodology that combining the measurement antenna and the link antenna to be one unified antenna in the OTA chamber.

  - The difficulty of link antenna replacement in current OTA chambers. Feedbacks from TE venders are welcome.

  - Feasibility verification based on test campaign with different devices.

  - Other issues on TAS methodology.

Sub-topic 2-1 Conclusion on test time reduction for EN-DC combinations

Agreement:

- Test time reduction is not essential demand for EN-DC bands combination consideration based on the conclusion that only one EN-DC band combination for each NR band is tested.

Sub-topic 2-2 Applicability of single point offset approach

Agreement:

- The single point offset approach can not be used without UE’s declaration of antenna pattern consistency.

2.4.2	Remaining Open issues

    - None

2.5	RAN5

2.5.1	Agreements

2.4.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

4.	References

RAN4#104-e

- R4-2211536	3GPP FR1 TRP/TRS Lab Alignment Measurement Results from Sporton International	Sporton International Inc

- R4-2211537	3GPP FR1 TRP/TRS Test Campaign Measurement Results from Sporton International	Sporton International Inc

- R4-2211557	On ENDC selection	Huawei Tech.(UK) Co.. Ltd

- R4-2211558	FR1 TRP TRS Lab Alignment Campaign Results	Huawei Tech.(UK) Co.. Ltd

- R4-2211559	On fast SAR measurement for TAS and TAA detection	Huawei Tech.(UK) Co.. Ltd

- R4-2211563	On TRP measurement under TxD	Huawei Tech.(UK) Co.. Ltd

- R4-2211570	TRP TRS lab alignment measurement from Huawei	Huawei Tech.(UK) Co.. Ltd

- R4-2211613	Element FR1 TRP/TRS Lab Alignment Campaign Measurement Results	Element Materials Technology

- R4-2211988	TP to TS 38.161 on ENDC low mid high test channel	Samsung

- R4-2211989	Discussion on FR1 TRP TRS performance requirement	Samsung

- R4-2212324	Commercial terminal TRP/TRS testing results of CMCC lab	CMCC

- R4-2212374	Remaining TRP and TRS EN-DC measurement aspects	Apple

- R4-2212375	Remaining issues with multiple antenna test methodologies	Apple

- R4-2212376	On harmonizing new alternative test methods	Apple

- R4-2212389	3GPP TRP TRS Performance measurement results from SRTC	SRTC

- R4-2212405	On TRP and TRS performance requirement phase	Apple

- R4-2212643	3GPP TRP TRS measurement results	CAICT

- R4-2212810	3GPP TS 38.161 v0.4.0	vivo

- R4-2212811	Measurement results for TRP TRS Performance Test Campaign	vivo

- R4-2212812	Proposals for concluding the Performance part work of TRP TRS WI	vivo

- R4-2212813	TP to TS 38.161 on general aspects	vivo

- R4-2212814	TP to TS 38.161 on TRP TRS requirements	vivo

- R4-2212815	Proposals on Test Tolerance for FR1 TRP TRS	vivo

- R4-2212816	Reply LS on TT and requirements framework	vivo

- R4-2212817	Outcome of 3GPP TRP TRS lab alignment activity	vivo

- R4-2212818	Analysis of TRP TRS Performance Test Campaign and Proposals for requirements	vivo

- R4-2213198	on the performance requirement for NR FR1 TRP TRS	Xiaomi

- R4-2213199	on the RAN5 LS on TT work for NR FR1 TRP TRS	Xiaomi

- R4-2213416	CR to TR 38.834 on TAA configuration	OPPO

- R4-2213417	3GPP TRP/TRS Performance measurement results	OPPO

- R4-2213419	Consideration on test time reduction	OPPO

- R4-2213423	OTA test method for transmit antenna switching	OPPO

- R4-2213424	OTA test method for TxD	OPPO

- R4-2213425	Percentile of CDF curve for final TRP TRS requirement	OPPO

- R4-2213426	Consideration on FR1 TRP TRS lab alignment	OPPO

- R4-2213570	Element FR1 TRP TRS Performance Measurement Results	Element Materials Technology

- R4-2214192	Email Discussion Summary for [104-e][332] FR1_TRP_TRS_Part1	Moderator (Vivo)

- R4-2214193	Email Discussion Summary for [104-e][333] FR1_TRP_TRS_Part2	Moderator (OPPO)

- R4-2214321	Email Discussion Summary for [104-e][332] FR1_TRP_TRS_Part1	Moderator (Vivo)

- R4-2214322	Email Discussion Summary for [104-e][333] FR1_TRP_TRS_Part2	Moderator (OPPO)

- R4-2214358	WF on FR1 TRP TRS for UE with multi-antenna and test time reduction	OPPO

- R4-2214359	WF on FR1 TRP TRS	vivo

- R4-2214542	CR to TR 38.834 on TAA configuration	 OPPO

- R4-2214795	TP to TS 38.161 on general aspects	vivo

- R4-2214796	TP to TS 38.161 on TRP TRS requirements	vivo

- R4-2214797	Reply LS on TT and requirements framework	vivo

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template