3GPP TSG RAN meeting #97-e									RP-222204

Electronic Meeting, September 12-16, 2022

## Status Report to TSG

Agenda item:			9.5.1.6

| WI / SI Name | Perf. part: Support of reduced capability NR devices |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_redcap-Perf |  |  |  |  |
| Unique ID | 900262 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-220966 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: | Performance part: 12/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: | Performance Part: 75% | Testing part: |  |

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Johan BERGMAN |
|  | Company | Ericsson |
|  | Email | johan.bergman@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	RAN4#104-e

236 contributions were submitted to this meeting (for details see agenda item 9.18 in Tdoc list)

RAN4 carried out the following offline email discussions:

- [104-e][223] NR_redcap_RRM_1

  - Summarized in R4-2214317

- [104-e][224] NR_redcap_RRM_2

  - Summarized in R4-2214274

- [104-e][328] NR_RedCap_Demod

  - Summarized in R4-2214275

RAN4 made the following agreements related to RRM performance requirements:

Testing principle was agreed. Test configurations (e.g. new SSB, SMTC configurations) for RedCap test cases was agreed. The workplan, including updated test cases and work split was agreed in R4-2213411. Following the agreed workplan, part 1 test cases were agreed at RAN4#104-e except few which were postponed to RAN4#104-bis-e. Applicability of cell specific RSRP offset was agreed in R4-2214484, and agreement was informed to RAN2. The exact value of offset will be discussed future meeting. Potential test cases for SDT were identified as follows: - - For 1 Rx. RedCap UEs in FR1, consider the test case with offsetRSRP if test case associated with sdt-RSRP-Threshold and cg-SDT-RSRP-ThresholdSSB is introduced in Rel-17 SDT test. - - For 1 Rx. RedCap UEs in FR1, consider the test case with offsetRSRPChange,CG-SDT under different RSRP change set-up if TA-validation for CG-SDT is introduced in Rel-17 SDT test.

RAN4 made the following agreements related to UE demodulation performance requirements (R4-2214394).

- Introduce UE demodulation and CSI reporting requirements for 2Rx RedCap UE supporting HD-FDD in FDD bands as well as 1Rx UE - The requirements with HD-FDD are the same as the corresponding (full-duplex) FDD requirements. - Interested companies can evaluate if the same requirements can be applied for both 2Rx HD-FDD and 2Rx FD-FDD. - Section names for RedCap UE demodulation and CSI reporting requirements - If new section is added for RedCap performance requirements, put ‘for RedCap’ (not ‘for RedCap UE’) for the section name - Applicable FR2 bands for RedCap UE - In FR2 bands, specify RedCap UE demodulation and CSI reporting requirements for FR2-1 only - Define 256QAM demodulation requirements for 1Rx RedCap UE in FR1 - Set MCS20 - CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD - Configure the following parameters for CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD: - CSI-RS periodicity and offset: 10/1 - CSI-Report periodicity and offset: 10/9 - CQI/RI/PMI delay: [14ms] - Interested companies are encouraged to evaluate the performance difference between CQI delay 14ms and 10ms in RAN4#104-bis-e. If significant performance degradation is observed compared with CQI delay 10ms, RAN4 will revisit the CQI/RI/PMI delay - If CQI delay 10ms is applied, consider CSI periodicity and offset is set to 10/5. - Lower test points for CQI reporting test in fading condition for 2Rx (FR1 FDD and TDD) - Set SNR=6/7dB for lower test points for CQI reporting test in fading condition for 2Rx - Static channel matrix used for 1Rx UE and SNR test point offset for CQI reporting tests - Interested companies are encouraged to evaluate the options for the static channel matrix and SNR test point offset. - Option 1: Set the static channel matrix in the frequency domain as H1RX=1j. Set SNR test point X=[0]dB lower than 2Rx test case. - Option 2: Keep the previous agreement on the static channel matrix in the frequency domain, that is,H1RX=11. Set SNR test point X=3dB lower than 2Rx test case. - Mapping of CQI index to information bit payload - For RedCap CSI reporting test using CQI table 1, apply the configuration in R4-2214394 for mapping of CQI index to information bit payload - FR1 - Reuse value 0 for the overhead parameter for TBS determination - Set 52RB for 10MHz/15kHz and 51RB for 20MHz/30kHz - FR2 - Reuse existing test for CQI reporting under static condition in clause 8.2.2.2.1 and corresponding CSI RMC for RedCap UE in FR2-1 - PMI feedback scheduling pattern (aperiodic CSI reporting) for both FD-FDD and HD-FDD - Configure the following parameters for PMI reporting tests for both FD-FDD and HD-FDD: - CSI request: 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0. - Reuse the FRC from Rel-15 PMI test (R.PDSCH 1-6.1 FDD) - Aperiodic Report Slot Offset: 3 slots - CQI/RI/PMI delay: 6ms - Define RI reporting requirements for RedCap 2Rx UEs - Apply Test 2 only - Replace fading CQI test for 2 Rx UE (high SNR point) by RI test case (Test 2)

#### 2.4.2	Remaining Open issues

UE demodulation requirements:

- Alignment of simulation results for UE demodulation requirements.

RRM performance requirements:

- Following the agreed work plan, part 2 test cases need to be discussed and agreed.

- Remaining test cases of part 1 (those which were postponed) need to be discussed and agreed.

## 4.	References

RAN4#104-e

236 contributions (for details see agenda item 9.18 in Tdoc list)