3GPP TSG RAN meeting #97   									RP-222219

Electronic Meeting, September 12-16, 2022

Status Report to TSG

Agenda item:			9.3.3.2

| WI / SI Name | Enhancement on NR QoE management and optimizations for diverse services |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_QoE_enh |  |  |  |  |
| Unique ID | 941008 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221803 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2023 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 10% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG3 |
| --- | --- | --- |
| Rapporteur | Name | Pei Li |
|  | Company | China Unicom |
|  | Email | lip104@chinaunicom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#119-e (Aug 2022)

- QoE measurements in RRC_IDLE INACTIVE

	- No need to send LS to SA4 since RAN3 is already sending an LS.

- The gNB can send the QoE configuration for MBS broadcast service to UE by RRC message in RRC_CONNECTED via dedicated signalling. The UE stores the configuration for QoE and performs the application layer measurement for MBS broadcast service.

- The baseline principles for QoE measurement collection for MBS services in RRC_INACTIVE and RRC_IDLE states are:

1)  	The UE is configured with IDLE/INACTIVE QoE via RRC.

2)	The UE buffers the QoE reports generated while in RRC IDLE/INACTIVE state.

- When the UE moves to RRC_CONNECTED state, the UE sends the QoE measurements availability indication to the gNB.

- For buffering of QoE reports generated in RRC IDLE/INACTIVE state, RAN2 should discuss at least the minimal memory size requirement.

- Left-over from R17

	- Not treated in this meeting.

2.2.2	Remaining Open issues

- FFS if configuration can be done in IDLE/INACTIVE states.

- FFS how does gNB determine which UEs can be configured with MBS QoE measurements

	- FFS if there is a new explicit indicator or new service type used for MBS QoE configuration in RRC_IDLE/RRC_INACTIVE. Wait for RAN3 progress and SA4 LS reply to RAN3.

- FFS if UE can setup/resume RRC connection just for QoE reporting, or whether the QoE reports are sent to the network when the UE moves to RRC CONNECTED state due to other reasons.

- For buffering of QoE reports generated in RRC IDLE/INACTIVE state, FFS if AS layer is responsible for storing the QoE reports (as in Rel-17).

2.3	RAN3

2.3.1	Agreements

RAN3#117-e (Aug 2022)

- Support for New Service Type and RRC_INACTIVE/RRC_IDLE states

	- Both signalling based and management based QoE measurements in RRC INACTIVE/IDLE mode shall be supported in Rel-18.

	- UE handles area scope checking for QoE measurements in RRC INACTIVE/IDLE mode.

	- Whether UE AS layer or UE APP layer handle the area scope is to be discussed based on RAN2 progress.

	- Support MBS broadcast service INACTIVE/IDLE QoE first.

	- UE shall keep the QoE configuration for MBS broadcast service configured in RRC_CONNECTED even when UE switches to RRC_IDLE and RRC_INACTIVE.

	- No LS on INACTIVE/IDLE QoE will be sent from RAN3 to RAN2 in this meeting.

	- If the UE receives the configuration in RRC connected state, a common QoE configuration mechanism is used to support QoE measurement configuration pertaining to MBS broadcast service for all RRC states, where the Rel-17 QoE configuration mechanism is adopted as baseline.

- Support QoE for NR-DC

	- MN is responsible to configure the s-based QoE to UE.

	- For M-based QoE configuration in NR-DC, coordination between MN and SN is needed. Details are FFS.

	- If the M-based QoE configuration is received by the MN, the MN should make the decision on the UE selection and on which node sends the QoE configuration to the UE.

	- If the M-based QoE configuration is received only by the SN, whether the MN or the SN performs UE selection and sends the QoE configuration to the UE needs to be further discussed.

	- QoE reports can be transmitted to either MN or SN and the reporting leg (MCG or SCG) can be changed during the application session. Send LS to RAN2.

	- WA: If QoE reports are received by the SN, SN can forward the QoE reports to MCE directly.

	- RAN3 should discuss and clarify the scenarios for QoE reporting transmitted over SN. Which SRB can be used for QoE reporting in SN depend on RAN2.

	- WA: MN and SN can generate RVQoE configurations.

	- MN and SN should coordinate about configuring a dual-connected UE with RVQoE measurements. The details of the coordination are FFS.

	- WA: UE can send RVQoE report to MN, MN then forward the RVQoE report to SN if needed, and vice versa.

- Left-over from R17

	- Introduce the slice scope information in the configuration container, and send LS out to SA4.

	- Definition of RVQoE value needs cooperation with SA4.

	- UE should include QoS flow information in the RVQoE report to RAN.

	- QoS flow information should be introduced as an explicit IE in the RAN visible QoE report over F1.

- QoE Measurement For Inter-RAT Handover

	- None.

- LS Out

	- LS on new service types for NR QoE[49].

	- LS to RAN2 on RAN3 agreement of QoE reporting in NR-DC[50].

	- LS to SA4 on R18 enhancement of NR QoE[51].

2.3.2	Remaining Open issues

	- Whether UE only reports the INACTIVE/IDLE QoE reports to gNB when the UE has entered to the RRC_CONNECTED due to other reasons shall be FFS.

	- The following aspects shall be discussed by RAN2:

which layer(e.g. app, as layer) is responsible to keep configured QoE configuration for MBS broadcast service at ue side when ue is in rrc_idle.

how long UE shall keep the QoE configuration for MBS broadcast service.

	- The use case and advantages of specifying a new QoE configuration mechanism for QoE measurements in INACTIVE/IDLE RRC states should be further clarified.​

	- RAN3 shall further clarify the use cases for high mobility scenario. whether enhancement is needed depends on the issues located during clarification.

	- FFS on how to control which leg is used for transmission of QoE reports in NR-DC.

	- FFS on whether QoE reports can be transmitted over MCG and SCG simultaneously, i.e., whether split SRB can be used to transmit QoE reports in NR-DC?

	- FFS whether a common or independent RVQoE configuration for MN and SN is sent to the UE.

	- FFS on whether both MN and SN may receive RVQoE reports from UE for NR-DC.

	- RAN3 to further discuss whether RAN visible QoE value should be generated directly by UE App layer, and/or with other involvement, e.g., UE AS layer.

	- RAN3 to further discuss what RAN3 wants as a RAN visible QoE value, and the following aspects can be considered:

	- whether RAN visible QoE value is calculated by one or more RAN visible QoE metrics

	- whether RAN visible QoE value is similar or different from MOS value defined in TS 26.909

	- other alternatives to define the RAN visible QoE value.

	- RAN3 to further discuss threshold-based triggers and event-based triggers for RAN visible QoE report, where the discussion should include but not limited to the clarification of the benefit of such triggers.

	- RAN3 to further discuss details on QoS flow information e.g., QoS flow ID, DRB ID, PDU session ID.

	- Further discuss whether the DU can activate/deactivate receiving the RAN visible QoE reports? Whether the DU can participate in assembling of RAN visible QoE configuration.

	- Further discuss whether OAM can send the priorities to NG-RAN for legacy QoE report.

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#119-e

- R2-2208619	Work Plan for Rel-18 NR QoE Enhancement	China Unicom	Work Plan	Rel-18	NR_QoE_enh

- R2-2207026	QoE measurement collection for IDLE and Inactive state	Qualcomm Incorporated	discussion	NR_QoE_enh-Core

- R2-2207427	IDLE/INACTIVE Mode QoE Measurements and Reporting	Apple	discussion	Rel-18	NR_QoE_enh-Core

- R2-2207532	Considerations on QoE measurements in RRC_IDLE and RRC_INACTIVE	Lenovo	discussion	Rel-18	NR_QoE_enh-Core

- R2-2207725	Discussion on QoE for MBS	Ericsson	discussion	Rel-17	NR_QoE_enh-Core

- R2-2207822	Discussion on MBS broadcast services	CATT	discussion	Rel-18	NR_QoE_enh-Core

- R2-2207992	QoE measurements for MBS broadcast services	Huawei, HiSilicon	discussion	Rel-18	NR_QoE_enh-Core

- R2-2208248	QMC enhancements for NR MBS	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_QoE_enh-Core

- R2-2208391	QoE measurement in RRC_IDLE and RRC_INACTIVE	Samsung	discussion	Rel-18

- R2-2208423	Discussion on QoE measurement in RRC_IDLE and RRC_INACTIVE	CMCC	discussion	Rel-18	NR_QoE_enh-Core

- R2-2208615	Discussion on Rel-18 QoE measurement	ZTE Corporation, Sanechips	discussion	Rel-18	NR_QoE_enh-Core

- R2-2208622	Discussion on MBS configuration and reporting for NR QoE in Rel-18	China Unicom	discussion	Rel-18	NR_QoE_enh

- R2-2207027	Discussion on Rel-17 leftover issues	Qualcomm Incorporated	discussion	NR_QoE_enh-Core

- R2-2207428	Views on Potential Enhancements of Existing QoE Features	Apple	discussion	Rel-18	NR_QoE_enh-Core

- R2-2207533	Discussion on Rel-17 leftover features for QoE	Lenovo	discussion	Rel-18	NR_QoE_enh-Core

- R2-2207724	Discussion on rel-17 leftovers	Ericsson	discussion	Rel-17	NR_QoE_enh-Core

- R2-2207823	Discussion on Rel-17 leftover issues for QoE	CATT	discussion	Rel-18	NR_QoE_enh-Core

- R2-2207993	Support of left-over features from Rel-17	Huawei, HiSilicon	discussion	Rel-18	NR_QoE_enh-Core

- R2-2208249	QMC enhancements for RAN overload	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_QoE_enh-Core

- R2-2208392	Timing information of measured samples	Samsung	discussion	Rel-18

- R2-2208616	Discussion on Rel-17 leftover issues for QoE	ZTE Corporation, Sanechips	discussion	Rel-18	NR_QoE_enh-Core

- R2-2208613	Recommended bitrate for XR services	MediaTek Beijing Inc.	discussion	Rel-18

- R2-2208629	On RAN visible QoE parameters for new services	China Telecom 	discussion

- R2-2209103 LS to SA4 on Rel-18 enhancement of NR QoE (R3-225227; contact: Huawei) Source: RAN3, To: S4, Cc: RAN2

- R2-2209109 LS to RAN2 on RAN3 agreement of QoE reporting in NR-DC (R3-225256; contact: China Unicom) Source: RAN3, To: RAN2

RAN3#117-e

- R3-224360	QMC Support for New Services and High-Mobility Scenarios	Ericsson

- R3-224361	QoE and RVQoE Measurement Support for MBS	Ericsson

- R3-224362	The Support for QoE and RVQoE Measurement and Reporting in NR-DC Scenarios	Ericsson

- R3-224363	The Enhancements of RAN Visible QoE Measurements and Reporting	Ericsson

- R3-224364	The Enhancements of QMC Rel-17 Features	Ericsson

- R3-224416	Discussion on the objectives of QoE enhancements	Lenovo

- R3-224417	QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE	Lenovo

- R3-224418	(TP to 38.423 & 38.420) Support of QoE measurement in RRC_INACTIVE	Lenovo

- R3-224419	QoE measurement in NR-DC	Lenovo

- R3-224420	(TP to TS 38.420) Support of QoE measurement in NR-DC	Lenovo

- R3-224457	On support of QMC for MBS and other service types surviving idle mode	Nokia, Nokia Shanghai Bell

- R3-224458	On support for QMC in NR-DC	Nokia, Nokia Shanghai Bell

- R3-224459	Initial observations on QMC support for Augmented Reality	Nokia, Nokia Shanghai Bell

- R3-224460	QMC enhancements for RAN overload	Nokia, Nokia Shanghai Bell

- R3-224589	Discussion on the support of R17 left-over features	Huawei

- R3-224610	QoE for new service types and high mobility scenarios	Qualcomm Incorporated

- R3-224611	QoE in RRC_IDLE and RRC_INACTIVE for MBS broadcast service	Qualcomm Incorporated

- R3-224612	Support for QoE in NR-DC	Qualcomm Incorporated

- R3-224613	Enhancements to RAN visible QoE	Qualcomm Incorporated

- R3-224758	Discussion on QMC in RRC_INACTIVE RRC_IDLE states	Xiaomi

- R3-224759	Discussion on QoE in NR-DC	Xiaomi

- R3-224760	Discussion on RVQoE value	Xiaomi

- R3-224761	Discussion on event-triggered RVQoE	Xiaomi

- R3-224788	Discussion on NR QoE for new service type	CATT

- R3-224789	Discussion on NR QoE in RRC_INACTIVE/ RRC_IDLE states	CATT

- R3-224790	Discussion on Support for legacy QoE in NR-DC	CATT

- R3-224791	Discussion on Support for RAN visible QoE in NR-DC	CATT

- R3-224792	Discussion on Left-over issues	CATT

- R3-224839	NR QoE Discussion on left over from R17	Samsung

- R3-224840	NR QoE Discussion on support for new service type and RRC inactive-idle state	Samsung

- R3-224841	NR QoE Discussion on support for NR-DC	Samsung

- R3-224842	Workplan for Rel-18 NR QoE Enhancement	China Unicom

- R3-224864	Discussion on QoE measurement in RRC_INACTIVE and RRC_IDLE states	China Unicom

- R3-224865	Discussion on QoE measurement in NR-DC	China Unicom

- R3-224866	Further discussion on R17 leftover issues	China Unicom

- R3-224869	Draft LS for R17 leftover issues	China Unicom

- R3-224875	Discussion on INACTIVE/IDLE QoE	ZTE Corporation

- R3-224876	Discussion on new service types and high mobility scenario	ZTE Corporation

- R3-224886	General considerations on R18 QoE enhancements	Huawei

- R3-224887	[Draft] LS to SA4 on R18 enhancement of NR QoE management and optimizations for diverse services	Huawei

- R3-224888	Initial discussions on the support of QoE measurement  in RRC_INACTIVE and RRC_IDLE states for MBS service	Huawei

- R3-224889	Discussions on the support for QoE in NR-DC	Huawei

- R3-224936	Discussion on the configuration and reporting of QoE and RVQoE in NR-DC	ZTE

- R3-224937	Discussion on MDT alignment and continuity in NR-DC	ZTE

- R3-224938	Discussion on R17 QoE left-over issues	ZTE

- R3-225010	CB: # QoE1_Inactive_Idle - Summary of email discussion	ZTE - moderator

- R3-225011	CB: # QoE2_NRDC - Summary of email discussion	China Unicom - moderator

- R3-225012	CB: # QoE3_Others - Summary of email discussion	Huawei - moderator

- R3-225255     LS on new service types for NR QoE

- R3-225256     LS to RAN2 on RAN3 agreement of QoE reporting in NR-DC

- R3-225227     LS to SA4 on R18 enhancement of NR QoE