3GPP TSG RAN Meeting #97-e									RP-222232

Electronic Meeting, September 12-16, 2022

## Status Report to TSG

Agenda item:			9.3.1.4

| WI / SI Name | NR MIMO evolution for downlink and uplink |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_evo_DL_UL |  |  |  |  |
| Unique ID | 940096 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213598 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 12/2023 | Performance part: 06/2024 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 20% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Yubo Yang |
|  | Company | Huawei |
|  | Email | yangyubo1@huawei.com |
| Rapporteur | Name | Riki Okawa |
|  | Company | NTT DOCOMO |
|  | Email | riki.ookawa.rp@nttdocomo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#110, the following agreements were made.

Multi-TRP enhancement

Agreement

On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, down-selection at least one alternative from the followings:

- Alt1-1: Use RRC parameter(s) in a CORESET configuration to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on the CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

- Alt1-2: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/DL TCI state(s) is associated with each CORESET group

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the CORESET group(s)

  - FFS: How to associate the indicated joint/DL TCI state(s) with each CORESET group

  - FFS: The UE applies the indicated joint/DL TCI state(s) to a CORESET according to the CORESET group(s) the CORESET belongs to, or the UE applies the indicated joint/DL TCI state(s) associated with the CORESET group(s) in which the beam indication DCI is received to all PDCCH receptions

- Alt2: The association between a CORESET and the indicated joint/DL TCI state(s) is determined based on a fixed rule, and the UE shall apply the indicated joint/DL TCI state(s) to the corresponding PDCCH receptions on the CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

- Alt3: Use MAC-CE to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on a CORESET

  - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE

Switching between multi-TRP and single TRP operation is not precluded

Agreement

On unified TCI framework extension, at least for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation

- FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP

- Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1

- Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17

Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based PDSCH reception and the required type(s) of TCI states (i.e., DL /UL/joint) are independently discussed in this AI

Agreement

On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2, down-selection one alternative from the followings:

- Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

- Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2

- Alt3: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/UL TCI state(s) is associated with each CORESET group. When a scheduling/activation DCI format 0_1/0_2 is received in a CORESET group, the indicated joint/UL TCI state(s) associated with the CORESET group is applied to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

  - FFS: Details of CORESET group(s)

FFS: PUSCH transmission scheduled/activated by a DCI format 0_0 and Type-1 CG-PUSCH

Agreement

On unified TCI framework extension for S-DCI based MTRP, to inform the association with joint/UL TCI state(s) indicated by DCI/MAC-CE for PUCCH transmission, down-selection at least one alternative from the followings:

- Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group

- Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group

- Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

- Alt4: Use DCI to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

Agreement

For multi-DCI based multi-TRP operation with two TAs, study how to handle overlapping part between two UL transmissions associated with two TAs, where the study includes:

- whether to introduce scheduling restriction in overlapping part

- whether to introduce dropping rules

- whether specification impact is needed, or if the issue can be handled via implementation

- whether to allow overlapped transmission in case the UE supports STxMP transmission (if STxMP feature is agreed in NR Rel-18)

Agreement

For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell.

Agreement

For multi-DCI based multi-TRP operation with two TAs, study the impact of two TAs for the following:

- RACH triggered by PDCCH order in intra-cell MTRP case

- RACH triggered by PDCCH order in inter-cell MTRP case

  - Which might require RACH enhancement as well

- UE triggered RACH by CBRA or CFRA in RRC connected mode

Further details of enhancements needed (if any)

Agreement

For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, downselect one of the options in RAN1#110bis-e:

- Option 1: Associate TAG to TCI-state/spatial relation

- Option 2: Associate TAG to CORESETPoolIndex

- Option 3: Associate TAG to DL RS group. For a UL transmission, UE adopts the TAG associated with the DL RS group to which the PL RS of the UL transmission belongs.

- Option 4: Associate TAG to target UL channels/RSs directly for semi-static UL channels/RSs (e.g. P CSI PUCCH, P SRS, CG PUSCH), and further discuss how to associate TAG to dynamic UL channels/RSs(e.g. via associating TAG to CORESETPoolIndex additionally, etc.)

Agreement

For multi-DCI multi-TRP operation with two TAs, up to two n-TimingAdvanceOffset value per serving cell is supported

CSI enhancement

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, support NTRP={1, 2, 3, 4} with equal priority.

Agreement

For the Rel-18 Type-II codebook for CJT mTRP, support RI={1,2,3,4}.

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, the following is supported:

- The CMR comprises K>1 NZP CSI-RS resources, where one resource corresponds to one TRP/TRP-group (i.e. K=NTRP)

  - Each of the CSI-RS resources has a same number of CSI-RS ports

- Note: The terms TRP and TRP-group are used for discussion purposes only (no spec impact is implied).

Agreement

For the Rel-18 Type-II codebook for CJT mTRP, support the following two modes:

- Mode 1: Per-TRP/TRP-group SD/FD basis selection which allows independent FD basis selection across N TRPs / TRP groups. Example formulation (N = number of TRPs or TRP groups):

- Mode 2: Per-TRP/TRP group (port-group or resource) SD basis selection and joint/common (across N TRPs) FD basis selection. Example formulation (N = number of TRPs or TRP groups):

- Striving for the two modes to share commonality in detailed designs such as parameter combinations, basis selection, TRP (group) selection, reference amplitude, W2 quantization schemes.

- FFS: Depending on the decision on SCI design, whether additional per-TRP/TRP-group amplitude scaling and/or co-phase is needed or not, and whether they are a part of W2s

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, down-select one from the following codebooks structures:

- Alt2A: Doppler-domain basis commonly selected for all SD/FD bases, e.g.

  - Note that  may be the identity as a special case

- Alt2B: Doppler-domain basis independently selected for different SD/FD bases

  - Note that  may be the identity as a special case

- Alt3. Reuse Rel-16/17 (F)eType-II codebook with multiple  and a single  and  report.

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, on the DD/TD basis waveforms:

- Down-select or combine from the following Doppler-/time-domain basis waveforms:

  - Alt1. Orthogonal DFT

    - TBD (by RAN1#110bis): whether rotation is used or not

    - FFS: identical or different rotation factors for different SD components

  - Alt2. Identity (i.e. no Doppler-/time-domain compression)

- FFS: Whether Doppler-/time-domain (DD/TD) basis vector length (N4) is RRC-configured or reported by the UE

- FFS: Whether the number of selected DD/TD basis vectors (for Alt1) is RRC-configured or reported by the UE

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the assumption of the UE-side prediction

- On the definition of UE-side prediction, down-select one from the following alternatives:

  - Alt1. UE “predicting” channel/CSI after the slot with a reference resource

  - Alt2. UE “predicting” channel/CSI after slot n (where the CSI is reported)

  - Alt3. UE “predicting” channel/CSI after the slot where CSI-RS resides

Agreement

The Rel-18 TRS-based TDCP reporting comprises stand-alone auxiliary feedback information to enable refinement of CSI reporting configuration, and/or codebook configuration parameters, and/or (to be confirmed in RAN1#110) gNB-side CSI prediction

- Not conditioned on other UCI parameters

- Not reported together with CQI/PMI/RI/(CRI) associated with a codebook

  - Note: This does not prevent TDCP reporting from being multiplexed with other UCI parameters on PUCCH and/or PUSCH

- Note: Aperiodic reporting is supported (per agreed Alt1 in RAN1#109-e)

Agreement

On the Type-II codebook refinement for CJT mTRP, down-select from the following TRP selection/determination schemes (where N is the number of cooperating TRPs assumed in PMI reporting) by RAN1#110bis-e:

- Alt1. N is gNB-configured via higher-layer (RRC) signalling

  - The N configured TRPs are gNB-configured via higher-layer (RRC) signalling

  - Note: only one transmission hypothesis is reported

- Alt2. N is UE-selected and reported as a part of CSI report where N{1,..., NTRP}

  - N is the number of cooperating TRPs, while NTRP is the maximum number of cooperating TRPs configured by gNB

  - In this case, the selection of N out of NTRP TRPs is also reported (FFS: exact reporting scheme)

  - FFS: Configuration of NTRP TRPs and the value of NTRP, whether explicit or implicit

  - Note: only one transmission hypothesis is reported. UE is not mandated to calculate CSI for multiple transmission hypotheses.

FFS: Whether S-TRP transmission hypothesis is also reported

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, for each layer, down-select one from the following alternatives by RAN1#110bis-e:

- Alt1. One group comprises one polarization across all TRPs/TRP-groups (Cgroup,phase=1, Cgroup,amp=2), one (common) SCI across all TRPs/TRP groups

- Alt2. One group comprises one polarization for one TRP/TRP-group (Cgroup,phase=N, Cgroup,amp=2N), per-TRP/TRP-group SCI

  - FFS: Quantization of N strongest coefficients

- Alt3. One group comprises one polarization for one TRP/TRP-group with a common phase reference across TRPs/TRP-groups (Cgroup,phase=1, Cgroup,amp=2N)

  - FFS: SCI, per-TRP/TRP-group vs. one (common) SCI across all TRPs/TRP groups

  - FFS: Quantization of N strongest coefficients

- Alt4. For a selected TRP/TRP-group, one group comprises one polarization, and for remaining N-1 TRPs/TRP-groups, one group comprises one polarization across remaining N-1 TRPs/TRP-groups (Cgroup,amp=2+2=4), with a common phase reference across all of N TRPs/TRP-groups (Cgroup,phase=1)

  - FFS: The selected TRP/TRP-group

FFS: The need for “strongest” TRP/TRP-group indicator in addition to SCI(s)

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, support DD/TD (compression) unit (analogous to PMI sub-band for Rel-16 codebook) as a codebook parameter.

- FFS: whether this parameter is defined as a function of another parameter

- FFS: whether this is used for PMI only or PMI/CQI

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities assuming the UE-side prediction, on the definition of UE-side prediction, down-select one from the following alternatives by RAN1#110bis-e:

- Alt1. UE “predicting” channel/CSI after the slot with a reference resource

- Alt2. UE “predicting” channel/CSI after slot n (where the CSI is reported)

Agreement

For the Rel-18 Type-II codebook for CJT mTRP based on the Rel-16 Type-II codebook, SD basis and FD basis are separate, each fully reusing the legacy Rel-16 DFT-based design

Agreement

The Rel-18 Type-II codebook for CJT mTRP comprises refinement of the following codebooks:

- Refinement of the Rel-16 eType-II regular codebook

- Refinement of the Rel-17 FeType-II port selection (PS) codebook, based on the same design details as the refinement of the Rel-16 eType-II regular codebook, except for the supported set of parameter combinations

Strive to maintain as much commonality between the Rel-16 and Rel-17 codebook enhancements to minimize workload.

Vivo and Lenovo raised concerns on the workload due to this agreement

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, down-select one from the following alternatives by RAN1#110bis-e:

- Alt1.B:  l ≥ nref

  - nref (a CSI reference resource slot) as boundary

- Alt2.B: l ≥ n

  - n (report slot) as boundary

Agreement

For the Rel-18 TRS-based TDCP reporting, down select one of the following alternatives by RAN1#110bis-e:

- AltA. Based on Doppler profile

  - E.g., Doppler spread derived from the 2nd moment of Doppler power spectrum, average Doppler shifts, Doppler shift per resource, maximum Doppler shift, relative Doppler shift, etc

- AltB. Based on time-domain correlation profile

  - E.g. Correlation within one TRS resource, correlation across multiple TRS resources

  - Note: The correlation over one or more lags of TRS resource may be considered.  The lags may be within one TRS burst or different TRS bursts

- AltC: CSI-RS resource and/or CSI reporting setting configuration parameter(s) to assist network

  - E.g. gNB configures UE with multiple choices on what to assist (e.g. two or more CSI-RS/report periodicities, or precoding schemes depending mainly on UE velocity), then UE report according to configuration; parameters correspond to CSI reporting periodicity, codebook type, etc.

Note: Different alternatives may or may not apply to different use cases

Agreement

For the Rel-18 TRS-based TDCP reporting, the use case of “aiding gNB-side CSI prediction” is refined to “aiding gNB implementation in CSI prediction for TDD”

Reference signal enhancement

Working Assumption

To increase the number of DMRS ports for PDSCH/PUSCH, support at least Opt.1 (introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6)).

- FFS: FD-OCC length for Rel.18 DMRS type 1 and type 2.

- FFS: Whether it is needed to handle potential performance issues of Opt 1. For example, study if there is performance loss in case of large delay spread scenario. If needed, how (e.g. additionally support other options).

Agreement

For enhanced FD-OCC length for DMRS of PDSCH/PUSCH, support the following FD-OCC length:

- For Rel.18 DMRS type 1, down select from the following in RAN1#110bis-e:

  - Opt.1-1: Length 6 FD-OCC is applied to 6 REs of DMRS within a PRB within an CDM group

  - Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group

- For Rel.18 DMRS type 2:

  - Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB within an CDM group

  - FFS: Support of length 6 FD-OCC

Agreement

Support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports.

- For MU-MIMO by different CDM groups, no MU-MIMO scheduling restriction of PUSCH/PDSCH (i.e. MU-MIMO between Rel.15 UE and Rel.18 UE is allowed).

- For MU-MIMO within a CDM group, study whether and how to support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH.

  - Note: the study includes MU-MIMO between Rel.15 UE and Rel.18 UE, and between Rel.18 UEs.

- Note: PUSCH above is CP-OFDM waveform.

Agreement

For support of more than 4 layers SU-MIMO PUSCH, study the following potential enhancements for PTRS-DMRS association.

- Whether to support more than 2-port UL PTRS.

- Whether to increase the DCI size of PTRS-DMRS association field in DCI format 0_1/0_2.

Agreement

For increased DMRS ports for enhanced FD-OCC, study whether/how to support DCI based switching between DMRS port(s) associated with length 2 FD-OCC and DMRS port(s) associated with length M FD-OCC (where M > 2).

Agreement

For > 4 layers PUSCH, support rank = 5,6,7,8 for both DMRS type 1/2, and for both single-symbol/double-symbol DMRS.

Agreement

For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

- SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

  - RAN1 should strive to minimize the number of schemes supported in Rel-18

- SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation.

  - Target usage includes antenna switching, codebook/non-codebook based SRS

Agreement

For 8 Tx SRS, at least support

- 8 ports in 1 SRS resource for ‘antennaSwitching’;

  - FFS 8 ports in one or multiple SRS resources for ‘codebook’

Above does not imply support for 8 ports in one or multiple OFDM symbols

Agreement

For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR)

Agreement

For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol.

FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

Agreement

For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols.

- Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

Enhanced uplink transmission

Working assumption

Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18:

- SDM scheme

- In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e.

- FFS: FDM-A scheme

- FFS: FDM-B scheme

- FFS: SFN-based transmission scheme

For schemes other than SDM, final decision to support or not will be made in RAN1#110bis-e.

Agreement

For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2} for single CW case.

- FFS on layer combinations of {1+3} and {3+1} considering the performance gain, system/UE complexity, specification efforts, etc.

- FFS: the option of using layer combination of 0+n and n+0 for dynamic switch between single-panel and STxMP (n=1 or 2, 3 or 4).

- This applies to SDM with 1 CW at least.

Agreement

To enhance the DMRS port indication for SDM scheme of STxMP PUSCH transmission in single-DCI based mTRP system, study the following aspects:

- Whether the indicated DMRS ports can be in different or same CDM group?

- How to determine the port partition among PUSCH layers.

- How to map DMRS ports with PUSCH layers from different panels.

- Whether to use one DCI field or two DCI fields for DMRS port indication

- How to indicate layer combination that is used to partition DMRS port partition among PUSCH layers.

- Other aspects are not precluded.

Agreement

Study and evaluate STxMP PUCCH based on the following:

- For single-DCI based STxMP PUCCH transmissions, companies to provide the detailed description of the scheme being evaluated along with evaluation results in contribution.

- For multi-DCI based STxMP PUCCH transmissions, transmitting two PUCCH resources with independent UCI payload to different TRPs with different UE panels that are fully or partially overlapping in time domain and partially/fully/non-overlapping in frequency domain can be considered.

- Note: Companies can reuse the EVM assumptions of Rel-18 STxMP as agreed in RAN1#109-e (other than the parameters that are specific to PUSCH) as well as Rel-17 EVM for PUCCH as agreed in RAN1#102-e (PUCCH format, # of RBs/symbols, UCI payload, and Frequency hopping as shown below).

  - Baseline scheme can be Rel-15 PUCCH or Rel-17 mTRP PUCCH repetition.

| Parameters | Potential values |
| --- | --- |
| Baseline scheme | Rel-15 PUCCH or Rel-17 mTRP PUCCH repetition |
| PUCCH format | Format 1 and 3. Other PUCCH Formats can be optionally considered. |
| # of RBs/symbols | PUCCH Format 1: 4 symbols, 1 RB PUCCH Format 3: 4 and 8 symbols, 1 RB Other combinations are not precluded. |
| UCI payload | 2 bits for PUCCH Format 1 (and Format 0, if considered). Companies to report assumptions on other PUCCH Formats |
| Frequency hopping | Reported by companies |

Agreement

8TX PUSCH is supported in Rel-18

Agreement

For 8TX PUSCH, at least support

- Ng=1, 2, 4

Note: The above does not restrict the Ng for the non-coherent case

Agreement

For evaluation purpose of codebook alternatives when a precoder based on Rel-15 DL Type I is used, following oversampling ratios are assumed

- (O1, O2) = (1,1), (2,1), (2,2)

- Note: Other values may be used and reported by companies

- Note: When deciding the supported O1, O2 combination, the signalling overhead, performance, UE complexity, etc should be considered

Agreement

RAN1 further studies Alt1b and Alt2a for down-selection of one of the two in RAN1 meeting #110b-e.

- Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

Agreement

Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability

- For uplink transmission with rank<=4, single CW is supported

- For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e

The above applies only with regards to the work scope of this agenda item.

Agreement

For SRS configuration for non-codebook UL transmission for an 8TX UE, down-select from

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- Alt2: Up to two SRS resource sets, each configured with up to 4 single-port SRS resources

- Alt3: Support both alternatives.

Agreement

Study low overhead solutions for SRI and/or transmitter precoder matrix indication for codebook-based, and SRI indication for non-codebook-based UL transmission by an 8TX UE,

- FFS using single or separate (exiting or new) fields for the indication, other solutions are not precluded.

- Note: Low overhead schemes for study include those using Rel-15 SRI/TPMI indication mechanisms

#### 2.1.2	Remaining Open issues

Multi-TRP enhancement

- Remaining specification details on extension of Rel-17 unified TCI framework, including:

  - Indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework

  - UL beam indication for, if specified, PUCCH/PUSCH to facilitate simultaneous multi-panel UL transmission, where unified TCI framework extension is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

- Remaining specification details on power control for UL single DCI for multi-TRP operation where unified TCI framework extension is assumed

- Remaining specification details on Two TAs for UL multi-DCI for multi-TRP operation

CSI enhancement

- Remaining specification details on CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, including:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

- Remaining specification details on Rel-16/17 Type-II codebook refinement for CJT mTRP and its associated CSI reporting, targeting FDD FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs

Reference signal enhancement

- Remaining specification details on DMRS enhancement to increase the number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM.

- Remaining specification details on DMRS enhancements for support of more than 4 layers SU-MIMO PUSCH.

- Remaining study (including evaluations based on agreed EVM), and specification of SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization

- Remaining specification details on SRS enhancements to enable 8 Tx UL operation

Enhanced uplink transmission

- Remaining specification details on UL precoding indication for PUSCH to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable):

  - No new codebook is introduced for multi-panel simultaneous transmission

  - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

- Remaining specification details on UL transmission for 8TX UE, including;

  - SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Consideration of coherence assumption, full/non-full power modes

## 2.2	RAN2

#### 2.2.1	Agreements

Not yet started

#### 2.2.2	Remaining Open issues

Not yet started

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

Not yet started

#### 2.4.2	Remaining Open issues

Not yet started

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template