3GPP TSG RAN meeting #97-e								        	RP-222274

Online Meeting, September 12th-16th, 2022

## Status Report to TSG

Agenda item:			9.5.4.10

| WI / SI Name | New WID on NR Repeaters |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_repeaters-Core |  |  |  |  |
| Unique ID | 900170 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213514 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2022 | Performance part: 12/2022 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 80% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN-4 |
| --- | --- | --- |
| Rapporteur | Name | Valentin Gheorghiu |
|  | Company | Qualcomm Incorporated |
|  | Email | vgheorgh@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

No TUs are needed for the baseband (RD) part so these are removed from the TU spreadsheet

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.2	RAN4

#### 2.2.1	Agreements

RAN4#104-e

The discussion on the repeaters conformance took place in two e-mail threads, the summaries can be found in R4-2214294 and R4-2214295.

Several agreements were reached and are captured in several WFs.

WF on [305] NR Repeater_RF Conformance (Part1) agreements in R4-2214367

## Sub-topic 2-1 Input signal spectral purity

Based on the email discussion summary in [1], the following options were identified for Input signal spectral purity:

- Option 1: E-UTRA repeater stimulus signal spectral purity requirements

- Option 2: check with ETSI ERM/MSG TFES on FR1/FR2 NR based repeater stimulus signal spectral purity requirements

As per Moderator proposal discussed during the GTW, the following WF is agreed:

- No need to check repeater stimulus signal spectral purity requirements with ETSI ERM/MSG TFES,

- For FR1, reuse the repeater stimulus signal spectral purity requirements from E-UTRA repeater specification, as baseline.

- For FR2: the repeater stimulus signal spectral purity requirements are FFS.

## Sub-topic 2-2 Naming of test configurations

Based on the email discussion summary in [1], the following options were identified for naming of test configurations:

- Option 1: NTCx

- Option 2: RTCx

As per discussion captured in [1], the following WF is agreed:

Use RTCx naming convention for NR repeater test configurations.

## Sub-topic 2-3 Carrier allocation

Based on the email discussion summary in [1], the following options were identified for Carrier allocation:

- Option 1: no limit on the number of carriers

- Option 2: a nominal maximum number of carriers

As per discussion captured in [1], the following WF is agreed:

A “nominal maximum number of carriers” agreed and further discuss the carrier definition and the details of number

  - Option 1: two times of number of passbands unless passband too small

  - Other options not precluded

Companies are encouraged to provide more inputs next meeting.

## Sub-topic 2-4 BW

Based on the email discussion summary in [1], the following option was identified:

- Option 1: Propose to use the nominal channel bandwidth for the definition of NR repeater testing signal

As per discussion captured in [1], the following WF is agreed:

Use the nominal channel bandwidth for the definition of NR repeater testing signal.

## Sub-topic 2-5 Clarification on NTC2 carrier allocation

Based on the email discussion summary in [1], the following it was discussed has RAN4 can clarify NTC2 and CACLR TC.

As per discussion captured in [1], the following WF is agreed:

For CACLR TC, carriers shall be allocated on top and bottom of each passband.

## Sub-topic 2-6 RF test channels

Based on the email discussion summary in [1], the following option was identified:

- Option 1: To adopt the RF channels in the following table for NR repeater conformance testing

|  | ## RF channel |  |  |
| --- | --- | --- | --- |
|  | Single carrier | Multi-carrier | Multi-carrier, Multi-band |
| ## Repeater output power | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Frequency stability ## Error Vector Magnitude | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Out of band gain | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Unwanted emissions, ## ACLR, spurious emission | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Input intermodulation | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Output intermodulation | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Adjacent Channel Rejection Ratio (ACRR) | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Transmit ON/OFF power | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Note: since there is no carrier related definition in repeater, carrier should be placed by nominal channel bandwidth of pass band. |  |  |  |

As per discussion captured in [1], the following WF is agreed:

Consider the above table as baseline for RF test channels for single carrier.

Further discussion needed to clarify multi-carrier and multi-band cases.

WF on NR repeater conformance test remaining issues  in R4-2214369

## 2.1 FR1 RF conformance test

### 2.1.1 FR1 declaration open issues

Agreements:

- NC CA no needs to be declared.

- Group delay:

  - Declaration with values for all repeaters

    - It’s not precluded that 0 can be declared if repeater can fulfil transient time requirements

    - Further work on the specification draft

### 2.1.2 Rx spurious emission limits test set up

Agreement:

- Use the following setup and add “Optional” in signal generator box

### 2.1.3 Repeater Tx/Rx spurious emissions MU and TT

Agreements:

- RAN4 consider MU values for BS Rx spurious emissions as baseline for repeater Rx spurious emissions.

- 30 MHz ≤ f ≤ 4 GHz: ±2.0 dB

- 4 GHz < f ≤ 19 GHz: ±4.0 dB

- 19 GHz < f ≤ 26 GHz: ±4.5 dB

- RAN4 adopt 0dB as TT values for Tx and Rx spurious emissions.

### 2.1.4 ALCR BW in MU

Agreement:

- BW should be replaced by the nominal repeater channel bandwidth for ACLR requirements.

### 2.1.5 UL/DL timing for TDD FR1 (and FR2) repeater

Agreement:

- Add generic description into specification: “UL/DL timing can be provided to the repeater”

## FR2 RF conformance test

### FR2 declarations open issues

Agreement:

- NC CA and long group delay follow FR1 agreement

### 2.2.2 TRP measurement with input antenna rotating together with repeater

Agreements:

- Keep the 3GPP description of providing input signal to the repeater generic and allow reasonable uncertainty for it, allowing test engineers to use appropriate solutions in various different test systems.

- Further work on the drafting TP

### 2.2.3 TDD off power measurement

Agreements:

- Take same approach as BS for transmit antenna gain assumption during on/off period.

- In TDD ON/OFF power measurement, input signal generator does not transmit towards repeater during the OFF-power measurement.

### 2.2.4 OTA test procedure

Agreements:

- “Measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off.  Verify measured result is enough below requirement limit”

- Further work on specification drafting

The following TPs for the draft conformance specifications were endorsed:

R4-2214865	TP to TS 38.115-1 clause 4.9 RF channels and test models

R4-2214866	TP to TS 38.115-2 clause 4.9 RF channels and test models

R4-2214785	TP to TS 38.115-1: Test Configurations and Requirement applicability

R4-2214786	TP to TS 38.115-2: Test Configurations and Requirement applicability

R4-2214789	TP to TS 38.115-1: In-band measurements Annex

R4-2214790	TP to TS 38.115-2: In-band measurements Annex

R4-2214740	TP for TS 38.115-1: Clause 3 definitions

R4-2214741	TP for TS 38.115-1: Clause 4.2-4.5

R4-2214742	TP for TS 38.115-2: Clause 3 definitions

R4-2214787	TP to TS 38.115-1: Frequency Stability, Out of band gain, unwanted emissions

R4-2214788	TP to TS 38.115-2: Frequency Stability, Out of band gain, unwanted emissions

R4-2214791	TP to TS 38.115-1: TDD Switching

R4-2214792	TP to TS 38.115-2: TDD Switching

R4-2214801	TP to TS 38.115-1: Manufacturer declarations for NR FR1 repeaters

R4-2214802	TP to TS 38.115-2 Manufacturer declarations for NR FR2 repeaters

R4-2214803	TP to TS 38.115-1 clause 6.7 Input intermodulation - conducted

R4-2214804	TP to TS 38.115-2 clause 6.7 OTA Input intermodulation

R4-2214840	TP for TS 38.115-1: Section 6.9

R4-2214841	TP for TS 38.115-1: Annex D

R4-2214842	TP for TS 38.115-2: section 6.8

R4-2214843	TP for TS 38.115-2: Annex D and E

R4-2214867	TP to TS 38.115-1 clause 6.6 EVM - conducted

R4-2214868	TP to TS 38.115-2 clause 6.6 OTA EVM

R4-2213977	TP to TS 38.115-2 Annex A Repeater stimulus signals (OTA)

All the endorsed TPs were compiled into draft versions of the respective specification and approved as follows:

TS 38.115-1 version 0.1.0 approved in R4-2211698

TS 38.115-2 version 0.1.0 approved in R4-2211699

#### 2.2.2	Remaining Open issues

Finalization of the repeater conformance requirements

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #104-e:

| R4-2211698 | TS 38.115-1 0.1.0 | CATT |
| --- | --- | --- |
| R4-2211699 | TP for TS 38.115-1: Clause 3 definitions | CATT |
| R4-2211700 | TP for TS 38.115-1: Clause 4.2-4.5 | CATT |
| R4-2211701 | TP for TS 38.115-1: Clause 5 operating bands | CATT |
| R4-2211702 | TP for TS 38.115-2: Clause 3 definitions | CATT |
| R4-2211703 | TP for TS 38.115-2: Clause 4.2-4.5 | CATT |
| R4-2211704 | TP for TS 38.115-2: Clause 5 operating bands | CATT |
| R4-2211705 | Discussion of the remaining issues for MU of FR1 and FR2 | CATT |
| R4-2211706 | Discussion of the remaining issues for declarations of FR1 and FR2 | CATT |
| R4-2211707 | Discussion of the remaining issues for FR1 and FR2 set up | CATT |
| R4-2211708 | Discussion of the remaining issues for TDD repeater test | CATT |
| R4-2212006 | Discussion on the requirements related to spurious emissions for FR1 repeater | NTT DOCOMO, INC. |
| R4-2212223 | Discussion on NR Repeater EMC performance assessment | ZTE Corporation |
| R4-2212224 | Draft CR to TS 38.114 Clauses 4.1, 4.2 and 9.1 | ZTE Corporation |
| R4-2212260 | Discussion onNR repeater ACLR requireents | NEC |
| R4-2212261 | CR to 38.106: NR repeater ACLR requirements | NEC |
| R4-2212262 | CR to 38.106: NR repeater ACRR requirements | NEC |
| R4-2212263 | CR to 38.106: NR repeater receiver spurious emissions requirements | NEC |
| R4-2212264 | CR to 38.106: Removal of unlicensed bands for NR repeaters | NEC |
| R4-2212308 | Draft CR for 38.106: delete bracket and add declaration of location requirement for1-C LA repeater | CMCC |
| R4-2212309 | Draft CR for 38.106: add requirements applicability for LA 1-C repeater | CMCC |
| R4-2212310 | Draft CR for 38.106: delete bracket for radiated related requirements | CMCC |
| R4-2212623 | TP to TS 38.115-1: Test Configurations and Requirement applicability | Ericsson |
| R4-2212624 | TP to TS 38.115-2: Test Configurations and Requirement applicability | Ericsson |
| R4-2212625 | TP to TS 38.115-1: Frequency Stability, Out of band gain, unwanted emissions | Ericsson |
| R4-2212626 | TP to TS 38.115-2: Frequency Stability, Out of band gain, unwanted emissions | Ericsson |
| R4-2212627 | TP to TS 38.115-1: In-band measurements Annex | Ericsson |
| R4-2212628 | TP to TS 38.115-2: In-band measurements Annex | Ericsson |
| R4-2212629 | TP to TS 38.115-1: TDD Switching | Ericsson |
| R4-2212630 | TP to TS 38.115-2: TDD Switching | Ericsson |
| R4-2212631 | CR to 38.106: Correction of LA ACLR requirements | Ericsson |
| R4-2212632 | CR to 38.106: Correction of LA ACLR requirements | Ericsson |
| R4-2212837 | TP to TS 38.115-1: Manufacturer declarations for NR FR1 repeaters | Nokia, Nokia Shanghai Bell |
| R4-2212838 | TP to TS 38.115-2 Manufacturer declarations for NR FR2 repeaters | Nokia, Nokia Shanghai Bell |
| R4-2212839 | TP to TS 38.115-1 clause 6.7 Input intermodulation - conducted | Nokia, Nokia Shanghai Bell |
| R4-2212840 | TP to TS 38.115-1 clause 6.8 Output intermodulation - conducted | Nokia, Nokia Shanghai Bell |
| R4-2212841 | On repeater OTA TRP measurement challenges | Nokia, Nokia Shanghai Bell |
| R4-2212842 | TP to TS 38.115-2 clause 6.7 OTA Input intermodulation | Nokia, Nokia Shanghai Bell |
| R4-2213712 | Further discussion on the supported bands for NR repeater | ZTE Corporation |
| R4-2213713 | Draft CR to TS38.106: the introduction of band n104 | ZTE Corporation |
| R4-2213714 | Discussion on NR repeater Stimulus signals | ZTE Corporation |
| R4-2213715 | Discussion on NR repeater test configuratio | ZTE Corporation |
| R4-2213716 | iscussions on NR repeater test cases | ZTE Corporation |
| R4-2213717 | Discussion on FR1 NR repeater test conformance testing | ZTE Corporation |
| R4-2213718 | Discussion on FR2 NR repeater test conformance testing | ZTE Corporation |
| R4-2213719 | TP for TS 38.115-1: section 4.10~4.12 | ZTE Corporation |
| R4-2213720 | TP for TS 38.115-1: Section 6.9 | ZTE Corporation |
| R4-2213721 | TP for TS 38.115-1: Annex D | ZTE Corporation |
| R4-2213722 | TP for TS 38.115-2: section 4.10~4.12 | ZTE Corporation |
| R4-2213723 | TP for TS 38.115-2: section 6.8 | ZTE Corporation |
| R4-2213724 | TP for TS 38.115-2: Annex D and E | ZTE Corporation |
| R4-2213928 | about Repeater OTA conformance testing | Keysight Technologies UK Ltd |
| R4-2213971 | TP to TS 38.115-1 clause 4.9 RF channels and test models | Nokia, Nokia Shanghai Bell |
| R4-2213972 | TP to TS 38.115-2 clause 4.9 RF channels and test models | Nokia, Nokia Shanghai Bell |
| R4-2213973 | TP to TS 38.115-1 clause 6.6 EVM - conducted | Nokia, Nokia Shanghai Bell |
| R4-2213974 | TP to TS 38.115-2 clause 6.6 OTA EVM | Nokia, Nokia Shanghai Bell |
| R4-2213976 | TP to TS 38.115-1 Annex A Repeater stimulus signals | Nokia, Nokia Shanghai Bell |
| R4-2213977 | TP to TS 38.115-2 Annex A Repeater stimulus signals (OTA) | Nokia, Nokia Shanghai Bell |
| R4-2213980 | CR to TS 38.106 with updates and corrections for conductive part | Nokia, Nokia Shanghai Bell |
| R4-2213981 | CR to TS 38.106 with updates and corrections for radiated part | Nokia, Nokia Shanghai Bell |
| R4-2214037 | Draft CR to TS 38.114: exclusion bands, performance assessment, performance criteria (4.4, 5.1, 5.2, 6.1, 6.2) | Huawei, HiSilicon |
| R4-2214038 | Draft CR to TS 38.114: text corrections aligning with the NR repeater core RF specification | Huawei, HiSilicon |
| R4-2214049 | CR to 38.114: Test configurations and radiation (8.1 and 8.2) | Nokia, Nokia Shanghai Bell |
| R4-2214164 | Email Discussion Summary for [104-e][304] NR_Repeater_RFMaintenance | Moderator (CMCC) |
| R4-2214165 | Email Discussion Summary for [104-e][305] NR_Repeater_RFConformance_Part1 | Moderator (Huawei) |
| R4-2214166 | Email Discussion Summary for [104-e][306] NR_Repeater_RFConformance_Part2 | Moderator (CATT) |
| R4-2214293 | Email Discussion Summary for [104-e][304] NR_Repeater_RFMaintenance | Moderator (CMCC) |
| R4-2214294 | Email Discussion Summary for [104-e][305] NR_Repeater_RFConformance_Part1 | Moderator (Huawei) |
| R4-2214295 | Email Discussion Summary for [104-e][306] NR_Repeater_RFConformance_Part2 | Moderator (CATT) |
| R4-2214367 | WF on [305] NR Repeater_RF Conformance (Part1) agreements | Huawei |
| R4-2214368 | TS 38.115-2 0.1.0 | ZTE |
| R4-2214369 | WF on NR repeater conformance test remaining issues | CATT |
| R4-2214529 | CR to 38.106: NR repeater receiver spurious emissions requirements | NEC |
| R4-2214532 | CR to 38.106: Correction of LA ACLR requirements | Ericsson |
| R4-2214555 | CR to TS 38.106 with updates and corrections for conductive part | Nokia, Nokia Shanghai Bell |
| R4-2214740 | TP for TS 38.115-1: Clause 3 definitions | CATT |
| R4-2214741 | TP for TS 38.115-1: Clause 4.2-4.5 | CATT |
| R4-2214742 | TP for TS 38.115-2: Clause 3 definitions | CATT |
| R4-2214759 | Draft CR to TS 38.114 Clauses 4.1, 4.2 and 9.1 | ZTE Corporation |
| R4-2214766 | Draft CR for 38.106: delete bracket and add declaration of location requirement for1-C LA repeater | CMCC |
| R4-2214767 | Draft CR for 38.106: add requirements applicability for LA 1-C repeater | CMCC |
| R4-2214785 | TP to TS 38.115-1: Test Configurations and Requirement applicability | Ericsson |
| R4-2214786 | TP to TS 38.115-2: Test Configurations and Requirement applicability | Ericsson |
| R4-2214787 | TP to TS 38.115-1: Frequency Stability, Out of band gain, unwanted emissions | Ericsson |
| R4-2214788 | TP to TS 38.115-2: Frequency Stability, Out of band gain, unwanted emissions | Ericsson |
| R4-2214789 | TP to TS 38.115-1: In-band measurements Annex | Ericsson |
| R4-2214790 | TP to TS 38.115-2: In-band measurements Annex | Ericsson |
| R4-2214791 | TP to TS 38.115-1: TDD Switching | Ericsson |
| R4-2214792 | TP to TS 38.115-2: TDD Switching | Ericsson |
| R4-2214801 | TP to TS 38.115-1: Manufacturer declarations for NR FR1 repeaters | Nokia, Nokia Shanghai Bell |
| R4-2214802 | TP to TS 38.115-2 Manufacturer declarations for NR FR2 repeaters | Nokia, Nokia Shanghai Bell |
| R4-2214803 | TP to TS 38.115-1 clause 6.7 Input intermodulation - conducted | Nokia, Nokia Shanghai Bell |
| R4-2214804 | TP to TS 38.115-2 clause 6.7 OTA Input intermodulation | Nokia, Nokia Shanghai Bell |
| R4-2214840 | TP for TS 38.115-1: Section 6.9 | ZTE Corporation |
| R4-2214841 | TP for TS 38.115-1: Annex D | ZTE Corporation |
| R4-2214842 | TP for TS 38.115-2: section 6.8 | ZTE Corporation |
| R4-2214843 | TP for TS 38.115-2: Annex D and E | ZTE Corporation |
| R4-2214865 | TP to TS 38.115-1 clause 4.9 RF channels and test models | Nokia, Nokia Shanghai Bell |
| R4-2214866 | TP to TS 38.115-2 clause 4.9 RF channels and test models | Nokia, Nokia Shanghai Bell |
| R4-2214867 | TP to TS 38.115-1 clause 6.6 EVM - conducted | Nokia, Nokia Shanghai Bell |
| R4-2214868 | TP to TS 38.115-2 clause 6.6 OTA EVM | Nokia, Nokia Shanghai Bell |
| R4-2214869 | TP to TS 38.115-2 Annex A Repeater stimulus signals (OTA) | Nokia, Nokia Shanghai Bell |
| R4-2214875 | Draft CR to TS 38.114: exclusion bands, performance assessment, performance criteria (4.4, 5.1, 5.2, 6.1, 6.2) | Huawei, HiSilicon |
| R4-2214876 | Draft CR to TS 38.114: text corrections aligning with the NR repeater core RF specification | Huawei, HiSilicon |
| R4-2214877 | CR to 38.114: Test configurations and radiation (8.1 and 8.2) | Nokia, Nokia Shanghai Bell |
| R4-2215205 | Big CR for TS 38.114 (Rel-17, CAT B) | ZTE |