3GPP TSG RAN meeting #97-e									RP-22xxxx

Electronic Meeting, September 12-16, 2022

## Status Report to TSG

Agenda item:			9.3.4.7

| WI / SI Name | Further enhancements on NR and MR-DC measurement gaps and measurements without gaps |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MG_enh2 |  |  |  |  |
| Unique ID | 950182 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221018 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall: 10% RAN4: 10% | Performance Part: Overall: 0% RAN4: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Ato (Tsang-Wei) YU, Rui HUANG |
|  | Company | MediaTek Inc., Intel Corporation |
|  | Email | ato.yu@mediatek.com, rui.huang@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #104-e (Aug, 2022, Electronic)

- The workplan for the core part is agreed in [1]

- RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG

  - The following agreements were captured in the WF [33]

  - Scope and scenario

    - RAN4 focuses on Case 1 and Case 2 first. RAN4 can continue discuss whether and when to discuss requirements for Pre-configured NCSG

    - Deprioritize 2G/3G measurements in Rel-18 MGE WI

    - Concurrent MUSIM gaps and other existing measurement gap types is not in the scope of this WI

    - Applying MAC signaling for activation/deactivation/reconfiguration for non-positioning measurements via Pre-MG is out of scope of this WI

    - Rel-17 MGE Pre-MG for RRC-based POS is in the scope of this WI

  - Max number of supported gaps

    - For the max number of gaps for Case 1 (Pre-configured MG and multiple concurrent MGs), the Rel-17 conclusions will be taken as the baseline.

    - For the max number of gaps for Case 2 (NCSG and multiple concurrent MGs), the Rel-17 conclusions will be taken as the baseline.

  - Collision definition and handling

    - For Case 1 (Pre-configured MG and multiple concurrent MGs), the baseline requirement considers collisions on Pre-MG is only considered when Pre-MG is activated.

    - When determining if a collision occurs between a NCSG instance and another gap instance, the baseline requirement considers the total NCSG duration, including both the VILs and the ML.

    - On gap collision handling, take priority rule and overlapping rules from Rel 17 concurrent gaps as the baseline

- RRM core requirements for measurements without gaps

  - The following agreements were captured in the WF [33]

  - Inter-RAT measurement without gap

    - Prioritize LTE (or NR) SA case when discussing inter-RAT NR (or LTE) measurements

    - For inter-RAT measurement without MG, including both inter-RAT NR measurement and inter-RAT LTE measurement, same numerology is to be supported

#### 2.4.2	Remaining Open issues

- RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG

  - The following open issues were captured in the WF [33]

  - Scope and scenario

    - Whether to define requirements for one Pre-configured MG and one NCSG

    - FFS whether to deprioritize MR-DC in Rel-18

    - Proponents are encouraged to clarify the proposal “consider legacy MG, Pre-MG, concurrent MG, and NCSG when we discuss joint MG requirements.” in the next meeting.

    - FFS Rel-17 MAC-CE based ePOS gap is in or out of scope of this WI

  - Inherit Rel-17 framework

    - Which Rel-17 Pre-MG principle(s) should be revisited

    - Which Rel-17 concurrent gap principle(s) should be revisited

  - Max number of supported gaps

    - FFS whether to increase the max number for Case 1

    - FFS whether to introduce new UE capability for # of supported Pre-MG for Case 1

    - FFS whether to increase the max number for Case 2

    - FFS whether to introduce new UE capability for # of supported NCSG for Case 2

    - Companies are encouraged to bring high-level principle about the possible gap combinations before going into details

  - Collision definition and handling

    - FFS whether Pre-MG priority can be further decided by the associated Mos being measured or other signalling parameters

    - FFS other potential enhancements regarding spare RF chains, interruption due to collision on RRT, revised proximity condition, … etc.

    - FFS whether to consider gap sharing rule

  - Measurement delay requirements

    - FFS whether the measurement reporting delay requirements with concurrent MGs defined in Rel-17 can be reused except that only activated gaps are considered when defining CSSF

    - FFS whether the pre-configured gap activation delay requirements need to be extended, when the activation procedures of multiple pre-configured gaps overlap.

    - FFS any change for the NCSG measurement reporting delay requirements is needed

    - Proponents are encouraged to clarify the proposal “requirements need to be distinguished if the UE has at least one idle RF chain available or not under NCSG + NCSG case.”

- RRM core requirements for measurements without gaps

  - The following open issues were captured in the WF [33]

  - [NFG] NeedForGapsInfoNR

    - Whether interruption is expected when UE reports ’no-gap’ and the interruption requirement, if allowed

    - Other aspect on whether to allow interruption

    - Requirement for intra-freq measurement without gap

    - Requirement for inter-freq measurement without gap

  - Inter-RAT measurement without gap

    - FFS whether to extend to MR-DC cases

    - FFS mx-numerology case and potential UE capability.

    - Whether to have extension to existing UE capabilities

    - On top of which UE capability to define the inter-RAT NR measurement requirements

    - On top of which UE capability to define the inter-RAT LTE measurement requirements

    - Target scenario for inter-RAT LTE measurement

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

- R4-2213651	work plan for Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps for RRM	MediaTek inc.

- R4-2211742	Initial discussion on RRM requirements for combination of pre-MG, concurrent MGs and NCSG	CATT

- R4-2211907	On R18 pre-configured MGs, multiple concurrent MGs and NCSG	Apple

- R4-2211934	Discussion on combination of pre-configured MGs, multiple concurrent MGs and NCSG	CMCC

- R4-2211965	RRM requirement for the combination of multiple concurrent gaps, pre-MG and NCSG	Xiaomi

- R4-2212059	Initial discussion on RRM requirements for joint operation of MGs	OPPO

- R4-2212131	Discussion on RRM requirements for pre-configured MGs, multiple concurrent MGs and NCSG	Intel Corporation

- R4-2212208	On joint requirements for Rel-17 measurement gap enhancements	Qualcomm Incorporated

- R4-2212766	Discussion on PreMG, ConMG, NCSG	Ericsson

- R4-2213060	Discussion on further enhancements of measurement gaps	Nokia, Nokia Shanghai Bell

- R4-2213061	Discussion on requirements for concurrent measurement gaps, pre-configured gaps and NCSG	Nokia, Nokia Shanghai Bell

- R4-2213287	Discussion on RRM requirements for joint considerations between pre-MG, concurrent MG and NCSG for NR and MR-DC	ZTE Corporation

- R4-2213448	Initial considerations on pre-configured MGs, multiple concurrent MGs and NCSG	vivo

- R4-2213563	Discussion on joint working of eMG features	Huawei, HiSilicon

- R4-2213652	RRM core requirements for pre-configured MGs multiple concurrent MGs and NCSG	MediaTek inc.

- R4-2213870	Discussion on RRM requirements for joint considerations between pre-MG, concurrent MG and NCSG for NR and MR-DC	ZTE Corporation

- R4-2211620	Initial view for RRM core requirements for measurements without gaps	Qualcomm Incorporated

- R4-2211743	Initial discussion on RRM requirements for measurement without gaps	CATT

- R4-2211908	On measurements without gaps	Apple

- R4-2211935	Discussion on measurements without gaps	CMCC

- R4-2211968	Discussion on RRM requirements for measurement without gaps	Xiaomi

- R4-2212060	Initial discussion on RRM requirements for measurements without gaps	OPPO

- R4-2212132	Discussion on RRM requirements for measurement without gaps	Intel Corporation

- R4-2212690	Discussion on further enhancements for gapless measurements	Nokia, Nokia Shanghai Bell

- R4-2212767	Discussion on gapless measurement	Ericsson

- R4-2213288	Discussion on RRM requirements for measurement without gaps for NR and MR-DC	ZTE Corporation

- R4-2213449	Initial considerations on measurements without gaps	vivo

- R4-2213564	Discussion on MG-less measurement in feMG	Huawei, HiSilicon

- R4-2213653	RRM core requirements for measurements without gaps	MediaTek inc.

- R4-2213871	Discussion on RRM requirements for measurement without gaps for NR and MR-DC	ZTE Corporation

- R4-2214155	Email Discussion Summary for [104-e][235] NR_MG_enh2	Moderator (Mediatek)

- R4-2214284	Email Discussion Summary for [104-e][235] NR_MG_enh2	Moderator (Mediatek)

- R4-2214346	WF on further enhancements on measurement gaps and measurements without gaps	MediaTek inc.

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template