3GPP TSG-RAN Meeting #97-e	RP-222344

Electronic Meeting, Sep 12 - 16, 2022

Agenda item:	9.12

Source:	Nokia, Nokia Shanghai Bell

Title:	Operation in the n77 frequency range in US and Canada

Document for:	Discussion

# 1	Introduction

Handling of NS_55 for US and NS_57 for Canada has been discussed for several meetings in RAN4 as well as RAN2 and requirements for non-CA case was already addressed. Although RAN4#104-e addressed CA case, CRs were not agreed. One of the reasons was RAN2 addressed both an n77 specific issue as well as a non-CA/CA NS mapping issue common to all bands and some companies preferred to wait for the RAN2 conclusion before RAN4 agrees relevant CRs.

On the other hand, RAN2#119-e agreed following CRs [1, 2] specific to the n77 CA issue, though the conclusion of the NS mapping issue was postponed. Hence, this contribution provides background of our companion CRs of [3, 4] which  address both US and Canada n77 issues without NC_CA_NS and CA_NS, respectively to complete the issues independently from NS mapping issue based on the agreed RAN2 CRs [1, 2].

# 2	Discussion

RAN2 undrestanding on the issue was summarized in CR coversheet of [1] or [2]. The below is an extract from [1].

----- Start of extract from the CR coversheet of [1] -----

The solution to the extended use of n77 sub-bands in the USA included introduction of:

- New UE capability (extendedBand-n77-r16 indicated by UE that support both the original n77 sub-range and the added (or extended) sub-range.

- New NS value (NS_55), indicated in system information in cells of the extended subrange to prevent idle mode UEs (that do not indicate support of the extendedBand-n77-r16 to camp and access the cell. Note that NS_55 do not imply any additional spectrum emission requirement (ie, same as NS_01).

In RRC, the gNb indicates the cell’s NS value in the field additionalSpectrumEmission. For NR CA, the field description of the field additionalSpectrumEmission requires that the ”Network configures the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured”. This ensures that a UE can unambiguously determine which additional spectrum emission table to apply across all carriers of the band. The UL emission requirements for intra-band cases for each are described by the NS_CA (intra-band contiguous case) and NS_NC_CA (intra-band non-contiguous case) in TS 38.101-1 [15], clause 6.2A.3 and TS 38.101-2 [39], clause 6.2A.3.

Furthermore, gNB is expected to signal the same values in dedicated signalling to UE (ServingCellConfigCommon) as is signalled in SIB1 (ServingCellConfigCommonSIB).

In combination, these two requirements are conflicting on how to configure UL CA in n77 (with at least one cell from original subrange and at least one cell from the extended subrange).

RAN2 agreed that for UL CA in n77. the network may configure either NS_55 or NS_01 for UL carrier(s) in the extended subrange, and NS_01 for the remaining uplink carrier(s) in this band.

----- End of extract from the CR coversheet of [1] -----

----- Start of extract from the main body of the CR of [1] -----

----- End of extract from the main body of the CR of [1] -----

Since the NS mapping issue will be furhter discussed in both RAN2 and RAN4, our companion CRs [3, 4] avoided using CA_NS or CA_NC_NS, respectively.

More specifically, our proposal, e.g., for US case, is to add following texts colored in red under Table 6.2A.3.1.2-1 in 38.101-1.

# 3	Conclusion

Background of our companion CRs of [3, 4] is provided. The CRs addressed both US and Canada n77 issues without NC_CA_NS and CA_NS, respectively to complete the issues independently from NS mapping issue.

# 4	References

[1] R2-2209137, Correction to additionalSpectrumEmission for UL CA in n77 for the US, Ericsson, Nokia, Nokia Shanghai Bell, T-Mobile USA, TSG RAN WG2 #119-e

[2] R2-2209139, Correction to additionalSpectrumEmission for UL CA in n77 for Canada, Ericsson, Nokia, Nokia Shanghai Bell, T-Mobile USA, TSG RAN WG2 #119-e

[3] 	RP-222350, Extension of operation in the n77 frequency range in US, Nokia, Nokia Shanghai Bell, TSG RAN Meeting #97-e

[4] RP-222353, Extension of operation in the n77 frequency range in Canada, Nokia, Nokia Shanghai Bell, TSG RAN Meeting #97-e