3GPP TSG RAN meeting #97e										RP-222388

Electronic Meeting, September 12-16, 2022

Status Report to TSG

Agenda item:			9.3.2.8

| WI / SI Name | Enhancements of NR Multicast and Broadcast Services |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_MBS_enh |  |  |  |  |
| Unique ID | 940099 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221458 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 10% | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Erlin Zeng |
|  | Company | CATT |
|  | Email | erlin.zeng@catt.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

The following agreements were made in R2#119-e.

The work plan in R2-2207770 is noted.

Multicast reception in RRC_INACTIVE

In Rel-18, multicast reception for UEs in INACTIVE supports at least the following scenarios, with the assumption that the UE already has a valid PTM configuration:

-	Scenario 1: a UE has been receiving multicast in CONNECTED, and it enters INACTIVE and continues the multicast reception.

-	Scenario 2: a UE has joined a multicast session and has been directed to INACTIVE, the UE starts to receive the multicast session

FFS for state changes, e.g. due to service being not provided in INACTIVE anymore etc.

It is up to gNB to decide whether a multicast session may be received by UE(s) in INACTIVE. FFS what information gNB may be provided to form such decision (related to SA2 discussion).

It is supported that gNB transmit one multicast session to both UEs in CONNECTED and INACTIVE in the same cell. FFS how the gNB configures this.

It is assumed the network can choose which UEs receive in RRC INACTIVE and which in RRC Connected and can move UEs between the states for Multicast service reception.

The following is taken as baseline: we assume the same PDCCH/PDSCH resources (e.g. resources used for MTCH) can be used for all UEs (including UEs in CONNECTED and/or INACTIVE states) for receiving the same multicast session. Different configuration/resources are not precluded as well. FFS what exactly can be common and what not (e.g. HARQ, SPS etc.) and what is needed in addition (to legacy PTM config).

For PTM configuration delivery, RAN2 further investigates the following solutions:

Option 1: Dedicated signalling

Option 2: Solution based on SIB+MCCH

We do not preclude some “mix” of the options

HARQ feedback and PTP are not supported for multicast reception in RRC_INACTIVE.

Multicast service continuity after cell reselection in RRC_INACTIVE state (i.e. without resuming RRC connection) will be supported (if the configuration of the new cell is available for the UE). FFS whether there are cases where the UE needs to resume the connection. FFS RAN3 impacts due to inter-gNB mobility.

Upon cell reselection to neighbour cells during active multicast session, if the configuration of the session is not available for the new cell for UEs in INACTIVE, then the UE is required to resume RRC connection to get the Multicast MRB configuration.

Shared processing for MBS broadcast and Unicast reception

RAN2 focuses on solutions taking multi-Rx UEs (i.e. no specific enhancements for 1Rx UEs).

2.2.2	Remaining Open issues

- Specify support of multicast reception by UEs in RRC_INACTIVE state

  - PTM configuration for UEs receiving multicast in RRC_INACTIVE state

  - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required)

- Specify Uu signalling enhancements to allow a UE to use shared processing for MBS broadcast and unicast reception, i.e., ‎including UE capability and related assistance information reporting regarding simultaneous unicast reception in RRC_CONNECTED and MBS broadcast reception from the same or different operators

2.3	RAN3

2.3.1	Agreements

About RAN Sharing

Agreements:

- LS to RAN Plenary on the scope of the RAN WID.

- NG-RAN shall be able to identify the MBS session signalling from different operators’ 5GCs aim at the same MBS session. The detail information is pending to SA2.

- The same PTM radio resource can be allocated in a shared cell for transmission of the same MBS service provided by different operators.

- The solution provided by RAN3 work on protocol in RAN sharing scenario should not have impact on Pre Rel-18 UE.

About Support for RRC Inactive state

Agreements:

- It is the common understanding that the following information, among others, may be taken into account by the gNB when deciding to enable UEs receiving multicast in RRC inactive state:

- Capability of UE (of whether support the mode “multicast over RRC inactive”)

- Rel-17 multicast context, e.g. the QoS parameters not associated to any specific UE;

- Parameters available at the local gNB without enhancement on interfaces, e.g. cell load.

2.3.2	Remaining Open issues

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

  - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]

- Study and if necessary, specify enhancements to improve the resource efficiency for MBS reception in RAN sharing scenarios[RAN3]

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R2-2206965	UE capabilities for MBS (S2-2203020; contact: Qualcomm)	SA2	LS in	Rel-18	FS_5MBS_Ph2	To:RAN1	Cc:RAN, RAN2, RAN3

- R2-2206973	Reply LS on UE capabilities for MBS (RP-221861; contact: Qualcomm)	RAN	LS in	Rel-18	FS_5MBS_Ph2	To:SA2	Cc:RAN1, RAN2, RAN3

- R2-2207770	Rel-18 NR MBS enhancement workplan	CATT	Work Plan	Rel-18	NR_MBS_enh-Core

- R2-2206987	Discussion on supporting group scheduling for RRC_INACTIVE UEs	FGI	discussion

- R2-2206988	Multicast reception in RRC_INACTIVE state	TD Tech Ltd	discussion	Rel-18

- R2-2206997	Discussion on multicast reception in RRC_INACTIVE state	OPPO	discussion	Rel-18	NR_MBS_enh

- R2-2207047	Considerations for Multicast Reception in RRC_INACTIVE	Samsung	discussion	Rel-18

- R2-2207191	Discussion on RAN based Notification Area for Multicast Mobility in RRC Inactive State	TCL Communication Ltd.	discussion	Rel-18

- R2-2207204	Overview considerations on Multicast reception in RRC_INACTIVE 	NEC Europe Ltd	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207227	Supporting Multicast Reception in RRC_INACTIVE	vivo	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207318	Discussion on possible approaches to support multicast for inactive UEs	Futurewei	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207412	State transition for UEs receiving Multicast in RRC_INACTIVE state	TCL Communication Ltd.	discussion

- R2-2207415	PTM configuration for UEs receiving Multicast in RRC_INACTIVE state	TCL Communication Ltd.	discussion

- R2-2207447	Multicast reception in RRC_INACTIVE state	Apple	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207481	Considerations on the multicast reception in RRC_INACTIVE	Beijing Xiaomi Software Tech	discussion	Rel-18

- R2-2207557	MBS inactive principles	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207566	Discussion on multicast enhancement for RRC INACTIVE state	MediaTek inc.	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207588	Multicast reception in RRC_INACTIVE	Huawei, HiSilicon	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207689	Discussion on Multicast Reception in RRC_INACTIVE	Spreadtrum Communications	discussion	Rel-18

- R2-2207698	PTM configuration for multicast reception in RRC_INACTIVE	Lenovo	discussion	Rel-18

- R2-2207699	Mobility and state transition for multicast reception in RRC_INACTIVE	Lenovo	discussion	Rel-18

- R2-2207720	Mobility of UEs receiving multicast in RRC_INACTIVE state	CANON Research Centre France	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207730	PTM Configuration in RRC_INACTIVE	SHARP Corporation	discussion	NR_MBS_enh-Core

- R2-2207771	Discussion on multicast reception in RRC_INACTIVE	CATT, CBN	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208093	MBS multicast reception in RRC_INACTIVE	Ericsson	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208096	Multicast reception by UEs in RRC_INACTIVE state	Qualcomm Incorporated	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208289	Multicast reception in RRC INACTIVE 	Kyocera 	discussion	Rel-18

- R2-2208312	Multicast reception in RRC_INACTIVE	LG Electronics Inc.	discussion	Rel-18

- R2-2208374	MBS support in RRC_INACTIVE	InterDigital, Inc.	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208441	Initial consideration on multicast reception in RRC_INACTIVE	CMCC	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208499	Multicast reception in RRC_INACTIVE	Intel Corporation	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208520	Discussion on user plane aspects for support of multicast in RRC_INACTIVE	LG Electronics Inc.	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208633	Multicast reception in RRC_INACTIVE	ZTE, Sanechips	discussion	Rel-18	NR_MBS_enh-Core

- R2-2206989	Simultaneous unicast reception and MBS broadcast reception	TD Tech Ltd	discussion	Rel-18

- R2-2206990	A new MCCH transmission method	Chengdu TD Tech, TD Tech	discussion	Rel-18

- R2-2206991	MBS reception interruption problem in LTE and NR	TD Tech Ltd	discussion	Rel-18	Withdrawn

- R2-2206998	Discussion on support of FTA in NR	OPPO	discussion	Rel-18	NR_MBS_enh

- R2-2207014	MBS reception interruption problem in LTE and NR	Chengdu TD Tech, TD Tech	discussion	Rel-18

- R2-2207184	Discussion on UE shared Processing for Broadcast and Unicast Services Reception	TCL Communication Ltd.	discussion	Rel-18

- R2-2207228	Supporting Shared Processing for MBS Broadcast and Unicast	vivo	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207448	Sharing processing of MBS broadcast and unicast reception	Apple	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207567	Discussion on broadcast coexistence and signaling enhancement	MediaTek inc.	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207589	Discussion on shared processing for MBS broadcast and unicast reception	Huawei, CBN, HiSilicon	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207690	Discussion on shared processing for MBS broadcast and Unicast Reception	Spreadtrum Communications	discussion	Rel-18

- R2-2207772	Discussions on shared processing for MBS broadcast and unicast reception	CATT, CBN	discussion	Rel-18	NR_MBS_enh-Core

- R2-2207808	Discussion on shared processing for MBS broadcast and unicast reception	Xiaomi	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208092	MBS broadcast and unicast reception with shared resources	Ericsson	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208097	Shared processing for MBS broadcast and unicast reception	Qualcomm Incorporated	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208182	Shared processing for MBS broadcast and unicast reception	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208290	Shared processing for simultaneous reception of MBS and unicast 	Kyocera 	discussion	Rel-18

- R2-2208442	Discussion on shared processing for broadcast and unicast reception	CMCC	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208548	Shared processing for simultaneous MBS broadcast and Unicast reception	Intel Corporation	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208591	Uu Signaling Enhancements for MBS	Samsung	discussion	Rel-18	NR_MBS_enh-Core

- R2-2208634	On shared processing for MBS broadcast and Unicast reception	ZTE, Sanechips	discussion	Rel-18	NR_MBS_enh-Core

- R3-224244	Support of MBS in RAN sharing scenarios	Qualcomm Incorporated	Agreement

- R3-224245	Enhancements to support Multicast reception by UEs in RRC_INACTIVE state	Qualcomm Incorporated	Agreement

- R3-224299	MBS RAN sharing scenario	NEC	Decision

- R3-224300	MBS Inactive Reception	NEC	Decision

- R3-224301	CR for RRC_INACTIVE MBS interested indication in 38.473	NEC	Agreement

- R3-224325	Consideration on MBS reception in RAN sharing scenarios	Huawei, CBN	Approval

- R3-224326	Multicast reception by UEs in RRC_INACTIVE state	Huawei, CBN	Approval

- R3-224438	PTM configuration for multicast reception in RRC_INACTIVE	Lenovo	Approval

- R3-224439	Mobility and state transition for multicast reception in RRC_INACTIVE	Lenovo	Approval

- R3-224479	On NR MBS Rel-18 RAN sharing	Ericsson	Agreement

- R3-224529	Principles for RAN support of Multicast in RRC INACTIVE state 	Nokia, Nokia Shanghai Bell

- R3-224531	Principles for mobility and state transition for Multicast in RRC INACTIVE state	Nokia, Nokia Shanghai Bell

- R3-224532	Dual Delivery Mode Support and Backwards Compatibility issues for multicast in RRC Inactive state	Nokia, Nokia Shanghai Bell

- R3-224581	Support of multicast reception in RRC_INACTIVE state - the bigger picture	Ericsson

- R3-224649	Rel-18 NR MBS enhancement work plan	CATT	Discussion

- R3-224650	Discussion on efficient MBS reception in RAN sharing scenario	CATT	Decision

- R3-224651	[Draft]LS on the scope of efficient MBS reception in RAN sharing scenario	CATT	Decision

- R3-224652	Initial thought on RAN3 aspects of multicast over RRC INACTIVE	CATT	Decision

- R3-224664	Comparison of MCCH solution and RRC Dedicated signalling solution	Huawei, CBN	Approval

- R3-224862	Initial discussion on the MBS RAN sharing	Samsung	Agreement

- R3-224914	Discussion on MBS reception in RAN sharing scenarios	CMCC	Discussion

- R3-224915	Multicast Reception in RRC_INACTIVE state	CMCC	Discussion

- R3-224945	Multicast reception in RRC_INACTIVE	ZTE	Decision

- R3-224946	MBS reception in RAN sharing scenarios	ZTE	Decision

- R3-224947	draft LS on MBS reception in RAN sharing scenarios	ZTE	Approval

- R3-225015       Summary of Offline Discussion – CB: # MBS2_Inactive CATT

- R3-225087       Summary of Offline Discussion – CB: # MBS1_NetworkSharin Samsung

- R3-225229       LS on the scope of resource efficiency for MBS reception in RAN sharing scenario  RAN3

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template