3GPP TSG RAN WG1 #110	R1-2208112

# Toulouse, France, August 22th – 26th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0348 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on a reference SCS configuration for co-DurationList in TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Qualcomm), ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 38.213-h20 specifies a reference SCS configuration for co-DurationList by subcarrierSpacing-r16. However, for operation with shared spectrum channel access in FR2-2, the reference SCS configuration for co-DurationList should be provided by subcarrierSpacing-r17 according to 38.331-h10 as shown below. Therefore, considering operation with shared spectrum channel access in FR1 and FR2-2 together, the reference SCS configuration for co-DurationList should be provided by “subcarrierSpacing” instead of “subcarrierSpacing-r16” --------------------38.331-h10---------------------- CO-DurationsPerCell-r16 ::= SEQUENCE { servingCellId-r16 ServCellIndex, positionInDCI-r16 INTEGER(0..maxSFI-DCI-PayloadSize-1), subcarrierSpacing-r16 SubcarrierSpacing, co-DurationList-r16 SEQUENCE (SIZE(1..64)) OF CO-Duration-r16 } CO-DurationsPerCell-r17 ::= SEQUENCE { servingCellId-r17 ServCellIndex, positionInDCI-r17 INTEGER(0..maxSFI-DCI-PayloadSize-1), subcarrierSpacing-r17 SubcarrierSpacing, co-DurationList-r17 SEQUENCE (SIZE(1..64)) OF CO-Duration-r17 |  |  |  |  |  |  |  |  |
| Summary of change: |  | change “subcarrierSpacing-r16” to “subcarrierSpacing” for the reference SCS configuration for co-DurationList |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect parameter name for operation with shared spectrum channel access in FR2-2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 11.1.1	UE procedure for determining slot format

This clause applies for a serving cell that is included in a set of serving cells configured to a UE by slotFormatCombToAddModList and slotFormatCombToReleaseList, availableRB-SetsToAddModList and availableRB-SetsToReleaseList, switchTriggerToAddModList and switchTriggerToReleaseList, or co-DurationsPerCellToAddModList and co-DurationsPerCellToReleaseList.

<Unchanged parts are omitted>

-	a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationsPerCell, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from co-DurationList. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by co-DurationList. If CO-DurationsPerCell is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats

-	a reference SCS configuration for co-DurationList, by subcarrierSpacing

<Unchanged parts are omitted>