3GPP TSG-RAN WG1 Meeting #110	R1-2208125

Toulouse, France, August 22nd – 26th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0340 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on the description about HARQ-feedbackEnablingforSPSactive in 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (ZTE) ,Huawei, HiSilicon, ASUSTek, Xiaomi, OPPO, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The paramter “HARQ-feedbackEnablingforSPSactive” is optional and a boolean value in TS 38.331 which means the parameter can be provided or not provided and indicate enabled or disabled (if provided). However, there are several places specifying UE reports ACK/NACK for the first SPS PDSCH regardless the corresponding HARQ feeback is enabled or disabled subject to condition “if HARQ-feedbackEnablingforSPSactive is provided” which means UE would reports ACK/NACK for the first SPS PDSCH even if HARQ-feedbackEnablingforSPSactive is provided and disabled and if the corresponding HARQ feeback is disabled by DownlinkHARQ-FeedbackDisabled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Further restrict the condition corresponding to “HARQ-feedbackEnablingforSPSactive is provided” to the case that it’s also enabled |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - UE would report ACK/NACK for the first SPS PDSCH even if HARQ-feedbackEnablingforSPSactive is provided and disabled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2, 9.1.3, 9.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 9.1.2	Type-1 HARQ-ACK codebook determination

<Unchanged parts are omitted>

If a UE is provided HARQ-feedbackEnabling-disablingperHARQprocess indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in PDSCH reception occasion m on serving cell c, the UE reports a NACK value for a HARQ-ACK information bit corresponding to the transport block in a Type-1 HARQ-ACK codebook and does not consider the transport block as received in the determination of Nm,creceived in clause 9.1.2.1. If the UE is also provided PDSCH-CodeBlockGroupTransmission, the UE reports NACK values for HARQ-ACK information bits corresponding to CBGs of the transport block in the Type-1 HARQ-ACK codebook and does not consider the CBGs as received in the determination of Nm,creceived,CBG in clause 9.1.2.1. If the UE is also provided HARQ-feedbackEnablingforSPSactive  = 'enabled', the UE considers a HARQ process associated with a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception.

<Unchanged parts are omitted>

### 9.1.3	Type-2 HARQ-ACK codebook determination

<Unchanged parts are omitted>

If a UE is provided HARQ-feedbackEnabling-disablingperHARQprocess indicating disabled HARQ-ACK information for a HARQ process associated with a transport block for PDCCH monitoring occasion m or for SPS PDSCH receptions on serving cell c, the UE does not multiplex a HARQ-ACK information bit corresponding to the transport block in a Type-2 HARQ-ACK codebook and does not consider the transport block as received in the determination of Nm,creceived or of NSPS,c in clause 9.1.3.1. If the UE is also provided PDSCH-CodeBlockGroupTransmission, the UE does not multiplex HARQ-ACK information bits corresponding to CBGs of the transport block in the Type-2 HARQ-ACK codebook and does not consider the CBGs as received in the determination of Nm,creceived,CBG in clause 9.1.3.1. If the UE is also provided HARQ-feedbackEnablingforSPSactive  = 'enabled', the UE considers a HARQ process associated with a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception.

<Unchanged parts are omitted>

### 9.1.4	Type-3 HARQ-ACK codebook determination

<Unchanged parts are omitted>

Set j=0

while c<NcellsDL

while h<NHARQ,cDL

if HARQ-feedbackEnabling-disablingperHARQprocess is not provided, or is provided and indicates enabled HARQ-ACK information for h, or HARQ-feedbackEnablingforSPSactive is provided and enabled and h corresponds to a transport block in a first SPS PDSCH reception after an activation of SPS PDSCH receptions

<Unchanged parts are omitted>