3GPP TSG RAN WG1 Meeting #110	R1-2208276

Toulouse, France, August 22 – 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0101 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on UE Power Saving Enhancements for NR in TS 38.211 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (MediaTek), Huawei, HiSilicon, ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - It is specified in clause 7.3.1.5 of TS 38.211 that PDSCH shall not be mapped to resource elements used for non-zero-power CSI-RS according to clause 7.4.1.5, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, or a PDSCH with SPS. Based on the description of clause 7.4.1.5, non-zero-power (NZP) CSI-RS can be defined for a non-zero-power CSI-RS configured by the TRS-ResourceSet IE. According to the above descripton of TS 38.211, the NZP-CSI-RS configured by TRS-ResourceSet IE shall not be used for PDSCH scheduled for a connected mode UE. This does not align with the agreement made in RAN1 #109-e that “TRS-ResourceSetConfig IE does not affect the UE behaviour in CONNECTED mode and can be ignored by UE in CONNECTED mode”. - The parameter l1 is not needed for the time-domain location determination of TRS configured for RRC idle/inactive state UEs. - The slot determination of of TRS configured for RRC idle/inactive state UEs is not captured. - It should be clarified that theTRS configured for RRC idle/inactive state UEs is only valid in the validity period in clause 10.4B in TS38.213. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that TRS configured TRS-ResourceSet IE does not affect PDSCH mapping scheduled for a UE in CONNECTED mode. - Remove the parameter l1 for TRS configured for RRC idle/inactive state UE. - Capture slot determination of of TRS configured for RRC idle/inactive state UEs. - Clarify that theTRS configured for RRC idle/inactive state UEs is only valid in the validity period. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect behaviour of UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.5, 7.4.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

#### 7.3.1.5	Mapping to virtual resource blocks

The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria:

-	they are in the virtual resource blocks assigned for transmission;

-	the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214];

-	the corresponding resource elements in the corresponding physical resource blocks are

-	not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2;

-	not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource;

-	not used for PT-RS according to clause 7.4.1.2;

-	not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214].

The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l.

#### < Unchanged parts are omitted >

##### 7.4.1.5.3	Mapping to physical resources

#### < Unchanged parts are omitted >

The time-domain locations l0∈0,1, …, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameters firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain2, respectively, in the CSI-RS-ResourceMapping IE or the CSI-RS-ResourceConfigMobility IE and defined relative to the start of a slot. For NZP CSI-RS configured by TRS-ResourceSet IE, the time-domain location l0∈0,1, …, 13 l1∈2, 3, …, 12 is provided by the higher-layer parameter firstOFDMSymbolInTimeDomain or firstOFDMSymbolInTimeDomain+4.

#### < Unchanged parts are omitted >

For a CSI-RS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, configured by the higher-layer parameter CSI-RS-CellMobility or configured by the higher-layer parameter TRS-ResourceSet-r17, the UE shall assume that the CSI-RS is transmitted in slots satisfying

where the periodicity  (in slots) and slot offset  are obtained from the higher-layer parameter CSI-ResourcePeriodicityAndOffset, slotConfig or periodicityAndOffset-r17. The UE shall assume that CSI-RS is transmitted in a candidate slot as described in clause 11.1 of [5, TS 38.213], clause 10.4B of [5, TS 38.213].

#### < Unchanged parts are omitted >