3GPP TSG-RAN WG1 Meeting #110	R1-2208278

# Toulouse, France, August 22nd – 26th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0365 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on UE Power Saving Enhancements for NR in TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (MediaTek), Huawei, HiSilicon, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clarify UE behaviors for the following cases: - During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, - If the DRX group of the serving cell is configured and enters outside Active Time, - If the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer in a PDCCH skipping duration, |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the rescpective UE behaviors in clause 10.4 of TS 38.214 as per the agreements in RAN1#110 meeting: - The UE shall not skip PDCCH monitoring on SpCell - The UE terminates PDCCH skipping for the serving cell - The UE resumes PDCCH monitoring or monitors PDCCH according to search space sets with group index 0 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification and UE behavior may not be aligned with network understanding |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================= Unchanged parts=========================

## 10.4	Search space set group switching and skipping of PDCCH monitoring

========================= Unchanged parts=========================

When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell.

If the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer in a PDCCH skipping duration, the UE

-  resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell, if the UE is not provided searchSpaceGroupIdList-r17 on the new active DL BWP

-  otherwise, monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell.

If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring an active DL BWP of on a serving cell and the timer is running, the UE

-	resets the timer after a slot of the active DL BWP of the serving cell if the UE detects a DCI format in a PDCCH reception in the slot for with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI

-	otherwise, decrements the timer value by one after a slot of the active DL BWP of the serving cell

========================= Unchanged parts =========================