3GPP TSG-RAN Meeting #97-e 																			RP-222568

Electronic Meeting, September 12 – 16, 2022

Agenda item:			9.1.5, 10.1.5

Source:	Moderator (RAN4 Chair)

Title:	Email discussion summary for [97e-08-R18-RAN4-OtherSpectrumRelated]

Document for:	Report

Introduction

In this email thread we will discussion the following topics:

- New basket WI proposal for adding new channel bandwidth for NR bands

- New basket WI proposal for simultaneous Rx/Tx band combinations

- New basket WI proposals for 4Rx

- New WI proposal of MPR requirements for LTE intra-band non-contiguous CA with >35MHz CC gap

- New WI proposal for reference sensitivity degradation mitigation for HPUE on sub-1GHz FDD bands

The following contributions will be covered.

| TDoc | Title | Source | Type | AI |
| --- | --- | --- | --- | --- |
| RP-222012 | Basket WID: adding new channel bandwidth(s) support to existing NR bands | Ericsson | WID new | 9.1.5 |
| RP-222514 | New WID: Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 | Huawei, HiSilicon | WID new | 9.1.5 |
| RP-222451 | Proposed new WID: 4Rx support for NR FR1 bands (<2.6GHz) in Rel-18 | ZTE, Sanechips | WID new | 9.1.5 |
| RP-222452 | Motivation on basket WID on 4Rx support for NR FR1 bands (<2.6GHz) in Rel-18 | ZTE, Sanechips | Discussion | 9.1.5 |
| RP-222307 | MPR requirements for LTE itrabCA with >35 MHz CC gap | Vodafone Ireland Plc | WID new | 10.1.4 |
| RP-222370 | Motivation for MPR requirements for LTE itrabCA with >35 MHz CC gap | Vodafone Ireland Plc | Discussion | 10.1.4 |
| RP-222340 | New WID: Reference sensitivity degradation mitigation for High-Power UEs in NR FDD sub-1GHz bands | MediaTek Inc | WID new | 9.1.5 |
| RP-222320 | New WID: PC1.5 Inter-band and Intra-band uplink CA | T-Mobile USA Inc. | WID new | 9.1.4 |

In this document, we capture comments and conclusions for this email thread.

Topic #1: Basket for adding new CBW for NR bands (RP-222012)

Sub-topic 1-2: Comments and responses on the proposed objectives

- Proposal 1-1: for objectives of basket WI for adding new CBW for NR bands

  - Change the objective for FR2 channel bandwidth to

    - FR2-1: {50 MHz, 100 MHz, 200 MHz, 400 MHz}

  - Change the way of working by adding the rule

    - The new request adding support for channel bandwidth in existing NR band should be submitted on RAN4 reflector before tdoc submission deadline the next RAN4 meeting (1 week before the meeting). When requesting the new channel bandwidth, the proponent should identify the expected impairments including A-MPR and/or MSD and capture them in "Additional information" column in Table 4.1.3-1.

Topic #2: Basket for simultaneous Rx/Tx band combinations (RP-222514)

Sub-topic 2-2: Comments and responses on the proposed objectives

- Proposal 2-1: for objectives of Basket for simultaneous Rx/Tx band combinations

  - Modify objective 1 as follows

    - 1. Identify feasibility for each requested FDD-TDD and TDD-TDD band combinations for CA, SUL, MR-DC supporting simultaneous Rx/Tx capability/operation based on technical analysis, especially for those with large MSD values.

  - Add one note under the objective 1 to clarify that there is no MSD threshold for which have mandatory simultaneous Rx-Tx and the simultaneous Rx-Tx capability will be discussed case by case in the WI

    - Note X: whether the simultaneous Rx-Tx capability could be supported or not depends on the evaluation of MSD for the requested band combinations case by case.

Topic #3: Basket for 4Rx support (RP-222451)

Sub-topic 3-2: Comments and responses on the proposed objectives

- Proposal 3-1: for objectives of Basket for 4Rx support, the following modification is proposed

  - Specify the 4Rx REFSENS requirement for NR FR1 bands

    - Support of 4Rx for the bands < 2.6Ghz added in this WI is optional

    - Note: it is not required for network to distinguish between smartphone and FWA UE types. Define the proper requirements for different UE types.

  - Add n25 and n85 together with contact person, supporting companies and other necessary information.

Topic #4: MPR requirements for LTE intra-band non-contiguous CA with >35MHz CC gap (RP-222307)

Sub-topic 4-2: Comments and responses on the proposed objectives

- Proposal 4-1: approve the revised WID proposal RP-222307 for which the supporting companies need be added.