3GPP TSG RAN meeting #97e										       RP-222655

Electronic Meeting, September 12 – 16, 2022

## Status Report to TSG

Agenda item:			9.5.1.1

| WI / SI Name | Extending current NR operation to 71GHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_ext_to_71GHz |  |  |  |  |
| Unique ID | 860041 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213540 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2022 | Performance part: 12/2022 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 45% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Jing Sun (RAN1); Aida Vera Lopez (RAN4) |
|  | Company | Qualcomm; Intel Corporation |
|  | Email | jingsun@qti.qualcomm.com; aida.l.vera.lopez@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

### 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

RAN1 completed the work. Remaining details will be completed in maintenance phase.

### 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

RAN2 completed the work. Remaining details will be completed in maintenance phase.

### 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

### 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	RF

General aspects and system parameters

LS reply to RAN1 on minimum guard period

- LS reply was approved (R4-2214890)

Answer from RAN4: RAN4 has considered the absolute switching time evaluated for FR2-1 in R15, i.e., the antenna switching time is 15 µsec with a view to reusing this value for FR2-2. The detailed evaluation results for the switching scenarios considered are included in the R15 reply LS R4-1710048 [1]. RAN4 will further consider this result and its feasibility for SRS transmission used for antenna switching and whether the switching time assumed could be improved for 480 and 960 kHz operation.

FR2 unlicensed operation

The following note for band n263 will be included in Table 5.2-1: NR operating bands in FR2 of TS 38.101-2

- Agreement:

  - NOTE 1: This band is for unlicensed operation and subject to regional and/or country specific regulatory requirements

Operating bands for CA

- Agreement:

  - Add NR CA band CA_n263 to Table 5-2A.1-1 in TS 38.101-2 together with a note clarifying that only contiguous CA is applicable for this band.

| NR CA Band | NR Band |
| --- | --- |
| CA_n257 | n257 |
| CA_n258 | n258 |
| CA_n259 | n259 |
| CA_n260 | n260 |
| CA_n261 | n261 |
| CA_n263 (Note) | n263 |
| NOTE: In this release of the specification, only contiguous CA is applicable for this operating band. |  |

NR CA bandwidth class

- Agreement:

  - No new bandwidth classes are specified for n*100 MHz

  - The discussion on the new channel bandwidth classes for FR2-2 is allowed in the maintenance of this WI

Channel spacing for CA

- Agreement:

  - Channel spacing for adjacent NR carriers in FR2-2

Nominal Channel spacing = ceil((BWChannel(1) + BWChannel(2))/100.8 MHz) * 50.4 MHz

  - Channel spacing for CA in FR2-2

Nominal Channel spacing = ceil((BWChannel(1) + BWChannel(2))/100.8 MHz) * 50.4 MHz

Configurations for intra-band contiguous CA

- Agreement:

  - This is related to NR CA bandwidth classes and therefore can also be discussed in maintenance stage

- The following configurations were introduced in endorsed draft CR R4-2214430:

| NR CA configuration / Bandwidth combination set / Fallback group |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configs. | Uplink CA configs. | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | Max aggregated BW (MHz) | BCS | Fallback group |
| CA_n263B | CA_n263A | 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n263C | CA_n263A | 400 | 400 | 400 |  |  |  |  |  | 1200 | 0 | 1 |
| CA_n263V | CA_n263A | 400 | 400 | 400 | 400 |  |  |  |  | 1600 | 0 | 1 |
| CA_n263W | CA_n263A | 400 | 400 | 400 | 400 | 400 |  |  |  | 2000 | 0 | 1 |
| CA_n263G | CA_n263A | 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n263H | CA_n263A | 100 | 100 | 100 |  |  |  |  |  | 300 | 0 | 3 |
| CA_n263I | CA_n263A | 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 | 3 |
| CA_n263J | CA_n263A | 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 | 3 |
| CA_n263K | CA_n263A | 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 | 3 |
| CA_n263L | CA_n263A | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 | 3 |
| CA_n263M | CA_n263A | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 | 3 |
| NOTE 1: Void NOTE 2: For the NR CA configuration with more than two component carries, the bandwidths in a BCS which may introduce combinations more than requested unintentionally should be listed in a row separately. NOTE 3: In this release of the specification, contiguous DL CA configurations within FR2-2 may only contain multiples of the same channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |

FR1+FR2-2 DC/CA band combinations

- Agreement:

  - Specify n48 + n263 within this work item and if n48 + n263 is completed, then the WI is viewed as completed.

    - Band combinations for n48 + n263 were introduced in endorsed draft CR R4-2214431

- Agreement:

  - RAN4 targets completing one example band combination FR2-2 DC/CA with an anchor in FR1 within the maintenance phase of the WI

  - Additional band combinations can be added in a release-independent manner as part of a Release 18 basket WI

  - To specify the example band combination FR2-2 DC/CA with an anchor in FR1, RAN4 could take a corresponding FR2-1 DC/CA combination with anchor in FR1 as a baseline in this meeting, with square brackets around the requirements, and aim to remove the brackets in the maintenance phase

Minimum guard band

- Agreement:

  - Remove square brackets in Table 5.3.3-1

Beam-direction switching time

- For Rel-17, no agreement was made on the FR2-2 specific beam-direction switching time value.

UE Tx agreements

Multi-band relaxation (MBR)

- Agreement for PC3:

  - Remove the brackets on the multi-band relaxation factors (∆MBP,n and ∆MBS,n) of band n263 and confirm both values are 1.0 dB.

MPR

- Agreement - power class 1

Table 6.2.2.1-3 MPRWT for power class 1, BWchannel = 100 MHz in FR2-2

| Modulation |  | MPRWT (dB), BWchannel = 100 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [5.5] | [0.0] | ≤ [3.5] |
|  | QPSK | ≤ [6.5] | [0.0] | ≤ [3.5] |
|  | 16 QAM | ≤ [7.0] | ≤ [2.5] | ≤ [2.5] |
|  | 64 QAM | ≤ [8.0] | ≤ [8.0] | ≤ [8.0] |
| CP-OFDM | QPSK | ≤ [8.0] | ≤ [1.5] | ≤ [3.5] |
|  | 16 QAM | ≤ [8.0] | ≤ [3.5] | ≤ [4.0] |
|  | 64 QAM | ≤ [9.5] | ≤ [9.5] | ≤ [9.5] |

Table 6.2.2.1-4 MPRWT for power class 1, BWchannel >= 400 MHz in FR2-2

| Modulation |  | MPRWT (dB), BWchannel = 400, 800, 1600, 2000 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [6.0] | ≤ [1.0] | ≤ [3.5] |
|  | QPSK | ≤ [6.0] | ≤ [1.0] | ≤ [4.0] |
|  | 16 QAM | ≤ [4.5] | ≤ [3.0] | ≤ [3.0] |
|  | 64 QAM | ≤ [8.0] | ≤ [8.0] | ≤ [8.0] |
| CP-OFDM | QPSK | ≤ [6.0] | ≤ [1.5] | ≤ [3.5] |
|  | 16 QAM | ≤ [6.0] | ≤ [4.0] | ≤ [5.5] |
|  | 64 QAM | ≤ [10.0] | ≤ [10.0] | ≤ [10.0] |

- Agreement - power class 3

Table 6.2.2.3-1a MPRWT for power class 3, BWchannel = 100 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 100 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ [0.5] |
|  | QPSK | 0.0 | ≤ [0.5] |
|  | 16 QAM | ≤ [3.0] | ≤ [3.0] |
|  | 64 QAM | ≤ [8.5] | ≤ [8.5] |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [1.5] |
|  | 16 QAM | ≤ [4.0] | ≤ [4.0] |
|  | 64 QAM | ≤ [10.0] | ≤ [10.0] |

Table 6.2.2.3-2b MPRWT for power class 3, BWchannel = 400 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [1.0] | ≤ 3.0 |
|  | QPSK | ≤ [1.0] | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ [9.5] | ≤ [9.0] |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 10.0 |

Table 6.2.2.3-2c MPRWT for power class 3, BWchannel >= 800 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [1.0] | ≤ 4.0 |
|  | QPSK | ≤ [1.0] | ≤ 4.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ [9.5] | ≤ [9.0] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 6.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 8.0 |
|  | 64 QAM | ≤ 10.5 | ≤ 10.5 |

A-MPR

- Agreement:

  - No A-MPR requirement needed for the EN 303753 emissions mask.

Minimum output power

- Agreement:

  - PC1 Pmin to be 4 dBm. PC2 and PC3 Pmin to be -13 dBm.

TX OFF power

- Agreement:

  - Reuse FR2-1 requirements for minimum output power and OFF power, specifically remove the [] from this table

EVM PTRS processing

- Agreement:

  - The DMRS based channel estimate shall utilize CPE-corrected DMRS symbols

  - The PTRS extraction and correction stage is used as the final refinement of the received signal.

  - For CP-OFDM, all non-DMRS symbols in a slot must be equipped with PTRS, and frequency density of PTRS tones maximized.

  - For DFT-s-OFDM, PTRS is specified with 4 symbols per group, and the groups are configured in a ‘head and tail’ configuration.

  - For DFT-s-OFDM, the number of PTRS groups is maximized so the ratio of PUSCH symbols to PTRS symbols stays at 1 or higher.

EVM compliance power levels

- Agreement (CCBW = 100 MHz Power class 1 and 2)

  - EVM compliance levels for FR2-2 CBW=100MHz in PC2 and PC1 are the same as FR2-1

- Agreement (CCBW ≥ 400 MHz Power class 1 and 2)

  - The numbers in the table of the proposal are agreeable, but further checking is needed.

- Agreement (CCBW = 100 MHz Power class 3)

  - EVM compliance levels for FR2-2 CBW=100MHz in PC3 is the same as FR2-1

- Agreement (CCBW ≥ 400 MHz Power class 3)

|  |  | Level |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| UE EIRP | dBm | [ -16 -13] | [ -13 -11] | [ -10 -8] | [ -7 -5] | [ -6 -4] |
| UE EIRP for UL 16 QAM | dBm | [ -13 -10] | [ -10 -8] | [ -7 -5] | [ -4 -2] | [ -3 -1] |
| UE EIRP for UL 64 QAM | dBm | [ -9 -6] | [ -6 -4] | [ -3 -1] | [ 0 2] | [ 1 3] |
| Operating conditions | Normal Conditions |  |  |  |  |  |
| NOTE 1: PTRS is configured for 16 QAM and 64 QAM |  |  |  |  |  |  |

Carrier leakage

- Agreement (PC1 carrier leakage for n263)

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 13.4 dBm | -25 |
| 0.4 dBm ≤ EIRP ≤ 13.4 dBm | -20 |

- Agreement (PC3 carrier leakage for n263)

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > -1.9 dBm | -25 |
| -14.9 dBm ≤ EIRP ≤ -1.9 dBm | -20 |

In-band emissions

- Agreement for power class 1

| IQ Image | dB | -25 | Output power > 23.4 dBm |
| --- | --- | --- | --- |
|  |  | -20 | Output power ≤ 23.4 dBm |
| Carrier leakage | dBc | -25 | Output power > 13.4 dBm |
|  |  | -20 | 0.4 dBm ≤ Output power ≤ 13.4 dBm |

- Agreement for power class 2

| IQ Image | dB | -25 | Output power > 19 dBm |
| --- | --- | --- | --- |
|  |  | -20 | Output power ≤ 19 dBm |
| Carrier leakage | dBc | -25 | Output power > 9 dBm |
|  |  | -20 | -13 dBm ≤ Output power ≤ 9 dBm |

- Agreement for power class 3

| IQ Image | dB | -25 | Output power > 8.1 dBm |
| --- | --- | --- | --- |
|  |  | -20 | Output power ≤ 8.1 dBm |
| Carrier leakage | dBc | -25 | Output power > -1.9 dBm |
|  |  | -20 | -14.9 dBm ≤ Output power ≤ 1.9 dBm |

Improved ON/ON transient period

- For Rel-17, no agreement was made to have an enhanced FR2-2 specific ON/ON transient period capability. Consequently, the current FR2 ON/ON transient period (5µs) also applies to FR2-2.

Beam correspondence

- Agreement:

  - Specify the requirement of beam correspondence for bit-1. FFS on bit-0 in the next meeting.

Tx CA agreements

Maximum output power for CA

- Agreement:

  - FR2-2 PC1 and PC3 power classes for CA are the same as for FR2-2 single carrier. Note this is the same approach as in FR2-1.

MPR for CA

- Agreement - PC1

Table 6.2A.2.2-2: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ [7.0] | ≤ [5.0] | ≤ [2.0] | ≤ [2.0] |
| QPSK | ≤ [8.0] | ≤ [6.0] | ≤ [3.0] | ≤ [3.0] |
| 16 QAM | ≤ [8.0] | ≤ [6.0] | ≤ [4.0] | ≤ [4.0] |
| 64 QAM | ≤ [10.0] | ≤ [10.0] | ≤ [10.0] | ≤ [10.0] |

- Agreement – PC3

Table 6.2A.2.4-2: Maximum power reduction (MPRWT_C_CA) for UE power class 3 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] |
| QPSK | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] |
| 16 QAM | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] |
| 64 QAM | ≤ [10.0] | ≤ [10.0] | ≤ [10.0] | ≤ [10.0] |

A-MPR for CA

- Agreement:

  - No CA A-MPR needed for the EN 303753 emissions mask.

Carrier leakage in CA

- Agreement for PC1 - n263

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 13.4 dBm | -25 |
| 0.4 dBm ≤ EIRP ≤ 13.4 dBm | -20 |

- Agreement for PC2 - n263

  - For n263 PC2 use the same value as in FR2-1 since the min peak EIRP values are nearly the same

- Agreement for PC3 - n263

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > -1.9 dBm | -25 |
| -14.9 dBm ≤ EIRP ≤ -1.9 dBm | -20 |

In-band emissions in CA

- Agreement for PC1 and PC3:

  - Re-use the FR2-1 CA in-band emissions method for PC1 and PC3 CA with the same output power values we are proposing for FR2-2 single carrier.

SEM for CA

- Agreement:

  - Re-use the FR2-1 CA SEM requirements for FR2-2.

ACLR for CA

- Agreement:

  - Use the FR2-2 single carrier 15 dB ACLR value for CA

  - Common understanding in RAN4 is that OBW requirement is more stringent than ACLR for FR2-2

UE Rx agreements

Uplink configuration for REFSENS

- Agreement:

Table 7.3.2.1-2: Uplink configuration for reference sensitivity

| Operating band | NR Band / Channel bandwidth / NRB / SCS / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz | SCS | Duplex mode |
| n257 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n258 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n260 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n261 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n262 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n263 | N/A | 64 | N/A | 256 | N/A | N/A | N/A | 120 kHz | TDD |
|  | N/A | N/A | N/A | [64] | 120 | [240] | N/A | 480 kHz |  |
|  | N/A | N/A | N/A | [32] | [60] | [120] | 144 | 960 kHz |  |

Adjacent channel selectivity

- Agreement:

  - Use 21 dB for CCBW ≤ 400 MHz and 20 dB for > 400 MHz

In-band blocking

- Agreement:

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |  |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 | 800 | 1600 | 2000 |  |
| PInterferer for band n263 | dBm | N/A | REFSENS + 33.5 dB | N/A | REFSENS + 33.5 dB | REFSENS + 33.5 dB | REFSENS + 33.5 dB | REFSENS + 33.5 dB |  |
| FIoffset | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 | ≤ -1600 & ≥ 1600 NOTE 5 | ≤ -3200 & ≥ 3200 | ≤ -4000 & ≥ 4000 |  |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 | FDL_low + 400 to FDL_high - 400 | FDL_low + 800 to FDL_high - 800 | FDL_low + 1600 to FDL_high - 1600 |  |

Spurious emissions

- Agreement:

  - Note 2 and Note 3 have been added to spurious emission requirements table

Table 7.9-1: General receiver spurious emission requirements

| Frequency range | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm (NOTE 2) | 1 |
| 1GHz f 2nd harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm (NOTE 3) |  |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH as defined in Annex C.3.1. NOTE 2: This maximum level does not apply for Band n263 for which -36 dBm applies. NOTE 3: This maximum level does not apply for Band n263 for which -30 dBm applies. |  |  |  |

Rx CA agreements

EIS relaxation for intra-band contiguous CA

- Agreement:

  - Use the same values as in FR2-1. For 1600-2000 MHz FR2-2 EIS relaxation dB value as [1.5].

Maximum input level for intra-band contiguous CA

- Agreement:

  - Use the FR2-1 max input for intra-band contiguous requirement for FR2-2.

Adjacent channel selectivity for intra-band contiguous CA

- Agreement:

Table 7.5A.1-1: Adjacent channel selectivity for intra-band contiguous CA

| Operating band | Units | Adjacent channel selectivity / CA bandwidth class |
| --- | --- | --- |
|  |  | All CA bandwidth class |
| n263 | dB | 21 for BWChannel_CA ≤ 400 MHz 20 for BWChannel_CA > 400 MHz |

Table 7.5A.1-2: Adjacent channel selectivity test parameters for intra-band contiguous CA, Case 1

| Rx Parameter | Units | All CA bandwidth Classes |
| --- | --- | --- |
| PInterferer for band n263 | dBm | Aggregated power + 19.5 for BWChannel_CA ≤ 400 MHz Aggregated power + 18.5 for BWChannel_CA > 400 MHz |

Table 7.5A.1-3: Adjacent channel selectivity test parameters for intra-band contiguous CA, Case 2

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n263 | dBm | -44.5 for BWChannel_CA ≤ 400 MHz -43.5 for BWChannel_CA > 400 MHz |

In-band blocking - CA

- Agreement:

Table 7.6A.2.1-1: In band blocking minimum requirements for intra-band contiguous CA

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Power in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| Pinterferer for band n257, n258, n261 | dBm | Aggregated power + 21.5 |
| Pinterferer for band n260, n262 | dBm | Aggregated power + 20.5 |
| Pinterferer for band n263 | dBm | Aggregated power + 19.5 |

BS RF agreements

In this meeting, RAN4 completed the core Tx and Rx requirements. Multiple CRs were endorsed finalizing TAE Tx requirements and ICS Rx requirements. Ongoing discussions regard conformance testing requirements.

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2211806 | Draft CR to TS 38.104 on correction of TAE requirements | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2214373 | WF on FR2-2 BS test environments | Ericsson | Approved |
| R4-2214374 | WF on FR2-2 BS conformance testing | Nokia, Nokia Shanghai Bell | Approved |
| R4-2214422 | WF on system parameters for FR2-2 | Intel Corporation | Approved |
| R4-2214430 | Draft CR for TS 38.101-2 on system parameter updates for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2214431 | Draft CR for TS 38.101-3 to add new NR_CADC 2BDL_xBUL combinations containing FR1 + FR2-2 | Charter Comm Inc. | Endorsed |
| R4-2214480 | Draft CR for TS 38.104 on system parameter updates for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2214883 | Draft CR to 38.101-2 on band n263 Tx aspects | Qualcomm Inc., Apple Inc. | Endorsed |
| R4-2214884 | Draft CR to 38.101-2 on band n263 Rx aspects | Qualcomm Inc., Ericsson | Endorsed |
| R4-2214890 | Reply LS for the minimum guard period between two SRS resources for antenna switching | CATT | Approved |
| R4-2215142 | Draft CR for n263 RMC | Huawei, HiSilicon | Endorsed |
| R4-2215202 | Big CR to 38.104 for Rel-17 NR extension up to 71 GHz maintenance | Nokia, Nokia Shanghai Bell | Agreed |
| R4-2215255 | Big CR of TS38.101-2 for FR2-2 UE requirements | Intel Corporation | Agreed |
| R4-2215256 | Big CR of TS38.101-3 to add new NR_CADC 2BDL_xBUL combinations containing FR1 + FR2-2 | Intel Corporation | Agreed |
| R4-2215257 | Big CR of TS38.104 on system parameter updates for FR2-2 | Nokia, Nokia Shanghai Bell | Agreed |

##### 2.4.1.2	Demod

During RAN4 #104e, draft CRs for PRACH and PDSCH requirements were endorsed. Discussions will continue to address remaining performance issues, including:

  - Requirements for 960kHz SCS

  - Requirements for larger CBW (for 480kHz and 960kHz SCS)

  - Requirements for MCS and PRB allocation

  - PN model and ICI impact for PXSCH requirements

  - PDCCH simulations

  - Requirements for SDR

  - CQI requirement configuration

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2212680 | Draft CR 38.104: PRACH requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2212681 | Draft CR 38.141-2: PRACH requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2214388 | WF on PUSCH demodulation requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Approved |
| R4-2214389 | WF on PRACH demodulation requirements for FR2-2 | Samsung | Approved |
| R4-2214390 | WF on FR2-2 UE demodulation requirements | Qualcomm Inc. | Approved |
| R4-2214500 | WF on PUCCH demodulation requirements for FR2-2 | Ericsson | Approved |
| R4-2214655 | WF on general aspects for demodulation requirements for FR2-2 | Huawei | Approved |
| R4-2214751 | Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz | Apple Inc. | Endorsed |
| R4-2214756 | Nokia_DraftCR_38101-4_PDCCH | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2214783 | draft CR on PDSCH requirements for 52.6 - 71 GHz band | Ericsson | Endorsed |
| R4-2214784 | draft CR on SDR requirements for 52.6 - 71 GHz band | Ericsson | Endorsed |
| R4-2214848 | Draft CR Introduction of FR2-2 PDSCH performance requirements in TS 38.101-4 | Huawei, HiSilicon | Endorsed |
| R4-2215123 | Draft CR for TS38.104 FRC tables for FR2-2 PUSCH demodulation requirements | Ericsson | Endorsed |
| R4-2215124 | Draft CR for TS38.141-2 FRC tables for FR2-2 PUSCH demodulation requirements | Ericsson | Endorsed |
| R4-2215125 | Draft CR Introduction of FR2-2 PUCCH performance requirements in TS 38.104 | Huawei, HiSilicon | Endorsed |
| R4-2215126 | Draft CR Introduction of FR2-2 PUSCH performance requirements in TS 38.141-2 | Huawei, HiSilicon | Endorsed |
| R4-2215138 | Draft CR - definition of FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |

##### 2.4.1.3	RRM

RAN4 completed the core part prior to RAN#96.

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2212683 | Draft CR Timing RRM requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2212900 | MRTD and MTTD values and definitions in CA and DC | Ericsson | Merged |
| R4-2212958 | Draft CR on timing requirements of FR2-2 | Huawei, HiSilicon | Merged |
| R4-2213027 | Draft CR on introduction of SSB configurations for FR2-2 | vivo | Endorsed |
| R4-2214077 | Draft CR for RSSI measurements for FR2-2 | Qualcomm Inc. | Merged |
| R4-2214215 | LS on Rx beam sweeping factor for RRM measurements | LG Electronics Inc. | Approved |
| R4-2214478 | WF on NR extension to 71 GHz RRM performance requirements | Qualcomm Inc. | Approved |
| R4-2214479 | Test cases and work split for NR extension to 71 GHz RRM performance requirements | Qualcomm Inc. | Approved |
| R4-2214577 | CR on QCL-ed assumption for RSSI measurement in FR2-2 | LG Electronics Inc. | Agreed |
| R4-2214584 | CR on Intra-frequency measurement requirements with CCA in FR2-2 | Nokia, Nokia Shanghai Bell | Agreed |
| R4-2214588 | CR on TS38.133 RRM requirements in FR2-2 | MediaTek Inc. | Agreed |
| R4-2214612 | CR on introduction of cell Re-selection, inter-frequency and intra-frequency measurements requirements for FR2-2 | vivo | Agreed |
| R4-2214971 | Draft CR Measurement procedure updates for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2214982 | Draft CR on beam sweeping factor for RRM requirements of FR2-2 | Huawei, HiSilicon | Endorsed |
| R4-2214983 | Draft CR on RSSI requirements of FR2-2 | Huawei, HiSilicon | Endorsed |
| R4-2214984 | Draft CR on test cases of HO for FR2-2 | Huawei, HiSilicon | Endorsed |
| R4-2214985 | Draft CR on test cases of timing requirements for FR2-2 | Huawei, HiSilicon | Endorsed |
| R4-2214986 | Draft CR on test cases of BWP switch for FR2-2 | Huawei, HiSilicon | Endorsed |
| R4-2214997 | Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2 | vivo | Endorsed |
| R4-2215046 | draft CR on Test Cases for cell re-selection for extending NR operation to 71GHz | Ericsson | Endorsed |
| R4-2215047 | draft CR on Test Cases for SCell activation to 71GHz | Ericsson | Endorsed |
| R4-2215104 | Draft CR for timing requirements for FR2-2 – MRTD-MTTD | Qualcomm Inc. | Endorsed |
| R4-2215146 | Draft CR on UE timing advance adjustment accuracy for Rel-17 NR extension to 71GHz | Apple Inc. | Endorsed |
| R4-2215224 | Big CR for NR extending to 71GHz RRM (Rel-17) | MCC, Qualcomm Inc. | Agreed |

#### 2.4.2	Remaining Open issues

None for the core part.

Remaining issues for the performance part are captured below:

- BS RF conformance test

  - Feasibility of test methods and test setups

  - MU evaluation for relevant test methods with input relevant for FR2-2

- Demodulation performance

  - Requirements for 960kHz SCS

  - Requirements for larger CBW (for 480kHz and 960kHz SCS)

  - Requirements for MCS and PRB allocation

  - PN model and ICI impact for PXSCH requirements

  - PDCCH simulations

  - Requirements for SDR

  - CQI requirement configuration

### 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

### 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g., a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #104-e

- R4-2211540, Views on channel model for FR2-2, Anritsu Corporation

- R4-2211626, Draft CR to 38.101-2 on band n263 Tx aspects, Qualcomm Inc.

- R4-2211627, Draft CR to 38.101-2 on band n263 Rx aspects, Qualcomm Inc.

- R4-2211628, 60GHz UE TX, Qualcomm Inc.

- R4-2211629, 60GHz UE RX, Qualcomm Inc.

- R4-2211644, Discussion on the BS RX RF requirements for 52.6-71GHz, CATT

- R4-2211645, Discussion on general part of conformance requirements for 52.6-71GHz, CATT

- R4-2211646, Discussion on the BS transmitter conformance testing for 52.6-71GHz, CATT

- R4-2211647, Discussion on the BS receiver conformance testing for 52.6-71GHz, CATT

- R4-2211689, Further discussion on general RRM requirements for extension to 71GHz, CATT

- R4-2211690, Further discussion on general RRM performance requirements for NR extension to 71 GHz, CATT

- R4-2211697, Reply LS for the minimum guard period between two SRS resources for antenna switching, CATT

- R4-2211806, Draft CR to TS 38.104 on correction of TAE requirements, Nokia, Nokia Shanghai Bell

- R4-2211807, Proposals on finalizing BS receiver requirements for extending current NR operation to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2211808, Draft CR to TS 38.104 on finalization of ICS interfering signal RB, Nokia, Nokia Shanghai Bell

- R4-2211809, Proposals on BS transmitter conformance testing for extending current NR operation to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2211810, Proposals on BS receiver conformance testing for extending current NR operation to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2211868, Discussion on PDSCH demod requirements for 52.6 - 71 GHz, Apple Inc.

- R4-2211869, Discussion on PDCCH demod requirements for 52.6 - 71 GHz, Apple Inc.

- R4-2211870, Discussion on CQI reporting requirements for 52.6 - 71 GHz, Apple Inc.

- R4-2211871, Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz, Apple Inc.

- R4-2211873, Remaining issues on system parameters for NR operation in 52.6GHz - 71GHz, Apple Inc.

- R4-2211874, UE Timing Advance adjustment accuracy for NR operation in 52.6GHz - 71GHz, Apple Inc.

- R4-2211875, Draft CR on UE timing advance adjustment accuracy for Rel-17 NR extension to 71GHz, Apple Inc.

- R4-2211876, Test case of UE transmit timing for NR operation in 52.6GHz - 71GHz, Apple Inc.

- R4-2211877, LBT impacts on RRM requirements for NR operation in 52.6GHz - 71GHz, Apple Inc.

- R4-2211878, Rx beam sweeping scaling factor for RRC_IDLE, Apple Inc.

- R4-2211879, Draft LS reply on TCI assumption for RSSI measurement for FR2-2, Apple Inc.

- R4-2211950, Multi-band relaxation for FR2-2 handheld UE, Murata Manufacturing Co Ltd.

- R4-2212105, On General for ext71GHz, Nokia, Nokia Shanghai Bell

- R4-2212106, On PDSCH requirements for ext71GHz, Nokia, Nokia Shanghai Bell

- R4-2212107, Nokia_ext71GHz_demod_results_PDSCH, Nokia, Nokia Shanghai Bell

- R4-2212108, On PDCCH and PBCH requirements for ext71GHz, Nokia, Nokia Shanghai Bell

- R4-2212109, Nokia_ext71GHz_demod_results_PDCCH_PBCH, Nokia, Nokia Shanghai Bell

- R4-2212110, Nokia_DraftCR_38101-4_PDCCH, Nokia, Nokia Shanghai Bell

- R4-2212111, On CSI reporting requirements for ext71GHz, Nokia, Nokia Shanghai Bell

- R4-2212118, UE feature list and general aspects for NR ext. to 71GHz,	Intel Corporation

- R4-2212119, Open issues for UE Tx requirements in FR2-2, Intel Corporation

- R4-2212150, draftCR to 38104 on BS radiated requirements for PUSCH FR2-2, Intel Corporation

- R4-2212151, draftCR to 38141-2 on BS radiated requirements for PUCCH FR2-2, Intel Corporation

- R4-2212214, LS on Rx beam sweeping factor for RRM measurements, LG Electronics Inc.

- R4-2212215, Discussion on TCI assumption for RSSI measurement for FR2-2, LG Electronics Inc.

- R4-2212216, CR on QCL-ed assumption for RSSI measurement in FR2-2, LG Electronics Inc.

- R4-2212225, Discussion on general issue on FR2-2 demodulation, Ericsson

- R4-2212226, Discussion on FR2-2 PUSCH demodulation requirements, Ericsson

- R4-2212227, Discussion on FR2-2 PUCCH demodulation requirements	, Ericsson

- R4-2212228, Discussion on FR2-2 PRACH demodulation requirements	, Ericsson

- R4-2212229, Simulation results for FR2-2 PUSCH demodulation requirements, Ericsson

- R4-2212230, Simulation results for FR2-2 PUCCH demodulation requirements, Ericsson

- R4-2212231, Simulation results for FR2-2 PRACH demodulation requirements, Ericsson

- R4-2212232, Draft CR for TS38.104 FRC tables for FR2-2 PUSCH demodulation requirements, Ericsson

- R4-2212233, Draft CR for TS38.141-2 FRC tables for FR2-2 PUSCH demodulation requirements, Ericsson

- R4-2212278, Draft CR to 38.101-2 on minimum peak EIRP for PC3 band n263, Murata Manufacturing Co Ltd.

- R4-2212372, Discussion on minimum UE EIRP scaling for FR2-2, Apple Inc.

- R4-2212373, CR for TS 38.101-2: Correcting oversight with UE EIRP CBW scaling for FR2-2, Apple Inc.

- R4-2212390, CR on Intra-frequency measurement requirements with CCA in FR2-2, Nokia, Nokia Shanghai Bell

- R4-2212402, Discussion on TCI assumption of RSSI in FR2-2, MediaTek inc.

- R4-2212403, CR on TS38.133 RRM requirements in FR2-2, MediaTek inc.

- R4-2212461, On transmitter test requirement level derivation for FR2-2, Ericsson

- R4-2212462, On receiver test requirement level derivation for FR2-2, Ericsson

- R4-2212463, Draft CR to 38.141-2: Addition of FR2-2 transmitter support in clause 6, Ericsson

- R4-2212464, Draft CR to 38.141-2: Addition of FR2-2 receiver support in clause 7, Ericsson

- R4-2212465, On general aspects relevant for conformance testing up to 71 GHz, Ericsson

- R4-2212570, The remaining issues of the PDSCH requirements in 52.6 – 71 GHz band, Ericsson

- R4-2212571, Simulation results for PDSCH demodulation in 52.6 – 71 GHz band, Ericsson

- R4-2212572, On the PDCCH and PBCH requirements in 52.6 – 71 GHz band, Ericsson

- R4-2212573, Simulation results for PDCCH and PBCH demodulation in FR2-2, Ericsson

- R4-2212574, SDR requirements in 52.6 – 71 GHz band, Ericsson

- R4-2212575, Simulation results for SDR requirements in 52.6 – 71 GHz band, Ericsson

- R4-2212576, CSI reporting requirements in 52.6 – 71 GHz band, Ericsson

- R4-2212577, Simulation results for CSI reporting requirements in FR2-2, Ericsson

- R4-2212578, draft CR on PDSCH requirements for 52.6 - 71 GHz band, Ericsson

- R4-2212579, draft CR on SDR requirements for 52.6 - 71 GHz band, Ericsson

- R4-2212673, Draft CR - definition of FR2-2, Nokia, Nokia Shanghai Bell

- R4-2212674, Discussion on PUSCH demodulation requirements for the extension to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2212675, PUSCH simulation results for the extension to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2212676, Discussion on PUCCH demodulation requirements for the extension to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2212677, PUCCH simulation results for the extension to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2212678, Discussion on PRACH demodulation requirements for the extension to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2212679, PRACH simulation results for demodulation requirements for the extension to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2212680, Draft CR 38.104: PRACH requirements for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2212681, Draft CR 38.141-2: PRACH requirements for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2212682, Draft CR Measurement procedure updates for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2212683, Draft CR Timing RRM requirements for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2212845, Contiguous DL CA system parameters for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2212846, Measurement uncertainty considerations for NR in 52.6GHz – 71GHz, Nokia, Nokia Shanghai Bell

- R4-2212899, MRTD and MTTD values and definitions in CA and DC, Ericsson

- R4-2212900, MRTD and MTTD values and definitions in CA and DC, Ericsson

- R4-2212910, Discussion on RRM performance in FR2-2, Nokia, Nokia Shanghai Bell

- R4-2212911, General Discussion on RRM requirements with CCA in FR2-2, Nokia, Nokia Shanghai Bell

- R4-2212912, General discussion on RRM requirements in FR2-2, Nokia, Nokia Shanghai Bell

- R4-2212956, Discussion on general RRM requirements for NR operation extending to 71GHz, Huawei, HiSilicon

- R4-2212957, Draft CR on beam sweeping factor for RRM requirements of FR2-2, Huawei, HiSilicon

- R4-2212958, Draft CR on timing requirements of FR2-2, Huawei, HiSilicon

- R4-2212959, Discussion on LBT impacts on RRM requirements for FR2-2, Huawei, HiSilicon

- R4-2212960, Draft CR on RSSI requirements of FR2-2, Huawei, HiSilicon

- R4-2212961, Discussion on performance requirements for NR operation extending to 71GHz, Huawei, HiSilicon

- R4-2212962, Draft CR on test cases of HO for FR2-2, Huawei, HiSilicon

- R4-2212963, Draft CR on test cases of timing requirements for FR2-2, Huawei, HiSilicon

- R4-2212964, Draft CR on test cases of BWP switch for FR2-2, Huawei, HiSilicon

- R4-2213023, Remain issues on RRM impacts for extending NR operation to 71GHz, vivo

- R4-2213024, Discussion on LBT impacts on RRM requirements, vivo

- R4-2213025, CR on introduction of cell Re-selection, inter-frequency and intra-frequency measurements requirements for FR2-2, vivo

- R4-2213026, Discussion on performance requirements for 52.6-71GHz, vivo

- R4-2213027, Draft CR on introduction of SSB configurations for FR2-2, vivo

- R4-2213028, Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2, vivo

- R4-2213203, draftCR on the UE RX requirement for band n263, Xiaomi

- R4-2213220, On UE Tx RF aspects for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2213221, On UE Rx RF aspects for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2213231, UE Rx requirements for band n263, Apple Inc.

- R4-2213232, SSB side conditions for band n263, Apple Inc.

- R4-2213347, draft CR on Test Cases for cell re-selection for extending NR operation to 71GHz, Ericsson

- R4-2213348, draft CR on Test Cases for SCell activation to 71GHz, Ericsson

- R4-2213349, General performance requirements for extending NR operation to 71GHz, Ericsson

- R4-2213350, LBT impacts on RRM requirements for extending NR operation to 71GHz, Ericsson

- R4-2213351, Remaining RRM requirements for extending NR operation to 71GHz, Ericsson

- R4-2213366, Draft CR for n263 RF Tx requirements, Huawei, HiSilicon

- R4-2213367, Draft CR for n263 RF Rx requirements, Huawei, HiSilicon

- R4-2213368, Draft CR for n263 RMC, Huawei, HiSilicon

- R4-2213369, On remaining RF requirements on n263, Huawei, HiSilicon

- R4-2213370, Draft reply LS on the minimum guard period between two SRS resources for antenna switching, Huawei, HiSilicon

- R4-2213466, CR on FR2-2 PC3 MPR, LG Electronics Finland

- R4-2213573, Discussion on MPR values for FR2-2 PC3, LG Electronics Finland

- R4-2213669, View on BS demodulation requirement for NR extended to 71GHz, Samsung

- R4-2213702, Further discussion on BS conformance testing for 52.6-71GHz, ZTE Corporation

- R4-2213744, UE Tx requirements for band n263, Apple Inc.

- R4-2213803, Discussion on general issues for FR2-2 demodulation requirements, Huawei, HiSilicon

- R4-2213804, Discussion on spec structure for FR2-2 demodulation and CSI requirements, Huawei, HiSilicon

- R4-2213805, Discussion on FR2-2 PDSCH requirements, Huawei, HiSilicon

- R4-2213806, Simulation results on FR2-2 PDSCH requirements, Huawei, HiSilicon

- R4-2213807, Discussion on FR2-2 PDCCH and PBCH requirements, Huawei, HiSilicon

- R4-2213808, Simulation results on FR2-2 PDCCH and PBCH requirements, Huawei, HiSilicon

- R4-2213809, Discussion on FR2-2 SDR requirements, Huawei, HiSilicon

- R4-2213810, Simulation results on FR2-2 SDR requirements, Huawei, HiSilicon

- R4-2213811, Discussion on FR2-2 CQI requirements, Huawei, HiSilicon

- R4-2213812, Discussion on FR2-2 PUSCH requirements, Huawei, HiSilicon

- R4-2213813, Simulation results on FR2-2 PUSCH requirements, Huawei, HiSilicon

- R4-2213814, Discussion on FR2-2 PUCCH requirements, Huawei, HiSilicon

- R4-2213815, Simulation results on FR2-2 PUCCH requirements, Huawei, HiSilicon

- R4-2213816, Discussion on FR2-2 PRACH requirements, Huawei, HiSilicon

- R4-2213817, Draft CR Introduction of FR2-2 PDSCH performance requirements in TS 38.101-4, Huawei, HiSilicon

- R4-2213818, Draft CR Introduction of FR2-2 PUCCH performance requirements in TS 38.104, Huawei, HiSilicon

- R4-2213819, Draft CR Introduction of FR2-2 PUSCH performance requirements in TS 38.141-2, Huawei, HiSilicon

- R4-2213925, About FR2-2 BS conformance test system, Tx system link budget, Keysight Technologies UK Ltd.

- R4-2213927, about FR2-2 BS conformance test system, Rx test link budget, Keysight Technologies UK Ltd.

- R4-2213967, UE Demodulation Requirements for FR2-2 (up to 71GHz), Qualcomm Inc.

- R4-2214077, Draft CR for RSSI measurements for FR2-2, Qualcomm Inc.

- R4-2214078, Draft CR for timing requirements for FR2-2 – MRTD-MTTD, Qualcomm Inc.

- R4-2214088, Email Discussion Summary for [104-e][110] NR_ext_to_71GHz_Part_1, Moderator (Intel)

- R4-2214089, Email Discussion Summary for [104-e][111] NR_ext_to_71GHz_Part_2, Moderator (Qualcomm)

- R4-2214138, Email Discussion Summary for [104-e][218] NR_ext_to_71GHz_RRM_1, Moderator (Huawei)

- R4-2214139, Email Discussion Summary for [104-e][219] NR_ext_to_71GHz_RRM_2, Moderator (Qualcomm)

- R4-2214168, Email Discussion Summary for [104-e][310] NR_exto71GHz_BSRF, Moderator (Nokia)

- R4-2214184, Email Discussion Summary for [104-e][324] NR_exto71GHz_Demod_Part1, Moderator (Nokia)

- R4-2214185, Email Discussion Summary for [104-e][325] NR_exto71GHz_Demod_Part2, Moderator (Qualcomm)

- R4-2214215, LS on Rx beam sweeping factor for RRM measurements, LG Electronics Inc.

- R4-2214228, Email Discussion Summary for [104-e][110] NR_ext_to_71GHz_Part_1, Moderator (Intel)

- R4-2214229, Email Discussion Summary for [104-e][111] NR_ext_to_71GHz_Part_2, Moderator (Qualcomm)

- R4-2214270, Email Discussion Summary for [104-e][218] NR_ext_to_71GHz_RRM_1, Moderator (Huawei)

- R4-2214271, Email Discussion Summary for [104-e][219] NR_ext_to_71GHz_RRM_2, Moderator (Qualcomm)

- R4-2214299, Email Discussion Summary for [104-e][310] NR_exto71GHz_BSRF, Moderator (Nokia)

- R4-2214313, Email Discussion Summary for [104-e][324] NR_exto71GHz_Demod_Part1, Moderator (Nokia)

- R4-2214314, Email Discussion Summary for [104-e][325] NR_exto71GHz_Demod_Part2, Moderator (Qualcomm)

- R4-2214373, WF on FR2-2 BS test environments, Ericsson

- R4-2214374, WF on FR2-2 BS conformance testing	Nokia, Nokia Shanghai Bell

- R4-2214388, WF on PUSCH demodulation requirements for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2214389, WF on PRACH demodulation requirements for FR2-2, Samsung

- R4-2214390, WF on FR2-2 UE demodulation requirements, Qualcomm Inc.

- R4-2214422, WF on system parameters for FR2-2, Intel Corporation

- R4-2214430, Draft CR for TS 38.101-2 on system parameter updates for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2214431, Draft CR for TS 38.101-3 to add new NR_CADC 2BDL_xBUL combinations containing FR1 + FR2-2, Charter Comm Inc.

- R4-2214476, WF on NR extension to 71 GHz RRM requirements (Part 1), Huawei, HiSilicon

- R4-2214477, LS reply on TCI assumption for RSSI measurement for FR2-2, Apple Inc.

- R4-2214478, WF on NR extension to 71 GHz RRM performance requirements, Qualcomm Inc.

- R4-2214479, Test cases and work split for NR extension to 71 GHz RRM performance requirements, Qualcomm Inc.

- R4-2214480, Draft CR for TS 38.104 on system parameter updates for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2214500, WF on PUCCH demodulation requirements for FR2-2, Ericsson

- R4-2214577, CR on QCL-ed assumption for RSSI measurement in FR2-2, LG Electronics Inc.

- R4-2214584, CR on Intra-frequency measurement requirements with CCA in FR2-2, Nokia, Nokia Shanghai Bell

- R4-2214588, CR on TS38.133 RRM requirements in FR2-2, MediaTek inc.

- R4-2214612, CR on introduction of cell Re-selection, inter-frequency and intra-frequency measurements requirements for FR2-2, vivo

- R4-2214655, WF on general aspects for demodulation requirements for FR2-2, Huawei

- R4-2214751, Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz, Apple Inc.

- R4-2214756, Nokia_DraftCR_38101-4_PDCCH, Nokia, Nokia Shanghai Bell

- R4-2214783, draft CR on PDSCH requirements for 52.6 - 71 GHz band	, Ericsson

- R4-2214784, draft CR on SDR requirements for 52.6 - 71 GHz band, Ericsson

- R4-2214848, Draft CR Introduction of FR2-2 PDSCH performance requirements in TS 38.101-4, Huawei, HiSilicon

- R4-2214883, Draft CR to 38.101-2 on band n263 Tx aspects, Qualcomm Inc., Apple Inc.

- R4-2214884, Draft CR to 38.101-2 on band n263 Rx aspects, Qualcomm Inc., Ericsson

- R4-2214890, Reply LS for the minimum guard period between two SRS resources for antenna switching, CATT

- R4-2214971, Draft CR Measurement procedure updates for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2214982, Draft CR on beam sweeping factor for RRM requirements of FR2-2, Huawei, HiSilicon

- R4-2214983, Draft CR on RSSI requirements of FR2-2, Huawei, HiSilicon

- R4-2214984, Draft CR on test cases of HO for FR2-2, Huawei, HiSilicon

- R4-2214985, Draft CR on test cases of timing requirements for FR2-2, Huawei, HiSilicon

- R4-2214986, Draft CR on test cases of BWP switch for FR2-2, Huawei, HiSilicon

- R4-2214997, Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2, vivo

- R4-2215046, draft CR on Test Cases for cell re-selection for extending NR operation to 71GHz, Ericsson

- R4-2215047, draft CR on Test Cases for SCell activation to 71GHz, Ericsson

- R4-2215104, Draft CR for timing requirements for FR2-2 – MRTD-MTTD, Qualcomm Inc.

- R4-2215123, Draft CR for TS38.104 FRC tables for FR2-2 PUSCH demodulation requirements, Ericsson

- R4-2215124, Draft CR for TS38.141-2 FRC tables for FR2-2 PUSCH demodulation requirements, Ericsson

- R4-2215125, Draft CR Introduction of FR2-2 PUCCH performance requirements in TS 38.104, Huawei, HiSilicon

- R4-2215126, Draft CR Introduction of FR2-2 PUSCH performance requirements in TS 38.141-2, Huawei, HiSilicon

- R4-2215138, Draft CR - definition of FR2-2, Nokia, Nokia Shanghai Bell

- R4-2215142, Draft CR for n263 RMC, Huawei, HiSilicon

- R4-2215146, Draft CR on UE timing advance adjustment accuracy for Rel-17 NR extension to 71GHz, Apple Inc.

- R4-2215159, WF on NR extension to 71 GHz RRM requirements (Part 1), Huawei, HiSilicon

- R4-2215202, Big CR to 38.104 for Rel-17 NR extension up to 71 GHz maintenance (Rel-17, CAT F), Nokia, Nokia Shanghai Bell

- R4-2215224, Big CR for NR extending to 71GHz RRM (Rel-17), MCC, Qualcomm Inc.

- R4-2215255, Big CR of TS38.101-2 for FR2-2 UE requirements, Intel Corporation

- R4-2215256, Big CR of TS38.101-3 to add new NR_CADC 2BDL_xBUL combinations containing FR1 + FR2-2, Intel Corporation

- R4-2215257, Big CR of TS38.104 on system parameter updates for FR2-2, Nokia, Nokia Shanghai Bell

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template