3GPP TSG RAN Meeting #97-e	RP-22xxxx

Electronic Meeting, September 12-16, 2022	(revision of RP-22xxxx)

- Source:	China Unicom

- Title:	New WID on high power for FR1 for FDD single band(s) with power class 2

- Document for:	Approval

Agenda Item:	9.1.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WID on high power for FR1 for FDD single band(s) with power class 2

Acronym: NR_PC2_UE_FDD_R18

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-18

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
|  |  |  | {optional free text} |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

# 3	Justification

Increasing the transmit power of UE has significant benefits on extending uplink coverage area and improving the experience of cell edge users. In Rel-16 several study/work items related to HPUE have been proposed and worked on to standardize the requirements for EN-DC (FDD 23dBm+TDD 23/26dBm) scenarios. In Rel-17, PC2 HPUE for NR CA and SUL is also specified.

However, extension of uplink coverage for lower-frequency NR SA FDD band is indeed very important for operators. The increment of UE transmit power would bring performance gain for both cell average and cell edge cases, which were verified under various power control parameters as one of conclusions in Rel-17 Study on high power UE (power class 2) for one NR FDD band[1].

In the WI on NR_PC2_UE_FDD, RF requirements including Tx power and tolerance, A-MPR and receiver sensitivity degradation requirements were specified for NR FDD band n1 and n3 under PC2 operation. And there are some further requests proposed by operators on PC2 for more NR FDD bands to fulfil the commercial deployment demands.

This basket Work Item is proposed to work on specifying RF requirements for PC2 of NR FDD bands according to operators’ requests.

[1] 3GPP TR38.861 V17.0.0 Study on high power UE (power class 2) for one NR FDD band

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objectives of the core part are as follows:

- Scope of this basket WI includes NR FDD bands

- Specify the band specific RF requirements for all listed power class 2 NR FDD bands including

  - UE maximum output power and Tx power tolerance

  - A-MPR requirements if needed

  - PC2 sensitivity degradation requirements if needed

Note 1: Ensure that the UE RF requirements of power class 2 UEs shall comply with those of power class 3 when the maximum transmit power is limited to 23dBm by gNB configuration.

Note 2: The ability to define 1Tx and 2Tx requirements is subject to data availability.

Note 3: Both 1Tx and 2Tx UE architecture are considered in this WI.

The power class 2 UE for NR FDD bands are defined in the table 1 below:

Table 1: Power class 2 NR FDD bands within FR1

| NR FDD band | contact name, company | Contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- |
| n8 | Basaier Jialade, China Unicom | basejld@chinaunicom.cn | CATT, Huawei, HiSilicon, ZTE, Sanechips, China Telecom | completed for PC3, new for PC2 |
| n5 | Basaier Jialade, China Unicom | basejld@chinaunicom.cn | CATT, SKT, Huawei, HiSilicon, ZTE, Sanechips, China Telecom | completed for PC3, new for PC2 |
| n28 | Basaier Jialade, China Unicom | basejld@chinaunicom.cn | CATT, Huawei, HiSilicon, ZTE, Sanechips | completed for PC3, new for PC2 |
| n26 | Frank Savaglio, Telstra | frank.savaglio@team.telstra.com | Qualcomm, Ericsson, Huawei, Hisilicon | completed for PC3, new for PC2 |
| n25 | Bill Shvodian, T-Mobile USA | bill.shvodian@t-mobile.com | Ericsson, Nokia, Nokia Shanghai Bell | completed for PC3, new for PC2 |
| n66 | Bill Shvodian, T-Mobile USA | bill.shvodian@t-mobile.com | Ericsson, Nokia, Nokia Shanghai Bell | completed for PC3, new for PC2 |
| n71 | Bill Shvodian, T-Mobile USA | bill.shvodian@t-mobile.com | Ericsson, Nokia, Nokia Shanghai Bell | completed for PC3, new for PC2 |
| n85 | Bill Shvodian, T-Mobile USA | bill.shvodian@t-mobile.com | Ericsson, Nokia, Nokia Shanghai Bell | completed for PC3, new for PC2 |
| n2 | Zheng Zhao, Verizon | zheng.zhao@verizonwrieless.com | Ericsson, Samsung, Qualcomm | completed for PC3, new for PC2 |
| n13 | Zheng Zhao, Verizon | zheng.zhao@verizonwrieless.com | Ericsson, Samsung, Qualcomm | completed for PC3, new for PC2 |

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Specify the necessary performance requirements such as release independence in TS 38.307.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 38.xxx | High power for FR1 for FDD single band(s) with power class 2 | TSG#101 | TSG#102 | Basaier Jialade, China Unicom, basejld@chinaunicom.cn |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | Add PC2 FDD requirements to User Equipment (UE) radio transmission and reception | TSG#102 | Core part |
| 38.307 | Specify release independence requirements | TSG#102 | Perf. part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

Basaier Jialade, China Unicom, basejld@chinaunicom.cn

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

RAN4

# 8	Aspects that involve other WGs

None

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

Supporting IM name

China Unicom

CATT

OPPO

Telstra

SKT

Huawei

HiSilicon

ZTE

Sanechips

T-Mobile USA

China Telecom

TURKCELL

BT

CMCC

Verizon