3GPP TSG-RAN Meeting #97-e 																			RP-22xxxx

Electronic Meeting, September 12 – 16, 2022

Agenda item:			9.1.5, 10.1.5

Source:	Moderator (RAN4 Chair)

Title:	Email discussion summary for [97e-38-CompanyCR-RAN4-WCS]

Document for:	Information

Introduction

In this email thread we will discussion the CR to remove [ ] for NS_21 A-MPR for n30.

The following contributions will be covered.

| TDoc | Title | Source | Type | AI |
| --- | --- | --- | --- | --- |
| RP-222490 | CR to 38.101-1 on new NS for Canadian WCS regulation R17 | Apple, Telus | CR | 14 |

In this document, we capture comments and conclusions for this email thread.

Topic #1: Remove square brackets of NS_21 A-MPR requirements

Companies’ contributions summary

The reason for change is provided below.

This CR is to remove the square brackets of NS_21 A-MPR requirements according to the proposal in R4-2215129 approved in RAN4 #104-e meeting.

Initial round

Comments & responses for Rel-16/17 38.101-1 CRs (RP-222490) for NS_21 A-MPR

Sub-topic 1-1: Comments for 38.101-1 CR RP-222490. Can this CR be approved?

------------------------------------------ Changes ------------------------------------------------------------------------

6.2.3.14	A-MPR for NS_21

Table 6.2.3.14-1: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 1.44MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS | Region A1b RBstart ≤ 1.44MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A2 LCRB > 5.4MHz/12/SCS | Region A3b RBend ≥ 7.74MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A3a RBend ≥ 7.74MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 10 | DFT-s-OFDM | PI/2 BPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | QPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | 16 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 64 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 256 QAM | 6 | 3 | 4 | 3 | 6 |
|  | CP-OFDM | QPSK | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 16 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 64 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 256 QAM | 6 | 4 | 5.5 | 4 | 6 |

Table 6.2.3.14-2: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 0.36MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS | Region A1b RBstart ≤ 0.36MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A2 LCRB > 2.52MHz/12/SCS | Region A3b RBend ≥ 3.96MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A3a RBend ≥ 3.96MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 5 | DFT-s-OFDM | PI/2 BPSK | ≤ 4.0 | ≤ 2.0 | ≤ 1.5 | [2.0 | [4.0 |
|  |  | QPSK | ≤ 4.5 | ≤ 2.5 | ≤ 2.0 | ≤ 2.5 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |
|  | CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |

------------------------------------------ Changes ------------------------------------------------------------------------

Companies are invited to provide the comments in the follow table.

| Company | Comments |
| --- | --- |

Summary

Moderator summarizes discussion status for initial round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

Intermediate round

Comments & responses

In this round, the following issues need be further discussed and addressed.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| XXX |  |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

Summary of Recommendations