3GPP TSG-RAN Meeting #97-e 																			RP-22xxxx

Electronic Meeting, September 12 – 16, 2022

Agenda item:			9.1.5, 10.1.5

Source:	Moderator (RAN4 Chair)

Title:	Email discussion summary for [97e-38-CompanyCR-RAN4-WCS]

Document for:	Information

Introduction

In this email thread we will discussion the CR to remove [ ] for NS_21 A-MPR for n30.

The following contributions will be covered.

| TDoc | Title | Source | Type | AI |
| --- | --- | --- | --- | --- |
| RP-222490 | CR to 38.101-1 on new NS for Canadian WCS regulation R17 | Apple, Telus | CR | 14 |

In this document, we capture comments and conclusions for this email thread.

Topic #1: Remove square brackets of NS_21 A-MPR requirements

Companies’ contributions summary

The reason for change is provided below.

This CR is to remove the square brackets of NS_21 A-MPR requirements according to the proposal in R4-2215129 approved in RAN4 #104-e meeting.

Initial round

Comments & responses for Rel-16/17 38.101-1 CRs (RP-222490) for NS_21 A-MPR

Sub-topic 1-1: Comments for 38.101-1 CR RP-222490. Can this CR be approved?

------------------------------------------ Changes ------------------------------------------------------------------------

6.2.3.14	A-MPR for NS_21

Table 6.2.3.14-1: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 1.44MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS | Region A1b RBstart ≤ 1.44MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A2 LCRB > 5.4MHz/12/SCS | Region A3b RBend ≥ 7.74MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A3a RBend ≥ 7.74MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 10 | DFT-s-OFDM | PI/2 BPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | QPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | 16 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 64 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 256 QAM | 6 | 3 | 4 | 3 | 6 |
|  | CP-OFDM | QPSK | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 16 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 64 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 256 QAM | 6 | 4 | 5.5 | 4 | 6 |

Table 6.2.3.14-2: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 0.36MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS | Region A1b RBstart ≤ 0.36MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A2 LCRB > 2.52MHz/12/SCS | Region A3b RBend ≥ 3.96MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A3a RBend ≥ 3.96MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 5 | DFT-s-OFDM | PI/2 BPSK | ≤ 4.0 | ≤ 2.0 | ≤ 1.5 | [2.0 | [4.0 |
|  |  | QPSK | ≤ 4.5 | ≤ 2.5 | ≤ 2.0 | ≤ 2.5 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |
|  | CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |

------------------------------------------ Changes ------------------------------------------------------------------------

Companies are invited to provide the comments in the follow table.

| Company | Comments |
| --- | --- |
| MediaTek | We are okay with the CR. |
| Qualcomm | Why wasn’t this CR presented for agreement in RAN4? We suggest presenting this to the upcoming RAN4 meeting next month. |
| ZTE | Fine with the CR to remove the square brackets in these two tables as long as the values are confirmed. |
| Samsung | We would like to confirm that removing [ ] on the A-MPR values is fine for us. |
| Huawei | We are fine with the proposed changes. |
| Ericsson | We also have similar view as QC that why this was not presented in RAN4? The values are in brackets because companies need to check if the values are OK or not. So our preference is to present this in the next RAN4 meeting instead of approving in RAN. |
| Apple | The CR with square backets was already approved in RAN #96-e meeting (RP-221868). In last RAN4 meeting, the proposal in R4-2215129 (a discussion paper) to remove square brackets was approved, while there was no associated CR. As there is urgent need for a Canadian Operator to make these A-MPR requirements officially standardized, we propose to approve the CR in this meeting. We think one meeting cycle time should already be sufficient for companies to check the A-MPR values. |

Summary

7 companies made comments. 2 companies preferred to further discuss and check in upcoming RAN4 meeting. 5 companies are OK with the change.

The proposal R4-2215129 was approved but no CR was proposed in last RAN4 meeting. The moderator wonder if companies still have concern to remove [ ]. And it is suggested to submit CRs during RAN4 meeting if it is urgent.

The moderator would like to give one more round for companies to check their views on the CR. If there was still different view, maybe we could agree the CR in October RAN4 meeting.

Intermediate round

Comments & responses

With clarification from proponents in the initial round, can we approve the CR RP-222490?

| Company | Comments |
| --- | --- |
| XXX |  |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

Summary of Recommendations