3GPP TSG-RAN4 Meeting #105	R4-2218028

# Toulouse, France, 14th  – 18th  November 2022,

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0755 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction CR on new NR DC LTE(xDL/1UL)+ NR(2DL/1UL) band combinations in Rel-18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R18_xBLTE_2BNR_yDL2UL-Core |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Completed NR DC on LTE x bands (xDL/1UL, x=1,2,3,4) and NR 2 bands (2DL/1UL) band combinations in RAN4 #104-e, RAN4#104BIS-e and RAN4#105 are added in TS 38.101-3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is implemented with the current version of TS 38.101-3 and all changes have been added in big CR. Reflect approved draft CR in R4-2211830, R4-2212720 and R4-2212721 in RAN4#104e Reflected approved draft CR in R4-2215903,R4-2215904, R4-2215905 and R4-2215906 in RAN4#104BIS-e . Reflected approved draft CR in R4-2218383, R4-2218384, R4-2219198, R4-2219470, R4-2220604, R4-2220605, R4-2220765, R4-2220766 and R4-2220781 in RAN4#105. Reflected TPs with following band combinations. LTE(1DL/1UL) + NR (2DL/1UL) EN-DC combinations within FR1 DL; UL. Reflected band combinations in RAN4#104e. DC_38A_n28A-n78A; DC_38A_n28A DC_38A_n78A. DC_(n)3AA-n8A; DC_(n)3AA1 DC_3A_n8A. DC_(n)3AA-n77A; DC_(n)3AA1 DC_3A_n77A. DC_(n)3AA-n77(2A); DC_(n)3AA1 DC_3A_n77A. Reflected band combinations in RAN4#104BIS-e. DC_3A-n26A-n78A; DC_3A_n26A DC_3A_n78A. DC_3C_n26A-n78A; DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A. DC_7A-n26A-n78A; DC_7A_n26A DC_7A_n78A. DC_7C_n26A-n78A; DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A. DC_20A_n1A-n75A; DC_20A_n1A. DC_7A_n1A-n75A; DC_7A_n1A. DC_1A-n26A-n78A; DC_1A_n26A DC_1A_n78A. DC_3A_n1A-n75A; DC_3A_n1A. DC_3C_n1A-n75A; DC_3C_n1A. Reflected band combinations in RAN4#105. DC_3A_n38A-n40A; DC_3A_n38A DC_3A_n40A. DC_41A_n1A-n78A; DC_41A_n1A DC_41A_n78A. DC_41C_n1A-n78A; DC_41A_n1A DC_41A_n78A. DC_8A_n38A-n40A; DC_8A_n38A DC_8A_n40A. LTE(2DL/1UL) + NR (2DL/1UL) EN-DC combinations within FR1 DL; UL. Reflected band combinations in RAN4#104e. DC_1A-38A_n28A-n78A; DC_1A_n28A DC_1A_n78A DC_38A_n28A DC_38A_n78A. DC_3A-38A_n28A-n78A; DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A. DC_3C-38A_n28A-n78A; DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A. Reflected band combinations in RAN4#104BIS-e. DC_1A-3A_n26A-n78A; DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A. DC_1A-3C_n26A-n78A; DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A. DC_1A-7A_n26A-n78A; DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7A_n78A. DC_1A-7C_n26A-n78A; DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A. DC_3A-7A_n26A-n78A; DC_3A_n78A DC_7A_n78A DC_3A_n26A DC_7A_n26A. DC_3A-7C_n26A-n78A; DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_7A_n26A DC_7C_n26A. DC_3C-7A_n26A-n78A; DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A. DC_3C-7C_n26A-n78A; DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A. DC_1A-38A_n7A-n78A; DC_1A_n78A. DC_7A-20A_n1A-n75A; DC_3A_n1A DC_7A_n1A. DC_3A-20A_n1A-n75A; DC_3A_n1A DC_20A_n1A. DC_3C-20A_n1A-n75A; DC_3A_n1A DC_3C_n1A DC_20A_n1A. DC_3A-7A_n1A-n75A; DC_3A_n1A DC_7A_n1A. DC_3C-7A_n1A-n75A; DC_3C_n1A DC_3A_n1A DC_7A_n1A. Reflected band combinations in RAN4#105. DC_1A-7A_n75A-n78A; DC_1A_n78A DC_7A_n78A. DC_3A-41A_n1A-n78A DC_3A-3A-41A_n1A-n78A; DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A. DC_3A-41C_n1A-n78A DC_3A-3A-41C_n1A-n78A; DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A. DC_8A-41A_n1A-n78A; DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A. DC_8A-41A_n1A-n78A; DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A. DC_20A-41A_n1A-n78A; DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A. DC_20A-41C_n1A-n78A; DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A. DC_3A-7A_n75A-n78A; DC_3A_n78A DC_7A_n78A. DC_1A-3A_n75A-n78A; DC_1A_n78A DC_3A_n78A. LTE(3DL/1UL) + NR (2DL/1UL) EN-DC combinations within FR1 DL; UL. Reflected band combinations in RAN4#104e. DC_1A-3A-38A_n28A-n78A; DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A. DC_1A-3C-38A_n28A-n78A; DC_1A_n28A DC_1A_n78A DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A. Reflected band combinations in RAN4#104BIS-e. DC_1A-3A-7A_n26A-n78A; DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7A_n78A. DC_1A-3C-7A_n26A-n78A; DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7A_n78A. DC_1A-3A-7C_n26A-n78A; DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A. DC_1A-3C-7C_n26A-n78A; DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A. Reflected band combinations in RAN4#105. DC_3A-8A-41A_n1A-n78A DC_3A-3A-8A-41A_n1A-n78A; DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A. DC_3A-8A-41C_n1A-n78A DC_3A-3A-8A-41C_n1A-n78A; DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A. DC_3A-20A-41A_n1A-n78A DC_3A-3A-20A-41A_n1A-n78A; DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A. DC_3A-20A-41C_n1A-n78A DC_3A-3A-20A-41C_n1A-n78A; DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A. DC_3A-7A-20A_n1A-n75A; DC_3A_n1A DC_7A_n1A DC_20A_n1A. DC_1A-3A-7A_n75A-n78A; DC_1A_n78A DC_3A_n78A DC_7A_n78A. LTE(1DL/1UL) + NR (2DL/1UL) EN-DC combinations within FR1 and FR2 DL; UL. Reflected band combinations in RAN4#104e. DC_3A_n34A-n258A; DC_3A_n34A DC_3A_n258A. DC_(n)3AA-n257A DC_(n)3AA-n257G DC_(n)3AA-n257H DC_(n)3AA-n257I DC_(n)3AA-n257J DC_(n)3AA-n257K DC_(n)3AA-n257L DC_(n)3AA-n257M; DC_(n)3AA1 DC_3A_n257A. DC_3A_n41A-n258A; DC_3A_n41A DC_3A_n258A. Reflected band combinations in RAN4#104BIS-e. DC_1A_n8A-n257A DC_1A_n8A-n257G DC_1A_n8A-n257H DC_1A_n8A-n257I DC_1A_n8A-n257J DC_1A_n8A-n257K DC_1A_n8A-n257L DC_1A_n8A-n257M; DC_1A_n8A DC_1A_n257A. DC_1A_n77A-n257J DC_1A_n77A-n257K DC_1A_n77A-n257L DC_1A_n77A-n257M DC_1A_n77A-n257A; DC_1A_n77A DC_1A_n257A. DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_3A_n77A-n257J DC_3A_n77A-n257K DC_3A_n77A-n257L DC_3A_n77A-n257M DC_3A_n77(2A)-n257A DC_3A_n77(2A)-n257G DC_3A_n77(2A)-n257H DC_3A_n77(2A)-n257I DC_3A_n77(2A)-n257J DC_3A_n77(2A)-n257K DC_3A_n77(2A)-n257L DC_3A_n77(2A)-n257M; DC_3A_n77A DC_3A_n257A. DC_8A_n3A-n257A DC_8A_n3A-n257G DC_8A_n3A-n257H DC_8A_n3A-n257I DC_8A_n3A-n257J DC_8A_n3A-n257K DC_8A_n3A-n257L DC_8A_n3A-n257M; DC_8A_n3A DC_8A_n257A. LTE(2DL/1UL) + NR (2DL/1UL) EN-DC combinations within FR1 and FR2 DL; UL. Reflected band combinations in RAN4#104e. DC_1A_(n)3AA-n257A DC_1A _(n)3AA-n257G DC_1A _(n)3AA-n257H DC_1A _(n)3AA-n257I DC_1A _(n)3AA-n257J DC_1A _(n)3AA-n257K DC_1A _(n)3AA-n257L DC_1A _(n)3AA-n257M; DC_1A_n3A DC_1A_n257A DC_(n)3AA DC_3A_n257A. DC_3A-8A_n78A-n257A2 DC_3A-8A_n78A-n257D2 DC_3A-8A_n78A-n257E2 DC_3A-8A_n78A-n257F2 DC_3A-8A_n78A-n257G2 DC_3A-8A_n78A-n257H2 DC_3A-8A_n78A-n257I2 DC_3A-8A_n78A-n257J2 DC_3A-8A_n78A-n257K2 DC_3A-8A_n78A-n257L2 DC_3A-8A_n78A-n257M2 DC_3C-8A_n78A-n257A DC_3C-8A_n78A-n257D DC_3C-8A_n78A-n257G DC_3C-8A_n78A-n257E DC_3C-8A_n78A-n257F DC_3C-8A_n78A-n257H DC_3C-8A_n78A-n257I DC_3C-8A_n78A-n257J DC_3C-8A_n78A-n257K DC_3C-8A_n78A-n257L DC_3C-8A_n78A-n257M; DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K. DC_7A-8A_n78A-n257A2 DC_7A-8A_n78A-n257D2 DC_7A-8A_n78A-n257E2 DC_7A-8A_n78A-n257F2 DC_7A-8A_n78A-n257G2 DC_7A-8A_n78A-n257H2 DC_7A-8A_n78A-n257I2 DC_7A-8A_n78A-n257J2 DC_7A-8A_n78A-n257K2 DC_7A-8A_n78A-n257L2 DC_7A-8A_n78A-n257M2; DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K. DC_7A-7A-8A_n78A-n257A2 DC_7A-7A-8A_n78A-n257D2 DC_7A-7A-8A_n78A-n257E2 DC_7A-7A-8A_n78A-n257F2 DC_7A-7A-8A_n78A-n257G2 DC_7A-7A-8A_n78A-n257H2 DC_7A-7A-8A_n78A-n257I2 DC_7A-7A-8A_n78A-n257J2 DC_7A-7A-8A_n78A-n257K2 DC_7A-7A-8A_n78A-n257L2 DC_7A-7A-8A_n78A-n257M2; DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K. DC_3A-3A-8A_n78A-n257A2 DC_3A-3A-8A_n78A-n257D2 DC_3A-3A-8A_n78A-n257E2 DC_3A-3A-8A_n78A-n257F2 DC_3A-3A-8A_n78A-n257G2 DC_3A-3A-8A_n78A-n257H2 DC_3A-3A-8A_n78A-n257I2 DC_3A-3A-8A_n78A-n257J2 DC_3A-3A-8A_n78A-n257K2 DC_3A-3A-8A_n78A-n257L2 DC_3A-3A-8A_n78A-n257M2; DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K. LTE(3DL/1UL) EN-DC combinations within FR1 and FR2 DL; UL. Reflected band combinations in RAN4#104e. DC_3A-7A-8A_n78A-n257A2 DC_3A-7A-8A_n78A-n257D2 DC_3A-7A-8A_n78A-n257E2 DC_3A-7A-8A_n78A-n257F2 DC_3A-7A-8A_n78A-n257G2 DC_3A-7A-8A_n78A-n257H2 DC_3A-7A-8A_n78A-n257I2 DC_3A-7A-8A_n78A-n257J2 DC_3A-7A-8A_n78A-n257K2 DC_3A-7A-8A_n78A-n257L2 DC_3A-7A-8A_n78A-n257M2; DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K. DC_3A-3A-7A-8A_n78A-n257A2 DC_3A-3A-7A-8A_n78A-n257D2 DC_3A-3A-7A-8A_n78A-n257E2 DC_3A-3A-7A-8A_n78A-n257F2 DC_3A-3A-7A-8A_n78A-n257G2 DC_3A-3A-7A-8A_n78A-n257H2 DC_3A-3A-7A-8A_n78A-n257I2 DC_3A-3A-7A-8A_n78A-n257J2 DC_3A-3A-7A-8A_n78A-n257K2 DC_3A-3A-7A-8A_n78A-n257L2 DC_3A-3A-7A-8A_n78A-n257M2; DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K. DC_3A-7A-7A-8A_n78A-n257A2 DC_3A-7A-7A-8A_n78A-n257D2 DC_3A-7A-7A-8A_n78A-n257E2 DC_3A-7A-7A-8A_n78A-n257F2 DC_3A-7A-7A-8A_n78A-n257G2 DC_3A-7A-7A-8A_n78A-n257H2 DC_3A-7A-7A-8A_n78A-n257I2 DC_3A-7A-7A-8A_n78A-n257J2 DC_3A-7A-7A-8A_n78A-n257K2 DC_3A-7A-7A-8A_n78A-n257L2 DC_3A-7A-7A-8A_n78A-n257M2; DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K. DC_3A-3A-7A-7A-8A_n78A-n257A2 DC_3A-3A-7A-7A-8A_n78A-n257D2 DC_3A-3A-7A-7A-8A_n78A-n257E2 DC_3A-3A-7A-7A-8A_n78A-n257F2 DC_3A-3A-7A-7A-8A_n78A-n257G2 DC_3A-3A-7A-7A-8A_n78A-n257H2 DC_3A-3A-7A-7A-8A_n78A-n257I2 DC_3A-3A-7A-7A-8A_n78A-n257J2 DC_3A-3A-7A-7A-8A_n78A-n257K2 DC_3A-3A-7A-7A-8A_n78A-n257L2 DC_3A-3A-7A-7A-8A_n78A-n257M2; DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K. |
| Consequences if not approved: |  | Corrections 38.101-3 are not made and new configurations are not included in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.2, 5.5B.4.3, 5.5B.4.4, 5.5B.4.5, 5.5B.5.2, 5.5B.6.2, 5.5B.6.3, 5.5B.6.4, 5.5B.6.5 6.2B.4.2.3.2, 6.2B.4.2.3.3, 6.2B.4.2.3.4, 6.2B.4.2.3.5, 7.3B.2.3.5.2, 7.3B.3.3.2, 7.3B.3.3.3, 7.3B.3.3.4, 7.3B.3.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | No |  |  |  |  |  |  |  |  |

<<Start of Change>>

5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n3A | DC_1A_n3A DC_3A_n3A2 |
| DC_1A-(n)3AA | DC_1A_n3A |
| DC_1A-3A_n5A DC_1A-3C_n5A | DC_1A_n5A DC_3A_n5A |
| DC_1A-3A_n7A DC_1A-3A_n7B DC_1A-3C_n7A DC_1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-1A-3A_n7A DC_1A-1A-3A_n7B DC_1A-1A-3C_n7A DC_1A-1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-3A-3A_n7A DC_1A-3A-3A_n7B | DC_1A_n7A DC_3A_n7A |
| DC_1A-1A-3A-3A_n7A DC_1A-1A-3A-3A_n7B | DC_1A_n7A DC_3A_n7A |
| DC_1A-3A_n8A | DC_1A_n8A DC_3A_n8A |
| DC_1A-3A_n28A DC_1A-3C_n28A | DC_1A_n28A DC_3A_n28A |
| DC_1A-1A-3A_n28A DC_1A-1A-3C_n28A | DC_1A_n28A DC_3A_n28A |
| DC_1A_n3A-n28A | DC_1A_n3A DC_1A_n28A |
| DC_1A-3A_n38A | DC_1A_n38A DC_3A_n38A |
| DC_1A_n3A-n38A | DC_1A_n3A DC_1A_n38A |
| DC_1A-3A_n40A | DC_1A_n40A DC_3A_n40A |
| DC_1A-3A_n41A5 DC_1A-3C_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A |
| DC_1A_n3A-n41A5 | DC_1A_n3A DC_1A_n41A |
| DC_1A-3A_n71A DC_1A-3A_n71B | DC_1A_n71A DC_3A_n71A |
| DC_1A-3A_n77A5 DC_1A-3A_n77C5 DC_1A-3C_n77A5 | DC_1A_n77A DC_3A_n77A DC_3C_n77A |
| DC_1A-3A_n77(2A)5 DC_1A-3C_n77(2A)5 | DC_1A_n77A DC_3A_n77A DC_3C_n77A |
| DC_1A-3A_n77(3A)5 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n78A5 DC_1A-3A_n78C5 DC_1A-3C_n78A5 | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A_n78(2A)5 DC_1A-3C_n78(2A)5 | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-1A-3A_n78A DC_1A-1A-3C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A_n3A-n8A | DC_1A_n3A DC_1A_n8A |
| DC_1A_n3A-n75A | DC_1A_n3A |
| DC_1A_n3A-n77A5 | DC_1A_n3A DC_1A_n77A |
| DC_1A_n3A-n77(2A) 5 | DC_1A_n3A DC_1A_n77A |
| DC_1A_n3A-n78A5 | DC_1A_n3A DC_1A_n78A |
| DC_1A_n3A-n79A | DC_1A_n3A DC_1A_n79A |
| DC_1A-3A_n79A5 DC_1A-3A_n79C5 | DC_1A_n79A DC_3A_n79A |
| DC_1A-5A_n77A | DC_1A_n77A DC_5A_n77A |
| DC_1A-5A_n77(2A) | DC_1A_n77A DC_5A_n77A |
| DC_1A-5A_n78A5 DC_1A-5A_n78C5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n78(2A)5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-1A-5A_n78A | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n79A | DC_1A_n79A DC_5A_n79A |
| DC_1A_n5A-n78A5 | DC_1A_n5A DC_1A_n78A |
| DC_1A-7A_n3A DC_1A-7C_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A |
| DC_1A-7A_n5A DC_1A-7C_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A |
| DC_1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-7A_n8A | DC_1A_n8A DC_7A_n8A |
| DC_1A-7A_n28A5 DC_1A-7C_n28A5 | DC_1A_n28A DC_7A_n28A DC_7C_n28A |
| DC_1A-1A-7A_n28A | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A_n40A | DC_1A_n40A DC_7A_n40A |
| DC_1A-7A_n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A_n77(2A) | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A_n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A_n77(2A) | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A_n78A5 DC_1A-7C_n78A5 DC_1A-7A_n78C5 | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n78(2A)5 DC_1A-7C_n78(2A)5 | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-1A-7A_n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-7A_n78A5 DC_1A-7A-7A_n78C5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-7A_n78(2A)5 | DC_1A_n78A DC_7A_n78A |
| DC_1A_n7A-n78A DC_1A_n7B-n78A | DC_1A_n7A DC_1A_n78A |
| DC_1A_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A_n28A | DC_1A_n28A DC_8A_n28A |
| DC_1A-8A_n40A | DC_1A_n40A DC_8A_n40A |
| DC_1A_n8A-n40A | DC_1A_n8A DC_1A_n40A |
| DC_1A-8A_n77A5 | DC_1A_n8A DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n77(2A)5 | DC_1A_n8A DC_1A_n77A DC_8A_n77A |
| DC_1A_n8A-n77A DC_1A_n8A-n77(2A) | DC_1A_n8A DC_1A_n77A |
| DC_1A-8A_n77(3A)5 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n78A5 | DC_1A_n78A DC_8A_n78A |
| DC_1A-8A_n78(2A)5 | DC_1A_n78A DC_8A_n78A |
| DC_1A_n8A-n78A5 | DC_1A_n8A DC_1A_n78A |
| DC_1A-8A_n79A5 | DC_1A_n79A DC_8A_n79A |
| DC_1A-11A_n3A | DC_1A_n3A DC_11A_n3A |
| DC_1A-11A_n28A | DC_1A_n28A DC_11A_n28A |
| DC_1A-11A_n41A5 | DC_1A_n41A DC_11A_n41A |
| DC_1A-11A_n77A5 | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n77(2A)5 | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n77(3A)5 | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n78A5 | DC_1A_n78A DC_11A_n78A |
| DC_1A-11A_n78(2A)5 | DC_1A_n78A DC_11A_n78A |
| DC_1A-11A_n79A5 | DC_1A_n79A DC_11A_n79A |
| DC_1A-18A_n3A | DC_1A_n3A DC_18A_n3A |
| DC_1A-18A_n28A | DC_1A_n28A DC_18A_n28A |
| DC_1A-18A_n41A | DC_1A_n41A DC_18A_n41A |
| DC_1A-18A_n77A5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A_n77(2A)5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A_n78A5 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n78(2A)5 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n79A | DC_1A_n79A DC_18A_n79A |
| DC_1A-19A_n77A5 DC_1A-19A_n77C5 | DC_1A_n77A DC_19A_n77A |
| DC_1A-19A_n77(2A)5 | DC_1A_n77A DC_19A_n77A |
| DC_1A-19A_n78A5 DC_1A-19A_n78C5 | DC_1A_n78A DC_19A_n78A |
| DC_1A-19A_n78(2A)5 | DC_1A_n78A DC_19A_n78A |
| DC_1A-19A_n79A5 DC_1A-19A_n79C5 | DC_1A_n79A DC_19A_n79A |
| DC_1A-20A_n3A DC_1C-20A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A_n7A | DC_1A_n7A DC_20A_n7A |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A_n28A | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A_n38A | DC_1A_n38A DC_20A_n38A |
| DC_1A-20A_n41A | DC_1A_n41A DC_20A_n41A |
| DC_1A-20A_n78A5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n78(2A)5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-21A_n28A13 | DC_1A_n28A DC_21A_n28A |
| DC_1A-21A_n77A5 DC_1A-21A_n77C5 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A_n77(2A)5 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A_n78A5 DC_1A-21A_n78C5 | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A_n78(2A)5 | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A_n79A5 DC_1A-21A_n79C5 | DC_1A_n79A DC_21A_n79A |
| DC_1A_n26A-n78A | DC_1A_n26A DC_1A_n78A |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A_n5A6 | DC_1A_n5A DC_28A_n5A |
| DC_1A-28A_n7A DC_1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A-1A-28A_n7A DC_1A-1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A_n28A-n40A | DC_1A_n28A DC_1A_n40A |
| DC_1A-28A_n40A | DC_1A_n40A DC_28A_n40A |
| DC_1A_n28A-n41A5 | DC_1A_n28A DC_1A_n41A |
| DC_1A_n28A-n75A | DC_1A_n28A |
| DC_1A-28A_n77A5 DC_1A-28A_n77C5 | DC_1A_n77A DC_28A_n77A |
| DC_1A-28A_n78A5 DC_1A-28A_n78C5 | DC_1A_n78A DC_28A_n78A |
| DC_1A-1A-28A_n78A | DC_1A_n78A DC_28A_n78A |
| DC_1A_n28A-n77A5 | DC_1A_n28A DC_1A_n77A |
| DC_1A_n28A-n77(2A)5 | DC_1A_n28A DC_1A_n77A |
| DC_1A_n28A-n78A5 | DC_1A_n28A DC_1A_n78A |
| DC_1A_n28A-n78(2A)5 | DC_1A_n28A DC_1A_n78A |
| DC_1A-28A_n79A5 DC_1A-28A_n79C5 | DC_1A_n79A DC_28A_n79A |
| DC_1A_n28A-n79A5 | DC_1A_n28A DC_1A_n79A |
| DC_1A-32A_n3A | DC_1A_n3A |
| DC_1A-32A_n8A | DC_1A_n8A |
| DC_1A-32A_n28A | DC_1A_n28A |
| DC_1A-32A_n78A DC_1A-32A_n78C | DC_1A_n78A |
| DC_1A-32A_n78(2A) | DC_1A_n78A |
| DC_1A-38A_n3A | DC_1A_n3A |
| DC_1A-38A_n8A | DC_1A_n8A DC_38A_n8A |
| DC_1A-38A_n28A | DC_1A_n28A DC_38A_n28A |
| DC_1A-(n)38AA | DC_1A_n38A |
| DC_1A_n38A-n78A | DC_1A_n38A DC_1A_n78A |
| DC_1A-38A_n78A | DC_1A_n78A |
| DC_1A-38A_n78(2A) | DC_1A_n78A |
| DC_1A-40A_n78A DC_1A-40C_n78A | DC_1A_n78A DC_40A_n78A |
| DC_1A-40A_n78(2A) DC_1A-40C_n78(2A) | DC_1A_n78A DC_40A_n78A |
| DC_1A_n40A-n78A | DC_1A_n40A DC_1A_n78A |
| DC_1A_n40A-n78(2A) | DC_1A_n40A DC_1A_n78A |
| DC_1A-41A_n3A5 DC_1A-41C_n3A5 | DC_1A_n3A DC_41A_n3A DC_41C_n3A |
| DC_1A-41A_n28A5 DC_1A-41C_n28A5 | DC_1A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-(n)41AA DC_1A-(n)41CA DC_1A-(n)41DA | DC_1A_n41A |
| DC_1A-41A_n41A DC_1A-41C_n41A | DC_1A_n41A |
| DC_1A-41A_n77A DC_1A-41C_n77A | DC_1A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | DC_1A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A_n41A-n77A | DC_1A_n41A DC_1A_n77A |
| DC_1A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A |
| DC_1A-41A_n78A DC_1A-41C_n78A | DC_1A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A_n41A-n78A | DC_1A_n41A DC_1A_n78A |
| DC_1A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A |
| DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | DC_1A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-41A_n79A5 DC_1A-41C_n79A5 | DC_1A_n79A |
| DC_1A-42A_n3A5 | DC_1A_n3A DC_42A_n3A |
| DC_1A-42C_n3A5 | DC_1A_n3A DC_42A_n3A DC_42C_n3A |
| DC_1A-42A_n28A5 | DC_1A_n28A DC_42A_n28A |
| DC_1A-42C_n28A5 | DC_1A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-42A_n77A15,16 DC_1A-42A_n77C15,16 DC_1A-42C_n77A15,16 DC_1A-42C_n77C15,16 DC_1A-42D_n77A15,16 DC_1A-42D_n77C15,16 DC_1A-42E_n77A15,16 DC_1A-42E_n77C15,16 | DC_1A_n77A |
| DC_1A-42A_n77(2A)15,16 DC_1A-42C_n77(2A)15,16 | DC_1A_n77A |
| DC_1A-42A_n78A15,16 DC_1A-42A_n78C15,16 DC_1A-42C_n78A15,16 DC_1A-42C_n78C15,16 DC_1A-42D_n78A15,16 DC_1A-42D_n78C15,16 DC_1A-42E_n78A15,16 DC_1A-42E_n78C15,16 | DC_1A_n78A |
| DC_1A-42A_n79A DC_1A-42A_n79C DC_1A-42C_n79A DC_1A-42C_n79C DC_1A-42D_n79A DC_1A-42D_n79C DC_1A-42E_n79A DC_1A-42E_n79C | DC_1A_n79A |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) | DC_1A_n78A |
| DC_1A_n77A-n79A DC_1A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A_SUL_n77A-n80A | DC_1A_n77A DC_1A_n80A |
| DC_1A_SUL_n77A-n84A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |
| DC_1A_n78A-n79A | DC_1A_n78A DC_1A_n79A |
| DC_1A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A |
| DC_1A_SUL_n78A-n84A5 | DC_1A_n78A, DC_1A_n84A_ULSUP-TDM_n78A |
| DC_1A_SUL_n79A-n84A | DC_1A_n79A, DC_1A_n84A_ULSUP-TDM_n79A |
| DC_2A_n2A-n38A | DC_2A_n38A |
| DC_2A_n2A-n41A | DC_2A_n41A |
| DC_2A_n2A-n66A | DC_2A_n66A |
| DC_2A_n2A-n71A | DC_2A_n71A |
| DC_2A_n2A-n77A14 DC_2A_n2A-n77C14 | DC_2A_n77A14 |
| DC_2A_n2A-n78A | DC_2A_n78A |
| DC_2A-4A_n28A | DC_2A_n28A DC_4A_n28A |
| DC_2A-4A_n38A | DC_2A_n38A DC_4A_n38A |
| DC_2A-4A_n41A | DC_2A_n41A DC_4A_n41A |
| DC_2A-5A_n2A | DC_5A_n2A DC_2A_n2A2 |
| DC_2A-5B_n2A | DC_5A_n2A |
| DC_2A-5A-5A_n2A | DC_5A_n2A |
| DC_2A-5A_n5A | DC_2A_n5A |
| DC_2A-2A-5A_n5A | DC_2A_n5A |
| DC_2A-(n)5AA | DC_2A_n5A DC_(n)5AA2 |
| DC_2A-2A-(n)5AA | DC_2A_n5A DC_(n)5AA2 |
| DC_2A-5A_n7A | DC_2A_n7A DC_5A_n7A |
| DC_2A-5A_n12A | DC_2A_n12A DC_5A_n12A |
| DC_2A-5A_n30A | DC_2A_n30A DC_5A_n30A |
| DC_2A-2A-5A_n30A | DC_2A_n30A DC_5A_n30A |
| DC_2A-5A_n48A DC_2A-5A_n48B | DC_2A_n48A DC_5A_n48A |
| DC_2A-5A_n66A DC_2A-5B_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-5A_n66A DC_2A-2A-5A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A_n71A | DC_2A_n71A DC_5A_n71A |
| DC_2A-5A_n77A14 DC_2A-5A_n77C14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-5A_n77(2A) | DC_2A_n77A DC_5A_n77A |
| DC_2A-2A-5A_n77A14 DC_2A-2A-5A_n77C14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-5A_n78A | DC_2A_n78A DC_5A_n78A |
| DC_2A-5A_n78(2A) | DC_2A_n78A DC_5A_n78A |
| DC_2A-7A_n2A | DC_7A_n2A |
| DC_2A-7A_n5A DC_2A-7C_n5A | DC_2A_n5A DC_7A_n5A |
| DC_2A-7A-7A_n5A | DC_2A_n5A DC_7A_n5A |
| DC_2A-7A_n7A | DC_2A_n7A DC_7A_n7A2 |
| DC_2A-7A_n25A15, 16 DC_2A-7A-7A_n25A15, 16 DC_2A-7C_n25A15, 16 | DC_7A_n25A |
| DC_2A-7A_n28A DC_2A-7C_n28A | DC_2A_n28A DC_7A_n28A |
| DC_2A_n5A-n77A14 DC_2A-2A_n5A-n77A14 DC_2A_n5A-n77C14 DC_2A-2A_n5A-n77C14 | DC_2A_n5A DC_2A_n77A14 |
| DC_2A-7A_n38A | 2A8 |
| DC_2A-2A-7A_n38A | 2A8 |
| DC_2A-7A_n66A DC_2A-7C_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7C_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A_n7A-n66A | DC_2A_n7A DC_7A_n66A |
| DC_2A_n7(2A)-n66A | DC_7A_n66A |
| DC_2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-7A_n77A DC_2A-7C_n77A | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-7A_n77A | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-7A_n77(2A) | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A_n78A DC_2A-7C_n78A | DC_2A_n78A DC_7A_n78A DC_7C_n78A |
| DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) | DC_2A_n78A DC_7A_n78A DC_7C_n78A |
| DC_2A-2A-7A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A_n7A-n78A | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7(2A)-n78A | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7A-n78(2A) | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7(2A)-n78(2A) | DC_2A_n7A DC_2A_n78A |
| DC_2A-7A-7A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-7A_n78(2A) | DC_2A_n78A DC_7A_n78A |
| DC_2A-8A_n2A | DC_2A_n2A2 DC_8A_n2A |
| DC_2A-12A_n2A | DC_12A_n2A |
| DC_2A-12A_n5A | DC_2A_n5A DC_12A_n5A |
| DC_2A-2A-12A_n5A | DC_2A_n5A DC_12A_n5A |
| DC_2A-12A_n7A | DC_2A_n7A DC_12A_n7A |
| DC_2A-12A_n7(2A) | DC_2A_n7A DC_12A_n7A |
| DC_2A-(n)12AA | DC_2A_n12A DC_(n)12AA2 |
| DC_2A-12A_n30A | DC_2A_n30A DC_12A_n30A |
| DC_2A-2A-12A_n30A | DC_2A_n30A DC_12A_n30A |
| DC_2A-12A_n41A | DC_2A_n41A DC_12A_n41A |
| DC_2A-2A-12A_n41A | DC_2A_n41A DC_12A_n41A |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-12A_n77A14 DC_2A-2A-12A_n77A14 | DC_2A_n77A14 DC_12A_n77A14 |
| DC_2A-12A_n77(2A) | DC_2A_n77A DC_12A_n77A |
| DC_2A-13A_n2A | DC_13A_n2A |
| DC_2A-12A_n78A | DC_2A_n78A DC_12A_n78A |
| DC_2A-12A_n78(2A) | DC_2A_n78A DC_12A_n78A |
| DC_2A-2A-12A_n78A | DC_2A_n78A DC_12A_n78A |
| DC_2A-13A_n5A | DC_2A_n5A |
| DC_2A-2A-13A_n5A | DC_2A_n5A |
| DC_2A-13A_n25A16,20 | DC_13A_n25A |
| DC_2A-13A_n48A DC_2A-13A_n48B | DC_2A_n48A DC_13A_n48A |
| DC_2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-13A_n77A14 DC_2A-13A_n77C14 DC_2A-2A-13A_n77C14 | DC_2A_n77A14 DC_13A_n77A14 |
| DC_2A-2A-13A_n77A | DC_2A_n77A14 DC_13A_n77A14 |
| DC_2A-14A_n2A | DC_2A_n2A2 DC_14A_n2A |
| DC_2A-14A_n5A | DC_2A_n5A DC_14A_n5A |
| DC_2A-2A-14A_n5A | DC_2A_n5A DC_14A_n5A |
| DC_2A-14A_n30A | DC_2A_n30A DC_14A_n30A |
| DC_2A-2A-14A_n30A | DC_2A_n30A DC_14A_n30A |
| DC_2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-14A_n77A14 DC_2A-2A-14A_n77A14 | DC_2A_n77A14 DC_14A_n77A14 |
| DC_2A-14A_n77(2A) | DC_2A_n77A DC_14A_n77A |
| DC_2A_n25A-n66A | DC_2A_n66A |
| DC_2A-28A_n7A | DC_2A_n7A DC_28A_n7A |
| DC_2A-28A_n66A | DC_2A_n66A DC_28A_n66A |
| DC_2A-28A_n78A | DC_2A_n78A DC_28A_n78A |
| DC_2A-29A_n30A | DC_2A_n30A |
| DC_2A-2A-29A_n30A | DC_2A_n30A |
| DC_2A-29A_n66A | DC_2A_n66A |
| DC_2A-2A-29A_n66A | DC_2A_n66A |
| DC_2A-29A_n77A14 DC_2A-2A-29A_n77A14 | DC_2A_n77A14 |
| DC_2A-29A_n78A | DC_2A_n78A |
| DC_2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-30A_n2A | DC_2A_n2A2 DC_30A_n2A |
| DC_2A-2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-30A_n77A14 DC_2A-2A-30A_n77A14 | DC_2A_n77A14 DC_30A_n77A14 |
| DC_2A-30A_n77(2A) | DC_2A_n77A DC_30A_n77A |
| DC_2A_n38A-n66A | DC_2A_n38A DC_2A_n66A |
| DC_2A_n38A-n71A | DC_2A_n38A DC_2A_n71A |
| DC_2A-38A_n78A | DC_2A_n78A DC_38A_n78A |
| DC_2A_n38A-n78A | DC_2A_n38A DC_2A_n78A |
| DC_2A_n41A-n66A DC_2A_n41C-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41(2A)-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41A-n71A DC_2A_n41C-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A_n2A3 DC_2A-46C_n2A3 DC_2A-46D_n2A3 DC_2A-46E_n2A3 | DC_2A_n2A2 |
| DC_2A-46A_n5A3 DC_2A-46C_n5A3 DC_2A-46D_n5A3 DC_2A-46E_n5A3 DC_2A-2A-46A_n5A3 DC_2A-2A-46C_n5A3 DC_2A-2A-46D_n5A3 | DC_2A_n5A |
| DC_2A-46A_n41A DC_2A-46C_n41A DC_2A-46D_n41A | DC_2A_n41A |
| DC_2A-46A_n41(2A) DC_2A-46C_n41(2A) DC_2A-46D_n41(2A) | DC_2A_n41A |
| DC_2A-46A_n66A DC_2A-46C_n66A DC_2A-46D_n66A DC_2A-46E_n66A | DC_2A_n66A |
| DC_2A-46A_n71A DC_2A-46C_n71A DC_2A-46D_n71A | DC_2A_n71A |
| DC_2A-46A_n77A | DC_2A_n77A |
| DC_2A-46A-46A_n77A | DC_2A_n77A |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | DC_2A_n2A2 DC_48A_n2A21 |
| DC_2A-48A_n5A | DC_2A_n5A DC_48A_n5A |
| DC_2A-48C_n5A DC_2A-48D_n5A DC_2A-48E_n5A | DC_2A_n5A |
| DC_2A_n48A-n66A | DC_2A_n48A DC_2A_n66A |
| DC_2A-48A_n71A | DC_2A_n71A DC_48A_n71A |
| DC_2A-48A_n12A | DC_2A_n12A DC_48A_n12A |
| DC_2A-48A_n48A | DC_2A_n48A |
| DC_2A-48A_n66A DC_2A-48C_n66A DC_2A-48D_n66A DC_2A-48E_n66A | DC_2A_n66A DC_48A_n66A |
| DC_2A-48A_n77A14,15,16 | DC_2A_n77A14 |
| DC_2A-48A-48A_n77A14,15,16 | DC_2A_n77A DC_48A_n77A |
| DC_2A-48A-48A-48A_n77A14,15,16 | DC_2A_n77A DC_48A_n77A |
| DC_2A-48C_n77A14,15,16 DC_2A-48D_n77A14,15,16 DC_2A-48E_n77A14,15,16 DC_2A-48A_n77C14,15,16 DC_2A-48C_n77C14,15,16 DC_2A-48D_n77C14,15,16 | DC_2A_n77A14 |
| DC_2A-66A_n2A | DC_2A_n2A2 DC_66A_n2A |
| DC_2A-66A-66A_n2A | DC_66A_n2A |
| DC_2A-66A_n5A DC_2A-66B_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A_n7A | DC_2A_n7A DC_66A_n7A |
| DC_2A-66A-66A_n7A | DC_2A_n7A DC_66A_n7A |
| DC_2A-66A_n12A | DC_2A_n12A DC_66A_n12A |
| DC_2A-66A_n25A16,20 | DC_66A_n25A |
| DC_2A-66A_n28A | DC_2A_n28A DC_66A_n28A |
| DC_2A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-2A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-66A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-66A_n41A14 DC_2A-66A_n41C DC_2C-66A_n41A | DC_2A_n41A DC_66A_n41A14 |
| DC_2A-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-2A-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-66A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-(n)66AA | DC_2A_n66A |
| DC_2A-2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-2A-66A-66A_n66A | DC_2A_n66A |
| DC_2A-66A_n71A DC_2A-66A_n71B DC_2A-66C_n71A DC_2C-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-2A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-66A_n77A14 DC_2A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-66A_n77(2A) | DC_2A_n77A DC_66A_n77A |
| DC_2A-2A-66A_n77A14 DC_2A-2A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-66A-66A_n77A14 DC_2A-66A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-2A-66A-66A_n77A14 DC_2A-2A-66A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A_n66A-n77A14 DC_2A_n66A-n77C14 DC_2A-2A_n66A-n77A14 DC_2A-2A_n66A-n77C14 | DC_2A_n77A14 DC_2A_n66A |
| DC_2A-66A_n78A DC_2A-2A-66A_n78A | DC_2A_n78A DC_66A_n78A |
| DC_2A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A |
| DC_2A_n66A-n78A DC_2A-2A_n66A-n78A | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66A-n78(2A) | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66(2A)-n78A | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66(2A)-n78(2A) | DC_2A_n66A DC_2A_n78A |
| DC_2A-66A-66A_n78A | DC_2A_n78A DC_66A_n78A |
| DC_2A-66A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A |
| DC_2A-71A_n2A | DC_71A_n2A |
| DC_2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-71A_n41A | DC_2A_n41A DC_71A_n41A |
| DC_2A-2A-71A_n41A | DC_2A_n41A DC_71A_n41A |
| DC_2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-71A_n71A | DC_2A_n71A |
| DC_2A-71A_n78A DC_2A-71A_n78(2A) | DC_71A_n78A DC_2A_n78A |
| DC_2A-2A-71A_n78A | DC_71A_n78A DC_2A_n78A |
| DC_2A_n71A-n78A | DC_2A_n71A DC_2A_n78A |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA |
| DC_3A_n1A-n7A | DC_3A_n1A DC_3A_n7A |
| DC_3C_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_3C_n1A DC_3C_n7A |
| DC_3A_n1A-n8A | DC_3A_n1A DC_3A_n8A |
| DC_3A-3A_n1A-n8A | DC_3A_n1A DC_3A_n8A |
| DC_3A_n1A-n28A | DC_3A_n1A DC_3A_n28A |
| DC_3C_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A |
| DC_3A_n1A-n38A | DC_3A_n1A DC_3A_n38A |
| DC_3A_n1A-n40A | DC_3A_n1A DC_3A_n40A |
| DC_3A_n1A-n41A | DC_3A_n1A DC_3A_n41A |
| DC_3A_n1A-n75A | DC_3A_n1A |
| DC_3C_n1A-n75A | DC_3C_n1A |
| DC_3A_n1A-n77A5 | DC_3A_n1A DC_3A_n77A |
| DC_3A_n1A-n78A5 DC_3C_n1A-n78A5 | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A |
| DC_3A_n1A-n78(2A)5 DC_3C_n1A-n78(2A)5 | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A |
| DC_3A-3A_n1A-n78A5 | DC_3A_n1A DC_3A_n78A |
| DC_3A_n1A-n79A5 | DC_3A_n1A DC_3A_n79A |
| DC_(n)3AA-n8A | DC_(n)3AA2 DC_3A_n8A |
| DC_3A_n3A-n41A | DC_3A_n41A DC_3A_n3A2 |
| DC_3A_n3A-n77A5 | DC_3A_n77A DC_3A_n3A2 |
| DC_(n)3AA-n77A | DC_(n)3AA2 DC_3A_n77A |
| DC_(n)3AA-n77(2A) | DC_(n)3AA2 DC_3A_n77A |
| DC_3A_n3A-n78A5 | DC_3A_n78A DC_3A_n3A2 |
| DC_3A_n5A-n40A | DC_3A_n5A DC_3A_n40A |
| DC_3A-5A_n77A | DC_3A_n77A DC_5A_n77A |
| DC_3A-5A_n77(2A) | DC_3A_n77A DC_5A_n77A |
| DC_3A-5A_n78A5 DC_3C-5A_n78A DC_3A-5A_n78C5 | DC_3A_n78A DC_5A_n78A |
| DC_3A-5A_n78(2A)5 | DC_3A_n78A DC_5A_n78A |
| DC_3A_n5A-n78A5, 14 DC_3C_n5A-n78A5, 14 | DC_3A_n5A DC_3A_n78A14 DC_3C_n78A14 |
| DC_3A-5A_n79A5 | DC_3A_n79A DC_5A_n79A |
| DC_3A-7A_n1A DC_3A-7C_n1A DC_3C-7A_n1A DC_3C-7C_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A |
| DC_3A-3A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A_n3A DC_3A-7C_n3A | DC_3A_n3A2 DC_7A_n3A |
| DC_3A-7A_n5A DC_3C-7A_n5A DC_3A-7C_n5A DC_3C-7C_n5A | DC_3A_n5A DC_7A_n5A DC_7C_n5A |
| DC_3A-7A_n7A DC_3C-7A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A2 |
| DC_3A-3A-7A_n7A | DC_3A_n7A DC_7A_n7A2 |
| DC_3A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-3A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-7A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-3A-7A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_7C_n28A |
| DC_3A-7A_n40A | DC_3A_n40A DC_7A_n40A |
| DC_3A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-3A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-3A-7A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A_n77(2A) | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A-7A_n77(2A) | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A_n78A5,14 DC_3C-7A_n78A5,14 DC_3A-7C_n78A5,14 DC_3C-7C_n78A5,14 DC_3A-7A_n78C5 | DC_3A_n78A14 DC_3C_n78A14 DC_7A_n78A14 DC_7C_n78A14 |
| DC_3A_n7A-n28A DC_3C_n7A-n28A | DC_3A_n7A DC_3A_n28A DC_3C_n7A |
| DC_3A-7A_n78(2A)5 DC_3C-7A_n78(2A)5 DC_3A-7C_n78(2A)5 DC_3C-7C_n78(2A)5 | DC_3A_n78A DC_7A_n78A DC_3C_n78A DC_7C_n78A |
| DC_3A-3A-7A_n78A5, 14 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A-7A-7A_n78A5, 14 DC_3A-7A-7A_n78C5 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A-7A-7A_n78(2A)5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-3A-7A-7A_n78A5, 14 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A_n7A-n78A5 DC_3A_n7B-n78A5 DC_3C_n7A-n78A5 DC_3C_n7B-n78A5 | DC_3A_n7A DC_3C_n7A DC_3A_n78A DC_3C_n78A |
| DC_3A-3A_n7A-n78A5 DC_3A-3A_n7B-n78A5 | DC_3A_n7A DC_3A_n7B DC_3A_n78A |
| DC_3A_n7A-n78(2A)5 DC_3C_n7A-n78(2A)5 | DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_3A-8A_n1A DC_3C-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-3A-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-3A_n8A-n78A5 | DC_3A_n8A DC_3A_n78A |
| DC_3A_n8A-n40A | DC_3A_n8A DC_3A_n40A |
| DC_3A-8A_n28A DC_3C-8A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3A-8A_n40A | DC_3A_n40A DC_8A_n40A |
| DC_3A-8A_n77A5 DC_3C-8A_n77A | DC_3A_n77A DC_3C_n77A DC_8A_n77A |
| DC_3A-8A_n77(2A) 5 DC_3C-8A_n77(2A) | DC_3A_n77A DC_3C_n77A DC_8A_n77A |
| DC_3A-8A_n77(3A) 5 | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n78A5, 14 DC_3C-8A_n78A5 | DC_3A_n78A14 DC_8A_n78A14 |
| DC_3A-8A_n78(2A) | DC_3A_n78A DC_8A_n78A |
| DC_3A-3A-8A_n78A5, 14 | DC_3A_n78A14 DC_8A_n78A14 |
| DC_3A-8A_n79A5 | DC_3A_n79A DC_8A_n79A |
| DC_3A_n8A-n78A5 | DC_3A_n8A DC_3A_n78A |
| DC_3A-11A_n28A | DC_3A_n28A DC_11A_n28A |
| DC_3A-11A_n77A5 | DC_3A_n77A DC_11A_n77A |
| DC_3A-11A_n77(2A) 5 | DC_3A_n77A DC_11A_n77A |
| DC_3A-11A_n77(3A) 5 | DC_3A_n77A DC_11A_n77A |
| DC_3A-18A_n3A | DC_3A_n3A2 DC_18A_n3A |
| DC_3A-18A_n28A | DC_3A_n28A DC_18A_n28A |
| DC_3A-18A_n41A | DC_3A_n41A DC_18A_n41A |
| DC_3A-18A_n77A | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A_n77(2A) | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A_n78A | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A_n78(2A) | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A_n79A | DC_3A_n79A DC_18A_n79A |
| DC_3A-19A_n1A | DC_3A_n1A DC_19A_n1A |
| DC_3A-19A_n77A5 DC_3A-19A_n77C5 | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A_n77(2A)5 | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A_n78A5 DC_3A-19A_n78C5 | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A_n78(2A)5 | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A_n79A5 DC_3A-19A_n79C5 | DC_3A_n79A DC_19A_n79A |
| DC_3A-20A_n1A DC_3C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-20A_n7A DC_3C-20A_n7A | DC_3A_n7A DC_3C_n7A DC_20A_n7A |
| DC_3A-20A_n8A | DC_3A_n8A DC_20A_n8A |
| DC_3A-20A_n28A5,6,16,20 DC_3C-20A_n28A5,6,16,20 | DC_3A_n28A DC_20A_n28A |
| DC_3A-20A_n41A | DC_3A_n41A DC_20A_n41A |
| DC_3C-20A_n41A | DC_3C_n41A DC_20A_n41A |
| DC_3A-20A_n38A | DC_3A_n38A DC_20A_n38A |
| DC_3A_n20A-n67A DC_3C_n20A-n67A | DC_3A_n20A |
| DC_3A-20A_n78A5 DC_3C-20A_n78A5 | DC_3A_n78A DC_3C_n78A DC_20A_n78A |
| DC_3A-20A_n78(2A)5 | DC_3A_n78A DC_20A_n78A |
| DC_3A_n20A-n78A | DC_3A_n20A DC_3A_n78A |
| DC_3A-21A_n1A10,11 | DC_3A_n1A DC_21A_n1A |
| DC_3A-21A_n28A13 | DC_3A_n28A DC_21A_n28A |
| DC_3A-21A_n77A5 DC_3A-21A_n77C5 | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A_n77(2A)5 | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A_n78A5 DC_3A-21A_n78C5 | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A_n78(2A)5 | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A_n79A5 DC_3A-21A_n79C5 | DC_3A_n79A DC_21A_n79A |
| DC_3A-n26A-n78A | DC_3A_n26A DC_3A_n78A |
| DC_3C_n26A-n78A | DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |
| DC_3A-28A_n1A DC_3C-28A_n1A | DC_28A_n1A DC_3A_n1A |
| DC_3A-28A_n3A | DC_3A_n3A2 DC_28A_n3A |
| DC_3A-28A_n5A DC_3C-28A_n5A | DC_3A_n5A DC_28A_n5A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |
| DC_3A-28A_n40A | DC_3A_n40A DC_28A_n40A |
| DC_3A-3A-28A_n7A DC_3A-3A-28A_n7B | DC_3A_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |
| DC_3A_n28A-n40A | DC_3A_n28A DC_3A_n40A |
| DC_3A_n28A-n41A5 | DC_3A_n28A DC_3A_n41A |
| DC_3A-28A_n41A5 | DC_3A_n41A DC_28A_n41A |
| DC_3A_n28A-n75A DC_3C_n28A-n75A | DC_3A_n28A |
| DC_3A-28A_n77A5 DC_3A-28A_n77C5 | DC_3A_n77A DC_28A_n77A |
| DC_3A-28A_n77(2A) 5 | DC_3A_n77A DC_28A_n77A |
| DC_3A_n28A-n77A5 | DC_3A_n28A DC_3A_n77A |
| DC_3A_n28A-n77(2A)5 | DC_3A_n28A DC_3A_n77A |
| DC_3A-28A_n78A5, 14 DC_3C-28A_n78A5, 14 DC_3A-28A_n78C5 | DC_3A_n78A14 DC_3C_n78A14 DC_28A_n78A14 |
| DC_3A-3A-28A_n78A | DC_3A_n78A DC_28A_n78A |
| DC_3A_n28A-n78A5, 14 DC_3C_n28A-n78A5, 14 | DC_3A_n28A DC_3A_n78A14 DC_3C_n78A14 |
| DC_3A_n28A-n78(2A)5 DC_3C_n28A-n78(2A)5 | DC_3A_n28A DC_3A_n78A DC_3C_n78A |
| DC_3A-28A_n79A5 DC_3A-28A_n79C5 | DC_3A_n79A DC_28A_n79A |
| DC_3A_n28A-n79A5 | DC_3A_n28A DC_3A_n79A |
| DC_3A-32A_n1A DC_3C-32A_n1A | DC_3A_n1A DC_3C_n1A |
| DC_3A-32A_n28A DC_3C-32A_n28A | DC_3A_n28A |
| DC_3A-32A_n78A DC_3C-32A_n78A DC_3A-32A_n78C | DC_3A_n78A DC_3C_n78A |
| DC_3A-32A_n78(2A) | DC_3A_n78A DC_3C_n78A |
| DC_3A-38A_n28A DC_3C-38A_n28A | DC_3A_n28A DC_38A_n28A |
| DC_3A_n38A-n40A | DC_3A_n38A DC_3A_n40A |
| DC_3A-38A_n78A | DC_3A_n78A |
| DC_3A-38A_n78(2A) | DC_3A_n78A |
| DC_3A_n38A-n78A | DC_3A_n38A DC_3A_n78A |
| DC_3C-38A_n78A | DC_3A_n78A DC_3C_n78A DC_38A_n78A |
| DC_3A-40A_n1A DC_3A-40C_n1A | DC_3A_n1A DC_40A_n1A |
| DC_3A_n40A-n41A DC_3A_n40A-n41C | DC_3A_n40A DC_3A_n41A |
| DC_3A-40A_n78A DC_3A-40C_n78A | DC_3A_n78A DC_40A_n78A |
| DC_3A-40A_n78(2A) DC_3A-40C_n78(2A) | DC_3A_n78A DC_40A_n78A |
| DC_3A_n40A-n78A | DC_3A_n40A DC_3A_n78A |
| DC_3A_n40A-n79A DC_3A_n40A-n79C | DC_3A_n40A DC_3A_n79A |
| DC_3A-41A_n3A DC_3A-41C_n3A | DC_3A_n3A2 DC_41A_n3A DC_41C_n3A |
| DC_3A-41A_n28A5 | DC_3A_n28A DC_41A_n28A |
| DC_3A-41C_n28A5 | DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_3A-41A_n41A DC_3A-41C_n41A DC_3A-41D_n41A | DC_3A_n41A |
| DC_3A-(n)41AA DC_3A-(n)41CA DC_3A-(n)41DA | DC_3A_n41A DC_(n)41AA |
| DC_3A-41A_n77A DC_3A-41C_n77A | DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) | DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_3A-41A_n78A DC_3A-41C_n78A | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A_n41A-n78A | DC_3A_n41A DC_3A_n78A |
| DC_3A_n41A-n78(2A) | DC_3A_n41A DC_3A_n78A |
| DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-42A_n1A5 DC_3A-42C_n1A5 | DC_3A_n1A DC_42A_n1A |
| DC_3A-42A_n28A5 | DC_3A_n28A DC_42A_n28A |
| DC_3A-42C_n28A5 | DC_3A_n28A DC_42A_n28A DC_42C_n28A |
| DC_3A-41A_n79A5 DC_3A-41C_n79A5 | DC_3A_n79A DC_41A_n79A |
| DC_3A_n41A-n77A | DC_3A_n41A DC_3A_n77A |
| DC_3A_n41A-n77(2A) | DC_3A_n41A DC_3A_n77A |
| DC_3A_n41A-n79A5 DC_3A_n41C-n79A5 DC_3A_n41A-n79C5 DC_3A_n41C-n79C5 | DC_3A_n41A DC_3A_n79A |
| DC_3A_SUL_n41A-n80A DC_3C_SUL_n41A-n80A | DC_3A_n41A DC_3C_n41A DC_3A_n80A_ULSUP-TDM_n41A DC_3C_n80A_ULSUP-TDM_n41A |
| DC_3A-42A_n77A15,16 DC_3A-42A_n77C15,16 DC_3A-42C_n77A15,16 DC_3A-42C_n77C15,16 DC_3A-42D_n77A15,16 DC_3A-42D_n77C15,16 DC_3A-42E_n77A15,16 DC_3A-42E_n77C15,16 | DC_3A_n77A |
| DC_3A-42A_n77(2A)15,16 DC_3A-42C_n77(2A)15,16 | DC_3A_n77A |
| DC_3A-42A_n78A15,16 DC_3A-42A_n78C15,16 DC_3A-42C_n78A15,16 DC_3A-42C_n78C15,16 DC_3A-42D_n78A15,16 DC_3A-42D_n78C15,16 DC_3A-42E_n78A15,16 DC_3A-42E_n78C15,16 | DC_3A_n78A |
| DC_3A-42A_n79A DC_3A-42A_n79C DC_3A-42C_n79A DC_3A-42C_n79C DC_3A-42D_n79A DC_3A-42D_n79C DC_3A-42E_n79A DC_3A-42E_n79C | DC_3A_n79A |
| DC_3A_n75A-n78A | DC_3A_n78A |
| DC_3A_n75A-n78(2A) | DC_3A_n78A |
| DC_3A_n77A-n79A | DC_3A_n77A DC_3A_n79A |
| DC_3A_n78A-n79A DC_3A_n78A-n79C | DC_3A_n78A DC_3A_n79A |
| DC_3A_SUL_n77A-n80A | DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A |
| DC_3A_SUL_n77A-n84A | DC_3A_n77A DC_3A_n84A |
| DC_3A_SUL_n78A-n80A5 DC_3C_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |
| DC_3A_SUL_n78A-n82A5 | DC_3A_n78A DC_3A_n82A |
| DC_3A_SUL_n78A-n84A | DC_3A_n78A DC_3A_n84A |
| DC_3A_SUL_n79A-n80A5 | DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A |
| DC_4A-7A_n28A | DC_4A_n28A DC_7A_n28A |
| DC_5A_n1A-n78A | DC_5A_n1A DC_5A_n78A |
| DC_5A_n2A-n77A14 DC_5A_n2A-n77C14 | DC_5A_n77A14 DC_5A_n2A |
| DC_5A_n2A-n78A | DC_5A_n2A DC_5A_n78A |
| DC_5A_n3A-n78A | DC_5A_n3A DC_5A_n78A |
| DC_5A_n5A-n77A14 DC_5A_n5A-n77C14 | DC_5A_n77A14 |
| DC_5A-7A_n2A | DC_7A_n2A |
| DC_5A-7A_n7A | DC_5A_n7A DC_7A_n7A2 |
| DC_5A-7A_n66A DC_5A-7C_n66A | DC_5A_n66A DC_7A_n66A |
| DC_5A-7A-7A_n66A | DC_5A_n66A DC_7A_n66A |
| DC_5A-7A_n77A | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A_n77A | DC_5A_n71A DC_7A_n71A |
| DC_5A-7A_n77(2A) | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A_n77(2A) | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A_n78A DC_5A-7A_n78C DC_5A-7C_n78A | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A_n78(2A) | DC_5A_n78A DC_7A_n78A |
| DC_5A_n7A-n78A | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7(2A)-n78A | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7A-n78(2A) | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7(2A)-n78(2A) | DC_5A_n7A DC_5A_n78A |
| DC_5A-7A-7A_n78A DC_5A-7A-7A_n78C | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A-7A_n78(2A) | DC_5A_n78A DC_7A_n78A |
| DC_5A-(n)12AA | DC_5A_n12A DC_(n)12AA2 |
| DC_5A-13A_n2A | DC_5A_n2A DC_13A_n2A |
| DC_5A-13A_n66A | DC_5A_n66A DC_13A_n66A |
| DC_5A-13A_n77A DC_5A-13A_n77C | DC_5A_n77A DC_13A_n77A |
| DC_5A-30A_n2A | DC_5A_n2A DC_30A_n2A |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A |
| DC_5A-30A_n77A14 | DC_5A_n77A14 DC_30A_n77A14 |
| DC_5A-30A_n77(2A) | DC_5A_n77A DC_30A_n77A |
| DC_5A_n38A-n66A | DC_5A_n38A DC_5A_n66A |
| DC_5A-41A_n79A | DC_5A_n79A DC_41A_n79A |
| DC_5A-46A_n66A | DC_5A_n66A DC_46A_n66A |
| DC_5A-48A_n5A | DC_48A_n5A |
| DC_5A-48A_n12A | DC_5A_n12A DC_48A_n12A |
| DC_5A-48A_n71A | DC_5A_n71A DC_48A_n71A |
| DC_5A-48A_n77A14,15,16 DC_5A-48C_n77A14,15,16 DC_5A-48D_n77A14,15,16 DC_5A-48A_n77C14,15,16 DC_5A-48C_n77C14,15,16 DC_5A-48D_n77C14,15,16 | DC_5A_n77A14 |
| DC_5A-66A_n2A DC_5B-66A_n2A DC_5A-66B_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-5A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-5A-66A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-66A_n5A | DC_66A_n5A |
| DC_5A-66A-66A_n5A | DC_66A_n5A |
| DC_5A-66A_n7A | DC_5A_n7A DC_66A_n7A |
| DC_5A-66A-66A_n7A | DC_5A_n7A DC_66A_n7A |
| DC_5A-66A_n12A | DC_5A_n12A DC_66A_n12A |
| DC_5A-66A_n30A | DC_5A_n30A DC_66A_n30A |
| DC_5A-66A-66A_n30A | DC_5A_n30A DC_66A_n30A |
| DC_5A-66A_n48A DC_5A-66A_n48B | DC_5A_n48A DC_66A_n48A |
| DC_5A-66A-66A_n48A DC_5A-66A-66A_n48B | DC_5A_n48A DC_66A_n48A |
| DC_5A-66A_n66A DC_5B-66A_n66A | DC_5A_n66A |
| DC_5A-5A-66A_n66A | DC_5A_n66A |
| DC_5A-66A-66A_n66A DC_5B-66A-66A_n66A | DC_5A_n66A |
| DC_5A-5A-66A-66A_n66A | DC_5A_n66A |
| DC_5A-66A_n71A | DC_5A_n71A DC_66A_n71A |
| DC_5A-66A_n77A14 DC_5A-66A_n77C14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A-66A_n77(2A) | DC_5A_n77A DC_66A_n77A |
| DC_5A-66A-66A_n77A14 DC_5A-66A-66A_n77C14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A_n66A-n77A14 DC_5A_n66A-n77C14 | DC_5A_n66A DC_5A_n77A14 |
| DC_5A-66A_n78A | DC_5A_n78A DC_66A_n78A |
| DC_5A-66A_n78(2A) | DC_5A_n78A DC_66A_n78A |
| DC_5A_n66A-n78A | DC_5A_n66A DC_5A_n78A |
| DC_5A-66A-66A_n78A | DC_5A_n78A DC_66A_n78A |
| DC_7A_n1A-n8A | DC_7A_n1A DC_7A_n8A |
| DC_7A-7A_n1A-n8A | DC_7A_n1A DC_7A_n8A |
| DC_7A_n1A-n40A | DC_7A_n1A DC_7A_n40A |
| DC_7A_n1A-n75A | DC_7A_n1A |
| DC_7A_n1A-n78A5 DC_7C_n1A-n78A5 | DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_7A-7A_n1A-n78A5 | DC_7A_n1A DC_7A_n78A |
| DC_7A_n2A-n66A | DC_7A_n2A DC_7A_n66A |
| DC_7A_n2A-n71A | DC_7A_n2A DC_7A_n71A |
| DC_7A_n2A-n78A | DC_7A_n2A DC_7A_n78A |
| DC_7A_n3A-n78A DC_7C_n3A-n78A | DC_7A_n3A DC_7A_n78A DC_7C_n3A DC_7C_n78A |
| DC_7A_n5A-n78A14 DC_7C_n5A-n78A14 | DC_7A_n5A DC_7C_n5A DC_7A_n78A14 DC_7C_n78A14 |
| DC_7A_n7A-n78A5 | DC_7A_n78A DC_7A_n7A2 |
| DC_7A_n7A-n78(2A) | DC_7A_n78A DC_7A_n7A2 |
| DC_7A-8A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-7A-8A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A_n3A | DC_7A_n3A DC_8A_n3A |
| DC_7A-8A_n28A | DC_7A_n28A DC_8A_n28A |
| DC_7A-8A_n40A | DC_7A_n40A DC_8A_n40A |
| DC_7A_n8A-n40A | DC_7A_n8A DC_7A_n40A |
| DC_7A-8A_n77A5 | DC_7A_n77A DC_8A_n77A |
| DC_7A-8A_n78A5, 14 | DC_7A_n78A14 DC_8A_n78A14 |
| DC_7A-8A_n78(2A) | DC_7A_n78A DC_8A_n78A |
| DC_7A-7A-8A_n78A5, 14 | DC_7A_n78A14 DC_8A_n78A14 |
| DC_7A-7A_n8A-n78A5 | DC_7A_n8A DC_7A_n78A |
| DC_7A_n8A-n78A5 | DC_7A_n8A DC_7A_n78A |
| DC_7A-12A_n2A | DC_7A_n2A DC_12A_n2A |
| DC_7A-12A_n66A | DC_7A_n66A DC_12A_n66A |
| DC_7A-12A_n78A DC_7A-12A_n78(2A) | DC_7A_n78A DC_12A_n78A |
| DC_7A-13A_n25A DC_7C-13A_n25A | DC_7A_n25A DC_13A_n25A |
| DC_7A-7A-13A_n25A | DC_7A_n25A DC_13A_n25A |
| DC_7A-13A_n66A DC_7C-13A_n66A | DC_7A_n66A DC_13A_n66A |
| DC_7A-7A-13A_n66A | DC_7A_n66A DC_13A_n66A |
| DC_7A-20A_n1A DC_7C-20A_n1A | DC_7A_n1A DC_7C_n1A DC_20A_n1A |
| DC_7A-20A_n3A DC_7C-20A_n3A | DC_7A_n3A DC_7C_n3A DC_20A_n3A |
| DC_7A-20A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A_n28A6,16,20 | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A_n78A5 | DC_7A_n78A DC_20A_n78A |
| DC_7A-20A_n78(2A)5 | DC_7A_n78A DC_20A_n78A |
| DC_7A_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7A-7A_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7C_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7A-25A_n77A DC_7C-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-7A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-25A-25A_n77A DC_7C-25A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-7A-25A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-25A_n78A DC_7C-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-7A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-25A-25A_n78A DC_7C-25A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-7A-25A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-n26A-n78A | DC_7A_n26A DC_7A_n78A |
| DC_7C_n26A-n78A | DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_7A-28A_n1A | DC_28A_n1A DC_7A_n1A |
| DC_7A-7A-28A_n1A | DC_28A_n1A DC_7A_n1A |
| DC_7A-28A_n2A | DC_7A_n2A DC_28A_n2A |
| DC_7A-28A_n3A DC_7C-28A_n3A | DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_7A-28A_n5A6 DC_7C-28A_n5A6 | DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_7A-28A_n7A | DC_7A_n7A2 DC_28A_n7A |
| DC_7A_n28A-n40A | DC_7A_n28A DC_7A_n40A |
| DC_7A-28A_n40A | DC_7A_n40A DC_28A_n40A |
| DC_7A-28A_n66A DC_7C-28A_n66A | DC_7A_n66A DC_28A_n66A |
| DC_7A-28A_n78A5,14 DC_7C-28A_n78A5,14 | DC_7A_n78A14 DC_7C_n78A14 DC_28A_n78A14 |
| DC_7A_n28A-n78A5,14 DC_7C_n28A-n78A14 | DC_7A_n28A DC_7A_n78A14 DC_7C_n28A DC_7C_n78A14 |
| DC_7A-29A_n78A DC_7C-29A_n78A | DC_7A_n78A |
| DC_7A-7A-29A_n78A | DC_7A_n78A |
| DC_7A-32A_n1A | DC_7A_n1A |
| DC_7A-32A_n3A DC_7C-32A_n3A | DC_7A_n3A |
| DC_7A-32A_n8A | DC_7A_n8A |
| DC_7A-32A_n28A | DC_7A_n28A |
| DC_7A-32A_n78A | DC_7A_n78A |
| DC_7A-40A_n1A DC_7A-40C_n1A | DC_7A_n1A DC_40A_n1A |
| DC_7A-40A_n78A DC_7A-40C_n78A | DC_7A_n78A DC_40A_n78A |
| DC_7A-40A_n78(2A) DC_7A-40C_n78(2A) | DC_7A_n78A DC_40A_n78A |
| DC_7A_n40A-n78A | DC_7A_n40A DC_7A_n78A |
| DC_7A-46A_n78A3 DC_7A-46C_n78A3 DC_7A-46D_n78A3 DC_7A-46E_n78A3 | DC_7A_n78A |
| DC_7A-66A_n2A | DC_7A_n2A DC_66A_n2A |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | DC_7A_n5A DC_66A_n5A |
| DC_7A-66A_n7A | DC_7A_n7A2 DC_66A_n7A |
| DC_7A-66A-66A_n7A | DC_7A_n7A2 DC_66A_n7A |
| DC_7A-66A_n25A DC_7C-66A_n25A | DC_7A_n25A DC_66A_n25A |
| DC_7A-7A-66A_n25A | DC_7A_n25A DC_66A_n25A |
| DC_7A-66A_n28A | DC_7A_n28A DC_66A_n28A |
| DC_7A-66A_n38A | 66A9 |
| DC_7A-66A_n66A DC_7C-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-7A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-66A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-7A-66A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A-66A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A_n66A-n71A | DC_7A_n66A DC_7A_n71A |
| DC_7A-66A_n77A DC_7C-66A_n77A | DC_7A_n77A DC_66A_n77A |
| DC_7A-7A-66A_n77A | DC_7A_n66A DC_66A_n77A |
| DC_7A-7A-66A_n77(2A) | DC_7A_n66A DC_66A_n77A |
| DC_7A-66A_n77(2A) DC_7C-66A_n77(2A) | DC_7A_n66A DC_66A_n77A |
| DC_7A_n66A-n77A DC_7C_n66A-n77A | DC_7A_n66A DC_7A_n77A |
| DC_7A-7A_n66A-n77A | DC_7A_n66A DC_7A_n77A |
| DC_7A_n66A-n78A DC_7C_n66A-n78A | DC_7A_n66A DC_7A_n78A |
| DC_7A-7A_n66A-n78A | DC_7A_n66A DC_7A_n78A |
| DC_7A-66A_n78A DC_7C-66A_n78A | DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) | DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_7A-7A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-66A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-66A-66A_n78A DC_7C-66A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-71A_n2A | DC_7A_n2A DC_71A_n2A |
| DC_7A-71A_n66A | DC_7A_n66A DC_71A_n66A |
| DC_7A-71A_n78A DC_7A-71A_n78(2A) | DC_7A_n78A DC_71A_n78A |
| DC_7A_n71A-n78A | DC_7A_n71A DC_7A_n78A |
| DC_7A_n78A-n79A DC_7A_n78A-n79C | DC_7A_n78A DC_7A_n79A |
| DC_7A_SUL_n78A-n80A | DC_7A_n78A DC_7A_n80A |
| DC_8A_n1A-n3A | DC_8A_n1A DC_8A_n3A |
| DC_8A_n1A-n28A | DC_8A_n1A DC_8A_n28A |
| DC_8A_n1A-n40A | DC_8A_n1A DC_8A_n40A |
| DC_8A_n1A-n77A5 DC_8A_n1A-n77(2A)5 | DC_8A_n1A DC_8A_n77A |
| DC_8A_n1A-n78A5 | DC_8A_n1A DC_8A_n78A |
| DC_8A-(n)3AA | DC_(n)3AA DC_8A_n3A |
| DC_8A_n3A-n28A | DC_8A_n3A DC_8A_n28A |
| DC_8A_n3A-n77A5 | DC_8A_n3A DC_8A_n77A |
| DC_8A_n3A-n77(2A) 5 | DC_8A_n3A DC_8A_n77A |
| DC_8A_n3A-n78A | DC_8A_n3A DC_8A_n78A |
| DC_8A_n3A-n79A | DC_8A_n3A DC_8A_n79A |
| DC_8A-11A_n1A | DC_8A_n1A DC_11A_n1A |
| DC_8A-11A_n3A | DC_8A_n3A DC_11A_n3A |
| DC_8A-11A_n28A | DC_8A_n28A DC_11A_n28A |
| DC_8A-11A_n77A5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n77(2A)5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n77(3A)5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n78A5 | DC_8A_n78A DC_11A_n78A |
| DC_8A-11A_n79A5 | DC_8A_n79A DC_11A_n79A |
| DC_8A-20A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_8A-20A_n3A | DC_8A_n3A DC_20A_n3A |
| DC_8A-20A_n28A6,16,19,20 | DC_8A_n28A DC_20A_n28A |
| DC_8A-20A_n78A | DC_8A_n78A DC_20A_n78A |
| DC_8A-28A_n3A | DC_8A_n3A DC_28A_n3A |
| DC_8A-28A_n78A | DC_8A_n78A DC_28A_n78A |
| DC_8A_n28A-n77A5 | DC_8A_n28A DC_8A_n77A |
| DC_8A_n28A-n77(2A)5 | DC_8A_n28A DC_8A_n77A |
| DC_8A_n28A-n78A5 | DC_8A_n28A DC_8A_n78A |
| DC_8A_n28A-n79A5 | DC_8A_n28A DC_8A_n79A |
| DC_8A-32A_n1A | DC_8A_n1A |
| DC_8A-32A_n3A | DC_8A_n3A |
| DC_8A-32A_n28A | DC_8A_n28A |
| DC_8A-32A_n78A | DC_8A_n78A |
| DC_8A-38A_n1A | DC_8A_n1A DC_38A_n1A |
| DC_8A_n38A-n40A | DC_8A_n38A DC_8A_n40A |
| DC_8A_n39A-n40A | DC_8A_n39A DC_8A_n40A |
| DC_8A_n39A-n41A | DC_8A_n39A DC_8A_n41A |
| DC_8A_n39A-n79A | DC_8A_n39A DC_8A_n79A |
| DC_8A-40A_n1A DC_8A-40C_n1A | DC_8A_n1A DC_40A_n1A |
| DC_8A_n40A-n41A | DC_8A_n40A DC_8A_n41A |
| DC_8A-40A_n78A DC_8A-40C_n78A | DC_8A_n78A DC_40A_n78A |
| DC_8A-40A_n78(2A) DC_8A-40C_n78(2A) | DC_8A_n78A DC_40A_n78A |
| DC_8A_n40A-n78A | DC_8A_n40A DC_8A_n78A |
| DC_8A_n40A-n79A | DC_8A_n40A DC_8A_n79A |
| DC_8A-41A_n1A DC_8A-41C_n1A | DC_8A_n1A DC_41A_n1A |
| DC_8A-41A_n3A5 DC_8A-41C_n3A5 | DC_8A_n3A DC_41A_n3A DC_41C_n3A |
| DC_8A-41A_n77A DC_8A-41C_n77A | DC_8A_n77A DC_41A_n77A DC_41C_n77A |
| DC_8A_n41A-n79A5 | DC_8A_n41A DC_8A_n79A |
| DC_8A-42A_n1A5 DC_8A-42C_n1A5 | DC_8A_n1A DC_42A_n1A DC_42C_n1A |
| DC_8A-42A_n3A5 | DC_8A_n3A DC_42A_n3A |
| DC_8A-42C_n3A5 | DC_8A_n3A DC_42A_n3A DC_42C_n3A |
| DC_8A-42A_n28A5 | DC_8A_n28A DC_42A_n28A |
| DC_8A-42C_n28A5 | DC_8A_n28A DC_42A_n28A DC_42C_n28A |
| DC_8A-42A_n77A15,16 DC_8A-42C_n77A15,16 | DC_8A_n77A |
| DC_8A-42A_n77(2A) 15,16 DC_8A-42C_n77(2A) 15,16 | DC_8A_n77A |
| DC_8A_SUL_n41A-n81A | DC_8A_n41A, DC_8A_n81A_ULSUP-TDM_n41A |
| DC_8A_n77A-n79A DC_8A_n77(2A)-n79A | DC_8A_n77A DC_8A_n79A |
| DC_8A_SUL_n78A-n80A | DC_8A_n78A DC_8A_n80A |
| DC_8A_SUL_n78A-n81A5 | DC_8A_n78A, DC_8A_n81A_ULSUP-TDM_n78A |
| DC_8A_SUL_n79A-n81A5 | DC_8A_n79A, DC_8A_n81A_ULSUP-TDM_n79A |
| DC_11A_n1A-n77A5 | DC_11A_n1A DC_11A_n77A |
| DC_11A_n1A-n77(2A)5 | DC_11A_n1A DC_11A_n77A |
| DC_11A_n3A-n28A | DC_11A_n3A DC_11A_n28A |
| DC_11A_n3A-n77A | DC_11A_n3A DC_11A_n77A |
| DC_11A_n3A-n77(2A) | DC_11A_n3A DC_11A_n77A |
| DC_11A_n3A-n79A5 | DC_11A_n3A DC_11A_n79A |
| DC_11A-18A_n3A | DC_11A_n3A DC_18A_n3A |
| DC_11A-18A_n28A | DC_11A_n28A |
| DC_11A-18A_n41A | DC_11A_n41A DC_18A_n41A |
| DC_11A-18A_n77A | DC_11A_n77A DC_18A_n77A |
| DC_11A-18A_n77(2A) | DC_11A_n77A DC_18A_n77A |
| DC_11A-18A_n78A | DC_11A_n78A DC_18A_n78A |
| DC_11A-18A_n78(2A) | DC_11A_n78A DC_18A_n78A |
| DC_11A_n28A-n77A5 | DC_11A_n28A DC_11A_n77A |
| DC_11A_n28A-n77(2A) 5 | DC_11A_n28A DC_11A_n77A |
| DC_11A_n77A-n79A DC_11A_n77(2A)-n79A | DC_11A_n77A DC_11A_n79A |
| DC_12A_n2A-n38A | DC_12A_n2A DC_12A_n38A |
| DC_12A_n2A-n41A | DC_12A_n2A DC_12A_n41A |
| DC_12A_n2A-n78A | DC_12A_n2A DC_12A_n78A |
| DC_12A-(n)5AA | DC_12A_n5A DC_(n)5AA2 |
| DC_12A_n7A-n66A | DC_12A_n7A DC_12A_n66A |
| DC_12A_n7(2A)-n66A | DC_12A_n7A DC_12A_n66A |
| DC_12A_n7A-n78A | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7(2A)-n78A | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7A-n78(2A) | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7(2A)-n78(2A) | DC_12A_n7A DC_12A_n78A |
| DC_12A-30A_n2A | DC_12A_n2A DC_30A_n2A |
| DC_12A-30A_n5A | DC_12A_n5A DC_30A_n5A |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A |
| DC_12A-30A_n77A14 | DC_12A_n77A14 DC_30A_n77A14 |
| DC_12A-30A_n77(2A) | DC_12A_n77A DC_30A_n77A |
| DC_12A-48A_n5A | DC_12A_n5A DC_48A_n5A |
| DC_12A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A_n5A | DC_12A_n5A DC_66A_n5A |
| DC_12A-66A-66A_n5A | DC_12A_n5A DC_66A_n5A |
| DC_12A-66A_n25A | DC_12A_n25A DC_66A_n25A |
| DC_12A-66A_n30A | DC_12A_n30A DC_66A_n30A |
| DC_12A-66A-66A_n30A | DC_12A_n30A DC_66A_n30A |
| DC_12A-66A_n41A | DC_12A_n41A DC_66A_n41A |
| DC_12A-66A_n66A | DC_12A_n66A DC_66A_n66A2 |
| DC_12A-66A_n77A14 DC_12A-66A-66A_n77A14 | DC_12A_n77A14 DC_66A_n77A14 |
| DC_12A-66A_n77(2A) | DC_12A_n77A DC_66A_n77A |
| DC_12A-66A_n78A DC_12A-66A_n78(2A) | DC_12A_n78A DC_66A_n78A |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | DC_12A_n66A DC_12A_n78A |
| DC_13A_n2A-n77A14 DC_13A_n2A-n77C14 | DC_13A_n2A DC_13A_n77A14 |
| DC_13A_n5A-n48A | DC_13A_n48A |
| DC_13A_n5A-n77A14 DC_13A_n5A-n77C14 | DC_13A_n77A14 |
| DC_13A_n7A-n78A | DC_13A_n7A DC_13A_n78A |
| DC_13A_n25A-n66A | DC_13A_n25A DC_13A_n66A |
| DC_13A-46A_n2A3 | DC_13A_n2A |
| DC_13A-46A_n5A | DC_13A_n5A |
| DC_13A-46A_n66A3 | DC_13A_n66A |
| DC_13A-46A_n77A DC_13A-46A-46A_n77A | DC_13A_n77A |
| DC_13A_n48A-n66A | DC_13A_n48A DC_13A_n66A |
| DC_13A-66A_n2A DC_13A-66B_n2A DC_13A-66C_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A_n5A DC_13A-66A-66A_n5A | DC_13A_n5A DC_66A_n5A |
| DC_13A-66A_n48A DC_13A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A_n66A DC_13A-66B_n66A | DC_13A_n66A |
| DC_13A-66A-66A_n66A | DC_13A_n66A |
| DC_13A-66A_n77A14 DC_13A-66A_n77C14 DC_13A-66A-66A_n77C14 | DC_13A_n77A14 DC_66A_n77A14 |
| DC_13A-66A-66A_n77A | DC_13A_n77A14 DC_66A_n77A14 |
| DC_13A_n66A-n77A14 DC_13A_n66A-n77C14 | DC_13A_n66A DC_13A_n77A14 |
| DC_13A-48A_n2A DC_13A-48B_n2A DC_13A-48C_n2A DC_13A-48D_n2A DC_13A-48E_n2A | DC_13A_n2A |
| DC_13A-48A_n66A DC_13A-48B_n66A DC_13A-48C_n66A DC_13A-48D_n66A DC_13A-48E_n66A | DC_13A_n66A |
| DC_13A-48A_n77A14,15,16 DC_13A-48A_n77C14,15,16 DC_13A-48C_n77A14,15,16 DC_13A-48C_n77C14,15,16 DC_13A-48D_n77A14,15,16 DC_13A-48D_n77C14,15,16 DC_13A-48A-48A_n77A14,15,16 | DC_13A_n77A14 |
| DC_14A-30A_n2A | DC_14A_n2A DC_30A_n2A |
| DC_14A-30A_n5A | DC_14A_n5A DC_30A_n5A |
| DC_14A-30A_n66A | DC_14A_n66A DC_30A_n66A |
| DC_14A-30A_n77A14 | DC_14A_n77A14 DC_30A_n77A14 |
| DC_14A-30A_n77(2A) | DC_14A_n77A DC_30A_n77A |
| DC_14A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A_n5A | DC_14A_n5A DC_66A_n5A |
| DC_14A-66A-66A_n5A | DC_14A_n5A DC_66A_n5A |
| DC_14A-66A_n30A DC_14A-66A-66A_n30A | DC_14A_n30A DC_66A_n30A |
| DC_14A-66A_n66A | DC_14A_n66A DC_66A_n66A2 |
| DC_14A-66A_n77A14 DC_14A-66A-66A_n77A14 | DC_14A_n77A14 DC_66A_n77A14 |
| DC_14A-66A_n77(2A) | DC_14A_n77A DC_66A_n77A |
| DC_18A_n3A-n41A | DC_18A_n3A DC_18A_n41A |
| DC_18A_n3A-n77A | DC_18A_n3A DC_18A_n77A |
| DC_18A_n3A-n78A | DC_18A_n3A DC_18A_n78A |
| DC_18A_n28A-n41A | DC_18A_n28A DC_18A_n41A |
| DC_18A-28A_n77A5 | DC_18A_n77A DC_28A_n77A |
| DC_18A_n28A-n77A5 | DC_18A_n28A DC_18A_n77A |
| DC_18A_n28A-n77(2A)5 | DC_18A_n28A DC_18A_n77A |
| DC_18A-28A_n78A5 | DC_18A_n78A DC_28A_n78A |
| DC_18A_n28A-n78A5 | DC_18A_n28A DC_18A_n78A |
| DC_18A_n28A-n78(2A)5 | DC_18A_n28A DC_18A_n78A |
| DC_18A-28A_n79A5 | DC_18A_n79A DC_28A_n79A |
| DC_18A-41A_n3A DC_18A-41C_n3A | DC_18A_n3A DC_41A_n3A DC_41C_n3A |
| DC_18A-41A_n77A DC_18A-41C_n77A | DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_18A-41A_n78A DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_18A_n41A-n77A | DC_18A_n41A DC_18A_n77A |
| DC_18A_n41A-n77(2A) | DC_18A_n41A DC_18A_n77A |
| DC_18A-42A_n77A15,16 DC_18A-42C_n77A15,16 | DC_18A_n77A |
| DC_18A_n41A-n78A | DC_18A_n41A DC_18A_n78A |
| DC_18A_n41A-n78(2A) | DC_18A_n41A DC_18A_n78A |
| DC_18A-42A_n78A15,16 DC_18A-42C_n78A15,16 | DC_18A_n78A |
| DC_18A-42A_n79A DC_18A-42C_n79A | DC_18A_n79A |
| DC_19A-21A_n1A | DC_19A_n1A DC_21A_n1A |
| DC_19A_n1A-n77A5 | DC_19A_n1A DC_19A_n77A |
| DC_19A_n1A-n78A5 | DC_19A_n1A DC_19A_n78A |
| DC_19A_n1A-n79A5 | DC_19A_n1A DC_19A_n79A |
| DC_19A-21A_n77A5 DC_19A-21A_n77C5 | DC_19A_n77A DC_21A_n77A |
| DC_19A-21A_n77(2A)5 | DC_19A_n77A DC_21A_n77A |
| DC_19A-21A_n78A5 DC_19A-21A_n78C5 | DC_19A_n78A DC_21A_n78A |
| DC_19A-21A_n78(2A)5 | DC_19A_n78A DC_21A_n78A |
| DC_19A-21A_n79A5 DC_19A-21A_n79C5 | DC_19A_n79A DC_21A_n79A |
| DC_19A-42A_n1A5,10,12 DC_19A-42C_n1A5,10,12 | DC_19A_n1A DC_42A_n1A |
| DC_19A-42A_n77A15,16 DC_19A-42A_n77C15,16 DC_19A-42C_n77A15,16 DC_19A-42C_n77C15,16 DC_19A-42D_n77A15,16 DC_19A-42D_n77C15,16 | DC_19A_n77A |
| DC_19A-42A_n78A15,16 DC_19A-42A_n78C15,16 DC_19A-42C_n78A15,16 DC_19A-42C_n78C15,16 DC_19A-42D_n78A15,16 DC_19A-42D_n78C15,16 | DC_19A_n78A |
| DC_19A-42A_n79A DC_19A-42A_n79C DC_19A-42C_n79A DC_19A-42C_n79C DC_19A-42D_n79A DC_19A-42D_n79C | DC_19A_n79A |
| DC_19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_20A_n1A-n7A | DC_20A_n1A DC_20A_n7A |
| DC_20A_n1A-n28A16,20 | DC_20A_n1A DC_20A_n28A |
| DC_20A_n1A-n67A | DC_20A_n1A |
| DC_20A_n1A-n75A | DC_20A_n1A |
| DC_20A_n1A-n78A | DC_20A_n1A DC_20A_n78A |
| DC_20A_n3A-n38A | DC_20A_n3A DC_20A_n38A |
| DC_20A_n3A-n67A | DC_20A_n3A |
| DC_20A_n3A-n78A | DC_20A_n3A DC_20A_n78A |
| DC_20A_n7A-n28A, 16, 20 | DC_20A_n7A DC_20A_n28A |
| DC_20A_n7A-n78A | DC_20A_n7A DC_20A_n78A |
| DC_20A_n8A-n75A6 | DC_20A_n8A |
| DC_20A_n8A-n78A | DC_20A_n78A DC_20A_n8A |
| DC_20A-28A_n1A | DC_20A_n1A DC_28A_n1A |
| DC_20A-28A_n3A | DC_20A_n3A DC_28A_n3A |
| DC_20A-28A_n78A | DC_20A_n78A DC_28A_n78A |
| DC_20A_n28A-n75A6,16,20 | DC_20A_n28A |
| DC_20A_n28A-n78A5,6,16,20 | DC_20A_n28A DC_20A_n78A |
| DC_20A-32A_n1A | DC_20A_n1A |
| DC_20A-32A_n3A | DC_20A_n3A |
| DC_20A-32A_n8A | DC_20A_n8A |
| DC_20A-32A_n28A16,20 | DC_20A_n28A |
| DC_20A-32A_n78A DC_20A-32A_n78C | DC_20A_n78A |
| DC_20A-32A_n78(2A) | DC_20A_n78A |
| DC_20A-38A_n1A | DC_20A_n1A DC_38A_n1A |
| DC_20A-38A_n3A | DC_20A_n3A |
| DC_20A-38A_n8A | DC_38A_n8A |
| DC_20A-(n)38AA | DC_20A_n38A |
| DC_20A-38A_n78A | DC_20A_n78A DC_38A_n78A |
| DC_20A-38A_n78(2A) | DC_20A_n78A |
| DC_20A_n38A-n78A | DC_20A_n38A DC_20A_n78A |
| DC_20A-40A_n1A DC_20A-40C_n1A | DC_20A_n1A DC_40A_n1A |
| DC_20A-40A_n78A DC_20A-40C_n78A | DC_20A_n78A DC_40A_n78A |
| DC_20A-40A_n78(2A) DC_20A-40C_n78(2A) | DC_20A_n78A DC_40A_n78A |
| DC_20A-41A_n41A DC_20A-41C_n41A | DC_41A_n41A |
| DC_20A_n41A-n78A | DC_20A_n41A DC_20A_n78A |
| DC_20A-(n)41AA DC_20A-(n)41CA DC_20A-(n)41DA | DC_20A_n41A |
| DC_20A_n75A-n78A5 | DC_20A_n78A |
| DC_20A_n76A-n78A5 | DC_20A_n78A |
| DC_20A_SUL_n78A-n80A | DC_20A_n78A DC_20A_n80A |
| DC_20A_SUL_n78A-n82A5 | DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A |
| DC_20A_SUL_n78A-n83A5 | DC_20A_n78A DC_20A_n83A |
| DC_20A_n78A-n92A DC_20A_n78(2A)-n92A | DC_20A_n78A DC_20A_n92A_ULSUP-TDM_n78A |
| DC_21A_n1A-n77A5 | DC_21A_n1A DC_21A_n77A |
| DC_21A_n1A-n78A5 | DC_21A_n1A DC_21A_n78A |
| DC_21A_n1A-n79A5 | DC_21A_n1A DC_21A_n79A |
| DC_21A-28A_n77A5 DC_21A-28A_n77C | DC_21A_n77A DC_28A_n77A |
| DC_21A_n28A-n77A5,13 | DC_21A_n28A DC_21A_n77A |
| DC_21A-28A_n78A5 DC_21A-28A_n78C | DC_21A_n78A DC_28A_n78A |
| DC_21A_n28A-n78A5,13 | DC_21A_n28A DC_21A_n78A |
| DC_21A-28A_n79A5 DC_21A-28A_n79C | DC_21A_n79A DC_28A_n79A |
| DC_21A_n28A-n79A5,13 | DC_21A_n28A DC_21A_n79A |
| DC_21A-42A_n1A510,12 DC_21A-42C_n1A510,12 | DC_21A_n1A DC_42A_n1A |
| DC_21A-42A_n77A15,16 DC_21A-42A_n77C15,16 DC_21A-42C_n77A15,16 DC_21A-42C_n77C15,16 DC_21A-42D_n77A15,16 DC_21A-42D_n77C15,16 DC_21A-42E_n77A15,16 DC_21A-42E_n77C15,16 | DC_21A_n77A |
| DC_21A-42A_n78A15,16 DC_21A-42A_n78C15,16 DC_21A-42C_n78A15,16 DC_21A-42C_n78C15,16 DC_21A-42D_n78A15,16 DC_21A-42D_n78C15,16 DC_21A-42E_n78A15,16 DC_21A-42E_n78C15,16 | DC_21A_n78A |
| DC_21A-42A_n79A DC_21A-42A_n79C DC_21A-42C_n79A DC_21A-42C_n79C DC_21A-42D_n79A DC_21A-42D_n79C DC_21A-42E_n79A DC_21A-42E_n79C | DC_21A_n79A |
| DC_28A-32A_n1A | DC_28A_n1A |
| DC_28A-32A_n3A | DC_28A_n3A |
| DC_28A-38A_n1A | DC_28A_n1A DC_38A_n1A |
| DC_28A-66A_n7A | DC_28A_n7A DC_66A_n7A |
| DC_28A-66A_n66A | DC_28A_n66A DC_66A_n66A2 |
| DC_21A_n77A-n79A | DC_21A_n77A DC_21A_n79A |
| DC_21A_n78A-n79A | DC_21A_n78A DC_21A_n79A |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A | DC_25A_n41A DC_41A_n41A |
| DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | DC_25A_n41A DC_41A_n41A |
| DC_25A-(n)41AA | DC_25A_n41A DC_(n)41AA |
| DC_25A-25A-(n)41AA | DC_25A_n41A DC_(n)41AA |
| DC_25A-(n)41CA DC_25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |
| DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |
| DC_25A-66A_n77A | DC_25A_n77A DC_66A_n77A |
| DC_25A-25A-66A_n77A | DC_25A_n77A DC_66A_n77A |
| DC_25A-66A_n78A | DC_25A_n78A DC_66A_n78A |
| DC_25A-25A-66A_n78A | DC_25A_n78A DC_66A_n78A |
| DC_28A-40A_n78A DC_28A-40C_n78A | DC_28A_n78A DC_40A_n78A |
| DC_28A-41A_n77A DC_28A-41C_n77A | DC_28A_n77A DC_41A_n77A |
| DC_28A-41A_n78A DC_28A-41C_n78A | DC_28A_n78A DC_41A_n78A |
| DC_28A-41A_n79A5 DC_28A-41C_n79A5 | DC_28A_n79A DC_41A_n79A |
| DC_28A_n1A-n40A | DC_28A_n1A DC_28A_n40A |
| DC_28A_n1A-n78A5 | DC_28A_n1A DC_28A_n78A |
| DC_28A_n3A-n77A5 | DC_28A_n3A DC_28A_n77A |
| DC_28A_n3A-n78A5 | DC_28A_n3A DC_28A_n78A |
| DC_28A_n5A-n78A5 | DC_28A_n5A DC_28A_n78A |
| DC_28A_n7A-n78A | DC_28A_n7A DC_28A_n78A |
| DC_28A_n7B-n78A | DC_28A_n7A DC_28A_n7B DC_28A_n78A |
| DC_28A_n8A-n78A5 | DC_28A_n8A DC_28A_n78A |
| DC_28A_n40A-n78A | DC_28A_n40A DC_28A_n78A |
| DC_28A_SUL_n41A-n83A5 | DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41A |
| DC_28A-42A_n77A15,16 DC_28A-42A_n77C15,16 DC_28A-42C_n77A15,16 DC_28A-42C_n77C15,16 | DC_28A_n77A |
| DC_28A-42A_n78A15,16 DC_28A-42A_n78C15,16 DC_28A-42C_n78A15,16 DC_28A-42C_n78C15,16 | DC_28A_n78A |
| DC_28A-42A_n79A DC_28A-42A_n79C DC_28A-42C_n79A DC_28A-42C_n79C | DC_28A_n79A |
| DC_28A_SUL_n78A-n83A5 | DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A |
| DC_29A-30A_n2A | DC_30A_n2A |
| DC_29A-30A_n66A | DC_30A_n66A |
| DC_29A-30A_n77A14 | DC_30A_n77A14 |
| DC_29A-66A_n2A | DC_66A_n2A |
| DC_29A-66A-66A_n2A | DC_66A_n2A |
| DC_29A-66A_n30A | DC_66A_n30A |
| DC_29A-66A-66A_n30A | DC_66A_n30A |
| DC_29A-66A_n77A14 DC_29A-66A-66A_n77A14 | DC_66A_n77A14 |
| DC_29A-66A_n78A | DC_66A_n78A |
| DC_30A-(n)5AA | DC_30A_n5A DC_(n)5AA2 |
| DC_30A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A_n66A | DC_30A_n66A DC_66A_n66A2 |
| DC_30A-66A_n77A14 DC_30A-66A-66A_n77A14 | DC_30A_n77A14 DC_66A_n77A14 |
| DC_30A-66A_n77(2A) | DC_30A_n77A DC_66A_n77A |
| DC_32A-38A_n1A | DC_38A_n1A |
| DC_32A-38A_n28A | DC_38A_n28A |
| DC_38A_n3A-n78A | DC_38A_n3A DC_38A_n78A |
| DC_38A_n28A-n78A | DC_38A_n28A DC_38A_n78A |
| DC_39A_n40A-n41A | DC_39A_n40A DC_39A_n41A |
| DC_39A_n40A-n79A | DC_39A_n40A DC_39A_n79A |
| DC_39A_n41A-n79A | DC_39A_n41A DC_39A_n79A |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | DC_40A_n1A DC_40A_n78A |
| DC_40A_n41A-n79A | DC_40A_n41A DC_40A_n79A |
| DC_40A-42A_n77A DC_40A-42A_n77C | DC_40A_n77A |
| DC_40A-42A_n78A | DC_40A_n78A |
| DC_41A_n1A-n3A | DC_41A_n1A DC_41A_n3A |
| DC_41C_n1A-n3A | DC_41A_n1A DC_41A_n3A |
| DC_41A_n1A-n77A | DC_41A_n1A DC_41A_n77A |
| DC_41C_n1A-n77A | DC_41A_n1A DC_41A_n77A DC_41C_n77A |
| DC_41A_n1A-n78A | DC_41A_n1A DC_41A_n78A |
| DC_41C_n1A-n78A | DC_41A_n1A DC_41A_n78A |
| DC_41A_n3A-n41A | DC_41A_n3A DC_41A_n41A |
| DC_41A_n3A-n77A | DC_41A_n3A DC_41A_n77A |
| DC_41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_41A_n3A-n78A | DC_41A_n3A DC_41A_n78A |
| DC_41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_41A_n28A-n41A | DC_41A_n28A |
| DC_41A_n28A-n77A | DC_41A_n28A DC_41A_n77A |
| DC_41C_n28A-n77A | DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_41A_n28A-n78A | DC_41A_n28A DC_41A_n78A |
| DC_41C_n28A-n78A | DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_(n)41AA-n78A DC_(n)41CA-n78A DC_(n)41DA-n78A | DC_41A_n78A |
| DC_41A_n41A-n77A | DC_41A_n77A |
| DC_41A_n41A-n78A DC_41C_n41A-n78A | DC_41A_n78A |
| DC_41A-42A_n77A15,16 DC_41A-42C_n77A15,16 DC_41C-42A_n77A15,16 DC_41C-42C_n77A15,16 | DC_41A_n77A |
| DC_41A-42A_n77(2A)15,16 DC_41A-42C_n77(2A)15,16 | DC_41A_n77A |
| DC_41A-42A_n78A15,16 DC_41A-42C_n78A15,16 DC_41C-42A_n78A15,16 DC_41C-42C_n78A15,16 | DC_41A_n78A |
| DC_41A-42A_n79A DC_41A-42C_n79A DC_41C-42A_n79A DC_41C-42C_n79A | DC_41A_n79A |
| DC_42A_n1A-n3A5 | DC_42A_n1A DC_42A_n3A |
| DC_42C_n1A-n3A5, | DC_42A_n1A DC_42A_n3A DC_42C_n1A DC_42C_n3A |
| DC_42A_n1A-n77A15,16 | DC_42A_n1A |
| DC_42C_n1A-n77A15,16 | DC_42A_n1A DC_42C_n1A |
| DC_42A_n1A-n78A15,16 DC_42C_n1A-n78A15,16 | N/A |
| DC_42A_n1A-n79A DC_42C_n1A-n79A | N/A |
| DC_42A_n3A-n28A | DC_42A_n3A DC_42A_n28A |
| DC_42C_n3A-n28A | DC_42A_n3A DC_42A_n28A DC_42C_n28A |
| DC_42A_n3A-n77A15,16 | DC_42A_n3A |
| DC_42A_n3A-n77(2A)15,16 | DC_42A_n3A |
| DC_42C_n3A-n77A15,16 | DC_42A_n3A DC_42C_n3A |
| DC_42C_n3A-n77(2A)15,16 | DC_42A_n3A DC_42C_n3A |
| DC_42A_n28A-n77A15,16 | DC_42A_n28A |
| DC_42A_n28A-n77(2A)15,16 | DC_42A_n28A |
| DC_42C_n28A-n77A15,16 | DC_42A_n28A DC_42C_n28A |
| DC_42C_n28A-n77(2A)15,16 | DC_42A_n28A DC_42C_n28A |
| DC_46A-48A_n2A3 DC_46C-48A_n2A3 DC_46D-48A_n2A3 DC_46E-48A_n2A3 | DC_48A_n2A |
| DC_46A-48A_n5A3 DC_46C-48A_n5A3 DC_46D-48A_n5A3 DC_46E-48A_n5A3 | DC_48A_n5A |
| DC_46A-48A_n66A3 DC_46C-48A_n66A3 DC_46D-48A_n66A3 DC_46E-48A_n66A3 | DC_48A_n66A |
| DC_46A-66A_n5A DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A DC_46A-66A-66A_n5A DC_46C-66A-66A_n5A DC_46D-66A-66A_n5A | DC_66A_n5A |
| DC_46A-66A_n25A DC_46C-66A_n25A DC_46D-66A_n25A | DC_66A_n25A |
| DC_46A-66A_n41A DC_46C-66A_n41A DC_46D-66A_n41A | DC_66A_n41A |
| DC_46A-66A_n41(2A) DC_46C-66A_n41(2A) DC_46D-66A_n41(2A) | DC_66A_n41A |
| DC_46A-66A_n71A DC_46C-66A_n71A DC_46D-66A_n71A | DC_66A_n71A |
| DC_46A-66A_n77A DC_46A-46A-66A_n77A | DC_66A_n77A |
| DC_48A-(n)5AA | DC_48A_n5A DC_(n)5AA2 |
| DC_48A-(n)12AA | DC_48A_n12A DC_(n)12AA2 |
| DC_48A_n25A-n48A | DC_48A_n25A |
| DC_48A_n48A-n66A | DC_48A_n66A |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | DC_66A_n2A DC_48A_n2A |
| DC_48A-66A_n5A DC_48B-66A_n5A DC_48C-66A_n5A DC_48D-66A_n5A DC_48E-66A_n5A | DC_66A_n5A |
| DC_48A-66A_n12A | DC_48A_n12A DC_66A_n12A |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | DC_48A_n25A DC_66A_n25A |
| DC_48A-66A_n48A | DC_66A_n48A |
| DC_48A-66A_n66A DC_48C-66A_n66A DC_48D-66A_n66A DC_48E-66A_n66A | DC_66A_n66A2 DC_48A_n66A |
| DC_48A-66A_n71A | DC_48A_n71A DC_66A_n71A |
| DC_48A-66A_n77A14,15,16 DC_48A-66A_n77C14,15,16 DC_48C-66A_n77A14,15,16 DC_48C-66A_n77C14,15,16 DC_48D-66A_n77A14,15,16 DC_48D-66A_n77C14,15,16 DC_48E-66A_n77A14,15,16 | DC_66A_n77A14 |
| DC_48A-48A-66A_n77A14,15,16 | DC_66A_n77A |
| DC_66A-(n)5AA | DC_66A_n5A DC_(n)5AA2 |
| DC_66A-66A-(n)5AA | DC_66A_n5A DC_(n)5AA2 |
| DC_66A_n2A-n38A | DC_66A_n2A DC_66A_n38A |
| DC_66A_n2A-n66A | DC_66A_n2A |
| DC_66A_n2A-n71A | DC_66A_n2A DC_66A_n71A |
| DC_66A_n2A-n77A14 DC_66A_n2A-n77C14 DC_66A-66A_n2A-n77C14 | DC_66A_n2A DC_66A_n77A14 |
| DC_66A-66A_n2A-n77A14 | DC_66A_n2A DC_66A_n77A14 |
| DC_66A_n2A-n78A | DC_66A_n2A DC_66A_n78A |
| DC_66A_n5A-n48A | DC_66A_n5A DC_66A_n48A |
| DC_66A_n5A-n77A14 DC_66A_n5A-n77C14 DC_66A-66A_n5A-n77C14 | DC_66A_n5A DC_66A_n77A14 |
| DC_66A-66A_n5A-n77A14 | DC_66A_n5A DC_66A_n77A14 |
| DC_66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7(2A)-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7(2A)-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7A-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7A-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7(2A)-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7(2A)-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |
| DC_66A_n38A-n66A | DC_66A_n38A DC_66A_n66A2 |
| DC_66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_66A_n66A-n77A14 DC_66A_n66A-n77C14 | DC_66A_n77A14 |
| DC_66A_n66A-n78A | DC_66A_n66A2 DC_66A_n78A |
| DC_66A-(n)12AA | DC_66A_n12A DC_(n)12AA2 |
| DC_66A-(n)71AA DC_66C-(n)71AA | DC_66A_n71A DC_(n)71AA |
| DC_66A_n25A-n41A DC_66A_n25A-n41C | DC_66A_n25A DC_66A_n41A |
| DC_66A_n25A-n41(2A) | DC_66A_n25A DC_66A_n41A |
| DC_66A_n25A-n48A | DC_66A_n25A DC_66A_n48A |
| DC_66A_n25A-n66A | DC_66A_n25A DC_66A_n66A2 |
| DC_66A_n38A-n71A | DC_66A_n38A DC_66A_n71A |
| DC_66A_n41A-n71A DC_66A_n41C-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A_n66A-n71A | DC_66A_n66A DC_66A_n71A |
| DC_66A-71A_n2A | DC_71A_n2A DC_66A_n2A |
| DC_66A-71A_n38A | DC_71A_n38A DC_66A_n38A |
| DC_66A-71A_n41A | DC_66A_n41A DC_71A_n41A |
| DC_66A-71A_n66A | DC_71A_n66A DC_66A_n66A2 |
| DC_66A-71A_n71A | DC_66A_n71A |
| DC_66A-71A_n78A DC_66A-71A_n78(2A) | DC_71A_n78A DC_66A_n78A |
| DC_66A_n71A-n78A | DC_66A_n71A DC_66A_n78A |
| DC_66A_SUL_n78A-n86A5 | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |
| DC_66A_SUL_n78(2A)-n86A5 | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |
| DC_71A_n2A-n41A | DC_71A_n2A DC_71A_n41A |
| DC_71A_n2A-n66A | DC_71A_n2A DC_71A_n66A |
| DC_71A_n2A-n78A | DC_71A_n2A DC_71A_n78A |
| DC_71A_n38A-n66A | DC_71A_n38A DC_71A_n66A |
| DC_71A_n38A-n78A | DC_71A_n38A DC_71A_n78A |
| DC_71A_n66A-n78A | DC_71A_n66A DC_71A_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 4: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 6: N/A NOTE 7: Void. NOTE 8: UL carrier shall be supported in Band 2 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 9: UL carrier shall be supported in Band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 10: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE 11: The frequency range in band 3 is restricted for this band combination to 1765 - 1785 MHz for the UL and 1860-1880 MHz for the DL. NOTE 12: The frequency range in band 42 is restricted for this band combination to 3440 - 3520 MHz. NOTE 13: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 17: Void. NOTE 18: Void. NOTE 19: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 20: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 21: The downlink DC_2_n2 RESSENS requirements only apply when the band n2 downlink carrier is configured closer to the uplink operating band than the E-UTRA Band 2 downlink carrier. |  |

5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A |
| DC_1A-3A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A |
| DC_1A-3A_n3A-n78A2 | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A |
| DC_1A-3A-5A_n77A DC_1A-3A-5A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A |
| DC_1A-3A-5A_n78A2 DC_1A-3A-5A_n78C2 DC_1A-3C-5A_n78A DC_1A-1A-3A-5A_n78A DC_1A-1A-3C-5A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A-5A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A_n5A-n78A2 DC_1A-3C_n5A-n78A2 | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-5A_n79A2 | DC_1A_n79A DC_3A_n79A DC_5A_n79A |
| DC_1A-3A-7A_n3A DC_1A-3A-7C_n3A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A |
| DC_1A-3A-7A_n5A DC_1A-3A-7C_n5A DC_1A-3C-7A_n5A DC_1A-3C-7C_n5A | DC_1A_n5A DC_3A_n5A DC_7A_n5A DC_7C_n5A |
| DC_1A-3A-7A_n7A DC_1A-3C-7A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 |
| DC_1A-1A-3A-7A_n7A DC_1A-1A-3C-7A_n7A DC_1A-3A-3A-7A_n7A DC_1A-1A-3A-3A-7A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 |
| DC_1A-3A-7A_n8A | DC_1A_n8A DC_3A_n8A DC_7A_n8A |
| DC_1A-3A-7A_n28A DC_1A-3A-7C_n28A DC_1A-3C-7A_n28A DC_1A-3C-7C_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_7C_n28A |
| DC_1A-1A-3A-7A_n28A DC_1A-1A-3C-7A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A |
| DC_1A-3A-7A_n38A12,13 | CA_1A-3A |
| DC_1A-3A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A |
| DC_1A-3A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A_n78A2 DC_1A-3A-7C_n78A DC_1A-3C-7A_n78A2 DC_1A-3C-7C_n78A DC_1A-3A-7A_n78C2 | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n78(2A) DC_1A-3C-7A_n78(2A) DC_1A-3A-7C_n78(2A) DC_1A-3C-7C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-1A-3A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A_n7A-n78A DC_1A-3A_n7B-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A |
| DC_1A-3A_n7A-n78(2A) DC_1A-3C_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_1A-3C_n7A-n78A DC_1A-3C_n7B-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_1A-3A-7A-7A_n78A2 DC_1A-1A-3C-7A_n78A DC_1A-3A-7A-7A_n78C2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-8A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A |
| DC_1A-3A-8A_n77A2 DC_1A-3C-8A_n77A | DC_1A_n77A DC_3A_n77A DC_3C_n77A DC_8A_n77A |
| DC_1A-3A-8A_n77(2A)2 DC_1A-3C-8A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_3C_n77A DC_8A_n77A |
| DC_1A-3A-8A_n77(3A)2 | DC_1A_n77A DC_3A_n77A DC_8A_n77A |
| DC_1A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A |
| DC_1A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A |
| DC_1A-3A-8A_n78A2 DC_1A-3C-8A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A |
| DC_1A-3A-8A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_8A_n78A |
| DC_1A-3A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A |
| DC_1A-3A-8A_n79A2 | DC_1A_n79A DC_3A_n79A DC_8A_n79A |
| DC_1A-3A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_11A_n28A |
| DC_1A-3A-11A_n77A2 | DC_1A_n77A DC_3A_n77A DC_11A_n77A |
| DC_1A-3A-11A_n77(2A) 2 DC_1A-3A-11A_n77(3A)2 | DC_1A_n77A DC_3A_n77A DC_11A_n77A |
| DC_1A-3A-18A_n3A | DC_1A_n3A DC_3A_n3A4 DC_18A_n3A |
| DC_1A-3A-18A_n28A | DC_1A_n28A DC_3A_n28A DC_18A_n28A |
| DC_1A-3A-18A_n41A | DC_1A_n41A DC_3A_n41A DC_18A_n41A |
| DC_1A-3A-18A_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DC_1A-3A-19A_n77A2 DC_1A-3A-19A_n77C2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A_n77(2A)2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A_n78A2 DC_1A-3A-19A_n78C2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A_n79A2 DC_1A-3A-19A_n79C2 | DC_1A_n79A DC_3A_n79A DC_19A_n79A |
| DC_1A-3A-20A_n7A | DC_1A_n7A DC_3A_n7A DC_20A_n7A |
| DC_1A-3A-20A_n8A | DC_1A_n8A DC_3A_n8A DC_20A_n8A |
| DC_1A-3A-20A_n28A3,8,14 DC_1A-3C-20A_n28A3,8,14 | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3A-20A_n38A | DC_3A_n38A DC_20A_n38A |
| DC_1A-3A-20A_n41A DC_1A-3C-20A_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A DC_20A_n41A |
| DC_1A-3A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-21A_n77A2 DC_1A-3A-21A_n77C2 | DC_1A_n77A DC_3A_n77A DC_21A_n77A |
| DC_1A-3A-21A_n77(2A)2 | DC_1A_n77A DC_3A_n77A DC_21A_n77A |
| DC_1A-3A-21A_n78A2 DC_1A-3A-21A_n78C2 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A_n79A2 DC_1A-3A-21A_n79C2 | DC_1A_n79A DC_3A_n79A DC_21A_n79A |
| DC_1A-3A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A |
| DC_1A-3C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-28A_n3A | DC_1A_n3A DC_3A_n3A4 DC_28A_n3A |
| DC_1A-3A-28A_n5A DC_1A-3C-28A_n5A | DC_1A_n5A DC_3A_n5A DC_28A_n5A |
| DC_1A-3A-28A_n7A DC_1A-3C-28A_n7A DC_1A-3A-28A_n7B DC_1A-3C-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-3A-3A-28A_n7A DC_1A-3A-3A-28A_n7B | DC_1A_n7A DC_3A_n7A DC_28A_n7A |
| DC_1A-1A-3A-28A_n7A DC_1A-1A-3C-28A_n7A DC_1A-1A-3A-28A_n7B DC_1A-1A-3C-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-1A-3A-3A-28A_n7A DC_1A-1A-3A-3A-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-3A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_28A_n40A |
| DC_1A-3A_n28A-n41A2 | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A |
| DC_1A-3A_n28A-n75A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3C_n28A-n75A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3A-28A_n77A2 DC_1A-3A-28A_n77C2 | DC_1A_n77A DC_3A_n77A DC_28A_n77A |
| DC_1A-3A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |
| DC_1A-3A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |
| DC_1A-3A-28A_n78A2 DC_1A-3C-28A_n78A2 DC_1A-3A-28A_n78C2 DC_1A-1A-3A-28A_n78A DC_1A-1A-3C-28A_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n79A2 DC_1A-3A-28A_n79C2 | DC_1A_n79A DC_3A_n79A DC_28A_n79A |
| DC_1A-3A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_3A_n28A DC_3A_n79A |
| DC_1A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A |
| DC_1A-3A_n28A-n78A2 DC_1A-3C_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A_n28A-n78(2A)2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-32A_n28A DC_1A-3C-32A_n28A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3A-32A_n78A DC_1A-3A-32A_n78C | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-32A_n78(2A) | DC_1A_n78A DC_3A_n78A |
| DC_1A-3C-32A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-38A_n28A DC_1A-3C-38A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_38A_n28A |
| DC_1A-3A-38A_n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-38A_n78(2A) | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A_n38A-n78A | DC_1A_n38A DC_1A_n78A DC_3A_n38A DC_3A_n78A |
| DC_1A-3A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A |
| DC_1A-3A-40A_n78A DC_1A-3A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_40A_n78A |
| DC_1A-3A-40A_n78(2A) DC_1A-3A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_40A_n78A |
| DC_1A-3A-41A_n3A DC_1A-3A-41C_n3A | DC_1A_n3A DC_3A_n3A4 DC_41A_n3A DC_41C_n3A |
| DC_1A-3A-41A_n28A2 DC_1A-3A-41C_n28A2 | DC_1A_n28A DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-3A-41A_n41A | DC_1A_n41A DC_3A_n41A |
| DC_1A-3A-(n)41AA | DC_1A_n41A DC_3A_n41A |
| DC_1A-3A-41A_n77A DC_1A-3A-41C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-3A-41A_n77(2A) DC_1A-3A-41C_n77(2A) | DC_1A_n77A DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-3A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A |
| DC_1A-3A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A |
| DC_1A-3A-41A_n78A DC_1A-3A-41C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-3A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A |
| DC_1A-3A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A |
| DC_1A-3A-41A_n78(2A) DC_1A-3A-41C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-3A-41A_n79A2 DC_1A-3A-41C_n79A2 | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-42A_n28A2 DC_1A-3A-42C_n28A2 | DC_1A_n28A DC_3A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-3A-42A_n77A7,8 DC_1A-3A-42A_n77C7,8 DC_1A-3A-42C_n77A7,8 DC_1A-3A-42C_n77C7,8 DC_1A-3A-42D_n77A7,8 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A-42A_n77(2A) 7,8 DC_1A-3A-42C_n77(2A) 7,8 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A-42A_n78A7,8 DC_1A-3A-42A_n78C7,8 DC_1A-3A-42C_n78A7,8 DC_1A-3A-42C_n78C7,8 DC_1A-3A-42D_n78A7,8 | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-42A_n79A DC_1A-3A-42A_n79C DC_1A-3A-42C_n79A DC_1A-3A-42C_n79C DC_1A-3A-42D_n79A | DC_1A_n79A DC_3A_n79A |
| DC_1A-3A_n75A-n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_3A_n77A DC_3A_n79A |
| DC_1A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A |
| DC_1A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A |
| DC_1A-3A_n78A-n79A | DC_1A_n78A DC_1A_n79A DC_3A_n78A DC_3A_n79A |
| DC_1A-3A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |
| DC_1A-5A-7A_n77A | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A_n77(2A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A-7A_n77A | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A-7A_n77(2A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A_n78A DC_1A-5A-7A_n78C DC_1A-1A-5A-7A_n78A | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A_n78(2A) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78A DC_1A-5A-7A-7A_n78C | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78(2A) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-41A_n79A | DC_1A_n79A DC_5A_n79A DC_41A_n79A |
| DC_1A-7A_n3A-n38A | DC_1A_n3A |
| DC_1A-7A_n3A-n78A DC_1A-7C_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_7A_n3A DC_7C_n3A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n5A-n78A DC_1A-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A |
| DC_1A-7A_n38A-n78A | DC_1A_n78A |
| DC_1A-7A-8A_n3A | DC_1A_n3A DC_7A_n3A DC_8A_n3A |
| DC_1A-7A-8A_n28A | DC_1A_n28A DC_7A_n28A DC_8A_n28A |
| DC_1A-7A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-8A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A-8A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_7A_n8A DC_7A_n78A |
| DC_1A-7A-20A_n3A DC_1A-7C-20A_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_20A_n3A |
| DC_1A-7A-20A_n8A | DC_1A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-7A-20A_n28A3,8,14 | DC_1A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-7A-20A_n38A12,13 | CA_1A-20A |
| DC_1A-7A-20A_n78A2 | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-20A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A-28A_n3A DC_1A-7C-28A_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_1A-7A-28A_n5A DC_1A-7C-28A_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-7A-28A_n40A | DC_1A_n40A DC_7A_n40A DC_28A_n40A |
| DC_1A-7A-28A_n78A DC_1A-7C-28A_n78A | DC_1A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-1A-7A-28A_n78A | DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A_n28A-n78A2 DC_1A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_1A-7A-32A_n3A DC_1A-7C-32A_n3A | DC_1A_n3A DC_7A_n3A |
| DC_1A-7A-32A_n8A | DC_1A_n8A DC_7A_n8A |
| DC_1A-7A-32A_n28A | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A-32A_n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-38A_n3A | DC_1A_n3A |
| DC_1A-7A-38A_n8A | DC_1A_n8A |
| DC_1A-7A-38A_n28A10 | DC_1A_n28A |
| DC_1A-7A-38A_n78A10 | DC_1A_n78A |
| DC_1A-7A-40A_n78A DC_1A-7A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-7A-40A_n78(2A) DC_1A-7A-40C_n78(2A) | DC_1A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-7A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-7A_n75A-n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-8A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A |
| DC_1A-8A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A |
| DC_1A-8A_n3A-n77(2A) 2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A |
| DC_1A-8A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_8A_n3A DC_8A_n79A |
| DC_1A-8A-11A_n3A | DC_1A_n3A DC_8A_n3A DC_11A_n3A |
| DC_1A-8A-11A_n28A | DC_1A_n28A DC_8A_n28A DC_11A_n28A |
| DC_1A-8A-11A_n77A2 | DC_1A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-8A-11A_n77(2A)2 DC_1A-8A-11A_n77(3A)2 | DC_1A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-8A-11A_n78A2 | DC_1A_n78A DC_8A_n78A DC_11A_n78A |
| DC_1A-8A-11A_n79A2 | DC_1A_n79A DC_8A_n79A DC_11A_n79A |
| DC_1A-8A-20A_n3A | DC_1A_n3A DC_8A_n3A DC_20A_n3A |
| DC_1A-8A-20A_n28A3,8,11,14 | DC_1A_n28A DC_8A_n28A DC_20A_n28A |
| DC_1A-8A-20A_n78A | DC_1A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-8A-28A_n3A | DC_1A_n3A DC_8A_n3A DC_28A_n3A |
| DC_1A-8A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n28A-n77(2A)2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A-28A_n78A | DC_1A_n78A DC_8A_n78A DC_28A_n78A |
| DC_1A-8A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-8A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_8A_n28A DC_8A_n79A |
| DC_1A-8A-32A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A-32A_n78A | DC_1A_n78A DC_8A_n78A |
| DC_1A-8A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_8A_n40A DC_8A_n78A |
| DC_1A-8A-40A_n78A DC_1A-8A-40C_n78A | DC_1A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-8A-40A_n78(2A) DC_1A-8A-40C_n78(2A) | DC_1A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-8A-42A_n3A2 DC_1A-8A-42C_n3A2 | DC_1A_n3A DC_8A_n3A DC_42A_n3A DC_42C_n3A |
| DC_1A-8A-42A_n28A2 DC_1A-8A-42C_n28A2 | DC_1A_n28A DC_8A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42A_n77A7,8 DC_1A-8A-42C_n77A7,8 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A-42A_n77(2A) 7,8 DC_1A-8A-42C_n77(2A) 7,8 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n77A-n79A DC_1A-8A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A |
| DC_1A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_11A_n3A DC_11A_n28A |
| DC_1A-11A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-11A_n3A-n77(2A) 2 | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-11A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_11A_n3A DC_11A_n79A |
| DC_1A-11A-18A_n3A | DC_1A_n3A DC_11A_n3A DC_18A_n3A |
| DC_1A-11A-18A_n28A | DC_1A_n28A DC_11A_n28A DC_18A_n28A |
| DC_1A-11A-18A_n41A | DC_1A_n41A DC_11A_n41A DC_18A_n41A |
| DC_1A-11A-18A_n77A | DC_1A_n77A DC_11A_n77A DC_18A_n77A |
| DC_1A-11A-18A_n77(2A) | DC_1A_n77A DC_11A_n77A DC_18A_n77A |
| DC_1A-11A-18A_n78A | DC_1A_n78A DC_11A_n78A DC_18A_n78A |
| DC_1A-11A-18A_n78(2A) | DC_1A_n78A DC_11A_n78A DC_18A_n78A |
| DC_1A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-11A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_18A_n3A DC_18A_n41A |
| DC_1A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_18A_n3A DC_18A_n77A |
| DC_1A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_18A_n3A DC_18A_n78A |
| DC_1A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_18A_n28A DC_18A_n41A |
| DC_1A-18A-28A_n77A | DC_1A_n77A DC_18A_n77A DC_28A_n77A |
| DC_1A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-18A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-18A-28A_n78A | DC_1A_n78A DC_18A_n78A DC_28A_n78A |
| DC_1A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-18A_n28A-n78(2A) | DC_1A_n28A DC_1A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-18A-28A_n79A2 | DC_1A_n79A DC_18A_n79A DC_28A_n79A |
| DC_1A-18A-41A_n3A DC_1A-18A-41C_n3A | DC_1A_n3A DC_18A_n3A DC_41A_n3A DC_41C_n3A |
| DC_1A-18A-41A_n77A DC_1A-18A-41C_n77A | DC_1A_n77A DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-18A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-18A-41A_n78A DC_1A-18A-41C_n78A | DC_1A_n78A DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-18A_n41A-n78A | DC_1A_n41A DC_18A_n41A DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n41A-n78(2A) | DC_1A_n41A DC_18A_n41A DC_1A_n78A DC_18A_n78A |
| DC_1A-18A-42A_n77A7,8 DC_1A-18A-42C_n77A7,8 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A-42A_n78A7,8 DC_1A-18A-42C_n78A7,8 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A-42A_n79A DC_1A-18A-42C_n79A | DC_1A_n79A DC_18A_n79A |
| DC_1A-19A-21A_n77A2 DC_1A-19A-21A_n77C2 | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A_n77(2A) 2 | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A_n78A2 DC_1A-19A-21A_n78C2 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A_n78(2A) 2 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A_n79A2 DC_1A-19A-21A_n79C2 | DC_1A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-19A-42A_n77A7,8 DC_1A-19A-42A_n77C7,8 DC_1A-19A-42C_n77A7,8 DC_1A-19A-42C_n77C7,8 | DC_1A_n77A DC_19A_n77A |
| DC_1A-19A-42A_n78A7,8 DC_1A-19A-42A_n78C7,8 DC_1A-19A-42C_n78A7,8 DC_1A-19A-42C_n78C7,8 | DC_1A_n78A DC_19A_n78A |
| DC_1A-19A-42A_n79A DC_1A-19A-42A_n79C DC_1A-19A-42C_n79A DC_1A-19A-42C_n79C | DC_1A_n79A DC_19A_n79A |
| DC_1A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_1A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_1A-20A_n3A-n38A | DC_1A_n3A DC_20A_n3A DC_1A_n38A DC_20A_n38A |
| DC_1A-20A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n7A-n78A | DC_1A_n7A DC_20A_n7A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-20A-28A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-20A_n28A-n75A | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A-28A_n78A | DC_1A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-20A_n28A-n78A2,3,8,14 | DC_1A_n28A DC_1A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-20A-32A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A-32A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A-32A_n28A8,14 | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A-32A_n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A-(n)38AA | DC_1A_n38A DC_20A_n38A |
| DC_1A-20A-38A_n8A | DC_1A_n8A DC_20A_n8A DC_38A_n8A |
| DC_1A-20A-38A_n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A-38A_n78(2A) | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n38A-n78A | DC_1A_n38A DC_20A_n38A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A-40A_n78A DC_1A-20A-40C_n78A | DC_1A_n78A DC_20A_n78A DC_40A_n78A |
| DC_1A-20A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_20A_n41A DC_20A_n78A |
| DC_1A-21A-28A_n77A2 | DC_1A_n77A DC_21A_n77A DC_28A_n77A |
| DC_1A-21A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_21A_n28A DC_21A_n77A |
| DC_1A-21A-28A_n78A2 | DC_1A_n78A DC_21A_n78A DC_28A_n78A |
| DC_1A-21A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_21A_n28A DC_21A_n78A |
| DC_1A-21A-28A_n79A2 | DC_1A_n79A DC_21A_n79A DC_28A_n79A |
| DC_1A-21A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_21A_n28A DC_21A_n79A |
| DC_1A-21A-42A_n77A7,8 DC_1A-21A-42A_n77C7,8 DC_1A-21A-42C_n77A7,8 DC_1A-21A-42C_n77C7,8 DC_1A-21A-42D_n77A7,8 DC_1A-21A-42D_n77C7,8 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A-42A_n78A7,8 DC_1A-21A-42A_n78C7,8 DC_1A-21A-42C_n78A7,8 DC_1A-21A-42C_n78C7,8 DC_1A-21A-42D_n78A7,8 DC_1A-21A-42D_n78C7,8 | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A-42A_n79A DC_1A-21A-42A_n79C DC_1A-21A-42C_n79A DC_1A-21A-42C_n79C DC_1A-21A-42D_n79A DC_1A-21A-42D_n79C | DC_1A_n79A DC_21A_n79A |
| DC_1A-21A_n77A-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A-21A_n78A-n79A | DC_1A_n78A DC_1A_n79A |
| DC_1A-28A_n3A-n77A2 | DC_28A_n3A DC_28A_n77A |
| DC_1A-28A_n3A-n78A2 | DC_1A_n3A DC_1A_n78A DC_28A_n3A DC_28A_n78A |
| DC_1A-28A_n5A-n78A2 | DC_1A_n5A DC_1A_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-28A_n7A-n78A | DC_1A_n7A DC_28A_n7A DC_1A_n78A DC_28A_n78A |
| DC_1A-28A_n7B-n78A | DC_1A_n7A DC_1A_n7B DC_28A_n7A DC_28A_n7B DC_1A_n78A DC_28A_n78A |
| DC_1A-28A-32A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A-40A_n78A DC_1A-28A-40C_n78A | DC_1A_n78A DC_28A_n78A DC_40A_n78A |
| DC_1A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-28A-42A_n77A7,8 DC_1A-28A-42A_n77C7,8 DC_1A-28A-42C_n77A7,8 DC_1A-28A-42C_n77C7,8 | DC_1A_n77A DC_28A_n77A |
| DC_1A-28A-42A_n78A7,8 DC_1A-28A-42A_n78C7,8 DC_1A-28A-42C_n78A7,8 DC_1A-28A-42C_n78C7,8 | DC_1A_n78A DC_28A_n78A |
| DC_1A-28A-42A_n79A DC_1A-28A-42A_n79C DC_1A-28A-42C_n79A DC_1A-28A-42C_n79C | DC_1A_n79A DC_28A_n79A |
| DC_1A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_41A_n3A |
| DC_1A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A |
| DC_1A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A |
| DC_1A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_38A_n3A DC_38A_n78A |
| DC_1A-38A_n7A-n78A | DC_1A_n78A |
| DC_1A-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_41A_n3A DC_41A_n77A |
| DC_1A-41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_1A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_41A_n3A DC_41A_n78A |
| DC_1A-41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_1A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_41A_n28A |
| DC_1A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A |
| DC_1A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_1A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A |
| DC_1A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_1A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_41A_n77A |
| DC_1A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_41A_n78A |
| DC_1A-42A_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A |
| DC_1A-42C_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A DC_42C_n3A DC_42C_n28A |
| DC_1A-42A_n3A-n77A7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A |
| DC_1A-42A_n3A-n77(2A) 7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A |
| DC_1A-42C_n3A-n77A7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-42C_n3A-n77(2A)7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-42A_n28A-n77A7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A |
| DC_1A-42A_n28A-n77(2A)7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A |
| DC_1A-42C_n28A-n77A7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-42C_n28A-n77(2A)7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-41A-42A_n77A7,8 DC_1A-41A-42C_n77A7,8 DC_1A-41C-42A_n77A7,8 DC_1A-41C-42C_n77A7,8 | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A-42A_n77(2A)7,8 DC_1A-41A-42C_n77(2A)7,8 | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A-42A_n78A7,8 DC_1A-41A-42C_n78A7,8 DC_1A-41C-42A_n78A7,8 DC_1A-41C-42C_n78A7,8 | DC_1A_n78A DC_41A_n78A |
| DC_1A-41A-42A_n79A DC_1A-41A-42C_n79A DC_1A-41C-42A_n79A DC_1A-41C-42C_n79A | DC_1A_n79A DC_41A_n79A |
| DC_1A-42A_n77A-n79A7,8 DC_1A-42C_n77A-n79A7,8 | DC_1A_n77A DC_1A_n79A |
| DC_1A-42A_n78A-n79A7,8 DC_1A-42C_n78A-n79A7,8 | DC_1A_n78A DC_1A_n79A |
| DC_2A-4A-7A_n28A | DC_2A_n28A DC_4A_n28A DC_7A_n28A |
| DC_2A-5A_n2A-n77A DC_2A-5A_n2A-n77C | DC_2A_n77A DC_5A_n2A DC_5A_n77A |
| DC_2A-5A_n2A-n78A | DC_5A_n2A DC_2A_n78A DC_5A_n78A |
| DC_2A-5A_n5A-n77A DC_2A-5A_n5A-n77C | DC_2A_n5A DC_2A_n77A DC_5A_n77A |
| DC_2A-5A-7A_n2A | DC_5A_n2A DC_7A_n2A |
| DC_2A-5A-7A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 |
| DC_2A-5A-7A_n66A DC_2A-5A-7C_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |
| DC_2A-5A-7A_n78A | DC_2A_n78A DC_5A_n78A DC_7A_n78A |
| DC_2A-2A-5A-7A_n66A DC_2A-5A-7A-7A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |
| DC_2A-5A-(n)12AA | DC_5A_n12A DC_2A_n12A DC_(n)12AA4 |
| DC_2A-12A-(n)5AA | DC_2A_n5A DC_12A_n5A DC_(n)5AA4 |
| DC_2A-5A-30A_n2A | DC_2A_n2A4 DC_5A_n2A DC_30A_n2A |
| DC_2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |
| DC_2A-2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |
| DC_2A-5A-30A_n77A9 DC_2A-2A-5A-30A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_30A_n77A9 |
| DC_2A-5A-48A_n12A | DC_2A_n12A DC_5A_n12A DC_48A_n12A |
| DC_2A-5A-48A_n77A7,8,9 DC_2A-5A-48A_n77C7,8,9 DC_2A-5A-48C_n77A7,8,9 DC_2A-5A-48C_n77C7,8,9 | DC_2A_n77A DC_5A_n77A |
| DC_2A-5A-66A_n2A DC_2A-5B-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-5A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-66A-66A_n2A DC_2A-5B-66A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-5A-66A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-5A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-5A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-5A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-5A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |
| DC_2A-5A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |
| DC_2A-5A-66A_n12A | DC_2A_n12A DC_5A_n12A DC_66A_n12A |
| DC_2A-5A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-2A-5A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-5A-66A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-5A-66A_n48A DC_2A-5A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |
| DC_2A-5A-66A-66A_n48A DC_2A-5A-66A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |
| DC_2A-5A-66A_n66A DC_2A-5B-66A_n66A | DC_2A_n66A DC_5A_n66A DC_66A_n66A4 |
| DC_2A-5A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-2A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-66A-66A_n66A DC_2A-5B-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-2A-5A-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-5A-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-66A_n71A | DC_2A_n71A DC_5A_n71A DC_66A_n71A |
| DC_2A-5A-66A_n77A9 DC_2A-5A-66A_n77C9 DC_2A-2A-5A-66A_n77C9 DC_2A-5A-66A-66A_n77C9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-2A-5A-66A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-5A-66A-66A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-5A-66A_n78A | DC_2A_n78A DC_5A_n78A DC_66A_n78A |
| DC_2A-5A_n66A-n77A DC_2A-5A_n66A-n77C | DC_2A_n66A DC_2A_n77A DC_5A_n66A DC_5A_n77A |
| DC_2A-5A_n66A-n78A | DC_2A_n66A DC_5A_n66A DC_2A_n78A DC_5A_n78A |
| DC_2A-7A_n2A-n78A | DC_7A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-12A_n2A | DC_7A_n2A DC_12A_n2A |
| DC_2A-7A-12A_n66A | DC_2A_n66A DC_7A_n66A DC_12A_n66A |
| DC_2A-2A-7A-12A_n66A | DC_2A_n66A DC_7A_n66A DC_12A_n66A |
| DC_2A-7A-12A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A |
| DC_2A-2A-7A-12A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A |
| DC_2A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7C-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7A-13A_n66A DC_2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-2A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7A-7A_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7C_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7A-28A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_2A-7A-28A_n66A DC_2A-7C-28A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A |
| DC_2A-7A-28A_n78A DC_2A-7C-28A_n78A | DC_2A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_2A-7A_n38A-n66A DC_2A-7C_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-7A_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-29A_n78A DC_2A-7C-29A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-7A-29A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A-7A_n38A-n78A DC_2A-7C_n38A-n78A | DC_2A_n78A |
| DC_2A-7A-7A_n38A-n78A | DC_2A_n78A |
| DC_2A-7A-66A_n2A | DC_7A_n2A DC_66A_n2A |
| DC_2A-7A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-7A-66A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7C-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7A-66A_n28A | DC_2A_n28A DC_7A_n28A DC_66A_n28A |
| DC_2A-7A-66A_n38A | 2A5 66A5 |
| DC_2A-2A-7A-66A_n38A | 2A5 66A5 |
| DC_2A-7A-66A_n66A DC_2A-7C-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-7A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-7A-66A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A_n71A | DC_2A_n71A DC_7A_n71A DC_66A_n71A |
| DC_2A-2A-7A-66A_n71A | DC_2A_n71A DC_7A_n71A DC_66A_n71A |
| DC_2A-7A-66A_n77A DC_2A-7C-66A_n77A | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-66A_n77(2A) DC_2A-7C-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-7A-66A_n77A | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-7A-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A_n66A-n77A DC_2A-7C_n66A-n77A DC_2A-7A-7A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-66A_n78A DC_2A-7C-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-2A-7A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A_n66A-n78A DC_2A-7C_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |
| DC_2A-7A-66A_n78(2A) DC_2A-7C-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |
| DC_2A-7A-7A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-66A-66A_n78A DC_2A-7C-66A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-66A-66A_n78(2A) DC_2A-7C-66A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-66A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-66A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-71A_n2A | DC_7A_n2A DC_71A_n2A |
| DC_2A-7A-71A_n66A | DC_2A_n66A DC_7A_n66A DC_71A_n66A |
| DC_2A-2A-7A-71A_n66A | DC_2A_n66A DC_7A_n66A DC_71A_n66A |
| DC_2A-7A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_71A_n78A |
| DC_2A-2A-7A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_71A_n78A |
| DC_2A-7A_n71A-n78A | DC_2A_n71A DC_7A_n71A DC_2A_n78A DC_7A_n78A |
| DC_2A-12A_n2A-n78A | DC_12A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-12A-30A_n2A | DC_12A_n2A DC_30A_n2A |
| DC_2A-12A-48A_n5A | DC_2A_n5A DC_12A_n5A DC_48A_n5A |
| DC_2A-12A-66A_n5A | DC_2A_n5A DC_12A_n5A DC_66A_n5A |
| DC_2A-12A-30A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A |
| DC_2A-2A-12A-30A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A |
| DC_2A-12A-30A_n77A9 DC_2A-2A-12A-30A_n77A9 | DC_2A_n77A9 DC_12A_n77A9 DC_30A_n77A9 |
| DC_2A-12A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_2A-12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_2A-12A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-2A-12A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-12A-66A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-12A-66A_n41A | DC_2A_n41A DC_12A_n41A DC_66A_n41A |
| DC_2A-2A-12A-66A_n41A | DC_2A_n41A DC_12A_n41A DC_66A_n41A |
| DC_2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |
| DC_2A-2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |
| DC_2A-12A-66A_n77A9 DC_2A-2A-12A-66A_n77A9 DC_2A-12A-66A-66A_n77A9 | DC_2A_n77A9 DC_12A_n77A9 DC_66A_n77A9 |
| DC_2A-12A-66A_n78A | DC_2A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-2A-12A-66A_n78A | DC_2A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-12A_n66A-n78A | DC_2A_n66A DC_12A_n66A DC_2A_n78A DC_12A_n78A |
| DC_2A-13A_n2A-n77A DC_2A-13A_n2A-n77C | DC_2A_n77A DC_13A_n2A DC_13A_n77A |
| DC_2A-13A_n5A-n77A9 DC_2A-2A-13A_n5A-n77A9 DC_2A-13A_n5A-n77C9 | DC_2A_n5A DC_2A_n77A DC_13A_n77A |
| DC_2A-13A_n25A-n66A8,14 | DC_2A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-13A-48A_n77A7,8,9 DC_2A-13A-48A_n77C7,8,9 DC_2A-13A-48C_n77A7,8,9 DC_2A-13A-48C_n77C7,8,9 | DC_2A_n77A DC_13A_n77A |
| DC_2A-13A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_2A-13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_2A-13A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-13A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-13A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-13A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-13A-66A_n48A DC_2A-13A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |
| DC_2A-13A-66A-66A_n48A DC_2A-13A-66A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |
| DC_2A-13A-66A_n66A DC_2A-2A-13A-66A_n66A DC_2A-13A-66A-66A_n66A DC_2A-2A-13A-66A-66A_n66A | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-13A-66B_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-13A-66A_n77A9 DC_2A-13A-66A_n77C9 DC_2A-2A-13A-66A_n77C9 DC_2A-2A-13A-66A-66A_n77A DC_2A-13A-66A-66A_n77C9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-2A-13A-66A_n77A9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-13A-66A-66A_n77A9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-13A_n66A-n77A9 DC_2A-13A_n66A-n77C9 DC_2A-2A-13A_n66A-n77A9 | DC_2A_n66A DC_2A_n77A9 DC_13A_n66A DC_13A_n77A9 |
| DC_2A-14A-30A_n2A | DC_2A_n2A4 DC_14A_n2A DC_30A_n2A |
| DC_2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-14A-30A_n77A9 DC_2A-2A-14A-30A_n77A9 | DC_2A_n77A9 DC_14A_n77A9 DC_30A_n77A9 |
| DC_2A-14A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-2A-14A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-14A-66A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-14A-66A_n77A9 DC_2A-2A-14A-66A_n77A9 DC_2A-14A-66A-66A_n77A9 | DC_2A_n77A9 DC_14A_n77A9 DC_66A_n77A9 |
| DC_2A-28A-66A_n7A | DC_2A_n7A DC_28A_n7A DC_66A_n7A |
| DC_2A-28A-66A_n66A | DC_2A_n66A DC_28A_n66A DC_66A_n66A4 |
| DC_2A-29A-30A_n2A | DC_2A_n2A4 DC_30A_n2A |
| DC_2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-29A-30A_n77A9 DC_2A-2A-29A-30A_n77A9 | DC_2A_n77A9 DC_30A_n77A9 |
| DC_2A-29A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |
| DC_2A-29A-66A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |
| DC_2A-29A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-29A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-29A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-29A-66A_n66A | DC_2A_n66A DC_66A_n66A4 |
| DC_2A-2A-29A-66A_n66A | DC_2A_n66A DC_66A_n66A4 |
| DC_2A-29A-66A_n77A9 | DC_2A_n77A9 DC_66A_n77A9 |
| DC_2A-29A-66A_n78A | DC_2A_n78A DC_66A_n78A |
| DC_2A-30A-(n)5AA DC_2A-2A-30A-(n)5AA | DC_2A_n5A DC_30A_n5A DC_(n)5AA4 |
| DC_2A-30A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-30A-66A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-30A-66A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-2A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-30A-66A_n77A9 DC_2A-2A-30A-66A_n77A9 DC_2A-30A-66A-66A_n77A9 | DC_2A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_2A-46A_n41A-n66A DC_2A-46C_n41A-n66A DC_2A-46D_n41A-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A-46A_n41A-n71A DC_2A-46C_n41A-n71A DC_2A-46D_n41A-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A_n41(2A)-n71A DC_2A-46C_n41(2A)-n71A DC_2A-46D_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A-48A_n2A DC_2A-46C-48A_n2A DC_2A-46D-48A_n2A DC_2A-46E-48A_n2A | DC_2A_n2A4 DC_48A_n2A |
| DC_2A-46A-48A_n5A DC_2A-46C-48A_n5A DC_2A-46D-48A_n5A DC_2A-46E-48A_n5A | DC_2A_n5A DC_48A_n5A |
| DC_2A-46A-48A_n66A DC_2A-46C-48A_n66A DC_2A-46D-48A_n66A DC_2A-46E-48A_n66A | DC_2A_n66A DC_48A_n66A |
| DC_2A-46A-66A_n5A DC_2A-46C-66A_n5A DC_2A-46D-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-46A-66A_n41A DC_2A-46C-66A_n41A DC_2A-46D-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-46A-66A_n41(2A) DC_2A-46C-66A_n41(2A) DC_2A-46D-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-46A-66A_n71A DC_2A-46C-66A_n71A DC_2A-46D-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-48A-(n)5AA | DC_2A_n5A DC_48A_n5A DC_(n)5AA4 |
| DC_2A-46A_n66A-n71A DC_2A-46C_n66A-n71A DC_2A-46D_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-48A_n48A-n66A | DC_2A_n48A DC_2A_n66A DC_48A_n66A |
| DC_2A-48A-66A_n2A DC_2A-48C-66A_n2A DC_2A-48D-66A_n2A DC_2A-48E-66A_n2A | DC_66A_n2A DC_48A_n2A DC_2A_n2A4 |
| DC_2A-48A-66A_n5A | DC_2A_n5A DC_48A_n5A DC_66A_n5A |
| DC_2A-48C-66A_n5A DC_2A-48D-66A_n5A DC_2A-48E-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-48A-66A_n12A | DC_2A_n12A DC_48A_n12A DC_66A_n12A |
| DC_2A-48A-66A_n66A DC_2A-48C-66A_n66A DC_2A-48D-66A_n66A DC_2A-48E-66A_n66A | DC_66A_n66A4 DC_48A_n66A DC_2A_n66A |
| DC_2A-48A-66A_n71A | DC_2A_n71A DC_48A_n71A DC_66A_n71A |
| DC_2A-48A-66A_n77A7,8,9 DC_2A-48C-66A_n77A7,8,9 DC_2A-48A-66A_n77C7,8,9 DC_2A-48C-66A_n77C7,8,9 DC_2A-48D-66A_n77A7,8,9 DC_2A-48E-66A_n77A7,8,9 | DC_2A_n77A DC_66A_n77A |
| DC_2A-66A_n2A-n77A DC_2A-66A_n2A-n77C | DC_2A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-66A-66A_n2A-n77A | DC_2A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-66A-(n)5AA DC_2A-2A-66A-(n)5AA DC_2A-66A-66A-(n)5AA | DC_2A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_2A-66A_n2A-n78A | DC_66A_n2A DC_2A_n78A DC_66A_n78A |
| DC_2A-66A_n5A-n77A9 DC_2A-2A-66A_n5A-n77A9 DC_2A-66A-66A_n5A-n77A9 DC_2A-66A_n5A-n77C9 | DC_2A_n5A DC_2A_n77A9 DC_5A_n77A DC_66A_n5A DC_66A_n77A9 |
| DC_2A-66A_n25A-n66A7,8 | DC_2A_n66A DC_66A_n25A |
| DC_2A-66A_n38A-n78A | DC_2A_n38A DC_2A_n78A DC_66A_n38A DC_66A_n78A |
| DC_2A-66A-71A_n38A | DC_2A_n38A DC_66A_n38A DC_71A_n38A |
| DC_2A-2A-66A-71A_n38A | DC_2A_n38A DC_66A_n38A DC_71A_n38A |
| DC_2A-66A-71A_n41A | DC_2A_n41A DC_66A_n41A DC_71A_n41A |
| DC_2A-2A-66A-71A_n41A | DC_2A_n41A DC_66A_n41A DC_71A_n41A |
| DC_2A-66A-71A_n66A | DC_2A_n66A DC_66A_n66A4 DC_71A_n66A |
| DC_2A-66A-71A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A-71A_n78A | DC_2A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-2A-66A-71A_n78A | DC_2A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-66A-(n)71AA DC_2A-66C-(n)71AA | DC_2A_n71A DC_66A_n71A DC_(n)71AA |
| DC_2A-66A_n41A-n71A DC_2A-66A_n41C-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A_n66A-n77A9 DC_2A-2A-66A_n66A-n77A9 DC_2A-66A_n66A-n77C9 | DC_2A_n66A DC_2A_n77A9 DC_66A_n77A9 |
| DC_2A-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_66A_n66A4 |
| DC_2A-66A-71A_n2A | DC_66A_n2A DC_71A_n2A |
| DC_2A-66A_n71A-n78A | DC_2A_n71A DC_66A_n71A DC_2A_n78A DC_66A_n78A |
| DC_2A-71A_n2A-n78A | DC_71A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-71A_n66A-n78A | DC_2A_n66A DC_71A_n66A DC_2A_n78A DC_71A_n78A |
| DC_3A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A |
| DC_3A-3A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A |
| DC_3A_n1A-n40A-n78A | DC_3A_n1A DC_3A_n40A DC_3A_n78A |
| DC_3A_n1A-n77A-n79A | DC_3A_n1A DC_3A_n77A DC_3A_n79A |
| DC_3A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A |
| DC_3A-5A-7A_n77A | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A_n77(2A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A-7A_n77A | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A-7A_n77(2A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A_n78A DC_3C-5A-7A_n78A DC_3A-5A-7A_n78C | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A_n78(2A) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n78A DC_3A-5A-7A-7A_n78C | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n78(2A) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A_n5A-n40A-n78A | DC_3A_n5A DC_3A_n40A DC_3A_78A |
| DC_3A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-3A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-7A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A |
| DC_3A-7A_n1A-n78A2 DC_3C-7A_n1A-n78A2 | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A DC_7A_n1A DC_7A_n78A |
| DC_3A-3A-7A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A |
| DC_3A-7A-7A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A |
| DC_3A-3A-7A-7A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A |
| DC_3A-7C_n1A-n78A DC_3C-7C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_3A-5A-41A_n79A | DC_3A_n79A DC_5A_n79A DC_41A_n79A |
| DC_3A-7A_n1A-n75A | DC_3A_n1A DC_7A_n1A |
| DC_3C-7A_n1A-n75A | DC_3C_n1A DC_3A_n1A DC_7A_n1A |
| DC_3A-7A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_3A_n78A DC_7A_n78A |
| DC_3A-7C_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_3A_n78A DC_7C_n78A DC_7A_n78A |
| DC_3A-7A_n5A-n78A9 DC_3A-7C_n5A-n78A9 DC_3C-7A_n5A-n78A9 DC_3C-7C_n5A-n78A9 | DC_3A_n5A DC_3A_n78A9 DC_3C_n78A9 DC_7A_n5A DC_7C_n5A DC_7A_n78A9 DC_7C_n78A9 |
| DC_3A-7A_n7A-n78A2 | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A |
| DC_3A-3A-7A_n7A-n78A2 | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A |
| DC_3C-7A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A-8A_n1A DC_3C-7A-8A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-3A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-3A-7A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-8A_n28A | DC_3A_n28A DC_7A_n28A DC_8A_n28A |
| DC_3A-7A-8A_n40A | DC_3A_n40A DC_7A_n40A DC_8A_n40A |
| DC_3A-7A-8A_n77A2 | DC_3A_n77A DC_7A_n77A DC_8A_n77A |
| DC_3A-7A-8A_n78A2, 9 | DC_3A_n78A9 DC_7A_n78A 9 DC_8A_n78A9 |
| DC_3A-7A-8A_n78(2A) | DC_3A_n78A, DC_7A_n78A, DC_8A_n78A |
| DC_3A-3A-7A-8A_n78A2, 9 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 |
| DC_3A-7A-7A-8A_n78A2, 9 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 |
| DC_3A-3A-7A-7A-8A_n78A2, 9 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 |
| DC_3A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-3A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-7A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-3A-7A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_3A-7A-20A_n1A DC_3C-7A-20A_n1A DC_3A-7C-20A_n1A DC_3C-7C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A DC_20A_n1A |
| DC_3A-7A-20A_n8A | DC_3A_n8A DC_7A_n8A DC_20A_n8A |
| DC_3A-7A-20A_n28A3,8,14 DC_3C-7A-20A_n28A3 | DC_3A_n28A DC_7A_n28A DC_20A_n28A |
| DC_3A-7A-20A_n38A12,13 | CA_3A-20A |
| DC_3A-7A-20A_n78A2 DC_3C-7A-20A_n78A2 | DC_3A_n78A DC_3C_n78A DC_20A_n78A DC_7A_n78A |
| DC_3A-7A-20A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_3A_n26A DC_7A_n26A |
| DC_3A-7C_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_7A_n26A DC_7C_n26A |
| DC_3C-7A_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A |
| DC_3C-7C_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |
| DC_3A-7A-28A_n1A DC_3C-7A-28A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_28A_n1A |
| DC_3A-7A-7A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_28A_n1A |
| DC_3A-7A-28A_n3A DC_3A-7C-28A_n3A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_3A-7A-28A_n5A DC_3A-7C-28A_n5A DC_3C-7A-28A_n5A DC_3C-7C-28A_n5A | DC_3A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_3A-7A-28A_n7A DC_3C-7A-28A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_3A-3A-7A-28A_n7A | DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_3A-7A-28A_n40A | DC_3A_n40A DC_7A_n40A DC_28A_n40A |
| DC_3A-7A-28A_n78A2, 9 DC_3A-7C-28A_n78A2, 9 DC_3C-7A-28A_n78A9 DC_3C-7C-28A_n78A9 | DC_3A_n78A9 DC_3C_n78A9 DC_7A_n78A9 DC_7C_n78A9 DC_28A_n78A9 |
| DC_3A-7A_n28A-n78A2, 9 DC_3A-7C_n28A-n78A9 DC_3C-7A_n28A-n78A9 DC_3C-7C_n28A-n78A9 | DC_3A_n28A DC_3A_n78A9 DC_3C_n78A9 DC_7A_n28A DC_7A_n78A9 DC_7C_n28A DC_7C_n78A9 |
| DC_3A-7A-32A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A-32A_n28A DC_3C-7A-32A_n28A | DC_3A_n28A DC_7A_n28A |
| DC_3A-7A-32A_n78A DC_3C-7A-32A_n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A-38A_n28A10 DC_3C-7A-38A_n28A10 | DC_3A_n28A |
| DC_3A-7A_n38A-n78A | DC_3A_n78A |
| DC_3A-7A-40A_n1A DC_3A-7A-40C_n1A | DC_3A_n1A DC_7A_n1A DC_40A_n1A |
| DC_3A-7A-40A_n78A DC_3A-7A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_3A-7A-40A_n78(2A) DC_3A-7A-40C_n78(2A) | DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_3A-7A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |
| DC_3A-7A_n75A-n78A | DC_3A_n78A DC_7A_n78A |
| DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_7A_n78A DC_7A_n80A |
| DC_3A-8A_n1A-n28A | DC_3A_n1A DC_8A_n1A DC_3A_n28A DC_8A_n28A |
| DC_3A-8A_n1A-n40A | DC_3A_n1A DC_8A_n1A DC_3A_n40A DC_8A_n40A |
| DC_3A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-3A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-8A-11A_n28A | DC_3A_n28A DC_8A_n28A DC_11A_n28A |
| DC_3A-8A-11A_n77A2 | DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_3A-8A-11A_n77(2A) 2 DC_3A-8A-11A_n77(3A)2 | DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_3A-8A-20A_n1A | DC_3A_n1A DC_8A_n1A DC_20A_n1A |
| DC_3A-8A-20A_n78A | DC_3A_n78A DC_8A_n78A DC_20A_n78A |
| DC_3A-8A_n28A-n77A2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_3A-8A_n28A-n77(2A)2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_3A-8A-28A_n78A | DC_3A_n78A DC_8A_n78A DC_28A_n78A |
| DC_3A-8A_n28A-n78A2 | DC_3A_n28A DC_3A_n78A DC_8A_n28A DC_8A_n78A |
| DC_3A-8A-32A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-8A-32A_n28A DC_3C-8A-32A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3A-8A-32A_n78A | DC_3A_n78A DC_8A_n78A |
| DC_3A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_8A_n40A DC_8A_n78A |
| DC_3A-8A-40A_n1A DC_3A-8A-40C_n1A | DC_3A_n1A DC_8A_n1A DC_40A_n1A |
| DC_3A-8A-40A_n78A DC_3A-8A-40C_n78A | DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-8A-40A_n78(2A) DC_3A-8A-40C_n78(2A) | DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-8A_n40A-n79A DC_3A-8A_n40A-n79C | DC_3A_n40A DC_3A_n79A DC_8A_n40A DC_8A_n79A |
| DC_3A-8A-42A_n77A7,8 DC_3A-8A-42C_n77A7,8 | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n77A-n79A | DC_3A_n77A DC_3A_n79A DC_8A_n77A DC_8A_n79A |
| DC_3A-8A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_8A_n78A DC_8A_n80A |
| DC_3A-11A_n28A-n77A2 | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-11A_n28A-n77(2A) 2 | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-18A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_18A_n3A DC_18A_n41A |
| DC_3A-18A_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_18A_n3A DC_18A_n77A |
| DC_3A-18A_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_18A_n3A DC_18A_n78A |
| DC_3A-18A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A |
| DC_3A-18A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_3A-18A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_3A-18A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_3A-18A_n28A-n78(2A) | DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_3A-18A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_3A-18A_n41A-n77(2A) | DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_3A-18A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_3A-18A_n41A-n78(2A) | DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_3A-18A-42A_n77A7,8 DC_3A-18A-42C_n77A7,8 | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A-42A_n78A7,8 DC_3A-18A-42C_n78A7,8 | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A-42A_n79A DC_3A-18A-42C_n79A | DC_3A_n79A DC_18A_n79A |
| DC_3A-19A_n1A-n77A2 | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A |
| DC_3A-19A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A |
| DC_3A-19A_n1A-n79A2 | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A |
| DC_3A-19A-21A_n77A2 DC_3A-19A-21A_n77C2 | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A_n78A2 DC_3A-19A-21A_n78C2 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A_n79A2 DC_3A-19A-21A_n79C2 | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-42A_n1A2 DC_3A-19A-42C_n1A2 | DC_3A_n1A DC_19A_n1A DC_42A_n1A |
| DC_3A-19A-42A_n77A7,8 DC_3A-19A-42A_n77C7,8 DC_3A-19A-42C_n77A7,8 DC_3A-19A-42C_n77C7,8 DC_3A-19A-42D_n77A7,8 DC_3A-19A-42D_n77C7,8 | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A-42A_n78A7,8 DC_3A-19A-42A_n78C7,8 DC_3A-19A-42C_n78A7,8 DC_3A-19A-42C_n78C7,8 DC_3A-19A-42D_n78A7,8 DC_3A-19A-42D_n78C7,8 | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A-42A_n79A2 DC_3A-19A-42A_n79C2 DC_3A-19A-42C_n79A2 DC_3A-19A-42C_n79C2 DC_3A-19A-42D_n79A DC_3A-19A-42D_n79C | DC_3A_n79A DC_19A_n79A |
| DC_3A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_3A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_3A-20A_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_20A_n1A DC_20A_n7A |
| DC_3C-20A_n1A-n7A | DC_3A_n1A DC_3C_n1A DC_3A_n7A DC_3C_n7A DC_20A_n1A DC_20A_n7A |
| DC_3A-20A_n1A-n28A8,14 | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_20A_n28A |
| DC_3C-20A_n1A-n28A8,14 | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_3C_n1A DC_20A_n28A |
| DC_3A-20A_n1A-n75A | DC_3A_n1A DC_20A_n1A |
| DC_3C-20A_n1A-n75A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A |
| DC_3C-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_3C_n1A DC_3C_n78A |
| DC_3A-20A_n7A-n28A8,14 | DC_3A_n7A DC_3A_n28A DC_20A_n7A DC_20A_n28A |
| DC_3C-20A_n7A-n28A8,14 | DC_3A_n7A DC_3A_n28A DC_3C_n7A DC_20A_n7A DC_20A_n28A |
| DC_3A-20A_n7A-n78A | DC_3A_n7A DC_3A_n78A DC_20A_n7A DC_20A_n78A |
| DC_3A-20A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_20A_n8A DC_20A_n78A |
| DC_3A-20A-28A_n1A | DC_3A_n1A DC_20A_n1A DC_28A_n1A |
| DC_3A-20A_n28A-n75A | DC_3A_n28A DC_20A_n28A |
| DC_3C-20A_n28A-n75A | DC_20A_n28A DC_3A_n28A |
| DC_3A-20A-28A_n78A | DC_3A_n78A DC_20A_n78A DC_28A_n78A |
| DC_3A-20A_n28A-n78A2,3,8,14 DC_3C-20A_n28A-n78A2,3,8,14 | DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-20A-32A_n1A DC_3C-20A-32A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-20A-32A_n28A8,14 DC_3C-20A-32A_n28A8,14 | DC_3A_n28A DC_20A_n28A |
| DC_3A-20A-32A_n78A | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A-38A_n78A | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A-38A_n78(2A) | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A_n38A-n78A | DC_3A_n78A DC_20A_n78A DC_3A_n38A DC_20A_n38A |
| DC_3A-20A-40A_n78A DC_3A-20A-40C_n78A | DC_3A_n78A DC_20A_n78A DC_40A_n78A |
| DC_3A-20A-40A_n78(2A) DC_3A-20A-40C_n78(2A) | DC_3A_n78A DC_20A_n78A DC_40A_n78A |
| DC_3A-20A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_20A_n41A DC_20A_n78A |
| DC_3A-20A_SUL_n78A-n80A DC_3C-20A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_20A_n78A DC_20A_n80A |
| DC_3A-21A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_21A_n28A DC_21A_n77A |
| DC_3A-21A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_21A_n28A DC_21A_n78A |
| DC_3A-21A_n28A-n79A2 | DC_3A_n28A DC_3A_n79A DC_21A_n28A DC_21A_n79A |
| DC_3A-21A-42A_n1A2 DC_3A-21A-42C_n1A2 | DC_3A_n1A DC_21A_n1A DC_42A_n1A |
| DC_3A-21A_n1A-n77A2 | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A |
| DC_3A-21A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A |
| DC_3A-21A_n1A-n79A2 | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A |
| DC_3A-21A-42A_n77A7,8 DC_3A-21A-42A_n77C7,8 DC_3A-21A-42C_n77A7,8 DC_3A-21A-42C_n77C7,8 DC_3A-21A-42D_n77A7,8 DC_3A-21A-42D_n77C7,8 | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A-42A_n78A7,8 DC_3A-21A-42A_n78C7,8 DC_3A-21A-42C_n78A7,8 DC_3A-21A-42C_n78C7,8 DC_3A-21A-42D_n78A7,8 DC_3A-21A-42D_n78C7,8 | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A-42A_n79A DC_3A-21A-42A_n79C DC_3A-21A-42C_n79A DC_3A-21A-42C_n79C DC_3A-21A-42D_n79A DC_3A-21A-42D_n79C | DC_3A_n79A DC_21A_n79A |
| DC_3A-21A_n77A-n79A | DC_3A_n77A DC_3A_n79A DC_21A_n77A DC_21A_n79A |
| DC_3A-21A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_21A_n78A DC_21A_n79A |
| DC_3A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_28A_n1A DC_28A_n40A |
| DC_3A-28A_n1A-n78A2 | DC_3A_n1A DC_28A_n1A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n3A-n78A2 | DC_3A_n3A4 DC_28A_n3A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n5A-n78A2 DC_3C-28A_n5A-n78A2 | DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_28A_n5A DC_28A_n78A |
| DC_3A-28A_n7A-n78A | DC_3A_n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A |
| DC_3A-3A-28A_n7A-n78A | DC_3A_n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A |
| DC_3A-3A-28A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A |
| DC_3C-28A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_3C-28A_n7B-n78A | DC_3A_n7A DC_3C_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_3A-28A-32A_n1A | DC_3A_n1A DC_28A_n1A |
| DC_3A-28A-40A_n78A DC_3A-28A-40C_n78A | DC_3A_n78A DC_28A_n78A DC_40A_n78A |
| DC_3A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_28A_n40A DC_28A_n78A |
| DC_3A-28A-41A_n78A DC_3A-28A-41C_n78A | DC_3A_n78A DC_28A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-28A-42A_n77A7,8 DC_3A-28A-42A_n77C7,8 DC_3A-28A-42C_n77A7,8 DC_3A-28A-42C_n77C7,8 | DC_3A_n77A DC_28A_n77A |
| DC_3A-28A-42A_n78A7,8 DC_3A-28A-42A_n78C7,8 DC_3A-28A-42C_n78A7,8 DC_3A-28A-42C_n78C7,8 | DC_3A_n78A DC_28A_n78A |
| DC_3A-28A-42A_n79A DC_3A-28A-42A_n79C DC_3A-28A-42C_n79A DC_3A-28A-42C_n79C | DC_3A_n79A DC_28A_n79A |
| DC_3A_n28A-n77A-n79A | DC_3A_n28A DC_3A_n77A DC_3A_n79A |
| DC_3A_n28A-n78A-n79A | DC_3A_n28A DC_3A_n78A DC_3A_n79A |
| DC_3A-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A |
| DC_3C-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A |
| DC_3A-32A_n1A-n78A DC_3C-32A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A |
| DC_3A-32A-38A_n28A DC_3C-32A-38A_n28A | DC_3A_n28A DC_38A_n28A |
| DC_3A-38A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_3C-38A_n28A-n78A | DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_3A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-41A_n1A-n78A DC_3A-3A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-41C_n1A-n78A DC_3A-3A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_3A-41A_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A |
| DC_3A-41C_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_3A-41A_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A |
| DC_3A-41C_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_3A-41A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_41A_n28A |
| DC_3A-41A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A |
| DC_3A-41C_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_3A-41A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |
| DC_3A-41C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_3A-41A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_41A_n77A |
| DC_3A-41A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_41A_n78A |
| DC_3A-41A-42A_n77A7,8 DC_3A-41A-42C_n77A7,8 DC_3A-41C-42A_n77A7,8 DC_3A-41C-42C_n77A7,8 | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A-42A_n77(2A)7,8 DC_3A-41A-42C_n77(2A)7,8 | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A-42A_n78A7,8 DC_3A-41A-42C_n78A7,8 DC_3A-41C-42A_n78A7,8 DC_3A-41C-42C_n78A7,8 | DC_3A_n78A DC_41A_n78A |
| DC_3A-41A-42A_n79A DC_3A-41A-42C_n79A DC_3A-41C-42A_n79A DC_3A-41C-42C_n79A | DC_3A_n79A DC_41A_n79A |
| DC_3A-42A_n1A-n77A7,8 DC_3A-42C_n1A-n77A7,8 | DC_3A_n1A DC_3A_n77A |
| DC_3A-42A_n1A-n78A7,8 DC_3A-42C_n1A-n78A7,8 | DC_3A_n1A DC_3A_n78A |
| DC_3A-42A_n1A-n79A DC_3A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A |
| DC_3A-42A_n28A-n77A7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_3A-42A_n28A-n77(2A)7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_3A-42C_n28A-n77A7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_3A-42C_n28A-n77(2A)7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_3A-42A_n77A-n79A7,8 DC_3A-42C_n77A-n79A7,8 | DC_3A_n77A DC_3A_n79A |
| DC_3A-42A_n78A-n79A7,8 DC_3A-42C_n78A-n79A7,8 | DC_3A_n78A DC_3A_n79A |
| DC_5A-7A_n2A-n78A | DC_5A_n2A DC_7A_n2A DC_5A_n78A DC_7A_n78A |
| DC_5A-7A-66A_n2A | DC_5A_n2A DC_7A_n2A DC_66A_n2A |
| DC_5A-7A-66A_n7A | DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_5A-7A-66A-66A_n7A | DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_5A-7A-66A_n66A DC_5A-7C-66A_n66A DC_5A-7A-7A-66A_n66A | DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_5A-7A-66A_n78A DC_5A-7C-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_5A-7A-66A-66A_n78A DC_5A-7C-66A-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_5A-7A_n66A-n78A | DC_5A_n66A DC_7A_n66A DC_5A_n78A DC_7A_n78A |
| DC_5A-30A-66A_n2A | DC_5A_n2A DC_30A_n2A DC_66A_n2A |
| DC_5A-30A-66A-66A_n2A | DC_5A_n2A DC_30A_n2A DC_66A_n2A |
| DC_5A-30A-66A_n66A | DC_5A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_5A-30A-66A_n77A9 DC_5A-30A-66A-66A_n77A9 | DC_5A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_5A-48A-(n)12AA | DC_5A_n12A DC_48A_n12A DC_(n)12AA4 |
| DC_5A-48A-66A_n12A | DC_5A_n12A DC_48A_n12A DC_66A_n12A |
| DC_5A-48A-66A_n71A | DC_5A_n71A DC_48A_n71A DC_66A_n71A |
| DC_5A-66A_n2A-n77A DC_5A-66A_n2A-n77C | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A |
| DC_5A-66A-66A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A |
| DC_5A-66A_n5A-n77A DC_5A-66A_n5A-n77C | DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_5A-66A-66A_n5A-n77A | DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_5A-48A-66A_n77A7,8,9 DC_5A-48A-66A_n77C7,8,9 DC_5A-48C-66A_n77A7,8,9 DC_5A-48C-66A_n77C7,8,9 | DC_5A_n77A DC_66A_n77A |
| DC_5A-66A_n2A-n78A | DC_5A_n2A DC_66A_n2A DC_5A_n78A DC_66A_n78A |
| DC_5A-66A-(n)12AA | DC_5A_n12A DC_66A_n12A DC_(n)12AA4 |
| DC_5A-66A_n66A-n77A9 DC_5A-66A_n66A-n77C9 | DC_5A_n66A DC_5A_n77A9 DC_66A_n77A9 |
| DC_7A_n1A-n8A-n78A2 | DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_7A-7A_n1A-n8A-n78A2 | DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_7A-8A_n1A-n40A | DC_7A_n1A DC_8A_n1A DC_7A_n40A DC_8A_n40A |
| DC_7A-8A_n1A-n78A2 | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_7A-7A-8A_n1A-n78A2 | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_7A-8A-20A_n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |
| DC_7A-8A-20A_n3A | DC_7A_n3A DC_8A_n3A DC_20A_n3A |
| DC_7A-8A-20A_n28A9 | DC_7A_n28A |
| DC_7A-8A-20A_n78A | DC_7A_n78A DC_8A_n78A DC_20A_n78A |
| DC_7A-8A-32A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A-32A_n78A | DC_7A_n78A DC_8A_n78A |
| DC_7A-8A-38A_n1A | DC_8A_n1A |
| DC_7A-8A_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_7A-8A-40A_n1A DC_7A-8A-40C_n1A | DC_7A_n1A DC_8A_n1A DC_40A_n1A |
| DC_7A-8A-40A_n78A DC_7A-8A-40C_n78A | DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_7A-8A-40A_n78(2A) DC_7A-8A-40C_n78(2A) | DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_7A-8A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A |
| DC_7A-12A_n2A-n78A | DC_7A_n2A DC_12A_n2A DC_7A_n78A DC_12A_n78A |
| DC_7A-12A-66A_n2A | DC_7A_n2A DC_12A_n2A DC_66A_n2A |
| DC_7A-12A-66A_n78A | DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_7A-12A_n66A-n78A | DC_7A_n66A DC_12A_n66A DC_7A_n78A DC_12A_n78A |
| DC_7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7A-7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7C-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7A-13A-66A_n66A DC_7C-13A-66A_n66A | DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_7A-7A-13A-66A_n66A | DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_7A-20A_n1A-n75A | DC_3A_n1A DC_7A_n1A |
| DC_7A-20A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |
| DC_7A-20A_n3A-n38A | DC_20A_n3A |
| DC_7A-20A_n3A-n78A | DC_7A_n3A DC_20A_n3A DC_7A_n78A DC_20A_n78A |
| DC_7A-20A_n8A-n78A | DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_7A-20A-28A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |
| DC_7A-20A-28A_n3A DC_7C-20A-28A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_7A-20A_n28A-n78A2,3 | DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_7A-20A-32A_n1A | DC_7A_n1A DC_20A_n1A |
| DC_7A-20A-32A_n3A DC_7C-20A-32A_n3A | DC_7A_n3A DC_20A_n3A |
| DC_7A-20A-32A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A-32A_n28A | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A-32A_n78A | DC_7A_n78A DC_20A_n78A |
| DC_7A-20A-38A_n1A | DC_20A_n1A |
| DC_7A-20A-38A_n3A | DC_20A_n3A |
| DC_7A-20A-38A_n8A | DC_20A_n8A |
| DC_7A-20A-38A_n78A10 | DC_20A_n78A |
| DC_7A-20A_n38A-n78A10 | DC_20A_n78A |
| DC_7A-25A-66A_n77A DC_7C-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-7A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-25A-25A-66A_n77A DC_7C-25A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-7A-25A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-25A-66A_n78A DC_7C-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-7A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-25A-25A-66A_n78A DC_7C-25A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-7A-25A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-28A_n1A-n40A | DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A |
| DC_7A-28A_n1A-n78A | DC_7A_n1A DC_28A_n1A DC_7A_n78A DC_28A_n78A |
| DC_7A-28A_n3A-n78A | DC_7A_n3A DC_28A_n3A DC_7A_n78A DC_28A_n78A |
| DC_7C-28A_n3A-n78A | DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_7A-28A_n5A-n78A DC_7C-28A_n5A-n78A | DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_7A-28A_n7A-n78A | DC_7A_n7A4 DC_28A_n7A DC_7A_n78A DC_28A_n78A |
| DC_7A-28A-32A_n1A | DC_7A_n1A DC_28A_n1A |
| DC_7A-28A-32A_n3A DC_7C-28A-32A_n3A | DC_7A_n3A DC_28A_n3A |
| DC_7A-28A-38A_n1A | DC_28A_n1A |
| DC_7A-28A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_7A-66A_n38A-n78A DC_7C-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_7A-7A-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_7A-28A-66A_n7A | DC_7A_n7A4 DC_28A_n7A DC_66A_n7A |
| DC_7A-28A-66A_n66A DC_7C-28A-66A_n66A | DC_7A_n66A DC_28A_n66A DC_66A_n66A4 |
| DC_7A-29A-66A_n78A DC_7C-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-32A_n1A-n78A | DC_7A_n1A DC_7A_n78A |
| DC_7A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-66A_n2A-n78A | DC_7A_n2A DC_66A_n2A DC_7A_n78A DC_66A_n78A |
| DC_7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7A-7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7C-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7A-66A_n66A-n77A DC_7C-66A_n66A-n77A DC_7A-7A-66A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_66A_n77A |
| DC_7A-66A_n66A-n78A DC_7C-66A_n66A-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_7A-7A-66A_n66A-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_7A-66A-71A_n2A | DC_7A_n2A DC_66A_n2A DC_71A_n2A |
| DC_7A-66A-71A_n78A | DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_7A-66A_n71A-n78A | DC_7A_n71A DC_66A_n71A DC_7A_n78A DC_66A_n78A |
| DC_7A-71A_n2A-n78A | DC_7A_n2A DC_71A_n2A DC_7A_n78A DC_71A_n78A |
| DC_7A-71A_n66A-n78A | DC_7A_n66A DC_71A_n66A DC_7A_n78A DC_71A_n78A |
| DC_8A_n1A-n3A-n77A | DC_8A_n1A DC_8A_n3A DC_8A_n77A |
| DC_8A_n3A-n28A-n77A2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_8A_n3A-n28A-n77(2A) 2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_8A_n3A-n28A-n79A | DC_8A_n3A DC_8A_n28A DC_8A_n79A |
| DC_8A_n3A-n77A-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_8A_n3A-n77(2A)-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_8A-11A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A |
| DC_8A-11A_n1A-n77(2A) | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A |
| DC_8A-11A_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A |
| DC_8A-11A_n3A-n77A2 | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_8A-11A_n3A-n77(2A) 2 | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_8A-11A_n3A-n79A | DC_8A_n3A DC_8A_n79A DC_11A_n3A DC_11A_n79A |
| DC_8A-11A_n28A-n77A2 | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n28A-n77(2A) 2 | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n77A-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_8A-11A_n77(2A)-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_8A-20A-28A_n78A | DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_8A-20A-32A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_8A_n28A-n77A-n79A | DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_8A-20A-38A_n1A | DC_8A_n1A DC_20A_n1A DC_38A_n1A |
| DC_8A-32A-38A_n1A | DC_8A_n1A DC_38A_n1A |
| DC_8A_n39A-n40A-n41A | DC_8A_n39A DC_8A_n40A DC_8A_n41A |
| DC_8A_n39A-n40A-n79A | DC_8A_n39A DC_8A_n40A DC_8A_n79A |
| DC_8A_n40A-n41A-n79A | DC_8A_n40A DC_8A_n41A DC_8A_n79A |
| DC_8A-41A_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_41A_n1A DC_41A_n3A |
| DC_8A-41C_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_41A_n1A DC_41A_n3A |
| DC_8A-41A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A |
| DC_8A-41C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A |
| DC_8A-40A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_8A-40C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_8A-41A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_8A-41C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_8A-41A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41A_n77A |
| DC_8A-41C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41C_n3A DC_41A_n77A DC_41C_n77A |
| DC_8A-42A_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42A_n3A |
| DC_8A-42C_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42C_n1A DC_42A_n3A DC_42C_n3A |
| DC_8A-42A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A |
| DC_8A-42C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A DC_42C_n1A |
| DC_8A-42A_n3A-n28A2 | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A |
| DC_8A-42C_n3A-n28A2 | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_8A-42A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A |
| DC_8A-42A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A |
| DC_8A-42C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A |
| DC_8A-42C_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A |
| DC_8A-42A_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_8A-42A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_8A-42C_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_8A-42C_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_12A-30A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_12A-30A-66A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_11A_n3A-n28A-n77A2 | DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_11A_n3A-n28A-n77(2A) 2 | DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_11A_n3A-n77A-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_11A_n3A-n77(2A)-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_12A-30A-66A_n66A | DC_12A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_12A-30A-66A_n77A9 DC_12A-30A-66A-66A_n77A9 | DC_12A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_12A-48A-(n)5AA | DC_12A_n5A DC_48A_n5A DC_(n)5AA4 |
| DC_12A-48A-66A_n5A | DC_12A_n5A DC_48A_n5A DC_66A_n5A |
| DC_12A-66A-(n)5AA | DC_12A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_12A-66A_n2A-n78A | DC_12A_n2A DC_66A_n2A DC_12A_n78A DC_66A_n78A |
| DC_13A-48A-66A_n77A9 DC_13A-48C-66A_n77A9 DC_13A-48A-66A_n77C9 DC_13A-48C-66A_n77C9 | DC_13A_n77A DC_66A_n77A |
| DC_13A-66A_n2A-n77A9 DC_13A-66A-66A_n2A-n77A9 DC_13A-66A_n2A-n77C9 | DC_13A_n2A DC_13A_n77A9 DC_66A_n2A DC_66A_n77A9 |
| DC_13A-66A_n5A-n48A | DC_13A_n48A DC_66A_n5A DC_66A_n48A |
| DC_13A-66A_n5A-n77A9 DC_13A-66A-66A_n5A-n77A9 DC_13A-66A_n5A-n77C9 DC_13A-66A-66A_n5A-n77C9 | DC_66A_n5A DC_13A_n77A DC_66A_n77A |
| DC_13A-66A_n66A-n77A9 DC_13A-66A_n66A-n77C | DC_13A_n66A DC_13A_n77A9 DC_66A_n77A9 |
| DC_14A-30A-66A_n2A | DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| DC_14A-30A-66A-66A_n2A | DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| DC_14A-30A-66A_n66A | DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_14A-30A-66A_n77A9 DC_14A-30A-66A-66A_n77A9 | DC_14A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_18A-41A_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A |
| DC_18A-41C_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_18A-41A_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A |
| DC_18A-41C_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_19A_n1A-n77A-n79A | DC_19A_n1A DC_19A_n77A DC_19A_n79A |
| DC_19A_n1A-n78A-n79A | DC_19A_n1A DC_19A_n78A DC_19A_n79A |
| DC_19A-21A_n1A-n77A2 | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A |
| DC_19A-21A_n1A-n78A2 | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A |
| DC_19A-21A_n1A-n79A2 | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A |
| DC_19A-21A-42A_n1A2 DC_19A-21A-42C_n1A2 | DC_19A_n1A DC_21A_n1A DC_42A_n1A |
| DC_19A-21A-42A_n77A DC_19A-21A-42A_n77C DC_19A-21A-42C_n77A DC_19A-21A-42C_n77C | DC_19A_n77A DC_21A_n77A |
| DC_19A-21A-42A_n78A DC_19A-21A-42A_n78C DC_19A-21A-42C_n78A DC_19A-21A-42C_n78C | DC_19A_n78A DC_21A_n78A |
| DC_19A-21A-42A_n79A DC_19A-21A-42A_n79C DC_19A-21A-42C_n79A DC_19A-21A-42C_n79C | DC_19A_n79A DC_21A_n79A |
| DC_19A-21A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A-21A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_19A-42A_n1A-n77A DC_19A-42C_n1A-n77A | DC_19A_n1A DC_19A_n77A |
| DC_19A-42A_n1A-n78A DC_19A-42C_n1A-n78A | DC_19A_n1A DC_19A_n78A |
| DC_19A-42A_n1A-n79A DC_19A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A |
| DC_19A-42A_n77A-n79A DC_19A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A-42A_n78A-n79A DC_19A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_20A-28A-32A_n1A | DC_20A_n1A DC_28A_n1A |
| DC_20A-28A-32A_n3A | DC_20A_n3A DC_28A_n3A |
| DC_20A-28A-38A_n1A | DC_20A_n1A DC_28A_n1A DC_38A_n1A |
| DC_20A-32A_n1A-n28A | DC_20A_n1A DC_20A_n28A |
| DC_20A-32A-38A_n1A | DC_20A_n1A DC_38A_n1A |
| DC_20A-38A_n3A-n78A | DC_20A_n3A DC_20A_n78A DC_38A_n3A DC_38A_n78A |
| DC_20A-41A_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_20A-41C_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_21A_n1A-n77A-n79A | DC_21A_n1A DC_21A_n77A DC_21A_n79A |
| DC_21A_n1A-n78A-n79A | DC_21A_n1A DC_21A_n78A DC_21A_n79A |
| DC_21A-28A-42A_n77A DC_21A-28A-42C_n77A | DC_21A_n77A DC_28A_n77A |
| DC_21A-28A-42A_n78A DC_21A-28A-42C_n78A | DC_21A_n78A DC_28A_n78A |
| DC_21A-28A-42A_n79A DC_21A-28A-42C_n79A | DC_21A_n79A DC_28A_n79A |
| DC_21A_n28A-n77A-n79A | DC_21A_n28A DC_21A_n77A DC_21A_n79A |
| DC_21A_n28A-n78A-n79A | DC_21A_n28A DC_21A_n78A DC_21A_n79A |
| DC_21A-42A_n1A-n77A DC_21A-42C_n1A-n77A | DC_21A_n1A DC_21A_n77A |
| DC_21A-42A_n1A-n78A DC_21A-42C_n1A-n78A | DC_21A_n1A DC_21A_n78A |
| DC_21A-42A_n1A-n79A DC_21A-42C_n1A-n79A | DC_21A_n1A DC_21A_n79A |
| DC_21A-42A_n77A-n79A DC_21A-42C_n77A-n79A | DC_21A_n77A DC_21A_n79A |
| DC_21A-42A_n78A-n79A DC_21A-42C_n78A-n79A | DC_21A_n78A DC_21A_n79A |
| DC_28A-32A-38A_n1A | DC_28A_n1A DC_38A_n1A |
| DC_28A-41A-42A_n78A DC_28A-41C-42A_n78A DC_28A-41A-42C_n78A DC_28A-41C-42C_n78A | DC_28A_n78A DC_41A_n78A DC_41C_n78A |
| DC_29A-30A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_29A-30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_29A-30A-66A_n66A | DC_30A_n66A DC_66A_n66A4 |
| DC_29A-30A-66A_n77A9 | DC_30A_n77A9 DC_66A_n77A9 |
| DC_30A-66A-(n)5AA | DC_30A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_42A_n1A-n77A-n79A7,8 | N/A |
| DC_42A_n1A-n78A-n79A7,8 | N/A |
| DC_42A_n3A-n28A-n77A7,8 | DC_42A_n3A DC_42A_n28A |
| DC_42A_n3A-n28A-n77(2A)7,8 | DC_42A_n3A DC_42A_n28A |
| DC_42C_n3A-n28A-n77A7,8 | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_42C_n3A-n28A-n77(2A)7,8 | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_46A-66A_n25A-n41A DC_46C-66A_n25A-n41A DC_46D-66A_n25A-n41A | DC_66A_n25A DC_66A_n41A |
| DC_46A-66A_n25A-n71A DC_46C-66A_n25A-n71A DC_46D-66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |
| DC_46A-66A_n41A-n71A DC_46C-66A_n41A-n71A DC_46D-66A_n41A-n71A | DC_66A_n41A DC_66A_n71A |
| DC_46A-66A_n41(2A)-n71A DC_46C-66A_n41(2A)-n71A DC_46D-66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |
| DC_48A-66A_n25A-n48A | DC_48A_n25A DC_66A_n25A DC_66A_n48A |
| DC_66A-71A_n2A-n78A | DC_66A_n2A DC_71A_n2A DC_66A_n78A DC_71A_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: UL carrier shall be supported in Band 2 or band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 6: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 8: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 9: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 10: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 11: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 12: Void. NOTE 13: Power imbalance between downlink carriers on Band 7 and band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |

5.5B.4.4	Inter-band EN-DC configurations within FR1 (five bands)

Table 5.5B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A_n78A DC_1A-3C-5A-7A_n78A DC_1A-3A-5A-7A_n78C | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A-7A_n78A DC_1A-3A-5A-7A-7A_n78C | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-1A-3A-5A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-41A_n79A | DC_1A_n79A DC_3A_n79A DC_5A_n79A DC_41A_n79A |
| DC_1A-3A-7A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7C_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n5A-n78A DC_1A-3C-7A_n5A-n78A DC_1A-3A-7C_n5A-n78A DC_1A-3C-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3C-7A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_1A-3A-7A-8A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_8A_n28A |
| DC_1A-3A-7A-8A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-7A-8A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-7A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_1A-3A-7A-20A_n8A | DC_1A_n8A DC_3A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-3A-7A-20A_n28A3 | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-3A-7A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A-20A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-3C-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-3A-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-3C-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A-28A_n3A DC_1A-3A-7C-28A_n3A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_1A-3A-7A-28A_n5A DC_1A-3C-7A-28A_n5A DC_1A-3A-7C-28A_n5A DC_1A-3C-7C-28A_n5A | DC_1A_n5A DC_3A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_1A-3A-7A-28A_n7A DC_1A-3C-7A-28A_n7A DC_1A-3A-3A-7A-28A_n7A DC_1A-1A-3C-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-1A-3A-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-1A-3A-3A-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-3A-7A_n28A-n38A | DC_1A_n28A7 DC_3A_n28A7 |
| DC_1A-3A-7A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A DC_28A_n40A |
| DC_1A-3A-7A-28A_n78A DC_1A-3A-7C-28A_n78A DC_1A-3C-7A-28A_n78A DC_1A-3C-7C-28A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-1A-3A-7A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-7A_n28A-n78A2 DC_1A-3A-7C_n28A-n78A DC_1A-3C-7A_n28A-n78A DC_1A-3C-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_1A-3A-7A-32A_n28A DC_1A-3C-7A-32A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A |
| DC_1A-3A-7A-32A_n78A DC_1A-3C-7A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_1A-3A-7A-38A_n28A7 DC_1A-3C-7A-38A_n28A7 | DC_1A_n28A DC_3A_n28A |
| DC_1A-3A-7A_n38A-n78A7 | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-7A-40A_n78A DC_1A-3A-7A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-3A-7A-40A_n78(2A) DC_1A-3A-7A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-3A-7A_n75A-n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-8A-40A_n78A DC_1A-3A-8A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-8A-40A_n78(2A) DC_1A-3A-8A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-7A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-3A-8A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A DC_11A_n28A |
| DC_1A-3A-8A-11A_n77A2 DC_1A-3A-8A-11A_n77(2A) 2 DC_1A-3A-8A-11A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-3A-8A-20A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-3A-8A_n28A-n77A2 DC_1A-3A-8A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-3A-8A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_28A_n78A |
| DC_1A-3A-8A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-3A-8A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A |
| DC_1A-3A-8A-42A_n77A DC_1A-3A-8A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_8A_n77A |
| DC_1A-3A-8A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_3A_n77A DC_3A_n79A DC_8A_n77A DC_8A_n79A |
| DC_1A-3A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_18A_n3A DC_18A_n41A |
| DC_1A-3A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A DC_18A_n3A DC_18A_n77A |
| DC_1A-3A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A DC_18A_n3A DC_18A_n78A |
| DC_1A-3A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A |
| DC_1A-3A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A1 DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-3A-18A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A1 DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-3A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A1 DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-3A-18A_n28A-n78(2A) | DC_1A_n28A DC_1A_n78A DC_3A_n28A1 DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-3A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-3A-18A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-3A-18A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_1A-3A-18A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_1A-3A-18A-42A_n77A DC_1A-3A-18A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A-42A_n78A DC_1A-3A-18A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A-42A_n79A DC_1A-3A-18A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DC_1A-3A-19A-21A_n77A2 DC_1A-3A-19A-21A_n77C2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-3A-19A-21A_n78A2 DC_1A-3A-19A-21A_n78C2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-3A-19A-21A_n79A2 DC_1A-3A-19A-21A_n79C2 | DC_1A_n79A DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-3A-19A-42A_n77A5,6 DC_1A-3A-19A-42A_n77C5,6 DC_1A-3A-19A-42C_n77A5,6 DC_1A-3A-19A-42C_n77C5,6 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A-42A_n78A5,6 DC_1A-3A-19A-42A_n78C5,6 DC_1A-3A-19A-42C_n78A5,6 DC_1A-3A-19A-42C_n78C5,6 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A-42A_n79A DC_1A-3A-19A-42A_n79C DC_1A-3A-19A-42C_n79A DC_1A-3A-19A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A |
| DC_1A-3A-20A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_20A_n7A DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-3A-20A_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3C-20A_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3A-20A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-3A-20A_n28A-n78A2,3,6,11 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-3A-20A-32A_n28A6,11 DC_1A-3C-20A-32A_n28A,6,11 | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3A-20A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A-38A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_1A_n38A DC_3A_n38A DC_20A_n38A |
| DC_1A-3A-20A-38A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A-40A_n78A DC_1A-3A-20A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_40A_n78A |
| DC_1A-3A-20A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_20A_n41A DC_20A_n78A |
| DC_1A-3A-21A-42A_n77A5,6 DC_1A-3A-21A-42A_n77C5,6 DC_1A-3A-21A-42C_n77A5,6 DC_1A-3A-21A-42C_n77C5,6 | DC_1A_n77A DC_3A_n77A DC_21A_n77A |
| DC_1A-3A-21A-42A_n78A5,6 DC_1A-3A-21A-42A_n78C5,6 DC_1A-3A-21A-42C_n78A5,6 DC_1A-3A-21A-42C_n78C5,6 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A-42A_n79A DC_1A-3A-21A-42A_n79C DC_1A-3A-21A-42C_n79A DC_1A-3A-21A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_21A_n79A |
| DC_1A-3A-21A_n77A-n79A | DC_3A_n77A DC_3A_n79A |
| DC_1A-3A-21A_n78A-n79A | DC_3A_n78A DC_3A_n79A |
| DC_1A-3A-28A_n3A-n78A2 | DC_1A_n3A DC_3A_n3A4 DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n5A-n78A2 DC_1A-3C-28A_n5A-n78A2 | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-3A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n7B-n78A | DC_1A_n7A DC_3A_n7A DC_28A_n7A DC_1A_n7B DC_3A_n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3C-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_1A-3C-28A_n7B-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_1A_n7B DC_3A_n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_1A-3A-28A-40A_n78A DC_1A-3A-28A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A DC_40A_n78A |
| DC_1A-3A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-3A-28A-42A_n77A DC_1A-3A-28A-42A_n77C DC_1A-3A-28A-42C_n77A DC_1A-3A-28A-42C_n77C | DC_1A_n77A DC_3A_n77A DC_28A_n77A |
| DC_1A-3A-28A-42A_n78A DC_1A-3A-28A-42A_n78C DC_1A-3A-28A-42C_n78A DC_1A-3A-28A-42C_n78C | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A-42A_n79A DC_1A-3A-28A-42A_n79C DC_1A-3A-28A-42C_n79A DC_1A-3A-28A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_28A_n79A |
| DC_1A-3A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_3A_n28A DC_3A_n77A DC_3A_n79A |
| DC_1A_n3A-n28A-n77A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_1A_n79A |
| DC_1A-3A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A DC_3A_n28A DC_3A_n78A DC_3A_n79A |
| DC_1A-3A-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A-3C-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_1A-3A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A |
| DC_1A-3A-41C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_1A-3A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A |
| DC_1A-3A-41C_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_1A-3A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_41A_n28A |
| DC_1A-3A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A |
| DC_1A-3A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_1A-3A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |
| DC_1A-3A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_1A-3A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-41A-42A_n77A5,6 DC_1A-3A-41A-42C_n77A5,6 DC_1A-3A-41C-42A_n77A5,6 DC_1A-3A-41C-42C_n77A5,6 | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A-42A_n77(2A)5,6 DC_1A-3A-41A-42C_n77(2A)5,6 | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A-42A_n78A5,6 DC_1A-3A-41A-42C_n78A5,6 DC_1A-3A-41C-42A_n78A5,6 DC_1A-3A-41C-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-41A-42A_n79A DC_1A-3A-41A-42C_n79A DC_1A-3A-41C-42A_n79A DC_1A-3A-41C-42C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-42A_n28A-n77A5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_1A-3A-42A_n28A-n77(2A)5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_1A-3A-42C_n28A-n77A5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-3A-42C_n28A-n77(2A)5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-7A-8A-20A_n3A | DC_1A_n3A DC_7A_n3A DC_8A_n3A DC_20A_n3A |
| DC_1A-7A-8A-20A_n28A3,9 | DC_1A_n28A DC_7A_n28A DC_8A_n28A DC_20A_n28A |
| DC_1A-7A-8A-20A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-7A-8A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-7A-8A-32A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A-8A-40A_n78A DC_1A-7A-8A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-7A-8A-40A_n78(2A) DC_1A-7A-8A-40C_n78(2A) | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-7A-20A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_20A_n3A DC_20A_n78A |
| DC_1A-7A-20A_n3A-n38A | DC_1A_n3A DC_20A_n3A |
| DC_1A-7A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-7A-20A-28A_n3A DC_1A-7C-20A-28A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-7A-20A_n28A-n78A2,3 | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-7A-20A-32A_n3A DC_1A-7C-20A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A |
| DC_1A-7A-20A-32A_n28A | DC_1A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-7A-20A-32A_n78A | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-20A-32A_n8A | DC_1A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-7A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-7A-20A-38A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-7A-20A-38A_n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-7A-20A_n38A-n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-7A-28A_n3A-n78A DC_1A-7C-28A_n3A-n78A | DC_1A_n3A DC_7A_n3A DC_28A_n3A DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A-28A_n5A-n78A DC_1A-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-7A-28A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A-28A-32A_n3A DC_1A-7C-28A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_28A_n3A |
| DC_1A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-7A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A |
| DC_1A-8A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A DC_8A_n3A DC_8A_n28A DC_8A_n79A |
| DC_1A-8A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A |
| DC_1A-8A-11A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-8A-11A_n3A-n77(2A) 2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-8A-11A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_8A_n3A DC_8A_n79A DC_11A_n3A DC_11A_n79A |
| DC_1A-8A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-11A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-8A-11A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-8A-20A-28A_n78A | DC_1A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-8A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A-42A_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A |
| DC_1A-8A-42C_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A |
| DC_1A-8A-42A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A |
| DC_1A-8A-42C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-8A-42C_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-8A-42A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_1A-8A-42A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_1A-8A-42C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42C_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-11A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_1A-11A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_1A-19A-21A-42A_n77A5,6 DC_1A-19A-21A-42A_n77C5,6 DC_1A-19A-21A-42C_n77A5,6 DC_1A-19A-21A-42C_n77C5,6 | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A-42A_n78A5,6 DC_1A-19A-21A-42A_n78C5,6 DC_1A-19A-21A-42C_n78A5,6 DC_1A-19A-21A-42C_n78C5,6 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A-42A_n79A DC_1A-19A-21A-42A_n79C DC_1A-19A-21A-42C_n79A DC_1A-19A-21A-42C_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-19A-42A_n77A-n79A5,6 DC_1A-19A-42C_n77A-n79A5,6 | DC_19A_n77A DC_19A_n79A |
| DC_1A-19A-42A_n78A-n79A5,6 DC_1A-19A-42C_n78A-n79A5,6 | DC_1A_n78A DC_1A_n79A DC_19A_n78A DC_19A_n79A |
| DC_1A-20A-28A-32A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-20A-38A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_38A_n3A DC_1A_n78A DC_20A_n78A DC_38A_n78A |
| DC_1A-21A-28A-42A_n77A5,6 DC_1A-21A-28A-42C_n77A5,6 | DC_1A_n77A DC_21A_n77A DC_28A_n77A |
| DC_1A-21A-28A-42A_n78A5,6 DC_1A-21A-28A-42C_n78A5,6 | DC_1A_n78A DC_21A_n78A DC_28A_n78A |
| DC_1A-21A-28A-42A_n79A DC_1A-21A-28A-42C_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A |
| DC_1A-21A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_21A_n28A DC_21A_n77A DC_21A_n79A |
| DC_1A-21A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A DC_21A_n28A DC_21A_n78A DC_21A_n79A |
| DC_1A-21A-42A_n77A-n79A5,6 DC_1A-21A-42C_n77A-n79A5,6 | DC_1A_n77A DC_1A_n79A |
| DC_1A-21A-42A_n78A-n79A5,6 DC_1A-21A-42C_n78A-n79A5,6 | DC_1A_n78A DC_1A_n79A DC_21A_n78A DC_21A_n79A |
| DC_1A-42A_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-42A_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-42C_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_1A-42C_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_2A-5A-7A-66A_n2A | DC_5A_n2A DC_7A_n2A DC_66A_n2A |
| DC_2A-5A-7A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-5A-7A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-5A-7A-66A_n66A DC_2A-5A-7C-66A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-5A-7A-7A-66A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-5A-7A-66A_n78A | DC_2A_n78A DC_5A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-5A-30A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_30A_n2A DC_66A_n2A |
| DC_2A-5A-30A-66A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-5A-30A-66A_n77A8 | DC_2A_n77A8 DC_5A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-5A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-5A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-5A-66A_n2A-n77A8 DC_2A-5A-66A-66A_n2A-n77A8 | DC_2A_n77A8 DC_5A_n77A8 DC_66A_n77A8 |
| DC_2A-5A-66A_n66A-n77A8 | DC_2A_n66A DC_2A_n77A8 DC_5A_n66A DC_5A_n77A8 DC_66A_n77A8 |
| DC_2A-7A-12A-66A_n2A | DC_7A_n2A DC_12A_n2A DC_66A_n2A |
| DC_2A-7A-12A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-2A-7A-12A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-7A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-7C-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-7A-13A-66A_n66A DC_2A-7C-13A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-7A-7A-13A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-7A-28A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A DC_66A_n7A |
| DC_2A-7A-28A-66A_n66A DC_2A-7C-28A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A DC_66A_n66A4 |
| DC_2A-7A-29A-66A_n78A DC_2A-7C-29A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-29A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-66A_n25A-n66A6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_2A-7A-7A-66A_n25A-n66A6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_2A-7C-66A_n25A-n66A,6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_2A-7A-66A_n66A-n77A DC_2A-7A-7A-66A_n66A-n77A DC_2A-7C-66A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-66A_n66A-n78A DC_2A-7C-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_2A-7A-7A-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_2A-7A-66A-71A_n2A | DC_7A_n2A DC_66A_n2A DC_71A_n2A |
| DC_2A-7A-66A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-2A-7A-66A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-12A-30A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_2A-12A-30A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-12A-30A-66A_n77A8 | DC_2A_n77A8 DC_12A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-13A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-13A-66A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-13A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-13A-66A_n66A-n77A8 DC_2A-2A-13A-66A_n66A-n77A8 | DC_2A_n66A DC_2A_n77A8 DC_13A_n66A DC_13A_n77A8 DC_66A_n77A8 |
| DC_2A-14A-30A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| DC_2A-14A-30A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-14A-30A-66A_n77A8 | DC_2A_n77A8 DC_14A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-29A-30A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-29A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-29A-30A-66A_n77A8 | DC_2A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-30A-66A-(n)5AA | DC_2A_n5A DC_30A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_2A-46A-66A_n41A-n71A DC_2A-46C-66A_n41A-n71A DC_2A-46D-66A_n41A-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_3A-7A-8A_n1A-n40A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_3A_n40A DC_7A_n40A DC_8A_n40A |
| DC_3A-7A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-3A-7A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-7A-7A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-3A-7A-7A-8A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-7A-8A-20A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_20A_n1A |
| DC_3A-7A-8A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_3A-7A-8A-32A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-8A-32A_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_3A-7A-8A-40A_n1A DC_3A-7A-8A-40C_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_40A_n1A |
| DC_3A-7A-8A-40A_n78A DC_3A-7A-8A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-7A-8A-40A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-7A-8A-40C_n78(2A) | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-7A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A |
| DC_3A-7A-20A_n1A-n75A | DC_3A_n1A DC_7A_n1A DC_20A_n1A |
| DC_3A-7A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |
| DC_3C-7A-20A_n1A-n78A | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |
| DC_3A-7A-20A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_3A-7A-20A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_20A_n1A DC_28A_n1A |
| DC_3A-7A-20A_n28A-n78A2,3,6,11 DC_3C-7A-20A_n28A-n78A2,3,6,11 | DC_3A_n28A DC_3A_n78A DC_3C_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-7A-20A-32A_n1A | DC_3A_n1A DC_7A_n1A DC_20A_n1A |
| DC_3A-7A-20A-32A_n78A | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-20A_n38A-n78A | DC_3A_n78A DC_20A_n78A |
| DC_3A-7A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A |
| DC_3A-7A-28A_n1A-n78A | DC_3A_n1A DC_7A_n1A DC_28A_n1A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7A-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7C-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_3A-7A-28A_n5A-n78A DC_3C-7A-28A_n5A-n78A DC_3A-7C-28A_n5A-n78A DC_3C-7C-28A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_3A-7A-28A_n7A-n78A | DC_3A_n7A DC_7A_n7A4 DC_28A_n7A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3C-7A-28A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_3A-7A-32A_n1A-n78A | DC_3A_n1A DC_7A_n1A DC_3A_n78A DC_7A_n78A |
| DC_3A-7A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-7A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-8A-11A_n28A-n77A2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-8A-11A_n28A-n77(2A) 2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-8A-20A-28A_n78A | DC_3A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_3A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-8A-41A_n1A-n78A DC_3A-3A-8A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-8A-41C_n1A-n78A DC_3A-3A-8A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-19A-21A-42A_n77A5,6 DC_3A-19A-21A-42A_n77C5,6 DC_3A-19A-21A-42C_n77A5,6 DC_3A-19A-21A-42C_n77C5,6 | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A-42A_n78A5,6 DC_3A-19A-21A-42A_n78C5,6 DC_3A-19A-21A-42C_n78A5,6 DC_3A-19A-21A-42C_n78C5,6 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A-42A_n79A DC_3A-19A-21A-42A_n79C DC_3A-19A-21A-42C_n79A DC_3A-19A-21A-42C_n79C | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-42A_n1A-n77A5,6 DC_3A-19A-42C_n1A-n77A5,6 | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A |
| DC_3A-19A-42A_n1A-n78A5,6 DC_3A-19A-42C_n1A-n78A5,6 | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A |
| DC_3A-19A-42A_n1A-n79A DC_3A-19A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A |
| DC_3A-20A-32A_n1A-n28A | DC_3A_n1A DC_20A_n1A DC_3A_n28A DC_20A_n28A |
| DC_3C-20A-32A_n1A-n28A | DC_3A_n1A DC_3C_n1A DC_20A_n1A DC_3A_n28A DC_20A_n28A |
| DC_3A-21A_n1A-n77A-n79A | DC_3A_n1A DC_3A_n77A DC_3A_n79A DC_21A_n1A DC_21A_n77A DC_21A_n79A |
| DC_3A-21A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A DC_21A_n1A DC_21A_n78A DC_21A_n79A |
| DC_3A-21A_n28A-n77A-n79A | DC_3A_n28A DC_3A_n77A DC_3A_n79A DC_21A_n28A DC_21A_n77A DC_21A_n79A |
| DC_3A-7A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_3A-3A-7A_n1A-n8A-n78A2 DC_3A-7A-7A_n1A-n8A-n78A2 DC_3A-3A-7A-7A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_3A-20A-41A_n1A-n78A DC_3A-3A-20A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-20A-41C_n1A-n78A DC_3A-3A-20A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-21A_n28A-n78A-n79A | DC_3A_n28A DC_3A_n78A DC_3A_n79A DC_21A_n28A DC_21A_n78A DC_21A_n79A |
| DC_3A-21A-42A_n1A-n77A5,6 DC_3A-21A-42C_n1A-n77A5,6 | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A |
| DC_3A-21A-42A_n1A-n78A5,6 DC_3A-21A-42C_n1A-n78A5,6 | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A |
| DC_3A-21A-42A_n1A-n79A DC_3A-21A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A |
| DC_3A-28A-41A-42A_n78A5,6 DC_3A-28A-41A-42C_n78A5,6 DC_3A-28A-41C-42A_n78A5,6 DC_3A-28A-41C-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_7A-8A-20A-32A_n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |
| DC_7A-8A-20A-38A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_7A-8A-32A-38A_n1A | DC_8A_n1A |
| DC_7A-8A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-8A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-20A-28A-32A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |
| DC_7A-20A-28A-32A_n3A DC_7C-20A-28A-32A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_7A-20A-28A-38A_n1A | DC_20A_n1A DC_28A_n1A |
| DC_7A-20A-32A-38A_n1A | DC_20A_n1A |
| DC_7A-20A-38A_n3A-n78A | DC_20A_n3A DC_20A_n78A |
| DC_7A-28A-32A-38A_n1A | DC_28A_n1A |
| DC_8A_n3A-n28A-n77A-n79A | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_8A-11A_n3A-n28A-n77A2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n3A-n28A-n77(2A) 2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n3A-n77A-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_8A-20A-32A-38A_n1A | DC_8A_n1A DC_20A_n1A DC_38A_n1A |
| DC_8A-42A_n3A-n28A-n77A5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_8A-42A_n3A-n28A-n77(2A)5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_8A-42C_n3A-n28A-n77A5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_8A-42C_n3A-n28A-n77(2A)5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_19A-21A-42A_n1A-n77A5,6 DC_19A-21A-42C_n1A-n77A5,6 | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A |
| DC_19A-21A-42A_n1A-n78A5,6 DC_19A-21A-42C_n1A-n78A5,6 | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A |
| DC_19A-21A-42A_n1A-n79A DC_19A-21A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A |
| DC_19A-21A-42A_n77A-n79A5,6 DC_19A-21A-42C_n77A-n79A5,6 | DC_19A_n77A DC_19A_n79A |
| DC_19A-21A-42A_n78A-n79A5,6 DC_19A-21A-42C_n78A-n79A5,6 | DC_19A_n78A DC_19A_n79A |
| DC_19A-42A_n1A-n77A-n79A5,6 | DC_19A_n1A DC_19A_n77A DC_19A_n79A |
| DC_19A-42A_n1A-n78A-n79A5,6 | DC_19A_n1A DC_19A_n78A DC_19A_n79A |
| DC_20A-28A-32A-38A_n1A | DC_20A_n1A DC_28A_n1A DC_38A_n1A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 4: Only single switched UL is supported NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 7: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 8: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 9: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 10: Void. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |

5.5B.4.5	Inter-band EN-DC configurations within FR1 (six bands)

Table 5.5B.4.5-1: Inter-band EN-DC configurations within FR1 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-7A-8A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-3A-7A-8A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-7A-8A-40A_n78A DC_1A-3A-7A-8A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-7A-8A-40A_n78(2A) DC_1A-3A-7A-8A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-7A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-3A-7A-20A_n28A-n78A2,3,6,7 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-3A-7A-20A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-7A-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-7C-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-3A-7A-28A_n5A-n78A DC_1A-3A-7C-28A_n5A-n78A DC_1A-3C-7A-28A_n5A-n78A DC_1A-3C-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-3A-7A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3C-7A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-7A-28A_n38A-n78A | DC_1A_n78A8 DC_3A_n78A8 DC_28A_n78A8 |
| DC_1A-3A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-3A-8A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-8A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-8A-20A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-7A-20A-28A-32A_n3A DC_1A-7C-20A-28A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-7A-20A-38A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A |
| DC_1A-8A_n3A-n28A-n77A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A-11A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-11A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-42A_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-8A-42A_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-8A-42C_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42C_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_3A-7A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-7A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-8A-20A-32A-38A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_7A-20A-28A-32A-38A_n1A | DC_20A_n1A DC_28A_n1A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 8: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

<<Unchanged parts skipped>>

5.5B.5.2	Inter-band EN-DC configurations including FR2 (three bands)

Table 5.5B.5.2-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n257A2 DC_1A-3A_n257D2 DC_1A-3A_n257E2 DC_1A-3A_n257F2 DC_1A-3A_n257G DC_1A-3A_n257H DC_1A-3A_n257I DC_1A-3A_n257J DC_1A-3A_n257K DC_1A-3A_n257L DC_1A-3A_n257M DC_1A-3C_n257A DC_1A-3C_n257D DC_1A-3C_n257E DC_1A-3C_n257F DC_1A-3C_n257G DC_1A-3C_n257H DC_1A-3C_n257I DC_1A-3C_n257J DC_1A-3C_n257K DC_1A-3C_n257L DC_1A-3C_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M |
| DC_1A-3A_n258A DC_1A-3A_n258B DC_1A-3A_n258C DC_1A-3A_n258D DC_1A-3A_n258E DC_1A-3A_n258F DC_1A-3A_n258G DC_1A-3A_n258H DC_1A-3A_n258I DC_1A-3A_n258J DC_1A-3A_n258K DC_1A-3A_n258L DC_1A-3A_n258M DC_1A-3C_n258A DC_1A-3C_n258B DC_1A-3C_n258C DC_1A-3C_n258D DC_1A-3C_n258E DC_1A-3C_n258F DC_1A-3C_n258G DC_1A-3C_n258H DC_1A-3C_n258I DC_1A-3C_n258J DC_1A-3C_n258K DC_1A-3C_n258L DC_1A-3C_n258M | DC_1A_n258A DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_3A_n258A DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3C_n258A DC_3C_n258D DC_3C_n258E DC_3C_n258F DC_3C_n258G DC_3C_n258H DC_3C_n258I |
| DC_1A-5A_n257A2 DC_1A-5A_n257D DC_1A-5A_n257E DC_1A-5A_n257F DC_1A-5A_n257G DC_1A-5A_n257H DC_1A-5A_n257I DC_1A-5A_n257J DC_1A-5A_n257K DC_1A-5A_n257L DC_1A-5A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I |
| DC_1A-7A_n257A2 DC_1A-7A_n257D DC_1A-7A_n257E DC_1A-7A_n257F DC_1A-7A_n257G DC_1A-7A_n257H DC_1A-7A_n257I DC_1A-7A_n257J DC_1A-7A_n257K DC_1A-7A_n257L DC_1A-7A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_1A-7A-7A_n257A2 DC_1A-7A-7A_n257D DC_1A-7A-7A_n257E DC_1A-7A-7A_n257F DC_1A-7A-7A_n257G DC_1A-7A-7A_n257H DC_1A-7A-7A_n257I DC_1A-7A-7A_n257J DC_1A-7A-7A_n257K DC_1A-7A-7A_n257L DC_1A-7A-7A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_1A-7A_n258A DC_1A-7A_n258B DC_1A-7A_n258C DC_1A-7A_n258D DC_1A-7A_n258E DC_1A-7A_n258F DC_1A-7A_n258G DC_1A-7A_n258H DC_1A-7A_n258I DC_1A-7A_n258J DC_1A-7A_n258K DC_1A-7A_n258L DC_1A-7A_n258M DC_1A-7C_n258A DC_1A-7C_n258B DC_1A-7C_n258C DC_1A-7C_n258D DC_1A-7C_n258E DC_1A-7C_n258F DC_1A-7C_n258G DC_1A-7C_n258H DC_1A-7C_n258I DC_1A-7C_n258J DC_1A-7C_n258K DC_1A-7C_n258L DC_1A-7C_n258M | DC_1A_n258A DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_7A_n258A DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n258A DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I |
| DC_1A-8A_n257A2 DC_1A-8A_n257D DC_1A-8A_n257E DC_1A-8A_n257F DC_1A-8A_n257G DC_1A-8A_n257H DC_1A-8A_n257I DC_1A-8A_n257J DC_1A-8A_n257K DC_1A-8A_n257L DC_1A-8A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_1A-8A_n258A DC_1A-8A_n258D DC_1A-8A_n258E DC_1A-8A_n258F DC_1A-8A_n258G DC_1A-8A_n258H DC_1A-8A_n258I DC_1A-8A_n258J DC_1A-8A_n258K DC_1A-8A_n258L DC_1A-8A_n258M | DC_1A_n258A DC_1A_n258D DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_8A_n258A DC_8A_n258D DC_8A_n258G DC_8A_n258H DC_8A_n258I |
| DC_1A-11A_n257A DC_1A-11A_n257D DC_1A-11A_n257G DC_1A-11A_n257H DC_1A-11A_n257I | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-18A_n257A2 DC_1A-18A_n257D DC_1A-18A_n257E DC_1A-18A_n257F DC_1A-18A_n257G DC_1A-18A_n257H DC_1A-18A_n257I DC_1A-18A_n257J DC_1A-18A_n257K DC_1A-18A_n257L DC_1A-18A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-19A_n257A2 DC_1A-19A_n257D2 DC_1A-19A_n257E2 DC_1A-19A_n257F2 DC_1A-19A_n257G DC_1A-19A_n257H DC_1A-19A_n257I DC_1A-19A_n257J DC_1A-19A_n257K DC_1A-19A_n257L DC_1A-19A_n257M | DC_1A_n257A DC_1A_257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-21A_n257A2 DC_1A-21A_n257D2 DC_1A-21A_n257E2 DC_1A-21A_n257F2 DC_1A-21A_n257G DC_1A-21A_n257H DC_1A-21A_n257I DC_1A-21A_n257J DC_1A-21A_n257K DC_1A-21A_n257L DC_1A-21A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_1A-28A_n257A2 DC_1A-28A_n257D2 DC_1A-28A_n257E2 DC_1A-28A_n257F2 DC_1A-28A_n257G2 DC_1A-28A_n257H2 DC_1A-28A_n257I2 DC_1A-28A_n257J DC_1A-28A_n257K DC_1A-28A_n257L DC_1A-28A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-28A_n258A DC_1A-28A_n258B DC_1A-28A_n258C DC_1A-28A_n258D DC_1A-28A_n258E DC_1A-28A_n258F DC_1A-28A_n258G DC_1A-28A_n258H DC_1A-28A_n258I DC_1A-28A_n258J DC_1A-28A_n258K DC_1A-28A_n258L DC_1A-28A_n258M | DC_1A_n258A DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_1A-41A_n257A DC_1A-41A_n257D DC_1A-41A_n257E DC_1A-41A_n257F DC_1A-41A_n257G DC_1A-41A_n257H DC_1A-41A_n257I DC_1A-41A_n257J DC_1A-41A_n257K DC_1A-41A_n257L DC_1A-41A_n257M DC_1A-41C_n257A DC_1A-41C_n257D DC_1A-41C_n257E DC_1A-41C_n257F DC_1A-41C_n257G DC_1A-41C_n257H DC_1A-41C_n257I DC_1A-41C_n257J DC_1A-41C_n257K DC_1A-41C_n257L DC_1A-41C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-42A_n257A DC_1A-42A_n257D DC_1A-42A_n257E DC_1A-42A_n257F DC_1A-42A_n257G DC_1A-42A_n257H DC_1A-42A_n257I DC_1A-42A_n257J DC_1A-42A_n257K DC_1A-42A_n257L DC_1A-42A_n257M DC_1A-42C_n257A DC_1A-42C_n257D DC_1A-42C_n257E DC_1A-42C_n257F DC_1A-42C_n257G DC_1A-42C_n257H DC_1A-42C_n257I DC_1A-42C_n257J DC_1A-42C_n257K DC_1A-42C_n257L DC_1A-42C_n257M DC_1A-42D_n257A DC_1A-42D_n257D DC_1A-42D_n257E DC_1A-42D_n257F DC_1A-42D_n257G DC_1A-42D_n257H DC_1A-42D_n257I DC_1A-42D_n257J DC_1A-42D_n257K DC_1A-42D_n257L DC_1A-42D_n257M DC_1A-42E_n257A DC_1A-42E_n257D DC_1A-42E_n257E DC_1A-42E_n257F DC_1A-42E_n257G DC_1A-42E_n257H DC_1A-42E_n257I DC_1A-42E_n257J DC_1A-42E_n257K DC_1A-42E_n257L DC_1A-42E_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_2A-5A_n257A2 | DC_2A_n257A DC_5A_n257A |
| DC_2A-5A_n260A DC_2A-5A_n260G DC_2A-5A_n260H DC_2A-5A_n260I DC_2A-5A_n260J DC_2A-5A_n260K DC_2A-5A_n260L DC_2A-5A_n260M | DC_2A_n260A DC_5A_n260A DC_2A_n260G DC_5A_n260G DC_2A_n260H DC_5A_n260H DC_2A_n260I DC_5A_n260I DC_2A_n260J DC_5A_n260J DC_2A_n260K DC_5A_n260K DC_2A_n260L DC_5A_n260L DC_2A_n260M DC_5A_n260M |
| DC_2A-2A-5A_n260A DC_2A-2A-5A_n260G DC_2A-2A-5A_n260H DC_2A-2A-5A_n260I DC_2A-2A-5A_n260J DC_2A-2A-5A_n260K DC_2A-2A-5A_n260L DC_2A-2A-5A_n260M | DC_2A_n260A DC_5A_n260A DC_2A_n260G DC_5A_n260G DC_2A_n260H DC_5A_n260H DC_2A_n260I DC_5A_n260I DC_2A_n260J DC_5A_n260J DC_2A_n260K DC_5A_n260K DC_2A_n260L DC_5A_n260L DC_2A_n260M DC_5A_n260M |
| DC_2A-5A_n261A DC_2A-5A_n261G DC_2A-5A_n261H DC_2A-5A_n261I DC_2A-5A_n261J DC_2A-5A_n261K DC_2A-5A_n261L DC_2A-5A_n261M | DC_2A_n261A DC_5A_n261A DC_2A_n261G DC_5A_n261G DC_2A_n261H DC_5A_n261H DC_2A_n261I DC_5A_n261I |
| DC_2A-5A_n261(2A) DC_2A-5A_n261(3A) DC_2A-5A_n261(4A) DC_2A-5A_n261(A-G) DC_2A-5A_n261(A-H) DC_2A-5A_n261(A-I) DC_2A-5A_n261(A-J) DC_2A-5A_n261(A-K) DC_2A-5A_n261(2A-G) DC_2A-5A_n261(A-L) DC_2A-5A_n261(A-2G) DC_2A-5A_n261(2A-H) DC_2A-5A_n261(2A-I) DC_2A-5A_n261(A-G-H) DC_2A-5A_n261(A-G-I) DC_2A-5A_n261(3A-G) DC_2A-5A_n261(G-H) DC_2A-5A_n261(G-I) DC_2A-5A_n261(G-J) DC_2A-5A_n261(2G) DC_2A-5A_n261(2H) DC_2A-5A_n261(H-I) | DC_2A_n261A DC_5A_n261A DC_2A_n261G DC_5A_n261G DC_2A_n261H DC_5A_n261H DC_2A_n261I DC_5A_n261I DC_2A_n261J DC_5A_n261J DC_2A_n261K DC_5A_n261K DC_2A_n261L DC_5A_n261L DC_2A_n261M DC_5A_n261M |
| DC_2A-12A_n260A DC_2A-12A_n260G DC_2A-12A_n260H DC_2A-12A_n260I DC_2A-12A_n260J DC_2A-12A_n260K DC_2A-12A_n260L DC_2A-12A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_12A_n260A DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M |
| DC_2A-2A-12A_n260A DC_2A-2A-12A_n260G DC_2A-2A-12A_n260H DC_2A-2A-12A_n260I DC_2A-2A-12A_n260J DC_2A-2A-12A_n260K DC_2A-2A-12A_n260L DC_2A-2A-12A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_12A_n260A DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M |
| DC_2A-13A_n257A2 | DC_2A_n257A DC_13A_n257A |
| DC_2A-13A_n260A2 DC_2A-13A_n260G DC_2A-13A_n260H DC_2A-13A_n260I DC_2A-13A_n260J DC_2A-13A_n260K DC_2A-13A_n260L DC_2A-13A_n260M | DC_2A_n260A DC_13A_n260A DC_2A_n260G DC_13A_n260G DC_2A_n260H DC_13A_n260H DC_2A_n260I DC_13A_n260I DC_2A_n260J DC_13A_n260J DC_2A_n260K DC_13A_n260K DC_2A_n260L DC_13A_n260L DC_2A_n260M DC_13A_n260M |
| DC_2A-13A_n260(2A) DC_2A-13A_n260(3A) DC_2A-13A_n260(4A) DC_2A-13A_n260(5A) DC_2A-13A_n260(6A) DC_2A-13A_n260(2G) DC_2A-13A_n260(2H) DC_2A-13A_n260(A-G) DC_2A-13A_n260(A-H) DC_2A-13A_n260(A-2G) DC_2A-13A_n260(2A-G) DC_2A-13A_n260(2A-2G) DC_2A-13A_n260(3A-G) DC_2A-13A_n260(G-H) | DC_2A_n260A DC_13A_n260A DC_2A_n260G DC_13A_n260G DC_2A_n260H DC_13A_n260H |
| DC_2A-13A_n261A DC_2A-13A_n261G DC_2A-13A_n261H DC_2A-13A_n261I DC_2A-13A_n261J DC_2A-13A_n261K DC_2A-13A_n261L DC_2A-13A_n261M | DC_2A_n261A DC_13A_n261A DC_2A_n261G DC_13A_n261G DC_2A_n261H DC_13A_n261H DC_2A_n260I DC_13A_n260I DC_2A_n260J DC_13A_n260J DC_2A_n260K DC_13A_n260K DC_2A_n260L DC_13A_n260L DC_2A_n260M DC_13A_n260M |
| DC_2A-2A-13A_n261A DC_2A-2A-13A_n261G DC_2A-2A-13A_n261H DC_2A-2A-13A_n261I DC_2A-2A-13A_n261J DC_2A-2A-13A_n261K DC_2A-2A-13A_n261L DC_2A-2A-13A_n261M | DC_2A_n261A DC_13A_n261A DC_2A_n261G DC_13A_n261G DC_2A_n261H DC_13A_n261H DC_2A_n261I DC_13A_n261I DC_2A_n260J DC_13A_n260J DC_2A_n260K DC_13A_n260K DC_2A_n260L DC_13A_n260L DC_2A_n260M DC_13A_n260M |
| DC_2A-13A_n261(2A) DC_2A-13A_n261(3A) DC_2A-13A_n261(4A) DC_2A-13A_n261(2G) DC_2A-13A_n261(2H) DC_2A-13A_n261(A-G) DC_2A-13A_n261(A-H) DC_2A-13A_n261(A-I) DC_2A-13A_n261(A-J) DC_2A-13A_n261(A-K) DC_2A-13A_n261(A-L) DC_2A-13A_n261(A-2G) DC_2A-13A_n261(A-G-H) DC_2A-13A_n261(A-G-I) DC_2A-13A_n261(2A-G) DC_2A-13A_n261(2A-H) DC_2A-13A_n261(2A-I) DC_2A-13A_n261(3A-G) DC_2A-13A_n261(G-H) DC_2A-13A_n261(G-I) DC_2A-13A_n261(G-J) DC_2A-13A_n261(H-I) | DC_2A_n261A DC_13A_n261A DC_2A_n261G DC_13A_n261G DC_2A_n261H DC_13A_n261H DC_2A_n261I DC_13A_n261I |
| DC_2A-14A_n260A DC_2A-14A_n260G DC_2A-14A_n260H DC_2A-14A_n260I DC_2A-14A_n260J DC_2A-14A_n260K DC_2A-14A_n260L DC_2A-14A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M |
| DC_2A-2A-14A_n260A DC_2A-2A-14A_n260G DC_2A-2A-14A_n260H DC_2A-2A-14A_n260I DC_2A-2A-14A_n260J DC_2A-2A-14A_n260K DC_2A-2A-14A_n260L DC_2A-2A-14A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M |
| DC_2A-29A_n260A DC_2A-29A_n260G DC_2A-29A_n260H DC_2A-29A_n260I DC_2A-29A_n260J DC_2A-29A_n260K DC_2A-29A_n260L DC_2A-29A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |
| DC_2A-2A-29A_n260A DC_2A-2A-29A_n260G DC_2A-2A-29A_n260H DC_2A-2A-29A_n260I DC_2A-2A-29A_n260J DC_2A-2A-29A_n260K DC_2A-2A-29A_n260L DC_2A-2A-29A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |
| DC_2A-30A_n260A DC_2A-30A_n260G DC_2A-30A_n260H DC_2A-30A_n260I DC_2A-30A_n260J DC_2A-30A_n260K DC_2A-30A_n260L DC_2A-30A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_2A-2A-30A_n260A DC_2A-2A-30A_n260G DC_2A-2A-30A_n260H DC_2A-2A-30A_n260I DC_2A-2A-30A_n260J DC_2A-2A-30A_n260K DC_2A-2A-30A_n260L DC_2A-2A-30A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_2A-46A_n258A DC_2A-46C_n258A DC_2A-46D_n258A | DC_2A_n258A |
| DC_2A-46A_n258(2A) DC_2A-46A_n258(3A) DC_2A-46A_n258(4A) DC_2A-46A_n258(5A) DC_2A-46C_n258(2A) DC_2A-46C_n258(3A) DC_2A-46C_n258(4A) DC_2A-46C_n258(5A) DC_2A-46D_n258(2A) DC_2A-46D_n258(3A) DC_2A-46D_n258(4A) DC_2A-46D_n258(5A) | DC_2A_n258A |
| DC_2A-46A_n260A DC_2A-46C_n260A DC_2A-46D_n260A DC_2A-46E_n260A DC_2A-46A_n260G DC_2A-46C_n260G DC_2A-46D_n260G DC_2A-46E_n260G DC_2A-46A_n260H DC_2A-46C_n260H DC_2A-46D_n260H DC_2A-46E_n260H DC_2A-46A_n260I DC_2A-46C_n260I DC_2A-46D_n260I DC_2A-46E_n260I DC_2A-46A_n260J DC_2A-46C_n260J DC_2A-46D_n260J DC_2A-46E_n260J DC_2A-46A_n260K DC_2A-46C_n260K DC_2A-46D_n260K DC_2A-46E_n260K DC_2A-46A_n260L DC_2A-46C_n260L DC_2A-46D_n260L DC_2A-46E_n260L DC_2A-46A_n260M DC_2A-46C_n260M DC_2A-46D_n260M DC_2A-46E_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |
| DC_2A-2A-46A_n260A DC_2A-2A-46C_n260A DC_2A-2A-46D_n260A DC_2A-2A-46E_n260A DC_2A-2A-46A_n260G DC_2A-2A-46C_n260G DC_2A-2A-46D_n260G DC_2A-2A-46E_n260G DC_2A-2A-46A_n260H DC_2A-2A-46C_n260H DC_2A-2A-46D_n260H DC_2A-2A-46E_n260H DC_2A-2A-46A_n260I DC_2A-2A-46C_n260I DC_2A-2A-46D_n260I DC_2A-2A-46E_n260I DC_2A-2A-46A_n260J DC_2A-2A-46C_n260J DC_2A-2A-46D_n260J DC_2A-2A-46E_n260J DC_2A-2A-46A_n260K DC_2A-2A-46C_n260K DC_2A-2A-46D_n260K DC_2A-2A-46E_n260K DC_2A-2A-46A_n260L DC_2A-2A-46C_n260L DC_2A-2A-46D_n260L DC_2A-2A-46E_n260L DC_2A-2A-46A_n260M DC_2A-2A-46C_n260M DC_2A-2A-46D_n260M DC_2A-2A-46E_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |
| DC_2A-46A_n261(A-G) DC_2A-46A_n261(A-H) DC_2A-46A_n261(A-I) DC_2A-46A_n261(A-J) DC_2A-46A_n261(A-K) DC_2A-46A_n261(A-L) DC_2A-46A_n261(G-H) DC_2A-46A_n261(2H) DC_2A-46A_n261(2G) DC_2A-46A_n261(2A) DC_2A-46C_n261(2A) DC_2A-46D_n261(2A) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A-46A-46A_n261(A-G) DC_2A-46A-46A_n261(A-H) DC_2A-46A-46A_n261(A-I) DC_2A-46A-46A_n261(A-J) DC_2A-46A-46A_n261(A-K) DC_2A-46A-46A_n261(A-L) DC_2A-46A-46A_n261(G-H) DC_2A-46A-46A_n261(2A) DC_2A-46A-46A_n261(2G) DC_2A-46A-46A_n261(2H) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A-46A-46A-46A_n261(A-G) DC_2A-46A-46A-46A_n261(A-H) DC_2A-46A-46A-46A_n261(A-I) DC_2A-46A-46A-46A_n261(A-J) DC_2A-46A-46A-46A_n261(A-K) DC_2A-46A-46A-46A_n261(A-L) DC_2A-46A-46A-46A_n261(G-H) DC_2A-46A-46A-46A_n261(2A) DC_2A-46A-46A-46A_n261(2G) DC_2A-46A-46A-46A_n261(2H) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A-46A_n261A DC_2A-46A_n261G DC_2A-46A_n261H DC_2A-46A_n261I DC_2A-46A_n261J DC_2A-46A_n261K DC_2A-46A_n261L DC_2A-46A_n261M DC_2A-46C_n261A DC_2A-46D_n261A | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M |
| DC_2A-46A-46A_n261A DC_2A-46A-46A_n261G DC_2A-46A-46A_n261H DC_2A-46A-46A_n261I DC_2A-46A-46A_n261J DC_2A-46A-46A_n261K DC_2A-46A-46A_n261L DC_2A-46A-46A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M |
| DC_2A-46A-46A-46A_n261A DC_2A-46A-46A-46A_n261G DC_2A-46A-46A-46A_n261H DC_2A-46A-46A-46A_n261I DC_2A-46A-46A-46A_n261J DC_2A-46A-46A-46A_n261K DC_2A-46A-46A-46A_n261L DC_2A-46A-46A-46A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M |
| DC_2A-66A_n257A2 DC_2A-66A_n257(2A) | DC_2A_n257A DC_66A_n257A |
| DC_2A-66A_n260A DC_2A-66A_n260G DC_2A-66A_n260H DC_2A-66A_n260I DC_2A-66A_n260J DC_2A-66A_n260K DC_2A-66A_n260L DC_2A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M |
| DC_2A-66A_n260(2A) DC_2A-66A_n260(3A) DC_2A-66A_n260(4A) DC_2A-66A_n260(5A) DC_2A-66A_n260(6A) DC_2A-66A_n260(2G) DC_2A-66A_n260(2H) DC_2A-66A_n260(A-G) DC_2A-66A_n260(A-H) DC_2A-66A_n260(A-2G) DC_2A-66A_n260(2A-G) DC_2A-66A_n260(2A-2G) DC_2A-66A_n260(3A-G) DC_2A-66A_n260(G-H) | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H |
| DC_2A-2A-66A_n260A DC_2A-2A-66A_n260G DC_2A-2A-66A_n260H DC_2A-2A-66A_n260I DC_2A-2A-66A_n260J DC_2A-2A-66A_n260K DC_2A-2A-66A_n260L DC_2A-2A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M |
| DC_2A-66A-66A_n260A DC_2A-66A-66A_n260G DC_2A-66A-66A_n260H DC_2A-66A-66A_n260I DC_2A-66A-66A_n260J DC_2A-66A-66A_n260K DC_2A-66A-66A_n260L DC_2A-66A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M |
| DC_2A-2A-66A-66A_n260A DC_2A-2A-66A-66A_n260G DC_2A-2A-66A-66A_n260H DC_2A-2A-66A-66A_n260I DC_2A-2A-66A-66A_n260J DC_2A-2A-66A-66A_n260K DC_2A-2A-66A-66A_n260L DC_2A-2A-66A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M |
| DC_2A-66A_n261A DC_2A-66A_n261G DC_2A-66A_n261H DC_2A-66A_n261I DC_2A-66A_n261J DC_2A-66A_n261K DC_2A-66A_n261L DC_2A-66A_n261M | DC_2A_n261A DC_66A_n261A DC_2A_n261G DC_66A_n261G DC_2A_n261H DC_66A_n261H DC_2A_n261I DC_66A_n261I DC_2A_n261J DC_66A_n261J DC_2A_n261K DC_66A_n261K DC_2A_n261L DC_66A_n261L DC_2A_n261M DC_66A_n261M |
| DC_2A-2A-66A_n261A DC_2A-2A-66A_n261G DC_2A-2A-66A_n261H DC_2A-2A-66A_n261I DC_2A-2A-66A_n261J DC_2A-2A-66A_n261K DC_2A-2A-66A_n261L DC_2A-2A-66A_n261M | DC_2A_n261A DC_66A_n261A DC_2A_n261G DC_66A_n261G DC_2A_n261H DC_66A_n261H DC_2A_n261I DC_66A_n261I DC_2A_n261J DC_66A_n261J DC_2A_n261K DC_66A_n261K DC_2A_n261L DC_66A_n261L DC_2A_n261M DC_66A_n261M |
| DC_2A-66A-66A_n261A DC_2A-66A-66A_n261G DC_2A-66A-66A_n261H DC_2A-66A-66A_n261I DC_2A-66A-66A_n261J DC_2A-66A-66A_n261K DC_2A-66A-66A_n261L DC_2A-66A-66A_n261M | DC_2A_n261A DC_66A_n261A DC_2A_n261G DC_66A_n261G DC_2A_n261H DC_66A_n261H DC_2A_n261I DC_66A_n261I DC_2A_n261J DC_66A_n261J DC_2A_n261K DC_66A_n261K DC_2A_n261L DC_66A_n261L DC_2A_n261M DC_66A_n261M |
| DC_2A-66A_n261(2A) DC_2A-66A_n261(3A) DC_2A-66A_n261(4A) DC_2A-66A_n261(2G) DC_2A-66A_n261(2H) DC_2A-66A_n261(A-G) DC_2A-66A_n261(A-H) DC_2A-66A_n261(A-I) DC_2A-66A_n261(A-J) DC_2A-66A_n261(A-K) DC_2A-66A_n261(A-L) DC_2A-66A_n261(A-2G) DC_2A-66A_n261(A-G-H) DC_2A-66A_n261(A-G-I) DC_2A-66A_n261(2A-G) DC_2A-66A_n261(2A-H) DC_2A-66A_n261(2A-I) DC_2A-66A_n261(3A-G) DC_2A-66A_n261(G-H) DC_2A-66A_n261(G-I) DC_2A-66A_n261(G-J) DC_2A-66A_n261(H-I) | DC_2A_n261A DC_66A_n261A DC_2A_n261G DC_66A_n261G DC_2A_n261H DC_66A_n261H DC_2A_n261I DC_66A_n261I |
| DC_2A-66A-66A_n261(2A) DC_2A-66A-66A_n261(3A) DC_2A-66A-66A_n261(4A) DC_2A-66A-66A_n261(A-G) DC_2A-66A-66A_n261(A-H) DC_2A-66A-66A_n261(A-I) DC_2A-66A-66A_n261(A-J) DC_2A-66A-66A_n261(A-K) DC_2A-66A-66A_n261(A-L) DC_2A-66A-66A_n261(A-2G) DC_2A-66A-66A_n261(2A-G) DC_2A-66A-66A_n261(2A-H) DC_2A-66A-66A_n261(2A-I) DC_2A-66A-66A_n261(A-G-H) DC_2A-66A-66A_n261(A-G-I) DC_2A-66A-66A_n261(3A-G) DC_2A-66A-66A_n261(2G) DC_2A-66A-66A_n261(G-H) DC_2A-66A-66A_n261(G-I) DC_2A-66A-66A_n261(G-J) DC_2A-66A-66A_n261(2H) DC_2A-66A-66A_n261(H-I) | DC_2A_n261A DC_66A_n261A DC_2A_n261G DC_66A_n261G DC_2A_n261H DC_66A_n261H DC_2A_n261I DC_66A_n261I |
| DC_3A-3A-7A_n257A DC_3A-3A-7A_n257D DC_3A-3A-7A_n257E DC_3A-3A-7A_n257F DC_3A-3A-7A_n257G DC_3A-3A-7A_n257H DC_3A-3A-7A_n257I DC_3A-3A-7A_n257J DC_3A-3A-7A_n257K DC_3A-3A-7A_n257L DC_3A-3A-7A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-3A-7A-7A_n257A DC_3A-3A-7A-7A_n257D DC_3A-3A-7A-7A_n257E DC_3A-3A-7A-7A_n257F DC_3A-3A-7A-7A_n257G DC_3A-3A-7A-7A_n257H DC_3A-3A-7A-7A_n257I DC_3A-3A-7A-7A_n257J DC_3A-3A-7A-7A_n257K DC_3A-3A-7A-7A_n257L DC_3A-3A-7A-7A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_(n)3AA-n257A DC_(n)3AA-n257G DC_(n)3AA-n257H DC_(n)3AA-n257I DC_(n)3AA-n257J DC_(n)3AA-n257K DC_(n)3AA-n257L DC_(n)3AA-n257M | DC _(n)3AA3 DC_3A_n257A |
| DC_3A-5A_n257A2 DC_3A-5A_n257D DC_3A-5A_n257E DC_3A-5A_n257F DC_3A-5A_n257G DC_3A-5A_n257H DC_3A-5A_n257I DC_3A-5A_n257J DC_3A-5A_n257K DC_3A-5A_n257L DC_3A-5A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I |
| DC_3A-7A_n257A2 DC_3A-7A_n257D DC_3A-7A_n257E DC_3A-7A_n257F DC_3A-7A_n257G DC_3A-7A_n257H DC_3A-7A_n257I DC_3A-7A_n257J DC_3A-7A_n257K DC_3A-7A_n257L DC_3A-7A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-7A_n258A DC_3A-7A_n258B DC_3A-7A_n258C DC_3A-7A_n258D DC_3A-7A_n258E DC_3A-7A_n258F DC_3A-7A_n258G DC_3A-7A_n258H DC_3A-7A_n258I DC_3A-7A_n258J DC_3A-7A_n258K DC_3A-7A_n258L DC_3A-7A_n258M DC_3C-7A_n258A DC_3C-7A_n258B DC_3C-7A_n258C DC_3C-7A_n258D DC_3C-7A_n258E DC_3C-7A_n258F DC_3C-7A_n258G DC_3C-7A_n258H DC_3C-7A_n258I DC_3C-7A_n258J DC_3C-7A_n258K DC_3C-7A_n258L DC_3C-7A_n258M DC_3A-7C_n258A DC_3A-7C_n258B DC_3A-7C_n258C DC_3A-7C_n258D DC_3A-7C_n258E DC_3A-7C_n258F DC_3A-7C_n258G DC_3A-7C_n258H DC_3A-7C_n258I DC_3A-7C_n258J DC_3A-7C_n258K DC_3A-7C_n258L DC_3A-7C_n258M DC_3C-7C_n258A DC_3C-7C_n258B DC_3C-7C_n258C DC_3C-7C_n258D DC_3C-7C_n258E DC_3C-7C_n258F DC_3C-7C_n258G DC_3C-7C_n258H DC_3C-7C_n258I DC_3C-7C_n258J DC_3C-7C_n258K DC_3C-7C_n258L DC_3C-7C_n258M | DC_3A_n258A DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3C_n258A DC_3C_n258D DC_3C_n258E DC_3C_n258F DC_3C_n258G DC_3C_n258H DC_3C_n258I DC_7A_n258A DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n258A DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I |
| DC_3A-7A-7A_n257A2 DC_3A-7A-7A_n257D DC_3A-7A-7A_n257E DC_3A-7A-7A_n257F DC_3A-7A-7A_n257G DC_3A-7A-7A_n257H DC_3A-7A-7A_n257I DC_3A-7A-7A_n257J DC_3A-7A-7A_n257K DC_3A-7A-7A_n257L DC_3A-7A-7A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-8A_n257A DC_3A-8A_n257D DC_3A-8A_n257E DC_3A-8A_n257F DC_3A-8A_n257G DC_3A-8A_n257H DC_3A-8A_n257I DC_3A-8A_n257J DC_3A-8A_n257K DC_3A-8A_n257L DC_3A-8A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_3C-8A_n257A DC_3C-8A_n257D DC_3C-8A_n257E DC_3C-8A_n257F DC_3C-8A_n257G DC_3C-8A_n257H DC_3C-8A_n257I DC_3C-8A_n257J DC_3C-8A_n257K DC_3C-8A_n257L DC_3C-8A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_3A-3A-8A_n257A DC_3A-3A-8A_n257D DC_3A-3A-8A_n257E DC_3A-3A-8A_n257F DC_3A-3A-8A_n257G DC_3A-3A-8A_n257H DC_3A-3A-8A_n257I DC_3A-3A-8A_n257J DC_3A-3A-8A_n257K DC_3A-3A-8A_n257L DC_3A-3A-8A_n257M | DC_3A_n257A DC_8A_n257A |
| DC_3A-8A_n258A DC_3A-8A_n258D DC_3A-8A_n258E DC_3A-8A_n258F DC_3A-8A_n258G DC_3A-8A_n258H DC_3A-8A_n258I DC_3A-8A_n258J DC_3A-8A_n258K DC_3A-8A_n258L DC_3A-8A_n258M | DC_3A_n258A DC_8A_n258A |
| DC_3A-11A_n257A DC_3A-11A_n257G DC_3A-11A_n257H DC_3A-11A_n257I | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_3A-18A_n257A DC_3A-18A_n257D DC_3A-18A_n257E DC_3A-18A_n257F DC_3A-18A_n257G DC_3A-18A_n257H DC_3A-18A_n257I DC_3A-18A_n257J DC_3A-18A_n257K DC_3A-18A_n257L DC_3A-18A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_3A-19A_n257A2 DC_3A-19A_n257D2 DC_3A-19A_n257E2 DC_3A-19A_n257F2 DC_3A-19A_n257G DC_3A-19A_n257H DC_3A-19A_n257I DC_3A-19A_n257J DC_3A-19A_n257K DC_3A-19A_n257L DC_3A-19A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_3A-21A_n257A2 DC_3A-21A_n257D2 DC_3A-21A_n257E2 DC_3A-21A_n257F2 DC_3A-21A_n257G DC_3A-21A_n257H DC_3A-21A_n257I DC_3A-21A_n257J DC_3A-21A_n257K DC_3A-21A_n257L DC_3A-21A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_3A-28A_n257A2 DC_3A-28A_n257D2 DC_3A-28A_n257E2 DC_3A-28A_n257F2 DC_3A-28A_n257G DC_3A-28A_n257H DC_3A-28A_n257I DC_3A-28A_n257J DC_3A-28A_n257K DC_3A-28A_n257L DC_3A-28A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-28A_n258A DC_3A-28A_n258B DC_3A-28A_n258C DC_3A-28A_n258D DC_3A-28A_n258E DC_3A-28A_n258F DC_3A-28A_n258G DC_3A-28A_n258H DC_3A-28A_n258I DC_3A-28A_n258J DC_3A-28A_n258K DC_3A-28A_n258L DC_3A-28A_n258M | DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3A_n258J DC_3A_n258K DC_3A_n258L DC_3A_n258M DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I DC_28A_n258J DC_28A_n258K DC_28A_n258L DC_28A_n258M |
| DC_3C-28A_n258A DC_3C-28A_n258B DC_3C-28A_n258C DC_3C-28A_n258D DC_3C-28A_n258E DC_3C-28A_n258F DC_3C-28A_n258G DC_3C-28A_n258H DC_3C-28A_n258I DC_3C-28A_n258J DC_3C-28A_n258K DC_3C-28A_n258L DC_3C-28A_n258M | DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3A_n258J DC_3A_n258K DC_3A_n258L DC_3A_n258M DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I DC_28A_n258J DC_28A_n258K DC_28A_n258L DC_28A_n258M |
| DC_3A-41A_n257A DC_3A-41A_n257D DC_3A-41A_n257E DC_3A-41A_n257F DC_3A-41A_n257G DC_3A-41A_n257H DC_3A-41A_n257I DC_3A-41A_n257J DC_3A-41A_n257K DC_3A-41A_n257L DC_3A-41A_n257M DC_3A-41C_n257A DC_3A-41C_n257D DC_3A-41C_n257E DC_3A-41C_n257F DC_3A-41C_n257G DC_3A-41C_n257H DC_3A-41C_n257I DC_3A-41C_n257J DC_3A-41C_n257K DC_3A-41C_n257L DC_3A-41C_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-42A_n257A2 DC_3A-42A_n257D2 DC_3A-42A_n257E2 DC_3A-42A_n257F2 DC_3A-42A_n257G DC_3A-42A_n257H DC_3A-42A_n257I DC_3A-42A_n257J DC_3A-42A_n257K DC_3A-42A_n257L DC_3A-42A_n257M DC_3A-42C_n257A2 DC_3A-42C_n257D2 DC_3A-42C_n257E2 DC_3A-42C_n257F2 DC_3A-42C_n257G DC_3A-42C_n257H DC_3A-42C_n257I DC_3A-42C_n257J DC_3A-42C_n257K DC_3A-42C_n257L DC_3A-42C_n257M DC_3A-42D_n257A2 DC_3A-42D_n257D DC_3A-42D_n257E DC_3A-42D_n257F DC_3A-42D_n257G DC_3A-42D_n257H DC_3A-42D_n257I DC_3A-42D_n257J DC_3A-42D_n257K DC_3A-42D_n257L DC_3A-42D_n257M DC_3A-42E_n257A2 DC_3A-42E_n257D DC_3A-42E_n257E DC_3A-42E_n257F DC_3A-42E_n257G DC_3A-42E_n257H DC_3A-42E_n257I DC_3A-42E_n257J DC_3A-42E_n257K DC_3A-42E_n257L DC_3A-42E_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_5A-7A_n257A2 DC_5A-7A_n257D DC_5A-7A_n257E DC_5A-7A_n257F DC_5A-7A_n257G DC_5A-7A_n257H DC_5A-7A_n257I DC_5A-7A_n257J DC_5A-7A_n257K DC_5A-7A_n257L DC_5A-7A_n257M | DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_5A-7A-7A_n257A DC_5A-7A-7A_n257D DC_5A-7A-7A_n257E DC_5A-7A-7A_n257F DC_5A-7A-7A_n257G DC_5A-7A-7A_n257H DC_5A-7A-7A_n257I DC_5A-7A-7A_n257J DC_5A-7A-7A_n257K DC_5A-7A-7A_n257L DC_5A-7A-7A_n257M | DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I |
| DC_5A-30A_n260A DC_5A-30A_n260G DC_5A-30A_n260H DC_5A-30A_n260I DC_5A-30A_n260J DC_5A-30A_n260K DC_5A-30A_n260L DC_5A-30A_n260M | DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_5A-66A_n260A DC_5A-66A_n260G DC_5A-66A_n260H DC_5A-66A_n260I DC_5A-66A_n260J DC_5A-66A_n260K DC_5A-66A_n260L DC_5A-66A_n260M | DC_5A_n260A DC_66A_n260A DC_5A_n260G DC_66A_n260G DC_5A_n260H DC_66A_n260H DC_5A_n260I DC_66A_n260I DC_5A_n260J DC_66A_n260J DC_5A_n260K DC_66A_n260K DC_5A_n260L DC_66A_n260L DC_5A_n260M DC_66A_n260M |
| DC_5A-66A-66A_n260A DC_5A-66A-66A_n260G DC_5A-66A-66A_n260H DC_5A-66A-66A_n260I DC_5A-66A-66A_n260J DC_5A-66A-66A_n260K DC_5A-66A-66A_n260L DC_5A-66A-66A_n260M | DC_5A_n260A DC_66A_n260A DC_5A_n260G DC_66A_n260G DC_5A_n260H DC_66A_n260H DC_5A_n260I DC_66A_n260I DC_5A_n260J DC_66A_n260J DC_5A_n260K DC_66A_n260K DC_5A_n260L DC_66A_n260L DC_5A_n260M DC_66A_n260M |
| DC_5A-66A_n261A DC_5A-66A_n261G DC_5A-66A_n261H DC_5A-66A_n261I DC_5A-66A_n261J DC_5A-66A_n261K DC_5A-66A_n261L DC_5A-66A_n261M | DC_5A_n261A DC_66A_n261A DC_5A_n261G DC_66A_n261G DC_5A_n261H DC_66A_n261H DC_5A_n261I DC_66A_n261I DC_5A_n261J DC_66A_n26JI DC_5A_n261K DC_66A_n261K DC_5A_n261L DC_66A_n261L DC_5A_n261M DC_66A_n261M |
| DC_5A-66A-66A_n261A DC_5A-66A-66A_n261G DC_5A-66A-66A_n261H DC_5A-66A-66A_n261I DC_5A-66A-66A_n261J DC_5A-66A-66A_n261K DC_5A-66A-66A_n261L DC_5A-66A-66A_n261M | DC_5A_n261A DC_66A_n261A DC_5A_n261G DC_66A_n261G DC_5A_n261H DC_66A_n261H DC_5A_n261I DC_66A_n261I DC_5A_n261J DC_66A_n26JI DC_5A_n261K DC_66A_n261K DC_5A_n261L DC_66A_n261L DC_5A_n261M DC_66A_n261M |
| DC_5A-66A_n261(2A) DC_5A-66A_n261(3A) DC_5A-66A_n261(4A) DC_5A-66A_n261(2G) DC_5A-66A_n261(2H) DC_5A-66A_n261(A-G) DC_5A-66A_n261(A-H) DC_5A-66A_n261(A-I) DC_5A-66A_n261(A-J) DC_5A-66A_n261(A-K) DC_5A-66A_n261(A-L) DC_5A-66A_n261(A-2G) DC_5A-66A_n261(2A-G) DC_5A-66A_n261(2A-H) DC_5A-66A_n261(2A-I) DC_5A-66A_n261(A-G-H) DC_5A-66A_n261(A-G-I) DC_5A-66A_n261(3A-G) DC_5A-66A_n261(G-H) DC_5A-66A_n261(G-I) DC_5A-66A_n261(G-J) DC_5A-66A_n261(H-I) | DC_5A_n261A DC_66A_n261A DC_5A_n261G DC_66A_n261G DC_5A_n261H DC_66A_n261H DC_5A_n261I DC_66A_n261I |
| DC_5A-66A-66A_n261(2A) DC_5A-66A-66A_n261(3A) DC_5A-66A-66A_n261(4A) DC_5A-66A-66A_n261(A-G) DC_5A-66A-66A_n261(A-H) DC_5A-66A-66A_n261(A-I) DC_5A-66A-66A_n261(A-J) DC_5A-66A-66A_n261(A-K) DC_5A-66A-66A_n261(A-L) DC_5A-66A-66A_n261(A-2G) DC_5A-66A-66A_n261(2A-G) DC_5A-66A-66A_n261(2A-H) DC_5A-66A-66A_n261(A-G-H) DC_5A-66A-66A_n261(A-G-I) DC_5A-66A-66A_n261(2A-I) DC_5A-66A-66A_n261(3A-G) DC_5A-66A-66A_n261(2G) DC_5A-66A-66A_n261(G-H) DC_5A-66A-66A_n261(G-I) DC_5A-66A-66A_n261(G-J) DC_5A-66A-66A_n261(2H) DC_5A-66A-66A_n261(H-I) | DC_5A_n261A DC_66A_n261A DC_5A_n261G DC_66A_n261G DC_5A_n261H DC_66A_n261H DC_5A_n261I DC_66A_n261I |
| DC_7A-8A_n257A DC_7A-8A_n257D DC_7A-8A_n257E DC_7A-8A_n257F DC_7A-8A_n257G DC_7A-8A_n257H DC_7A-8A_n257I DC_7A-8A_n257J DC_7A-8A_n257K DC_7A-8A_n257L DC_7A-8A_n257M | DC_7A_n257A DC_8A_n257A |
| DC_7A-7A-8A_n257A DC_7A-7A-8A_n257D DC_7A-7A-8A_n257E DC_7A-7A-8A_n257F DC_7A-7A-8A_n257G DC_7A-7A-8A_n257H DC_7A-7A-8A_n257I DC_7A-7A-8A_n257J DC_7A-7A-8A_n257K DC_7A-7A-8A_n257L DC_7A-7A-8A_n257M | DC_7A_n257A DC_8A_n257A |
| DC_7A-8A_n258A DC_7A-8A_n258D DC_7A-8A_n258E DC_7A-8A_n258F DC_7A-8A_n258G DC_7A-8A_n258H DC_7A-8A_n258I DC_7A-8A_n258J DC_7A-8A_n258K DC_7A-8A_n258L DC_7A-8A_n258M | DC_7A_n258A DC_8A_n258A |
| DC_7A-28A_n257A DC_7A-28A_n257G DC_7A-28A_n257H DC_7A-28A_n257I | DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_7A-28A_n258A DC_7A-28A_n258B DC_7A-28A_n258C DC_7A-28A_n258D DC_7A-28A_n258E DC_7A-28A_n258F DC_7A-28A_n258G DC_7A-28A_n258H DC_7A-28A_n258I DC_7A-28A_n258J DC_7A-28A_n258K DC_7A-28A_n258L DC_7A-28A_n258M DC_7C-28A_n258A DC_7C-28A_n258B DC_7C-28A_n258C DC_7C-28A_n258D DC_7C-28A_n258E DC_7C-28A_n258F DC_7C-28A_n258G DC_7C-28A_n258H DC_7C-28A_n258I DC_7C-28A_n258J DC_7C-28A_n258K DC_7C-28A_n258L DC_7C-28A_n258M | DC_7A_n258A DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n258A DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_8A-11A_n257A DC_8A-11A_n257D DC_8A-11A_n257G DC_8A-11A_n257H DC_8A-11A_n257I | DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_11A-18A_n257A DC_11A-18A_n257G DC_11A-18A_n257H DC_11A-18A_n257I | DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_12A-30A_n260A DC_12A-30A_n260G DC_12A-30A_n260H DC_12A-30A_n260I DC_12A-30A_n260J DC_12A-30A_n260K DC_12A-30A_n260L DC_12A-30A_n260M | DC_12A_n260A DC_30A_n260A DC_12A_n260G DC_30A_n260G DC_12A_n260H DC_30A_n260H DC_12A_n260I DC_30A_n260I DC_12A_n260J DC_30A_n260J DC_12A_n260K DC_30A_n260K DC_12A_n260L DC_30A_n260L DC_12A_n260M DC_30A_n260M |
| DC_12A-66A_n260A DC_12A-66A_n260G DC_12A-66A_n260H DC_12A-66A_n260I DC_12A-66A_n260J DC_12A-66A_n260K DC_12A-66A_n260L DC_12A-66A_n260M | DC_12A_n260A DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_12A-66A-66A_n260A DC_12A-66A-66A_n260G DC_12A-66A-66A_n260H DC_12A-66A-66A_n260I DC_12A-66A-66A_n260J DC_12A-66A-66A_n260K DC_12A-66A-66A_n260L DC_12A-66A-66A_n260M | DC_12A_n260A DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_13A-66A_n257A2 | DC_13A_n257A DC_66A_n257A |
| DC_13A-66A_n260A DC_13A-66A_n260G DC_13A-66A_n260H DC_13A-66A_n260I DC_13A-66A_n260J DC_13A-66A_n260K DC_13A-66A_n260L DC_13A-66A_n260M | DC_13A_n260A DC_66A_n260A DC_13A_n260G DC_66A_n260G DC_13A_n260H DC_66A_n260H DC_13A_n260I DC_66A_n260I DC_13A_n260J DC_66A_n260J DC_13A_n260K DC_66A_n260K DC_13A_n260L DC_66A_n260L DC_13A_n260M DC_66A_n260M |
| DC_13A-66A-66A_n260A DC_13A-66A-66A_n260G DC_13A-66A-66A_n260H DC_13A-66A-66A_n260I DC_13A-66A-66A_n260J DC_13A-66A-66A_n260K DC_13A-66A-66A_n260L DC_13A-66A-66A_n260M | DC_13A_n260A DC_66A_n260A DC_13A_n260G DC_66A_n260G DC_13A_n260H DC_66A_n260H DC_13A_n260I DC_66A_n260I DC_13A_n260J DC_66A_n260J DC_13A_n260K DC_66A_n260K DC_13A_n260L DC_66A_n260L DC_13A_n260M DC_66A_n260M |
| DC_13A-66A_n260(2A) DC_13A-66A_n260(3A) DC_13A-66A_n260(4A) DC_13A-66A_n260(5A) DC_13A-66A_n260(6A) DC_13A-66A_n260(2G) DC_13A-66A_n260(2H) DC_13A-66A_n260(A-G) DC_13A-66A_n260(A-H) DC_13A-66A_n260(A-2G) DC_13A-66A_n260(2A-G) DC_13A-66A_n260(2A-2G) DC_13A-66A_n260(3A-G) DC_13A-66A_n260(G-H) | DC_13A_n260A DC_66A_n260A DC_13A_n260G DC_66A_n260G DC_13A_n260H DC_66A_n260H |
| DC_13A-66A-66A_n260(2A) DC_13A-66A-66A_n260(3A) DC_13A-66A-66A_n260(4A) DC_13A-66A-66A_n260(5A) DC_13A-66A-66A_n260(6A) DC_13A-66A-66A_n260(2G) DC_13A-66A-66A_n260(2H) DC_13A-66A-66A_n260(A-G) DC_13A-66A-66A_n260(A-H) DC_13A-66A-66A_n260(A-2G) DC_13A-66A-66A_n260(2A-G) DC_13A-66A-66A_n260(2A-2G) DC_13A-66A-66A_n260(3A-G) DC_13A-66A-66A_n260(G-H) | DC_13A_n260A DC_66A_n260A DC_13A_n260G DC_66A_n260G DC_13A_n260H DC_66A_n260H |
| DC_13A-66A_n261A DC_13A-66A_n261G DC_13A-66A_n261H DC_13A-66A_n261I DC_13A-66A_n261J DC_13A-66A_n261K DC_13A-66A_n261L DC_13A-66A_n261M | DC_13A_n261A DC_66A_n261A DC_13A_n261G DC_66A_n261G DC_13A_n261H DC_66A_n261H DC_13A_n261I DC_66A_n261I DC_13A_n261J DC_66A_n261J DC_13A_n261K DC_66A_n261K DC_13A_n261L DC_66A_n261L DC_13A_n261M DC_66A_n261M |
| DC_13A-66A-66A_n261A DC_13A-66A-66A_n261G DC_13A-66A-66A_n261H DC_13A-66A-66A_n261I DC_13A-66A-66A_n261J DC_13A-66A-66A_n261K DC_13A-66A-66A_n261L DC_13A-66A-66A_n261M | DC_13A_n261A DC_66A_n261A DC_13A_n261G DC_66A_n261G DC_13A_n261H DC_66A_n261H DC_13A_n261I DC_66A_n261I DC_13A_n261J DC_66A_n261J DC_13A_n261K DC_66A_n261K DC_13A_n261L DC_66A_n261L DC_13A_n261M DC_66A_n261M |
| DC_13A-66A_n261(2A) DC_13A-66A_n261(3A) DC_13A-66A_n261(4A) DC_13A-66A_n261(2G) DC_13A-66A_n261(2H) DC_13A-66A_n261(A-G) DC_13A-66A_n261(A-H) DC_13A-66A_n261(A-I) DC_13A-66A_n261(A-J) DC_13A-66A_n261(A-K) DC_13A-66A_n261(A-L) DC_13A-66A_n261(A-2G) DC_13A-66A_n261(A-G-H) DC_13A-66A_n261(A-G-I) DC_13A-66A_n261(2A-G) DC_13A-66A_n261(2A-H) DC_13A-66A_n261(2A-I) DC_13A-66A_n261(3A-G) DC_13A-66A_n261(G-H) DC_13A-66A_n261(G-I) DC_13A-66A_n261(G-J) DC_13A-66A_n261(H-I) | DC_13A_n261A DC_66A_n261A DC_13A_n261G DC_66A_n261G DC_13A_n261H DC_66A_n261H DC_13A_n261I DC_66A_n261I |
| DC_13A-66A-66A_n261(2A) DC_13A-66A-66A_n261(3A) DC_13A-66A-66A_n261(4A) DC_13A-66A-66A_n261(2G) DC_13A-66A-66A_n261(2H) DC_13A-66A-66A_n261(A-G) DC_13A-66A-66A_n261(A-H) DC_13A-66A-66A_n261(A-I) DC_13A-66A-66A_n261(A-J) DC_13A-66A-66A_n261(A-K) DC_13A-66A-66A_n261(A-L) DC_13A-66A-66A_n261(A-2G) DC_13A-66A-66A_n261(A-G-H) DC_13A-66A-66A_n261(A-G-I) DC_13A-66A-66A_n261(2A-G) DC_13A-66A-66A_n261(2A-H) DC_13A-66A-66A_n261(2A-I) DC_13A-66A-66A_n261(3A-G) DC_13A-66A-66A_n261(G-H) DC_13A-66A-66A_n261(G-I) DC_13A-66A-66A_n261(G-J) DC_13A-66A-66A_n261(H-I) | DC_13A_n261A DC_66A_n261A DC_13A_n261G DC_66A_n261G DC_13A_n261H DC_66A_n261H DC_13A_n261I DC_66A_n261I |
| DC_13A-46A_n261A DC_13A-46A_n261G DC_13A-46A_n261H DC_13A-46A_n261I DC_13A-46A_n261J DC_13A-46A_n261K DC_13A-46A_n261L DC_13A-46A_n261M | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I DC_13A_n261J DC_13A_n261K DC_13A_n261L DC_13A_n261M |
| DC_13A-46A-46A_n261A DC_13A-46A-46A_n261G DC_13A-46A-46A_n261H DC_13A-46A-46A_n261I DC_13A-46A-46A_n261J DC_13A-46A-46A_n261K DC_13A-46A-46A_n261L DC_13A-46A-46A_n261M | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I DC_13A_n261J DC_13A_n261K DC_13A_n261L DC_13A_n261M |
| DC_13A-46A_n261(A-G) DC_13A-46A_n261(A-H) DC_13A-46A_n261(G-H) DC_13A-46A_n261(2A) DC_13A-46A_n261(2G) DC_13A-46A_n261(2H) | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I |
| DC_13A-46A-46A_n261(A-G) DC_13A-46A-46A_n261(A-H) DC_13A-46A-46A_n261(G-H) DC_13A-46A-46A_n261(2A) DC_13A-46A-46A_n261(2G) DC_13A-46A-46A_n261(2H) | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I |
| DC_14A-30A_n260A DC_14A-30A_n260G DC_14A-30A_n260H DC_14A-30A_n260I DC_14A-30A_n260J DC_14A-30A_n260K DC_14A-30A_n260L DC_14A-30A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_14A-66A_n260A DC_14A-66A_n260G DC_14A-66A_n260H DC_14A-66A_n260I DC_14A-66A_n260J DC_14A-66A_n260K DC_14A-66A_n260L DC_14A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_14A-66A-66A_n260A DC_14A-66A-66A_n260G DC_14A-66A-66A_n260H DC_14A-66A-66A_n260I DC_14A-66A-66A_n260J DC_14A-66A-66A_n260K DC_14A-66A-66A_n260L DC_14A-66A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_18A-28A_n257A2 | DC_18A_n257A DC_28A_n257A |
| DC_18A-42A_n257A DC_18A-42A_n257D DC_18A-42A_n257E DC_18A-42A_n257F DC_18A-42A_n257G DC_18A-42A_n257H DC_18A-42A_n257I DC_18A-42A_n257J DC_18A-42A_n257K DC_18A-42A_n257L DC_18A-42A_n257M DC_18A-42C_n257A DC_18A-42C_n257D DC_18A-42C_n257E DC_18A-42C_n257F DC_18A-42C_n257G DC_18A-42C_n257H DC_18A-42C_n257I DC_18A-42C_n257J DC_18A-42C_n257K DC_18A-42C_n257L DC_18A-42C_n257M | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_18A-41A_n257A DC_18A-41A_n257G DC_18A-41A_n257H DC_18A-41A_n257I DC_18A-41C_n257A DC_18A-41C_n257G DC_18A-41C_n257H DC_18A-41C_n257I | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_19A-21A_n257A2 DC_19A-21A_n257D2 DC_19A-21A_n257E2 DC_19A-21A_n257F2 DC_19A-21A_n257G DC_19A-21A_n257H DC_19A-21A_n257I DC_19A-21A_n257J DC_19A-21A_n257K DC_19A-21A_n257L DC_19A-21A_n257M | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_19A-42A_n257A2 DC_19A-42A_n257D2 DC_19A-42A_n257E2 DC_19A-42A_n257F2 DC_19A-42A_n257G2 DC_19A-42A_n257H2 DC_19A-42A_n257I2 DC_19A-42A_n257J DC_19A-42A_n257K DC_19A-42A_n257L DC_19A-42A_n257M DC_19A-42C_n257A2 DC_19A-42C_n257D DC_19A-42C_n257E DC_19A-42C_n257F DC_19A-42C_n257G2 DC_19A-42C_n257H2 DC_19A-42C_n257I2 DC_19A-42C_n257J DC_19A-42C_n257K DC_19A-42C_n257L DC_19A-42C_n257M DC_19A-42D_n257A DC_19A-42D_n257D2 DC_19A-42D_n257E2 DC_19A-42D_n257F2 | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_21A-28A_n257A2 DC_21A-28A_n257D2 DC_21A-28A_n257E2 DC_21A-28A_n257F2 | DC_21A_n257A DC_21A_n257D DC_28A_n257A DC_28A_n257D |
| DC_21A-42A_n257A2 DC_21A-42A_n257D2 DC_21A-42A_n257E2 DC_21A-42A_n257F2 DC_21A-42A_n257G DC_21A-42A_n257H DC_21A-42A_n257I DC_21A-42A_n257J DC_21A-42A_n257K DC_21A-42A_n257L DC_21A-42A_n257M DC_21A-42C_n257A2 DC_21A-42C_n257D DC_21A-42C_n257E DC_21A-42C_n257F DC_21A-42C_n257G DC_21A-42C_n257H DC_21A-42C_n257I DC_21A-42C_n257J DC_21A-42C_n257K DC_21A-42C_n257L DC_21A-42C_n257M DC_21A-42D_n257A DC_21A-42D_n257D DC_21A-42D_n257E DC_21A-42D_n257F DC_21A-42D_n257G DC_21A-42D_n257H DC_21A-42D_n257I DC_21A-42D_n257J DC_21A-42D_n257K DC_21A-42D_n257L DC_21A-42D_n257M DC_21A-42E_n257A DC_21A-42E_n257D DC_21A-42E_n257E DC_21A-42E_n257F DC_21A-42E_n257G DC_21A-42E_n257H DC_21A-42E_n257I DC_21A-42E_n257J DC_21A-42E_n257K DC_21A-42E_n257L DC_21A-42E_n257M | DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_28A-41A_n257A DC_28A-41A_n257G DC_28A-41A_n257H DC_28A-41A_n257I DC_28A-41C_n257A DC_28A-41C_n257G DC_28A-41C_n257H DC_28A-41C_n257I | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_28A-42A_n257A2 DC_28A-42A_n257D2 DC_28A-42A_n257G2 DC_28A-42A_n257H2 DC_28A-42A_n257I2 DC_28A-42A_n257J DC_28A-42A_n257K DC_28A-42A_n257L DC_28A-42A_n257M DC_28A-42C_n257A2 DC_28A-42C_n257D2 DC_28A-42C_n257G2 DC_28A-42C_n257H2 DC_28A-42C_n257I2 DC_28A-42C_n257J DC_28A-42C_n257K DC_28A-42C_n257L DC_28A-42C_n257M | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_29A-30A_n260A DC_29A-30A_n260G DC_29A-30A_n260H DC_29A-30A_n260I DC_29A-30A_n260J DC_29A-30A_n260K DC_29A-30A_n260L DC_29A-30A_n260M | DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_29A-66A_n260A DC_29A-66A_n260G DC_29A-66A_n260H DC_29A-66A_n260I DC_29A-66A_n260J DC_29A-66A_n260K DC_29A-66A_n260L DC_29A-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_30A-66A_n260A DC_30A-66A_n260G DC_30A-66A_n260H DC_30A-66A_n260I DC_30A-66A_n260J DC_30A-66A_n260K DC_30A-66A_n260L DC_30A-66A_n260M | DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_30A-66A-66A_n260A DC_30A-66A-66A_n260G DC_30A-66A-66A_n260H DC_30A-66A-66A_n260I DC_30A-66A-66A_n260J DC_30A-66A-66A_n260K DC_30A-66A-66A_n260L DC_30A-66A-66A_n260M | DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_40A-42A_n257A DC_40A-42A_n257D DC_40A-42A_n257E DC_40A-42A_n257F DC_40A-42A_n257G DC_40A-42A_n257H DC_40A-42A_n257I DC_40A-42A_n257J DC_40A-42A_n257K DC_40A-42A_n257L DC_40A-42A_n257M DC_40A-42C_n257A DC_40A-42C_n257D DC_40A-42C_n257E DC_40A-42C_n257F DC_40A-42C_n257G DC_40A-42C_n257H DC_40A-42C_n257I DC_40A-42C_n257J DC_40A-42C_n257K DC_40A-42C_n257L DC_40A-42C_n257M DC_40A-40A-42A_n257A DC_40A-40A-42A_n257D DC_40A-40A-42A_n257E DC_40A-40A-42A_n257F DC_40A-40A-42A_n257G DC_40A-40A-42A_n257H DC_40A-40A-42A_n257I DC_40A-40A-42A_n257J DC_40A-40A-42A_n257K DC_40A-40A-42A_n257L DC_40A-40A-42A_n257M DC_40A-40A-42C_n257A DC_40A-40A-42C_n257D DC_40A-40A-42C_n257E DC_40A-40A-42C_n257F DC_40A-40A-42C_n257G DC_40A-40A-42C_n257H DC_40A-40A-42C_n257I DC_40A-40A-42C_n257J DC_40A-40A-42C_n257K DC_40A-40A-42C_n257L DC_40A-40A-42C_n257M DC_40A-42A-42A_n257A DC_40A-42A-42A_n257D DC_40A-42A-42A_n257E DC_40A-42A-42A_n257F DC_40A-42A-42A_n257G DC_40A-42A-42A_n257H DC_40A-42A-42A_n257I DC_40A-42A-42A_n257J DC_40A-42A-42A_n257K DC_40A-42A-42A_n257L DC_40A-42A-42A_n257M DC_40C-42A-42A_n257A DC_40C-42A-42A_n257D DC_40C-42A-42A_n257E DC_40C-42A-42A_n257F DC_40C-42A-42A_n257G DC_40C-42A-42A_n257H DC_40C-42A-42A_n257I DC_40C-42A-42A_n257J DC_40C-42A-42A_n257K DC_40C-42A-42A_n257L DC_40C-42A-42A_n257M DC_40A-40A-42A-42A_n257A DC_40A-40A-42A-42A_n257D DC_40A-40A-42A-42A_n257E DC_40A-40A-42A-42A_n257F DC_40A-40A-42A-42A_n257G DC_40A-40A-42A-42A_n257H DC_40A-40A-42A-42A_n257I DC_40A-40A-42A-42A_n257J DC_40A-40A-42A-42A_n257K DC_40A-40A-42A-42A_n257L DC_40A-40A-42A-42A_n257M | DC_40A_n257A DC_42A_n257A |
| DC_41A-42A_n257A DC_41A-42A_n257D DC_41A-42A_n257E DC_41A-42A_n257F DC_41A-42A_n257G DC_41A-42A_n257H DC_41A-42A_n257I DC_41A-42A_n257J DC_41A-42A_n257K DC_41A-42A_n257L DC_41A-42A_n257M DC_41A-42C_n257A DC_41A-42C_n257D DC_41A-42C_n257E DC_41A-42C_n257F DC_41A-42C_n257G DC_41A-42C_n257H DC_41A-42C_n257I DC_41A-42C_n257J DC_41A-42C_n257K DC_41A-42C_n257L DC_41A-42C_n257M DC_41C-42A_n257A DC_41C-42A_n257D DC_41C-42A_n257E DC_41C-42A_n257F DC_41C-42A_n257G DC_41C-42A_n257H DC_41C-42A_n257I DC_41C-42A_n257J DC_41C-42A_n257K DC_41C-42A_n257L DC_41C-42A_n257M DC_41C-42C_n257A DC_41C-42C_n257D DC_41C-42C_n257E DC_41C-42C_n257F DC_41C-42C_n257G DC_41C-42C_n257H DC_41C-42C_n257I DC_41C-42C_n257J DC_41C-42C_n257K DC_41C-42C_n257L DC_41C-42C_n257M | DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_46A-48A_n260A DC_46C-48A_n260A DC_46D-48A_n260A DC_46A-48C_n260A DC_46A-48D_n260A DC_46C-48C_n260A DC_46C-48D_n260A DC_46D-48C_n260A DC_46D-48D_n260A | DC_48A_n260A DC_48C_n260A |
| DC_46A-48A_n260(2A) DC_46C-48A_n260(2A) DC_46D-48A_n260(2A) DC_46A-48C_n260(2A) DC_46A-48D_n260(2A) DC_46C-48C_n260(2A) DC_46C-48D_n260(2A) DC_46D-48C_n260(2A) DC_46D-48D_n260(2A) DC_46A-48A_n260(3A) DC_46C-48A_n260(3A) DC_46D-48A_n260(3A) DC_46A-48C_n260(3A) DC_46A-48D_n260(3A) DC_46C-48C_n260(3A) DC_46C-48D_n260(3A) DC_46D-48C_n260(3A) DC_46D-48D_n260(3A) DC_46A-48A_n260(4A) DC_46C-48A_n260(4A) DC_46D-48A_n260(4A) DC_46A-48C_n260(4A) DC_46A-48D_n260(4A) DC_46C-48C_n260(4A) DC_46C-48D_n260(4A) DC_46D-48C_n260(4A) DC_46D-48D_n260(4A) | DC_48A_n260A DC_48C_n260A |
| DC_46A-48A_n261A DC_46C-48A_n261A DC_46D-48A_n261A DC_46A-48C_n261A DC_46A-48D_n261A DC_46C-48C_n261A DC_46C-48D_n261A DC_46D-48C_n261A DC_46D-48D_n261A | DC_48A_n261A DC_48C_n261A |
| DC_46A-48A_n261(2A) DC_46C-48A_n261(2A) DC_46D-48A_n261(2A) DC_46A-48C_n261(2A) DC_46A-48D_n261(2A) DC_46C-48C_n261(2A) DC_46C-48D_n261(2A) DC_46D-48C_n261(2A) DC_46D-48D_n261(2A) | DC_48A_n261A DC_48C_n261A |
| DC_46A-66A_n258A DC_46C-66A_n258A DC_46D-66A_n258A | DC_66A_n258A |
| DC_46A-66A_n258(2A) DC_46A-66A_n258(3A) DC_46A-66A_n258(4A) DC_46A-66A_n258(5A) DC_46C-66A_n258(2A) DC_46C-66A_n258(3A) DC_46C-66A_n258(4A) DC_46C-66A_n258(5A) DC_46D-66A_n258(2A) DC_46D-66A_n258(3A) DC_46D-66A_n258(4A) DC_46D-66A_n258(5A) | DC_66A_n258A |
| DC_46A-66A_n260A DC_46C-66A_n260A DC_46D-66A_n260A DC_46E-66A_n260A DC_46A-66A_n260G DC_46C-66A_n260G DC_46D-66A_n260G DC_46E-66A_n260G DC_46A-66A_n260H DC_46C-66A_n260H DC_46D-66A_n260H DC_46E-66A_n260H DC_46A-66A_n260I DC_46C-66A_n260I DC_46D-66A_n260I DC_46E-66A_n260I DC_46A-66A_n260J DC_46C-66A_n260J DC_46D-66A_n260J DC_46E-66A_n260J DC_46A-66A_n260K DC_46C-66A_n260K DC_46D-66A_n260K DC_46E-66A_n260K DC_46A-66A_n260L DC_46C-66A_n260L DC_46D-66A_n260L DC_46E-66A_n260L DC_46A-66A_n260M DC_46C-66A_n260M DC_46D-66A_n260M DC_46E-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_46A-66A-66A_n260A DC_46C-66A-66A_n260A DC_46D-66A-66A_n260A DC_46E-66A-66A_n260A DC_46A-66A-66A_n260G DC_46C-66A-66A_n260G DC_46D-66A-66A_n260G DC_46E-66A-66A_n260G DC_46A-66A-66A_n260H DC_46C-66A-66A_n260H DC_46D-66A-66A_n260H DC_46E-66A-66A_n260H DC_46A-66A-66A_n260I DC_46C-66A-66A_n260I DC_46D-66A-66A_n260I DC_46E-66A-66A_n260I DC_46A-66A-66A_n260J DC_46C-66A-66A_n260J DC_46D-66A-66A_n260J DC_46E-66A-66A_n260J DC_46A-66A-66A_n260K DC_46C-66A-66A_n260K DC_46D-66A-66A_n260K DC_46E-66A-66A_n260K DC_46A-66A-66A_n260L DC_46C-66A-66A_n260L DC_46D-66A-66A_n260L DC_46E-66A-66A_n260L DC_46A-66A-66A_n260M DC_46C-66A-66A_n260M DC_46D-66A-66A_n260M DC_46E-66A-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_46A-66A_n260(2A) DC_46C-66A_n260(2A) DC_46D-66A_n260(2A) | DC_66A_n260A |
| DC_46A-66A_n261A DC_46A-66A_n261G DC_46A-66A_n261H DC_46A-66A_n261I DC_46A-66A_n261J DC_46A-66A_n261K DC_46A-66A_n261L DC_46A-66A_n261M DC_46C-66A_n261A DC_46D-66A_n261A | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |
| DC_46A-46A-66A_n261A DC_46A-46A-66A_n261G DC_46A-46A-66A_n261H DC_46A-46A-66A_n261I DC_46A-46A-66A_n261J DC_46A-46A-66A_n261K DC_46A-46A-66A_n261L DC_46A-46A-66A_n261M | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |
| DC_46A-46A-46A-66A_n261A DC_46A-46A-46A-66A_n261G DC_46A-46A-46A-66A_n261H DC_46A-46A-46A-66A_n261I DC_46A-46A-46A-66A_n261J DC_46A-46A-46A-66A_n261K DC_46A-46A-46A-66A_n261L DC_46A-46A-46A-66A_n261M | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |
| DC_46A-66A_n261(2A) DC_46C-66A_n261(2A) DC_46D-66A_n261(2A) DC_46A-66A_n261(2G) DC_46A-66A_n261(3A) DC_46A-66A_n261(A-G) DC_46A-66A_n261(A-H) DC_46A-66A_n261(A-I) DC_46A-66A_n261(A-J) DC_46A-66A_n261(A-K) DC_46A-66A_n261(A-L) DC_46A-66A_n261(G-H) DC_46A-66A_n261(2H) DC_46A-66A_n261(2A-G) DC_46A-66A_n261(2A-H) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_46A-46A-66A_n261(A-G) DC_46A-46A-66A_n261(A-H) DC_46A-46A-66A_n261(A-I) DC_46A-46A-66A_n261(A-J) DC_46A-46A-66A_n261(A-K) DC_46A-46A-66A_n261(A-L) DC_46A-46A-66A_n261(G-H) DC_46A-46A-66A_n261(2A) DC_46A-46A-66A_n261(2G) DC_46A-46A-66A_n261(3A) DC_46A-46A-66A_n261(2H) DC_46A-46A-66A_n261(2A-G) DC_46A-46A-66A_n261(2A-H) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_46A-46A-46A-66A_n261(2A) DC_46A-46A-46A-66A_n261(2G) DC_46A-46A-46A-66A_n261(3A) DC_46A-46A-46A-66A_n261(A-G) DC_46A-46A-46A-66A_n261(A-H) DC_46A-46A-46A-66A_n261(A-I) DC_46A-46A-46A-66A_n261(A-J) DC_46A-46A-46A-66A_n261(A-K) DC_46A-46A-46A-66A_n261(A-L) DC_46A-46A-46A-66A_n261(G-H) DC_46A-46A-46A-66A_n261(2H) DC_46A-46A-46A-66A_n261(2A-G) DC_46A-46A-46A-66A_n261(2A-H) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations. NOTE 3: Only single switched UL is supported. |  |

<<Unchanged parts skipped>>

5.5B.6	Inter-band EN-DC including FR1 and FR2

5.5B.6.1	Void

5.5B.6.2	Inter-band EN-DC configurations including FR1 and FR2 (three bands)

Table 5.5B.6.2-1: Inter-band EN-DC configurations including FR1 and FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n3A-n257A2 DC_1A_n3A-n257G2 DC_1A_n3A-n257H2 DC_1A_n3A-n257I2 | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n8A-n257A DC_1A_n8A-n257G DC_1A_n8A-n257H DC_1A_n8A-n257I DC_1A_n8A-n257J DC_1A_n8A-n257K DC_1A_n8A-n257L DC_1A_n8A-n257M | DC_1A_n8A DC_1A_n257A |
| DC_1A_n28A-n257A2 DC_1A_n28A-n257G2 DC_1A_n28A-n257H2 DC_1A_n28A-n257I2 | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n40A-n258A DC_1A_n40A-n258D DC_1A_n40A-n258G DC_1A_n40A-n258E DC_1A_n40A-n258F DC_1A_n40A-n258H DC_1A_n40A-n258I DC_1A_n40A-n258J DC_1A_n40A-n258K DC_1A_n40A-n258L DC_1A_n40A-n258M | DC_1A_n40A DC_1A_n258A |
| DC_1A_n77A-n257J DC_1A_n77A-n257K DC_1A_n77A-n257L DC_1A_n77A-n257M DC_1A_n77A-n257A | DC_1A_n77A DC_1A_n257A |
| DC_1A_n77A-n257A2 DC_1A_n77A-n257D2 DC_1A_n77A-n257E2 DC_1A_n77A-n257F2 DC_1A_n77A-n257G2 DC_1A_n77A-n257H2 DC_1A_n77A-n257I2 DC_1A_n77C-n257A2 DC_1A_n77C-n257D2 DC_1A_n77C-n257E2 DC_1A_n77C-n257F2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I |
| DC_1A_n77(2A)-n257A2 DC_1A_n77(2A)-n257D2 DC_1A_n77(2A)-n257G2 DC_1A_n77(2A)-n257H2 DC_1A_n77(2A)-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n77A-n258A | DC_1A_n77A DC_1A_n258A |
| DC_1A_n78A-n257A2 DC_1A_n78A-n257D2 DC_1A_n78A-n257E2 DC_1A_n78A-n257F2 DC_1A_n78C-n257A2 DC_1A_n78C-n257D2 DC_1A_n78C-n257E2 DC_1A_n78C-n257F2 | DC_1A_n78A DC_1A_n257A DC_1A_n257D DC_1A_n78A-n257A |
| DC_1A_n78A-n257G2 DC_1A_n78A-n257H2 DC_1A_n78A-n257I2 DC_1A_n78A-n257J2 DC_1A_n78A-n257K2 DC_1A_n78A-n257L2 DC_1A_n78A-n257M2 DC_1A_n78C-n257G DC_1A_n78C-n257H DC_1A_n78C-n257I DC_1A_n78C-n257J DC_1A_n78C-n257K DC_1A_n78C-n257L DC_1A_n78C-n257M | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I |
| DC_1A_n78A-n258A DC_1A_n78A-n258D DC_1A_n78A-n258E DC_1A_n78A-n258F DC_1A_n78A-n258G DC_1A_n78A-n258H DC_1A_n78A-n258I DC_1A_n78A-n258J DC_1A_n78A-n258K DC_1A_n78A-n258L DC_1A_n78A-n258M | DC_1A_n78A DC_1A_n258A |
| DC_1A_n79A-n257A2 DC_1A_n79A-n257D2 DC_1A_n79A-n257E2 DC_1A_n79A-n257F2 DC_1A_n79A-n257G2 DC_1A_n79A-n257H2 DC_1A_n79A-n257I2 DC_1A_n79C-n257A2 DC_1A_n79C-n257D2 DC_1A_n79C-n257E2 DC_1A_n79C-n257F2 | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I |
| DC_1A_n79A-n258A | DC_1A_n79A DC_1A_n258A |
| DC_2A_n12A-n258A | DC_2A_n258A DC_2A_n12A |
| DC_2A_n12A-n260A | DC_2A_n260A DC_2A_n12A |
| DC_2A_n12A-n261A | DC_2A_n261A DC_2A_n12A |
| DC_2A_n41A-n260A | DC_2A_n41A |
| DC_2A_n41A-n260(2A) DC_2A_n41A-n260(3A) DC_2A_n41A-n260(4A) | DC_2A_n41A |
| DC_2A_n41A-n261A | DC_2A_n41A |
| DC_2A_n41A-n261(2A) | DC_2A_n41A |
| DC_2A_n71A-n261A | DC_2A_n261A DC_2A_n71A |
| DC_2A_n71A-n261(2A) | DC_2A_n261A DC_2A_n71A |
| DC_3A_n1A-n257A2 DC_3A_n1A-n257D2 DC_3A_n1A-n257E2 DC_3A_n1A-n257F2 DC_3A_n1A-n257G2 DC_3A_n1A-n257H2 DC_3A_n1A-n257I2 DC_3A_n1A-n257J2 DC_3A_n1A-n257K2 DC_3A_n1A-n257L2 DC_3A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |
| DC_3A-3A_n1A-n257A2 DC_3A-3A_n1A-n257D2 DC_3A-3A_n1A-n257E2 DC_3A-3A_n1A-n257F2 DC_3A-3A_n1A-n257G2 DC_3A-3A_n1A-n257H2 DC_3A-3A_n1A-n257I2 DC_3A-3A_n1A-n257J2 DC_3A-3A_n1A-n257K2 DC_3A-3A_n1A-n257L2 DC_3A-3A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |
| DC_3A_n28A-n257A2 DC_3A_n28A-n257G2 DC_3A_n28A-n257H2 DC_3A_n28A-n257I2 | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_3A_n34A-n258A | DC_3A_n34A DC_3A_n258A |
| DC_3A_n40A-n258A DC_3A_n40A-n258D DC_3A_n40A-n258E DC_3A_n40A-n258F DC_3A_n40A-n258G DC_3A_n40A-n258H DC_3A_n40A-n258I DC_3A_n40A-n258J DC_3A_n40A-n258K DC_3A_n40A-n258L DC_3A_n40A-n258M | DC_3A_n40A DC_3A_n258A DC_3A_n40A-n258A |
| DC_3A_n41A-n258A | DC_3A_n41A DC_3A_n258A |
| DC_3A_n77A-n257A2 DC_3A_n77A-n257D2 DC_3A_n77A-n257E2 DC_3A_n77A-n257F2 DC_3A_n77A-n257G2 DC_3A_n77A-n257H2 DC_3A_n77A-n257I2 DC_3A_n77C-n257A2 DC_3A_n77C-n257D2 DC_3A_n77C-n257E2 DC_3A_n77C-n257F2 | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I |
| DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_3A_n77A-n257J DC_3A_n77A-n257K DC_3A_n77A-n257L DC_3A_n77A-n257M DC_3A_n77(2A)-n257A DC_3A_n77(2A)-n257G DC_3A_n77(2A)-n257H DC_3A_n77(2A)-n257I DC_3A_n77(2A)-n257J DC_3A_n77(2A)-n257K DC_3A_n77(2A)-n257L DC_3A_n77(2A)-n257M | DC_3A_n77A DC_3A_n257A |
| DC_3A_n77(2A)-n257A2 DC_3A_n77(2A)-n257D2 DC_3A_n77(2A)-n257G2 DC_3A_n77(2A)-n257H2 DC_3A_n77(2A)-n257I2 | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_3A_n77A-n258A | DC_3A_n77A DC_3A_n258A DC_3A_n77A-n258A |
| DC_3A_n78A-n257A2 DC_3A_n78A-n257D2 DC_3A_n78A-n257E2 DC_3A_n78A-n257F2 DC_3A_n78A-n257G2 DC_3A_n78A-n257H2 DC_3A_n78A-n257I2 DC_3A_n78A-n257J2 DC_3A_n78A-n257K2 DC_3A_n78A-n257L2 DC_3A_n78A-n257M2 DC_3A_n78C-n257A2 DC_3A_n78C-n257D2 DC_3A_n78C-n257E2 DC_3A_n78C-n257F2 DC_3A_n78C-n257G DC_3A_n78C-n257H DC_3A_n78C-n257I DC_3A_n78C-n257J DC_3A_n78C-n257K DC_3A_n78C-n257L DC_3A_n78C-n257M | DC_3A_n78A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I |
| DC_3C_n78A-n257A2 DC_3C_n78A-n257D2 DC_3C_n78A-n257E2 DC_3C_n78A-n257F2 DC_3C_n78A-n257G2 DC_3C_n78A-n257H2 DC_3C_n78A-n257I2 DC_3C_n78A-n257J2 DC_3C_n78A-n257K2 DC_3C_n78A-n257L2 DC_3C_n78A-n257M2 | DC_3A_n78A DC_3A_n257A |
| DC_3A_n78A-n258A DC_3A_n78A-n258D DC_3A_n78A-n258E DC_3A_n78A-n258F DC_3A_n78A-n258G DC_3A_n78A-n258H DC_3A_n78A-n258I DC_3A_n78A-n258J DC_3A_n78A-n258K DC_3A_n78A-n258L DC_3A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3A_n258J DC_3A_n258K |
| DC_3A-3A_n78A-n257A2 DC_3A-3A_n78A-n257D2 DC_3A-3A_n78A-n257E2 DC_3A-3A_n78A-n257F2 DC_3A-3A_n78A-n257G2 DC_3A-3A_n78A-n257H2 DC_3A-3A_n78A-n257I2 DC_3A-3A_n78A-n257J2 DC_3A-3A_n78A-n257K2 DC_3A-3A_n78A-n257L2 DC_3A-3A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |
| DC_3A_n79A-n257A2 DC_3A_n79A-n257D2 DC_3A_n79A-n257E2 DC_3A_n79A-n257F2 DC_3A_n79A-n257G2 DC_3A_n79A-n257H2 DC_3A_n79A-n257I2 DC_3A_n79C-n257A2 DC_3A_n79C-n257D2 DC_3A_n79C-n257E2 DC_3A_n79C-n257F2 | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I |
| DC_3A_n79A-n258A DC_3A_n79A-n258D DC_3A_n79A-n258E DC_3A_n79A-n258F DC_3A_n79A-n258G DC_3A_n79A-n258H DC_3A_n79A-n258I DC_3A_n79A-n258J DC_3A_n79A-n258K DC_3A_n79A-n258L | DC_3A_n79A DC_3A_n258A DC_3A_n79A-n258A |
| DC_5A_n78A-n257A2 DC_5A_n78A-n257D DC_5A_n78A-n257E DC_5A_n78A-n257F DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_5A_n78A-n257J DC_5A_n78A-n257K DC_5A_n78A-n257L DC_5A_n78A-n257M DC_5A_n78C-n257A DC_5A_n78C-n257D DC_5A_n78C-n257E DC_5A_n78C-n257F DC_5A_n78C-n257G DC_5A_n78C-n257H DC_5A_n78C-n257I DC_5A_n78C-n257J DC_5A_n78C-n257K DC_5A_n78C-n257L DC_5A_n78C-n257M | DC_5A_n78A DC_5A_n257A |
| DC_7A_n1A-n257A2 DC_7A_n1A-n257D2 DC_7A_n1A-n257E2 DC_7A_n1A-n257F2 DC_7A_n1A-n257G2 DC_7A_n1A-n257H2 DC_7A_n1A-n257I2 DC_7A_n1A-n257J2 DC_7A_n1A-n257K2 DC_7A_n1A-n257L2 DC_7A_n1A-n257M2 | DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A-7A_n1A-n257A2 DC_7A-7A_n1A-n257D2 DC_7A-7A_n1A-n257E2 DC_7A-7A_n1A-n257F2 DC_7A-7A_n1A-n257G2 DC_7A-7A_n1A-n257H2 DC_7A-7A_n1A-n257I2 DC_7A-7A_n1A-n257J2 DC_7A-7A_n1A-n257K2 DC_7A-7A_n1A-n257L2 DC_7A-7A_n1A-n257M2 | DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A_n40A-n258A DC_7A_n40A-n258D DC_7A_n40A-n258G DC_7A_n40A-n258E DC_7A_n40A-n258F DC_7A_n40A-n258H DC_7A_n40A-n258I DC_7A_n40A-n258J DC_7A_n40A-n258K DC_7A_n40A-n258L DC_7A_n40A-n258M | DC_7A_n40A DC_7A_n258A |
| DC_7A_n78A-n257A2 DC_7A_n78A-n257D2 DC_7A_n78A-n257E2 DC_7A_n78A-n257F2 DC_7A_n78A-n257G2 DC_7A_n78A-n257H2 DC_7A_n78A-n257I2 DC_7A_n78A-n257J2 DC_7A_n78A-n257K2 DC_7A_n78A-n257L2 DC_7A_n78A-n257M2 DC_7A_n78C-n257A DC_7A_n78C-n257D DC_7A_n78C-n257E DC_7A_n78C-n257F DC_7A_n78C-n257G DC_7A_n78C-n257H DC_7A_n78C-n257I DC_7A_n78C-n257J DC_7A_n78C-n257K DC_7A_n78C-n257L DC_7A_n78C-n257M | DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A-7A_n78A-n257A2 DC_7A-7A_n78A-n257D2 DC_7A-7A_n78A-n257E2 DC_7A-7A_n78A-n257F2 DC_7A-7A_n78A-n257G2 DC_7A-7A_n78A-n257H2 DC_7A-7A_n78A-n257I2 DC_7A-7A_n78A-n257J2 DC_7A-7A_n78A-n257K2 DC_7A-7A_n78A-n257L2 DC_7A-7A_n78A-n257M2 DC_7A-7A_n78C-n257A DC_7A-7A_n78C-n257D DC_7A-7A_n78C-n257E DC_7A-7A_n78C-n257F DC_7A-7A_n78C-n257G DC_7A-7A_n78C-n257H DC_7A-7A_n78C-n257I DC_7A-7A_n78C-n257J DC_7A-7A_n78C-n257K DC_7A-7A_n78C-n257L DC_7A-7A_n78C-n257M | DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_7A_n78A-n257A |
| DC_7A_n78A-n258A DC_7A_n78A-n258D DC_7A_n78A-n258E DC_7A_n78A-n258F DC_7A_n78A-n258G DC_7A_n78A-n258H DC_7A_n78A-n258I DC_7A_n78A-n258J DC_7A_n78A-n258K DC_7A_n78A-n258L DC_7A_n78A-n258M DC_7C_n78A-n258A DC_7C_n78A-n258G DC_7C_n78A-n258H DC_7C_n78A-n258I DC_7C_n78A-n258J DC_7C_n78A-n258K DC_7C_n78A-n258L DC_7C_n78A-n258M | DC_7A_n78A DC_7A_n258A DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7C_n78A DC_7C_n258A DC_7C_n258G DC_7C_n258H DC_7C_n258I |
| DC_7A_n79A-n257A DC_7A_n79A-n257G DC_7A_n79A-n257H DC_7A_n79A-n257I DC_7A_n79A-n257J DC_7A_n79A-n257K DC_7A_n79A-n257L DC_7A_n79A-n257M | DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_7A_n257L DC_7A_n257M DC_7A_n79A |
| DC_7A_n79A-n258A DC_7A_n79A-n258G DC_7A_n79A-n258H DC_7A_n79A-n258I DC_7A_n79A-n258J DC_7A_n79A-n258K DC_7A_n79A-n258L DC_7A_n79A-n258M | DC_7A_n258A DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7A_n258J DC_7A_n258K DC_7A_n258L DC_7A_n258M DC_7A_n79A |
| DC_8A_n1A-n257A2 DC_8A_n1A-n257D2 DC_8A_n1A-n257E2 DC_8A_n1A-n257F2 DC_8A_n1A-n257G2 DC_8A_n1A-n257H2 DC_8A_n1A-n257I2 DC_8A_n1A-n257J2 DC_8A_n1A-n257K2 DC_8A_n1A-n257L2 DC_8A_n1A-n257M2 | DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_8A_n3A-n257A DC_8A_n3A-n257G DC_8A_n3A-n257H DC_8A_n3A-n257I DC_8A_n3A-n257J DC_8A_n3A-n257K DC_8A_n3A-n257L DC_8A_n3A-n257M | DC_8A_n3A DC_8A_n257A |
| DC_8A_n34A-n258A | DC_8A_n34A DC_8A_n258A |
| DC_8A_n40A-n258A DC_8A_n40A-n258D DC_8A_n40A-n258E DC_8A_n40A-n258F DC_8A_n40A-n258G DC_8A_n40A-n258H DC_8A_n40A-n258I DC_8A_n40A-n258J DC_8A_n40A-n258K DC_8A_n40A-n258L DC_8A_n40A-n258M | DC_8A_n40A DC_8A_n258A DC_8A_n40A-n258A |
| DC_8A_n41A-n258A | DC_8A_n41A DC_8A_n258A |
| DC_8A_n77A-n257A2 DC_8A_n77A-n257D2 DC_8A_n77A-n257G2 DC_8A_n77A-n257H2 DC_8A_n77A-n257I2 | DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_8A_n77(2A)-n257A2 DC_8A_n77(2A)-n257D2 DC_8A_n77(2A)-n257G2 DC_8A_n77(2A)-n257H2 DC_8A_n77(2A)-n257I2 | DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_8A_n78A-n257A2 DC_8A_n78A-n257D2 DC_8A_n78A-n257E2 DC_8A_n78A-n257F2 DC_8A_n78A-n257G2 DC_8A_n78A-n257H2 DC_8A_n78A-n257I2 DC_8A_n78A-n257J2 DC_8A_n78A-n257K2 DC_8A_n78A-n257L2 DC_8A_n78A-n257M2 | DC_8A_n78A DC_8A_n257A |
| DC_8A_n78A-n258A DC_8A_n78A-n258D DC_8A_n78A-n258E DC_8A_n78A-n258F DC_8A_n78A-n258G DC_8A_n78A-n258H DC_8A_n78A-n258I DC_8A_n78A-n258J DC_8A_n78A-n258K DC_8A_n78A-n258L DC_8A_n78A-n258M | DC_8A_n78A DC_8A_n258A |
| DC_8A_n79A-n258A | DC_8A_n79A-n258A |
| DC_11A_n77A-n257A2 DC_11A_n77A-n257D2 DC_11A_n77A-n257G2 DC_11A_n77A-n257H2 DC_11A_n77A-n257I2 | DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_11A_n77(2A)-n257A2 DC_11A_n77(2A)-n257D2 DC_11A_n77(2A)-n257G2 DC_11A_n77(2A)-n257H2 DC_11A_n77(2A)-n257I2 | DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_18A_n3A-n257A DC_18A_n3A-n257G DC_18A_n3A-n257H DC_18A_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_18A_n78A-n257A DC_18A_n78A-n257G DC_18A_n78A-n257H DC_18A_n78A-n257I | DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_19A_n77A-n257A2 DC_19A_n77A-n257D2 DC_19A_n77A-n257E2 DC_19A_n77A-n257F2 DC_19A_n77A-n257G2 DC_19A_n77A-n257H2 DC_19A_n77A-n257I2 DC_19A_n77C-n257A2 DC_19A_n77C-n257D2 DC_19A_n77C-n257E2 DC_19A_n77C-n257F2 | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n77A-n257A DC_19A_n77A-n257G DC_19A_n77A-n257H DC_19A_n77A-n257I |
| DC_19A_n78A-n257A2 DC_19A_n78A-n257D2 DC_19A_n78A-n257E2 DC_19A_n78A-n257F2 DC_19A_n78A-n257G2 DC_19A_n78A-n257H2 DC_19A_n78A-n257I2 DC_19A_n78C-n257A2 DC_19A_n78C-n257D2 DC_19A_n78C-n257E2 DC_19A_n78C-n257F2 | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n78A-n257A DC_19A_n78A-n257G DC_19A_n78A-n257H DC_19A_n78A-n257I |
| DC_19A_n79A-n257A2 DC_19A_n79A-n257D2 DC_19A_n79A-n257E2 DC_19A_n79A-n257F2 DC_19A_n79A-n257G2 DC_19A_n79A-n257H2 DC_19A_n79A-n257I2 DC_19A_n79C-n257A2 DC_19A_n79C-n257D2 DC_19A_n79C-n257E2 DC_19A_n79C-n257F2 | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_21A_n77A-n257A2 DC_21A_n77A-n257G2 DC_21A_n77A-n257H2 DC_21A_n77A-n257I2 | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_21A_n78A-n257A2 DC_21A_n78A-n257G2 DC_21A_n78A-n257H2 DC_21A_n78A-n257I2 | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_21A_n79A-n257A2 DC_21A_n79A-n257G2 DC_21A_n79A-n257H2 DC_21A_n79A-n257I2 | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_28A_n3A-n257A2 DC_28A_n3A-n257G2 DC_28A_n3A-n257H2 DC_28A_n3A-n257I2 | DC_28A_n3A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n7A-n258A DC_28A_n7A-n258B DC_28A_n7A-n258C DC_28A_n7A-n258D DC_28A_n7A-n258E DC_28A_n7A-n258F DC_28A_n7A-n258G DC_28A_n7A-n258H DC_28A_n7A-n258I DC_28A_n7A-n258J DC_28A_n7A-n258K DC_28A_n7A-n258L DC_28A_n7A-n258M DC_28A_n7B-n258A DC_28A_n7B-n258B DC_28A_n7B-n258C DC_28A_n7B-n258D DC_28A_n7B-n258E DC_28A_n7B-n258F DC_28A_n7B-n258G DC_28A_n7B-n258H DC_28A_n7B-n258I DC_28A_n7B-n258J DC_28A_n7B-n258K DC_28A_n7B-n258L DC_28A_n7B-n258M | DC_28A_n7A DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_28A_n77A-n257A2 DC_28A_n77A-n257D2 DC_28A_n77A-n257G2 DC_28A_n77A-n257H2 DC_28A_n77A-n257I2 | DC_28A_n77A DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n77(2A)-n257A2 DC_28A_n77(2A)-n257D2 DC_28A_n77(2A)-n257G2 DC_28A_n77(2A)-n257H2 DC_28A_n77(2A)-n257I2 | DC_28A_n77A DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n78A-n257A2 DC_28A_n78A-n257G2 DC_28A_n78A-n257H2 DC_28A_n78A-n257I2 | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n8A-n258A | DC_28A_n8A DC_28A_n258A |
| DC_28A_n78A-n258A DC_28A_n78A-n258G DC_28A_n78A-n258H DC_28A_n78A-n258I DC_28A_n78A-n258J DC_28A_n78A-n258K DC_28A_n78A-n258L DC_28A_n78A-n258M | DC_28A_n78A DC_28A_n258A |
| DC_39A_n40A-n258A | DC_39A_n40A DC_39A_n258A DC_39A_n40A-n258A |
| DC_39A_n41A-n258A DC_39A_n41C-n258A | DC_39A_n41A DC_39A_n258A DC_39A_n41A-n258A |
| DC_39A_n79A-n258A DC_39A_n79C-n258A | DC_39A_n79A DC_39A_n258A DC_39A_n79A-n258A |
| DC_40A_n41A-n258A DC_40A_n41C-n258A | DC_40A_n41A DC_40A_n258A DC_40A_n41A-n258A |
| DC_40A_n77A-n257A DC_40A_n77A-n257D DC_40A_n77A-n257E DC_40A_n77A-n257F DC_40A_n77A-n257G DC_40A_n77A-n257H DC_40A_n77A-n257I DC_40A_n77A-n257J DC_40A_n77A-n257K DC_40A_n77A-n257L DC_40A_n77A-n257M DC_40A_n77C-n257A DC_40A_n77C-n257D DC_40A_n77C-n257E DC_40A_n77C-n257F DC_40A_n77C-n257G DC_40A_n77C-n257H DC_40A_n77C-n257I DC_40A_n77C-n257J DC_40A_n77C-n257K DC_40A_n77C-n257L DC_40A_n77C-n257M | DC_40A_n257A DC_40A_n257D DC_40A_n257E DC_40A_n257F DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M |
| DC_40A_n78A-n257A DC_40A_n78A-n257D DC_40A_n78A-n257E DC_40A_n78A-n257F DC_40A_n78A-n257G DC_40A_n78A-n257H DC_40A_n78A-n257I DC_40A_n78A-n257J DC_40A_n78A-n257K DC_40A_n78A-n257L DC_40A_n78A-n257M | DC_40A_n257A DC_40A_n257D DC_40A_n257E DC_40A_n257F DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M |
| DC_40A_n79A-n258A DC_40A_n79C-n258A | DC_40A_n79A DC_40A_n258A |
| DC_41A_n3A-n257A2 DC_41A_n3A-n257G2 DC_41A_n3A-n257H2 DC_41A_n3A-n257I2 DC_41C_n3A-n257A2 DC_41C_n3A-n257G2 DC_41C_n3A-n257H2 DC_41C_n3A-n257I2 | DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n28A-n257A2 DC_41A_n28A-n257G2 DC_41A_n28A-n257H2 DC_41A_n28A-n257I2 DC_41C_n28A-n257A2 DC_41C_n28A-n257G2 DC_41C_n28A-n257H2 DC_41C_n28A-n257I2 | DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n77A-n257A DC_41A_n77A-n257G DC_41A_n77A-n257H DC_41A_n77A-n257I DC_41C_n77A-n257A DC_41C_n77A-n257G DC_41C_n77A-n257H DC_41C_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n78A-n257A DC_41A_n78A-n257G DC_41A_n78A-n257H DC_41A_n78A-n257I DC_41C_n78A-n257A DC_41C_n78A-n257G DC_41C_n78A-n257H DC_41C_n78A-n257I | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n79A-n258A DC_41A_n79C-n258A | DC_41A_n79A DC_41A_n258A DC_41A_n79A-n258A |
| DC_42A_n77A-n257A DC_42A_n77A-n257D DC_42A_n77A-n257E DC_42A_n77A-n257F DC_42A_n77A-n257G DC_42A_n77A-n257H DC_42A_n77A-n257I DC_42A_n77A-n257J DC_42A_n77A-n257K DC_42A_n77A-n257L DC_42A_n77A-n257M DC_42A_n77C-n257A DC_42A_n77C-n257D DC_42A_n77C-n257E DC_42A_n77C-n257F DC_42A_n77C-n257G DC_42A_n77C-n257H DC_42A_n77C-n257I DC_42A_n77C-n257J DC_42A_n77C-n257K DC_42A_n77C-n257L DC_42A_n77C-n257M DC_42C_n77A-n257A DC_42C_n77A-n257G DC_42C_n77A-n257H DC_42C_n77A-n257I | DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257K DC_42A_n257L DC_42A_n257M |
| DC_42A_n78A-n257A DC_42A_n78A-n257D DC_42A_n78A-n257E DC_42A_n78A-n257F DC_42A_n78A-n257G DC_42A_n78A-n257H DC_42A_n78A-n257I DC_42A_n78A-n257J DC_42A_n78A-n257K DC_42A_n78A-n257L DC_42A_n78A-n257M DC_42C_n78A-n257A DC_42C_n78A-n257G DC_42C_n78A-n257H DC_42C_n78A-n257I | DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257L DC_42A_n257M DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_42A_n79A-n257A DC_42A_n79A-n257G DC_42A_n79A-n257H DC_42A_n79A-n257I DC_42C_n79A-n257A DC_42C_n79A-n257G DC_42C_n79A-n257H DC_42C_n79A-n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_66A_n5A-n260A DC_66A_n5A-n260G DC_66A_n5A-n260H DC_66A_n5A-n260I DC_66A_n5A-n260J DC_66A_n5A-n260K DC_66A_n5A-n260L DC_66A_n5A-n260M | DC_66A_n5A DC_66A_n260A DC_66A_n5A-n260A |
| DC_66A_n5A-n260(2A) DC_66A_n5A-n260(3A) DC_66A_n5A-n260(4A) DC_66A_n5A-n260(5A) DC_66A_n5A-n260(6A) DC_66A_n5A-n260(2H) DC_66A_n5A-n260(2G) DC_66A_n5A-n260(A-2G) DC_66A_n5A-n260(A-H) DC_66A_n5A-n260(A-G) DC_66A_n5A-n260(G-H) DC_66A_n5A-n260(2A-G) DC_66A_n5A-n260(2A-2G) DC_66A_n5A-n260(3A-G) | DC_66A_n5A-n260A |
| DC_66A_n5A-n261A DC_66A_n5A-n261G DC_66A_n5A-n261H DC_66A_n5A-n261I DC_66A_n5A-n261J DC_66A_n5A-n261K DC_66A_n5A-n261L DC_66A_n5A-n261M | DC_66A_n5A-n261A |
| DC_66A_n5A-n261(2A) DC_66A_n5A-n261(3A) DC_66A_n5A-n261(4A) DC_66A_n5A-n261(2G) DC_66A_n5A-n261(2H) DC_66A_n5A-n261(A-G) DC_66A_n5A-n261(A-H) DC_66A_n5A-n261(A-I) DC_66A_n5A-n261(A-J) DC_66A_n5A-n261(A-K) DC_66A_n5A-n261(G-H) DC_66A_n5A-n261(G-I) DC_66A_n5A-n261(G-J) DC_66A_n5A-n261(H-I) DC_66A_n5A-n261(A-2G) DC_66A_n5A-n261(A-G-H) DC_66A_n5A-n261(A-G-I) DC_66A_n5A-n261(2A-G) DC_66A_n5A-n261(2A-H) DC_66A_n5A-n261(2A-I) DC_66A_n5A-n261(3A-G) | DC_66A_n5A-n261A |
| DC_66A_n12A-n258A | DC_66A_n12A DC_66A_n258A |
| DC_66A_n12A-n260A | DC_66A_n12A DC_66A_n260A |
| DC_66A_n12A-n261A | DC_66A_n12A DC_66A_n261A |
| DC_66A_n41A-n260A | DC_66A_n41A |
| DC_66A_n41A-n260(2A) DC_66A_n41A-n260(3A) DC_66A_n41A-n260(4A) | DC_66A_n41A |
| DC_66A_n41A-n261A | DC_66A_n41A |
| DC_66A_n41A-n261(2A) | DC_66A_n41A |
| DC_66A_n71A-n260A | DC_66A_n71A DC_66A_n260A |
| DC_66A_n71A-n260(2A) | DC_66A_n71A DC_66A_n260A |
| DC_66A_n71A-n261A | DC_66A_n71A DC_66A_n261A |
| DC_66A_n71A-n261(2A) | DC_66A_n71A DC_66A_n261A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. |  |

5.5B.6.3	Inter-band EN-DC configurations including FR1 and FR2 (four bands)

Table 5.5B.6.3-1: Inter-band EN-DC configurations including FR1 and FR2 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_(n)3AA-n257A DC_1A _(n)3AA-n257G DC_1A _(n)3AA-n257H DC_1A _(n)3AA-n257I DC_1A _(n)3AA-n257J DC_1A _(n)3AA-n257K DC_1A _(n)3AA-n257L DC_1A _(n)3AA-n257M | DC_1A_n3A DC_1A_n257A DC_(n)3AA3 DC_3A_n257A |
| DC_1A-3A_n28A-n257A2 DC_1A-3A_n28A-n257G2 DC_1A-3A_n28A-n257H2 DC_1A-3A_n28A-n257I2 | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_1A-3A_n77A-n257A2 | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_3A_n257A |
| DC_1A-3A_n77A-n257D2 | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257D DC_3A_n257A DC_3A_n257D |
| DC_1A-3A_n77A-n257G2 | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n257A DC_3A_n257G DC_1A_n77A-n257A DC_1A_n77A-n257G DC_3A_n77A-n257A DC_3A_n77A-n257G |
| DC_1A-3A_n77A-n257H2 | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H |
| DC_1A-3A_n77A-n257I2 | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I |
| DC_1A-3A_n78A-n257A2 DC_1A-3A_n78A-n257D2 DC_1A-3A_n78A-n257E2 DC_1A-3A_n78A-n257F2 DC_1A-3A_n78A-n257G2 DC_1A-3A_n78A-n257H2 DC_1A-3A_n78A-n257I2 DC_1A-3A_n78A-n257J2 DC_1A-3A_n78A-n257K2 DC_1A-3A_n78A-n257L2 DC_1A-3A_n78A-n257M2 DC_1A-3C_n78A-n257A DC_1A-3C_n78A-n257D DC_1A-3C_n78A-n257E DC_1A-3C_n78A-n257F DC_1A-3C_n78A-n257G DC_1A-3C_n78A-n257H DC_1A-3C_n78A-n257I DC_1A-3C_n78A-n257J DC_1A-3C_n78A-n257K DC_1A-3C_n78A-n257L DC_1A-3C_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I |
| DC_1A-3A_n78C-n257A DC_1A-3A_n78C-n257D DC_1A-3A_n78C-n257E DC_1A-3A_n78C-n257F DC_1A-3A_n78C-n257G DC_1A-3A_n78C-n257H DC_1A-3A_n78C-n257I DC_1A-3A_n78C-n257J DC_1A-3A_n78C-n257K DC_1A-3A_n78C-n257L DC_1A-3A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I |
| DC_1A-3A_n78A-n258A DC_1A-3A_n78A-n258D DC_1A-3A_n78A-n258E DC_1A-3A_n78A-n258F DC_1A-3A_n78A-n258G DC_1A-3A_n78A-n258H DC_1A-3A_n78A-n258I DC_1A-3A_n78A-n258J DC_1A-3A_n78A-n258K DC_1A-3A_n78A-n258L DC_1A-3A_n78A-n258M | DC_1A_n78A DC_1A_n258A DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_3A_n78A DC_3A_n258A DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I |
| DC_1A-3A_n79A-n257A2 DC_1A-3A_n79A-n257G2 DC_1A-3A_n79A-n257H2 DC_1A-3A_n79A-n257I2 | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I |
| DC_1A-5A_n78A-n257A DC_1A-5A_n78A-n257D DC_1A-5A_n78A-n257E DC_1A-5A_n78A-n257F DC_1A-5A_n78A-n257G DC_1A-5A_n78A-n257H DC_1A-5A_n78A-n257I DC_1A-5A_n78A-n257J DC_1A-5A_n78A-n257K DC_1A-5A_n78A-n257L DC_1A-5A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_1A-5A_n78C-n257A DC_1A-5A_n78C-n257D DC_1A-5A_n78C-n257E DC_1A-5A_n78C-n257F DC_1A-5A_n78C-n257G DC_1A-5A_n78C-n257H DC_1A-5A_n78C-n257I DC_1A-5A_n78C-n257J DC_1A-5A_n78C-n257K DC_1A-5A_n78C-n257L DC_1A-5A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_1A-7A_n78A-n257A DC_1A-7A_n78A-n257D DC_1A-7A_n78A-n257E DC_1A-7A_n78A-n257F DC_1A-7A_n78A-n257G DC_1A-7A_n78A-n257H DC_1A-7A_n78A-n257I DC_1A-7A_n78A-n257J DC_1A-7A_n78A-n257K DC_1A-7A_n78A-n257L DC_1A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-7A_n78C-n257A DC_1A-7A_n78C-n257D DC_1A-7A_n78C-n257E DC_1A-7A_n78C-n257F DC_1A-7A_n78C-n257G DC_1A-7A_n78C-n257H DC_1A-7A_n78C-n257I DC_1A-7A_n78C-n257J DC_1A-7A_n78C-n257K DC_1A-7A_n78C-n257L DC_1A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-7A-7A_n78A-n257A DC_1A-7A-7A_n78A-n257D DC_1A-7A-7A_n78A-n257E DC_1A-7A-7A_n78A-n257F DC_1A-7A-7A_n78A-n257G DC_1A-7A-7A_n78A-n257H DC_1A-7A-7A_n78A-n257I DC_1A-7A-7A_n78A-n257J DC_1A-7A-7A_n78A-n257K DC_1A-7A-7A_n78A-n257L DC_1A-7A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-7A-7A_n78C-n257A DC_1A-7A-7A_n78C-n257D DC_1A-7A-7A_n78C-n257E DC_1A-7A-7A_n78C-n257F DC_1A-7A-7A_n78C-n257G DC_1A-7A-7A_n78C-n257H DC_1A-7A-7A_n78C-n257I DC_1A-7A-7A_n78C-n257J DC_1A-7A-7A_n78C-n257K DC_1A-7A-7A_n78C-n257L DC_1A-7A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-7A_n78A-n258A DC_1A-7A_n78A-n258D DC_1A-7A_n78A-n258E DC_1A-7A_n78A-n258F DC_1A-7A_n78A-n258G DC_1A-7A_n78A-n258H DC_1A-7A_n78A-n258I DC_1A-7A_n78A-n258J DC_1A-7A_n78A-n258K DC_1A-7A_n78A-n258L DC_1A-7A_n78A-n258M | DC_1A_n78A DC_1A_n258A DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_7A_n78A DC_7A_n258A DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I |
| DC_1A-8A_n40A-n258A DC_1A-8A_n40A-n258D DC_1A-8A_n40A-n258E DC_1A-8A_n40A-n258F DC_1A-8A_n40A-n258G DC_1A-8A_n40A-n258H DC_1A-8A_n40A-n258I DC_1A-8A_n40A-n258J DC_1A-8A_n40A-n258K DC_1A-8A_n40A-n258L DC_1A-8A_n40A-n258M | DC_1A_n40A DC_1A_n258A DC_8A_n40A DC_8A_n258A |
| DC_1A-8A_n77A-n257A2 DC_1A-8A_n77A-n257D2 DC_1A-8A_n77A-n257G2 DC_1A-8A_n77A-n257H2 DC_1A-8A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_1A-8A_n77(2A)-n257A2 DC_1A-8A_n77(2A)-n257D2 DC_1A-8A_n77(2A)-n257G2 DC_1A-8A_n77(2A)-n257H2 DC_1A-8A_n77(2A)-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_1A-8A_n78A-n257A2 DC_1A-8A_n78A-n257D2 DC_1A-8A_n78A-n257E2 DC_1A-8A_n78A-n257F2 DC_1A-8A_n78A-n257G2 DC_1A-8A_n78A-n257H2 DC_1A-8A_n78A-n257I2 DC_1A-8A_n78A-n257J2 DC_1A-8A_n78A-n257K2 DC_1A-8A_n78A-n257L2 DC_1A-8A_n78A-n257M2 | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A |
| DC_1A-8A_n78A-n258A DC_1A-8A_n78A-n258D DC_1A-8A_n78A-n258E DC_1A-8A_n78A-n258F DC_1A-8A_n78A-n258G DC_1A-8A_n78A-n258H DC_1A-8A_n78A-n258I DC_1A-8A_n78A-n258J DC_1A-8A_n78A-n258K DC_1A-8A_n78A-n258L DC_1A-8A_n78A-n258M | DC_1A_n78A DC_1A_n258A DC_8A_n78A DC_8A_n258A |
| DC_1A-11A_n77A-n257A2 DC_1A-11A_n77A-n257D2 DC_1A-11A_n77A-n257G2 DC_1A-11A_n77A-n257H2 DC_1A-11A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-11A_n77(2A)-n257A2 DC_1A-11A_n77(2A)-n257D2 DC_1A-11A_n77(2A)-n257G2 DC_1A-11A_n77(2A)-n257H2 DC_1A-11A_n77(2A)-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-18A_n3A-n257A DC_1A-18A_n3A-n257G DC_1A-18A_n3A-n257H DC_1A-18A_n3A-n257I | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-18A_n78A-n257A DC_1A-18A_n78A-n257G DC_1A-18A_n78A-n257H DC_1A-18A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-19A_n77A-n257A2 DC_1A-19A_n77A-n257G2 DC_1A-19A_n77A-n257H2 DC_1A-19A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-19A_n78A-n257A2 DC_1A-19A_n78A-n257G2 DC_1A-19A_n78A-n257H2 DC_1A-19A_n78A-n257I2 | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-19A_n79A-n257A2 DC_1A-19A_n79A-n257G2 DC_1A-19A_n79A-n257H2 DC_1A-19A_n79A-n257I2 | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_1A-21A_n77A-n257A2 DC_1A-21A_n77A-n257G2 DC_1A-21A_n77A-n257H2 DC_1A-21A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_1A-21A_n78A-n257A2 DC_1A-21A_n78A-n257G2 DC_1A-21A_n78A-n257H2 DC_1A-21A_n78A-n257I2 | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_1A-21A_n79A-n257A2 DC_1A-21A_n79A-n257G2 DC_1A-21A_n79A-n257H2 DC_1A-21A_n79A-n257I2 | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_1A-28A_n3A-n257A2 DC_1A-28A_n3A-n257G2 DC_1A-28A_n3A-n257H2 DC_1A-28A_n3A-n257I2 | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n3A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-28A_n78A-n257A2 DC_1A-28A_n78A-n257G2 DC_1A-28A_n78A-n257H2 DC_1A-28A_n78A-n257I2 | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-41A_n3A-n257A2 DC_1A-41A_n3A-n257G DC_1A-41A_n3A-n257H DC_1A-41A_n3A-n257I | DC_41A_n3A DC_41A_n257A DC_41A_n257I |
| DC_1A-41C_n3A-n257A2 DC_1A-41C_n3A-n257G DC_1A-41C_n3A-n257H DC_1A-41C_n3A-n257I | DC_41A_n3A DC_41A_n257A DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257I |
| DC_1A-41A_n28A-n257A2 DC_1A-41A_n28A-n257G2 DC_1A-41A_n28A-n257H2 DC_1A-41A_n28A-n257I2 DC_1A-41C_n28A-n257A2 DC_1A-41C_n28A-n257G2 DC_1A-41C_n28A-n257H2 DC_1A-41C_n28A-n257I2 | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-41A_n77A-n257A DC_1A-41A_n77A-n257G DC_1A-41A_n77A-n257H DC_1A-41A_n77A-n257I DC_1A-41C_n77A-n257A DC_1A-41C_n77A-n257G DC_1A-41C_n77A-n257H DC_1A-41C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-41A_n78A-n257A DC_1A-41A_n78A-n257G DC_1A-41A_n78A-n257H DC_1A-41A_n78A-n257I DC_1A-41C_n78A-n257A DC_1A-41C_n78A-n257G DC_1A-41C_n78A-n257H DC_1A-41C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-42A_n77A-n257A DC_1A-42A_n77A-n257G DC_1A-42A_n77A-n257H DC_1A-42A_n77A-n257I DC_1A-42C_n77A-n257A DC_1A-42C_n77A-n257G DC_1A-42C_n77A-n257H DC_1A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I |
| DC_1A-42A_n78A-n257A DC_1A-42A_n78A-n257G DC_1A-42A_n78A-n257H DC_1A-42A_n78A-n257I DC_1A-42C_n78A-n257A DC_1A-42C_n78A-n257G DC_1A-42C_n78A-n257H DC_1A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I |
| DC_1A-42A_n79A-n257A DC_1A-42A_n79A-n257G DC_1A-42A_n79A-n257H DC_1A-42A_n79A-n257I DC_1A-42C_n79A-n257A DC_1A-42C_n79A-n257G DC_1A-42C_n79A-n257H DC_1A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I |
| DC_2A-66A_n41A-n260A | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n41A-n260(2A) DC_2A-66A_n41A-n260(3A) DC_2A-66A_n41A-n260(4A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n41A-n261A | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n41A-n261(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n71A-n261A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A_n71A-n261(2A) | DC_2A_n71A DC_66A_n71A |
| DC_3A_n1A-n78A-n257A2 DC_3A_n1A-n78A-n257D2 DC_3A_n1A-n78A-n257E2 DC_3A_n1A-n78A-n257F2 DC_3A_n1A-n78A-n257G2 DC_3A_n1A-n78A-n257H2 DC_3A_n1A-n78A-n257I2 DC_3A_n1A-n78A-n257J2 DC_3A_n1A-n78A-n257K2 DC_3A_n1A-n78A-n257L2 DC_3A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A |
| DC_3A-3A_n1A-n78A-n257A2 DC_3A-3A_n1A-n78A-n257D2 DC_3A-3A_n1A-n78A-n257E2 DC_3A-3A_n1A-n78A-n257F2 DC_3A-3A_n1A-n78A-n257G2 DC_3A-3A_n1A-n78A-n257H2 DC_3A-3A_n1A-n78A-n257I2 DC_3A-3A_n1A-n78A-n257J2 DC_3A-3A_n1A-n78A-n257K2 DC_3A-3A_n1A-n78A-n257L2 DC_3A-3A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A |
| DC_3A-5A_n78A-n257A DC_3A-5A_n78A-n257D DC_3A-5A_n78A-n257E DC_3A-5A_n78A-n257F DC_3A-5A_n78A-n257G DC_3A-5A_n78A-n257H DC_3A-5A_n78A-n257I DC_3A-5A_n78A-n257J DC_3A-5A_n78A-n257K DC_3A-5A_n78A-n257L DC_3A-5A_n78A-n257M | DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_3A-5A_n78C-n257A DC_3A-5A_n78C-n257D DC_3A-5A_n78C-n257E DC_3A-5A_n78C-n257F DC_3A-5A_n78C-n257G DC_3A-5A_n78C-n257H DC_3A-5A_n78C-n257I DC_3A-5A_n78C-n257J DC_3A-5A_n78C-n257K DC_3A-5A_n78C-n257L DC_3A-5A_n78C-n257M | DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_3A-7A_n1A-n257A2 DC_3A-7A_n1A-n257D2 DC_3A-7A_n1A-n257E2 DC_3A-7A_n1A-n257F2 DC_3A-7A_n1A-n257G2 DC_3A-7A_n1A-n257H2 DC_3A-7A_n1A-n257I2 DC_3A-7A_n1A-n257J2 DC_3A-7A_n1A-n257K2 DC_3A-7A_n1A-n257L2 DC_3A-7A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-3A-7A_n1A-n257A2 DC_3A-3A-7A_n1A-n257D2 DC_3A-3A-7A_n1A-n257E2 DC_3A-3A-7A_n1A-n257F2 DC_3A-3A-7A_n1A-n257G2 DC_3A-3A-7A_n1A-n257H2 DC_3A-3A-7A_n1A-n257I2 DC_3A-3A-7A_n1A-n257J2 DC_3A-3A-7A_n1A-n257K2 DC_3A-3A-7A_n1A-n257L2 DC_3A-3A-7A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-7A-7A_n1A-n257A2 DC_3A-7A-7A_n1A-n257D2 DC_3A-7A-7A_n1A-n257E2 DC_3A-7A-7A_n1A-n257F2 DC_3A-7A-7A_n1A-n257G2 DC_3A-7A-7A_n1A-n257H2 DC_3A-7A-7A_n1A-n257I2 DC_3A-7A-7A_n1A-n257J2 DC_3A-7A-7A_n1A-n257K2 DC_3A-7A-7A_n1A-n257L2 DC_3A-7A-7A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-3A-7A-7A_n1A-n257A2 DC_3A-3A-7A-7A_n1A-n257D2 DC_3A-3A-7A-7A_n1A-n257E2 DC_3A-3A-7A-7A_n1A-n257F2 DC_3A-3A-7A-7A_n1A-n257G2 DC_3A-3A-7A-7A_n1A-n257H2 DC_3A-3A-7A-7A_n1A-n257I2 DC_3A-3A-7A-7A_n1A-n257J2 DC_3A-3A-7A-7A_n1A-n257K2 DC_3A-3A-7A-7A_n1A-n257L2 DC_3A-3A-7A-7A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-7A_n40A-n258A DC_3A-7A_n40A-n258D DC_3A-7A_n40A-n258G DC_3A-7A_n40A-n258E DC_3A-7A_n40A-n258F DC_3A-7A_n40A-n258H DC_3A-7A_n40A-n258I DC_3A-7A_n40A-n258J DC_3A-7A_n40A-n258K DC_3A-7A_n40A-n258L DC_3A-7A_n40A-n258M | DC_3A_n40A DC_7A_n40A DC_3A_n258A DC_7A_n258A |
| DC_3A-3A-7A_n78A-n257A2 DC_3A-3A-7A_n78A-n257D2 DC_3A-3A-7A_n78A-n257E2 DC_3A-3A-7A_n78A-n257F2 DC_3A-3A-7A_n78A-n257G2 DC_3A-3A-7A_n78A-n257H2 DC_3A-3A-7A_n78A-n257I2 DC_3A-3A-7A_n78A-n257J2 DC_3A-3A-7A_n78A-n257K2 DC_3A-3A-7A_n78A-n257L2 DC_3A-3A-7A_n78A-n257M2 DC_3A-3A-7A-7A_n78A-n257A2 DC_3A-3A-7A-7A_n78A-n257D2 DC_3A-3A-7A-7A_n78A-n257E2 DC_3A-3A-7A-7A_n78A-n257F2 DC_3A-3A-7A-7A_n78A-n257G2 DC_3A-3A-7A-7A_n78A-n257H2 DC_3A-3A-7A-7A_n78A-n257I2 DC_3A-3A-7A-7A_n78A-n257J2 DC_3A-3A-7A-7A_n78A-n257K2 DC_3A-3A-7A-7A_n78A-n257L2 DC_3A-3A-7A-7A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_3A-7A_n78A-n257A2 DC_3A-7A_n78A-n257D2 DC_3A-7A_n78A-n257E2 DC_3A-7A_n78A-n257F2 DC_3A-7A_n78A-n257G2 DC_3A-7A_n78A-n257H2 DC_3A-7A_n78A-n257I2 DC_3A-7A_n78A-n257J2 DC_3A-7A_n78A-n257K2 DC_3A-7A_n78A-n257L2 DC_3A-7A_n78A-n257M2 DC_3A-7A_n78C-n257A DC_3A-7A_n78C-n257D DC_3A-7A_n78C-n257E DC_3A-7A_n78C-n257F DC_3A-7A_n78C-n257G DC_3A-7A_n78C-n257H DC_3A-7A_n78C-n257I DC_3A-7A_n78C-n257J DC_3A-7A_n78C-n257K DC_3A-7A_n78C-n257L DC_3A-7A_n78C-n257M | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_3A-7A-7A_n78A-n257A2 DC_3A-7A-7A_n78A-n257D2 DC_3A-7A-7A_n78A-n257E2 DC_3A-7A-7A_n78A-n257F2 DC_3A-7A-7A_n78A-n257G2 DC_3A-7A-7A_n78A-n257H2 DC_3A-7A-7A_n78A-n257I2 DC_3A-7A-7A_n78A-n257J2 DC_3A-7A-7A_n78A-n257K2 DC_3A-7A-7A_n78A-n257L2 DC_3A-7A-7A_n78A-n257M2 DC_3A-7A-7A_n78C-n257A DC_3A-7A-7A_n78C-n257D DC_3A-7A-7A_n78C-n257E DC_3A-7A-7A_n78C-n257F DC_3A-7A-7A_n78C-n257G DC_3A-7A-7A_n78C-n257H DC_3A-7A-7A_n78C-n257I DC_3A-7A-7A_n78C-n257J DC_3A-7A-7A_n78C-n257K DC_3A-7A-7A_n78C-n257L DC_3A-7A-7A_n78C-n257M | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_3A-7A_n78A-n258A DC_3A-7A_n78A-n258D DC_3A-7A_n78A-n258E DC_3A-7A_n78A-n258F DC_3A-7A_n78A-n258G DC_3A-7A_n78A-n258H DC_3A-7A_n78A-n258I DC_3A-7A_n78A-n258J DC_3A-7A_n78A-n258K DC_3A-7A_n78A-n258L DC_3A-7A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_7A_n78A DC_7A_n258A DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I |
| DC_3A-8A_n1A-n257A2 DC_3A-8A_n1A-n257D2 DC_3A-8A_n1A-n257E2 DC_3A-8A_n1A-n257F2 DC_3A-8A_n1A-n257G2 DC_3A-8A_n1A-n257H2 DC_3A-8A_n1A-n257I2 DC_3A-8A_n1A-n257J2 DC_3A-8A_n1A-n257K2 DC_3A-8A_n1A-n257L2 DC_3A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-8A_n1A-n257A2 DC_3A-3A-8A_n1A-n257D2 DC_3A-3A-8A_n1A-n257E2 DC_3A-3A-8A_n1A-n257F2 DC_3A-3A-8A_n1A-n257G2 DC_3A-3A-8A_n1A-n257H2 DC_3A-3A-8A_n1A-n257I2 DC_3A-3A-8A_n1A-n257J2 DC_3A-3A-8A_n1A-n257K2 DC_3A-3A-8A_n1A-n257L2 DC_3A-3A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-8A_n40A-n258A DC_3A-8A_n40A-n258D DC_3A-8A_n40A-n258E DC_3A-8A_n40A-n258F DC_3A-8A_n40A-n258G DC_3A-8A_n40A-n258H DC_3A-8A_n40A-n258I DC_3A-8A_n40A-n258J DC_3A-8A_n40A-n258K DC_3A-8A_n40A-n258L DC_3A-8A_n40A-n258M | DC_3A_n40A DC_3A_n258A DC_8A_n40A DC_8A_n258A |
| DC_3A-8A_n78A-n257A2 DC_3A-8A_n78A-n257D2 DC_3A-8A_n78A-n257E2 DC_3A-8A_n78A-n257F2 DC_3A-8A_n78A-n257G2 DC_3A-8A_n78A-n257H2 DC_3A-8A_n78A-n257I2 DC_3A-8A_n78A-n257J2 DC_3A-8A_n78A-n257K2 DC_3A-8A_n78A-n257L2 DC_3A-8A_n78A-n257M2 DC_3C-8A_n78A-n257A DC_3C-8A_n78A-n257D DC_3C-8A_n78A-n257G DC_3C-8A_n78A-n257E DC_3C-8A_n78A-n257F DC_3C-8A_n78A-n257H DC_3C-8A_n78A-n257I DC_3C-8A_n78A-n257J DC_3C-8A_n78A-n257K DC_3C-8A_n78A-n257L DC_3C-8A_n78A-n257M | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-8A_n78A-n257A2 DC_3A-3A-8A_n78A-n257D2 DC_3A-3A-8A_n78A-n257E2 DC_3A-3A-8A_n78A-n257F2 DC_3A-3A-8A_n78A-n257G2 DC_3A-3A-8A_n78A-n257H2 DC_3A-3A-8A_n78A-n257I2 DC_3A-3A-8A_n78A-n257J2 DC_3A-3A-8A_n78A-n257K2 DC_3A-3A-8A_n78A-n257L2 DC_3A-3A-8A_n78A-n257M2 | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-8A_n78A-n258A DC_3A-8A_n78A-n258D DC_3A-8A_n78A-n258E DC_3A-8A_n78A-n258F DC_3A-8A_n78A-n258G DC_3A-8A_n78A-n258H DC_3A-8A_n78A-n258I DC_3A-8A_n78A-n258J DC_3A-8A_n78A-n258K DC_3A-8A_n78A-n258L DC_3A-8A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_8A_n78A DC_8A_n258A |
| DC_3A-18A_n78A-n257A DC_3A-18A_n78A-n257G DC_3A-18A_n78A-n257H DC_3A-18A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_3A-19A_n77A-n257A2 DC_3A-19A_n77A-n257G2 DC_3A-19A_n77A-n257H2 DC_3A-19A_n77A-n257I2 | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_19A_n77A-n257A DC_19A_n77A-n257G DC_19A_n77A-n257H DC_19A_n77A-n257I |
| DC_3A-19A_n78A-n257A2 DC_3A-19A_n78A-n257G2 DC_3A-19A_n78A-n257H2 DC_3A-19A_n78A-n257I2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_19A_n78A-n257A DC_19A_n78A-n257G DC_19A_n78A-n257H DC_19A_n78A-n257I |
| DC_3A-19A_n79A-n257A2 DC_3A-19A_n79A-n257G2 DC_3A-19A_n79A-n257H2 DC_3A-19A_n79A-n257I2 | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_3A-21A_n77A-n257A2 DC_3A-21A_n77A-n257G2 DC_3A-21A_n77A-n257H2 DC_3A-21A_n77A-n257I2 | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_3A-21A_n78A-n257A2 DC_3A-21A_n78A-n257G2 DC_3A-21A_n78A-n257H2 DC_3A-21A_n78A-n257I2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_3A-21A_n79A-n257A2 DC_3A-21A_n79A-n257G2 DC_3A-21A_n79A-n257H2 DC_3A-21A_n79A-n257I2 | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_3A-28A_n77A-n257A2 DC_3A-28A_n77A-n257D2 DC_3A-28A_n77A-n257G2 DC_3A-28A_n77A-n257H2 DC_3A-28A_n77A-n257I2 | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n77A DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-28A_n77(2A)-n257A2 DC_3A-28A_n77(2A)-n257D2 DC_3A-28A_n77(2A)-n257G2 DC_3A-28A_n77(2A)-n257H2 DC_3A-28A_n77(2A)-n257I2 | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n77A DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-28A_n78A-n257A2 DC_3A-28A_n78A-n257G2 DC_3A-28A_n78A-n257H2 DC_3A-28A_n78A-n257I2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-41A_n28A-n257A2 DC_3A-41A_n28A-n257G2 DC_3A-41A_n28A-n257H2 DC_3A-41A_n28A-n257I2 DC_3A-41C_n28A-n257A2 DC_3A-41C_n28A-n257G2 DC_3A-41C_n28A-n257H2 DC_3A-41C_n28A-n257I2 | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-41A_n77A-n257A DC_3A-41A_n77A-n257G DC_3A-41A_n77A-n257H DC_3A-41A_n77A-n257I DC_3A-41C_n77A-n257A DC_3A-41C_n77A-n257G DC_3A-41C_n77A-n257H DC_3A-41C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-41A_n78A-n257A DC_3A-41A_n78A-n257G DC_3A-41A_n78A-n257H DC_3A-41A_n78A-n257I DC_3A-41C_n78A-n257A DC_3A-41C_n78A-n257G DC_3A-41C_n78A-n257H DC_3A-41C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-42A_n77A-n257A DC_3A-42A_n77A-n257G DC_3A-42A_n77A-n257H DC_3A-42A_n77A-n257I DC_3A-42C_n77A-n257A DC_3A-42C_n77A-n257G DC_3A-42C_n77A-n257H DC_3A-42C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I |
| DC_3A-42A_n78A-n257A DC_3A-42A_n78A-n257G DC_3A-42A_n78A-n257H DC_3A-42A_n78A-n257I DC_3A-42C_n78A-n257A DC_3A-42C_n78A-n257G DC_3A-42C_n78A-n257H DC_3A-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I |
| DC_3A-42A_n79A-n257A DC_3A-42A_n79A-n257G DC_3A-42A_n79A-n257H DC_3A-42A_n79A-n257I DC_3A-42C_n79A-n257A DC_3A-42C_n79A-n257G DC_3A-42C_n79A-n257H DC_3A-42C_n79A-n257I | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I |
| DC_3A-42A_n79A-n257A DC_3A-42A_n79A-n257G DC_3A-42A_n79A-n257H DC_3A-42A_n79A-n257I DC_3A-42C_n79A-n257A DC_3A-42C_n79A-n257G DC_3A-42C_n79A-n257H DC_3A-42C_n79A-n257I | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_5A-7A_n78A-n257A DC_5A-7A_n78A-n257D DC_5A-7A_n78A-n257E DC_5A-7A_n78A-n257F DC_5A-7A_n78A-n257G DC_5A-7A_n78A-n257H DC_5A-7A_n78A-n257I DC_5A-7A_n78A-n257J DC_5A-7A_n78A-n257K DC_5A-7A_n78A-n257L DC_5A-7A_n78A-n257M | DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_5A-7A_n78C-n257A DC_5A-7A_n78C-n257D DC_5A-7A_n78C-n257E DC_5A-7A_n78C-n257F DC_5A-7A_n78C-n257G DC_5A-7A_n78C-n257H DC_5A-7A_n78C-n257I DC_5A-7A_n78C-n257J DC_5A-7A_n78C-n257K DC_5A-7A_n78C-n257L DC_5A-7A_n78C-n257M | DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_5A-7A-7A_n78A-n257A DC_5A-7A-7A_n78A-n257D DC_5A-7A-7A_n78A-n257E DC_5A-7A-7A_n78A-n257F DC_5A-7A-7A_n78A-n257G DC_5A-7A-7A_n78A-n257H DC_5A-7A-7A_n78A-n257I DC_5A-7A-7A_n78A-n257J DC_5A-7A-7A_n78A-n257K DC_5A-7A-7A_n78A-n257L DC_5A-7A-7A_n78A-n257M | DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_5A-7A-7A_n78C-n257A DC_5A-7A-7A_n78C-n257D DC_5A-7A-7A_n78C-n257E DC_5A-7A-7A_n78C-n257F DC_5A-7A-7A_n78C-n257G DC_5A-7A-7A_n78C-n257H DC_5A-7A-7A_n78C-n257I DC_5A-7A-7A_n78C-n257J DC_5A-7A-7A_n78C-n257K DC_5A-7A-7A_n78C-n257L DC_5A-7A-7A_n78C-n257M | DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_7A_n1A-n78A-n257A2 DC_7A_n1A-n78A-n257D2 DC_7A_n1A-n78A-n257E2 DC_7A_n1A-n78A-n257F2 DC_7A_n1A-n78A-n257G2 DC_7A_n1A-n78A-n257H2 DC_7A_n1A-n78A-n257I2 DC_7A_n1A-n78A-n257J2 DC_7A_n1A-n78A-n257K2 DC_7A_n1A-n78A-n257L2 DC_7A_n1A-n78A-n257M2 | DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_7A-7A_n1A-n78A-n257A2 DC_7A-7A_n1A-n78A-n257D2 DC_7A-7A_n1A-n78A-n257E2 DC_7A-7A_n1A-n78A-n257F2 DC_7A-7A_n1A-n78A-n257G2 DC_7A-7A_n1A-n78A-n257H2 DC_7A-7A_n1A-n78A-n257I2 DC_7A-7A_n1A-n78A-n257J2 DC_7A-7A_n1A-n78A-n257K2 DC_7A-7A_n1A-n78A-n257L2 DC_7A-7A_n1A-n78A-n257M2 | DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_7A-8A_n1A-n257A2 DC_7A-8A_n1A-n257D2 DC_7A-8A_n1A-n257E2 DC_7A-8A_n1A-n257F2 DC_7A-8A_n1A-n257G2 DC_7A-8A_n1A-n257H2 DC_7A-8A_n1A-n257I2 DC_7A-8A_n1A-n257J2 DC_7A-8A_n1A-n257K2 DC_7A-8A_n1A-n257L2 DC_7A-8A_n1A-n257M2 | DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_7A-7A-8A_n1A-n257A2 DC_7A-7A-8A_n1A-n257D2 DC_7A-7A-8A_n1A-n257E2 DC_7A-7A-8A_n1A-n257F2 DC_7A-7A-8A_n1A-n257G2 DC_7A-7A-8A_n1A-n257H2 DC_7A-7A-8A_n1A-n257I2 DC_7A-7A-8A_n1A-n257J2 DC_7A-7A-8A_n1A-n257K2 DC_7A-7A-8A_n1A-n257L2 DC_7A-7A-8A_n1A-n257M2 | DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_7A-8A_n40A-n258A DC_7A-8A_n40A-n258D DC_7A-8A_n40A-n258E DC_7A-8A_n40A-n258F DC_7A-8A_n40A-n258G DC_7A-8A_n40A-n258H DC_7A-8A_n40A-n258I DC_7A-8A_n40A-n258J DC_7A-8A_n40A-n258K DC_7A-8A_n40A-n258L DC_7A-8A_n40A-n258M | DC_7A_n40A DC_7A_n258A DC_8A_n40A DC_8A_n258A |
| DC_7A-8A_n78A-n257A2 DC_7A-8A_n78A-n257D2 DC_7A-8A_n78A-n257E2 DC_7A-8A_n78A-n257F2 DC_7A-8A_n78A-n257G2 DC_7A-8A_n78A-n257H2 DC_7A-8A_n78A-n257I2 DC_7A-8A_n78A-n257J2 DC_7A-8A_n78A-n257K2 DC_7A-8A_n78A-n257L2 DC_7A-8A_n78A-n257M2 | DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_7A-7A-8A_n78A-n257A2 DC_7A-7A-8A_n78A-n257D2 DC_7A-7A-8A_n78A-n257E2 DC_7A-7A-8A_n78A-n257F2 DC_7A-7A-8A_n78A-n257G2 DC_7A-7A-8A_n78A-n257H2 DC_7A-7A-8A_n78A-n257I2 DC_7A-7A-8A_n78A-n257J2 DC_7A-7A-8A_n78A-n257K2 DC_7A-7A-8A_n78A-n257L2 DC_7A-7A-8A_n78A-n257M2 | DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_7A-8A_n78A-n258A DC_7A-8A_n78A-n258D DC_7A-8A_n78A-n258E DC_7A-8A_n78A-n258F DC_7A-8A_n78A-n258G DC_7A-8A_n78A-n258H DC_7A-8A_n78A-n258I DC_7A-8A_n78A-n258J DC_7A-8A_n78A-n258K DC_7A-8A_n78A-n258L DC_7A-8A_n78A-n258M | DC_7A_n78A DC_7A_n258A DC_8A_n78A DC_8A_n258A |
| DC_7A-28A_n78A-n257A DC_7A-28A_n78A-n257G DC_7A-28A_n78A-n257H DC_7A-28A_n78A-n257I | DC_7A_n78A DC_28A_n78A DC_7A_n257A DC_28A_n257A |
| DC_8A_n1A-n78A-n257A2 | DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_8A-11A_n77A-n257A2 DC_8A-11A_n77A-n257D2 DC_8A-11A_n77A-n257G2 DC_8A-11A_n77A-n257H2 DC_8A-11A_n77A-n257I2 | DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_8A-11A_n77(2A)-n257A2 DC_8A-11A_n77(2A)-n257D2 DC_8A-11A_n77(2A)-n257G2 DC_8A-11A_n77(2A)-n257H2 DC_8A-11A_n77(2A)-n257I2 | DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_18A-41A_n3A-n257A DC_18A-41A_n3A-n257G DC_18A-41A_n3A-n257H DC_18A-41A_n3A-n257I DC_18A-41C_n3A-n257A DC_18A-41C_n3A-n257G DC_18A-41C_n3A-n257H DC_18A-41C_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_18A-42A_n78A-n257A DC_18A-42A_n78A-n257G DC_18A-42A_n78A-n257H DC_18A-42A_n78A-n257I DC_18A-42C_n78A-n257A DC_18A-42C_n78A-n257G DC_18A-42C_n78A-n257H DC_18A-42C_n78A-n257I | DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_19A-21A_n77A-n257A2 DC_19A-21A_n77A-n257G2 DC_19A-21A_n77A-n257H2 DC_19A-21A_n77A-n257I2 | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_19A-21A_n78A-n257A2 DC_19A-21A_n78A-n257G2 DC_19A-21A_n78A-n257H2 DC_19A-21A_n78A-n257I2 | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_19A-21A_n79A-n257A2 DC_19A-21A_n79A-n257G2 DC_19A-21A_n79A-n257H2 DC_19A-21A_n79A-n257I2 | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_19A-42A_n77A-n257A DC_19A-42A_n77A-n257G DC_19A-42A_n77A-n257H DC_19A-42A_n77A-n257I DC_19A-42C_n77A-n257A DC_19A-42C_n77A-n257G DC_19A-42C_n77A-n257H DC_19A-42C_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-42A_n78A-n257A DC_19A-42A_n78A-n257G DC_19A-42A_n78A-n257H DC_19A-42A_n78A-n257I DC_19A-42C_n78A-n257A DC_19A-42C_n78A-n257G DC_19A-42C_n78A-n257H DC_19A-42C_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-42A_n79A-n257A DC_19A-42A_n79A-n257G DC_19A-42A_n79A-n257H DC_19A-42A_n79A-n257I DC_19A-42C_n79A-n257A DC_19A-42C_n79A-n257G DC_19A-42C_n79A-n257H DC_19A-42C_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_21A-42A_n77A-n257A DC_21A-42A_n77A-n257G DC_21A-42A_n77A-n257H DC_21A-42A_n77A-n257I DC_21A-42C_n77A-n257A DC_21A-42C_n77A-n257G DC_21A-42C_n77A-n257H DC_21A-42C_n77A-n257I | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_21A-42A_n78A-n257A DC_21A-42A_n78A-n257G DC_21A-42A_n78A-n257H DC_21A-42A_n78A-n257I DC_21A-42C_n78A-n257A DC_21A-42C_n78A-n257G DC_21A-42C_n78A-n257H DC_21A-42C_n78A-n257I | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_21A-42A_n79A-n257A DC_21A-42A_n79A-n257G DC_21A-42A_n79A-n257H DC_21A-42A_n79A-n257I DC_21A-42C_n79A-n257A DC_21A-42C_n79A-n257G DC_21A-42C_n79A-n257H DC_21A-42C_n79A-n257I | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_28A-41A_n78A-n257A DC_28A-41A_n78A-n257G DC_28A-41A_n78A-n257H DC_28A-41A_n78A-n257I DC_28A-41C_n78A-n257A DC_28A-41C_n78A-n257G DC_28A-41C_n78A-n257H DC_28A-41C_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_28A-42A_n78A-n257A DC_28A-42A_n78A-n257G DC_28A-42A_n78A-n257H DC_28A-42A_n78A-n257I DC_28A-42C_n78A-n257A DC_28A-42C_n78A-n257G DC_28A-42C_n78A-n257H DC_28A-42C_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_40A-42A_n77A-n257A DC_40A-42A_n77A-n257D DC_40A-42A_n77A-n257E DC_40A-42A_n77A-n257F DC_40A-42A_n77A-n257G DC_40A-42A_n77A-n257H DC_40A-42A_n77A-n257I DC_40A-42A_n77A-n257J DC_40A-42A_n77A-n257K DC_40A-42A_n77A-n257L DC_40A-42A_n77A-n257M DC_40A-42A_n77C-n257A DC_40A-42A_n77C-n257D DC_40A-42A_n77C-n257E DC_40A-42A_n77C-n257F DC_40A-42A_n77C-n257G DC_40A-42A_n77C-n257H DC_40A-42A_n77C-n257I DC_40A-42A_n77C-n257J DC_40A-42A_n77C-n257K DC_40A-42A_n77C-n257L DC_40A-42A_n77C-n257M | DC_40A_n257A DC_40A_n257D DC_40A_n257E DC_40A_n257F DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_40A_n77A |
| DC_40A-42A_n78A-n257A DC_40A-42A_n78A-n257D DC_40A-42A_n78A-n257E DC_40A-42A_n78A-n257F DC_40A-42A_n78A-n257G DC_40A-42A_n78A-n257H DC_40A-42A_n78A-n257I DC_40A-42A_n78A-n257J DC_40A-42A_n78A-n257K DC_40A-42A_n78A-n257L DC_40A-42A_n78A-n257M | DC_40A_n257A DC_40A_n257D DC_40A_n257E DC_40A_n257F DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_40A_n78A |
| DC_41A-42A_n77A-n257A DC_41A-42A_n77A-n257G DC_41A-42A_n77A-n257H DC_41A-42A_n77A-n257I DC_41A-42C_n77A-n257A DC_41A-42C_n77A-n257G DC_41A-42C_n77A-n257H DC_41A-42C_n77A-n257I DC_41C-42A_n77A-n257A DC_41C-42A_n77A-n257G DC_41C-42A_n77A-n257H DC_41C-42A_n77A-n257I DC_41C-42C_n77A-n257A DC_41C-42C_n77A-n257G DC_41C-42C_n77A-n257H DC_41C-42C_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_41A-42A_n78A-n257A DC_41A-42A_n78A-n257G DC_41A-42A_n78A-n257H DC_41A-42A_n78A-n257I DC_41A-42C_n78A-n257A DC_41A-42C_n78A-n257G DC_41A-42C_n78A-n257H DC_41A-42C_n78A-n257I DC_41C-42A_n78A-n257A DC_41C-42A_n78A-n257G DC_41C-42A_n78A-n257H DC_41C-42A_n78A-n257I DC_41C-42C_n78A-n257A DC_41C-42C_n78A-n257G DC_41C-42C_n78A-n257H DC_41C-42C_n78A-n257I | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 3: Only single switched UL is supported. |  |

5.5B.6.4	Inter-band EN-DC configurations including FR1 and FR2 (five bands)

Table 5.5B.6.4-1: Inter-band EN-DC configurations including FR1 and FR2 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A_n78A-n257A DC_1A-3A-5A_n78A-n257D DC_1A-3A-5A_n78A-n257E DC_1A-3A-5A_n78A-n257F DC_1A-3A-5A_n78A-n257G DC_1A-3A-5A_n78A-n257H DC_1A-3A-5A_n78A-n257I DC_1A-3A-5A_n78A-n257J DC_1A-3A-5A_n78A-n257K DC_1A-3A-5A_n78A-n257L DC_1A-3A-5A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_1A-3A-5A_n78C-n257A DC_1A-3A-5A_n78C-n257D DC_1A-3A-5A_n78C-n257E DC_1A-3A-5A_n78C-n257F DC_1A-3A-5A_n78C-n257G DC_1A-3A-5A_n78C-n257H DC_1A-3A-5A_n78C-n257I DC_1A-3A-5A_n78C-n257J DC_1A-3A-5A_n78C-n257K DC_1A-3A-5A_n78C-n257L DC_1A-3A-5A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I |
| DC_1A-3A-7A_n78A-n257A DC_1A-3A-7A_n78A-n257D DC_1A-3A-7A_n78A-n257E DC_1A-3A-7A_n78A-n257F DC_1A-3A-7A_n78A-n257G DC_1A-3A-7A_n78A-n257H DC_1A-3A-7A_n78A-n257I DC_1A-3A-7A_n78A-n257J DC_1A-3A-7A_n78A-n257K DC_1A-3A-7A_n78A-n257L DC_1A-3A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-7A_n78C-n257A DC_1A-3A-7A_n78C-n257D DC_1A-3A-7A_n78C-n257E DC_1A-3A-7A_n78C-n257F DC_1A-3A-7A_n78C-n257G DC_1A-3A-7A_n78C-n257H DC_1A-3A-7A_n78C-n257I DC_1A-3A-7A_n78C-n257J DC_1A-3A-7A_n78C-n257K DC_1A-3A-7A_n78C-n257L DC_1A-3A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-7A-7A_n78A-n257A DC_1A-3A-7A-7A_n78A-n257D DC_1A-3A-7A-7A_n78A-n257E DC_1A-3A-7A-7A_n78A-n257F DC_1A-3A-7A-7A_n78A-n257G DC_1A-3A-7A-7A_n78A-n257H DC_1A-3A-7A-7A_n78A-n257I DC_1A-3A-7A-7A_n78A-n257J DC_1A-3A-7A-7A_n78A-n257K DC_1A-3A-7A-7A_n78A-n257L DC_1A-3A-7A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-7A-7A_n78C-n257A DC_1A-3A-7A-7A_n78C-n257D DC_1A-3A-7A-7A_n78C-n257E DC_1A-3A-7A-7A_n78C-n257F DC_1A-3A-7A-7A_n78C-n257G DC_1A-3A-7A-7A_n78C-n257H DC_1A-3A-7A-7A_n78C-n257I DC_1A-3A-7A-7A_n78C-n257J DC_1A-3A-7A-7A_n78C-n257K DC_1A-3A-7A-7A_n78C-n257L DC_1A-3A-7A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-7A_n78A-n258A DC_1A-3A-7A_n78A-n258D DC_1A-3A-7A_n78A-n258E DC_1A-3A-7A_n78A-n258F DC_1A-3A-7A_n78A-n258G DC_1A-3A-7A_n78A-n258H DC_1A-3A-7A_n78A-n258I DC_1A-3A-7A_n78A-n258J DC_1A-3A-7A_n78A-n258K DC_1A-3A-7A_n78A-n258L DC_1A-3A-7A_n78A-n258M | DC_1A_n78A DC_1A_n258A DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_3A_n78A DC_3A_n258A DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_7A_n78A DC_7A_n258A DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I |
| DC_1A-3A-8A_n78A-n257A2 DC_1A-3A-8A_n78A-n257D2 DC_1A-3A-8A_n78A-n257E2 DC_1A-3A-8A_n78A-n257F2 DC_1A-3A-8A_n78A-n257G2 DC_1A-3A-8A_n78A-n257H2 DC_1A-3A-8A_n78A-n257I2 DC_1A-3A-8A_n78A-n257J2 DC_1A-3A-8A_n78A-n257K2 DC_1A-3A-8A_n78A-n257L2 DC_1A-3A-8A_n78A-n257M2 DC_1A-3C-8A_n78A-n257A DC_1A-3C-8A_n78A-n257D DC_1A-3C-8A_n78A-n257E DC_1A-3C-8A_n78A-n257F DC_1A-3C-8A_n78A-n257G DC_1A-3C-8A_n78A-n257H DC_1A-3C-8A_n78A-n257I DC_1A-3C-8A_n78A-n257J DC_1A-3C-8A_n78A-n257K DC_1A-3C-8A_n78A-n257L DC_1A-3C-8A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_8A_n78A DC_8A_n257A |
| DC_1A-3A-18A_n78A-n257A DC_1A-3A-18A_n78A-n257G DC_1A-3A-18A_n78A-n257H DC_1A-3A-18A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-3A-21A_n77A-n257A2 DC_1A-3A-21A_n77A-n257G2 DC_1A-3A-21A_n77A-n257H2 DC_1A-3A-21A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_1A-3A-21A_n78A-n257A2 DC_1A-3A-21A_n78A-n257G2 DC_1A-3A-21A_n78A-n257H2 DC_1A-3A-21A_n78A-n257I2 | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_1A-3A-21A_n79A-n257A2 DC_1A-3A-21A_n79A-n257G2 DC_1A-3A-21A_n79A-n257H2 DC_1A-3A-21A_n79A-n257I2 | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_1A-3A-28A_n78A-n257A2 DC_1A-3A-28A_n78A-n257G2 DC_1A-3A-28A_n78A-n257H2 DC_1A-3A-28A_n78A-n257I2 | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-3A-41A_n28A-n257A2 DC_1A-3A-41A_n28A-n257G DC_1A-3A-41A_n28A-n257H DC_1A-3A-41A_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257A DC_41A_n257I |
| DC_1A-3A-41C_n28A-n257A2 DC_1A-3A-41C_n28A-n257G DC_1A-3A-41C_n28A-n257H DC_1A-3A-41C_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257A DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257I |
| DC_1A-3A-41A_n77A-n257A DC_1A-3A-41A_n77A-n257G DC_1A-3A-41A_n77A-n257H DC_1A-3A-41A_n77A-n257I DC_1A-3A-41C_n77A-n257A DC_1A-3A-41C_n77A-n257G DC_1A-3A-41C_n77A-n257H DC_1A-3A-41C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-3A-41A_n78A-n257A DC_1A-3A-41A_n78A-n257G DC_1A-3A-41A_n78A-n257H DC_1A-3A-41A_n78A-n257I DC_1A-3A-41C_n78A-n257A DC_1A-3A-41C_n78A-n257G DC_1A-3A-41C_n78A-n257H DC_1A-3A-41C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-3A-42A_n77A-n257A DC_1A-3A-42A_n77A-n257G DC_1A-3A-42A_n77A-n257H DC_1A-3A-42A_n77A-n257I DC_1A-3A-42C_n77A-n257A DC_1A-3A-42C_n77A-n257G DC_1A-3A-42C_n77A-n257H DC_1A-3A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-42A_n78A-n257A DC_1A-3A-42A_n78A-n257G DC_1A-3A-42A_n78A-n257H DC_1A-3A-42A_n78A-n257I DC_1A-3A-42C_n78A-n257A DC_1A-3A-42C_n78A-n257G DC_1A-3A-42C_n78A-n257H DC_1A-3A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-5A-7A_n78A-n257A DC_1A-5A-7A_n78A-n257D DC_1A-5A-7A_n78A-n257E DC_1A-5A-7A_n78A-n257F DC_1A-5A-7A_n78A-n257G DC_1A-5A-7A_n78A-n257H DC_1A-5A-7A_n78A-n257I DC_1A-5A-7A_n78A-n257J DC_1A-5A-7A_n78A-n257K DC_1A-5A-7A_n78A-n257L DC_1A-5A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-5A-7A_n78C-n257A DC_1A-5A-7A_n78C-n257D DC_1A-5A-7A_n78C-n257E DC_1A-5A-7A_n78C-n257F DC_1A-5A-7A_n78C-n257G DC_1A-5A-7A_n78C-n257H DC_1A-5A-7A_n78C-n257I DC_1A-5A-7A_n78C-n257J DC_1A-5A-7A_n78C-n257K DC_1A-5A-7A_n78C-n257L DC_1A-5A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-5A-7A-7A_n78A-n257A DC_1A-5A-7A-7A_n78A-n257D DC_1A-5A-7A-7A_n78A-n257E DC_1A-5A-7A-7A_n78A-n257F DC_1A-5A-7A-7A_n78A-n257G DC_1A-5A-7A-7A_n78A-n257H DC_1A-5A-7A-7A_n78A-n257I DC_1A-5A-7A-7A_n78A-n257J DC_1A-5A-7A-7A_n78A-n257K DC_1A-5A-7A-7A_n78A-n257L DC_1A-5A-7A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-5A-7A-7A_n78C-n257A DC_1A-5A-7A-7A_n78C-n257D DC_1A-5A-7A-7A_n78C-n257E DC_1A-5A-7A-7A_n78C-n257F DC_1A-5A-7A-7A_n78C-n257G DC_1A-5A-7A-7A_n78C-n257H DC_1A-5A-7A-7A_n78C-n257I DC_1A-5A-7A-7A_n78C-n257J DC_1A-5A-7A-7A_n78C-n257K DC_1A-5A-7A-7A_n78C-n257L DC_1A-5A-7A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-7A-28A_n78A-n257A DC_1A-7A-28A_n78A-n257G DC_1A-7A-28A_n78A-n257H DC_1A-7A-28A_n78A-n257I | DC_1A_n78A DC_7A_n78A DC_28A_n78A DC_1A_n257A DC_7A_n257A DC_28A_n257A |
| DC_1A-8A-11A_n77A-n257A2 DC_1A-8A-11A_n77A-n257D2 DC_1A-8A-11A_n77A-n257G2 DC_1A-8A-11A_n77A-n257H2 DC_1A-8A-11A_n77A-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-8A-11A_n77(2A)-n257A2 DC_1A-8A-11A_n77(2A)-n257D2 DC_1A-8A-11A_n77(2A)-n257G2 DC_1A-8A-11A_n77(2A)-n257H2 DC_1A-8A-11A_n77(2A)-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_8A_n77A DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_11A_n77A DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_1A-18A-41A_n3A-n257A DC_1A-18A-41A_n3A-n257G DC_1A-18A-41A_n3A-n257H DC_1A-18A-41A_n3A-n257I DC_1A-18A-41C_n3A-n257A DC_1A-18A-41C_n3A-n257G DC_1A-18A-41C_n3A-n257H DC_1A-18A-41C_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_41A_n3A DC_41C_n3A DC_41A_n257A DC_41C_n257A DC_18A_n257I DC_41A_n257I DC_41C_n257I |
| DC_1A-18A-42A_n78A-n257A DC_1A-18A-42A_n78A-n257G DC_1A-18A-42A_n78A-n257H DC_1A-18A-42A_n78A-n257I DC_1A-18A-42C_n78A-n257A DC_1A-18A-42C_n78A-n257G DC_1A-18A-42C_n78A-n257H DC_1A-18A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-19A-42A_n77A-n257A DC_1A-19A-42A_n77A-n257G DC_1A-19A-42A_n77A-n257H DC_1A-19A-42A_n77A-n257I DC_1A-19A-42C_n77A-n257A DC_1A-19A-42C_n77A-n257G DC_1A-19A-42C_n77A-n257H DC_1A-19A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-19A-42A_n78A-n257A DC_1A-19A-42A_n78A-n257G DC_1A-19A-42A_n78A-n257H DC_1A-19A-42A_n78A-n257I DC_1A-19A-42C_n78A-n257A DC_1A-19A-42C_n78A-n257G DC_1A-19A-42C_n78A-n257H DC_1A-19A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-19A-42A_n79A-n257A DC_1A-19A-42A_n79A-n257G DC_1A-19A-42A_n79A-n257H DC_1A-19A-42A_n79A-n257I DC_1A-19A-42C_n79A-n257A DC_1A-19A-42C_n79A-n257G DC_1A-19A-42C_n79A-n257H DC_1A-19A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_1A-21A-42A_n77A-n257A DC_1A-21A-42A_n77A-n257G DC_1A-21A-42A_n77A-n257H DC_1A-21A-42A_n77A-n257I DC_1A-21A-42C_n77A-n257A DC_1A-21A-42C_n77A-n257G DC_1A-21A-42C_n77A-n257H DC_1A-21A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_1A-21A-42A_n78A-n257A DC_1A-21A-42A_n78A-n257G DC_1A-21A-42A_n78A-n257H DC_1A-21A-42A_n78A-n257I DC_1A-21A-42C_n78A-n257A DC_1A-21A-42C_n78A-n257G DC_1A-21A-42C_n78A-n257H DC_1A-21A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_1A-21A-42A_n79A-n257A DC_1A-21A-42A_n79A-n257G DC_1A-21A-42A_n79A-n257H DC_1A-21A-42A_n79A-n257I DC_1A-21A-42C_n79A-n257A DC_1A-21A-42C_n79A-n257G DC_1A-21A-42C_n79A-n257H DC_1A-21A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_1A-28A-42A_n78A-n257A DC_1A-28A-42A_n78A-n257G DC_1A-28A-42A_n78A-n257H DC_1A-28A-42A_n78A-n257I DC_1A-28A-42C_n78A-n257A DC_1A-28A-42C_n78A-n257G DC_1A-28A-42C_n78A-n257H DC_1A-28A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-41A-42A_n77A-n257A DC_1A-41A-42A_n77A-n257G DC_1A-41A-42A_n77A-n257H DC_1A-41A-42A_n77A-n257I DC_1A-41A-42C_n77A-n257A DC_1A-41A-42C_n77A-n257G DC_1A-41A-42C_n77A-n257H DC_1A-41A-42C_n77A-n257I DC_1A-41C-42A_n77A-n257A DC_1A-41C-42A_n77A-n257G DC_1A-41C-42A_n77A-n257H DC_1A-41C-42A_n77A-n257I DC_1A-41C-42C_n77A-n257A DC_1A-41C-42C_n77A-n257G DC_1A-41C-42C_n77A-n257H DC_1A-41C-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-41A-42A_n78A-n257A DC_1A-41A-42A_n78A-n257G DC_1A-41A-42A_n78A-n257H DC_1A-41A-42A_n78A-n257I DC_1A-41A-42C_n78A-n257A DC_1A-41A-42C_n78A-n257G DC_1A-41A-42C_n78A-n257H DC_1A-41A-42C_n78A-n257I DC_1A-41C-42A_n78A-n257A DC_1A-41C-42A_n78A-n257G DC_1A-41C-42A_n78A-n257H DC_1A-41C-42A_n78A-n257I DC_1A-41C-42C_n78A-n257A DC_1A-41C-42C_n78A-n257G DC_1A-41C-42C_n78A-n257H DC_1A-41C-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-5A-7A-7A_n78A-n257A DC_3A-5A-7A-7A_n78A-n257D DC_3A-5A-7A-7A_n78A-n257E DC_3A-5A-7A-7A_n78A-n257F DC_3A-5A-7A-7A_n78A-n257G DC_3A-5A-7A-7A_n78A-n257H DC_3A-5A-7A-7A_n78A-n257I DC_3A-5A-7A-7A_n78A-n257J DC_3A-5A-7A-7A_n78A-n257K DC_3A-5A-7A-7A_n78A-n257L DC_3A-5A-7A-7A_n78A-n257M DC_3A-5A-7A-7A_n78C-n257A DC_3A-5A-7A-7A_n78C-n257D DC_3A-5A-7A-7A_n78C-n257E DC_3A-5A-7A-7A_n78C-n257F DC_3A-5A-7A-7A_n78C-n257G DC_3A-5A-7A-7A_n78C-n257H DC_3A-5A-7A-7A_n78C-n257I DC_3A-5A-7A-7A_n78C-n257J DC_3A-5A-7A-7A_n78C-n257K DC_3A-5A-7A-7A_n78C-n257L DC_3A-5A-7A-7A_n78C-n257M DC_3A-5A-7A_n78A-n257A DC_3A-5A-7A_n78A-n257D DC_3A-5A-7A_n78A-n257E DC_3A-5A-7A_n78A-n257F DC_3A-5A-7A_n78A-n257G DC_3A-5A-7A_n78A-n257H DC_3A-5A-7A_n78A-n257I DC_3A-5A-7A_n78A-n257J DC_3A-5A-7A_n78A-n257K DC_3A-5A-7A_n78A-n257L DC_3A-5A-7A_n78A-n257M DC_3A-5A-7A_n78C-n257A DC_3A-5A-7A_n78C-n257D DC_3A-5A-7A_n78C-n257E DC_3A-5A-7A_n78C-n257F DC_3A-5A-7A_n78C-n257G DC_3A-5A-7A_n78C-n257H DC_3A-5A-7A_n78C-n257I DC_3A-5A-7A_n78C-n257J DC_3A-5A-7A_n78C-n257K DC_3A-5A-7A_n78C-n257L DC_3A-5A-7A_n78C-n257M | DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_3A-7A_n1A-n78A-n257A2 DC_3A-7A_n1A-n78A-n257D2 DC_3A-7A_n1A-n78A-n257E2 DC_3A-7A_n1A-n78A-n257F2 DC_3A-7A_n1A-n78A-n257G2 DC_3A-7A_n1A-n78A-n257H2 DC_3A-7A_n1A-n78A-n257I2 DC_3A-7A_n1A-n78A-n257J2 DC_3A-7A_n1A-n78A-n257K2 DC_3A-7A_n1A-n78A-n257L2 DC_3A-7A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_3A-3A-7A_n1A-n78A-n257A2 DC_3A-3A-7A_n1A-n78A-n257D2 DC_3A-3A-7A_n1A-n78A-n257E2 DC_3A-3A-7A_n1A-n78A-n257F2 DC_3A-3A-7A_n1A-n78A-n257G2 DC_3A-3A-7A_n1A-n78A-n257H2 DC_3A-3A-7A_n1A-n78A-n257I2 DC_3A-3A-7A_n1A-n78A-n257J2 DC_3A-3A-7A_n1A-n78A-n257K2 DC_3A-3A-7A_n1A-n78A-n257L2 DC_3A-3A-7A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_3A-7A-7A_n1A-n78A-n257A2 DC_3A-7A-7A_n1A-n78A-n257D2 DC_3A-7A-7A_n1A-n78A-n257E2 DC_3A-7A-7A_n1A-n78A-n257F2 DC_3A-7A-7A_n1A-n78A-n257G2 DC_3A-7A-7A_n1A-n78A-n257H2 DC_3A-7A-7A_n1A-n78A-n257I2 DC_3A-7A-7A_n1A-n78A-n257J2 DC_3A-7A-7A_n1A-n78A-n257K2 DC_3A-7A-7A_n1A-n78A-n257L2 DC_3A-7A-7A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_3A-3A-7A-7A_n1A-n78A-n257A2 DC_3A-3A-7A-7A_n1A-n78A-n257D2 DC_3A-3A-7A-7A_n1A-n78A-n257E2 DC_3A-3A-7A-7A_n1A-n78A-n257F2 DC_3A-3A-7A-7A_n1A-n78A-n257G2 DC_3A-3A-7A-7A_n1A-n78A-n257H2 DC_3A-3A-7A-7A_n1A-n78A-n257I2 DC_3A-3A-7A-7A_n1A-n78A-n257J2 DC_3A-3A-7A-7A_n1A-n78A-n257K2 DC_3A-3A-7A-7A_n1A-n78A-n257L2 DC_3A-3A-7A-7A_n1A-n78A-n257M2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A |
| DC_3A-7A-8A_n1A-n257A2 DC_3A-7A-8A_n1A-n257D2 DC_3A-7A-8A_n1A-n257E2 DC_3A-7A-8A_n1A-n257F2 DC_3A-7A-8A_n1A-n257G2 DC_3A-7A-8A_n1A-n257H2 DC_3A-7A-8A_n1A-n257I2 DC_3A-7A-8A_n1A-n257J2 DC_3A-7A-8A_n1A-n257K2 DC_3A-7A-8A_n1A-n257L2 DC_3A-7A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-7A-8A_n1A-n257A2 DC_3A-3A-7A-8A_n1A-n257D2 DC_3A-3A-7A-8A_n1A-n257E2 DC_3A-3A-7A-8A_n1A-n257F2 DC_3A-3A-7A-8A_n1A-n257G2 DC_3A-3A-7A-8A_n1A-n257H2 DC_3A-3A-7A-8A_n1A-n257I2 DC_3A-3A-7A-8A_n1A-n257J2 DC_3A-3A-7A-8A_n1A-n257K2 DC_3A-3A-7A-8A_n1A-n257L2 DC_3A-3A-7A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-7A-7A-8A_n1A-n257A2 DC_3A-7A-7A-8A_n1A-n257D2 DC_3A-7A-7A-8A_n1A-n257E2 DC_3A-7A-7A-8A_n1A-n257F2 DC_3A-7A-7A-8A_n1A-n257G2 DC_3A-7A-7A-8A_n1A-n257H2 DC_3A-7A-7A-8A_n1A-n257I2 DC_3A-7A-7A-8A_n1A-n257J2 DC_3A-7A-7A-8A_n1A-n257K2 DC_3A-7A-7A-8A_n1A-n257L2 DC_3A-7A-7A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-7A-7A-8A_n1A-n257A2 DC_3A-3A-7A-7A-8A_n1A-n257D2 DC_3A-3A-7A-7A-8A_n1A-n257E2 DC_3A-3A-7A-7A-8A_n1A-n257F2 DC_3A-3A-7A-7A-8A_n1A-n257G2 DC_3A-3A-7A-7A-8A_n1A-n257H2 DC_3A-3A-7A-7A-8A_n1A-n257I2 DC_3A-3A-7A-7A-8A_n1A-n257J2 DC_3A-3A-7A-7A-8A_n1A-n257K2 DC_3A-3A-7A-7A-8A_n1A-n257L2 DC_3A-3A-7A-7A-8A_n1A-n257M2 | DC_3A_n1A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n1A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n1A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-7A-8A_n40A-n258A DC_3A-7A-8A_n40A-n258D DC_3A-7A-8A_n40A-n258E DC_3A-7A-8A_n40A-n258F DC_3A-7A-8A_n40A-n258G DC_3A-7A-8A_n40A-n258H DC_3A-7A-8A_n40A-n258I DC_3A-7A-8A_n40A-n258J DC_3A-7A-8A_n40A-n258K DC_3A-7A-8A_n40A-n258L DC_3A-7A-8A_n40A-n258M | DC_3A_n40A DC_3A_n258A DC_7A_n40A DC_7A_n258A DC_8A_n40A DC_8A_n258A |
| DC_3A-7A-8A_n78A-n257A2 DC_3A-7A-8A_n78A-n257D2 DC_3A-7A-8A_n78A-n257E2 DC_3A-7A-8A_n78A-n257F2 DC_3A-7A-8A_n78A-n257G2 DC_3A-7A-8A_n78A-n257H2 DC_3A-7A-8A_n78A-n257I2 DC_3A-7A-8A_n78A-n257J2 DC_3A-7A-8A_n78A-n257K2 DC_3A-7A-8A_n78A-n257L2 DC_3A-7A-8A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-7A-8A_n78A-n257A2 DC_3A-3A-7A-8A_n78A-n257D2 DC_3A-3A-7A-8A_n78A-n257E2 DC_3A-3A-7A-8A_n78A-n257F2 DC_3A-3A-7A-8A_n78A-n257G2 DC_3A-3A-7A-8A_n78A-n257H2 DC_3A-3A-7A-8A_n78A-n257I2 DC_3A-3A-7A-8A_n78A-n257J2 DC_3A-3A-7A-8A_n78A-n257K2 DC_3A-3A-7A-8A_n78A-n257L2 DC_3A-3A-7A-8A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-7A-7A-8A_n78A-n257A2 DC_3A-7A-7A-8A_n78A-n257D2 DC_3A-7A-7A-8A_n78A-n257E2 DC_3A-7A-7A-8A_n78A-n257F2 DC_3A-7A-7A-8A_n78A-n257G2 DC_3A-7A-7A-8A_n78A-n257H2 DC_3A-7A-7A-8A_n78A-n257I2 DC_3A-7A-7A-8A_n78A-n257J2 DC_3A-7A-7A-8A_n78A-n257K2 DC_3A-7A-7A-8A_n78A-n257L2 DC_3A-7A-7A-8A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-3A-7A-7A-8A_n78A-n257A2 DC_3A-3A-7A-7A-8A_n78A-n257D2 DC_3A-3A-7A-7A-8A_n78A-n257E2 DC_3A-3A-7A-7A-8A_n78A-n257F2 DC_3A-3A-7A-7A-8A_n78A-n257G2 DC_3A-3A-7A-7A-8A_n78A-n257H2 DC_3A-3A-7A-7A-8A_n78A-n257I2 DC_3A-3A-7A-7A-8A_n78A-n257J2 DC_3A-3A-7A-7A-8A_n78A-n257K2 DC_3A-3A-7A-7A-8A_n78A-n257L2 DC_3A-3A-7A-7A-8A_n78A-n257M2 | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_8A_n78A DC_8A_n257A DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K |
| DC_3A-7A-8A_n78A-n258A DC_3A-7A-8A_n78A-n258D DC_3A-7A-8A_n78A-n258E DC_3A-7A-8A_n78A-n258F DC_3A-7A-8A_n78A-n258G DC_3A-7A-8A_n78A-n258H DC_3A-7A-8A_n78A-n258I DC_3A-7A-8A_n78A-n258J DC_3A-7A-8A_n78A-n258K DC_3A-7A-8A_n78A-n258L DC_3A-7A-8A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_7A_n78A DC_7A_n258A DC_8A_n78A DC_8A_n258A |
| DC_3A-7A-28A_n78A-n257A DC_3A-7A-28A_n78A-n257G DC_3A-7A-28A_n78A-n257H DC_3A-7A-28A_n78A-n257I | DC_3A_n78A DC_7A_n78A DC_28A_n78A DC_3A_n257A DC_7A_n257A DC_28A_n257A |
| DC_3A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-3A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-18A-42A_n78A-n257A DC_3A-18A-42A_n78A-n257G DC_3A-18A-42A_n78A-n257H DC_3A-18A-42A_n78A-n257I DC_3A-18A-42C_n78A-n257A DC_3A-18A-42C_n78A-n257G DC_3A-18A-42C_n78A-n257H DC_3A-18A-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-41A-42A_n77A-n257A DC_3A-41A-42A_n77A-n257G DC_3A-41A-42A_n77A-n257H DC_3A-41A-42A_n77A-n257I DC_3A-41A-42C_n77A-n257A DC_3A-41A-42C_n77A-n257G DC_3A-41A-42C_n77A-n257H DC_3A-41A-42C_n77A-n257I DC_3A-41C-42A_n77A-n257A DC_3A-41C-42A_n77A-n257G DC_3A-41C-42A_n77A-n257H DC_3A-41C-42A_n77A-n257I DC_3A-41C-42C_n77A-n257A DC_3A-41C-42C_n77A-n257G DC_3A-41C-42C_n77A-n257H DC_3A-41C-42C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-28A-41A_n78A-n257A DC_3A-28A-41A_n78A-n257G DC_3A-28A-41A_n78A-n257H DC_3A-28A-41A_n78A-n257I DC_3A-28A-41C_n78A-n257A DC_3A-28A-41C_n78A-n257G DC_3A-28A-41C_n78A-n257H DC_3A-28A-41C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-28A-42A_n78A-n257A DC_3A-28A-42A_n78A-n257G DC_3A-28A-42A_n78A-n257H DC_3A-28A-42A_n78A-n257I DC_3A-28A-42C_n78A-n257A DC_3A-28A-42C_n78A-n257G DC_3A-28A-42C_n78A-n257H DC_3A-28A-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-41A-42A_n78A-n257A DC_3A-41A-42A_n78A-n257G DC_3A-41A-42A_n78A-n257H DC_3A-41A-42A_n78A-n257I DC_3A-41A-42C_n78A-n257A DC_3A-41A-42C_n78A-n257G DC_3A-41A-42C_n78A-n257H DC_3A-41A-42C_n78A-n257I DC_3A-41C-42A_n78A-n257A DC_3A-41C-42A_n78A-n257G DC_3A-41C-42A_n78A-n257H DC_3A-41C-42A_n78A-n257I DC_3A-41C-42C_n78A-n257A DC_3A-41C-42C_n78A-n257G DC_3A-41C-42C_n78A-n257H DC_3A-41C-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_7A-8A_n1A-n78A-n257A2 | DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_7A-7A-8A_n1A-n78A-n257A2 | DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_19A-21A-42A_n77A-n257A DC_19A-21A-42A_n77A-n257G DC_19A-21A-42A_n77A-n257H DC_19A-21A-42A_n77A-n257I DC_19A-21A-42C_n77A-n257A DC_19A-21A-42C_n77A-n257G DC_19A-21A-42C_n77A-n257H DC_19A-21A-42C_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-21A-42A_n78A-n257A DC_19A-21A-42A_n78A-n257G DC_19A-21A-42A_n78A-n257H DC_19A-21A-42A_n78A-n257I DC_19A-21A-42C_n78A-n257A DC_19A-21A-42C_n78A-n257G DC_19A-21A-42C_n78A-n257H DC_19A-21A-42C_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-21A-42A_n79A-n257A DC_19A-21A-42A_n79A-n257G DC_19A-21A-42A_n79A-n257H DC_19A-21A-42A_n79A-n257I DC_19A-21A-42C_n79A-n257A DC_19A-21A-42C_n79A-n257G DC_19A-21A-42C_n79A-n257H DC_19A-21A-42C_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_28A-41A-42A_n78A-n257A DC_28A-41A-42A_n78A-n257G DC_28A-41A-42A_n78A-n257H DC_28A-41A-42A_n78A-n257I DC_28A-41A-42C_n78A-n257A DC_28A-41A-42C_n78A-n257G DC_28A-41A-42C_n78A-n257H DC_28A-41A-42C_n78A-n257I DC_28A-41C-42A_n78A-n257A DC_28A-41C-42A_n78A-n257G DC_28A-41C-42A_n78A-n257H DC_28A-41C-42A_n78A-n257I DC_28A-41C-42C_n78A-n257A DC_28A-41C-42C_n78A-n257G DC_28A-41C-42C_n78A-n257H DC_28A-41C-42C_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. |  |

5.5B.6.5	Inter-band EN-DC configurations including FR1 and FR2 (six bands)

Table 5.5B.6.5-1: Inter-band EN-DC configurations including FR1 and FR2 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A-7A_n78A-n257A DC_1A-3A-5A-7A-7A_n78A-n257D DC_1A-3A-5A-7A-7A_n78A-n257E DC_1A-3A-5A-7A-7A_n78A-n257F DC_1A-3A-5A-7A-7A_n78A-n257G DC_1A-3A-5A-7A-7A_n78A-n257H DC_1A-3A-5A-7A-7A_n78A-n257I DC_1A-3A-5A-7A-7A_n78A-n257J DC_1A-3A-5A-7A-7A_n78A-n257K DC_1A-3A-5A-7A-7A_n78A-n257L DC_1A-3A-5A-7A-7A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-5A-7A-7A_n78C-n257A DC_1A-3A-5A-7A-7A_n78C-n257D DC_1A-3A-5A-7A-7A_n78C-n257E DC_1A-3A-5A-7A-7A_n78C-n257F DC_1A-3A-5A-7A-7A_n78C-n257G DC_1A-3A-5A-7A-7A_n78C-n257H DC_1A-3A-5A-7A-7A_n78C-n257I DC_1A-3A-5A-7A-7A_n78C-n257J DC_1A-3A-5A-7A-7A_n78C-n257K DC_1A-3A-5A-7A-7A_n78C-n257L DC_1A-3A-5A-7A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-5A-7A_n78A-n257A DC_1A-3A-5A-7A_n78A-n257D DC_1A-3A-5A-7A_n78A-n257E DC_1A-3A-5A-7A_n78A-n257F DC_1A-3A-5A-7A_n78A-n257G DC_1A-3A-5A-7A_n78A-n257H DC_1A-3A-5A-7A_n78A-n257I DC_1A-3A-5A-7A_n78A-n257J DC_1A-3A-5A-7A_n78A-n257K DC_1A-3A-5A-7A_n78A-n257L DC_1A-3A-5A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-5A-7A_n78C-n257A DC_1A-3A-5A-7A_n78C-n257D DC_1A-3A-5A-7A_n78C-n257E DC_1A-3A-5A-7A_n78C-n257F DC_1A-3A-5A-7A_n78C-n257G DC_1A-3A-5A-7A_n78C-n257H DC_1A-3A-5A-7A_n78C-n257I DC_1A-3A-5A-7A_n78C-n257J DC_1A-3A-5A-7A_n78C-n257K DC_1A-3A-5A-7A_n78C-n257L DC_1A-3A-5A-7A_n78C-n257M | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_5A_n78A-n257A DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_7A_n78A-n257A DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I |
| DC_1A-3A-7A-28A_n38A-n257A DC_1A-3A-7A-28A_n38A-n257G DC_1A-3A-7A-28A_n38A-n257H DC_1A-3A-7A-28A_n38A-n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-3A-7A-28A_n78A-n257A DC_1A-3A-7A-28A_n78A-n257G DC_1A-3A-7A-28A_n78A-n257H DC_1A-3A-7A-28A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-3A-18A-42A_n78A-n257A DC_1A-3A-18A-42A_n78A-n257G DC_1A-3A-18A-42A_n78A-n257H DC_1A-3A-18A-42A_n78A-n257I DC_1A-3A-18A-42C_n78A-n257A DC_1A-3A-18A-42C_n78A-n257G DC_1A-3A-18A-42C_n78A-n257H DC_1A-3A-18A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-28A-42A_n78A-n257A DC_1A-3A-28A-42A_n78A-n257G DC_1A-3A-28A-42A_n78A-n257H DC_1A-3A-28A-42A_n78A-n257I DC_1A-3A-28A-42C_n78A-n257A DC_1A-3A-28A-42C_n78A-n257G DC_1A-3A-28A-42C_n78A-n257H DC_1A-3A-28A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-41A-42A_n77A-n257A DC_1A-3A-41A-42A_n77A-n257G DC_1A-3A-41A-42A_n77A-n257H DC_1A-3A-41A-42A_n77A-n257I DC_1A-3A-41C-42A_n77A-n257A DC_1A-3A-41C-42A_n77A-n257G DC_1A-3A-41C-42A_n77A-n257H DC_1A-3A-41C-42A_n77A-n257I DC_1A-3A-41A-42C_n77A-n257A DC_1A-3A-41A-42C_n77A-n257G DC_1A-3A-41A-42C_n77A-n257H DC_1A-3A-41A-42C_n77A-n257I DC_1A-3A-41C-42C_n77A-n257A DC_1A-3A-41C-42C_n77A-n257G DC_1A-3A-41C-42C_n77A-n257H DC_1A-3A-41C-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-41A-42A_n78A-n257A DC_1A-3A-41A-42A_n78A-n257G DC_1A-3A-41A-42A_n78A-n257H DC_1A-3A-41A-42A_n78A-n257I DC_1A-3A-41A-42C_n78A-n257A DC_1A-3A-41A-42C_n78A-n257G DC_1A-3A-41A-42C_n78A-n257H DC_1A-3A-41A-42C_n78A-n257I DC_1A-3A-41C-42A_n78A-n257A DC_1A-3A-41C-42A_n78A-n257G DC_1A-3A-41C-42A_n78A-n257H DC_1A-3A-41C-42A_n78A-n257I DC_1A-3A-41C-42C_n78A-n257A DC_1A-3A-41C-42C_n78A-n257G DC_1A-3A-41C-42C_n78A-n257H DC_1A-3A-41C-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-7A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-3A-7A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-7A-7A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-3A-7A-7A-8A_n1A-n78A-n257A2 | DC_3A_n1A DC_3A_n78A DC_3A_n257A DC_7A_n1A DC_7A_n78A DC_7A_n257A DC_8A_n1A DC_8A_n78A DC_8A_n257A |
| DC_3A-28A-41A-42A_n78A-n257A DC_3A-28A-41A-42A_n78A-n257G DC_3A-28A-41A-42A_n78A-n257H DC_3A-28A-41A-42A_n78A-n257I DC_3A-28A-41A-42C_n78A-n257A DC_3A-28A-41A-42C_n78A-n257G DC_3A-28A-41A-42C_n78A-n257H DC_3A-28A-41A-42C_n78A-n257I DC_3A-28A-41C-42A_n78A-n257A DC_3A-28A-41C-42A_n78A-n257G DC_3A-28A-41C-42A_n78A-n257H DC_3A-28A-41C-42A_n78A-n257I DC_3A-28A-41C-42C_n78A-n257A DC_3A-28A-41C-42C_n78A-n257G DC_3A-28A-41C-42C_n78A-n257H DC_3A-28A-41C-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. |  |

<<Unchanged parts skipped>>

6.2B.4.2.3.2	ΔTIB,c for EN-DC three bands

Table 6.2B.4.2.3.2-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |  |
| DC_1-3_n3 DC_1_(n)3 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_1-3_n5 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_1-3_n7 | 0.6 |  | 0.6 |  | 0.6 |  |
| DC_1-3_n8 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_1_n3-n8 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_1-3_n28 | 0.3 |  | 0.3 |  | 0.6 |  |
| DC_1_n3-n28 | 0.3 |  | 0.3 |  | 0.6 |  |
| DC_1-3_n38 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_1-3_n40 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_1-3_n41 | 0.5 |  | 0.5 |  | 0.33 / 0.84 |  |
| DC_1_n3-n41 | 0.5 |  | 0.5 |  | 0.33 / 0.84 |  |
| DC_1-41_n3 | 0.5 |  | 0.33 / 0.84 |  | 0.5 |  |
| DC_1_n3-n75 | 0.5 |  | 0.5 |  | - |  |
| DC_1-3_n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_1_n3-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_1-3_n71 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_1-3_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_1-3_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_1_n3-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_1_n3-n79 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_1-5_n77 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_1-5_n78 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_1-5_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_1-7_n3 | 0.6 |  | 0.6 |  | 0.6 |  |
| DC_1-7_n5 | 0.5 |  | 0.6 |  | 0.3 |  |
| DC_1-7_n7 | 0.5 |  | 0.6 |  | 0.6 |  |
| DC_1-7_n8 | 0.5 |  | 0.6 |  | 0.6 |  |
| DC_1-7_n28 | 0.5 |  | 0.6 |  | 0.6 |  |
| DC_1-7_n38 | 0.5 |  | - |  | - |  |
| DC_1-7_n40 | 0.6 |  | 0.8 |  | 0.9 |  |
| DC_1-7_n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_1-7_n78 DC_1-7-7_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_1_n7-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_1-8_n3 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_1-8_n28 | 0.3 |  | 0.6 |  | 0.6 |  |
| DC_1_n8-n40 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_1-8_n77 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_1_n8-n77 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_1-8_n78 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_1_n8-n78 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_1-8_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_1-11_n3 | 0.3 |  | 0.8 |  | 0.9 |  |
| DC_1-11_n28 | 0.3 |  | 0.4 |  | 0.6 |  |
| DC_1-11_n41 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_1-11_n77 | 0.6 |  | 0.4 |  | 0.8 |  |
| DC_1-11_n78 | 0.3 |  | 0.4 |  | 0.8 |  |
| DC_1-11_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_1-18_n3 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_1-18_n28 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_1-18_n41 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_1-18_n77 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_1-18_n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_1-19_n77 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_1-19_n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_1-19_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_1-20_n3 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_1-20_n7 | 0.5 |  | 0.3 |  | 0.6 |  |
| DC_1-20_n8 | 0.3 |  | 0.4 |  | 0.4 |  |
| DC_1-20_n28 | 0.5 |  | 0.6 |  | 0.6 |  |
| DC_1-20_n38 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_1-20_n41 | 0.5 |  | 0.3 |  | 0.51 / 1.22 |  |
| DC_1-20_n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_1-21_n28 | 0.3 |  | 0.4 |  | 0.6 |  |
| DC_1-21_n77 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_1-21_n78 | 0.6 |  | 0.4 |  | 0.8 |  |
| DC_1-21_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_1_n26-n78 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_1-28_n3 | 0.3 |  | 0.6 |  | 0.3 |  |
| DC_1-28_n5 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_1-28_n7 | 0.5 |  | 0.6 |  | 0.6 |  |
| DC_1_n28-n75 | 0.3 |  | 0.7 |  | - |  |
| DC_1-28_n77 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_1-28_n78 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_1_n28-n78 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_1_n28-n79 | 0.3 |  | 0.6 |  | - |  |
| DC_1_n28-n40 | 0.6 |  | 0.3 |  | 0.5 |  |
| DC_1_n28-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_1-28_n40 | 0.6 |  | 0.3 |  | 0.5 |  |
| DC_1-32_n3 | 0.5 |  | - |  | 0.5 |  |
| DC_1-32_n8 | 0.5 |  | - |  | 0.3 |  |
| DC_1-32_n28 | 0.3 |  | - |  | 0.7 |  |
| DC_1-32_n78 | 0.5 |  | - |  | 0.8 |  |
| DC_1-38_n3 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_1-38_n7 | 0.5 |  | - |  | - |  |
| DC_1-38_n8 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_1-38_n28 | 0.5 |  | 0.5 |  | 0.6 |  |
| DC_1-(n)38 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_1-38_n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_1_n38-n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_1-40_n78 | 0.6 |  | 0.35 |  | 0.85 |  |
| DC_1_n40-n78 | 0.3 |  | 0.5 |  | 0.8 |  |
| DC_1-41_n3 | 0.5 |  | 0.33 / 0.84 |  | 0.5 |  |
| DC_1-41_n28 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_1-(n)41 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_1-41_n41 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_1-41_n77 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_1_n41-n77 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_1-41_n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_1_n41-n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_1-41_n79 | 0.5 |  | 0.5 |  | - |  |
| DC_1-42_n3 | 0.3 |  | 0.8 |  | 0.6 |  |
| DC_1-42_n28 | 0.3 |  | 0.8 |  | 0.8 |  |
| DC_1-42_n77 | 0.6 |  | 0.8 |  | 0.8 |  |
| DC_1-42_n78 | 0.3 |  | 0.8 |  | 0.8 |  |
| DC_1-42_n79 | 0.3 |  | 0.8 |  | - |  |
| DC_1_n77-n79 | 0.6 |  | 0.8 |  | - |  |
| DC_1_SUL_n77-n80 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_1_SUL_n77-n84 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_1_SUL_n78-n84 | 0.3 |  | 0.8 |  | 0.3 |  |
| DC_1_n78-n79 | 0.3 |  | 0.8 |  | 0.5 |  |
| DC_1_n75-n78 | 0.5 |  | - |  | 0.8 |  |
| DC_1_SUL_n78-n80 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_2_n2-n38 | 0.5 |  | 0.5 |  | 0.9 |  |
| DC_2_n2-n41 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2_n2-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2_n2-n71 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2_n2-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_2_n2-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_2-4_n28 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_2-4_n38 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2-4_n41 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2-5_n2 DC_2-5-5_n2 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-5_n5 DC_2-2-5_n5 DC_2-(n)5 DC_2-2-(n)5 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-5_n7 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-5_n12 | 0.3 |  | 0.8 |  | 0.4 |  |
| DC_2-5_n30 DC_2-2-5_n30 | 0.5 |  | 0.3 |  | 0.3 |  |
| DC_2-5_n48 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_2-5_n66 DC_2-5-5_n66 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-5_n71 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_2-5_n77 DC_2-2-5_n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_2-5_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_2-7_n5 DC_2-7-7_n5 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-7_n7 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2-7_n25 DC_2-7-7_n25 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2-7_n28 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_2_n5-n77 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_2-7_n38 DC_2-2-7_n38 | 0.5 |  | - |  | - |  |
| DC_2-7_n71 | 0.5 |  | 0.5 |  | 0.6 |  |
| DC_2-7_n66 DC_2-7-7_n66 DC_2_n7-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2-7_n77 DC_2-7-7_n77 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_2-7_n78 DC_2-2-7_n78 | 0.5 |  | 0.5 |  | - |  |
| DC_2_n7-n78 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_2-8_n2 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-12_n2 | 0.3 |  | 0.3 |  | - |  |
| DC_2-12_n5 DC_2-2-12_n5 | 0.3 |  | 0.4 |  | 0.8 |  |
| DC_2-12_n7 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2_(n)12 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-12_n30 DC_2-2-12_n30 | 0.5 |  | 0.3 |  | 0.3 |  |
| DC_2-12_n41 DC_2-2-12_n41 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-12_n66, DC_2-2-12_n66 | 0.5 |  | 0.8 |  | 0.5 |  |
| DC_2-12_n77 DC_2-2-12_n77 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_2-12_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_2_n38-n66 | 0.5 |  | 0.9 |  | 0.5 |  |
| DC_2-13_n2 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-13_n5 DC_2-2-13_n5 | 0.3 |  | 0.5 |  | 0.3 |  |
| DC_2-13_n25 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-13_n48 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_2-13_n66 DC_2-2-13_n66 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-13_n77 DC_2-2-13_n77 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_2-14_n2 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-14_n5 DC_2-2-14_n5 | 0.3 |  | 0.4 |  | 0.8 |  |
| DC_2-14_n30 DC_2-2-14_n30 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-14_n66 DC_2-2-14_n66 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-14_n77 DC_2-2-14_n77 | 0.5 |  | 0.3 |  | 0.8 |  |
| DC_2_n25-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2-28_n7 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-28_n66 | 0.5 |  | 0.6 |  | 0.5 |  |
| DC_2-28_n78 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_2-29_n30 DC_2-2-29_n30 | 0.5 |  | - |  | 0.3 |  |
| DC_2-29_n66 DC_2-2-29_n66 | 0.5 |  | - |  | 0.5 |  |
| DC_2-29_n77 DC_2-2-29_n77 | 0.6 |  | - |  | 0.8 |  |
| DC_2-29-n78 | 0.6 |  | - |  | 0.8 |  |
| DC_2-30_n2 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-30_n5, DC_2-2-30_n5 | 0.5 |  | 0.3 |  | 0.3 |  |
| DC_2-30_n66, DC_2-2-30_n66 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-30_n77 DC_2-2-30_n77 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_2_n38-n71 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_2-38_n78 | 0.6 |  | 0.9 |  | 0.8 |  |
| DC_2_n38-n78 | 0.6 |  | 0.9 |  | 0.8 |  |
| DC_2_n41-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2_n41-n71 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_2_n41-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2_n41-n71 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_2-46_n5 DC_2-2-46_n5 | 0.3 |  | - |  | 0.3 |  |
| DC_2-46_n41 | 0.5 |  | - |  | 0.41 / 0.92 |  |
| DC_2-46_n66 | 0.5 |  | - |  | 0.5 |  |
| DC_2-46_n77 DC_2-46-46_n77 | 0.6 |  | - |  | 0.8 |  |
| DC_2-48_n2 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_2-48_n5 | 0.6 |  | 0.8 |  | 0.3 |  |
| DC_2-48_n12 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_2-48_n48 | 0.6 |  | 0.8 |  | 0.8 |  |
| DC_2-48_n66 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_2-48_n71 | 0.6 |  | 0.8 |  | 0.3 |  |
| DC_2-48_n77 DC_2-48-48_n77 DC_2-48-48-48_n77 | 0.3 |  | 0.6 |  | 0.5 |  |
| DC_2-66_n2 DC_2-66-66_n2 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2-66_n5, DC_2-2-66_n5, DC_2-66-66_n5, DC_2-2-66-66_n5, DC_2-66-66-66_n5 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_2-66-n7 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2-66_n12 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_2-66_n25 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2-66-n28 | 0.5 |  | 0.5 |  | 0.6 |  |
| DC_2-66_n30 DC_2-2-66_n30 DC_2-66-66_n30 DC_2-2-66-66_n30 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_2-66_n38 DC_2-2-66_n38 DC_2-66-66_n38 | 0.5 |  | 0.5 |  | 0.9 |  |
| DC_2-66_n41 | 0.5 |  | 0.5 |  | 0.81 / 1.32 |  |
| DC_2-66_n48 DC_2-66-66_n48 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_2-66_n66 DC_2-2-66-66_n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2_(n)66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2-66_n71 DC_2_n66-n71 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_2-66_n77 DC_2-2-66_n77 DC_2-66-66_n77 DC_2-2-66-66_n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_2_n66-n77 DC_2-2_n66-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_2-66_n78 DC_2-66-66_n78 DC_2_n66-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_2-71_n38 DC_2-2-71_n38 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-71_n41 DC_2-2-71_n41 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-71_n66 DC_2-2-71_n66 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-71_n71 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-(n)71 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-71_n78 DC_2-2-71_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_2_n71-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3_n1-n7 | 0.6 |  | 0.6 |  | 0.6 |  |
| DC_3_n1-n8 DC_3-3_n1-n8 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_3_n1-n28 | 0.3 |  | 0.3 |  | 0.6 |  |
| DC_3_n1-n38 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_3_n1-n40 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_3_n1-n41 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_3_n1-n75 | 0.5 |  | 0.5 |  | - |  |
| DC_3_n1-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3_n1-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_(n)3-n8 | 0.3 |  | - |  | 0.3 |  |
| DC_(n)3-n77 | - |  | 0.6 |  | 0.8 |  |
| DC_3_n1-n79 | 0.3 |  | 0.3 |  | - |  |
| DC_3_n3-n41 | 0.5 |  | 0.5 |  | 0.33 / 0.84 |  |
| DC_3_n3-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3_n3-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3_n5-n40 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_3-5_n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3-5_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3-5_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_3-7_n1 DC_3-3-7_n1 DC_3-7-7_n1 DC_3-3-7-7_n1 | 0.3 |  | 0.6 |  | 0.5 |  |
| DC_3-7_n3 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_3-7_n5 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_3-7_n7 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_3-7_n8 DC_3-3-7_n8 DC_3-7-7_n8 DC_3-3-7-7_n8 | 0.5 |  | 0.5 |  | 0.6 |  |
| DC_3-7_n28 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_3_n7-n28 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_3-7_n38 | 0.5 |  | - |  | - |  |
| DC_3-7_n40 | 0.6 |  | 0.8 |  | 0.9 |  |
| DC_3-7_n77 DC_3-3-7_n77 DC_3-7-7_n77 DC_3-3-7-7_n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3-7_n78 DC_3-7-7_n78 DC_3-3-7_n78 DC_3-3-7-7_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3_n7-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3-8_n1 DC_3-3-8_n1 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_3-8_n40 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_3_n8-n40 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_3-8_n28 | 0.3 |  | 0.6 |  | 0.5 |  |
| DC_3-8_n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3-8_n78 DC_3-3-8_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3_n8-n78 DC_3-3_n8-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3-8_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_3-11_n28 | 0.8 |  | 0.9 |  | 0.6 |  |
| DC_3-11_n77 | 0.8 |  | 0.9 |  | 0.8 |  |
| DC_3-18_n3 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_3-18_n28 | 0.3 |  | 0.5 |  | 0.3 |  |
| DC_3-18_n41 | 0.6 |  | 0.3 |  | 0.33 / 0.84 |  |
| DC_3-18_n77 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_3-18_n78 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_3-18_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_3-19_n1 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_3-19_n77 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_3-19_n78 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_3-19_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_3-20_n1 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_3-20_n7 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_3-20_n8 | 0.3 |  | 0.4 |  | 0.4 |  |
| DC_3-20_n28 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_3-20_n38 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_3-20_n41 | 0.5 |  | 0.3 |  | 0.53 / 1.24 |  |
| DC_3_n20-n67 | 0.3 |  | 0.5 |  | 0.3 |  |
| DC_3-20_n78 | 0.5 |  | 0.3 |  | 0.8 |  |
| DC_3_n20-n78 | 0.5 |  | 0.3 |  | 0.8 |  |
| DC_3-21_n1 | 0.8 |  | 0.9 |  | 0.3 |  |
| DC_3-21_n28 | 0.8 |  | 0.9 |  | 0.3 |  |
| DC_3-21_n77 | 0.8 |  | 0.9 |  | 0.8 |  |
| DC_3-21_n78 | 0.8 |  | 0.9 |  | 0.8 |  |
| DC_3-21_n79 | 0.8 |  | 0.9 |  | - |  |
| DC_3_n26-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_3-28_n1 | 0.3 |  | 0.6 |  | 0.3 |  |
| DC_3-28_n3 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_3-28_n5 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_3-28_n7 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_3_n28-n40 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_3-28_n40 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_3-28_n41 | 0.5 |  | 0.5 |  | 0.33 / 0.84 |  |
| DC_3_n28-n75 | 0.3 |  | 0.3 |  | - |  |
| DC_3-28_n77 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_3_n28-n77 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_3-28_n78 | 0.5 |  | 0.3 |  | 0.8 |  |
| DC_3_n28-n78 | 0.5 |  | 0.3 |  | 0.8 |  |
| DC_3_n28-n79 | 0.3 |  | 0.3 |  | - |  |
| DC_3-32_n1 | 0.5 |  | - |  | 0.5 |  |
| DC_3-32_n28 | 0.3 |  | - |  | 0.3 |  |
| DC_3-32_n78 | 0.6 |  | - |  | 0.8 |  |
| DC_3-38_n7 | 0.5 |  | - |  | - |  |
| DC_3-38_n28 | 0.5 |  | 0.5 |  | 0.6 |  |
| DC_3_n38-n40 | 0.5 |  | 0.53 |  | 0.5 |  |
| DC_3-38_n78 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_3_n38-n78 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_3-40_n1 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_3_n40-n41 | 0.5 |  | 0.5 |  | 0.53 / 0.84 |  |
| DC_3-40_n78 | 0.6 |  | 0.35 |  | 0.85 |  |
| DC_3_n40-n78 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_3_n40-n79 | 0.5 |  | 0.5 |  | - |  |
| DC_3-41_n3 | 0.5 |  | 0.33 / 0.84 |  | 0.5 |  |
| DC_3-41_n28 | 0.5 |  | 0.33 / 0.84 |  | 0.3 |  |
| DC_3-(n)41 | 0.5 |  | 0.33 / 0.84 |  | 0.33 / 0.84 |  |
| DC_3-41_n41 | 0.5 |  | 0.33 / 0.84 |  | 0.33 / 0.84 |  |
| DC_3-41_n77 DC_3_n41-n77 | 0.6 |  | 0.33 / 0.84 |  | 0.8 |  |
| DC_3-41_n78 | 0.6 |  | 0.33 / 0.84 |  | 0.8 |  |
| DC_3_n41-n78 | 0.6 |  | 0.33 / 0.84 |  | 0.8 |  |
| DC_3-41_n79 | 0.6 |  | 0.33 / 0.84 |  | - |  |
| DC_3_n41-n79 | 0.6 |  | 0.33 / 0.84 |  | - |  |
| DC_3_SUL_n41-n80 | 0.5 |  | 0.33 / 0.84 |  | 0.5 |  |
| DC_3-42_n1 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_3-42_n28 | 0.6 |  | 0.8 |  | 0.8 |  |
| DC_3-42_n77 | 0.6 |  | 0.8 |  | 0.8 |  |
| DC_3-42_n78 | 0.6 |  | 0.8 |  | 0.8 |  |
| DC_3-42_n79 | 0.6 |  | 0.8 |  | - |  |
| DC_3_n75-n78 | 0.6 |  | - |  | 0.8 |  |
| DC_3_n77-n79 | 0.6 |  | 0.8 |  | - |  |
| DC_3_SUL_n77-n80 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_3_SUL_n77-n84 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_3_n78-n79 | 0.6 |  | 0.8 |  | 0.5 |  |
| DC_3_SUL_n78-n80 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_3_SUL_n78-n82 | 0.5 |  | 0.8 |  | 0.3 |  |
| DC_3_SUL_n78-n84 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_4-7_n28 | 0.5 |  | 0.5 |  | 0.6 |  |
| DC_5_n1-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5_n2-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5_n2-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5_n3-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5_n5-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5-7_n7 | 0.5 |  | 0.3 |  | 0.3 |  |
| DC_5-7_n66 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_5-7_n71 | 0.5 |  | 0.3 |  | 0.6 |  |
| DC_5-7_n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5-7_n78 DC_5-7-7_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5_n7-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5_(n)12 | 0.8 |  | 0.4 |  | 0.4 |  |
| DC_5-13_n2 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_5-13_n66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_5-13_n77 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_5-30_n2 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_5-30_n66 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_5-30_n77 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_5_n38-n66 | 0.5 |  | 0.8 |  | 0.5 |  |
| DC_5-41_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_5-46_n66 | 0.3 |  | - |  | 0.3 |  |
| DC_5-48_n12 | 0.8 |  | 0.3 |  | 0.4 |  |
| DC_5-48_n71 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_5-48_n77 | 0.6 |  | 0.8 |  | 0.8 |  |
| DC_5-66_n2 DC_5-5-66_n2 DC_5-66-66_n2 DC_5-5-66-66_n2 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_5-66_n5 DC_5-66-66_n5 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_5-66-n7 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_5-66_n12 | 0.3 |  | 0.8 |  | 0.8 |  |
| DC_5-66_n30 DC_5-66-66_n30 | 0.3 |  | 0.5 |  | 0.3 |  |
| DC_5-66_n48 DC_5-66-66_n48 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_5-66_n66 DC_5-5-66_n66 DC_5-66-66_n66 DC_5-5-66-66_n66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_5-66_n71 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_5-66_n77 DC_5-66-66_n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5_n66-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5-66_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5_n66-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_5-66_n66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_7_n1-n8 DC_7-7_n1-n8 | 0.6 |  | 0.5 |  | 0.5 |  |
| DC_7_n1-n40 | 0.8 |  | 0.6 |  | 0.9 |  |
| DC_7_n1-n75 | 0.6 |  | 0.5 |  | - |  |
| DC_7_n1-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_7_n2-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7_n2-n71 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_7_n2-n78 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_7_n3-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_7_n7-n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_7-8_n1 DC_7-7-8_n1 | 0.6 |  | 0.6 |  | 0.5 |  |
| DC_7-8_n3 | 0.5 |  | 0.6 |  | 0.5 |  |
| DC_7-8_n28 | 0.3 |  | 0.6 |  | 0.5 |  |
| DC_7-8_n40 | 0.5 |  | 0.6 |  | 0.6 |  |
| DC_7_n8-n40 | 0.5 |  | 0.6 |  | 0.6 |  |
| DC_7-8_n77 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_7-8_n78 DC_7-7-8_n78 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_7_n8-n78 DC_7-7_n8-n78 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_7-12_n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7-12_n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_7-13_n25 DC_7-7-13_n25 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_7-13_n66 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_7-20_n1 | 0.6 |  | 0.3 |  | 0.5 |  |
| DC_7-20_n3 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_7-20_n8 | 0.3 |  | 0.4 |  | 0.4 |  |
| DC_7-20_n28 | 0.3 |  | 0.6 |  | 0.6 |  |
| DC_7-20_n38 | - |  | 0.3 |  | - |  |
| DC_7-20_n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_7-25_n77 DC_7-7-25_n77 DC_7-25-25_n77 DC_7-7-25-25_n77 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_7-25_n78 DC_7-7-25_n78 DC_7-25-25_n78 DC_7-7-25-25_n78 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_7_n25-n66 DC_7-7_n25-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7_n26-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_7-28_n1 DC_7-7-28_n1 | 0.6 |  | 0.6 |  | 0.5 |  |
| DC_7-28_n2 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_7-28_n3 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_7-28_n5 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_7-28_n7 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_7_n28-n40 | 0.5 |  | 0.3 |  | 0.6 |  |
| DC_7-28_n40 | 0.5 |  | 0.3 |  | 0.6 |  |
| DC_7-28_n66 | 0.5 |  | 0.6 |  | 0.5 |  |
| DC_7-28_n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_7_n28-n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_7-29_n78 | 0.5 |  | - |  | 0.8 |  |
| DC_7-32_n1 | 0.6 |  | - |  | 0.5 |  |
| DC_7-32_n3 | 0.7 |  | - |  | 0.7 |  |
| DC_7-32_n8 | 0.7 |  | - |  | 0.6 |  |
| DC_7-32_n28 | 0.3 |  | - |  | 0.7 |  |
| DC_7-32_n78 | 0.5 |  | - |  | 0.8 |  |
| DC_7-38_n3 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7_n38-n78 | - |  | - |  | 0.8 |  |
| DC_7_n78-n79 | 0.5 |  | 0.8 |  | 0.8 |  |
| DC_7-40_n1 | 0.8 |  | 0.9 |  | 0.6 |  |
| DC_7-40-n78 | 0.5 |  | 0.35 |  | 0.85 |  |
| DC_7-46_n78 | 0.5 |  | - |  | 0.8 |  |
| DC_7-66_n5 DC_7-66-66_n5 DC_7-7-66_n5 DC_7-7-66-66_n5 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_7-66_n7 DC_7-66-66_n7 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7-66_n25 DC_7-7-66_n25 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7-66_n28 | 0.5 |  | 0.5 |  | 0.6 |  |
| DC_7-66_n38 | - |  | 0.5 |  | - |  |
| DC_7-66_n66 DC_7-7-66_n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7-66_n71 DC_7-66-66_n71 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7_n66-n71 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7-66_n77 DC_7-7-66_n77 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_7_n66-n77 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_7-66_n78 DC_7-7-66_n78 DC_7-66-66_n78 DC_7-7-66-66_n78 | 0.5 |  | 0.5 |  | - |  |
| DC_7_n66-n78 DC_7-7_n66-n78 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_7-71_n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7-71_n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_7_n71-n78 | 0.3 |  | 0.5 |  | 0.8 |  |
| DC_7_SUL_n78-n80 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_8_n1-n3 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_8_n1-n28 | 0.6 |  | 0.3 |  | 0.6 |  |
| DC_8_n1-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_8_n1-n40 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_8_n1-n78 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_8_(n)3 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_8_n3-n28 | 0.6 |  | 0.3 |  | 0.5 |  |
| DC_8_n3-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_8_n3-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_8_n3-n79 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_8-11_n3 | 0.3 |  | 0.8 |  | 0.9 |  |
| DC_8-11_n28 | 0.6 |  | 0.4 |  | 0.6 |  |
| DC_8-11_n77 | 0.6 |  | 0.4 |  | 0.8 |  |
| DC_8-11_n78 | 0.6 |  | 0.4 |  | 0.8 |  |
| DC_8-20_n1 | 0.4 |  | 0.4 |  | 0.3 |  |
| DC_8-20_n3 | 0.4 |  | 0.4 |  | 0.3 |  |
| DC_8-20_n28 | 0.6 |  | 0.5 |  | 0.5 |  |
| DC_8-20_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_8_n28-n77 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_8_n28-n78 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_8-32_n1 | 0.3 |  | - |  | 0.5 |  |
| DC_8-32_n3 | 0.3 |  | - |  | 0.8 |  |
| DC_8-32_n28 | 0.5 |  | - |  | 0.5 |  |
| DC_8-38_n1 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_8_n38-n40 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_8_n39-n40 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_8_n39-n79 | 0.3 |  | 0.3 |  | - |  |
| DC_8-40_n1 | 0.3 |  | 0.5 |  | 0.3 |  |
| DC_8-40-n78 | 0.6 |  | 0.35 |  | 0.85 |  |
| DC_8_n40-n41 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_8_n40-n79 | 0.3 |  | 0.3 |  | - |  |
| DC_8-41_n1 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_8-41_n3 | 0.3 |  | 0.33 / 0.84 |  | 0.5 |  |
| DC_8-41_n77 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_8_n41-n79 | 0.3 |  | 0.3 |  | - |  |
| DC_8_SUL_n41-n81 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_8-42_n1 | 0.6 |  | 0.8 |  | 0.3 |  |
| DC_8-42_n3 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_8-42_n28 | 0.6 |  | 0.8 |  | 0.8 |  |
| DC_8-42_n77 | 0.6 |  | 0.8 |  | 0.8 |  |
| DC_8_n77-n79 | 0.6 |  | 0.8 |  | 0.5 |  |
| DC_8_SUL_n78-n80 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_8_SUL_n78- n81 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_11_n1-n77 | 0.4 |  | 0.6 |  | 0.8 |  |
| DC_11_n3-n28 | 0.8 |  | 0.9 |  | 0.6 |  |
| DC_11_n3-n77 | 0.8 |  | 0.9 |  | 0.8 |  |
| DC_11_n3-n79 | 0.8 |  | 0.9 |  | 0.8 |  |
| DC_11-18_n77 | 0.4 |  | 0.3 |  | 0.8 |  |
| DC_11-18_n78 | 0.4 |  | 0.3 |  | 0.8 |  |
| DC_11_n28-n77 | 0.4 |  | 0.6 |  | 0.8 |  |
| DC_11_n77-n79 | - |  | 0.5 |  | - |  |
| DC_12_n2-n38 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_12_n2-n41 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_12_n2-n78 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_12_(n)5 | 0.8 |  | 0.4 |  | 0.8 |  |
| DC_12_n7-n66 | 0.8 |  | 0.5 |  | 0.5 |  |
| DC_12_n7-n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_12-30_n2 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_12-30_n5 | 0.8 |  | 0.8 |  | 0.3 |  |
| DC_12-30_n66 | 0.8 |  | 0.3 |  | 0.5 |  |
| DC_12-30_n77 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_12-48_n5 | 0.4 |  | 0.3 |  | 0.8 |  |
| DC_12-66_n2 | 0.8 |  | 0.5 |  | 0.5 |  |
| DC_12-66_n5 DC_12-66-66_n5 | 0.8 |  | 0.8 |  | 0.3 |  |
| DC_12-66_n25 | 0.8 |  | 0.5 |  | 0.5 |  |
| DC_12-66_n30 DC_12-66-66_n30 | 0.8 |  | 0.5 |  | 0.3 |  |
| DC_12-66_n41 | 0.6 |  | 0.5 |  | 0.81 / 1.32 |  |
| DC_12-66_n77 DC_12-66-66_n77 | 0.8 |  | 0.6 |  | 0.8 |  |
| DC_12-66_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_12-66_n66 | 0.8 |  | 0.3 |  | 0.3 |  |
| DC_12_n66-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_13_n2-n77 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_13_n5-n48 | 0.4 |  | 0.8 |  | 0.3 |  |
| DC_13_n5-n77 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_13_n7-n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_13_n25-n66 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_13-46_n2 | 0.3 |  | - |  | 0.3 |  |
| DC_13-46_n5 | 0.5 |  | - |  | 0.5 |  |
| DC_13-46_n66 | 0.3 |  | - |  | 0.3 |  |
| DC_13-46_n77 DC_13-46-46_n7 | 0.5 |  | - |  | 0.8 |  |
| DC_13-48_n2 | 0.3 |  | 0.8 |  | 0.6 |  |
| DC_13-48_n66 | 0.3 |  | 0.8 |  | 0.6 |  |
| DC_13_n48-n66 | 0.3 |  | 0.8 |  | 0.6 |  |
| DC_13-48_n77 | 0.5 |  | 0.8 |  | 0.8 |  |
| DC_13-66_n2 DC_13-66-66_n2 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_13-66_n5 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_13-66_n48 DC_13-66-66_n48 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_13-66_n66 DC_13-66-66_n66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_13-66_n77 DC_13-66-66_n77 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_13_n66-n77 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_14-30_n2 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_14-30_n5 | 0.8 |  | 0.8 |  | 0.3 |  |
| DC_14-30_n66 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_14-30_n77 | 0.5 |  | 0.3 |  | 0.8 |  |
| DC_14-66_n2 DC_14-66-66_n2 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_14-66_n5 DC_14-66-66_n5 | 0.8 |  | 0.8 |  | 0.3 |  |
| DC_14-66_n30 DC_14-66-66_n30 | 0.3 |  | 0.5 |  | 0.3 |  |
| DC_14-66_n66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_14-66_n77 DC_14-66-66_n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_18_n3-n41 | 0.3 |  | 0.5 |  | 0.3 |  |
| DC_18_n3-n77 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_18_n3-n78 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_18_n28-n41 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_18-28_n77 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_18_n28-n77 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_18-28_n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_18_n28-n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_18-28_n79 | 0.5 |  | 0.5 |  | - |  |
| DC_18-41_n3 | 0.3 |  | 0.33 / 0.84 |  | 0.5 |  |
| DC_18-41_n77 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_18_n41-n77 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_18-41_n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_18_n41-n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_18-42_n77 | 0.3 |  | 0.8 |  | 0.8 |  |
| DC_18-42_n78 | 0.3 |  | 0.8 |  | 0.8 |  |
| DC_18-42_n79 | 0.3 |  | 0.8 |  | - |  |
| DC_19_n1-n77 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_19_n1-n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_19_n1-n79 | 0.3 |  | 0.3 |  | - |  |
| DC_19-21_n1 | 0.3 |  | 0.4 |  | 0.3 |  |
| DC_19-21_n77 | 0.3 |  | 0.4 |  | 0.8 |  |
| DC_19-21_n78 | 0.3 |  | 0.4 |  | 0.8 |  |
| DC_19-21_n79 | 0.3 |  | 0.4 |  | - |  |
| DC_19-42_n1 | 0.3 |  | 0.8 |  | 0.3 |  |
| DC_19-42_n77 | 0.3 |  | 0.8 |  | 0.8 |  |
| DC_19-42_n78 | 0.3 |  | 0.8 |  | 0.8 |  |
| DC_19-42_n79 | 0.3 |  | 0.8 |  | - |  |
| DC_19_n77-n79 | 0.3 |  | 0.8 |  | - |  |
| DC_19_n78-n79 | 0.3 |  | 0.8 |  | 0.5 |  |
| DC_20_n1-n7 | 0.3 |  | 0.5 |  | 0.6 |  |
| DC_20_n1-n28 | 0.3 |  | 0.6 |  | 0.6 |  |
| DC_20_n1-n67 | 0.6 |  | 0.5 |  | 0.6 |  |
| DC_20_n1-n75 | 0.3 |  | 0.5 |  | - |  |
| DC_20_n1-n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_20_n3-n67 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_20_n3-n78 | 0.3 |  | 0.5 |  | 0.8 |  |
| DC_20_n7-n28 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_20_n8-n75 | 0.4 |  | 0.4 |  | - |  |
| DC_20_n7-n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_20_n8-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_20-28_n1 | 0.6 |  | 0.6 |  | 0.5 |  |
| DC_20-28_n3 | 0.5 |  | 0.6 |  | 0.5 |  |
| DC_20_n28-n75 | 0.5 |  | 0.7 |  | - |  |
| DC_20_n28-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_20-32_n1 | 0.3 |  | - |  | 0.5 |  |
| DC_20-32_n3 | 0.3 |  | - |  | 0.5 |  |
| DC_20-32_n8 | 0.4 |  | - |  | 0.4 |  |
| DC_20-32_n28 | 0.5 |  | - |  | 0.7 |  |
| DC_20-32_n78 | 0.5 |  | - |  | 0.8 |  |
| DC_20-38_n1 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_20-38_n3 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_20-(n)38 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_20-38_n78 | 0.6 |  | - |  | 0.8 |  |
| DC_20_n38-n78 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_20-40-n1 | 0.3 |  | 0.5 |  | 0.3 |  |
| DC_20-40_n78 | 0.6 |  | 0.35 |  | 0.85 |  |
| DC_20_n41-n78 | 0.5 |  | 0.3 |  | 0.8 |  |
| DC_20_n75-n78 | 0.5 |  | - |  | 0.8 |  |
| DC_20_n76-n78 | 0.5 |  | - |  | 0.8 |  |
| DC_20_SUL_n78-n80 | 0.3 |  | 0.8 |  | 0.5 |  |
| DC_20_SUL_n78-n82 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_20_SUL_n78-n83 | 0.8 |  | 0.8 |  | 0.8 |  |
| DC_20_n78-n92 | 0.6 |  | - |  | 0.8 |  |
| DC_21_n1-n77 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_21_n1-n78 | 0.4 |  | 0.6 |  | 0.8 |  |
| DC_21_n1-n79 | 0.3 |  | 0.3 |  | - |  |
| DC_21_n28-n77 | 0.4 |  | 0.5 |  | 0.8 |  |
| DC_21_n28-n78 | 0.4 |  | 0.5 |  | 0.8 |  |
| DC_21_n28-n79 | 0.4 |  | - |  | 0.3 |  |
| DC_21-42_n1 | 0.4 |  | 0.8 |  | 0.3 |  |
| DC_21-42_n77 | 0.4 |  | 0.8 |  | 0.8 |  |
| DC_21-42_n78 | 0.4 |  | 0.8 |  | 0.8 |  |
| DC_21-42_n79 | 0.4 |  | 0.8 |  | - |  |
| DC_21_n77-n79 | 0.4 |  | 0.8 |  | - |  |
| DC_21_n78-n79 | 0.4 |  | 0.8 |  | 0.5 |  |
| DC_25-41_n41 DC_25_(n)41 DC_25-25-41_n41 DC_25-25_(n)41 | 0.5 |  | 0.41 / 0.92 |  | 0.41 / 0.92 |  |
| DC_25-66_n77 DC_25-25-66_n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_25-66_n78 DC_25-25-66_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_28_n1-n40 | 0.6 |  | 0.3 |  | 0.5 |  |
| DC_28_n1-n78 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_28_n3-n77 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_28_n3-n78 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_28_n7-n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_28_n8-n78 | 0.5 |  | 0.6 |  | 0.3 |  |
| DC_28_n40-n78 | 0.5 |  | 0.35 |  | 0.85 |  |
| DC_28-32_n1 | 0.6 |  | - |  | 0.5 |  |
| DC_28-32_n3 | 0.3 |  | - |  | 0.3 |  |
| DC_28-38_n1 | 0.6 |  | 0.5 |  | 0.5 |  |
| DC_28-41_n77 | 0.5 |  | 0.3 |  | 0.8 |  |
| DC_28-41_n78 | 0.5 |  | 0.3 |  | 0.8 |  |
| DC_28-41_n79 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_28_SUL_n41-n83 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_28-42_n77 | 0.5 |  | 0.8 |  | 0.8 |  |
| DC_28-42_n78 | 0.5 |  | 0.8 |  | 0.8 |  |
| DC_28-42_n79 | 0.5 |  | 0.8 |  | - |  |
| DC_28-66_n7 | 0.6 |  | 0.5 |  | 0.5 |  |
| DC_28-66_n66 | 0.6 |  | 0.3 |  | 0.3 |  |
| DC_28_SUL_n78-n83 | 0.5 |  | 0.8 |  | 0.5 |  |
| DC_29-30_n2 | - |  | 0.3 |  | 0.5 |  |
| DC_29-30_n66 | - |  | 0.3 |  | 0.5 |  |
| DC_29-30_n77 | - |  | 0.3 |  | 0.5 |  |
| DC_29-66_n2 DC_29-66-66_n2 | - |  | 0.5 |  | 0.5 |  |
| DC_29-66_n30 DC_29-66-66_n30 | - |  | 0.5 |  | 0.3 |  |
| DC_29-66_n77 | - |  | 0.6 |  | 0.8 |  |
| DC_29-66_n78 | - |  | 0.6 |  | 0.8 |  |
| DC_30-(n)5 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_30-66_n2 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_30-66_n5, DC_30-66-66_n5, DC_30-66-66-66_n5 | 0.3 |  | 0.5 |  | 0.3 |  |
| DC_30-66_n66 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_30-66_n77 DC_30-66-66_n77 | 0.3 |  | 0.6 |  | 0.8 |  |
| DC_32-38_n1 | - |  | 0.5 |  | 0.5 |  |
| DC_32-38_n28 | - |  | 0.7 |  | 0.6 |  |
| DC_38_n3-n78 | 0.5 |  | 0.6 |  | 0.8 |  |
| DC_38_n28-n78 | 0.3 |  | 0.5 |  | 0.8 |  |
| DC_39_n40-n41 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_39_n40-n79 | 0.3 |  | - |  | 0.8 |  |
| DC_39_n41-n79 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_40_n1-n78 | 0.5 |  | 0.3 |  | 0.8 |  |
| DC_40-42_n77 | 0.45 |  | 0.55 |  | 0.55 |  |
| DC_40-42_n78 | 0.45 |  | 0.55 |  | 0.55 |  |
| DC_41_n1-n3 | 0.53 |  | 0.5 |  | 0.84 |  |
| DC_41_n1-n77 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_41_n1-n78 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_41_n3-n41 | 0.33 / 084 |  | 0.5 |  | 0.33 / 084 |  |
| DC_41_n3-n77 | 0.33 / 084 |  | 0.6 |  | 0.8 |  |
| DC_41_n3-n78 | 0.33 / 084 |  | 0.6 |  | 0.8 |  |
| DC_41_n28-n41 | 0.33 / 084 |  | 0.3 |  | 0.33 / 084 |  |
| DC_41_n28-n77 | 0.3 |  | 0.5 |  | 0.8 |  |
| DC_41_n28-n78 | 0.3 |  | 0.5 |  | 0.8 |  |
| DC_41_n41-n77 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_41_n41-n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_(n)41-n78 | 0.3 |  | 0.3 |  | 0.8 |  |
| DC_41-42_n77 | 0.5 |  | 0.8 |  | 0.8 |  |
| DC_41-42_n78 | 0.5 |  | 0.8 |  | 0.8 |  |
| DC_41-42_n79 | 0.3 |  | 0.8 |  | - |  |
| DC_42_n1-n3 | 0.8 |  | 0.3 |  | 0.6 |  |
| DC_42_n1-n77 | 0.8 |  | 0.6 |  | 0.8 |  |
| DC_42_n1-n78 | 0.8 |  | 0.3 |  | 0.8 |  |
| DC_42_n1-n79 | 0.8 |  | 0.3 |  | - |  |
| DC_42_n3-n28 | 0.8 |  | 0.6 |  | 0.8 |  |
| DC_42_n3-n77 | 0.8 |  | 0.6 |  | 0.8 |  |
| DC_42_n28-n77 | 0.5 |  | 0.8 |  | 0.8 |  |
| DC_46-48_n5 | - |  | 0.8 |  | 0.3 |  |
| DC_46-48_n66 | - |  | 0.8 |  | 0.6 |  |
| DC_46-66_n5 DC_46-66-66_n5 | - |  | 0.3 |  | 0.3 |  |
| DC_46-66_n25 | - |  | 0.5 |  | 0.5 |  |
| DC_46-66_n77 DC_46-46-66_n77 | - |  | 0.6 |  | 0.8 |  |
| DC_48_(n)5 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_48_(n)12 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_48_n25-n48 | 0.8 |  | 0.6 |  | 0.8 |  |
| DC_48_n48-n66 | 0.8 |  | 0.8 |  | 0.6 |  |
| DC_48-66_n2 | 0.8 |  | 0.6 |  | 0.6 |  |
| DC_48-66_n12 | 0.8 |  | 0.6 |  | 0.3 |  |
| DC_48-66_n25 | 0.8 |  | 0.6 |  | 0.6 |  |
| DC_48-66_n48 | 0.8 |  | 0.6 |  | 0.6 |  |
| DC_48-66_n71 | 0.8 |  | 0.6 |  | 0.3 |  |
| DC_48-66_n5 | 0.8 |  | 0.6 |  | 0.3 |  |
| DC_48-66_n66 | 0.8 |  | 0.6 |  | 0.6 |  |
| DC_48-66_n77 | 0.8 |  | 0.6 |  | 0.8 |  |
| DC_66_n2-n38 | 0.5 |  | 0.5 |  | 0.9 |  |
| DC_66_n2-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_66_n2-n71 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_66_n2-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_66_n2-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_66-(n)5 DC_66-66-(n)5 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_66_n5-n48 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_66_n5-n77 | 0.6 |  | 0.3 |  | 0.8 |  |
| DC_66_n7-n78 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_66_(n)12 | 0.8 |  | 0.5 |  | 0.8 |  |
| DC_66_n25-n41 | 0.5 |  | 0.5 |  | 0.81 / 1.32 |  |
| DC_66_n25-n48 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_66_n25-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_66_n25-n71 | 0.5 |  | 0.5 |  | 0.3 |  |
| DC_66_n38-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_66_n38-n71 | 0.5 |  | 0.8 |  | 0.5 |  |
| DC_66_n38-n78 | 0.6 |  | 0.5 |  | 0.8 |  |
| DC_66_n41-n71 | 0.5 |  | 0.81 / 1.32 |  | 0.6 |  |
| DC_66_n66-n71 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_66_n66-n77 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_66_n66-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_66_(n)71 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_66-71_n38 | 0.5 |  | 0.5 |  | 0.8 |  |
| DC_66-71_n41 | 0.5 |  | 0.6 |  | 0.81 / 1.32 |  |
| DC_66-71_n66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_66-71_n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_66_n71-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_66_SUL_n78-n86 | 0.6 |  | 0.8 |  | 0.6 |  |
| DC_71_n2-n41 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_71_n2-n66 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_71_n2-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| DC_71_n38-n66 | 0.5 |  | 0.8 |  | 0.5 |  |
| DC_71_n38-n78 | 0.5 |  | 0.3 |  | 0.8 |  |
| DC_71_n66-n78 | 0.6 |  | 0.6 |  | 0.8 |  |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |  |  |  |

6.2B.4.2.3.3	ΔTIB,c for EN-DC four bands

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-3_n3-n41 | 0.5 | 0.5 | 0.5 | 0.34/0.85 |
| DC_1-3_n3-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3_n3-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-5_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-5_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-5_n79 | 0.3 | 0.3 | 0.3 | - |
| DC_1-3-7_n3 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-7_n5 | 0.6 | 0.6 | 0.6 | 0.3 |
| DC_1-3-7_n7 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-7_n8 | 0.6 | 0.6 | 0.6 | 0.3 |
| DC_1-3-7_n28 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-7_n38 | 0.6 | 0.6 | - | - |
| DC_1-3-7_n40 | 0.6 | 0.6 | 0.8 | 0.9 |
| DC_1-3-7_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-7_n78 DC_1-3-7-7_n78 | 0.7 | 0.7 | 0.7 | 0.8 |
| DC_1-3_n7-n78 | 0.7 | 0.7 | 0.7 | 0.8 |
| DC_1-3-8_n28 | 0.3 | 0.3 | 0.6 | 0.6 |
| DC_1-3-8_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-8_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3_n8-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-8_n79 | 0.3 | 0.3 | 0.3 | - |
| DC_1-3-11_n28 | 0.3 | 0.8 | 0.9 | 0.6 |
| DC_1-3-11_n77 | 0.6 | 0.8 | 0.9 | 0.8 |
| DC_1-3-18_n3 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_1-3-18_n28 | 0.3 | 0.3 | 0.3 | 0.6 |
| DC_1-3-18_n41 | 0.3 | 0.3 | 0.3 | 0.34 |
| DC_1-3-28_n3 | 0.3 | 0.3 | 0.6 | 0.3 |
| DC_1-3-18_n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-18_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-18_n79 | 0.3 | 0.3 | 0.3 | - |
| DC_1-3-19_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-19_n79 | 0.3 | 0.3 | 0.3 | - |
| DC_1-3-20_n7 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_1-3-20_n8 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-20_n28 | 0.3 | 0.3 | 0.6 | 0.6 |
| DC_1-3-20_n38 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_1-3-20_n41 | 0.5 | 0.5 | 0.3 | 0.84 / 1.35 |
| DC_1-3-20_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-21_n77 | 0.6 | 0.8 | 0.9 | 0.8 |
| DC_1-3-21_n78 | 0.6 | 0.8 | 0.9 | 0.8 |
| DC_1-3-21_n79 | 0.3 | 0.8 | 0.9 | - |
| DC_1-3_n26-n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_1-3-28_n5 | 0.3 | 0.3 | 0.6 | 0.6 |
| DC_1-3-28_n7 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-3-28_n40 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_1-3_n28-n75 | 0.3 | 0.3 | 0.6 | - |
| DC_1-3-28_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1_n3-n28-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-28_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3_n28-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3-28_n79 | 0.6 | 0.6 | 0.6 | - |
| DC_1_n3-n28-n79 | 0.6 | 0.6 | 0.6 | - |
| DC_1-3_n28-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-3_n28-n79 | 0.3 | 0.3 | 0.6 | - |
| DC_1-3-32_n28 | 0.3 | 0.3 | - | 0.6 |
| DC_1-3-38_n28 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_1-3-32_n78 | 0.6 | 0.6 | - | 0.8 |
| DC_1-3-38_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-3_n38-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_1-3_n40-n78 | 0.5 | 0.6 | 0.36 | 0.86 |
| DC_1-3-40_n78 | 0.6 | 0.6 | 0.39 | 0.89 |
| DC_1-3-41_n3 | 0.5 | 0.5 | 0.34 / 0.85 | 0.5 |
| DC_1-3-41_n28 | 0.5 | 0.5 | 0.34 / 0.85 | 0.6 |
| DC_1-3-41_n41 | 0.5 | 0.5 | 0.34 / 0.85 | 0.34 / 0.85 |
| DC_1-3_(n)41 | 0.5 | 0.5 | 0.34 / 0.85 | 0.34 / 0.85 |
| DC_1-3-41_n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-3_n41-n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-3-41_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-3_n41-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_1-3-41_n79 | 0.5 | 0.5 | 0.34 / 0.85 | - |
| DC_1-3-42_n28 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_1-3-42_n77 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_1-3-42_n78 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_1-3-42_n79 | 0.6 | 0.6 | 0.8 | - |
| DC_1-3_n75-n78 | 0.6 | 0.6 | - | 0.8 |
| DC_1-3_n77-n79 | 0.6 | 0.6 | 0.8 | - |
| DC_1_n3-n77-n79 | 0.6 | 0.6 | 0.8 | - |
| DC_1-3_n78-n79 | 0.6 | 0.6 | 0.8 | - |
| DC_1-3_SUL_n78-n80 | 0.6 | 0.6 | 0.8 | 0.6 |
| DC_1-5-7_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-5-7_n78 DC_1-5-7-7_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-5-41_n79 | 0.5 | 0.3 | 0.5 | - |
| DC_1-7_n3-n38 | 0.6 | 0.6 | 0.6 | 0.5 |
| DC_1-7_n3-n78 | 0.5 | 0.2 | 0.6 | 0.8 |
| DC_1-7_n7-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7-8_n3 | 0.6 | 0.6 | 0.3 | 0.6 |
| DC_1-7-8_n28 | 0.5 | 0.6 | 0.6 | 0.6 |
| DC_1-7-8_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7_n8-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7-20_n3 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_1-7-20_n8 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-7-20_n28 | 0.5 | 0.6 | 0.6 | 0.6 |
| DC_1-7-20_n38 | 0.5 | - | 0.3 | - |
| DC_1-7-20_n78 | 0.6 | 0.7 | 0.4 | 0.8 |
| DC_1-7_n26-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7-28_n3 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_1-7-28_n5 | 0.3 | 0.3 | 0.6 | 0.6 |
| DC_1-7-28_n7 | 0.5 | 0.6 | 0.6 | 0.6 |
| DC_1-7-28_n40 | 0.6 | 0.8 | 0.6 | 0.9 |
| DC_1-7-28_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7_n28-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-7-32_n3 | 0.6 | 0.6 | - | 0.6 |
| DC_1-7-32_n8 | 0.7 | 0.7 | - | 0.6 |
| DC_1-7-32_n28 | 0.5 | 0.6 | - | 0.7 |
| DC_1-7-38_n3 | 0.6 | - | - | 0.6 |
| DC_1-7-38_n78 | 0.3 | - | - | 0.8 |
| DC_1-7-32_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-7-38_n8 | 0.5 | - | - | 0.5 |
| DC_1-7-38_n28 | 0.3 | - | - | 0.6 |
| DC_1-7-40_n78 | 0.6 | 0.5 | 0.39 | 0.89 |
| DC_1-7_n40-n78 | 0.6 | 0.5 | 0.5 | 0.8 |
| DC_1-7_n75-n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-8_n3-n28 | 0.3 | 0.6 | 0.3 | 0.6 |
| DC_1-8_n3-n77 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_1-8_n3-n79 | 0.3 | 0.3 | 0.3 | 0.8 |
| DC_1-8-11_n3 | 0.3 | 0.3 | 0.8 | 0.9 |
| DC_1-8-11_n28 | 0.3 | 0.6 | 0.4 | 0.6 |
| DC_1-8-11_n77 | 0.6 | 0.6 | 0.4 | 0.8 |
| DC_1-8-11_n78 | 0.3 | 0.6 | 0.4 | 0.8 |
| DC_1-8-11_n79 | 0.3 | 0.3 | 0.4 | - |
| DC_1-8-20_n3 | 0.3 | 0.4 | 0.4 | 0.3 |
| DC_1-8-20_n28 | 0.3 | 0.6 | 0.6 | 0.6 |
| DC_1-8-20_n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-8-28_n3 | 0.3 | 0.6 | 0.6 | 0.3 |
| DC_1-8_n28-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-8-28_n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-8_n28-n78 | 0.3 | 0.6 | 0.5 | 0.8 |
| DC_1-8_n28-n79 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-8-32_n3 | 0.5 | 0.3 | - | 0.8 |
| DC_1-8-32_n78 | 0.5 | 0.6 | - | 0.8 |
| DC_1-8-40_n78 | 0.6 | 0.6 | 0.39 | 0.89 |
| DC_1-8-42_n3 | 0.3 | 0.6 | 0.8 | 0.6 |
| DC_1-8-42_n28 | 0.3 | 0.6 | 0.8 | 0.8 |
| DC_1-8_n40-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_1-8-42_n77 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_1-8_n77-n79 | 0.6 | 0.6 | 0.8 | 0.5 |
| DC_1-11_n3-n28 | 0.3 | 0.8 | 0.9 | 0.6 |
| DC_1-11_n3-n77 | 0.6 | 0.8 | 0.9 | 0.8 |
| DC_1-11_n3-n79 | 0.3 | 0.8 | 0.9 | 0.8 |
| DC_1-11-18_n3 | 0.3 | 0.9 | 0.3 | 0.8 |
| DC_1-11-18_n28 | 0.3 | 0.4 | 0.4 | 0.6 |
| DC_1-11-18_n41 | 0.5 | 0.4 | 0.3 | 0.5 |
| DC_1-11-18_n77 | 0.6 | 0.4 | 0.3 | 0.8 |
| DC_1-11-18_n78 | 0.3 | 0.4 | 0.3 | 0.8 |
| DC_1-11_n77-n79 | 0.6 | 0.4 | 0.8 | - |
| DC_1-18_n3-n41 | 0.3 | 0.3 | 0.3 | 0.34 |
| DC_1-18_n3-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_1-18_n3-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_1-18_n28-n41 | 0.3 | 0.3 | 0.5 | 0.34 |
| DC_1-18-28_n77 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_1-18_n28-n77 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_1-18-28_n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_1-18_n28-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_1-18-28_n79 | 0.3 | 0.5 | 0.5 | - |
| DC_1-18-41_n77 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_1-18_n41-n77 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_1-18-41_n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_1-18_n41-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_1-18-42_n77 | 0.3 | 0.3 | 0.8 | 0.8 |
| DC_1-18-42_n78 | 0.3 | 0.3 | 0.8 | 0.8 |
| DC_1-18-42_n79 | 0.3 | 0.3 | 0.8 | - |
| DC_1-19-42_n77 | 0.6 | 0.3 | 0.8 | 0.8 |
| DC_1-19-42_n78 | 0.3 | 0.3 | 0.8 | 0.8 |
| DC_1-19-42_n79 | 0.3 | 0.3 | 0.8 | - |
| DC_1-19_n77-n79 | 0.3 | 0.3 | 0.8 | - |
| DC_1-19_n78-n79 | 0.3 | 0.3 | 0.8 | - |
| DC_1-20_n3-n38 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_1-20_n3-n78 | 0.3 | 0.6 | 0.3 | 0.8 |
| DC_1-20_n7-n78 | 0.5 | 0.3 | 0.3 | 0.8 |
| DC_1-20_n8-n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-20-28_n3 | 0.3 | 0.6 | 0.6 | 0.3 |
| DC_1-20_n28-n75 | 0.3 | 0.6 | 0.7 | - |
| DC_1-20-28_n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-20_n28-n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_1-20-32_n3 | 0.5 | 0.3 | - | 0.5 |
| DC_1-20-32_n28 | 0.3 | 0.6 | - | 0.7 |
| DC_1-20-32_n78 | 0.3 | 0.3 | - | 0.8 |
| DC_1-20-38_n3 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_1-20_(n)38 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_1-20-38_n8 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_1-20-38_n78 | 0.3 | 0.6 | - | 0.8 |
| DC_1-20-40_n78 | 0.5 | 0.3 | 0.59 | 0.89 |
| DC_1-20_n41-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_1-21-28_n77 | 0.6 | 0.4 | 0.6 | 0.8 |
| DC_1-21-28_n78 | 0.3 | 0.4 | 0.6 | 0.8 |
| DC_1-21-28_n79 | 0.3 | 0.4 | 0.6 | - |
| DC_1-21_n28-n77 | 0.3 | 0.4 | 0.6 | 0.8 |
| DC_1-21_n28-n78 | 0.6 | 0.4 | 0.6 | 0.8 |
| DC_1-21_n28-n79 | 0.3 | 0.4 | 0.6 | - |
| DC_1-21-42_n77 | 0.6 | 0.4 | 0.8 | 0.8 |
| DC_1-21-42_n78 | 0.3 | 0.4 | 0.8 | 0.8 |
| DC_1-21-42_n79 | 0.3 | 0.4 | 0.8 | - |
| DC_1-21_n77-n79 | 0.3 | 0.3 | 0.8 |  |
| DC_1-21_n78-n79 | 0.3 | 0.3 | 0.8 |  |
| DC_1-28_n3-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-28_n3-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-28_n7-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_1-28-32_n3 | 0.5 | 0.6 | - | 0.5 |
| DC_1-28-40_n78 | 0.5 | 0.5 | 0.36 | 0.86 |
| DC_1-28_n40-n78 | 0.5 | 0.5 | 0.36 | 0.86 |
| DC_1-28-42_n77 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_1-28-42_n78 | 0.3 | 0.6 | 0.8 | 0.8 |
| DC_1-28-42_n79 | 0.3 | 0.6 | 0.8 |  |
| DC_1_n28-n77-n79 | 0.6 | 0.6 | 0.8 | 0.5 |
| DC_1_n28-n78-n79 | 0.3 | 0.6 | 0.8 | 0.5 |
| DC_1-38_n3-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_1-38_n7-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_1-38_n28-n78 | 0.5 | 0.5 | 0.5 | 0.8 |
| DC_1-41_n3-n41 | 0.5 | 0.34 / 0.85 | 0.5 | 0.34 / 0.85 |
| DC_1-41_n3-n77 | 0.6 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_1-41_n3-n78 | 0.6 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_1-41_n28-n41 | 0.5 | 0.34 / 0.85 | 0.5 | 0.34 / 0.85 |
| DC_1-41_n28-n77 | 0.6 | 0.5 | 0.5 | 0.8 |
| DC_1-41_n28-n78 | 0.5 | 0.5 | 0.5 | 0.8 |
| DC_1-41_n41-n77 | 0.5 | 0.5 | 0.5 | 0.8 |
| DC_1-41_n41-n78 | 0.5 | 0.5 | 0.5 | 0.8 |
| DC_1-41-42_n77 | 0.5 | 0.5 | 0.8 | 0.8 |
| DC_1-41-42_n78 | 0.5 | 0.5 | 0.8 | 0.8 |
| DC_1-41-42_n79 | 0.5 | 0.5 | 0.8 | - |
| DC_1-42_n3-n28 | 0.3 | 0.8 | 0.6 | 0.8 |
| DC_1-42_n3-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_1-42_n77-n79 | 0.6 | 0.8 | 0.8 | - |
| DC_1-42_n28-n77 | 0.6 | 0.8 | 0.8 | 0.8 |
| DC_1-42_n78-n79 | 0.3 | 0.8 | 0.8 | - |
| DC_2-4-7_n28 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_2-5_n2-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5_n5-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5-7_n2 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-5-7_n7 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-5-7_n66 DC_2-2-5-7_n66 DC_2-5-7-7_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-5-7_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5_(n)12 | 0.3 | 0.8 | 0.4 | 0.4 |
| DC_2-12_(n)5 | - | 0.5 | 0.3 | 0.5 |
| DC_2-5-30_n2 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_2-5-30_n66 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_2-5-30_n77 DC_2-2-5-30_n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_2-5-48_n12 | 0.6 | 0.8 | 0.8 | 0.4 |
| DC_2-5-48_n71 | 0.6 | 0.5 | 0.8 | 0.5 |
| DC_2-5-48_n77 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_2-5-66_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-5-66_n5 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-5-66_n7 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-5-66_n12 | 0.3 | 0.5 | 0.5 | 0.3 |
| DC_2-5-66_n30 DC_2-2-5-66_n30 DC_2-5-66-66_n30 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-5-66_n48 DC_2-5-66-66_n48 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_2-5-66_n66 DC_2-5-5-66_n66 DC_2-5-66-66_n66 DC_2-2-5-66-66_n66 DC_2-5-5-66-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-5-66_n71 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-5-66_n77 DC_2-2-5-66_n77 DC_2-5-66-66_n77 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_2-5-66_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5_n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-5_n66-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_2-7_n2-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_2-7-12_n2 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7-12_n66 DC_2-2-7-12_n66 | 0.5 | 0.5 | 0.8 | 0.5 |
| DC_2-7-12_n78 DC_2-2-7-12_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-7-13_n25 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7-13_n66 DC_2-7-7-13_n66 DC_2-2-7-7-13_n66 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7_n25-n66 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-7-28_n7 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7-28_n66 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_2-7-28_n78 | 0.5 | 0.5 | 0.3 | 0.8 |
| DC_2-7-29_n78 DC_2-7-7-29_n78 | 0.6 | 0.5 | - | 0.8 |
| DC_2-7_n38-n66 DC_2-7-7_n38-n66 | 0.5 | - | - | 0.5 |
| DC_2-7_n38-n78 DC_2-7-7_n38-n78 | 0.6 | - | - | 0.8 |
| DC_2-7-66_n2 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n7 DC_2-7-66-66_n7 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n25 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n28 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_2-7-66_n38 DC_2-2-7-66_n38 | 0.5 | - | 0.5 | - |
| DC_2-7-66_n66 DC_2-7-7-66_n66 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n71 DC_2-2-7-66_n71 | 0.5 | 0.5 | 0.5 | 0.3 |
| DC_2-7-66_n77 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_2-7-66_n78 DC_2-7-7-66_n78 DC_2-7-66-66_n78 DC_2-7-7-66-66_n78 DC_2-7_n66-n78 DC_2-7-7_n66-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_2-7_n66-n77 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_2-7-71_n2 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_2-7-71_n66 DC_2-2-7-71_n66 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-7-71_n78 DC_2-2-7 -71_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-7_n71-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-12_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-12-30_n2 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_2-12-30_n66 | 0.5 | 0.8 | 0.3 | 0.5 |
| DC_2-12-30_n77 DC_2-2-12-30_n77 | 0.6 | 0.5 | 0.3 | 0.8 |
| DC_2-12-48_n5 | 0.6 | 0.4 | 0.8 | 0.8 |
| DC_2-12-66_n5 | 0.5 | 0.8 | 0.5 | 0.8 |
| DC_2-12-66_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-12-66_n30 DC_2-2-12-66_n30 DC_2-12-66-66_n30 | 0.5 | 0.8 | 0.5 | 0.3 |
| DC_2-12-66_n66 | 0.5 | 0.8 | 0.5 | 0.5 |
| DC_2-12-66_n77 DC_2-2-12-66_n77 DC_2-12-66-66_n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_2-12-66_n78 DC_2-2-12-66_n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_2-12_n66-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_2-13_n25-n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-13-48_n77 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_2-13-66_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-13-66_n5 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-13-66_n48 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_2-13-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-13-66_n77 DC_2-2-13-66_n77 DC_2-2-13-66-66_n77 DC_2-13-66-66_n77 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_2-13_n66-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_2-14-30_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-14-30_n66 | 0.5 | 0.3 | 0.3 | 0.5 |
| DC_2-14-30_n77 DC_2-2-14-30_n77 | 0.6 | 0.5 | 0.3 | 0.8 |
| DC_2-14-66_n2 DC_2-14-66-66_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-14-66_n30 DC_2-2-14-66_n30 DC_2-14-66-66_n30 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-14-66_n66 DC_2-2-14-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-14-66_n77 DC_2-2-14-66_n77 DC_2-14-66-66_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-28-66_n7 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_2-28-66_n66 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_2-29-30_n2 | 0.5 | - | 0.3 | 0.5 |
| DC_2-29-30_n66 | 0.5 | - | 0.3 | 0.5 |
| DC_2-29-30_n77 DC_2-2-29-30_n77 | 0.6 | - | 0.3 | 0.8 |
| DC_2-29-66_n2 DC_2-29-66-66_n2 | 0.5 | - | 0.5 | 0.5 |
| DC_2-29-66_n30 DC_2-2-29-66_n30 DC_2-29-66-66_n30 | 0.5 | - | 0.5 | 0.3 |
| DC_2-29-66_n66 | 0.5 | - | 0.5 | 0.5 |
| DC_2-29-66_n77 | 0.6 | - | 0.6 | 0.8 |
| DC_2-29-66_n78 | 0.6 | - | 0.6 | 0.8 |
| DC_2-30-(n)5 DC_2-2-30-(n)5 | 0.5 | 0.3 | 0.3 | 0.3 |
| DC_2-30-66_n2 DC_2-30-66-66_n2 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-30-66_n5 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-30-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_2-30-66_n77 DC_2-2-30-66_n77 DC_2-30-66-66_n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_2-46_n41-n66 | 0.5 | - | 0.5 | 0.5 |
| DC_2-46_n41-n71 | 0.5 | - | 0.5 | 0.6 |
| DC_2-46-48_n2 | 0.6 | - | 0.8 | 0.6 |
| DC_2-46-48_n5 | 0.6 | - | 0.8 | 0.3 |
| DC_2-46-48_n66 | 0.6 | - | 0.8 | 0.6 |
| DC_2-46-66_n5 | 0.5 | - | 0.5 | 0.3 |
| DC_2-46-66_n41 | 0.5 | - | 0.5 | 0.81 / 1.32 |
| DC_2-46-66_n71 | - | - | 0.3 | 0.3 |
| DC_2-48-66_n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_2-48_n48-n66 | 0.6 | 0.8 | 0.8 | 0.6 |
| DC_2-48_(n)5 | 0.6 | 0.3 | 0.8 | 0.3 |
| DC_2-46_n66_n71 | 0.5 | - | 0.5 | 0.3 |
| DC_2-48-66_n2 | 0.6 | 0.8 | 0.6 | 0.6 |
| DC_2-48-66_n5 | 0.6 | 0.8 | 0.6 | - |
| DC_2-48-66_n12 | 0.6 | 0.8 | 0.6 | 0.3 |
| DC_2-48-66_n66 | 0.6 | 0.8 | 0.6 | 0.6 |
| DC_2-48-66_n71 | 0.6 | 0.8 | 0.6 | 0.3 |
| DC_2-66_n2-n77 DC_2-66-66_n2-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-66_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-66_(n)5 DC_2-2-66_(n)5 DC_2-66-66_(n)5 | 0.5 | 0.3 | 0.5 | 0.3 |
| DC_2-66_n5-n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_2-66_n25-n66 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-66_n38-n78 | 0.6 | 0.6 | 0.9 | 0.8 |
| DC_2-66_n41-n71 | 0.5 | 0.5 | 0.81 / 1.32 | 0.8 |
| DC_2-66_n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-66_n66-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-66-71_n2 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-66-71_n38 DC_2-2-66-71_n38 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-66-71_n41 DC_2-2-66-71_n41 | 0.5 | 0.5 | 0.8 | 0.81 / 1.32 |
| DC_2-66-71_n66 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-66-(n)71 | 0.5 | 0.5 | 0.3 | 0.3 |
| DC_2-66-71_n71 | 0.5 | 0.5 | 0.3 | 0.3 |
| DC_2-66-71_n78 DC_2-2-66-71_n78 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-66_n71-n78 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_2-71_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_2-71_n66-n78 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_3_n1-n40-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3_n1-n77-n79 | 0.6 | 0.6 | 0.8 | 0.5 |
| DC_3_n1-n78-n79 | 0.6 | 0.3 | 0.8 | 0.5 |
| DC_3-5-7_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-5-7_n78 DC_3-5-7-7_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3_n5-n40-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-5-41_n79 | 0.5 | 0.33 | 0.34 / 0.85 | - |
| DC_3-7_n1-n8 DC_3-3-7_n1-n8 DC_3-3-7-7_n1-n8 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_3-7_n1-n40 | 0.6 | 0.8 | 0.6 | 0.9 |
| DC_3-7_n1-n75 | 0.6 | 0.6 | 0.6 | - |
| DC_3-7_n1-n78 | 0.7 | 0.7 | 0.7 | 0.8 |
| DC_3-7_n3-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7_n7-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7-8_n1 DC_3-3-7-8_n1 DC_3-7-7-8_n1 DC_3-3-7-7-8_n1 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_3-7-8_n28 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_3-7-8_n40 | 0.5 | 0.5 | 0.6 | 0.6 |
| DC_3-7-8_n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7-8_n78 DC_3-3-7-8_n78 DC_3-7-7-8_n78 DC_3-3-7-7-8_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7_n8-n78 DC_3-3-7_n8-n78 DC_3-7-7_n8-n78 DC_3-3-7-7_n8-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7-20_n1 | 0.6 | 0.6 | 0.3 | 0.6 |
| DC_3-7-20_n8 | 0.6 | 0.6 | 0.6 | 0.6 |
| DC_3-7-20_n28 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_3-7-20_n38 | 0.5 | - | 0.3 | - |
| DC_3-7-20_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_3-7_n26-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7-28_n1 DC_3-7-7-28_n1 | 0.6 | 0.6 | 0.5 | 0.6 |
| DC_3-7-28_n3 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_3-7-28_n5 | 0.5 | 0.5 | 0.4 | 0.4 |
| DC_3-7-28_n7 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_3-7-28_n40 | 0.6 | 0.8 | 0.3 | 0.9 |
| DC_3-7-28_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7_n28-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7-32_n1 | 0.6 | 0.6 | - | 0.6 |
| DC_3-7-32_n28 | 0.5 | 0.5 | - | 0.3 |
| DC_3-7-32_n78 | 0.6 | 0.6 | - | 0.8 |
| DC_3-7-38_n28 | 0.3 | - | - | 0.3 |
| DC_3-7-40_n1 | 0.6 | 0.8 | 0.9 | 0.6 |
| DC_3-7-40_n78 | 0.6 | 0.5 | 0.39 | 0.89 |
| DC_3-7_n40-n78 | 0.6 | 0.5 | 0.5 | 0.8 |
| DC_3-7_n75-n78 | 0.6 | 0.6 | - | 0.8 |
| DC_3-7_SUL_n78-n80 | 0.6 | 0.6 | 0.8 | 0.6 |
| DC_3-8_n1-n28 | 0.3 | 0.6 | 0.3 | 0.6 |
| DC_3-3-8_n1-n78 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_3_n1-n8-n78 | 0.5 | 0.5 | 0.5 | 0.6 |
| DC_3-8_n1-n78 DC_3-3-8_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-8-11_n28 | 0.8 | 0.6 | 0.9 | 0.6 |
| DC_3-8-11_n77 | 0.8 | 0.6 | 0.9 | 0.8 |
| DC_3-8-20_n1 | 0.3 | 0.4 | 0.4 | 0.3 |
| DC_3-8-20_n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-8_n28-n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-8-28_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-8_n28-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-8-32_n1 | 0.5 | 0.3 | - | 0.8 |
| DC_3-8-32_n28 | 0.3 | 0.3 | - | 0.6 |
| DC_3-8-32_n78 | 0.8 | 0.6 | - | 0.8 |
| DC_3-8-40_n1 | 0.5 | 0.5 | 0.6 | 0.5 |
| DC_3-8-40_n78 | 0.6 | 0.6 | 0.39 | 0.89 |
| DC_3-8_n40-n78 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_3-8_n40-n79 | 0.5 | 0.3 | 0.5 | - |
| DC_3-8-42_n77 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_(n)3-n8-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-8_n77-n79 | 0.6 | 0.6 | 0.8 | 0.5 |
| DC_3-8_SUL_n78-n80 | 0.6 | 0.6 | 0.8 | 0.6 |
| DC_3-11_n28-n77 | 0.8 | 0.9 | 0.6 | 0.8 |
| DC_3-18_n3-n41 | 0.6 | 0.3 | 0.6 | 0.34 / 0.85 |
| DC_3-18_n3-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-18_n3-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-18_n28-n41 | 0.6 | 0.3 | 0.5 | 0.34 / 0.85 |
| DC_3-18_n28-n77 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_3-18_n28-n78 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_3-18_n41-n77 | 0.6 | 0.3 | 0.5 | 0.8 |
| DC_3-18_n41-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-18-42_n77 | 0.3 | 0.3 | 0.8 | 0.8 |
| DC_3-18-42_n78 | 0.3 | 0.3 | 0.8 | 0.8 |
| DC_3-18-42_n79 | 0.6 | 0.3 | 0.8 | - |
| DC_3-19_n1-n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-19_n1-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-19_n1-n79 | 0.3 | 0.3 | 0.3 | - |
| DC_3-19-21_n77 | 0.8 | 0.3 | 0.9 | 0.8 |
| DC_3-19-21_n78 | 0.8 | 0.3 | 0.9 | 0.8 |
| DC_3-19-21_n79 | 0.8 | 0.3 | 0.9 | - |
| DC_3-19-42_n1 | 0.6 | 0.3 | 0.8 | - |
| DC_3-19-42_n77 | 0.6 | 0.3 | 0.8 | 0.8 |
| DC_3-19-42_n78 | 0.6 | 0.3 | 0.8 | 0.8 |
| DC_3-19-42_n79 | 0.6 | 0.3 | 0.8 | - |
| DC_3-19_n77-n79 | 0.6 | 0.3 | 0.8 | - |
| DC_3-19_n78-n79 | 0.6 | 0.3 | 0.8 | - |
| DC_3-20_n1-n7 | 0.6 | 0.3 | 0.6 | 0.6 |
| DC_3-20_n1-n28 | 0.3 | 0.3 | 0.6 | 0.6 |
| DC_3-20_n1-n75 | 0.5 | 0.3 | 0.5 | - |
| DC_3-20_n1-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-20_n1-n28 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_3-20_n7-n28 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_3-20_n8-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-20-28_n1 | 0.3 | 0.6 | 0.6 | 0.3 |
| DC_3-20_n28-n75 | 0.3 | 0.5 | 0.5 | - |
| DC_3-20-28_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-20_n28-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-20-32_n1 | 0.5 | 0.3 | - | 0.5 |
| DC_3-20-32_n28 | 0.3 | 0.5 | - | 0.5 |
| DC_3-20-32_n78 | 0.5 | 0.3 | - | 0.8 |
| DC_3-20-38_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-20_n38-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_3-20-40_n78 | 0.6 | 0.5 | 0.36 | 0.86 |
| DC_3-20_n41-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_3_20_SUL_n78-n80 | 0.5 | 0.3 | 0.8 | 0.5 |
| DC_3-21_n1-n77 | 0.8 | 0.9 | 0.6 | 0.8 |
| DC_3-21_n1-n78 | 0.8 | 0.9 | 0.6 | 0.8 |
| DC_3-21_n1-n79 | 0.8 | 0.9 | 0.3 | - |
| DC_3-21_n28-n77 | 0.8 | 0.9 | 0.5 | 0.8 |
| DC_3-21_n28-n78 | 0.8 | 0.9 | 0.5 | 0.8 |
| DC_3-21_n28-n79 | 0.8 | 0.9 | 0.3 | - |
| DC_3-21-42_n1 | 0.8 | 0.9 | 0.8 | 0.6 |
| DC_3-21-42_n77 | 0.8 | 0.9 | 0.8 | 0.8 |
| DC_3-21-42_n78 | 0.8 | 0.9 | 0.8 | 0.8 |
| DC_3-21-42_n79 | 0.8 | 0.9 | 0.8 | - |
| DC_3-21_n77-n79 | 0.8 | 0.9 | 0.8 | - |
| DC_3-21_n78-n79 | 0.8 | 0.9 | 0.8 | - |
| DC_3-28_n1-n40 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_3-28_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-28_n3-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_3-28_n7-n78 DC_3-3-28_n7-n78 | 1.0 | 0.5 | 0.8 | 0.8 |
| DC_3-28-32_n1 | 0.3 | 0.6 | - | 0.3 |
| DC_3-28-40_n78 | 0.6 | 0.5 | 0.36 | 0.86 |
| DC_3-28_n40-n78 | 0.6 | 0.5 | 0.36 | 0.86 |
| DC_3-28-41_n78 | 1.0 | 0.5 | 0.34 / 0.85 | 0.8 |
| DC_3-28-42_n77 | 0.6 | 0.5 | 0.8 | 0.8 |
| DC_3-28-42_n78 | 0.6 | 0.5 | 0.8 | 0.8 |
| DC_3-28-42_n79 | 0.6 | 0.5 | 0.8 | - |
| DC_3_n28-n77-n79 | 0.6 | 0.5 | 0.8 | 0.5 |
| DC_3_n28-n78-n79 | 0.6 | 0.5 | 0.8 | 0.5 |
| DC_3-32_n1-n28 | 0.3 | - | 0.3 | 0.6 |
| DC_3-32_n1-n78 | 0.6 | - | 0.6 | 0.8 |
| DC_3-32-38_n28 | 0.7 | - | 0.7 | 0.6 |
| DC_3-38_n28-n78 | 1.0 | 0.3 | 0.5 | 0.8 |
| DC_3-40_n1-n78 | 0.6 | 0.36 | 0.5 | 0.86 |
| DC_3_n40-n41-n79 | 0.5 | 0.5 | 0.54/0.85 | 0.8 |
| DC_3-41_n1-n78 DC_3-3-41_n1-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_3-41_n3-n41 | 0.5 | 0.34 / 0.85 | 0.5 | 0.34 / 0.85 |
| DC_3-41_n3-n77 | 0.6 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_3-41_n3-n78 | 0.6 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_3-41_n28-n41 | 0.6 | 0.34 / 0.85 | 0.5 | 0.34 / 0.85 |
| DC_3-41_n28-n77 | 0.6 | 0.34 / 0.85 | 0.5 | 0.8 |
| DC_3-41_n28-n78 | 1.0 | 0.34 / 0.85 | 0.5 | 0.8 |
| DC_3-41_n41-n77 | 0.6 | 0.34 / 0.85 | 0.34 / 0.85 | 0.8 |
| DC_3-41_n41-n78 | 0.6 | 0.34 / 0.85 | 0.34 / 0.85 | 0.8 |
| DC_3-41-42_n77 | 1.0 | 0.34 / 0.85 | 0.8 | 0.8 |
| DC_3-41-42_n78 | 1.0 | 0.34 / 0.85 | 0.8 | 0.8 |
| DC_3-41-42_n79 | 1.0 | 0.34 / 0.85 | 0.8 | - |
| DC_3-42_n1-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_3-42_n1-n78 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_3-42_n1-n79 | 0.6 | 0.8 | 0.6 | - |
| DC_3-42_n28-n77 | 0.6 | 0.8 | 0.8 | 0.8 |
| DC_3-42_n77-n79 | 0.6 | 0.8 | 0.8 | - |
| DC_3-42_n78-n79 | 0.6 | 0.8 | 0.8 | - |
| DC_5-7_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_5-7-66_n2 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_5-7-66_n7 DC_5-7-66-66_n7 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_5-7-66_n66 DC_5-7-7-66_n66 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_5-7_n66-n78 | 0.5 | 0.8 | 1.0 | 0.8 |
| DC_5-7-66_n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_5-30-66_n2 | 0.3 | 0.3 | 0.5 | 0.5 |
| DC_5-30-66_n66 | 0.3 | 0.3 | 0.5 | 0.5 |
| DC_5-30-66_n77 DC_5-30-66-66_n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_5-48_(n)12 | 0.8 | 0.4 | 0.3 | 0.8 |
| DC_5-48-66_n12 | 0.8 | 0.8 | 0.6 | 0.4 |
| DC_5-48-66_n71 | 0.5 | 0.8 | 0.6 | 0.5 |
| DC_5-48-66_n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_5-66_n2-n77 DC_5-66-66_n2-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_5-66_n2-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_5-66_n5-n77 DC_5-66-66_n5-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_5-66_(n)12 | 0.3 | 0.8 | 0.8 | 0.8 |
| DC_5-66_n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7-8_n1-n40 | 0.8 | 0.6 | 0.6 | 0.9 |
| DC_7-8_n1-n78 DC_7-7-8_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7-8-20_n1 | 0.6 | 0.6 | 0.6 | 0.5 |
| DC_7-8-20_n3 | 0.5 | 0.6 | 0.4 | 0.5 |
| DC_7-8_n28-n78 | 0.5 | 0.6 | 0.5 | 0.8 |
| DC_7-8-32_n1 | 0.7 | 0.6 | - | 0.7 |
| DC_7-8-32_n78 | 0.7 | 0.6 | - | 0.8 |
| DC_7-8-38_n1 | - | 0.5 | - | 0.5 |
| DC_7-8-40_n1 | 0.8 | 0.6 | 0.9 | 0.6 |
| DC_7-8-40_n78 | 0.5 | 0.6 | 0.39 | 0.89 |
| DC_7-8_n40-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_7-12_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7-12-66_n2 | 0.5 | 0.8 | 0.5 | 0.5 |
| DC_7-12-66_n78 | 0.8 | 0.5 | 1.0 | 0.8 |
| DC_7-12_n66-n78 | 0.8 | 0.5 | 1.0 | 0.8 |
| DC_7-13_n25-n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_7-13-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_7-20_n1-n75 | 0.7 | 0.3 | 0.7 | - |
| DC_7-20_n1-n78 | 0.7 | 0.4 | 0.6 | 0.8 |
| DC_7-20_n3-n38 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_7-20_n3-n78 | 0.5 | 0.6 | 0.5 | 0.8 |
| DC_7-20_n8-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_7-20-28_n1 | 0.6 | 0.6 | 0.6 | 0.5 |
| DC_7-20-28_n3 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_7-20_n28-n78 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_7-20-32_n1 | 0.6 | 0.3 | - | 0.5 |
| DC_7-20-32_n3 | 0.7 | 0.3 | - | 0.3 |
| DC_7-20-32_n8 | 0.7 | 0.6 | - | 0.6 |
| DC_7-20-32_n28 | 0.3 | 0.5 | - | 0.7 |
| DC_7-20-32_n78 | 0.7 | 0.5 | - | 0.8 |
| DC_7-20-38_n1 | - | 0.3 | - | 0.5 |
| DC_7-20-38_n3 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_7-20-38_n8 | - | 0.6 | - | 0.6 |
| DC_7-20-38_n78 | - | 0.6 | - | 0.8 |
| DC_7-28_n1-n40 | 0.3 | 0.2 | - | 0.8 |
| DC_7-28_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7-28_n3-n78 | 0.8 | 0.5 | 1.0 | 0.8 |
| DC_7-28_n7-n78 | 0.3 | 0.3 | 0.3 | 0.8 |
| DC_7-28-66_n7 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_7-28-66_n66 | 0.5 | 0.6 | 0.5 | 0.5 |
| DC_7-28-32_n1 | 0.7 | 0.6 | - | 0.7 |
| DC_7-28-32_n3 | 0.7 | 0.3 | - | 0.7 |
| DC_7-28-38_n1 | - | 0.6 | - | 0.5 |
| DC_7-28_n40-n78 | 0.5 | 0.3 | 0.5 | 0.8 |
| DC_7-29-66_n78 | 0.5 | - | 0.6 | 0.8 |
| DC_7-32_n1-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_7-38_n3-n78 | - | - | 0.6 | 0.8 |
| DC_7-40_n1-n78 | 0.5 | 0.56 | 0.6 | 0.86 |
| DC_7-66_n2-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_7-66_n25-n66 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_7-66_n38-n78 DC_7-7-66_n38-n78 | - | 0.6 | - | 0.8 |
| DC_7-66_n66-n77 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_7-66_n66-n78 DC_7-7-66_n66-n78 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_7-66-71_n2 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_7-66-71_n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_7-66_n71-n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_7-71_n2-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_7-71_n66-n78 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_8_n1-n3-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_8_n3-n28-n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_8_n3-n28-n79 | 0.6 | 0.5 | 0.3 | 0.8 |
| DC_8_n3-n77-n79 | 0.6 | 0.6 | 0.8 | 0.8 |
| DC_8-11_n1-n77 | 0.6 | 0.4 | 0.6 | 0.8 |
| DC_8-11_n3-n28 | 0.6 | 0.8 | 0.9 | 0.6 |
| DC_8-11_n3-n77 | 0.6 | 0.8 | 0.9 | 0.8 |
| DC_8-11_n3-n79 | 0.3 | 0.8 | 0.9 | 0.8 |
| DC_8-11_n28-n77 | 0.6 | 0.4 | 0.6 | 0.8 |
| DC_8-11_n77-n79 | 0.6 | 0.4 | 0.8 | 0.5 |
| DC_8-20-28_n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| DC_8-20-32_n1 | 0.4 | 0.4 | - | 0.5 |
| DC_8-20-38_n1 | 0.6 | 0.5 | 0.5 | 0.5 |
| DC_8_n28-n77-n79 | 0.6 | 0.5 | 0.8 | 0.8 |
| DC_8-32-38_n1 | 0.3 | - | 0.5 | 0.5 |
| DC_8_n39-n40-n41 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_8_n39-n40-n79 | 0.3 | 0.3 | 0.3 | 0.8 |
| DC_8_n40-n41-n79 | 0.3 | 0.3 | 0.3 | - |
| DC_8-40_n1-n78 | 0.3 | 0.56 | 0.5 | 0.86 |
| DC_8-41_n1-n3 | 0.3 | 0.54 / 0.85 | 0.5 | 0.5 |
| DC_8-41_n1-n77 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_8-41_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_8-41_n3-n77 | 0.6 | 0.310 / 0.811 | 0.6 | 0.8 |
| DC_8-42_n1-n3 | 0.6 | 0.8 | 0.3 | 0.6 |
| DC_8-42_n1-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_8-42_n3-n28 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_8-42_n3-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| DC_8-42_n28-n77 | 0.6 | 0.8 | 0.8 | 0.8 |
| DC_11_n3-n28-n77 | 0.8 | 0.9 | 0.6 | 0.8 |
| DC_11_n3-n77-n79 | 0.8 | 0.9 | 0.8 | 0.8 |
| DC_12-30-66_n2 | 0.8 | 0.3 | 0.5 | 0.5 |
| DC_12-30-66_n66 | 0.8 | 0.3 | 0.5 | 0.5 |
| DC_12-30-66_n77 DC_12-30-66-66_n77 | 0.8 | 0.3 | 0.6 | 0.8 |
| DC_12-48_(n)5 | 0.8 | 0.4 | 0.3 | 0.8 |
| DC_12-48-66_n5 | 0.8 | 0.8 | 0.8 | 0.3 |
| DC_12-66_(n)5 | 0.3 | 0.8 | 0.8 | 0.3 |
| DC_12-66_n2-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_13-48-66_n77 | 0.3 | 0.8 | 0.6 | 0.8 |
| DC_13-66_n2-n77 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_13-66_n5-n48 | 0.4 | 0.6 | 0.8 | 0.8 |
| DC_13-66_n5-n77 DC_13-66-66_n5-n77 | 0.5 | 0.6 | 0.6 | 0.8 |
| DC_13-66_n66-n77 | 0.3 | 0.6 | 0.6 | 0.8 |
| DC_14-30-66-n2 | 0.3 | 0.3 | 0.5 | 0.5 |
| DC_14-30-66_n66 | 0.5 | 0.3 | 0.5 | 0.5 |
| DC_14-30-66_n77 DC_14-30-66-66_n77 | 0.6 | 0.3 | 0.6 | 0.8 |
| DC_18-41_n3-n77 | 0.3 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_18-41_n3-n78 | 0.3 | 0.34 / 0.85 | 0.6 | 0.8 |
| DC_19_n1-n77-n79 | 0.3 | 0.6 | 0.8 | 0.5 |
| DC_19_n1-n78-n79 | 0.3 | 0.3 | 0.8 | 0.5 |
| DC_19-21_n1-n77 | 0.3 | 0.4 | 0.3 | 0.8 |
| DC_19-21_n1-n78 | 0.3 | 0.4 | 0.6 | 0.8 |
| DC_19-21_n1-n79 | 0.3 | 0.4 | 0.3 | - |
| DC_19-21-42_n1 | 0.3 | 0.4 | 0.8 | 0.3 |
| DC_19-21-42_n77 | 0.3 | 0.4 | 0.8 | 0.8 |
| DC_19-21-42_n78 | 0.3 | 0.4 | 0.8 | 0.8 |
| DC_19-21-42_n79 | 0.3 | 0.4 | 0.8 | - |
| DC_19-21_n77-n79 | 0.3 | 0.4 | 0.8 | - |
| DC_19-21_n78-n79 | 0.3 | 0.4 | 0.8 | - |
| DC_19-42_n1-n77 | 0.3 | 0.8 | 0.6 | 0.8 |
| DC_19-42_n1-n78 | 0.3 | 0.8 | 0.3 | 0.8 |
| DC_19-42_n1-n79 | 0.3 | 0.8 | 0.3 | - |
| DC_19-42_n77-n79 | 0.3 | 0.8 | 0.8 | - |
| DC_19-42_n78-n79 | 0.3 | 0.8 | 0.8 | - |
| DC_20-28-32_n1 | 0.6 | 0.6 | - | 0.5 |
| DC_20-28-32_n3 | 0.5 | 0.6 | - | 0.5 |
| DC_20-28-38_n1 | 0.6 | 0.6 | 0.5 | 0.5 |
| DC_20-32_n1-n28 | 0.6 | - | 0.3 | 0.7 |
| DC_20-32-38_n1 | 0.3 | - | 0.5 | 0.5 |
| DC_20-38_n3-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| DC_20-41_n1-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| DC_21_n1-n77-n79 | 0.4 | 0.6 | 0.8 | 0.5 |
| DC_21_n1-n78-n79 | 0.4 | 0.6 | 0.8 | 0.5 |
| DC_21-28-42_n77 | 0.4 | 0.5 | 0.8 | 0.8 |
| DC_21-28-42_n78 | 0.4 | 0.5 | 0.8 | 0.8 |
| DC_21-28-42_n79 | 0.4 | 0.5 | 0.8 | - |
| DC_21_n28-n77-n79 | 0.4 | 0.5 | 0.8 | 0.5 |
| DC_21_n28-n78-n79 | 0.4 | 0.5 | 0.8 | - |
| DC_21-42_n1-n77 | 0.4 | 0.8 | 0.6 | 0.8 |
| DC_21-42_n1-n78 | 0.4 | 0.8 | 0.3 | 0.8 |
| DC_21-42_n1-n79 | 0.4 | 0.8 | 0.3 | - |
| DC_21-42_n77-n79 | 0.4 | 0.8 | 0.8 | - |
| DC_21-42_n78-n79 | 0.4 | 0.8 | 0.8 | - |
| DC_28-32-38_n1 | 0.7 | - | 0.5 | 0.5 |
| DC_28-41-42_n78 | 0.5 | 0.3 | 0.8 | 0.8 |
| DC_29-30-66_n2 DC_29-30-66-66_n2 | - | 0.3 | 0.5 | 0.5 |
| DC_29-30-66_n66 | - | 0.3 | 0.5 | 0.5 |
| DC_29-30-66_n77 | - | 0.3 | 0.6 | 0.8 |
| DC_30-66-(n)5 | 0.3 | 0.3 | 0.5 | 0.3 |
| DC_42_n1-n77-n79 | 0.8 | 0.6 | 0.8 | - |
| DC_42_n1-n78-n79 | 0.8 | 0.3 | 0.8 | - |
| DC_42_n3-n28-n77 | 0.8 | 0.6 | 0.8 | 0.8 |
| DC_46-66_n25-n41 | - | 0.5 | 0.5 | 0.41 / 0.92 |
| DC_46-66_n25-n71 | - | 0.5 | 0.5 | 0.3 |
| DC_46-66_n41-n71 | - | 0.5 | 0.41 / 0.92 | 0.6 |
| DC_48-66_n25-n48 | 0.8 | 0.6 | 0.6 | 0.8 |
| DC_66-71_n2-n78 | 0.5 | 0.3 | 0.5 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 3: The values in the table reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 7: Void. NOTE 8: Void. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

6.2B.4.2.3.4	ΔTIB,c for EN-DC five bands

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |  |  |
| DC_1-3-5-7_n77 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-5-7_n78 DC_1-3-5-7-7_n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-5-41_n79 | 0.5 | 0.5 | 0.3 |  | 0.53 / 0.84 | - |  |
| DC_1-3-7_n3-n78 | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.8 |  |
| DC_1-3-7_n7-n78 | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.8 |  |
| DC_1-3-7-8_n28 | 0.5 | 0.5 | 0.6 |  | 0.6 | 0.6 |  |
| DC_1-3-7-8_n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-7_n8-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-7-20_n8 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_1-3-7-20_n28 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_1-3-7-20_n38 | 0.3 | 0.3 | - |  | 0.3 | - |  |
| DC_1-3-7-20_n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_1-3-7_n26-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-7-28_n3 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_1-3-7-28_n5 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_1-3-7-28_n7 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_1-3-7_n28-n38 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_1-3-7-28_n40 | 0.6 | 0.6 | 0.8 |  | 0.6 | 0.9 |  |
| DC_1-3-7-28_n78 | 0.7 | 0.7 | 0.7 |  | 0.6 | 0.8 |  |
| DC_1-3-7_n28-n78 | 0.7 | 0.7 | 0.7 |  | 0.6 | 0.8 |  |
| DC_1-3-7-32_n28 | 0.6 | 0.6 | 0.6 |  | - | 0.6 |  |
| DC_1-3-7-32_n78 | 0.7 | 0.7 | 0.7 |  | - | 0.8 |  |
| DC_1-3-7-38_n28 | 0.6 | 0.6 | - |  | - | 0.5 |  |
| DC_1-3-7-40_n78 | 0.6 | 0.6 | 0.5 |  | 0.35 | 0.85 |  |
| DC_1-3-7_n40-n78 | 0.6 | 0.6 | 0.8 |  | 0.9 | 0.8 |  |
| DC_1-3-7_n75-n78 | 0.7 | 0.7 | 0.7 |  | - | 0.8 |  |
| DC_1-3-8-11_n28 | 0.3 | 0.8 | 0.6 |  | 0.9 | 0.6 |  |
| DC_1-3-8-11_n77 | 0.6 | 0.8 | 0.6 |  | 0.9 | 0.8 |  |
| DC_1-3-8-20_n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-8_n28-n77 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-8-28_n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-8_n28-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-8_n77-n79 | 0.6 | 0.6 | 0.6 |  | 0.8 | 0.5 |  |
| DC_1-3-8-32_n78 | 0.6 | 0.6 | 0.6 |  | - | 0.8 |  |
| DC_1-3-8-40_n78 | 0.6 | 0.6 | 0.6 |  | 0.35 | 0.85 |  |
| DC_1-3-8-42_n77 | 0.6 | 0.6 | 0.6 |  | 0.8 | 0.8 |  |
| DC_1-3-11_n28-n77 | 0.6 | 0.8 | 0.9 |  | 0.6 | 0.8 |  |
| DC_1-3-18_n3-n41 | 0.5 | 0.5 | 0.3 |  | 0.5 | 0.33 / 0.84 |  |
| DC_1-3-18_n3-n77 | 0.6 | 0.6 | 0.3 |  | 0.6 | 0.8 |  |
| DC_1-3-18_n3-n78 | 0.6 | 0.6 | 0.3 |  | 0.6 | 0.8 |  |
| DC_1-3-18_n28-n41 | 0.5 | 0.5 | 0.3 |  | 0.6 | 0.33 / 0.84 |  |
| DC_1-3-18_n28-n77 | 0.3 | 0.3 | 0.3 |  | 0.6 | 0.8 |  |
| DC_1-3-18_n28-n77 | 0.3 | 0.3 | 0.3 |  | 0.6 | 0.8 |  |
| DC_1-3-18_n41-n77 | 0.5 | 0.5 | 0.3 |  | 0.33 / 0.84 | 0.8 |  |
| DC_1-3-18_n41-n78 | 0.5 | 0.5 | 0.3 |  | 0.33 / 0.84 | 0.8 |  |
| DC_1-3-18-42_n77 | 0.6 | 0.6 | 0.3 |  | 0.8 | 0.8 |  |
| DC_1-3-18-42_n78 | 0.6 | 0.6 | 0.3 |  | 0.8 | 0.8 |  |
| DC_1-3-18-42_n79 | 0.6 | 0.6 | 0.3 |  | 0.8 | - |  |
| DC_1-3-19-21_n77 | 0.6 | 0.8 | 0.3 |  | 0.9 | 0.8 |  |
| DC_1-3-19-21_n78 | 0.6 | 0.8 | 0.3 |  | 0.9 | 0.8 |  |
| DC_1-3-19-21_n79 | 0.3 | 0.8 | 0.3 |  | 0.9 | - |  |
| DC_1-3-19-42_n77 | 0.6 | 0.6 | 0.3 |  | 0.8 | 0.8 |  |
| DC_1-3-19-42_n78 | 0.6 | 0.6 | 0.3 |  | 0.8 | 0.8 |  |
| DC_1-3-19-42_n79 | 0.6 | 0.6 | 0.3 |  | 0.8 | - |  |
| DC_1-3-20_n7-n78 | 0.6 | 0.6 | 0.3 |  | 0.6 | 0.8 |  |
| DC_1-3-20_n8-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-20_n28-n75 | 0.3 | 0.3 | 0.6 |  | 0.6 | - |  |
| DC_1-3-20-28_n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-20_n28-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-20-32_n28 | 0.3 | 0.3 | 0.6 |  | - | 0.6 |  |
| DC_1-3-20-32_n78 | 0.6 | 0.6 | 0.6 |  | - | 0.8 |  |
| DC_1-3-20-38_n78 | 0.3 | 0.6 | 0.6 |  | 0.5 | 0.8 |  |
| DC_1-3-20_n38-n78 | 0.3 | 0.6 | 0.6 |  | 0.5 | 0.8 |  |
| DC_1-3-20-40_n78 | 0.5 | 0.5 | 0.3 |  | 0.55 | 0.85 |  |
| DC_1-3-20_n41-n78 | 0.5 | 0.5 | 0.3 |  | 0.5 | 0.8 |  |
| DC_1-3-21-42_n77 | 0.6 | 0.8 | 0.9 |  | 0.8 | 0.6 |  |
| DC_1-3-21-42_n78 | 0.6 | 0.8 | 0.9 |  | 0.8 | 0.6 |  |
| DC_1-3-21-42_n79 | 0.6 | 0.8 | 0.9 |  | 0.8 | - |  |
| DC_1-3-21_n77-n79 | 0.6 | 0.8 | 0.9 |  | 0.8 | - |  |
| DC_1-3-21_n78-n79 | 0.6 | 0.8 | 0.9 |  | 0.8 | - |  |
| DC_1-3-28_n3-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-3-28_n7-n78 | 0.7 | 0.7 | 0.6 |  | 0.7 | 0.8 |  |
| DC_1-3-28-40_n78 | 0.5 | 0.5 | 0.6 |  | 0.5 | 0.8 |  |
| DC_1-3-28_n40-n78 | 0.5 | 0.6 | 0.5 |  | 0.35 | 0.85 |  |
| DC_1-3-28-42_n77 | 0.6 | 0.6 | 0.6 |  | 0.8 | 0.8 |  |
| DC_1-3-28-42_n78 | 0.6 | 0.6 | 0.6 |  | 0.8 | 0.8 |  |
| DC_1-3-28-42_n79 | 0.6 | 0.6 | 0.6 |  | 0.8 | - |  |
| DC_1-3_n28-n77-n79 | 0.6 | 0.6 | 0.6 |  | 0.8 | 0.5 |  |
| DC_1_n3-n28-n77-n79 | 0.6 | 0.6 | 0.6 |  | 0.8 | 0.8 |  |
| DC_1-3_n28-n78-n79 | 0.3 | 0.6 | 0.6 |  | 0.8 | 0.5 |  |
| DC_1-3-38_n28-n78 | 0.5 | 0.6 | 0.3 |  | 0.5 | 0.8 |  |
| DC_1-3-41_n3-n41 | 0.5 | 0.5 | 0.33 / 0.84 |  | 0.5 | 0.33 / 0.84 |  |
| DC_1-3-41_n3-n77 | 0.6 | 0.6 | 0.5 |  | 0.6 | 0.8 |  |
| DC_1-3-41_n3-n78 | 0.6 | 0.6 | 0.5 |  | 0.6 | 0.8 |  |
| DC_1-3-41_n28-n41 | 0.3 | 0.3 | 0.33 / 0.84 |  | 0.6 | 0.33 / 0.84 |  |
| DC_1-3-41_n28-n77 | 0.6 | 0.6 | 0.33 / 0.84 |  | 0.5 | 0.8 |  |
| DC_1-3-41_n28-n78 | 0.5 | 0.6 | 0.33 / 0.84 |  | 0.5 | 0.8 |  |
| DC_1-3-41_n41-n77 | 0.6 | 0.6 | 0.5 |  | 0.5 | 0.8 |  |
| DC_1-3-41_n41-n78 | 0.6 | 0.6 | 0.5 |  | 0.5 | 0.8 |  |
| DC_1-3-41-42_n77 | 0.6 | 0.6 | 0.5 |  | 0.8 | 0.8 |  |
| DC_1-3-41-42_n78 | 0.6 | 0.6 | 0.5 |  | 0.8 | 0.8 |  |
| DC_1-3-41-42_n79 | 0.6 | 0.6 | 0.5 |  | 0.8 | - |  |
| DC_1-3-42_n28-n77 | 0.6 | 0.6 | 0.8 |  | 0.8 | 0.8 |  |
| DC_1-7-8-20 _n28 | 0.5 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_1-7-8-20_n78 | 0.6 | 0.7 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-7-8_n28-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-7-8-32_n78 | 0.7 | 0.7 | 0.6 |  | 0.8 | - |  |
| DC_1-7-8-40_n78 | 0.6 | 0.5 | 0.6 |  | 0.35 | 0.85 |  |
| DC_1-7-20_n3-n38 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.5 |  |
| DC_1-7-20_n3-n78 | 0.6 | 0.7 | 0.4 |  | 0.5 | 0.8 |  |
| DC_1-7-20_n8-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-7-20-28 _n3 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_1-7-20_n28-n78 | 0.6 | 0.7 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-7-20-32_n3 | 0.7 | 0.7 | 0.3 |  | - | 0.7 |  |
| DC_1-7-20-32_n8 | 0.7 | 0.7 | 0.6 |  | - | 0.6 |  |
| DC_1-7-20-32_n28 | 0.5 | 0.6 | 0.6 |  | - | 0.7 |  |
| DC_1-7-20-32_n78 | 0.6 | 0.7 | 0.4 |  | - | 0.8 |  |
| DC_1-7-20-38_n3 | 0.6 | 0.5 | 0.5 |  | 0.5 | 0.6 |  |
| DC_1-7-20-38_n8 | 0.5 | - | 0.6 |  | - | 0.6 |  |
| DC_1-7-20-38_n78 | 0.6 | 0.7 | 0.6 |  | - | 0.8 |  |
| DC_1-7-28_n3-n78 | 0.7 | 0.7 | 0.6 |  | 0.7 | 0.6 |  |
| DC_1-7-28_n7-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-7-28-32_n3 | 0.6 | 0.6 | 0.6 |  | - | 0.6 |  |
| DC_1-7-28_n40-n78 | 0.6 | 0.5 | 0.3 |  | 0.5 | 0.8 |  |
| DC_1-7-38_n3-n78 | 0.6 | - | - |  | 0.6 | 0.8 |  |
| DC_1-8_n3-n28-n77 | 0.6 | 0.6 | 0.8 |  | 0.6 | 0.8 |  |
| DC_1-8_n3-n28-n79 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-8_n3-n77-n79 | 0.6 | 0.6 | 0.8 |  | 0.8 | 0.8 |  |
| DC_1-8-11_n3-n28 | 0.3 | 0.6 | 0.8 |  | 0.9 | 0.6 |  |
| DC_1-8-11_n3-n77 | 0.6 | 0.6 | 0.8 |  | 0.9 | 0.8 |  |
| DC_1-8-11_n3-n79 | 0.3 | 0.3 | 0.8 |  | 0.9 | 0.8 |  |
| DC_1-8-11_n28-n77 | 0.6 | 0.6 | 0.4 |  | 0.6 | 0.8 |  |
| DC_1-8-11_n77-n79 | 0.6 | 0.6 | 0.4 |  | 0.8 | 0.5 |  |
| DC_1-8-20-28_n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_1-8_n28-n77-n79 | 0.6 | 0.6 | 0.6 |  | 0.8 | 0.8 |  |
| DC_1-8-42_n3-n28 | 0.3 | 0.6 | 0.8 |  | 0.6 | 0.8 |  |
| DC_1-8-42_n3-n77 | 0.6 | 0.6 | 0.8 |  | 0.8 | 0.8 |  |
| DC_1-8-42_n28-n77 | 0.6 | 0.6 | 0.8 |  | 0.8 | 0.8 |  |
| DC_1-11_n3-n28-n77 | 0.6 | 0.8 | 0.9 |  | 0.6 | 0.8 |  |
| DC_1-11_n3-n77-n79 | 0.6 | 0.8 | 0.9 |  | 0.8 | 0.8 |  |
| DC_1-18-41_n3-n77 | 0.6 | 0.3 | 0.33 / 0.84 |  | 0.6 | 0.8 |  |
| DC_1-18-41_n3-n78 | 0.6 | 0.3 | 0.33 / 0.84 |  | 0.6 | 0.8 |  |
| DC_1-19-21-42_n77 | 0.3 | 0.3 | 0.4 |  | 0.8 | 0.8 |  |
| DC_1-19-21-42_n78 | 0.3 | 0.3 | 0.4 |  | 0.8 | 0.8 |  |
| DC_1-19-21-42_n79 | 0.3 | 0.3 | 0.4 |  | 0.8 | - |  |
| DC_1-19-42_n77-n79 | 0.6 | 0.3 | 0.8 |  | 0.8 | - |  |
| DC_1-19-42_n78-n79 | 0.3 | 0.3 | 0.8 |  | 0.8 | - |  |
| DC_1-20-28-32_n3 | 0.5 | 0.6 | 0.6 |  | - | 0.5 |  |
| DC_1-20-38_n3-n78 | 0.5 | 0.6 | 0.5 |  | 0.6 | 0.8 |  |
| DC_1-21-28-42_n77 | 0.6 | 0.4 | 0.6 |  | 0.8 | 0.8 |  |
| DC_1-21-28-42_n78 | 0.3 | 0.4 | 0.6 |  | 0.8 | 0.8 |  |
| DC_1-21-28-42_n79 | 0.3 | 0.4 | 0.6 |  | 0.8 | - |  |
| DC_1-21_n28-n77-n79 | 0.6 | 0.4 | 0.6 |  | 0.8 | 0.5 |  |
| DC_1-21_n28-n78-n79 | 0.6 | 0.4 | 0.6 |  | 0.8 | 0.5 |  |
| DC_1-21-42_n77-n79 | 0.6 | 0.4 | 0.8 |  | 0.8 | - |  |
| DC_1-42_n3-n28-n77 | 0.6 | 0.8 | 0.8 |  | 0.8 | 0.8 |  |
| DC_1-21-42_n78-n79 | 0.3 | 0.4 | 0.8 |  | 0.8 | - |  |
| DC_2-5-7-66_n2 | 0.5 | 0.3 | 0.5 |  | 0.5 | 0.5 |  |
| DC_2-5-7-66_n7 DC_2-5-7-66-66_n7 | 0.5 | 0.3 | 0.5 |  | 0.5 | 0.5 |  |
| DC_2-5-7-66_n66 | 0.5 | 0.3 | 0.5 |  | 0.5 | 0.5 |  |
| DC_2-5-7-66_n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_2-5-66_n2-n77 DC_2-5-66-66_n2-n77 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_2-5-66_n5-n77 DC_2-5-66-66_n5-n77 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_2-5-30-66_n2 | 0.5 | 0.3 | 0.3 |  | 0.5 | 0.5 |  |
| DC_2-5-30-66_n66 | 0.5 | 0.3 | 0.3 |  | 0.5 | 0.5 |  |
| DC_2-5-30-66_n77 | 0.6 | 0.6 | 0.3 |  | 0.6 | 0.8 |  |
| DC_2-5-66_n66-n77 | 0.5 | 0.3 | 0.5 |  | 0.5 | 0.8 |  |
| DC_2-7-12-66_n2 | 0.5 | 0.5 | 0.8 |  | 0.5 | 0.5 |  |
| DC_2-7-12-66_n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_2-7-13_n25-n66 | 0.5 | 0.5 | 0.3 |  | 0.5 | 0.5 |  |
| DC_2-7-13-66_n66 | 0.5 | 0.5 | 0.3 |  | 0.5 | 0.5 |  |
| DC_2-7-28-66_n7 | 0.5 | 0.5 | 0.6 |  | 0.5 | 0.5 |  |
| DC_2-7-28-66_n66 | 0.5 | 0.5 | 0.6 |  | 0.5 | 0.5 |  |
| DC_2-7-29-66_n78 DC_2-7-7-29-66_n78 | 0.6 | 0.5 | - |  | 0.6 | 0.8 |  |
| DC_2-7-66_n25-n66 | 0.5 | 0.5 | 0.5 |  | 0.5 | 0.5 |  |
| DC_2-7-66_n66-n77 | 0.6 | 0.5 | 0.6 |  | 0.6 | 0.8 |  |
| DC_2-7-66_n66-n78 DC_2-7-7-66_n66-n78 | 0.6 | 0.5 | 0.6 |  | 0.6 | 0.8 |  |
| DC_2-7-66-71_n2 | 0.5 | 0.5 | 0.5 |  | 0.3 | 0.5 |  |
| DC_2-7-66-71_n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_2-12-30-66_n2 | 0.5 | 0.8 | 0.3 |  | 0.5 | 0.5 |  |
| DC_2-12-30-66_n66 | 0.5 | 0.8 | 0.3 |  | 0.5 | 0.5 |  |
| DC_2-12-30-66_n77 | 0.6 | 0.8 | 0.3 |  | 0.6 | 0.8 |  |
| DC_2-13-66_n2-n77 DC_2-13-66-66_n2-n77 | 0.6 | 0.3 | 0.6 |  | 0.6 | 0.8 |  |
| DC_2-13-66_n5-n77 DC_2-2-13-66_n5-n77 DC_2-13-66-66_n5-n77 | 0.6 | 0.5 | 0.6 |  | 0.6 | 0.8 |  |
| DC_2-13-66_n66-n77 DC_2-2-13-66_n66-n77 | 0.6 | 0.3 | 0.6 |  | 0.6 | 0.8 |  |
| DC_2-14-30-66_n2 | 0.5 | 0.3 | 0.3 |  | 0.5 | 0.5 |  |
| DC_2-14-30-66_n66 | 0.5 | 0.3 | 0.3 |  | 0.5 | 0.5 |  |
| DC_2-14-30-66_n77 | 0.6 | 0.6 | 0.3 |  | 0.6 | 0.8 |  |
| DC_2-29-30-66_n2 | 0.5 | - | 0.3 |  | 0.5 | 0.5 |  |
| DC_2-29-30-66_n66 | 0.5 | - | 0.3 |  | 0.5 | 0.5 |  |
| DC_2-29-30-66_n77 | 0.6 | - | 0.3 |  | 0.6 | 0.8 |  |
| DC_2-30-66-(n)5 | 0.5 | 0.3 | 0.3 |  | 0.5 | 0.3 |  |
| DC_2-46-66_n41-n71 | 0.5 | - | 0.5 |  | 0.41 / 0.92 | 0.6 |  |
| DC_3-7-8_n1-n40 | 0.5 | 0.8 | 0.6 |  | 0.6 | 0.9 |  |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78 DC_3-7-7-8_n1-n78 DC_3-3-7-7-8_n1-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_3-7_n1-n8-n78 DC_3-3-7_n1-n8-n78 DC_3-7-7_n1-n8-n78 DC_3-3-7-7_n1-n8-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_3-7-8-20_n1 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_3-7-8_n28-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_3-7-8-32_n1 | 0.6 | 0.7 | 0.6 |  | - | 0.7 |  |
| DC_3-7-8-32_n78 | 0.6 | 0.6 | 0.6 |  | - | 0.8 |  |
| DC_3-7-8-40_n78 | 0.6 | 0.5 | 0.6 |  | 0.55 | 0.85 |  |
| DC_3-7-8_n40-n78 | 0.6 | 0.5 | 0.6 |  | 0.55 | 0.85 |  |
| DC_3-7-20_n1-n75 | 0.7 | 0.7 | 0.3 |  | 0.7 | - |  |
| DC_3-7-20_n1-n78 | 0.6 | 0.7 | 0.6 |  | 0.6 | 0.8 |  |
| DC_3-7-20_n8-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_3-7-20-28_n1 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.6 |  |
| DC_3-7-20_n28-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_3-7-20-32_n1 | 0.7 | 0.7 | 0.3 |  | - | 0.7 |  |
| DC_3-7-20-32_n78 | 0.6 | 0.6 | 0.3 |  | - | 0.8 |  |
| DC_3-7-28_n1-n40 | 0.6 | 0.8 | 0.6 |  | 0.6 | 0.9 |  |
| DC_3-7-28_n1-n78 | 0.7 | 0.7 | 0.6 |  | 0.7 | 0.6 |  |
| DC_3-7-28_n3-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_3-7-28_n7-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_3-7-28_n40-n78 | 0.6 | 0.5 | 0.3 |  | 0.5 | 0.8 |  |
| DC_3-7-32_n1-n78 | 0.3 | 0.3 | - |  | 0.3 | 0.5 |  |
| DC_3-7-40_n1-n78 | 0.6 | 0.5 | 0.35 |  | 0.6 | 0.85 |  |
| DC_3-8-11_n28-n77 | 0.8 | 0.6 | 0.9 |  | 0.6 | 0.8 |  |
| DC_3-8-20-28_n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_3-8-40_n1-n78 | 0.6 | 0.6 | 0.35 |  | 0.6 | 0.85 |  |
| DC_3-8-41_n1-n78 DC_3-3-8-41_n1-n78 | 0.6 | 0.6 | 0.6 |  | 0.6 | 0.8 |  |
| DC_3-19-21-42_n77 | 0.8 | 0.3 | 0.9 |  | 0.8 | 0.8 |  |
| DC_3-19-21-42_n78 | 0.8 | 0.3 | 0.9 |  | 0.8 | 0.8 |  |
| DC_3-19-21-42_n79 | 0.8 | 0.3 | 0.9 |  | 0.8 | - |  |
| DC_3-19-42_n1-n77 | 0.6 | 0.3 | 0.8 |  | 0.6 | 0.8 |  |
| DC_3-19-42_n1-n78 | 0.6 | 0.3 | 0.8 |  | 0.6 | 0.8 |  |
| DC_3-19-42_n1-n79 | 0.6 | 0.3 | 0.8 |  | 0.6 | - |  |
| DC_3-20-32_n1-n28 | 0.3 | 0.6 | - |  | 0.3 | 0.6 |  |
| DC_3-20-41_n1-n78 DC_3-3-20-41_n1-n78 | 0.5 | 0.3 | 0.5 |  | 0.5 | 0.8 |  |
| DC_3-21_n1-n77-n79 | 0.8 | 0.9 | 0.6 |  | 0.8 | 0.5 |  |
| DC_3-21_n1-n78-n79 | 0.8 | 0.9 | 0.6 |  | 0.8 | 0.5 |  |
| DC_3-21_n28-n77-n79 | 0.8 | 0.9 | 0.5 |  | 0.8 | 0.5 |  |
| DC_3-21_n28-n78-n79 | 0.8 | 0.9 | 0.5 |  | 0.8 | 0.5 |  |
| DC_3-21-42_n1-n77 | 0.8 | 0.9 | 0.8 |  | 0.6 | 0.6 |  |
| DC_3-21-42_n1-n78 | 0.8 | 0.9 | 0.8 |  | 0.6 | 0.6 |  |
| DC_3-21-42_n1-n79 | 0.8 | 0.9 | 0.8 |  | 0.6 | - |  |
| DC_3-28-41-42_n78 | 1.0 | 0.5 | 0.33 / 0.84 |  | 0.8 | 0.8 |  |
| DC_7-8-20-32_n1 | 0.7 | 0.6 | 0.7 |  | - | 0.5 |  |
| DC_7-8-40_n1-n78 | 0.5 | 0.6 | 0.35 |  | 0.6 | 0.85 |  |
| DC_7-20-28-32_n1 | 0.7 | 0.6 | 0.6 |  | - | 0.7 |  |
| DC_7-20-28-32_n3 | 0.7 | 0.6 | 0.5 |  | - | 0.7 |  |
| DC_7-20-32-38_n1 | - | 0.3 | - |  | - | 0.7 |  |
| DC_7-20-38_n3-n78 | 0.5 | 0.6 | 0.5 |  | 0.5 | 0.8 |  |
| DC_8_n3-n28-n77-n79 | 0.2 | 0.2 | 0.2 |  | 0.5 | 0.5 |  |
| DC_8-11_n3-n28-n77 | 0.6 | 0.8 | 0.9 |  | 0.6 | 0.8 |  |
| DC_8-11_n3-n77-n79 | 0.6 | 0.8 | 0.9 |  | 0.8 | 0.8 |  |
| DC_8-42_n3-n28-n77 | 0.6 | 0.8 | 0.6 |  | 0.8 | 0.8 |  |
| DC_19-21-42_n1-n77 | 0.3 | 0.4 | 0.8 |  | 0.3 | 0.8 |  |
| DC_19-21-42_n1-n78 | 0.3 | 0.4 | 0.8 |  | 0.3 | 0.8 |  |
| DC_19-21-42_n1-n79 | 0.3 | 0.4 | 0.8 |  | 0.3 | - |  |
| DC_19-21-42_n77-n79 | 0.3 | 0.4 | 0.8 |  | 0.8 | - |  |
| DC_19-21-42_n78-n79 | 0.3 | 0.4 | 0.8 |  | 0.8 | - |  |
| DC_19-42_n1-n77-n79 | 0.3 | 0.8 | 0.6 |  | 0.8 | 0.5 |  |
| DC_19-42_n1-n78-n79 | 0.3 | 0.8 | 0.3 |  | 0.8 | 0.5 |  |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |  |  |

6.2B.4.2.3.5	ΔTIB,c for EN-DC six bands

Table 6.2B.4.2.3.5-1: ΔTIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration |  |  | ΔTIB,c for E-UTRA band / NR band (dB)3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component band in order of bands in configuration4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-3-7-8_n28-n78 |  |  | 0.6 |  | 0.6 |  | 0.6 |  |  | 0.6 |  | 0.6 |  | 0.8 |  |  |
| DC_1-3-7-8-32_n78 |  |  | 0.6 |  | 0.6 |  | 0.6 |  |  | 0.6 |  | - |  | 0.8 |  |  |
| DC_1-3-7-8-40_n78 |  |  | 0.6 |  | 0.6 |  | 0.5 |  |  | 0.6 |  | 0.31 |  | 0.81 |  |  |
| DC_1-3-7-20_n8-n78 |  |  | 0.6 |  | 0.6 |  | 0.6 |  |  | 0.6 |  | 0.6 |  | 0.8 |  |  |
| DC_1-3-7-20_n28-n78 |  |  | 0.7 |  | 0.7 |  | 0.7 |  |  | 0.6 |  | 0.6 |  | 0.8 |  |  |
| DC_1-3-7-20-32_n78 |  |  | 0.7 |  | 0.7 |  | 0.7 |  |  | 0.4 |  | - |  | 0.8 |  |  |
| DC_1-3-7-20_n38-n78 |  |  | 0.6 |  | 0.6 |  | - |  |  | 0.6 |  | - |  | 0.8 |  |  |
| DC_1-3-7-28_n3-n78 |  |  | 0.7 |  | 0.7 |  | 0.7 |  |  | 0.6 |  | 0.7 |  | 0.8 |  |  |
| DC_1-3-7-28_n7-n78 |  |  | 0.7 |  | 0.7 |  | 0.7 |  |  | 0.6 |  | 0.7 |  | 0.8 |  |  |
| DC_1-3-7-28_n38-n78 |  | 0.7 | 0.7 | 0.7 |  | 0.6 | 0.7 | 0.8 |  |
| DC_1-3-7-28_n40-n78 |  | 0.6 | 0.6 | 0.8 |  | 0.3 | 0.9 | 0.8 |  |
| DC_1-3-8-11_n28-n77 |  |  | 0.6 |  | 0.8 |  | 0.6 |  |  | 0.9 |  | 0.6 |  | 0.8 |  |  |
| DC_1-3-8-20-28_n78 |  |  | 0.3 |  | 0.3 |  | 0.6 |  |  | 0.6 |  | 0.6 |  | 0.8 |  |  |
| DC_1-7-20-28-32_n3 |  |  | 0.6 |  | 0.6 |  | 0.6 |  |  | 0.6 |  | - |  | 0.6 |  |  |
| DC_1-7-20-38_n3-n78 |  |  | 0.6 |  | 0.7 |  | 0.6 |  |  | 0.5 |  | 0.6 |  | 0.8 |  |  |
| DC_1-8_n3-n28-n77-n79 |  |  | 0.6 |  | 0.6 |  | 0.8 |  |  | 0.6 |  | 0.8 |  | 0.8 |  |  |
| DC_1-8-11_n3-n28-n77 |  |  | 0.6 |  | 0.6 |  | 0.8 |  |  | 0.9 |  | 0.6 |  | - |  |  |
| DC_1-8-42_n3-n28-n77 |  |  | 0.6 |  | 0.6 |  | 0.8 |  |  | 0.8 |  | - |  | 0.8 |  |  |
| DC_3-7-8-40_n1-n78 |  |  | 0.6 |  | 0.5 |  | 0.6 |  |  | 0.32 |  | 0.6 |  | 0.82 |  |  |
| DC_7-8-20-32-38_n1 |  |  | - |  | 0.6 |  | 0.6 |  |  | - |  | - |  | 0.7 |  |  |
| DC_7-20-28-32-38_n1 |  |  | - |  | 0.6 |  | 0.6 |  |  | - |  | - |  | 0.7 |  |  |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 3: “-” denotes ΔTIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<Unchanged parts skipped>>

7.3B.2.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-0: MSD test points for Pcell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A-(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
| DC_1A-3A_n71A DC_1A-3A_n71B | 1 | 1960 | 5 | 25 | 2150 | 5 | IMD4 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_1A_n3A-n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
| DC_1A_n3A-n41A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 |
| DC_1A_n3A-n75A | n75 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
| DC_1A_n3A-n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n79 | 4950 | 40 | 216 | 4950 | 4.7 | IMD5 |
| DC_1A-7A_n28A DC_1A-7C_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 7 | 2533 | 10 | 50 | 2653 | 30.0 | IMD2 |
| DC_1A-7A_n40A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_1A_n8A-n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
| DC_1A_n8A-n77(2A) | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | 1.5 | IMD5 |
| DC_1A_n8A-n77A | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
| DC_1A_n8A-n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3960 | 10 | 50 | 3960 | 8.8 | IMD3 |
| DC_1A_n8A-n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
| DC_1A_n8A-n77(2A) | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | 3.3 | IMD5 |
| DC_1A-8A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A_n8A-n77(2A) | 8 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-3A_n77A DC_1A-3A_n77(2A) DC_1A-3A_n77(3A) DC_1A-3C_n77A DC_1A-3A_n77C DC_1A-3C_n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_1A-3A_n78A DC_1A-3C_n78A DC_1A-3A_n78C DC_1A-3A_n78(2A) DC_1A-3C_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 | IMD2 |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | IMD5 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| DC_1A_n3A-n77A DC_1A_n3A-n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n77 | 3360 | 10 | 50 | 3360 | 11.2 | IMD4 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
| DC_1A_n3A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 27.9 | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
| DC_1A-5A_n77A DC_1A-5A_n77(2A) | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n77 | 3405 | 10 | 50 | 3405 | N/A | N/A |
| DC_1A-5A_n78A DC_1A-5A_n78C | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |
| DC_1A-7A_n77A DC_1A-7A_n77(2A) DC_1A-7A-7A_n77A DC_1A-7A-7A_n77(2A) | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD44 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A-7A_n78A DC_1A-7C_n78A DC_1A-7A_n78(2A) DC_1A-7C_n78(2A) DC_1A-7A_n78C DC_1A-1A-7A_n78A DC_1A-7A-7A_n78C | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A_n7A-n78A DC_1A_n7B-n78A DC_1A_n7A-n78(2A) | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |
| DC_1A-5A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 8.9 | IMD4 |
|  | n79 | 4907.5 | 40 | 216 | 4907.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 5 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A-8A_n28A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 3.3 | IMD5 |
| DC_1A-8A_n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 8.0 | IMD4 |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
|  | 1 | 1945 | 5 | 25 | 2135 | 5.3 | IMD5 |
|  | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
| DC_1A-8A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 3.3 | IMD5 |
| DC_1A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) | n77 | 3960 | 10 | 50 | 3960 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 14.4 | IMD3 |
| DC_1A-8A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n79 | 4815 | 40 | 216 | 4815 | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 15.8 | IMD3 |
| DC_1A-8A_n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n79 | 4845 | 40 | 216 | 4845 | N/A | N/A |
|  | 1 | 1955 | 5 | 25 | 2145 | 8.2 | IMD4 |
| DC_1A_n8A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | 8.0 | IMD4 |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
| DC_1A_n8A-n78A | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3745 | 10 | 50 | 3745 | 14.9 | IMD3 |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n8 | 895 | 5 | 25 | 940 | 3.3 | IMD5 |
|  | n78 | 3380 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-11A_n3A | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 11 | 1432 | 5 | 25 | 1480 | 15.2 | IMD3 |
| DC_1A-11A_n28A | 11 | 1440 | 5 | 25 | 1488 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | N/A | N/A |
|  | 1 | 1960 | 5 | 25 | 2150 | 28.3 | IMD21 |
| DC_1A-11A_n41A | 11 | 1442 | 5 | 25 | 1490 | N/A | N/A |
|  | n41 | 2520 | 10 | 50 | 2520 | N/A | N/A |
|  | 1 | 1966 | 5 | 25 | 2156 | 10.2 | IMD4 |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
|  | 11 | 1442 | 5 | 25 | 1490 | 10.6 | IMD4 |
| DC_1A-11A_n77A DC_1A-11A_n77(2A) DC_1A-11A_n77(3A) | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | IMD2 |
|  | n77 | 3441 | 10 | 50 | 3441 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | IMD2 |
|  | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | N/A |
| DC_1A-11A_n78A DC_1A-11A_n78(2A) | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | IMD2 |
|  | n78 | 3441 | 10 | 50 | 3441 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | IMD2 |
|  | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |
|  | n78 | 3578 | 10 | 50 | 3578 | N/A | N/A |
| DC_1A-11A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 11 | 1435 | 5 | 25 | 1483 | 10.2 | IMD4 |
|  | n79 | 4427 | 40 | 216 | 4427 | N/A | N/A |
|  | 1 | 1928 | 5 | 25 | 2118 | 15.6 | IMD3 |
|  | 11 | 1431 | 5 | 25 | 1479 | N/A | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
| DC_1A-18A_n77A DC_1A-18A_n77(2A) | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| DC_1A-18A_n78A DC_1A-18A_n78(2A) | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 819 | 5 | 25 | 864 | N/A | N/A |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | N/A |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | IMD3 |
|  | n79 | 4737.5 | 40 | 216 | 4737.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | IMD4 |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | IMD4 |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | N/A |
|  | n79 | 4592.5 | 40 | 216 | 4592.5 | N/A | N/A |
| DC_1A-19A_n77A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 1 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | 19 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | IMD5 |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A_n28A-n41A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | n41 | 2653 | 10 | 50 | 2653 | 30.1 | IMD2 |
|  | 1 | 1923 | 5 | 25 | 2113 | N/A | N/A |
|  | n28 | 707 | 5 | 25 | 762 | 29.3 | IMD2 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 730 | 10 | 50 | 785 | 4.5 | IMD5 |
|  | n41 | 2510 | 10 | 50 | 2510 | N/A | N/A |
| DC_1A-20A_n7A | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
| DC_1A-20A_n8A | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 11.5 | IMD4 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
| DC_1A-20A_n38A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 20 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n38 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | IMD3 |
| DC_1A-20A_n78(2A) | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-21A_n28A10 | 1 | 1975.3 | 5 | 25 | 2165.3 | 16.1 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
|  | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 | IMD5 |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |
| DC_1A-21A_n79A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  |  | n79 |  | N/A |  | N/A |  | N/A | N/A |  | N/A |  | N/A |  |
| DC_1A_n26A-n78A |  | n1 |  | 1950 |  | 5 |  | 25 | 2140 |  | N/A |  | N/A |  |
|  |  | n26 |  | 830 |  | 5 |  | 25 | 875 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3610 |  | 10 |  | 50 | 3610 |  | 15.7 |  | IMD3 |  |
|  |  | 1 |  | 1975 |  | 5 |  | 25 | 2165 |  | N/A |  | N/A |  |
|  |  | n26 |  | 840 |  | 5 |  | 25 | 885 |  | 3.1 |  | IMD5 |  |
|  |  | n78 |  | 3405 |  | 10 |  | 50 | 3405 |  | N/A |  | N/A |  |
| DC_1A-28A_n3A |  | 1 |  | 1949 |  | 5 |  | 25 | 2139 |  | 11.0 |  | IMD4 |  |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
| DC_1A-28A_n7A DC_1A-1A-28A_n7A DC_1A-28A_n7B DC_1A-1A-28A_n7B | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 28 | 730 | 10 | 50 | 785 | 4.5 | IMD5 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
| DC_1A-28A_n40A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 8.9 | IMD4 |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 | IMD5 |
|  | n77 | 3352 | 10 | 50 | 3352 | N/A | N/A |
| DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | N/A |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 | IMD3 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | IMD4 |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | IMD5 |
|  | 28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | N/A |
| DC_1A_n28A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n40 | 2374 | 5 | 25 | 2374 | 10.1 | IMD4 |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 713 | 5 | 25 | 768 | 8.6 | IMD4 |
|  | n40 | 2314 | 5 | 25 | 2314 | N/A | N/A |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 4.2 | IMD5 |
| DC_1A_n28A-n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | 15.2 | IMD39 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n79 | 4630 | 40 | 216 | 4630 | 14.9 | IMD34 |
| DC_1A-32A_n3A | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
| DC_1A-32A_n78A DC_1A-32A_n78C DC_1A-32A_n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 31.8 | IMD2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 0 | IMD5 |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A-38A_n78A DC_1A-38A_n78(2A) | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 38 | 2590 | 5 | 25 | 2590 | 12.7 | IMD4 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
| DC_1A_n38A-n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n38 | 2590 | 10 | 50 | 2590 | 12.7 | IMD4 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
| DC_1A-40A_n78A DC_1A-40C_n78A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 40 | 2340 | 5 | 25 | 2340 | 10.6 | IMD4 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.1 | IMD4 |
|  | 40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n78 | 3430 | 10 | 50 | 3430 | N/A | N/A |
| DC_1A_n40A-n78A DC_1A_n40A-n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 10.6 | IMD4 |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_1A-41A_n3A DC_1A-41C_n3A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 41 | 2653 | 10 | 50 | 2653 | 30 | IMD2 |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
| DC_1A-41A_n77A DC_1A-41C_n77A DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 41 | 2510 | 5 | 25 | 2510 | N/A | IMD4 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 11.0 | N/A |
|  | 41 | 2510 | 5 | 25 | 2510 | N/A | IMD5 |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A |  |
| DC_1A-41A_n78A DC_1A-41C_n78A DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 41 | 2515 | 5 | 25 | 2515 | 12 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
| DC_1A_n41A-n77A DC_1A_n41A-n77(2A) DC_1A_n41A-n78A DC_1A_n41A-n78(2A) | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n41 | 2515 | 10 | 50 | 2515 | 11.5 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2650 | 10 | 25 | 2650 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | 19.6 | IMD3 |
| DC_1A-41A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 41 | 2530 | 5 | 25 | 2530 | 29.4 | IMD2 |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A | N/A |
| DC_1A-42A_n3A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | 42 | 3425 | 5 | 25 | 3425 | 13.0 | IMD4 |
| DC_1A-42A_n28A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | 42 | 3416 | 5 | 25 | 3416 | 15.7 | IMD3 |
| DC_1A-42A_n28A | 42 | 3580 | 5 | 25 | 3580 | N/A | N/A |
|  | n28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | 1 | 1944 | 5 | 25 | 2134 | 15.7 | IMD3 |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | IMD5 |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | IMD3 |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
| DC_1A_SUL_n77A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |
| DC_1A_SUL_n77A-n80A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n75 | - | - | - | 1470 | 30.4 | IMD2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5 |
| DC_1A_SUL_n78A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |
|  | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_2A_n2A-n66A | 2 | 1875 | 5 | 25 | 1955 | N/A | N/A |
|  | n2 | 1895 | 5 | 25 | 1975 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_2A_n2A-n77A | 2 | 1875 | 5 | 25 | 1955 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  |  |  |  |  |  | 28.712 |  |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD44 |
|  |  |  |  |  |  | 10.712 |  |
|  | n77 | 3735 | 10 | 50 | 3735 | N/A | N/A |
| DC_2A_n2A-n78A | 2 | 1852.5 | 5 | 25 | 1932.5 | N/A | N/A |
|  | n2 | 1862.5 | 5 | 25 | 1942.5 | 26 | IMD24 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_2A-4A_n28A | 2 | 1880 | 5 | 25 | 1960 | 11.0 | IMD4 |
|  | 4 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | N/A |
| DC_2A-4A_n41A | 2 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 |
|  | 4 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_2A-5A_n12A8 | 2 | 1900 | 5 | 25 | 1980 | 5.9 | IMD5 |
|  | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |
| DC_2A-5A_n30A | 2 | 1870 | 5 | 25 | 1959 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | 9.7 | IMD4 |
|  | n30 | 2310 | 10 | 50 | 2355 | N/A | N/A |
| DC_2A-5A_n48A DC_2A-5A_n48B | 2 | 1882 | 5 | 25 | 1962 | 15.6 | IMD3 ¦ fn48-2*fB5¦ |
|  | 5 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n48 | 3640 | 5 | 25 | 3640 | N/A | N/A |
| DC_2A-5A_n71A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n71 | 686.5 | 5 | 25 | 640.5 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | 4.2 | IMD5 |
| DC_2A_n5A-n77A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.0 | IMD3 |
| DC_2A_n5A-n77A11 | 2 | 1907 | 5 | 25 | 1987 | N/A | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 3.8 | IMD5 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
| DC_2A-5A_n77A11 | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
| DC_2A-5A_n77C11 DC_2A-5A_n77(2A)11 DC_2A-2A-5A_n77A11 | 5 | 842.5 | 5 | 25 | 887.5 | 3.8 | IMD5 |
| DC_2A-2A-5A_n77C11 | n77 | 3305 | 5 | 25 | 3305 | N/A | N/A |
|  | 2 | 1907 | 5 | 25 | 1987 | 16.5 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n77 | 3680 | 5 | 25 | 3680 | N/A | N/A |
| DC_2A-5A_n78A DC_2A-5A_n78(2A) | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 5 | 842.5 | 5 | 25 | 887.5 | 3.8 | IMD5 |
|  | n78 | 3305 | 5 | 25 | 3305 | N/A | N/A |
|  | 2 | 1907 | 5 | 25 | 1987 | 16.5 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n78 | 3680 | 5 | 25 | 3680 | N/A | N/A |
| DC_2A-7A_n5A DC_2A-7C_n5A DC_2A-7A-7A_n5A | 2 | 1855 | 10 | 50 | 1935 | N/A | N/A |
|  | 7 | 2575 | 10 | 50 | 2685 | 30.0 | IMD2 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_2A-7A_n28A DC_2A-7C_n28A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 7 | 1720 | 5 | 25 | 2120 | 29.0 | IMD2 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | N/A |
| DC_2A-7A_n77A DC_2A-7C_n77A DC_2A-7A-7A_n77A DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) DC_2A-7A-7A_n77(2A) | 2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3525 | 10 | 50 | 3475 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | 3.4 | IMD5 |
|  | n77 | 4120 | 10 | 50 | 4120 | N/A | N/A |
| DC_2A-7A_n78A DC_2A-2A-7A_n78A DC_2A-7C_n78A DC_2A-7A-7A_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) DC_2A-7A-7A_n78(2A) | 2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3525 | 10 | 50 | 3475 | N/A | N/A |
| DC_2A_n7A-n78A, DC_2A_n7(2A)-n78A DC_2A_n7A-n78(2A) DC_2A_n7(2A)-n78(2A) | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3775 | 10 | 50 | 3775 | 4.2 | IMD5 |
| DC_2-8_n2 | 2 | 1860 | 5 | 25 | 1940 | 4 | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
| DC_2A-12A_n5A DC_2A-2A-12A_n5A | 2 | 1900 | 5 | 25 | 1980 | 5.9 | IMD5 |
|  | 12 | 705 | 5 | 25 | 735 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |
| DC_2A-12A_n7A DC_2A-12A_n7(2A) | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 12 | 701.5 | 5 | 25 | 731.5 | 4.5 | IMD5 |
|  | n7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |
| DC_2A-12A_n41A DC_2A-2A-12A_n41A | 2 | 1872 | 5 | 25 | 1952 | 26 | IMD2 |
|  | 12 | 708 | 5 | 50 | 738 | N/A | N/A |
|  | n41 | 2660 | 10 | 50 | 2660 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 12 | 708 | 5 | 50 | 738 | 28.7 | IMD24 |
|  | n41 | 2638 | 10 | 50 | 2638 | N/A | N/A |
| DC_2A-12A_n66A | 2 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | 12 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_2A-12A_n77A DC_2A-12A_n77(2A) | 2 | 1880 | 5 | 25 | 1960 | 16.5 | IMD39,11 |
| DC_2A-2A-12A_n77A | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | N/A |
| DC_2A-12A_n78A DC_2A-2A-12A_n78A DC_2A-12A_n78(2A) | 2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
| DC_2A-13A_n48A DC_2A-13A_n48B | 2 | 1903.5 | 5 | 25 | 1983.5 | 15.6 | IMD3 ¦ fn48-2*fB13¦ |
|  | 13 | 784.5 | 5 | 25 | 753.5 | N/A | N/A |
|  | n48 | 3552.5 | 5 | 25 | 3552.5 | N/A | N/A |
| DC_2A-13A_n66A DC_2A-2A-13A_n66A | 2 | 1860 | 5 | 25 | 1940 | 6.2 | IMD4 |
|  | 13 | 780 | 10 | 50 | 749 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
| DC_2A-13A_n77A | 2 | 1864 | 5 | 25 | 1944 | 16.0 | IMD3 |
| DC_2A-13A_n77C | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
| DC_2A-2A-13A_n77A DC_2A-2A-13A_n77C | n77 | 3510 | 5 | 25 | 3510 | N/A | N/A |
| DC_2A-14A_n77A DC_2A-14A_n77(2A) | 2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
| DC_2A-2A-14A_n77A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_2_n25-n66 | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | 4 | IMD5 |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
| DC_2A_n38A-n71A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n38 | 2586 | 10 | 50 | 2586 | 29.2 | IMD2 |
|  | n71 | 686 | 5 | 25 | 640 | N/A | N/A |
| DC_2A_n38A-n78A | 2 | 1870 | 5 | 25 | 1950 | N/A | N/A |
|  | n38 | 2610 | 10 | 50 | 2610 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | 14.8 | IMD3 |
| DC_2A-14A_n66A | 2 | 1874 | 5 | 25 | 1954 | 7.2 | IMD4 |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_2A-28A_n66A | 2 | 1900 | 5 | 25 | 1980 | 11 | IMD4 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
| DC_2A-30A_n77A DC_2A-30A_n77(2A) | 2 | 1906 | 5 | 25 | 1986 | 8.6 | IMD411 |
| DC_2A-2A-30A_n77A | 30 | 2312 | 5 | 25 | 2357 | N/A | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 2 | 1905 | 5 | 25 | 1985 | N/A | N/A |
|  | 30 | 2309 | 5 | 25 | 2354 | 10.6 | IMD411 |
|  | n77 | 3361 | 10 | 50 | 3361 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | 30 | 2309 | 5 | 25 | 2354 | 3.4 | IMD5 |
|  | n77 | 3967 | 10 | 50 | 3967 | N/A | N/A |
| DC_2A-38A_n78A | 2 | 1852.5 | 5 | 25 | 1932.5 | 16 | IMD39 |
|  | 38 | 2617.5 | 5 | 25 | 2617.5 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
| DC_2A_n41A-n71A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
|  | n71 | 676 | 5 | 50 | 630 | 28.7 | IMD2 |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2586 | 10 | 50 | 2586 | 29.2 | IMD2 |
|  | n71 | 686 | 5 | 50 | 640 | N/A | N/A |
| DC_2A-46A_n5A5 DC_2A-46C_n5A5 DC_2A-46D_n5A5 DC_2A-46E_n5A5 | 2 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_2A-2A-46A_n5A5 DC_2A-2A-46C_n5A5 DC_2A-2A-46D_n5A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD4, IMD5 |
|  | n5 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_2A-46A_n66A5 DC_2A-46C_n66A5 DC_2A-46D_n66A5 DC_2A-46E_n66A5 | 2 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD3, IMD5 |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_2A-46A_n77A5 DC_2A-46A-46A_n77A5 | 2 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | n2 | 1853 | 5 | 25 | 1933 | N/A | N/A |
|  | 48 | 3590 | 20 | 100 | 3590 | N/A | N/A |
|  | 2 | 1889 | 5 | 25 | 1969 | 12 | IMD4 |
| DC_2A-48A_n5A | 2 | 1870 | 5 | 25 | 1950 | 16.9 | IMD3 |
| DC_2A-48C_n5A | 48 | 3610 | 10 | 50 | 3610 | N/A | N/A |
| DC_2A-48D_n5A | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_2A-48E_n5A | 2 | 1890 | 5 | 25 | 1970 | N/A | N/A |
|  | 48 | 3570 | 5 | 25 | 3570 | 16.2 | IMD3 |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |
| DC_2A-48A_n66A DC_2A-48C_n66A DC_2A-48D_n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 28.3 | IMD2 |
|  | 48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
| DC_2A_n48A-n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
| DC_2A-48E_n66A | n48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 20 | IMD3 |
| DC_2A-66A_n2A | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
| DC_2A-66A-66A_n2A | n2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
| DC_2A-66A_n5A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 7.2 | IMD4 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_2A-66A_n25A | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_2A-66A_n28A | 2 | 1880 | 5 | 25 | 1960 | 11.0 | IMD4 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | N/A |
| DC_2A-66A_n41A DC_2A-66A_n41C DC_2A-66A_n41(2A) | 2 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1905 | 5 | 25 | 1985 | N/A | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 12.1 | IMD4 |
|  | n48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1880 | 5 | 25 | 1960 | 28.3 | IMD5 |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
| DC_2A-66A_n77A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
| DC_2A-66A_n77C DC_2A-66A_n77(2A) DC_2A-2A-66A_n77A DC_2A-2A-66A_n77C DC_2A-66A-66A_n77A DC_2A-66A-66A_n77C DC_2A-2A-66A-66A_n77A DC_2A-2A-66A-66A_n77C | 66 | 1715 | 5 | 25 | 2115 | 29.2 | IMD2 |
|  | n77 | 3970 | 5 | 25 | 3970 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | M/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 10.4 | IMD4 |
|  | n77 | 3500 | 5 | 25 | 3500 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | M/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | 4.0 | IMD5 |
|  | n77 | 3915 | 5 | 25 | 3915 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3720 | 5 | 25 | 3720 | N/A | N/A |
| DC_2A-66A_n77A11 DC_2A-66A_n77C11 DC_2A-66A_n77(2A)11 DC_2A-2A-66A_n77A11 DC_2A-2A-66A_n77C11 DC_2A-66A-66A_n77A11 DC_2A-66A-66A_n77C11 DC_2A-2A-66A-66A_n77A11 DC_2A-2A-66A-66A_n77C11 | 2 | 1860 | 5 | 25 | 1940 | 9.1 | IMD4 |
|  | 66 | 1775 | 5 | 25 | 2195 | N/A | N/A |
|  | n77 | 3385 | 5 | 25 | 3385 | N/A | N/A |
| DC_2A-66A_n77A | 2 | 1855 | 5 | 25 | 1935 | 4.2 | IMD5 |
| DC_2A-66A_n77C DC_2A-66A_n77(2A) DC_2A-2A-66A_n77A DC_2A-2A-66A_n77C DC_2A-66A-66A_n77A DC_2A-66A-66A_n77C DC_2A-2A-66A-66A_n77A DC_2A-2A-66A-66A_n77C | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n77 | 3540 | 5 | 25 | 3540 | N/A | N/A |
| DC_2A_n66A-n77A11 DC_2A-2A_n66A-n77A11 | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 29.2 | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | N/A |
|  | 2 | 1853 | 5 | 25 | 1933 | N/A | N/A |
|  | n66 | 1713 | 5 | 25 | 2113 | N/A | N/A |
|  | n77 | 3566 | 10 | 50 | 3566 | 29.4 | IMD2 |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) DC_2A_n66A-n78A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
| DC_2A-2A-66A_n78A | 66/n66 | 1760 | 5 | 25 | 2160 | 10.3 | IMD4 |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | 2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
| DC_2A-2A-66A_n78A | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | 9.1 | IMD4 |
| DC_2A-2A-66A_n78A | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | 2.1 | IMD5 |
| DC_2A-2A-66A_n78A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | N/A |
| DC_2A_n66A-n78A DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | 8.9 | IMD4 |
| DC_2A-71A_n38A DC_2A-2A-71A_n38A | 2 | 1862 | 5 | 25 | 1942 | 26 | IMD2 |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
|  | n38 | 2610 | 10 | 50 | 2610 | N/A | N/A |
| DC_2A-71A_n41A DC_2A-2A-71A_n41A | 2 | 1862 | 5 | 25 | 1942 | 26 | IMD2 |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
|  | n41 | 2610 | 10 | 50 | 2610 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 71 | 676 | 5 | 50 | 630 | 28.7 | IMD24 |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
| DC_2A-71A_n78A DC_2A-2A-71A_n78A | 2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
| DC_2A_n71A-n78A | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | 8 | IMD3 |
| DC_3A_n1A-n28A DC_3C_n1A-n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_3A_n1A-n40A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |
| DC_3A_n1A-n41A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n41 |  | 2507.5 |  | 5 |  | 25 |  |  | 2507.5 |  | 5.0 |  | IMD5 |  |
| DC_3A_n1A-n75A | n75 |  | N/A |  | 5 |  | 25 |  |  | 1480 |  | 15.2 |  | IMD34 |  |
|  | n1 |  | 1960 |  | 5 |  | 25 |  |  | 2150 |  | N/A |  | N/A |  |
|  | 3 |  | 1720 |  | 5 |  | 25 |  |  | 1815 |  | N/A |  | N/A |  |
| DC_3C_n1A-n75A | n75 |  | N/A |  | 5 |  | 25 |  |  | 1495 |  | [FFS] |  | IMD3 |  |
|  | n1 |  | 1975 |  | 5 |  | 25 |  |  | 2165 |  | N/A |  | N/A |  |
|  | 3 |  | 1730 1749.8 |  | 20 20 |  | 1(RBSTART=20) 1(RBSTART=79) |  |  | 1825 1844.8 |  | N/A N/A |  | N/A N/A |  |
| DC_3A_n1A-n77A | 3 |  | 1750 |  | 5 |  | 25 |  |  | 1845 |  | N/A |  | N/A |  |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_3A_n1A-n78A DC_3C_n1A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 3.5 | IMD5 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_(n)3AA-n8A | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1837.5 | 4.5 | IMD5 |
|  | n3 | 1747.5 | 5 | 25 | 1842.5 | 6.4 | IMD5 |
| DC_3A_n3A-n41A | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | 8.2 | IMD4 |
|  | n41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |
| DC_3A-5A_n77A DC_3A-5A_n77(2A) | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 5 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n77 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A-5A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 5 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-5A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 18.5 | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |
|  | 5 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 | 1875 | N/A | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 30.0 | IMD21 |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
| DC_3A-7A_n8A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2 IMD33 |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
| DC_3A-18A_n3A | 3 | 1719 | 5 | 25 | 1814 | 4 | IMD4 ¦2*fn3-2*fB18¦ |
|  | 18 | 823 | 5 | 25 | 868 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_3-18_n41 | 18 | 820 | 5 | 25 | 865 | 28.9 | IMD2 |
|  | 3 | 1765 | 5 | 25 | 1860 | N/A | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 19.0 | IMD3 |
|  | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | n41 | 2585 | 5 | 25 | 2585 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 28.8 | IMD2 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | MSD | N/A |
| DC_3A-18A_n77A DC_3A-18A_n77(2A)DC_3A-18A_n78A DC_3A-18A_n78(2A) | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 18 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-19A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 19 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A_n7A-n28A | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
| DC_3C_n7A-n28A | n7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | n28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n7 | 2562 | 5 | 25 | 2682 | 17.0 | IMD3 |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
| DC_3A-7A_n40A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
| DC_3A-7A_n77(2A) | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
| DC_3A-7A-7A_n77(2A) | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3475 | 10 | 50 | 3475 | N/A | N/A |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | 7 | 2550 | 5 | 25 | 2670 | 5.2 | IMD5 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | 3.4 | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A DC_3A-7A_n78(2A) DC_3C-7A_n78(2A) DC_3A-7C_n78(2A) DC_3C-7C_n78(2A) DC_3A-7A_n78C DC_3A-7A-7A_n78C | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |
| DC_3A-8A_n40A | 3 | 1779 | 5 | 25 | 1874 | 4 | IMD5 |
|  | 8 | 912 | 5 | 25 | 957 | N/A | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | N/A |
| DC_3A-8A_n77A DC_3A-8A_n77(2A) DC_3A-8A_n77(3A) DC_3C-8A_n77A DC_3C-8A_n77(2A) | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 9.7 | IMD4 |
| DC_3A-8A_n77A DC_3A-8A_n77(2A) DC_3A-8A_n77(3A) DC_3C-8A_n77A DC_3C-8A_n77(2A) | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A-8A_n78A DC_3A-3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A_n8A-n78A | 3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | 16.3 | IMD3 |
| DC_3A-8A_n79A | 3 | 1755 | 5 | 25 | 1850 | N/A | N/A |
|  | n79 | 4465 | 40 | 216 | 4465 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 15.3 | IMD3 |
| DC_3A-8A_n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 8.8 | IMD4 |
| DC_3A_n7A-n78A DC_3A_n7B-n78A DC_3C_n7A-n78A DC_3C_n7B-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_3A_n7A-n78(2A) | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
| DC_3C_n7A-n78(2A) | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
| DC_3A-11A_n77A DC_3A-11A_n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 3675 | 10 | 50 | 3675 | N/A | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 8.8 | IMD4 |
|  | 11 | 1435.4 | 5 | 25 | 1483.4 | N/A | N/A |
|  | n77 | 3905 | 10 | 50 | 3905 | N/A | N/A |
|  | 3 | 1753 | 5 | 25 | 1848 | 3.4 | IMD57 |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 19 | 840 | 5 | 25 | 885 | 18.5 | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_3A-20A_n7A DC_3C-20A_n7A | 3 | 1737 | 5 | 25 | 1832 | N/A | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
| DC_3A-20A_n8A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 27 | IMD2 |
| DC_3A-20A_n8A | 3 | 1765 | 5 | 25 | 1860 | 14.5 | IMD4 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | 3 | 1733 | 5 | 25 | 1828 | 9.4 | IMD4 |
| DC_3A-20A_n38A | 3 | 1779 | 5 | 25 | 1874 | N/A | N/A |
|  | 20 | 852 | 10 | 20 | 811 | 26.0 | IMD21 |
|  | n38 | 2590 | 10 | 50 | 2590 | N/A | N/A |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1744 | 5 | 25 | 1839 | 26.0 | IMD2 |
|  | n41 | 2680 | 10 | 50 | 2680 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | N/A | N/A |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1779 | 5 | 25 | 1874 | N/A | N/A |
|  | n41 | 2590 | 10 | 50 | 2590 | N/A | N/A |
|  | 20 | 852 | 10 | 50 | 811 | 26.0 | IMD2 |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n41 | 2660 | 10 | 50 | 2660 | N/A | N/A |
|  | 20 | 841 | 5 | 25 | 800 | 12.5 | IMD3 |
| DC_3_n20-n67 | 3 | 1765 | 5 | 25 | 1860 | N/A | N/A |
|  | n20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n67 | N/A | 5 | 25 | 746 | 10.1 | IMD4 |
| DC_3A_20A_SUL_n78A-n80A DC_3C_20A_SUL_n78A-n80A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
| DC_3A-20A_n78A DC_3C-20A_n78A DC_3A-20A_n78(2A) | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 | IMD4 |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 3 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | N/A |
| DC_3A-21A_n79A | 3 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A-28A_n1A DC_3C-28A_n1A | 3 | 1725 | 5 | 25 | 1820 | 4 | IMD5 |
|  | 28 | 710 | 5 | 25 | 765 | N/A | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
| DC_3A-28A_n5A DC_3C-28A_n5A | 3 | 1735 | 5 | 25 | 1830 | 8.7 | IMD4 |
|  | 28 | 705 | 5 | 25 | 798 | N/A | N/A |
|  | n5 | 845 | 5 | 25 | 874 | N/A | N/A |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | 28 | 730 | 5 | 25 | 785 | 9.4 | IMD4 |
|  | n5 | 845 | 5 | 25 | 874 | N/A | N/A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |
|  | 28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |
| DC_3A_n28A-n75A DC_3C_n28A-n75A | B3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 708 | 5 | 25 | 763 | N/A | N/A |
|  | n75 | - | - | - | 1436 | 3.3 | IMD5 |
| DC_3A_n28A-n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | IMD3 |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |
| DC_3A-28A_n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | N/A |
|  | 28 | 735 | 5 | 25 | 790 | 26.0 | IMD21 |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
| DC_3A_n28A-n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 261 | IMD2 ¦fn41-fB3¦ |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | N/A |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | n41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 ¦fB3+fn28¦ |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  |  | n41 |  | 2687 |  | 5 |  | 25 | 2687 |  | 15.9 |  | IMD3 ¦2*fB3-fn28¦ |  |
| DC_3A_n26A-n78A |  | 3 |  | 1730 |  | 5 |  | 25 | 1825 |  | N/A |  | N/A |  |
| DC_3C_n26A-n78A |  | n26 |  | 839 |  | 5 |  | 25 | 884 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3408 |  | 10 |  | 50 | 3408 |  | 16.1 |  | IMD3 |  |
|  |  | 3 |  | 1730 |  | 5 |  | 25 | 1825 |  | N/A |  | N/A |  |
|  |  | n26 |  | 839 |  | 5 |  | 25 | 884 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3512 |  | 10 |  | 50 | 3512 |  | 4.5 |  | IMD5 |  |
| DC_3A-28A_n78A DC_3C-28A_n78A DC_3A-3A-28A_n78A |  | 3 |  | 1775 |  | 5 |  | 25 | 1870 |  | 17.3 |  | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 760 | N/A | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | N/A |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 5.7 | IMD5 |
|  | 28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A_n28A-n78A DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | IMD5 |
| DC_3A_n28A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n79 | 4585 | 40 | 216 | 4585 | 9.4 | IMD44 |
| DC_3A_SUL_n77A-n84A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n84 | 1922.5 | 5 | 25 |  | N/A | N/A |
|  | n77 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_3A_n40A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 4.8 | IMD5 |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 4.4 | IMD5 |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | N/A |
| DC_3A_n40A-n79A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2330 | 5 | 25 | 2330 | N/A | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | 4.7 | IMD5 |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2330 | 5 | 25 | 2330 | 3.2 | IMD5 |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
| DC_3A_n41A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |
|  | n79 | 4440 | 40 | 216 | 4440 | 30.8 | IMD24 |
| DC_3A-42A_n1A DC_3A-42C_n1A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | 42 | 3425 | 5 | 25 | 3425 | 13.0 | IMD4 |
|  | n1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
| DC_3A_n75A-n78A DC_3A_n75A-n78(2A) | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | n75 | - | - | - | 1514.5 | 10.0 | IMD2 |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
|  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | IMD3 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | 4.2 | IMD5 |
| DC_3A_SUL_n78A-n82A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | n82 | 840 | 5 | 25 |  | N/A | N/A |
| DC_3A_SUL_n78A-n84A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n84 | 1922.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A-32A_n1A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
| DC_3C-32A_n1A | 32 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | n1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
| DC_3A-32A_n78A DC_3C-32A_n78A DC_3A-32A_n78C DC_3A-32A_n78(2A) | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 4.9 | IMD4 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1475 | 0 | IMD5 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
| DC_3A-38A_n28A DC_3C-38A_n28A | 38 | 2575 | 5 | 25 | 2575 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 26 | IMD2 |
| DC_3A-38A_n78A DC_3C-38A_n78A | 3 | 1735 | 5 | 25 | 1830 | 16.4 | IMD35 |
|  | 38 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
| DC_3A-40A_n1A DC_3A-40C_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |
| DC_3A-40A_n78A DC_3A-40C_n78A | 3 | 1775 | 5 | 25 | 1870 | 9.1 | IMD4 |
|  | 40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
|  | n78 | 3325 | 10 | 50 | 3325 | N/A | N/A |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 40 | 2360 | 5 | 25 | 2360 | 4.4 | IMD5 |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | N/A |
| DC_3A-41A_n3A DC_3A-41C_n3A | 3 | 1770 | 5 | 25 | 1865 | 8.2 | IMD4 ¦2*fB41-2*fn3¦ |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26 | IMD2 |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 41 | 2687 | 5 | 25 | 2687 | 15.9 | IMD3 |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) DC_3A_n41A-n77A DC_3A_n41A-n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 41/n41 | 2640 | 5 | 25 | 2640 | 5.3 | IMD5 |
|  | 41/n41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A-41A_n78A DC_3A-41C_n78A DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A_n41A-n78A DC_3A_n41A-n78(2A) | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.4 | IMD3 |
| DC_3A-41A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | N/A |
|  | 41 | 2670 | 5 | 25 | 2670 | 30.2 | IMD2 |
|  | 41 | 2570 | 5 | 25 | 2570 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 29.4 | IMD2 |
| DC_4A-7A_n28A | 4 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | 18.0 | IMD3 |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
| DC_5_n1-n78 | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3610 | 10 | 50 | 3610 | 15.7 | IMD3 |
| DC_5A_n2A-n77A11 | n2 | 1907 | 5 | 25 | 1987 | 16.5 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n77 | 3680 | 5 | 25 | 3680 | N/A | N/A |
| DC_5A_n5A-n77A11 | 5 | 834 | 5 | 25 | 879 | N/A | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n77 | 3391 | 10 | 50 | 3391 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | n5 | 837 | 5 | 25 | 882 | 5.5 | IMD5 |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | N/A |
| DC_5A-7A_n7A | 5 | 834 | 5 | 25 | 879 | 12 | IMD34 |
|  | 7 | 2527 | 10 | 50 | 2647 | N/A | N/A |
|  | n7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
| DC_5A_n2A-n78A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | 16.0 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n2 | 1907 | 5 | 25 | 1987 | 16.5 | IMD3 |
|  | n78 | 3680 | 10 | 25 | 3680 | N/A | N/A |
| DC_5A_n3A-n78A | 5 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3408 | 10 | 50 | 3408 | 16.1 | IMD3 |
|  | 5 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3512 | 10 | 50 | 3512 | 4.5 | IMD5 |
|  | 5 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n3 | 1767 | 5 | 25 | 1862 | 15.7 | IMD3 |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_5A-7A_n66A DC_5A-7C_n66A DC_5A-7A-7A_n66A | 5 | 835 | 5 | 25 | 880 | 17.8 | IMD3 |
|  | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | 7 | 2504 | 5 | 25 | 2624 | 29.0 | IMD21 |
|  | 66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |
| DC_5A-7A_n71A | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | 6.5 | IMD5 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | N/A |
|  | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
| DC_5A-7A_n77A | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
| DC_5A-7A_n77(2A) | n77 | 3489 | 10 | 50 | 3489 | N/A | N/A |
| DC_5A-7A-7A_n77A | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD21 |
| DC_5A-7A-7A_n77(2A) | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n77 | 3429 | 10 | 50 | 3429 | N/A | N/A |
| DC_5A-7A_n78A DC_5A-7A_n78C DC_5A-7A-7A_n78C | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_5A_n7A-n78A, DC_5A_n7(2A)-n78A DC_5A_n7A-n78(2A) DC_5A_n7(2A)-n78(2A) | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | 29.7 | IMD2 |
| DC_5A-13A_n66A | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 9.4 | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_5A-13A_n77A11 | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
| DC_5A-13A_n77C11 | n77 | 4110 | 10 | 50 | 4110 | N/A | N/A |
|  | 13 | 781 | 5 | 20 | 750 | 4.4 | IMD5 |
|  | 13 | 782 | 5 | 20 | 751 | N/A | N/A |
|  | n77 | 4013 | 10 | 50 | 4013 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 4.5 | IMD5 |
| DC_5A-30A_n2A | 5 | 835 | 5 | 25 | 880 | 8 | IMD4 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | N/A |
| DC_5A-30A_n77A DC_5A-30A_n77(2A) | 5 | 835 | 5 | 25 | 880 | 15.2 | IMD34 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 13.2 | IMD311 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | N/A |
| DC_5A_n38A-n66A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n38 | 2590 | 10 | 50 | 2590 | 28.9 | IMD2 |
| DC_5A_41A_n78A | 5 | 860 | 5 | 25 | 885 | 30.2 | IMD2 |
|  | 41 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |
|  | 5 | 856.5 | 5 | 25 | 881.5 | 3.1 | IMD5 |
|  | 41 | 2620.5 | 5 | 25 | 2620.5 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |
| DC_5A-41A_n79A | 5 | 835 | 5 | 25 | 880 | 23.9 | IMD3 |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | N/A |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 41 | 2517.5 | 5 | 25 | 2517.5 | 1.8 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
| DC_5A-46A_n66A | 5 | 847 | 5 | 25 | 892 | N/A | N/A |
|  | 46 | 5163 | 10 | 50 | 5163 | 9.04 | IMD4 ¦2*fB5+2*fn66¦ |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_5A-48A_n12A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 48 | 3650 | 5 | 25 | 3650 | 4.4 | IMD5 |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | 5.9 | IMD5 |
|  | 48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |
| DC_5A-48A_n71A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 48 | 3590 | 5 | 25 | 3590 | 4.4 | IMD5 |
|  | n71 | 690 | 5 | 25 | 644 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | 5.9 | IMD5 |
|  | 48 | 3600 | 5 | 25 | 3600 | N/A | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | N/A |
| DC_5A-66A_n2A DC_5B-66A_n2A DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | 5 | 834 | 5 | 25 | 879 | N/A | N/A |
| DC_5A-66B_n2A | 66 | 1712 | 5 | 25 | 2132 | 7.2 | IMD4 |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
| DC_5A-66A_n7A DC_5A-66A-66A_n7A | 5 | 835 | 5 | 25 | 880 | 18.0 | IMD3 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
| DC_5A-66A_n30A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 66 | 1725 | 5 | 25 | 2125 | 4 | IMD5 |
|  | n30 | 2307.5 | 5 | 50 | 2352.5 | N/A | N/A |
| DC_5A-66A_n71A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 66 | 1761 | 5 | 25 | 2161 | 13 | IMD3 |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | 4.2 | IMD5 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
| DC_5A-66A_n77A | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
| DC_5A-66A_n77C DC_5A-66A_n77(2A) DC_5A-66A-66A_n77A DC_5A-66A-66A_n77C | 66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
| DC_5A-66A-66A_n78A | 66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_5A_n66A-n78A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.6 | IMD3 |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | n66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_5A_n66A-n77A | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | n66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 5 | 845 | 5 | 25 | 890 | N/A | N/A |
|  | n66 | 1785 | 5 | 25 | 2185 | N/A | N/A |
|  | n77 | 3475 | 10 | 50 | 3475 | 16.1 | IMD3 |
| DC_7A_n1A-n40A | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n40 | 2335 | 5 | 25 | 2335 | N/A | N/A |
|  |  | n1 |  | 1940 |  | 5 |  | 25 | 2130 |  | 15.2 |  | IMD3 |  |
| DC_7A_n1A-n75A |  | n1 |  | 1977.5 |  | 5 |  | 25 | 2167.5 |  | N/A |  | N/A |  |
|  |  | 7 |  | 2502.5 |  | 5 |  | 25 | 2622.5 |  | N/A |  | N/A |  |
|  |  | 75 |  | N/A |  | 5 |  | N/A | 1454.5 |  | 15.2 |  | IMD3 |  |
| DC_7A_n1A-n78A DC_7C_n1A-n78A |  | 7 |  | 2520 |  | 5 |  | 25 | 2640 |  | N/A |  | N/A |  |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | 9.0 | IMD4 |
|  | n78 | 3610 | 10 | 50 | 3610 | N/A | N/A |
| DC_7A_n2A-n71A | 7 | 2530 | 5 | 25 | 2530 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n71 | 676 | 5 | 25 | 630 | 28.7 | IMD2 |
| DC_7A_n2A-n78A | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | n78 | 3525 | 10 | 50 | 3525 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n78 | 3775 | 10 | 50 | 3775 | 4.2 | IMD5 |
| DC_7A_n3A-n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 15.6 | IMD3 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A_n8A-n40A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n8 | 905 | 5 | 25 | 950 | N/A | N/A |
|  | n40 | 2345 | 5 | 25 | 2345 | 3.0 | IMD5 |
| DC_7A-8A_n3A | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 18.0 | IMD3 |
| DC_7A-8A_n3A | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2+IMD33 |
| DC_7A-8A_n77A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 30.5 | IMD2 |
|  | n77 | 3470 | 10 | 50 | 3470 | N/A | N/A |
| DC_7A-8A_n77A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.1 | IMD5 |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A-8A_n77A | 7 | 2530 | 5 | 25 | 2650 | 28 | IMD2 |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 30.5 | IMD2 |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.1 | IMD5 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 | 2650 | 28 | IMD2 |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| DC_7A_n8A-n78A | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3455 | 10 | 50 | 3455 | 28.5 | IMD2 |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 29.7 | IMD2 |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |
| DC_7A-12A_n2A | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |
|  | 12 | 701.5 | 5 | 25 | 731.5 | 5.3 | IMD5 |
|  | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 7 | 2501 | 5 | 25 | 2621 | 30.8 | IMD2 |
|  | 12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |
|  | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
| DC_7A-12A_n66A | 7 | 2515 | 5 | 25 | 2635 | N/A | N/A |
|  | 12 | 712 | 5 | 25 | 742 | 31 | IMD2 |
|  | n66 | 1773 | 5 | 25 | 2173 | N/A | N/A |
| DC_7A-12A_n78A DC_7A-12A_n78(2A) | 7 | 2542 | 5 | 25 | 2662 | 29.6 | IMD2 |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 12 | 710 | 5 | 25 | 740 | 30.8 | IMD24 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
| DC_7A-13A_n66A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 31 | IMD2 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_7A-13A_n66A | 7 | 2540 | 5 | 25 | 2660 | 18 | IMD3 |
|  | 13 | 780 | 5 | 25 | 749 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
| DC_7A-13A_n25A DC_7A-7A-13A_n25A DC_7C-13A_n25A | 7 | 2542 | 10 | 50 | 2662 | 27.6 | IMD2 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |
| DC_7A-20A_n1A DC_7C-20A_n1A | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |
|  | n3 | 1737 | 5 | 25 | 1832 | N/A | N/A |
|  | 7 | 2510 | 10 | 50 | 2630 | 26.0 | IMD21 |
|  | 20 | 855 | 5 | 25 | 896 | N/A | N/A |
|  | n3 | 1775 | 10 | 50 | 1870 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 836 | 5 | 25 | 795 | 17.4 | IMD3 |
| DC_7A-20A_n8A | 7 | 2520 | 5 | 25 | 2640 | 21.1 | IMD3 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2504 | 5 | 25 | 2624 | 18.8 | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
| DC_7A-20A_n28A | 20 | 842 | 5 | 25 | 801 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | 7 | 2520 | 10 | 50 | 2640 | 5.9 | IMD5 |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
| DC_7A-20A_n78(2A) | 20 | 851 | 5 | 25 | 810 | 30.5 | IMD2 |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
| DC_7A-20A_n78(2A) | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_7A-20A_n78A | 7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 |
| DC_7A-20A_n78(2A) | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_7A-25A_n77A DC_7A-7A-25A_n77A DC_7C-25A_n77A DC_7C-25A-25A_n77A DC_7A-25A-25A_n77A DC_7A-7A-25A-25A_n77A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | 25 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | 3.4 | IMD5 |
|  | 25 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | n77 | 4120 | 10 | 50 | 4120 | N/A | N/A |
| DC_7A-25A_n78A DC_7A-7A-25A_n78A DC_7C-25A_n78A DC_7A-25A-25A_n78A DC_7A-7A-25A-25A_n78A DC_7C-25A-25A_n78A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | 25 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  |  | n78 |  | 3525 |  | 10 |  | 50 | 3525 |  | N/A |  | N/A |  |
| DC_7A_n26A-n78A |  | 7 |  | 2550 |  | 5 |  | 25 | 2670 |  | N/A |  | N/A |  |
| DC_7C_n26A-n78A |  | n26 |  | 834 |  | 5 |  | 25 | 879 |  | 30.2 |  | IMD2 |  |
|  |  | n78 |  | 3429 |  | 10 |  | 50 | 3429 |  | N/A |  | N/A |  |
|  |  | 7 |  | 2525 |  | 5 |  | 25 | 2645 |  | N/A |  | N/A |  |
|  |  | n26 |  | 830 |  | 5 |  | 25 | 875 |  | 3.3 |  | IMD5 |  |
|  |  | n78 |  | 3350 |  | 10 |  | 50 | 3350 |  | N/A |  | N/A |  |
|  |  | 7 |  | 2540 |  | 5 |  | 25 | 2660 |  | N/A |  | N/A |  |
|  |  | n26 |  | 835 |  | 5 |  | 25 | 880 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3375 |  | 10 |  | 50 | 3375 |  | 29.7 |  | IMD2 |  |
|  |  | 7 |  | 2550 |  | 5 |  | 25 | 2670 |  | N/A |  | N/A |  |
|  |  | n26 |  | 835 |  | 5 |  | 25 | 880 |  | N/A |  | N/A |  |
|  |  | n78 |  | 3430 |  | 10 |  | 50 | 3430 |  | 9.7 |  | IMD4 |  |
| DC_7A-28A_n1A |  | 7 |  | 2535 |  | 5 |  | 25 | 2655 |  | N/A |  | N/A |  |
| DC_7A-7A-28A_n1A | 28 | 725 | 5 | 25 | 780 | 4.3 | IMD5 |
|  | n1 | 1950 | 5 | 25 | 2165 | N/A | N/A |
|  | 7 | 2545 | 5 | 25 | 2665 | 29.0 | IMD2 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
| DC_7A-28A_n2A | 7 | 2510 | 10 | 50 | 2630 | 27.6 | IMD2 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
| DC_7A-28A_n3A DC_7C-28A_n3A | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2685 | 18 | IMD3 |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 | 2725 | N/A | N/A |
|  | 28 | 721 | 5 | 25 | 776 | 4.4 | IMD5 |
|  | n5 | 829 | 5 | 25 | 854 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 874 | N/A | N/A |
| DC_7A-28A_n40A | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_7A-28A_n66A DC_7C-28A_n66A | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796 | 20.0 | IMD2 |
|  | n66 | 1747 | 5 | 25 | 2147 | N/A | N/A |
| DC_7A-28A_n78A | 7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |
|  | 28 | 727.5 | 5 | 25 | 782.5 | 28.8 | IMD2 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |
|  | 28 | 727.5 | 5 | 25 | 782.5 | 3.0 | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3460 | N/A | N/A |
|  | 7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | IMD2 |
| DC_7A-29A_n78A DC_7C-29A_n78A DC_7A-7A-29A_n78A | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | 29 | N/A | N/A | N/A | 720 | 3.0 | IMD5 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
| DC_7A-32A_n1A | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1454.5 | 15.2 | IMD3 |
| DC_7A-32A_n3A | 7 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n3 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | 32 | - | - | - | 1470 | 10.5 | IMD4 |
| DC_7A-32A_n78A | n78 | 3560.5 | 10 | 50 | 3560.5 | N/A | N/A |
|  | 7 | 2517.5 | 5 | 25 | 2637.5 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1474.5 | 17.6 | IMD3 |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1492 | 4.9 | IMD4 |
| DC_7A-40A_n1A DC_7A-40C_n1A | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7 | 2530 | 5 | 25 | 2650 | 32.1 | IMD3 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_7A-40A_n78A DC_7A-40C_n78A | 7 | 2510 | 5 | 25 | 2630 | 10.1 | IMD4 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | N/A | N/A |
|  | 40 | 2310 | 5 | 25 | 2310 | 8.7 | IMD4 |
|  | n78 | 3785 | 10 | 50 | 3785 | N/A | N/A |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | 7 | 2505 | 10 | 50 | 2625 | 30.0 | IMD26 |
|  | 66 | 1775 | 10 | 50 | 2175 | N/A | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
| DC_7A-66A_n7A DC_7A-66A-66A_n7A | 7 | 2555 | 10 | 50 | 2675 | 15 | IMD4 |
|  | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2515 | 10 | 50 | 2635 | N/A | N/A |
| DC_7A-66A_n28A | 7 | 2565 | 5 | 25 | 2685 | 18.0 | IMD3 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
| DC_7A-66A_n77A DC_7A-7A-66A_n77A DC_7A-7A-66A_n77(2A) DC_7A-66A_n77(2A) DC_7C-66A_n77A DC_7C-66A_n77(2A) | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 ¦2*fB7-2*fn77¦ |
|  | n77 | 3625 | 10 | 50 | 3475 | N/A | N/A |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | 7 | 2550 | 5 | 25 | 2670 | 5.2 | IMD5 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | 3.4 | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 |
|  | n77 | 3625 | 10 | 50 | 3625 | N/A | N/A |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2542 | 5 | 25 | 2662 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3344 | 10 | 50 | 3344 | 16.0 | IMD3 |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 4040 | 10 | 50 | 4040 | 4.2 | IMD5 |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-7A-66A_n78A DC_7A-66A-66A_n78A DC_7A-7A-66A-66A_n78A DC_7C-66A-66A_n78A DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) DC_7A-7A-66A_n78(2A) DC_7A-66A-66A_n78(2A) DC_7A-7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | 66/n66 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 |
|  | n78 | 3625 | 10 | 50 | 3475 | N/A | N/A |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A | 7 | 2542 | 5 | 25 | 2662 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3344 | 10 | 50 | 3344 | 16.0 | IMD3 |
| DC_7A-71A_n2A | n2 | 1859 | 5 | 25 | 1933 | N/A | N/A |
|  | 7 | 2505 | 5 | 25 | 2625 | N/A | N/A |
|  | 71 | 692 | 5 | 25 | 646 | 30.8 | IMD2 |
| DC_7A-71A_n78A DC_7A-71A_n78(2A) | 7 | 2550 | 5 | 25 | 2670 | 29.6 | IMD2 |
|  | 71 | 680 | 5 | 25 | 634 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | 71 | 686 | 5 | 25 | 640 | 3.0 | IMD5 |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |
| DC_7A_n71A-n78A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3714 | 10 | 50 | 3714 | 9.7 | IMD4 |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | n71 | 671 | 5 | 25 | 625 | 3.9 | IMD5 |
| DC_7A_n78A-n79A DC_7A_n78A-n79C | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3600 | 10 | 50 | 3600 | N/A | N/A |
|  | n79 | 4680 | 10 | 50 | 4680 | 20.6 | IMD34,9,13 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3770 | 10 | 50 | 3770 | 6.4 | IMD413 |
|  | n79 | 4450 | 10 | 50 | 4450 | N/A | N/A |
| DC_7A_SUL_n78A-n80A | n80 | 1730 | 5 | 25 |  | N/A | N/A |
|  | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
| DC_8A_n1A-n28A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.6 | IMD4 |
| DC_8A_n1A-n40A | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
|  | n1 | 1945 | 5 | 25 | 2135 | 3.3 | IMD5 |
| DC_8A_n1A-n77A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | n77 | 3745 | 10 | 50 | 3745 | 14.9 | IMD31 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3960 | 10 | 50 | 3960 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 14.4 | IMD3 |
| DC_8A_n1A-n78A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | n78 | 3745 | 10 | 50 | 3745 | 14.9 | IMD3 |
| DC_8A_n3A-n28A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 30.4 | IMD2 |
| DC_8A_n3A-n77A DC_8A_n3A-n77(2A) | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.3 | IMD3 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_8A_n3A-n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3550 | 10 | 50 | 3550 | 16.1 | IMD3 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | 4.5 | IMD5 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 15.7 | IMD3 |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |
| DC_8A_n3A-n79A | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4425 | 40 | 216 | 4425 | 15.7 | IMD39 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 8.8 | IMD4 |
| DC_8A-11A_n1A | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 16.6 | IMD35 |
| DC_8A-11A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 18.8 | IMD3 |
| DC_8A-11A_n77A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n77 | 3791 | 10 | 50 | 3791 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 18.2 | IMD3 |
| DC_8A-11A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 18.8 | IMD3 |
| DC_8A-11A_n78A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n78 | 3791 | 10 | 50 | 3791 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 18.2 | IMD3 |
| DC_8A-11A_n79A | 8 | 882.5 | 5 | 25 | 927.5 | N/A | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 11 | 1430.4 | 5 | 25 | 1478.4 | 1.2 | IMD5 |
| DC_8A-11A_n79A | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |
|  | n79 | 4810 | 40 | 216 | 4810 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 2.8 | IMD5 |
| DC_8-20_n1 | n1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 11.5 | IMD4 |
| DC_8-20_n3 | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 27 | IMD24 |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 930 | 27 | IMD24 |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_8A-20A_n28A | 8 | 901 | 5 | 25 | 946 | [23.5] | IMD3 |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 773 | N/A | N/A |
| DC_8A-20A_n78A | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |
|  | 20 | 841 | 5 | 25 | 800 | 12.1 | IMD4 |
|  | 8 | 895 | 5 | 25 | 940 | 12.1 | IMD4 |
|  | n78 | 3481 | 10 | 50 | 3481 | N/A | N/A |
|  | 20 | 847 | 5 | 25 | 806 | N/A | N/A |
| DC_8A_n28A-n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n77 | 3473 | 10 | 50 | 3473 | 10.3 | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.6 | IMD4 |
|  | n77 | 3495 | 10 | 50 | 3495 | N/A | N/A |
| DC_8A_n28A-n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n78 | 3455 | 10 | 50 | 3455 | 10.3 | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.6 | IMD4 |
|  | n78 | 3495 | 10 | 50 | 3495 | N/A | N/A |
| DC_8A_n28A-n79A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | N/A |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | 0.0 | IMD5 |
|  | 8 | 905 | 5 | 25 | 950 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 3.9 | IMD5 |
| DC_8A-32A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 32 | 1443 | 5 | 25 | 1491 | 18.8 | IMD3 |
| DC_8A_n39A-n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | 4680 | 40 | 216 | 4680 | 15.9 | IMD3 |
| DC_8A_n39A-n79A | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | 4560 | 40 | 216 | 4560 | 12.1 | IMD4 |
| DC_8A_n39A-n79A | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n39 | 1907.5 | 10 | 50 | 1907.5 | 13.8 | IMD4 |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |
| DC_8A-40A_n1A DC_8A-40C_n1A | 8 | 885 | 5 | 25 | 930 | 8.0 | IMD4 |
|  | 40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
| DC_8A-40A_n78A DC_8A-40C_n78A | 8 | 905 | 5 | 25 | 950 | 30.5 | IMD2 |
|  | 40 | 2380 | 5 | 25 | 2380 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 935 | 19.8 | IMD3 |
|  | 40 | 2320 | 5 | 25 | 2320 | N/A | N/A |
|  | n78 | 3705 | 10 | 50 | 3705 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 40 | 2395 | 5 | 25 | 2395 | 28 | IMD2 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
| DC_8A_n40A-n79A | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | N/A |
|  | n79 | 4960 | 40 | 216 | 4960 | 10.7 | IMD4 |
|  | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | 9.2 | IMD4 |
|  | n79 | 4960 | 40 | 216 | 4960 | N/A | N/A |
| DC_8A-41A_n1A | 41 | 2500 | 5 | 25 | 2500 | N/A | N/A |
| DC_8A-41C_n1A | n1 | 1977 | 5 | 25 | 2167 | N/A | N/A |
|  | 8 | 886 | 5 | 25 | 931 | 4.5 | IMD5 |
| DC_8A-41A_n3A | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
| DC_8A-41C_n3A | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 41 | 2665 | 5 | 25 | 2665 | 27.4 | IMD21 |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 28.9 | IMD21 |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | N/A |
| DC_8A-41A_n77A | 8 | 905 | 5 | 25 | 950 | 29.1 | IMD21, 4 |
| DC_8A-41C_n77A | 41 | 2630 | 10 | 50 | 2630 | N/A | N/A |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | 41 | 2650 | 5 | 25 | 2650 | 28.0 | IMD2 |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| DC_8A_n41A-n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | N/A |
|  | n79 | 4470 | 40 | 216 | 4470 | 16.3 | IMD3 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | 15.5 | IMD3 |
|  | n79 | 4470 | 40 | 216 | 4470 | N/A | N/A |
| DC_8A-42A_n1A | 42 | 3405 | 10 | 50 | 3405 | N/A | N/A |
| DC_8A-42C_n1A | n1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 3.3 | IMD5 |
| DC_8A-42A_n3A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | 42 | 3540 | 5 | 25 | 3540 | 16.3 | IMD3 |
| DC_8A-42A_n28A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 42 | 3443 | 5 | 25 | 3443 | 8.7 | IMD4 |
| DC_8A_SUL_n78A-n80A | n80 | 1755 | 10 | 50 |  | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 8 | IMD4 |
|  | n80 | 1750 | 10 | 50 |  | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3550 | 10 | 50 | 3550 | 8 | IMD33 |
| DC_11A_n1A-n77A DC_11A_n1A-n77(2A) | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | 29.6 | IMD21 |
|  | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 30.8 | IMD21 |
| DC_11A_n3A-n28A | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |
|  | n3 | 1753 | 5 | 25 | 1848 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 3.0 | IMD5 |
| DC_11A_n3A-n77A DC_11A_n3A-n77(2A) | 11 | 1440 | 5 | 25 | 1488 | N/A | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | 10.8 | IMD4 |
|  | 11 | 1440 | 5 | 25 | 1488 | N/A | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 29.0 | IMD2 |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_11A_n3A-n79A | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 | 16.2 | IMD3 |
|  | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |
|  | n79 | 4735 | 40 | 216 | 4735 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | 17.8 | IMD3 |
| DC_11A-18A_n77A | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |
| DC_11A-18A_n77(2A) | n77 | 3706 | 10 | 50 | 3706 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 18.7 | IMD3 |
| DC_11A-18A_n78A | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |
| DC_11A-18A_n78(2A) | n78 | 3706 | 10 | 50 | 3706 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 18.7 | IMD3 |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n77 | 3629 | 10 | 50 | 3629 | 17.5 | IMD3 |
|  | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |
|  | n77 | 3684 | 10 | 50 | 3684 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 15.8 | IMD3 |
| DC_12A_n2A-n38A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n38 | 2608 | 10 | 50 | 2608 | 28.7 | IMD2 |
| DC_12A_n2A-n41A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2608 | 5 | 25 | 2608 | 28.7 | IMD2 |
| DC_12_n2-n78 | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 16.5 | IMD3 |
|  | n78 | 3375 | 10 | 50 | 3375 | N/A | N/A |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n78 | 3315 | 10 | 50 | 3315 | 16.0 | IMD3 |
| DC_12A_n7A-n78A, DC_12A_n7(2A)-n78A DC_12A_n7A-n78(2A) DC_12A_n7(2A)-n78(2A) | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3624 | 10 | 50 | 3624 | 9 | IMD4 |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
|  | n7 | 2542 | 5 | 25 | 2662 | 29.6 | IMD2 |
| DC_12A-30A_n2A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 30 | 2308 | 5 | 25 | 2353 | 12.0 | IMD4 |
|  | n2 | 1885 | 5 | 25 | 1965 | N/A | N/A |
| DC_12A-30A_n5A | 12 | 702 | 5 | 25 | 732 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 18.8 | IMD3 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
| DC_12A-30A_n77A DC_12A-30A_n77(2A) | 12 | 710 | 5 | 25 | 740 | 15.2 | IMD34 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | N/A |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 13.2 | IMD3 |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| DC_12A-66A_n5A DC_12A-66A-66A_n5A | 12 | 712 | 5 | 25 | 742 | 9.4 | IMD4 |
|  | 66 | 1745 | 5 | 25 | 2145 | N/A | N/A |
|  | n5 | 829 | 5 | 25 | 874 | N/A | N/A |
| DC_12A-66A_n77A DC_12A-66A_n77(2A) | 12 | 710 | 5 | 25 | 740 | 15.2 | IMD311 |
| DC_12A-66A-66A_n77A | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | N/A |
|  | 12 | 707 | 5 | 25 | 737 | N/A | N/A |
|  | 66 | 1726 | 5 | 25 | 2126 | 13.2 | IMD3 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_13A_n2A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1896 | 5 | 25 | 1976 | N/A | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | 17.3 | IMD3 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 16.0 | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | N/A |
| DC_13A_n5A-n77A11 | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n77 | 4013 | 10 | 50 | 4013 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 885 | 4.5 | IMD5 |
| DC_13A_n25A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | n66 | 1736 | 5 | 25 | 2156 | 7.2 | IMD4 |
| DC_13A_n25A-n66A | 13 | 780 | 5 | 25 | 749 | N/A | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | 6.2 | IMD4 |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
| DC_13A_n48A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3584 | 5 | 25 | 3584 | 2.8 | IMD5 |
|  | n66 | 1716 | 5 | 25 | 2116 | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 |
| DC_13A-66A_n2A DC_13A-66A-66A_n2A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
| DC_13A-66B_n2A | 66 | 1736 | 5 | 25 | 2156 | 7..2 | IMD4 |
| DC_13A-66C_n2A | n2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
| DC_13A-66A_n5A | 13 | 781 | 5 | 25 | 750 | 9.4 | IMD4 |
| DC_13A-66A-66A_n5A | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |
| DC_12A-66A_n25A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_12A-66A_n41A | 12 | 712 | 5 | 25 | 742 | 31 | IMD2 |
|  | 66 | 1773 | 5 | 25 | 2173 | N/A | N/A |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | N/A |
| DC_12A-66A_n78A | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 17.1 | IMD3 |
|  | n78 | 3580 | 5 | 25 | 3580 | N/A | N/A |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 | 5 | 25 | 733 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 16.5 | IMD3 |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | N/A |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 | 5 | 25 | 733 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n78 | 3754 | 10 | 50 | 3754 | 4.1 | IMD5 |
| DC_13A_n7A-n78A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n78 | 3432 | 10 | 50 | 3432 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 27.9 | IMD2 |
| DC_13A_n7A-n78A | 13 | 749 | 5 | 25 | 780 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3622 | 10 | 50 | 3622 | 9 | IMD4 |
| DC_13A_n7A-n78A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n78 | 3312 | 10 | 50 | 3312 | 29.0 | IMD2 |
| DC_13A-46A_n2A5 | 13 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n2 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_13A-46A_n66A5 | 13 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD4, IMD5 |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_13A-46A_n77A5 DC_13A-46A-46A_n77A5 | 13 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD3, IMD4, IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_13A-66A_n48A DC_13A-66A_n48B DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | 66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
| DC_13A-66A_n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
| DC_13A-66A_n77C DC_13A-66A-66A_n77A DC_13A-66A-66A_n77C | 66 | 1756 | 5 | 25 | 2156 | 17.1 | IMD3 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_13A-66A_n77A11 | 13 | 781 | 5 | 25 | 750 | 15.2 | IMD3 |
| DC_13A-66A_n77C11 DC_13A-66A-66A_n77A11 DC_13A-66A-66A_n77C11 | 66 | 1710 | 5 | 25 | 2110 | N/A | N/A |
|  | n77 | 4170 | 10 | 50 | 4170 | N/A | N/A |
| DC_14A-30A_n5A | 14 | 795 | 5 | 25 | 765 | N/A | N/A |
|  | 30 | 2308 | 5 | 25 | 2353 | 5.9 | IMD5 |
|  | n5 | 827 | 5 | 25 | 872 | N/A | N/A |
| DC_14A-30A_n77A DC_14A-30A_n77(2A) | 14 | 793 | 5 | 25 | 763 | 15.2 | IMD34 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3857 | 10 | 50 | 3857 | N/A | N/A |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 13.2 | IMD3 |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | N/A |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1762 | 5 | 25 | 2162 | 7.6 | IMD4 |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |
| DC_14A-66A_n5A | 14 | 792 | 5 | 25 | 762 | 9.4 | IMD4 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | N/A | N/A |
| DC_14A-66A_n77A DC_14A-66A_n77(2A) | 14 | 793 | 5 | 25 | 763 | 15.2 | IMD311 |
| DC_14A-66A-66A_n77A | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | N/A |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 13.2 | IMD3 |
|  | n77 | 3741 | 10 | 50 | 3741 | N/A | N/A |
| DC_18A_n3A-n41A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n41 | 2540 | 10 | 50 | 2540 | 29.4 | IMD2 |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 28.2 | IMD2 |
| DC_18A_n3A-n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | 16.3 | IMD3 |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | 15.7 | IMD3 |
|  | n77 | 3505 | 10 | 50 | 3505 | N/A | N/A |
| DC_18A_n3A-n78A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 15.2 | IMD33 |
| DC_18A-28A_n77A DC_18A_n28A-n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | 28/n28 | 723 | 5 | 25 | 778 | 4.4 | IMD5 |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | N/A |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | 3.9 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n77 | 3757 | 10 | 50 | 3757 | N/A | N/A |
| DC_18A-28A_n78A | 18 | 819 | 5 | 25 | 864 | 3.8 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n78 | 3756 | 10 | 50 | 3756 | N/A | N/A |
| DC_18A_n28A-n77A DC_18A_n28A-n78A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | N/A | N/A |
|  | n77/n78 | 3770 | 10 | 50 | 3770 | 4.0 | IMD5 |
| DC_18A-41A_n3A DC_18A-41C_n3A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | 16.0 | IMD3 |
|  | 18 | 820 | 5 | 25 | 865 | 28.9 | IMD21 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | N/A | N/A |
| DC_18A_n41A-n77A DC_18A_n41A-n77(2A) DC_18A_n41A-n78A DC_18A_n41A-n78(2A) | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_18A_n41A-n77A DC_18A_n41A-n78A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n41 | 2570 | 5 | 25 | 2570 | N/A | N/A |
|  | n77/n78 | 3390 | 10 | 50 | 3390 | 30.1 | IMD2 |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n77/n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | n41 | 2630 | 5 | 25 | 2630 | 28.5 | IMD2 |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_19A_n1A-n77A DC_19A_n1A-n78A | 19 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n77/n78 | 3655 | 10 | 50 | 3655 | [21.4] | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | n77/n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 | 882.5 | 18.7 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 9.0 | IMD4 |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | N/A |
| DC_19A-21A_n79A | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 3.8 | IMD5 |
|  |  | n79 |  | 4850 |  | 40 |  | 216 | 4850 |  | N/A |  | N/A |  |
| DC_20A-n1A_n75A |  | n1 |  | 1950.5 |  | 5 |  | 50 | 2140.5 |  | N/A |  | N/A |  |
|  |  | 20 |  | 852.5 |  | 5 |  | 25 | 811.5 |  | N/A |  | N/A |  |
|  |  | n75 |  | N/A |  | 5 |  | N/A | 1459.5 |  | 4.0 |  | IMD5 |  |
| DC_20A_n1A-n78A |  | 20 |  | 845 |  | 5 |  | 25 | 804 |  | N/A |  | N/A |  |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | 16.0 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | 15.3 | IMD3 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_20_n3-n67 | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |
|  | n67 | N/A | 5 | 25 | 746 | 9.4 | IMD4 |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | IMD3 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | N/A |
| DC_20A_n7A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | n7 | 2550 | 10 | 50 | 2675 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | 28.8 | IMD21 |
| DC_20A_n8A-n78A | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n78 | 3567 | 10 | 50 | 3567 | 10.3 | IMD4 |
|  | n8 | 895 | 5 | 25 | 940 | 12.1 | IMD4 |
|  | n78 | 3481 | 10 | 50 | 3481 | N/A | N/A |
|  | 20 | 847 | 5 | 25 | 806 | N/A | N/A |
| DC_20A-38A_n1A | n1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 20 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | 38 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20A-38A_n78A DC_20A-38A_n78(2A | 20 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | 38 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 20 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 38 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20A_n38A-n78A | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | n38 | 2600 | 10 | 50 | 2600 | 30.9 | IMD2 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
| DC_20A_n7A-n28A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
|  | n7 | 2512 | 5 | 25 | 2632 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 13.9 | IMD3 |
|  | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n7 | 2550 | 10 | 50 | 2670 | 5.9 | IMD5 |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
| DC_20A_SUL_n78A-n80A | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n80 | 1735 | 5 | 25 |  | N/A | N/A |
| DC_20A_n41A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n41 | 2675 | 10 | 50 | 2675 | 29.8 | IMD2 |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | 28.8 | IMD2 |
| DC_21A_n1A-n77A DC_21A_n1A-n78A | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD24 |
|  | n77/n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3 |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | IMD4 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_21A-28A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | IMD5 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_21A_n28A-n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
| DC_21A_n28A-n78A | n28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD39 |
|  | n77/n78 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | n28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n77/n78 | 3634.5 | 10 | 50 | 3634.5 | 17.3 | IMD39 |
| DC_21A_n28A-n79A | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n28 | 735.5 | 5 | 25 | 790.5 | 2.8 | IMD5 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 21 | 1460.4 | 5 | 25 | 1508.4 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | [6.3] | IMD44 |
| DC_21A-42A_n1A | 21 | 1452 | 5 | 25 | 1500 | 31.4 | IMD2 |
|  | 42 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
| DC_28A_n1A-n40A | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2374 | 5 | 25 | 2374 | 10.1 | IMD4 |
| DC_28A_n1A-n78A | 28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_28A_n3A-n77A | 28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | IMD3 |
| DC_28A_n7A-n78A DC_28A_n7B-n78A | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_28A-38A_n1A | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 28 | 720 | 5 | 25 | 775 | 4.5 | IMD5 |
|  | 38 | 2575 | 5 | 25 | 2575 | N/A | N/A |
| DC_28A-41A_n77A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | 29.5 | IMD2 |
| DC_28A-41A_n77A | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 30.8 | IMD2 |
| DC_28A-41A_n77A | 41 | 2567.5 | 10 | 50 | 2567.5 | N/A | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | N/A |
|  | 28 | 727.5 | 5 | 25 | 782.5 | 3.0 | IMD5 |
| DC_28A-41A_n78A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | 29.5 | IMD2 |
| DC_28A-41A_n78A | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 30.8 | IMD2 |
| DC_28A-41A_n79A | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n79 | 4739 | 40 | 216 | 4739 | N/A | N/A |
|  | 41 | 2510 | 5 | 25 | 2510 | 8.6 | IMD4 |
| DC_28A-41A_n79A | 41 | 2650 | 5 | 25 | 2650 | N/A | N/A |
|  | n79 | 4502 | 40 | 216 | 4502 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 15.9 | IMD3 |
| DC_28A-42A_n79A | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
| DC_28A-42A_n79C | 42 | 3420 | 5 | 25 | 3420 | 15.3 | IMD3 |
| DC_28A-42C_n79A | n79 | 4880 | 40 | 216 | 4880 | N/A | N/A |
| DC_28A-42C_n79C | 28 | 745 | 5 | 25 | 800 | 16.2 | IMD2 |
|  | 42 | 3597.5 | 5 | 25 | 3597.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_28A-66A_n7A | 28 | 735 | 5 | 25 | 790 | 27.6 | IMD2 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n7 | 2505 | 5 | 50 | 2625 | N/A | N/A |
| DC_28A-66A_n66A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 66 | 1729 | 5 | 25 | 2129 | 11.0 | IMD4 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |
|  | n79 | 4515 | 40 | 216 | 4515 | 29.3 | IMD2 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
|  | n78 | 3715 | 10 | 50 | 3715 | 28.8 | IMD2 |
| DC_20A-28A_n3A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | 28 | 730 | 5 | 25 | 785 | 9.4 | IMD4 |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
| DC_20A_n28A-n78A, DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
|  | n28, n83 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3314 | 10 | 50 | 3314 | 8.7 | IMD4 |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | n28 | 744 | 5 | 25 | 799 | 9.4 | IMD4 |
| DC_20A-32A_n1A | n1 | 1950.5 | 5 | 50 | 2140.5 | N/A | N/A |
|  | 20 | 852.5 | 5 | 25 | 811.5 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1459.5 | 4.0 | IMD5 |
| DC_20A-38A_n3A | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | 38 | 2610 | 5 | 25 | 2610 | 28.4 | IMD21 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| DC_20A-40A_n1A DC_20A-40C_n1A | 20 | 841 | 5 | 25 | 800 | 8.0 | IMD4 |
|  | 40 | 2330 | 5 | 25 | 2330 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
| DC_20A-40A_n78A DC_20A-40C_n78A DC_20A-40A_n78(2A) DC_20A-40C_n78(2A) | 20 | 856 | 5 | 25 | 815 | 19.8 | IMD3 |
|  | 40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |
|  | n79 | 4873 | 40 | 216 | 4873 | 30.1 | IMD2 |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | N/A |
|  | n78 | 3487 | 10 | 50 | 3487 | 29.8 | IMD2 |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A | 25 | N/A | 5 | N/A | 1992.5 | 8.5 | IMD7 |
| DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | 41 | 2502.5 | 5 | 1 (RBstart=0) | 2502.5 | N/A | N/A |
| DC_25A-(n)41CA DC_25A-(n)41DA DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | n41 | 2670 | 5 | 1 (RBstart=9) | 2670 | N/A | N/A |
| DC_25A-66A_n77A DC_25A-25A-66A_n77A | 25 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 66 | 1715 | 5 | 25 | 2115 | 29.2 | IMD2 |
|  | n77 | 3970 | 10 | 25 | 3970 | N/A | N/A |
|  | 25 | 1880 | 5 | 25 | 1960 | M/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 10.4 | IMD4 |
|  | n77 | 3500 | 10 | 25 | 3500 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | M/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | 4.0 | IMD5 |
|  | n77 | 3915 | 10 | 25 | 3915 | N/A | N/A |
|  | 25 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3720 | 10 | 25 | 3720 | N/A | N/A |
|  | 25 | 1860 | 5 | 25 | 1940 | 9.1 | IMD411 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n77 | 3385 | 10 | 25 | 3385 | N/A | N/A |
|  | 25 | 1855 | 5 | 25 | 1935 | 4.2 | IMD5 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n77 | 3540 | 10 | 25 | 3540 | N/A | N/A |
| DC_25A-66A_n78A DC_25A-25A-66A_n78A | 25 | 1880 | 5 | 25 | 1960 | M/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 10.4 | IMD4 |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |
|  | 25 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |
|  | 25 | 1880 | 5 | 25 | 1960 | 9.1 | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 25 | 1900 | 5 | 25 | 1980 | 4.2 | IMD5 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n78 | 3645 | 10 | 25 | 3645 | N/A | N/A |
| DC_28A_n8A-n78A | 28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3458 | 10 | 50 | 3458 | 9.1 | IMD4 |
|  | 28 | 713 | 5 | 25 | 768 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | 4.3 | IMD5 |
|  | n78 | 3787 | 10 | 50 | 3787 | N/A | N/A |
| DC_28A-40A_n78A DC_28A-40C_n78A | 28 | N/A | 5 | 25 | 800.5 | 11 | IMD3 |
|  | 40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | N/A | N/A |
|  | 40 | 2320 | 5 | 25 | 2320 | 15.7 | IMD3 |
|  | n78 | 3750 | 10 | 50 | 3750 | N/A | N/A |
| DC_29A-30A_n66A | 29 | N/A | 5 | 25 | 719.5 | 4.5 | IMD5 |
|  | 30 | 2307.5 | 5 | 25 | 2352.5 | N/A | N/A |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |
| DC_29A-30A_n77A | 29 | N/A | 5 | N/A | 722 | 15.2 | IMD34 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3898 | 10 | 50 | 3898 | N/A | N/A |
| DC_29A-66A_n77A | 29 | N/A | 5 | N/A | 722 | 15.2 | IMD311 |
| DC_29A-66A-66A_n77A | 66 | 1734 | 5 | 25 | 2134 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
| DC_30A-66A_n5A, DC_30A-66A-66A_n5A, DC_30A-66A-66A-66A_n5A | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | 2.5 | IMD5 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_30A-66A_n77A DC_30A-66A_n77(2A) | 30 | 2310 | 5 | 25 | 2355 | 29.2 | IMD211 |
| DC_30A-66A-66A_n77A | 66 | 1745 | 5 | 25 | 2145 | N/A | N/A |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 3.4 | IMD5 |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 8.7 | IMD411 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_38A_n28A-n78A | 38 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3360 | 10 | 25 | 3360 | 28.2 | IMD29 |
|  | 38 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | 30.8 | IMD24 |
|  | n78 | 3400 | 10 | 25 | 3400 | N/A | N/A |
| DC_39A_n40A-n79A | 39 | 1917.5 | 5 | 25 | 1917.5 | N/A | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | 5.8 | IMD4 |
| DC_39A_n41A-n79A | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | 29.8 | IMD24 |
|  | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |
|  | n41 | 2620 | 10 | 50 | 2620 | 30.2 | IMD24 |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |
| DC_40A_n1A-n78A | 40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 |
|  | 40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 9.1 | IMD4 |
|  | n78 | 3430 | 10 | 50 | 3430 | N/A | N/A |
| DC_41A_n1A-n77A | 41 | 2650 | 5 | 25 | 2650 | N/A | TDD |
| DC_41C_n1A-n77A | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD |
|  | n77 | 3330 | 10 | 50 | 3330 | 19.6 | TDD |
|  | 41 | 2510 | 5 | 25 | 2510 | N/A | TDD |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A | TDD |
|  | n1 | 1930 | 5 | 25 | 2120 | 11.0 | FDD |
| DC_41A_n3A-n77A DC_41C_n3A-n77A DC_41A_n3A-n78A DC_41C_n3A-n78A | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
|  | n77/n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 41 | 2580 | 5 | 25 | 2580 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77/n78 | 3440 | 10 | 50 | 3440 | 16.8 | IMD34 |
| DC_41A_n28A-n77A DC_41C_n28A-n77A DC_41A_n28A-n78A DC_41C_n28A-n78A | 41 | 2580 | 5 | 25 | 2580 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n77/n78 | 3323 | 10 | 50 | 3323 | 28.2 | IMD21 |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 30.8 | IMD21 |
|  | n77/n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |
| DC_46A-48A_n5A5 DC_46C-48A_n5A5 DC_46D-48A_n5A5 DC_46E-48A_n5A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |
|  | 48 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n5 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_46A-48A_n66A5 DC_46C-48A_n66A5 DC_46D-48A_n66A5 DC_46E-48A_n66A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |
|  | 48 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_46A-66A_n5A | 46 | 5163 | 10 | 50 | 5163 | 9.0 | IMD4 |
| DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A DC_46A-66A-66A_n5A DC_46C-66A-66A_n5A DC_46D-66A-66A_n5A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n5 | 847 | 5 | 25 | 892 | N/A | N/A |
| DC_46A-66A_n25A4 DC_46C-66A_n25A4 DC_46D-66A_n25A4 | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_46A-66A_n77A5 DC_46A-46A-66A_n77A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |
|  | 66 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | 48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 12.1 | IMD4 |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | N/A |
| DC_48A-66A_n12A | 48 | 3580 | 5 | 25 | 3580 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 17.1 | IMD3 |
|  | n12 | 710 | 5 | 25 | 740 | N/A | N/A |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | 8.3 | IMD4 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |
| DC_48A-66A_n66A DC_48C-66A_n66A | 48 | 3660 | 20 | 100 | 3660 | N/A | N/A |
| DC_48D-66A_n66A | 66 | 1775 | 5 | 25 | 2175 | 4.0 | IMD5 |
| DC_48E-66A_n66A | n66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
| DC_48A-66A_n71A | 48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
|  | 66 | 1774 | 5 | 25 | 2174 | 15.8 | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | 48 | 3697.5 | 5 | 25 | 3697.5 | 13.0 | IMD4 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
| DC_66A_n2A-n66A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | 4.0 | IMD5 |
| DC_66A_n2A-n77A | n2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_66_n2-n78 | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 34.9 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | 20.9 | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 21.1 | IMD4 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 2.1 | IMD5 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | N/A |
| DC_66A_n5A-n48A | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | N/A | N/A |
|  | n48 | 3582 | 5 | 25 | 3582 | 3.3 | IMD5 |
| DC_66A_n5A-n77A | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | 16.6 | IMD39 |
| DC_66A_n7A-n78A, DC_66A-66A_n7A-n78 DC_66A_n7(2A)-n78A DC_66A-66A_n7(2A)-n78A DC_66A_n7A-n78(2A) DC_66A-66A_n7A-n78(2A) DC_66A-66A_n7(2A)-n78(2A) | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
| DC_66A_n25A-n41A | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | 5 | 11.0 |
| DC_66A_n25A-n48A | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 28.3 | IMD2 |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
| DC_66A_n25A-n66A | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  | n66 | 1717.5 | 5 | 25 | 2117.5 | 23 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n25 | 1873 | 5 | 25 | 1953 | N/A | N/A |
|  | n66 | 1719 | 5 | 25 | 2119 | 4 | IMD5 |
| DC_66A_n38A-n78A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n38 | 2610 | 10 | 50 | 2610 | N/A | N/A |
|  | n78 | 3460 | 10 | 50 | 3460 | 15.0 | IMD3 |
| DC_66A_n66A-n71A | 66 | 1752 | 5 | 25 | 2152 | N/A | N/A |
|  | n66 | 1718 | 5 | 25 | 2118 | 5.0 | IMD4 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |
| DC_66A_n66A-n77A | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | 31 | IMD2 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
| DC_66A_n66A-n78A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n66 | 1725 | 5 | 25 | 2125 | 2.8 | IMD5 |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| DC_66A_n71A-n78A | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
|  | n78 | 3709 | 5 | 25 | 3709 | 13.0 | IMD4 |
| DC_71A_n2A-n41A | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2586 | 5 | 25 | 2586 | 29.2 | IMD2 |
|  | 71 | 686 | 5 | 50 | 640 | N/A | N/A |
|  | n2 | 1862 | 5 | 25 | 1942 | 26 | IMD2 |
|  | n41 | 2610 | 5 | 25 | 2610 | N/A | N/A |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
| DC_71A_n2A-n78A | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | 8.0 | IMD3 |
|  | n2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
| DC_71A_n38A-n78A | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n38 | 2615 | 10 | 50 | 2615 | N/A | N/A |
|  | n78 | 3308 | 10 | 50 | 3308 | 29.1 | IMD2 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3308 | 10 | 50 | 3308 | N/A | N/A |
|  | n38 | 2615 | 10 | 50 | 2615 | 28.7 | IMD2 |
| DC_71A_n66A-n78A | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 15.5 | IMD3 |
|  | 71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
|  | n78 | 3697.5 | 10 | 50 | 3697.5 | 13.0 | IMD4 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 2: For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified NOTE 3: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. DC_2A-46A_n77A; DC_2A_n77A; fc_2A + fc_n77A; BW_2A + BW_n77A. DC_2A-46A_n77A; DC_2A_n77A; -fc_2A + 2*fc_n77A; -BW_2A + 2*BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 2*fc_13A + fc_n77A; 2*BW_13A + BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 3*fc_13A + fc_n77A; 3*BW_13A + BW_n77A. DC_13A-46A_n2A; DC_13A_n2A; 2*fc_n2A + 2*fc_13A; 2*BW_n2A+2*BW_13A. DC_13A-46A_n77A; DC_13A_n77A; -3*fc_13A + 2*fc_n77A; -3*BW_13A + 2*BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; fc_66A + fc_n77A; BW_66A + BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; -fc_66A + 2*fc_n77A; -BW_66A + 2*BW_n77A. DC_13A-46A_n66A; DC_13A_n66A; 3*fc_13A + fc_n66A; BW_13A + 2*BW_n66A. DC_13A-46A_n66A; DC_13A_n66A; 2*fc_13A + 3*fc_n66A; BW_13A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; fc_48A + fc_n66A; BW_48A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; 2*fc_48A + fc_n66A; 2*BW_48A + BW_n66A. DC_2A-46_n5A; DC_2A_n5A; 2*fc_2A + 2*fc_n5A; BW_2A + 2*BW_n5A. DC_2A-46_n5A; DC_2A_n5A; fc_2A + 4*fc_n5A; BW_2*2A + BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + fc_n5A; BW_48A + 2*BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + 2*fc_n5A; BW_2*48A + BW_n5A. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: This band is also subject to IMD2 which is not specified. The frequency range below 3400MHz in n77 is not used for this combination. NOTE 8: Band 5 is also affected by IMD5 from UL DC_2A_n12A, but MSD value is not specified as there is only partial overlap of IMD5 with DL carrier. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 10: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2 fall in B1 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped.NOTE 12: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 13: For the DC band combination, simultaneous Rx/Tx capability is allowed between n78 and n79 NOTE 14: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |

<<Unchanged parts skipped>>

7.3B.3.3.2	ΔRIB,c for EN-DC three bands

Table 7.3B.3.3.2-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |  |  |  |
| DC_1-3_n28 | - |  | - |  | 0.2 |  |
| DC_1_n3-n28 | - |  | - |  | 0.2 |  |
| DC_1-3_n41 | - |  | - |  | 03 / 0.54 |  |
| DC_1_n3-n41 | - |  | - |  | 03 / 0.54 |  |
| DC_1-41_n3 | - |  | - |  | 03 / 0.54 |  |
| DC_1-3_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1_n3-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1-3_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1_n3-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1_n3-n79 | - |  | - |  | 0.5 |  |
| DC_1-5_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1-5_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1-7_n8 | - |  | - |  | 0.2 |  |
| DC_1-7_n28 | - |  | - |  | 0.2 |  |
| DC_1-7_n38 | - |  | - |  | 0.2 |  |
| DC_1-7_n40 | - |  | 0.3 |  | 0.8 |  |
| DC_1-7_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1-7_n78 DC_1-7-7_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1_n7-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1-8_n28 | - |  | 0.2 |  | 0.2 |  |
| DC_1_n8-n40 | - |  | 0.2 |  | 0.5 |  |
| DC_1-8_n77 | - |  | 0.2 |  | 0.5 |  |
| DC_1_n8-n77 | - |  | 0.2 |  | 0.5 |  |
| DC_1-8_n78 | - |  | 0.2 |  | 0.5 |  |
| DC_1_n8-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1-11_n3 | - |  | 0.3 |  | 0.5 |  |
| DC_1-11_n28 | - |  | - |  | 0.2 |  |
| DC_1-11_n77 | 0.2 |  | - |  | 0.5 |  |
| DC_1-11_n78 | - |  | - |  | 0.5 |  |
| DC_1-18_n77 | - |  | - |  | 0.5 |  |
| DC_1-18_n78 | - |  | - |  | 0.5 |  |
| DC_1-19_n77 | - |  | - |  | 0.5 |  |
| DC_1-19_n78 | - |  | - |  | 0.5 |  |
| DC_1-19_n79 | 0.3 |  | 0.3 |  | - |  |
| DC_1-20_n28 | - |  | 0.2 |  | 0.2 |  |
| DC_1-20_n78 | - |  | - |  | 0.5 |  |
| DC_1-21_n28 | - |  | - |  | 0.2 |  |
| DC_1-21_n77 | - |  | - |  | 0.5 |  |
| DC_1-21_n78 | 0.2 |  | - |  | 0.5 |  |
| DC_1-20_n38 | - |  | 0.2 |  | - |  |
| DC_1_n26-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1-28-n3 | - |  | 0.2 |  | - |  |
| DC_1-28_n7 | - |  | 0.2 |  | - |  |
| DC_1_n28-n40 | - |  | 0.2 |  | - |  |
| DC_1-28_n40 | - |  | 0.2 |  | - |  |
| DC_1_n28-n75 | 0.2 |  | - |  | - |  |
| DC_1-28_n77 | - |  | 0.2 |  | 0.5 |  |
| DC_1_n28-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_1-28_n78 | - |  | 0.2 |  | 0.5 |  |
| DC_1_n28-n78 | - |  | 0.2 |  | 0.5 |  |
| DC_1_n28-n79 | - |  | 0.2 |  | - |  |
| DC_1-32_n28 | - |  | - |  | 0.2 |  |
| DC_1-32_n78 | - |  | - |  | 0.5 |  |
| DC_1-38_n7 | - |  | 0.2 |  | - |  |
| DC_1-38_n28 | - |  | - |  | 0.2 |  |
| DC_1-38_n78 | - |  | - |  | 0.5 |  |
| DC_1_n38-n78 | - |  | - |  | 0.5 |  |
| DC_1-40-n78 | 0.2 |  | 0.45 |  | 0.55 |  |
| DC_1-41_n78 | - |  | - |  | 0.5 |  |
| DC_1_n41-n78 | - |  | - |  | 0.5 |  |
| DC_1-41_n3 | - |  | 03/0.54 |  | - |  |
| DC_1-41_n28 | - |  | - |  | 0.2 |  |
| DC_1-41_n77 | - |  | - |  | 0.5 |  |
| DC_1_n41-n77 | - |  | - |  | 0.5 |  |
| DC_1-41_n78 | - |  | - |  | 0.5 |  |
| DC_1-42_n3 | - |  | 0.5 |  | 0.2 |  |
| DC_1-42_n28 | - |  | 0.5 |  | 0.5 |  |
| DC_1-42_n77 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_1-42_n78 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_1-42_n79 | - |  | 0.5 |  | - |  |
| DC_1_n75-n78 | - |  | - |  | 0.5 |  |
| DC_1_n77-n79 | 0.2 |  | - |  | 0.5 |  |
| DC_1_SUL_n77-n80 | 0.2 |  | 0.5 |  | - |  |
| DC_1_SUL_n77-n84 | 0.2 |  | 0.5 |  | - |  |
| DC_1_n78-n79 | - |  | 0.5 |  | - |  |
| DC_1_SUL_n78-n80 | 0.2 |  | 0.5 |  | - |  |
| DC_1-SUL_n78-n84 | - |  | 0.5 |  | - |  |
| DC_2_n2-n66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2_n2-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2_n2-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2-4-n28 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_2-4_n38 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_2-4_n41 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_2-5_n12 | - |  | 0.5 |  | 0.3 |  |
| DC_2-5_n30 DC_2-2-5_n30 | 0.4 |  | - |  | 0.5 |  |
| DC_2-5_n48 | 0.2 |  | - |  | 0.5 |  |
| DC_2-5_n66 DC_2-5-5_n66 | 0.3 |  | - |  | 0.3 |  |
| DC_2-5_n77 DC_2-2-5_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2_n5-n77 | 0.2 |  | - |  | 0.5 |  |
| DC_2-5_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2-7_n38 DC_2-2-7_n38 | - |  | - |  | 0.2 |  |
| DC_2-7_n66 DC_2-7-7_n66 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_2_n7-n66 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_2-7_n71 | - |  | - |  | 0.2 |  |
| DC_2-7_n77 DC_2-7-7_n77 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_2_n7-n78 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_2-12_n5 DC_2-2-12_n5 | - |  | 0.3 |  | 0.5 |  |
| DC_2-12_n30 DC_2-2-12_n30 | 0.4 |  | - |  | 0.5 |  |
| DC_2-12_n66 DC_2-2-12_n66 | 0.3 |  | 0.5 |  | 0.3 |  |
| DC_2-12_n77 DC_2-2-12_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2-12_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2-13_n48 | 0.2 |  | - |  | 0.5 |  |
| DC_2-13_n66 DC_2-2-13_n66 | 0.3 |  | - |  | 0.3 |  |
| DC_2-13_n77 DC_2-2-13_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2-14_n5 DC_2-2-14_n5 | - |  | 0.3 |  | 0.5 |  |
| DC_2-14_n30 DC_2-2-14_n30 | 0.3 |  | - |  | 0.3 |  |
| DC_2-14_n66 DC_2-2-14_n66 | 0.3 |  | - |  | 0.3 |  |
| DC_2-14_n77 DC_2-2-14_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2_n25-n66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-28_n66 | 0.3 |  | 0.2 |  | 0.3 |  |
| DC_2-28_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2-29_n30 DC_2-2-29_n30 | 0.3 |  | - |  | 0.3 |  |
| DC_2-29_n66 DC_2-2-29_n66 | 0.3 |  | - |  | 0.3 |  |
| DC_2-29_n77 DC_2-2-29_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2-29-n78 | 0.2 |  | - |  | 0.5 |  |
| DC_2-30_n2 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_2-30_n5 DC_2-2-30_n5 | 0.4 |  | 0.5 |  | - |  |
| DC_2-30_n66 DC_2-2-30_n66 | 0.4 |  | 0.5 |  | 0.4 |  |
| DC_2-30_n77 DC_2-2-30_n77 | 0.2 |  | - |  | 0.5 |  |
| DC_2_n38-n66 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_2-38_n78 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2_n38-n78 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_2_n41-n66 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_2-48_n2 | 0.2 |  | 0.5 |  | 0.2 |  |
| DC_2-48_n5 | 0.2 |  | 0.5 |  | - |  |
| DC_2-48_n12 | 0.2 |  | 0.5 |  | - |  |
| DC_2-48_n48 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_2-48_n66 | 0.3 |  | 0.5 |  | 0.3 |  |
| DC_2-48_n77 DC_2-48-48_n77 DC_2-48-48-48_n77 | - |  | 0.2 |  | 0.1 |  |
| DC_2-48_n71 | 0.2 |  | 0.5 |  | - |  |
| DC_2-66_n2 DC_2-66-66_n2 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-66_n5 DC_2-2-66_n5 DC_2-66-66_n5 DC_2-2-66-66_n5 DC_2-66-66-66_n5 | 0.3 |  | 0.3 |  | - |  |
| DC_2-66-n7 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_2-66_n12 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_2-66_n25 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-66-n28 | 0.3 |  | 0.3 |  | 0.2 |  |
| DC_2-66_n30 DC_2-2-66_n30 DC_2-66-66_n30 DC_2-2-66-66_n30 | 0.4 |  | 0.4 |  | 0.5 |  |
| DC_2-66_n38 DC_2-2-66_n38 DC_2-66-66_n38 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_2-66_n41 | 0.3 |  | 0.5 |  | 0.51 / 12 |  |
| DC_2-66_n48 DC_2-66-66_n48 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_2-66_n66 DC_2-2-66-66_n66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2_(n)66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_2-66_n71 | 0.3 |  | 0.3 |  | - |  |
| DC_2_n66-n71 | 0.3 |  | 0.3 |  | - |  |
| DC_2-66_n77 DC_2-2-66_n77 DC_2-66-66_n77 DC_2-2-66-66_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2_n66-n77 DC_2-2_n66-n77 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_2-66_n78 DC_2-66-66_n78 DC_2_n66-n78 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_2-71_n66 DC_2-2-71_n66 | 0.3 |  | - |  | 0.3 |  |
| DC_2-71_n78 DC_2-2-71_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_2_n71-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3_n1-n28 | - |  | - |  | 0.2 |  |
| DC_3_n1-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3_n1-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3_n3-n41 | - |  | - |  | 03/0.54 |  |
| DC_3_n3-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_(n)3-n77 | - |  | 0.2 |  | 0.5 |  |
| DC_3_n3-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3-5_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3-5_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3-7_n38 | - |  | - |  | 0.2 |  |
| DC_3-7_n40 | - |  | 0.3 |  | 0.8 |  |
| DC_3-7_n77 DC_3-3-7_n77 DC_3-7-7_n77 DC_3-3-7-7_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3-7_n8 DC_3-3-7_n8 DC_3-7-7_n8 DC_3-3-7-7_n8 | - |  | - |  | 0.2 |  |
| DC_3-7_n78 DC_3-7-7_n78 DC_3-3-7_n78 DC_3-3-7-7_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3_n7-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3-8_n28 | - |  | 0.2 |  | 0.1 |  |
| DC_3-8_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3-8_n78 DC_3-3-8_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3_n8-n78 DC_3-3_n8-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3-11_n28 | 0.3 |  | 0.5 |  | 0.2 |  |
| DC_3-11_n77 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_3-18_n41 | - |  | - |  | 03 / 0.54 |  |
| DC_3-18-n77 | 0.2 |  | - |  | 0.5 |  |
| DC_3-18-n78 | 0.2 |  | - |  | 0.5 |  |
| DC_3-19_n77 | 0.2 |  | - |  | 0.5 |  |
| DC_3-19_n78 | 0.2 |  | - |  | 0.5 |  |
| DC_3-20_n28 | - |  | 0.1 |  | 0.1 |  |
| DC_3-20_n38 | - |  | 0.2 |  | - |  |
| DC_3_n20-n67 | - |  | 0.1 |  | 0.1 |  |
| DC_3-20_n78 | 0.2 |  | - |  | 0.5 |  |
| DC_3_n20-n78 | 0.2 |  | - |  | 0.5 |  |
| DC_3-21_n1 | 0.3 |  | 0.5 |  | - |  |
| DC_3-21_n28 | 0.3 |  | 0.5 |  | - |  |
| DC_3-21_n77 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_3-21_n78 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_3-21_n79 | 0.3 |  | 0.5 |  | - |  |
| DC_3_n26-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3-28_n1 | - |  | 0.2 |  | - |  |
| DC_3-28_n5 | - |  | 0.1 |  | 0.1 |  |
| DC_3-28_n41 | - |  | - |  | 03 / 0.54 |  |
| DC_3_n28-n75 | 0.5 |  | 0.5 |  | - |  |
| DC_3-28_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3_n28-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_3-28_n78 | 0.2 |  | - |  | 0.5 |  |
| DC_3_n28-n78 | 0.2 |  | - |  | 0.5 |  |
| DC_3-32_n28 | 0.5 |  | - |  | 0.5 |  |
| DC_3-32_n78 | 0.2 |  | - |  | 0.5 |  |
| DC_3-38_n7 | - |  | 0.2 |  | - |  |
| DC_3-38_n28 | - |  | - |  | 0.2 |  |
| DC_3-38_n78 | 0.2 |  | 0.4 |  | 0.5 |  |
| DC_3_n38-n78 | 0.5 |  | - |  | 0.5 |  |
| DC_3_n40-n41 | - |  | - |  | 03 / 0.54 |  |
| DC_3-40-n78 | 0.2 |  | 0.45 |  | 0.55 |  |
| DC_3-41_n3 | - |  | 03/0.54 |  | - |  |
| DC_3-41_n28 | - |  | 03/0.54 |  | - |  |
| DC_3-41_n41 | - |  | 03 / 0.54 |  | 03 / 0.54 |  |
| DC_3-(n)41 | - |  | 03 / 0.54 |  | 03 / 0.54 |  |
| DC_3-41-n77 | 0.2 |  | 03 / 0.54 |  | 0.54 |  |
| DC_3-41_n78 DC_3_n41-n78 | 0.2 |  | 03 / 0.54 |  | 0.54 |  |
| DC_3-41-n79 | 0.2 |  | 03 / 0.54 |  | - |  |
| DC_3_n41-n79 | 0.2 |  |  |  | - |  |
| DC_3_SUL_n41-n80 | - |  | 03 / 0.54 |  | - |  |
| DC_3-42_n1 | 0.2 |  | 0.5 |  | 0.2 |  |
| DC_3-42_n28 | 0.2 |  | 0.5 |  | 0.2 |  |
| DC_3-42_n77 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_3-42_n78 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_3-42_n79 | 0.2 |  | 0.5 |  | - |  |
| DC_3_n75-n78 | 0.2 |  | - |  | 0.5 |  |
| DC_3_n77-n79 | 0.2 |  | 0.5 |  | - |  |
| DC_3_SUL_n77-n80 | 0.2 |  | 0.5 |  | - |  |
| DC_3_SUL_n77-n84 | 0.2 |  | 0.5 |  | - |  |
| DC_3_n78-n79 | 0.2 |  | 0.5 |  | - |  |
| DC_3-SUL_n78-n80 | 0.2 |  | 0.5 |  | - |  |
| DC_3-SUL_n78-n82 | 0.2 |  | 0.5 |  | - |  |
| DC_3_SUL_n78-n84 | 0.2 |  | 0.5 |  | - |  |
| DC_4-7_n28 | 0.5 |  | 0.5 |  | 0.2 |  |
| DC_5_n1-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_5_n2-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_5_n3-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_5_n5-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_5-7_n66 | - |  | 0.5 |  | 0.5 |  |
| DC_5-7_n71 | - |  | - |  | 0.2 |  |
| DC_5-7_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_5-7_n78, DC_5-7-7_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_5_n7-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_5_(n)12 | 0.5 |  | 0.3 |  | 0.3 |  |
| DC_5-13_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_5-30_n2 | - |  | 0.5 |  | 0.4 |  |
| DC_5_30_n66 | - |  | 0.5 |  | 0.4 |  |
| DC_5-30_n77 | 0.2 |  | - |  | 0.5 |  |
| DC_5_n38-n66 | 0.2 |  | - |  | - |  |
| DC_5-48_n12 | 0.5 |  | - |  | 0.3 |  |
| DC_5-48_n77 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_5-66_n2 DC_5-5-66_n2 DC_5-66-66_n2 DC_5-5-66-66_n2 | - |  | 0.3 |  | 0.3 |  |
| DC_5-66-n7 | - |  | 0.5 |  | 0.5 |  |
| DC_5-66_n12 | - |  | 0.5 |  | 0.5 |  |
| DC_5-66_n30 DC_5-66-66_n30 | - |  | 0.4 |  | 0.5 |  |
| DC_5-66_n48 DC_5-66-66_n48 | - |  | 0.2 |  | 0.5 |  |
| DC_5-66_n77 DC_5-66-66_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_5_n66-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_5-66_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_5_n66-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_7_n1-n8 DC_7-7_n1-n8 | - |  | - |  | 0.2 |  |
| DC_7_n1-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_7_n2-n66 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_7_n2-n71 | 0.3 |  | 0.3 |  | - |  |
| DC_7_n2-n78 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_7_n3-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_7_n7-n78 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7-8_n1 DC_7-7-8_n1 | - |  | 0.2 |  | - |  |
| DC_7-8_n28 | - |  | 0.2 |  | 0.1 |  |
| DC_7-8_n40 | - |  | 0.2 |  | 0.5 |  |
| DC_7_n8-n40 | - |  | 0.2 |  | 0.5 |  |
| DC_7-8_n3 | - |  | 0.2 |  | - |  |
| DC_7-8_n77 | - |  | 0.2 |  | 0.5 |  |
| DC_7-8_n78 DC_7-7-8_n78 | - |  | 0.2 |  | 0.5 |  |
| DC_7_n8-n78 DC_7-7_n8-n78 | - |  | 0.2 |  | 0.5 |  |
| DC_7-12_n66 | 0.5 |  | 0.1 |  | 0.5 |  |
| DC_7-12_n78 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_7-13_n66 | 0.5 |  | - |  | 0.5 |  |
| DC_7-20_n28 | - |  | 0.2 |  | 0.2 |  |
| DC_7-20_n38 | - |  | - |  | 0.2 |  |
| DC_7-20_n78 | - |  | - |  | 0.5 |  |
| DC_7-25_n77 DC_7-7-25_n77 DC_7-25-25_n77 DC_7-7-25-25_n77 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_7-25_n78 DC_7-7-25_n78 DC_7-25-25_n78 DC_7-7-25-25_n78 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_7_n25-n66 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_7_n26-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_7-28_n1 DC_7-7-28_n1 | - |  | 0.2 |  | - |  |
| DC_7_n28-n40 | - |  | - |  | 0.5 |  |
| DC_7-28_n40 | - |  | - |  | 0.5 |  |
| DC_7-28_n66 | - |  | 0.2 |  | - |  |
| DC_7-28_n78 | - |  | - |  | 0.5 |  |
| DC_7_n28-n78 | - |  | - |  | 0.5 |  |
| DC_7-29_n78 | - |  | - |  | 0.5 |  |
| DC_7-32_n8 | - |  | - |  | 0.2 |  |
| DC_7-32_n28 | - |  | - |  | 0.2 |  |
| DC_7-32_n78 | - |  | - |  | 0.5 |  |
| DC_7-38_n78 | - |  | - |  | 0.5 |  |
| DC_7_n38-n78 | - |  | - |  | 0.5 |  |
| DC_7-40_n1 | 0.3 |  | 0.8 |  | - |  |
| DC_7_n1-n40 | 0.3 |  | - |  | 0.8 |  |
| DC_7-40-n78 | - |  | 0.45 |  | 0.55 |  |
| DC_7-46_n78 | - |  | - |  | 0.5 |  |
| DC_7-66_n7 DC_7-66-66_n7 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7-66_n25 DC_7-7-66_n25 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_7-66-n28 | 0.5 |  | 0.5 |  | 0.2 |  |
| DC_7-66_n38 | - |  | - |  | 0.2 |  |
| DC_7-66_n66 DC_7-7-66_n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7-66_n77 DC_7-7-66_n77 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7_n66-n77 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7_n66-n78 DC_7-7_n66-n78 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_7-66_n71 DC_7-66-66_n71 | 0.5 |  | 0.5 |  | 0.1 |  |
| DC_7_n66-n71 | 0.5 |  | 0.5 |  | 0.1 |  |
| DC_7_n71-n78 | - |  | 0.2 |  | 0.5 |  |
| DC_7-71_n66 | 0.5 |  | 0.1 |  | 0.5 |  |
| DC_7-71_n78 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_7_SUL_n78-n80 | 0.2 |  | 0.5 |  | - |  |
| DC_7_n78-n79 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_8_n1-n28 | 0.2 |  | - |  | 0.2 |  |
| DC_8_n1-n40 | 0.2 |  | - |  | 0.5 |  |
| DC_8_n1-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_8_n3-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_8_n3-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_8_n3-n79 | - |  | - |  | 0.5 |  |
| DC_8-11_n1 | 0.3 |  | 0.4 |  | 0.3 |  |
| DC_8_n1-n78 | 0.2 |  | - |  | 0.5 |  |
| DC_8_n3-n28 | 0.2 |  | - |  | 0.1 |  |
| DC_8-11_n3 | - |  | 0.3 |  | 0.5 |  |
| DC_8-11_n28 | 0.2 |  | - |  | 0.2 |  |
| DC_8-11_n77 | 0.2 |  | - |  | 0.5 |  |
| DC_8-11_n78 | 0.2 |  | - |  | 0.2 |  |
| DC_8-11_n79 | 0.3 |  | 0.4 |  | - |  |
| DC_8-20_n28 | 0.2 |  | - |  | 0.1 |  |
| DC_8-20_n78 | 0.2 |  | - |  | 0.5 |  |
| DC_8_n28-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_8-32_n3 | - |  | 0.5 |  | 0.3 |  |
| DC_8_n39-n40 | - |  | 0.3 |  | 0.3 |  |
| DC_8-40_n1 | 0.2 |  | 0.5 |  | - |  |
| DC_8-40-n78 | 0.2 |  | 0.45 |  | 0.55 |  |
| DC_8-41_n3 | - |  | 03/0.54 |  | - |  |
| DC_8-41_n77 | 0.2 |  | - |  | 0.5 |  |
| DC_8-42_n1 | 0.2 |  | 0.5 |  | - |  |
| DC_8-42_n3 | 0.2 |  | 0.5 |  | 0.2 |  |
| DC_8-42_n28 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_8-42_n77 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_8_SUL_n78-n80 | 0.2 |  | 0.5 |  | - |  |
| DC_8_n28-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_8_n77-n79 | 0.2 |  | 0.5 |  | - |  |
| DC_8-SUL_n78-n81 | 0.2 |  | 0.2 |  | - |  |
| DC_11_n1-n77 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_11_n3-n28 | 0.3 |  | 0.5 |  | 0.2 |  |
| DC_11_n3-n77 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_11_n3-n79 | 0.3 |  | 0.5 |  | 0.5 |  |
| DC_11-18_n77 | - |  | - |  | 0.5 |  |
| DC_11-18_n78 | - |  | - |  | 0.5 |  |
| DC_11_n28-n77 | - |  | 0.2 |  | 0.5 |  |
| DC_12_(n)5 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_12_n2-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_12_n7-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_12_n7-n78 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_12-30_n2 | - |  | 0.5 |  | 0.4 |  |
| DC_12-30_n5 | 0.5 |  | 0.5 |  | - |  |
| DC_12-30_n66 | 0.5 |  | 0.5 |  | 0.4 |  |
| DC_12-30_n77 | 0.2 |  | - |  | 0.5 |  |
| DC_12-48_n5 | 0.3 |  | - |  | 0.5 |  |
| DC_12-66_n2 | 0.5 |  | 0.3 |  | 0.3 |  |
| DC_12-66_n5 DC_12-66-66_n5 | 0.5 |  | 0.5 |  | - |  |
| DC_12-66_n25 | 0.5 |  | 0.3 |  | 0.3 |  |
| DC_12-66_n30 DC_12-66-66_n30 | 0.5 |  | 0.4 |  | 0.5 |  |
| DC_12-66_n41 | 0.5 |  | 0.5 |  | 0.51 / 12 |  |
| DC_12-66_n77 DC_12-66-66_n77 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_12-66_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_12_n66-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_13_n2-n77 | - |  | 0.2 |  | 0.5 |  |
| DC_13_n5-n48 | 0.3 |  | 0.5 |  | - |  |
| DC_13_n5-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_13_n7-n78 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_13_n25-n66 | - |  | 0.3 |  | 0.3 |  |
| DC_13-48_n2 | - |  | 0.5 |  | 0.2 |  |
| DC_13-48_n66 | - |  | 0.5 |  | 0.2 |  |
| DC_13_n48-n66 | - |  | 0.5 |  | 0.2 |  |
| DC_13-48_n77 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_13-66_n2 DC_13-66-66_n2 | - |  | 0.3 |  | 0.3 |  |
| DC_13-66_n48 DC_13-66-66_n48 | - |  | 0.2 |  | 0.5 |  |
| DC_13-66_n77 DC_13-66-66_n77 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_13_n66-n77 | - |  | 0.2 |  | 0.5 |  |
| DC_14-30_n2 | - |  | 0.5 |  | 0.4 |  |
| DC_14-30_n5 | 0.5 |  | 0.5 |  | - |  |
| DC_14-30_n66 | - |  | 0.5 |  | 0.4 |  |
| DC_14-30_n77 | 0.2 |  | - |  | 0.5 |  |
| DC_14-66_n2 DC_14-66-66_n2 | - |  | 0.3 |  | 0.3 |  |
| DC_14-66_n5 DC_14-66-66_n5 | 0.5 |  | 0.5 |  | - |  |
| DC_14-66_n30 DC_14-66-66_n30 | - |  | 0.4 |  | 0.5 |  |
| DC_14-66_n77 DC_14-66-66_n77 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_18_n3-n77 | - |  | 0.2 |  | 0.5 |  |
| DC_18_n3-n78 | - |  | 0.2 |  | 0.5 |  |
| DC_18-28_n77 | - |  | - |  | 0.5 |  |
| DC_18_n28-n77 | - |  | - |  | 0.5 |  |
| DC_18-28_n78 | - |  | - |  | 0.5 |  |
| DC_18_n28-n78 | - |  | - |  | 0.5 |  |
| DC_18-41_n3 | - |  | 03 / 0.54 |  | - |  |
| DC_18-41_n77 | - |  | - |  | 0.5 |  |
| DC_18_n41-n77 | - |  | - |  | 0.5 |  |
| DC_18-41_n78 | - |  | - |  | 0.5 |  |
| DC_18_n41-n78 | - |  | - |  | 0.5 |  |
| DC_18-42_n77 | - |  | 0.5 |  | 0.5 |  |
| DC_18-42_n78 | - |  | 0.5 |  | 0.5 |  |
| DC_18-42_n79 | - |  | 0.5 |  | - |  |
| DC_19_n1-n77 | - |  | - |  | 0.5 |  |
| DC_19_n1-n78 | - |  | - |  | 0.5 |  |
| DC_19_n1-n79 | 0.3 |  | 0.3 |  | - |  |
| DC_19-21_n77 | - |  | - |  | 0.5 |  |
| DC_19-21_n78 | - |  | - |  | 0.5 |  |
| DC_19-42_n1 | - |  | 0.5 |  | - |  |
| DC_19-42_n77 | - |  | 0.5 |  | 0.5 |  |
| DC_19-42_n78 | - |  | 0.5 |  | 0.5 |  |
| DC_19-42_n79 | - |  | 0.5 |  | - |  |
| DC_19_n77-n79 | - |  | 0.5 |  | - |  |
| DC_19_n78-n79 | - |  | 0.5 |  | - |  |
| DC_20_n1-n28 | - |  | 0.2 |  | 0.2 |  |
| DC_20_n1-n67 | - |  | 0.2 |  | 0.2 |  |
| DC_20_n3-n67 | 0.1 |  | - |  | 0.1 |  |
| DC_20_n1-n78 | - |  | - |  | 0.5 |  |
| DC_20_n3-n78 | - |  | 0.2 |  | 0.5 |  |
| DC_20_n7-n28 | 0.2 |  | - |  | 0.2 |  |
| DC_20_n7-n78 | - |  | - |  | 0.5 |  |
| DC_20_n8-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_20-28_n1 | 0.2 |  | 0.2 |  | - |  |
| DC_20-28_n3 | 0.3 |  | 0.2 |  | 0.3 |  |
| DC_20_n28-n75 | - |  | 0.2 |  | - |  |
| DC_20_n28-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_20-32_n28 | - |  | - |  | 0.2 |  |
| DC_20-32_n78 | - |  | - |  | 0.5 |  |
| DC_20-38_n1 | 0.2 |  | - |  | - |  |
| DC_20-38_n78 | - |  | 0.4 |  | 0.5 |  |
| DC_20_n38-n78 | 0.2 |  | - |  | 0.5 |  |
| DC_20-40-n78 | 0.2 |  | 0.45 |  | 0.55 |  |
| DC_20_n41-n78 | - |  | - |  | 0.5 |  |
| DC_20-(n)41 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_20_n75-n78 | - |  | - |  | 0.5 |  |
| DC_20_n76-n78 | - |  | - |  | 0.5 |  |
| DC_20_SUL_n78-n80 | - |  | 0.5 |  | - |  |
| DC_20-SUL_n78-n82 | - |  | 0.5 |  | - |  |
| DC_20-SUL_n78-n83 | 0.2 |  | 0.5 |  | - |  |
| DC_20_n78-n92 | - |  | 0.5 |  | - |  |
| DC_21_n1-n77 | - |  | - |  | 0.5 |  |
| DC_21_n1-n78 | - |  | 0.2 |  | 0.5 |  |
| DC_21_n28-n77 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_21_n28-n78 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_21-42_n1 | - |  | 0.5 |  | - |  |
| DC_21-42_n77 | - |  | 0.5 |  | 0.5 |  |
| DC_21-42_n78 | - |  | 0.5 |  | 0.5 |  |
| DC_21-42_n79 | - |  | 0.5 |  | - |  |
| DC_21_n77-n79 | - |  | 0.5 |  | - |  |
| DC_21_n78-n79 | - |  | 0.5 |  | - |  |
| DC_25-41_n41 DC_25_(n)41 DC_25-25-41_n41 DC_25-25_(n)41 | - |  | 01 / 0.52 |  | 01 / 0.52 |  |
| DC_25-66_n77 DC_25-25-66_n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_25-66_n78 DC_25-25-66_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_28-SUL_n78-n83 | 0.2 |  | 0.5 |  | - |  |
| DC_28_n1-n40 | 0.2 |  | - |  | - |  |
| DC_28_n1-n78 | 0.2 |  | - |  | 0.5 |  |
| DC_28_n3-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_28_n3-n78 | - |  | 0.2 |  | 0.5 |  |
| DC_28_n7-n78 | - |  | - |  | 0.5 |  |
| DC_28-32_n1 | 0.2 |  | - |  | - |  |
| DC_28-38_n1 | 0.2 |  | - |  | - |  |
| DC_28-40_n78 | 0.2 |  | 0.4 |  | 0.5 |  |
| DC_28_n40-n78 | 0.2 |  | 0.45 |  | 0.55 |  |
| DC_28-41_n77 | 0.2 |  | - |  | 0.5 |  |
| DC_28-41_n78 | 0.2 |  | - |  | 0.5 |  |
| DC_28-41_n79 | - |  | - |  | 0.5 |  |
| DC_28-42_n77 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_28-42_n78 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_28-42_n79 | 0.2 |  | 0.5 |  | - |  |
| DC_28-66_n7 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_28-66_n66 | 0.2 |  | - |  | - |  |
| DC_29-30_n2 | - |  | 0.3 |  | 0.5 |  |
| DC_29-30-n66 | - |  | 0.5 |  | 0.4 |  |
| DC_29-30_n77 | 0.2 |  | - |  | 0.5 |  |
| DC_29-66_n2 DC_29-66-66_n2 | - |  | 0.3 |  | 0.3 |  |
| DC_29-66_n30 DC_29-66-66_n30 | - |  | 0.4 |  | 0.5 |  |
| DC_29-66_n77 DC_29-66-66_n77 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_29-66-n78 | - |  | 0.2 |  | 0.5 |  |
| DC_30-66_n2 | 0.5 |  | 0.4 |  | 0.4 |  |
| DC_30-66_n5 DC_30-66-66_n5 DC_30-66-66-66_n5 | - |  | 0.4 |  | 0.5 |  |
| DC_30-66-n66 | 0.5 |  | 0.5 |  | 0.4 |  |
| DC_30-66_n77 DC_30-66-66_n77 | 0.5 |  | 0.4 |  | 0.5 |  |
| DC_32-38_n28 | - |  | - |  | 0.2 |  |
| DC_38_n3-n78 | - |  | 0.5 |  | 0.5 |  |
| DC_38_n28-n78 |  |  | 0.2 |  | 0.5 |  |
| DC_39_n40-n79 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_39_n41-n79 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_40_n1-n78 | 0.4 |  | 0.2 |  | 0.5 |  |
| DC_41_n1-n78 | - |  | - |  | 0.5 |  |
| DC_41_n1-n3 | 03 / 0.54 |  | - |  | - |  |
| DC_41_n1-n77 | - |  | - |  | 0.5 |  |
| DC_40-42_n77 | 0.45 |  | 0.55 |  | 0.55 |  |
| DC_40-42_n78 | 0.45 |  | 0.55 |  | 0.55 |  |
| DC_41_n3-n41 | 03 / 0.54 |  | - |  | 03 / 0.54 |  |
| DC_41_n3-n77 | 03 / 0.54 |  | 0.2 |  | 0.5 |  |
| DC_41_n3-n78 | 03 / 0.54 |  | 0.2 |  | 0.5 |  |
| DC_41_n28-n77 | - |  | 0.2 |  | 0.5 |  |
| DC_41_n28-n78 | - |  | 0.2 |  | 0.5 |  |
| DC_41_n41-n77 | - |  | - |  | 0.5 |  |
| DC_41_n41-n78 | - |  | - |  | 0.5 |  |
| DC_(n)41-n78 | - |  | - |  | 0.5 |  |
| DC_41-42_n77 | - |  | 0.5 |  | 0.5 |  |
| DC_41-42_n78 | - |  | 0.5 |  | 0.5 |  |
| DC_41-42_n79 | - |  | 0.5 |  | - |  |
| DC_42_n1-n3 | 0.5 |  | - |  | 0.2 |  |
| DC_42_n1-n77 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_42_n1-n78 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_42_n1-n79 | 0.5 |  | - |  | - |  |
| DC_42_n3-n28 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_42_n3-n77 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_42_n28-n77 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_46-48_n5 | - |  | 0.5 |  | - |  |
| DC_46-48_n66 | - |  | 0.5 |  | 0.3 |  |
| DC_48_n25-n48 | 0.4 |  | 0.3 |  | 0.4 |  |
| DC_48_n48-n66 | 0.4 |  | 0.4 |  | 0.3 |  |
| DC_46-66_n41 | - |  | 0.5 |  | 0.51 / 12 |  |
| DC_48-66_n2 | 0.5 |  | 0.3 |  | 0.3 |  |
| DC_48-66_n5 | 0.5 |  | 0.2 |  | - |  |
| DC_48-66_n12 | 0.5 |  | 0.2 |  | - |  |
| DC_48-66_n25 | 0.5 |  | 0.2 |  | 0.2 |  |
| DC_48-66_n48 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_48-66_n66 | 0.5 |  | 0.2 |  | 0.2 |  |
| DC_48-66_n71 | 0.5 |  | 0.2 |  | - |  |
| DC_48-66_n77 | 0.5 |  | 0.2 |  | 0.5 |  |
| DC_66_n2-n38 | 0.5 |  | 0.3 |  | 0.5 |  |
| DC_66_n2-n66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_66_n2-n71 | 0.3 |  | 0.3 |  | - |  |
| DC_66_n2-n77 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_66_n2-n78 | 0.3 |  | 0.3 |  | 0.5 |  |
| DC_66_n5-n48 | 0.2 |  | - |  | 0.5 |  |
| DC_66_n5-n77 | 0.2 |  | - |  | 0.5 |  |
| DC_66_n7-n78 | 0.2 |  | 0.5 |  | 0.5 |  |
| DC_66_n25-n41 | 0.5 |  | 0.5 |  | 0.51 / 12 |  |
| DC_66_n25-n48 | 0.3 |  | 0.3 |  | 0.4 |  |
| DC_66_n25-n66 | 0.3 |  | 0.3 |  | 0.3 |  |
| DC_66_n25-n71 | 0.3 |  | 0.5 |  | - |  |
| DC_66_n38-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_66_n38-n71 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_66_n38-n78 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_66_n41-n71 | 0.5 |  | 0.51 / 12 |  | 0.5 |  |
| DC_66_n66-n77 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_66_n66-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_66-71_n38 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_66-71_n41 | 0.5 |  | 0.5 |  | 0.51 / 12 |  |
| DC_66-71_n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_66_n71-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_66-SUL_n78-n86 | 0.2 |  | 0.5 |  | - |  |
| DC_71_n2-n66 | - |  | 0.3 |  | 0.3 |  |
| DC_71_n2-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| DC_71_n38-n66 | 0.5 |  | 0.5 |  | 0.5 |  |
| DC_71_n38-n78 | 0.2 |  | - |  | 0.5 |  |
| DC_71_n66-n78 | 0.2 |  | 0.2 |  | 0.5 |  |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 6: This band is subject to IMD3 also which MSD is not specified. NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |  |  |  |

7.3B.3.3.3	ΔRIB,c for EN-DC four bands

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-3_n3-n41 | - | - | - | 03 / 0.54 |
| DC_1-3_n3-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-5_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3_n3-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-5_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-7_n28 | - | - | - | 0.2 |
| DC_1-3-7_n40 | - | - | 0.3 | 0.8 |
| DC_1-3-7_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-7_n78 DC_1-3-7-7_n78 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_1-3_n7-n78 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_1-3-8_n28 | - | - | 0.2 | 0.2 |
| DC_1-3-8_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-8_n3-n79 | - | - | - | 0.5 |
| DC_1-3-8_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3_n8-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-11_n28 | - | 0.3 | 0.5 | 0.2 |
| DC_1-3-11_n77 | 0.2 | 0.3 | 0.5 | 0.5 |
| DC_1-3-18_n28 | - | - | - | 0.2 |
| DC_1-3-18_n41 | - | - | - | 0.26 |
| DC_1-3-28_n3 | - | - | 0.2 | - |
| DC_1-3-18_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-18_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-19_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-20_n28 | - | - | 0.2 | 0.2 |
| DC_1-3-20_n41 | - | - | - | 01 / 0.54 |
| DC_1-3-20_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-21_n77 | 0.2 | 0.3 | 0.5 | 0.5 |
| DC_1-3-21_n78 | 0.2 | 0.3 | 0.5 | 0.5 |
| DC_1-3-21_n79 | - | 0.3 | 0.5 | - |
| DC_1-3_n26-n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-28_n5 | - | - | 0.2 | 0.2 |
| DC_1-3-28_n7 | - | - | 0.2 | - |
| DC_1-3-28_n40 | - | - | 0.2 | - |
| DC_1-3_n28-n75 | 0.2 | - | 0.2 | - |
| DC_1-3-28_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3_n28-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1_n3-n28-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-28_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-3-28_n79 | 0.2 | 0.2 | 0.2 | - |
| DC_1-3_n28-n79 | 0.2 | 0.2 | 0.2 | - |
| DC_1_n3-n28-n79 | 0.2 | 0.2 | 0.2 | - |
| DC_1-3-32_n28 | - | 0.5 | - | 0.5 |
| DC_1-3-32_n78 | - | - | - | 0.5 |
| DC_1-3-38_n28 | - | - | - | 0.2 |
| DC_1-3_n38-n78 | - | 0.2 | - | 0.5 |
| DC_1-3-38_n78 | 0.2 | 0.2 | 0.4 | 0.5 |
| DC_1-3-40_n78 | 0.2 | 0.2 | 0.48 | 0.58 |
| DC_1-3_n40-n78 | - | 0.2 | 0.45 | 0.55 |
| DC_1-3-41_n3 | - | - | 03 / 0.54 | - |
| DC_1-3-41_n28 | - | - | 03 / 0.54 | 0.2 |
| DC_1-3-41_n41 | - | - | 03 / 0.54 | 03 / 0.54 |
| DC_1-3_(n)41 | - | - | 03 / 0.54 | 03 / 0.54 |
| DC_1-3-41_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3_n41-n77 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-41_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3_n41-n78 | 0.2 | 0.2 | - | 0.5 |
| DC_1-3-41_n79 | - | - | 03 / 0.54 | - |
| DC_1-3-42_n28 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_1-3-42_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_1-3-42_n78 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_1-3-42_n79 | 0.2 | 0.2 | 0.5 | - |
| DC_1-3_n75-n78 | - | - | - | 0.5 |
| DC_1-3_n77-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_1_n3-n77-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_1-3_n78-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_1-3_SUL_n78-n80 | 0.2 | 0.2 | 0.5 | - |
| DC_1-5-7_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-5-7_n78 DC_1-5-7-7_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7_n3-n38 | - | - | - | 0.2 |
| DC_1-7_n3-n78 | - | - | - | 0.5 |
| DC_1-7_n7-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7-8_n28 | - | - | 0.2 | 0.2 |
| DC_1-7-8_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7_n8-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7-20_n28 | - | - | 0.2 | 0.2 |
| DC_1-7-20_n38 | - | - | - | 0.2 |
| DC_1-7-20_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7_n26-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7-28_n3 | - | - | 0.2 | - |
| DC_1-7-28_n5 | - | - | 0.2 | 0.2 |
| DC_1-7-28_n7 | - | - | 0.2 | - |
| DC_1-7-28_n40 | - | 0.3 | 0.2 | 0.8 |
| DC_1-7-28_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-7-32_n8 | - | - | - | 0.2 |
| DC_1-7-32_n28 | - | - | - | 0.2 |
| DC_1-7-32_n78 | 0.6 | 0.6 | - | 0.8 |
| DC_1-7-38_n8 | - | - | 0.2 | - |
| DC_1-7-38_n28 | - | - | 0.2 | 0.2 |
| DC_1-7-38_n78 | 0.6 | 0.6 | - | 0.8 |
| DC_1-7-40_n78 | 0.2 | - | 0.48 | 0.58 |
| DC_1-7_n40-n78 | 0.2 | - | 0.4 | 0.5 |
| DC_1-7_n75-n78 | 0.6 | 0.6 | - | 0.8 |
| DC_1-8_n3-n28 | - | 0.2 | - | 0.2 |
| DC_1-8_n3-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-8-11_n3 | - | - | 0.3 | 0.5 |
| DC_1-8-11_n28 | - | 0.2 | - | 0.2 |
| DC_1-8-11_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_1-8-11_n78 | - | 0.2 | - | 0.5 |
| DC_1-8-20_n28 | - | 0.2 | 0.2 | 0.2 |
| DC_1-8-20_n78 | - | 0.2 | - | 0.5 |
| DC_1-8-28_n3 | - | 0.2 | 0.2 | - |
| DC_1-8_n28-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-8-28_n78 | - | 0.2 | 0.2 | 0.5 |
| DC_1-8_n28-n78 | - | 0.2 | 0.2 | 0.5 |
| DC_1-8_n28-n79 | 0.3 | 0.3 | 0.6 | 0.5 |
| DC_1-8-32_n3 | - | - | 0.5 | 0.3 |
| DC_1-8-32_n78 | - | 0.2 | - | 0.5 |
| DC_1-8_n40-n78 | - | 0.2 | 0.4 | 0.5 |
| DC_1-8-40_n78 | 0.2 | 0.2 | 0.48 | 0.58 |
| DC_1-8-42_n3 | - | 0.2 | 0.5 | 0.2 |
| DC_1-8-42_n28 | - | 0.2 | 0.5 | 0.5 |
| DC_1-8-42_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_1-8_n77-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_1-11_n3-n28 | - | 0.3 | 0.5 | 0.2 |
| DC_1-11_n3-n77 | 0.2 | 0.3 | 0.5 | 0.5 |
| DC_1-11-18_n77 | 0.2 | - | - | 0.5 |
| DC_1-11-18_n78 | - | - | - | 0.5 |
| DC_1-11_n28-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_1-18_n3-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_1-18_n3-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_1-11_n3-n79 | - | 0.3 | 0.5 | 0.5 |
| DC_1-11-18_n3 | - | 0.5 | - | 0.3 |
| DC_1-11-18_n28 | - | - | - | 0.1 |
| DC_1-11_n77-n79 | 0.2 | - | 0.5 | - |
| DC_1-18_n28-n41 | - | - | 0.2 | - |
| DC_1-18-28_n77 | - | - | - | 0.5 |
| DC_1-18_n28-n77 | - | - | - | 0.5 |
| DC_1-18-28_n78 | - | - | - | 0.5 |
| DC_1-18_n28-n78 | - | - | - | 0.5 |
| DC_1-18-41_n3 | - | - | 03 / 0.54 | - |
| DC_1-18-41_n77 | 0.2 | - | - | 0.5 |
| DC_1-18_n41-n77 | 0.2 | - | - | 0.5 |
| DC_1-18-41_n78 | - | - | - | 0.5 |
| DC_1-18_n41-n78 | - | - | - | 0.5 |
| DC_1-18-42_n77 | - | - | 0.5 | 0.5 |
| DC_1-18-42_n78 | - | - | 0.5 | 0.5 |
| DC_1-18-42_n79 | - | - | 0.5 | - |
| DC_1-19-42_n77 | 0.2 | - | 0.5 | 0.5 |
| DC_1-19-42_n78 | - | - | 0.5 | 0.5 |
| DC_1-19-42_n79 | - | - | 0.5 | - |
| DC_1-19_n77-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_1-19_n78-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_1-20_n3-n78 | - | - | - | 0.5 |
| DC_1-20_n7-n78 | - | - | - | 0.5 |
| DC_1-20_n8-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-20-28_n3 | - | 0.2 | 0.2 | - |
| DC_1-20_n28-n75 | - | 0.2 | 0.2 | - |
| DC_1-20-28_n78 | - | 0.2 | 0.2 | 0.5 |
| DC_1-20_n28-n78 | - | 0.2 | 0.2 | 0.5 |
| DC_1-20-32_n8 | 0.5 | 0.4 | - | 0.4 |
| DC_1-20-32_n28 | - | 0.2 | - | 0.2 |
| DC_1-20-32_n78 | - | - | - | 0.5 |
| DC_1-20-38_n78 | - | - | 0.4 | 0.5 |
| DC_1-20-40_n78 | - | - | - | 0.88 |
| DC_1-20_n41-n78 | - | - | - | 0.5 |
| DC_1-21_n28-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_1-21_n28-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_1-21_n28-n79 | 0.3 | - | 0.3 | - |
| DC_1-21-42_n77 | 0.2 | - | 0.5 | 0.5 |
| DC_1-21-42_n78 | - | - | 0.5 | 0.5 |
| DC_1-21-42_n79 | - | - | 0.5 | - |
| DC_1-21_n77-n79 | - | - | 0.5 | - |
| DC_1-21_n78-n79 | - | - | 0.5 | - |
| DC_1-28_n3-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-28_n3-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-28_n7-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_1-28-32_n3 | - | 0.2 | - | - |
| DC_1-28-40_n78 | - | 0.2 | 0.45 | 0.55 |
| DC_1-28_n40-n78 | - | 0.2 | 0.45 | 0.55 |
| DC_1-28-42_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_1-28-42_n78 | - | 0.2 | 0.5 | 0.5 |
| DC_1-28-42_n79 | - | 0.2 | 0.5 | - |
| DC_1_n28-n77-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_1_n28-n78-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_1-38_n3-n78 | - | - | 0.2 | 0.5 |
| DC_1-38_n7-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_1-38_n28-n78 | - | - | 0.2 | 0.5 |
| DC_1-41_n3-n41 | - | 03 / 0.54 | - | 03 / 0.54 |
| DC_1-41_n3-n77 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_1-41_n3-n78 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_1-41_n28-n41 | - | 03 / 0.54 | - | 03 / 0.54 |
| DC_1-41_n28-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_1-41_n28-n78 | - | - | 0.2 | 0.5 |
| DC_1-41_n41-n77 | - | - | - | 0.5 |
| DC_1-41_n41-n78 | - | - | - | 0.5 |
| DC_1-41-42_n77 | - | - | 0.5 | 0.5 |
| DC_1-41-42_n78 | - | - | 0.5 | 0.5 |
| DC_1-41-42_n79 | - | - | 0.5 | - |
| DC_1-41-42_n79 | - | - | 0.5 | - |
| DC_1-42_n3-n28 | - | 0.5 | 0.2 | 0.5 |
| DC_1-42_n3-n77 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_1-42_n28-n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_1-42_n77-n79 | 0.2 | 0.5 | 0.5 | - |
| DC_1-42_n78-n79 | 0.2 | 0.5 | 0.5 | - |
| DC_2-4-7_n28 | 0.3 | 0.5 | 0.5 | 0.2 |
| DC_2-5_n2-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-5_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-5_n5-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-5-7_n66 DC_2-2-5-7_n66 DC_2-5-7-7_n66 | 0.3 | - | 0.5 | 0.5 |
| DC_2-5-7_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-5_(n)12 | - | 0.5 | 0.3 | 0.3 |
| DC_2-12_(n)5 | - | 0.5 | 0.5 | - |
| DC_2-5-30_n2 | 0.4 | - | 0.5 | 0.4 |
| DC_2-5-30_n66 | 0.4 | - | 0.5 | 0.4 |
| DC_2-5-30_n77 DC_2-2-5-30_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_2-5-48_n12 | 0.2 | 0.5 | 0.5 | 0.3 |
| DC_2-5-48_n71 | 0.2 | - | 0.5 | - |
| DC_2-5-48_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_2-5-66_n2 | 0.3 | - | 0.3 | 0.3 |
| DC_2-5-66_n5 | 0.3 | - | 0.3 | - |
| DC_2-5-66_n7 | 0.3 | - | 0.5 | 0.5 |
| DC_2-5-66_n12 | 0.2 | 0.5 | 0.5 | 0.3 |
| DC_2-5-66_n30 DC_2-2-5-66_n30 DC_2-5-66-66_n30 | 0.4 | - | 0.4 | 0.5 |
| DC_2-5-66_n48 DC_2-5-66-66_n48 | 0.3 | - | 0.3 | 0.5 |
| DC_2-5-66_n66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-5-66_n71 | 0.3 | - | 0.3 | - |
| DC_2-5-66_n77 DC_2-2-5-66_n77 DC_2-5-66-66_n77 | 0.3 | - | 0.3 | 0.5 |
| DC_2-5_n66-n77 | 0.3 | 0.2 | 0.3 | 0.5 |
| DC_2-5-66_n78 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_2-5_n66-n78 | 0.3 | - | 0.3 | 0.5 |
| DC_2-7_n2-n78 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_2-7-12_n66 DC_2-2-7-12_n66 | 0.3 | 0.5 | 0.5 | 0.3 |
| DC_2-7-12_n78 DC_2-2-7-12_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-7-13_n66 DC_2-7-7-13_n66 DC_2-2-7-7-13_n66 | 0.3 | 0.5 | - | 0.5 |
| DC_2-7_n25-n66 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_2-7-28_n66 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_2-7-28_n78 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_2-7-29_n78 DC_2-7-7-29_n78 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_2-7_n38-n66 DC_2-7-7_n38-n66 | 0.3 | - | - | 0.5 |
| DC_2-7_n38-n78 DC_2-7-7_n38-n78 | 0.2 | - | - | 0.5 |
| DC_2-7-66_n2 | 0.3 | 0.5 | 0.5 | 0.3 |
| DC_2-7-66_n7 DC_2-7-66-66_n7 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n25 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n28 | 0.3 | 0.5 | 0.5 | 0.2 |
| DC_2-7-66_n38 DC_2-2-7-66_n38 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n66 DC_2-7-7-66_n66 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n71 DC_2-2-7-66_n71 | 0.3 | 0.5 | 0.5 | - |
| DC_2-7-66_n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_2-7_n66-n77 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-66_n78 DC_2-2-7-66_n78 DC_2-7-7-66_n78 DC_2-7-66-66_n78 DC_2-7-7-66-66_n78 | 0.3 | - | 0.3 | 0.5 |
| DC_2-7_n66-n78 DC_2-7-7_n66-n78 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_2-7-71_n2 | - | - | 0.2 | - |
| DC_2-7-71_n66 DC_2-2-7-71_n66 | 0.3 | 0.5 | - | 0.3 |
| DC_2-7-71_n78 DC_2-2-7 -71_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-7_n71-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-12_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-12-30_n2 | 0.4 | - | 0.5 | 0.4 |
| DC_2-12-30_n66 | 0.4 | 0.5 | 0.5 | 0.4 |
| DC_2-12-30_n77 DC_2-2-12-30_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_2-12-48_n5 | 0.3 | 0.3 | 0.5 | 0.5 |
| DC_2-12-66_n5 | 0.3 | 0.5 | 0.5 | 0.3 |
| DC_2-12-66_n2 | 0.3 | 0.5 | 0.3 | 0.3 |
| DC_2-12-66_n30 DC_2-2-12-66_n30 DC_2-12-66-66_n30 | 0.4 | 0.5 | 0.4 | 0.5 |
| DC_2-12-66_n41 DC_2-2-12-66_n41 | 0.5 | 0.8 | 0.5 | 0.5 |
| DC_2-12-66_n66 | 0.3 | 0.5 | 0.3 | 0.3 |
| DC_2-12-66_n77 DC_2-2-12-66_n77 DC_2-12-66-66_n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_2-12-66_n78 DC_2-2-12-66_n78 | 0.3 | - | 0.3 | 0.5 |
| DC_2-12_n66-n78 | 0.3 | - | 0.3 | 0.5 |
| DC_2-13_n2-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_2-13_n5-n77 DC_2-2-13_n5-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-13_n25-n66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-13-48_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-13-66_n2 | 0.3 | - | 0.3 | 0.3 |
| DC_2-13-66_n5 | 0.3 | - | 0.3 | - |
| DC_2-13-66_n48 | 0.3 | - | 0.3 | 0.5 |
| DC_2-13-66_n66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-13-66_n77 DC_2-2-13-66_n77 DC_2-2-13-66-66_n77 DC_2-13-66-66_n77 | 0.3 | - | 0.3 | 0.5 |
| DC_2-13_n66-n77 | 0.3 | - | 0.3 | 0.5 |
| DC_2-14-30_n2 | 0.3 | - | 0.3 | 0.3 |
| DC_2-14-30_n66 | 0.4 | - | 0.5 | 0.4 |
| DC_2-14-30_n77 DC_2-2-14-30_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_2-14-66_n2 DC_2-14-66-66_n2 | 0.3 | - | 0.3 | 0.3 |
| DC_2-14-66_n30 DC_2-2-14-66_n30 DC_2-14-66-66_n30 | 0.4 | - | 0.4 | 0.5 |
| DC_2-14-66_n66 DC_2-2-14-66_n66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-14-66_n77 DC_2-2-14-66_n77 DC_2-14-66-66_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_2-28-66_n7 | 0.3 | 0.2 | 0.5 | 0.5 |
| DC_2-28-66_n66 | 0.3 | 0.2 | 0.3 | 0.3 |
| DC_2-29-30_n2 | 0.4 | - | 0.5 | 0.4 |
| DC_2-29-30_n66 | 0.4 | - | 0.5 | 0.4 |
| DC_2-29-30_n77 DC_2-2-29-30_n77 | 0.2 | 0.2 | - | 0.5 |
| DC_2-29-66_n2 DC_2-29-66-66_n2 | 0.3 | - | 0.3 | 0.3 |
| DC_2-29-66_n30 DC_2-2-29-66_n30 DC_2-29-66-66_n30 | 0.4 | - | 0.4 | 0.5 |
| DC_2-29-66_n66 | 0.3 | - | 0.3 | 0.3 |
| DC_2-29-66_n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_2-29-66_n78 | 0.3 | - | 0.3 | 0.5 |
| DC_2-30-(n)5 DC_2-2-30-(n)5 | 0.4 | - | 0.5 | - |
| DC_2-30-66_n2 DC_2-30-66-66_n2 | 0.4 | 0.5 | 0.4 | 0.4 |
| DC_2-30-66_n5 | 0.4 | 0.5 | 0.4 | - |
| DC_2-30-66_n66 | 0.4 | 0.5 | 0.4 | 0.4 |
| DC_2-30-66_n77 DC_2-2-30-66_n77 DC_2-30-66-66_n77 | 0.2 | 0.5 | 0.4 | 0.5 |
| DC_2-46_n41-n66 | 0.3 | - | 0.5 | 0.5 |
| DC_2-46_n41-n71 | - | - | - | 0.2 |
| DC_2-46-48_n2 | 0.3 | - | 0.5 | 0.3 |
| DC_2-46-48_n5 | 0.2 | - | 0.5 | - |
| DC_2-46-48_n66 | 0.3 | - | 0.5 | 0.3 |
| DC_2-46-66_n5 | 0.3 | - | 0.3 | - |
| DC_2-46-66_n41 | 0.3 | - | 0.5 | 0.51 / 12 |
| DC_2-48_(n)5 | 0.2 | - | 0.5 | - |
| DC_2-48_n48-n66 | 0.3 | 0.4 | 0.4 | 0.3 |
| DC_2-48-66_n2 | 0.3 | 0.5 | 0.3 | 0.3 |
| DC_2-48-66_n5 | 0.3 | 0.5 | 0.3 | - |
| DC_2-48-66_n12 | 0.3 | 0.5 | 0.3 | - |
| DC_2-48-66_n66 | 0.3 | 0.5 | 0.3 | 0.3 |
| DC_2-48-66_n71 | 0.3 | 0.5 | 0.3 | - |
| DC_2-48-66_n77 | 0.3 | 0.5 | 0.3 | 0.5 |
| DC_2-66_n2-n77 DC_2-66-66_n2-n77 | 0.2 | 0.3 | 0.3 | 0.5 |
| DC_2-66_n2-n78 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_2-66_(n)5 DC_2-2-66_(n)5 DC_2-66-66_(n)5 | 0.3 | - | 0.3 | - |
| DC_2-66_n5-n77 | 0.3 | 0.3 | - | 0.5 |
| DC_2-66_n25-n66 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_2-66_n38-n78 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_2-66_n41-n71 | 0.3 | 0.3 | 0.51 / 12 | 0.5 |
| DC_2-66-71_n38 DC_2-2-66-71_n38 | 0.3 | 0.5 | - | 0.5 |
| DC_2-66-71_n41 DC_2-2-66-71_n41 | 0.3 | 0.3 | 0.5 | 0.51 / 12 |
| DC_2-66-71_n66 | 0.3 | 0.3 | - | 0.3 |
| DC_2-66-(n)71 | 0.3 | 0.3 | - | - |
| DC_2-66-71_n71 | 0.3 | 0.3 | - | - |
| DC_2-66-71_n78 DC_2-2-66-71_n78 | 0.3 | 0.5 | - | 0.5 |
| DC_2-66_n71-n78 | 0.3 | 0.5 | - | 0.5 |
| DC_2-66_n66-n77 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_2-66_n66-n78 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_2-66-71_n2 | 0.3 | 0.3 | - | 0.3 |
| DC_2-71_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_2-71_n66-n78 | 0.3 | - | 0.5 | 0.5 |
| DC_3_n1-n40-n78 | 0.2 | 0.2 | 0.48 | 0.5 |
| DC_3_n1-n77-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_3_n1-n78-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_3-5-7_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-5-7_n78 DC_3-5-7-7_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3_n5-n40-n78 | 0.2 | 0.2 | 0.48 | 0.5 |
| DC_3-5-41_n79 | - | - | 03 / 0.54 | - |
| DC_3-7_n1-n8 DC_3-3-7_n1-n8 DC_3-7-7_n1-n8 DC_3-3-7-7_n1-n8 | - | - | - | 0.2 |
| DC_3-7_n1-n40 | - | 0.3 | - | 0.8 |
| DC_3-7_n1-n78 | 0.3 | 0.3 | 0.3 | 0.5 |
| DC_3-7_n3-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-7_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-8_n1 DC_3-3-7-8_n1 DC_3-7-7-8_n1 DC_3-3-7-7-8_n1 | - | - | 0.2 | - |
| DC_3-7-8_n28 | - | - | 0.2 | 0.1 |
| DC_3-7-8_n40 | - | - | 0.2 | 0.5 |
| DC_3-7-8_n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-8_n78 DC_3-3-7-8_n78 DC_3-7-7-8_n78 DC_3-3-7-7-8_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7_n7-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-20_n28 | - | - | 0.2 | 0.1 |
| DC_3-7-20_n38 | - | - | - | 0.2 |
| DC_3-7-20_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_3-7_n26-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-28_n1 DC_3-7-7-28_n1 | - | - | 0.2 | - |
| DC_3-7-28_n40 | - | 0.3 | - | 0.8 |
| DC_3-7-28_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-7-32_n28 | 0.5 | - | - | 0.5 |
| DC_3-7-32_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_3-7-38_n28 | - | - | 0.2 | 0.2 |
| DC_3-7-40_n1 | - | 0.3 | 0.8 | - |
| DC_3-7_n40-n78 | 0.2 | - | 0.48 | 0.58 |
| DC_3-7_n75-n78 | 0.2 | 0.2 | - | 0.5 |
| DC_3-7_SUL_n78-n80 | 0.2 | 0.2 | 0.5 | - |
| DC_3-8_n77-n79 | 0.6 | 0.3 | 0.8 | - |
| DC_3-8_n1-n28 | - | 0.2 | - | 0.2 |
| DC_3-8_n1-n40 | - | - | 0.1 | 0.2 |
| DC_3-8_n1-n78 DC_3-3-8_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-8-11_n28 | 0.3 | 0.2 | 0.5 | 0.2 |
| DC_3-8-11_n77 | 0.3 | 0.2 | 0.5 | 0.5 |
| DC_3-8-20_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_3-8_n28-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-8-28_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-8_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-8-32_n1 | - | - | 0.5 | 0.3 |
| DC_3-8-32_n28 | - | - | - | 0.2 |
| DC_3-8-32_n78 | 0.3 | 0.2 | 0.5 | 0.5 |
| DC_3-8-40_n1 | - | - | 0.2 | 0.1 |
| DC_3-8-40_n78 | 0.2 | 0.2 | 0.48 | 0.58 |
| DC_3-8_n40-n78 | 0.2 | 0.2 | 0.4 | 0.5 |
| DC_3-8-42_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_(n)3-n8-n77 | 0.6 | 0.6 | 0.3 | 0.8 |
| DC_3-8_SUL_n78-n80 | 0.2 | 0.2 | 0.5 | - |
| DC_3-11_n28-n77 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_3-18_n3-n41 | 0.2 | - | 0.2 | - |
| DC_3-18_n3-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_3-18_n3-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_3-18_n28-n41 | 0.2 | - | 0.2 | - |
| DC_3-18_n28-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_3-18_n28-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_3-18_n41-n77 | 0.2 | - | - | 0.5 |
| DC_3-18_n41-n78 | 0.2 | - | - | 0.5 |
| DC_3-18-42_n77 | - | - | 0.5 | 0.5 |
| DC_3-18-42_n78 | - | - | 0.5 | 0.5 |
| DC_3-18-42_n79 | 0.2 | - | 0.5 | - |
| DC_3-19_n1-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_3-19_n1-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_3-19-21_n77 | 0.3 | - | 0.5 | 0.5 |
| DC_3-19-21_n78 | 0.3 | - | 0.5 | 0.5 |
| DC_3-19-21_n79 | 0.3 | - | 0.5 | - |
| DC_3-19-42_n1 | 0.2 | - | 0.5 | 0.2 |
| DC_3-19-42_n77 | 0.2 | - | 0.5 | 0.5 |
| DC_3-19-42_n78 | 0.2 | - | 0.5 | 0.5 |
| DC_3-19-42_n79 | 0.2 | - | 0.5 | - |
| DC_3-19_n77-n79 | 0.2 | - | 0.5 | - |
| DC_3-19_n78-n79 | 0.2 | - | 0.5 | - |
| DC_3-20_n1-n28 | - | - | 0.2 | 0.2 |
| DC_3-20_n1-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_3-20_n7-n28 | - | 0.1 | - | 0.1 |
| DC_3-20_n8-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-20-28_n1 | - | 0.2 | 0.2 | - |
| DC_3-20_n28-n75 | 0.5 | - | 0.5 | - |
| DC_3-20-28_n78 | 0.2 | 0.1 | 0.2 | 0.5 |
| DC_3-20_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-20-32_n28 | 0.5 | - | - | 0.5 |
| DC_3-20-32_n78 | 0.2 | - | - | 0.5 |
| DC_3-20-38_n78 | 0.2 | 0.2 | 0.4 | 0.5 |
| DC_3-20_n38-n78 | 0.2 | 0.2 | 0.4 | 0.5 |
| DC_3-20-40_n78 | 0.2 | 0.2 | 0.45 | 0.55 |
| DC_3-20_n41-n78 | - | - | - | 0.5 |
| DC_3_20_SUL_n78-n80 | 0.2 | - | 0.5 | - |
| DC_3-21_n1-n77 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_3-21_n1-n78 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_3-21_n1-n79 | 0.3 | 0.5 | - | - |
| DC_3-21_n28-n77 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_3-21_n28-n78 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_3-21_n28-n79 | 0.3 | 0.5 | 0.3 | - |
| DC_3-21-42_n1 | 0.3 | 0.5 | 0.5 | 0.2 |
| DC_3-21-42_n77 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_3-21-42_n78 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_3-21-42_n79 | 0.3 | 0.5 | 0.5 | - |
| DC_3-21_n77-n79 | 0.3 | 0.5 | 0.5 | - |
| DC_3-21_n78-n79 | 0.3 | 0.5 | 0.5 | - |
| DC_3-28_n1-n40 | - | 0.2 | - | - |
| DC_3-28_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_3-28_n3-n78 | - | 0.2 | - | 0.5 |
| DC_3-28_n7-n78 DC_3-3-28_n7-n78 | 0.5 | 0.2 | 0.4 | 0.5 |
| DC_3-28-32_n1 | 0.5 | 0.5 | - | - |
| DC_3-28-40_n78 | 0.2 | 0.2 | 0.45 | 0.55 |
| DC_3-28_n40-n78 | 0.2 | 0.2 | 0.45 | 0.55 |
| DC_3-28-41_n78 | 0.5 | 0.2 | 0.43 / 0.54 | 0.5 |
| DC_3-28-42_n77 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_3-28-42_n78 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_3-28-42_n79 | 0.2 | 0.2 | 0.5 | - |
| DC_3_n28-n77-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_3_n28-n78-n79 | 0.2 | 0.2 | 0.5 | - |
| DC_3-32_n1-n28 | - | - | 0.2 | 0.2 |
| DC_3-32_n1-n78 | - | - | - | 0.5 |
| DC_3-32-38_n28 | - | - | - | 0.2 |
| DC_3-38_n28-n78 | 0.5 | 0.4 | 0.2 | 0.5 |
| DC_3-40_n1-n78 | 0.2 | 0.45 | - | 0.55 |
| DC_3_n40-n41-n79 | - | - | 03/0.54 | 0.5 |
| DC_3-41_n1-n78 DC_3-3-41_n1-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_3-41_n3-n41 | - | 03 / 0.54 | - | 03 / 0.54 |
| DC_3-41_n3-n77 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_3-41_n3-n78 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_3-41_n28-n41 | 0.2 | 03 / 0.54 | 0.2 | 03 / 0.54 |
| DC_3-41_n28-n77 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_3-41_n28-n78 | 0.5 | 0.43 / 0.54 | 0.2 | 0.5 |
| DC_3-41_n41-n77 | 0.2 | 03 / 0.54 | 03 / 0.54 | 0.5 |
| DC_3-41_n41-n78 | 0.2 | 03 / 0.54 | 03 / 0.54 | 0.5 |
| DC_3-41-42_n77 | 0.5 | 03 / 0.54 | 0.5 | 0.5 |
| DC_3-41-42_n78 | 0.5 | 03 / 0.54 | 0.5 | 0.5 |
| DC_3-41-42_n79 | 0.5 | 03 / 0.54 | 0.5 | - |
| DC_3-42_n1-n77 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_3-42_n1-n78 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_3-42_n1-n79 | 0.2 | 0.5 | 0.2 | - |
| DC_3-42_n28-n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_3-42_n77-n79 | 0.2 | 0.5 | 0.5 | - |
| DC_3-42_n78-n79 | 0.2 | 0.5 | 0.5 | - |
| DC_5-7_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_5-7-7_n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_5-7-66_n2 | - | 0.5 | 0.5 | 0.3 |
| DC_5-7-66_n7 DC_5-7-66-66_n7 | - | 0.5 | 0.5 | 0.5 |
| DC_5-7-66_n66 DC_5-7-7-66_n66 | 0.3 | - | 0.3 | 0.3 |
| DC_5-7_n66-n78 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_5-7-66_n78 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_5-30-66_n2 | - | 0.5 | 0.4 | 0.4 |
| DC_5-30-66_n66 | - | 0.5 | 0.4 | 0.4 |
| DC_5-30-66_n77 DC_5-30-66-66_n77 | 0.2 | 0.5 | 0.4 | 0.5 |
| DC_5-48_(n)12 | 0.5 | 0.3 | - | 0.5 |
| DC_5-48-66_n12 | 0.5 | 0.5 | 0.2 | 0.3 |
| DC_5-48-66_n71 | - | 0.5 | 0.2 | - |
| DC_5-48-66_n77 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_5-66_n2-n77 DC_5-66-66_n2-n77 | 0.2 | 0.3 | 0.3 | 0.5 |
| DC_5-66_n2-n78 | - | 0.3 | 0.3 | 0.5 |
| DC_5-66_n5-n77 DC_5-66-66_n5-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_5-66_(n)12 | - | 0.5 | 0.5 | 0.5 |
| DC_5-66_n66-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-8_n1-n78 | 0.3 | 0.2 | - | 0.8 |
| DC_7_n1-n8-n78 | 0.3 | - | 0.2 | 0.8 |
| DC_7-8_n1-n78 DC_7-7-8_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-8-20_n1 | - | 0.2 | 0.2 | - |
| DC_7-8-20_n3 | - | 0.2 | - | - |
| DC_7-8_n28-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-8-32_n1 | - | 0.2 | - | - |
| DC_7-8-32_n78 | - | 0.2 | - | 0.5 |
| DC_7-8-38_n1 | - | - | 0.2 | - |
| DC_7-8-40_n1 | 0.3 | 0.2 | 0.8 | - |
| DC_7-8-40_n78 | - | 0.2 | 0.48 | 0.58 |
| DC_7-8_n40-n78 | - | 0.2 | 0.4 | 0.5 |
| DC_7-12_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-12-66_n2 | 0.5 | 0.5 | 0.3 | 0.3 |
| DC_7-12-66_n78 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-12_n66-n78 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-13_n25-n66 | 0.5 | - | 0.3 | 0.5 |
| DC_7-13-66_n66 | 0.5 | - | 0.5 | 0.5 |
| DC_7-20_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-20_n3-n38 | - | 0.2 | - | 0.2 |
| DC_7-20_n3-n78 | - | - | - | 0.5 |
| DC_7-20_n8-n78 | - | 0.2 | 0.2 | 0.5 |
| DC_7-20-28_n1 | - | 0.2 | 0.2 | - |
| DC_7-20-28_n3 | - | 0.2 | 0.1 | - |
| DC_7-20_n28-n78 | - | 0.2 | 0.2 | 0.5 |
| DC_7-20-32_n8 | - | 0.2 | - | 0.2 |
| DC_7-20-32_n28 | - | - | - | 0.2 |
| DC_7-20-32_n78 | - | - | - | 0.5 |
| DC_7-20-38_n3 | - | - | 0.2 | - |
| DC_7-20-38_n8 | - | 0.2 | 0.2 | 0.2 |
| DC_7-20-38_n78 | - | - | 0.4 | 0.6 |
| DC_7-28_n1-n40 | 0.3 | 0.2 | - | 0.8 |
| DC_7-28_n3-n78 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-28_n7-n78 | - | - | - | 0.5 |
| DC_7-28-32_n1 | - | 0.2 | - | - |
| DC_7-28-38_n1 | - | 0.2 | 0.2 | - |
| DC_7-28_n40-n78 | - | 0.2 | 0.4 | 0.5 |
| DC_7-29-66_n78 | 0.5 | - | 0.5 | 0.5 |
| DC_7-32_n1-n78 | 0.6 | - | 0.6 | 0.8 |
| DC_7-38_n3-n78 | 0.5 | 0.5 | 0.2 | 0.5 |
| DC_7-66_n38-n78 DC_7-7-66_n38-n78 | - | 0.2 | - | 0.5 |
| DC_7-28_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-28-66_n7 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-28-66_n66 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_7-40_n1-n78 | - | 0.45 | 0.2 | 0.55 |
| DC_7-66_n2-n78 | - | 0.3 | 0.3 | 0.5 |
| DC_7-66_n25-n66 | 0.5 | 0.5 | 0.3 | 0.5 |
| DC_7-66_n66-n77 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_7-66_n66-n78 DC_7-7-66_n66-n78 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_7-66-71_n2 | 0.5 | 0.5 | - | 0.3 |
| DC_7-66-71_n78 | 0.2 | 0.2 | - | 0.5 |
| DC_7-66_n71-n78 | 0.2 | 0.2 | - | 0.5 |
| DC_7-71_n2-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7-71_n66-n78 | 0.2 | - | 0.2 | 0.5 |
| DC_8_n1-n3-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_8_n3-n28-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_8_n3-n77-n79 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_8-11_n1-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_8-11_n3-n28 | 0.2 | 0.3 | 0.5 | 0.2 |
| DC_8-11_n3-n77 | 0.2 | 0.3 | 0.5 | - |
| DC_8-11_n3-n79 | - | 0.3 | 0.5 | 0.5 |
| DC_8-11_n28-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_8-11_n77-n79 | 0.2 | - | 0.5 | - |
| DC_8-20-28_n78 | 0.2 | 0.1 | 0.2 | 0.5 |
| DC_8_n28-n77-n79 | 0.2 | 0.2 | 0.5 | 0.5 |
| DC_8_n39-n40-n79 | - | 0.3 | 0.3 | 0.5 |
| DC_8-40_n1-n78 | 0.2 | 0.45 | - | 0.55 |
| DC_8-41_n1-n3 | - | 03 / 0.54 | - | - |
| DC_8-41_n1-n77 | 0.2 | - | 0.2 | 0.5 |
| DC_8-41_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_8-41_n3-n77 | 0.2 | 09 / 0.510 | 0.2 | 0.5 |
| DC_8-42_n1-n3 | 0.2 | 0.5 | - | 0.2 |
| DC_8-42_n1-n77 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_8-42_n3-n28 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_8-42_n3-n77 | 0.2 | 0.5 | 0.2 | 0.5 |
| DC_8-42_n28-n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_11_n3-n28-n77 | 0.3 | 0.5 | 0.2 | 0.5 |
| DC_11_n3-n77-n79 | 0.3 | 0.5 | 0.5 | 0.5 |
| DC_12-30-66_n2 | 0.5 | 0.5 | 0.4 | 0.4 |
| DC_12-30-66_n66 | 0.5 | 0.5 | 0.4 | 0.4 |
| DC_12-30-66_n77 DC_12-30-66-66_n77 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_12-48_(n)5 | 0.5 | 0.3 | - | 0.5 |
| DC_12-48-66_n5 | 0.5 | 0.5 | 0.5 | - |
| DC_12-66_(n)5 | - | 0.5 | 0.5 | - |
| DC_12-66_n2-n78 | - | 0.3 | 0.3 | 0.5 |
| DC_13-48-66_n77 | - | 0.5 | 0.2 | 0.5 |
| DC_13-66_n2-n77 | - | 0.2 | 0.2 | 0.5 |
| DC_13-66_n5-n48 | 0.3 | 0.2 | 0.5 | 0.5 |
| DC_13-66_n5-n77 DC_13-66-66_n5-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_13-66_n66-n77 | - | 0.2 | 0.2 | 0.5 |
| DC_14-30-66-n2 | - | 0.5 | 0.4 | 0.4 |
| DC_14-30-66_n66 | - | 0.5 | 0.4 | 0.4 |
| DC_14-30-66_n77 DC_14-30-66-66_n77 | 0.2 | 0.5 | 0.5 | 0.5 |
| DC_18-41_n3-n77 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_18-41_n3-n78 | 0.2 | 03 / 0.54 | 0.2 | 0.5 |
| DC_19_n1-n77-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_19_n1-n78-n79 | 0.3 | 0.3 | 0.5 | - |
| DC_19-21_n1-n77 | - | - | - | 0.5 |
| DC_19-21_n1-n78 | - | - | 0.2 | 0.5 |
| DC_19-21-42_n1 | - | - | 0.5 | - |
| DC_19-21-42_n77 | - | - | 0.5 | 0.5 |
| DC_19-21-42_n78 | - |  | 0.5 | 0.5 |
| DC_19-21-42_n79 | - | - | 0.5 | - |
| DC_19-21_n77-n79 | - | - | 0.5 | - |
| DC_19-21_n78-n79 | - | - | 0.5 | - |
| DC_19-42_n1-n77 | - | 0.5 | 0.2 | 0.5 |
| DC_19-42_n1-n78 | - | 0.5 | - | 0.5 |
| DC_19-42_n1-n79 | - | 0.5 | - | - |
| DC_19-42_n77-n79 | - | 0.5 | 0.5 | - |
| DC_19-42_n78-n79 | - | 0.5 | 0.5 | - |
| DC_20-28-32_n1 | 0.2 | 0.2 | - | - |
| DC_20-28-32_n3 | 0.3 | 0.2 | - | 0.3 |
| DC_20-28-38_n1 | 0.2 | 0.2 | - | - |
| DC_20-32_n1-n28 | 0.2 | - | - | 0.2 |
| DC_20-38_n3-n78 | 0.2 | 0.4 | 0.2 | 0.5 |
| DC_20-41_n1-n78 | - | - | - | 0.5 |
| DC_21_n1-n77-n79 | - | 0.2 | 0.5 | - |
| DC_21_n1-n78-n79 | - | 0.2 | 0.5 | - |
| DC_21-28-42_n77 | - | 0.2 | 0.5 | 0.5 |
| DC_21-28-42_n78 | - | 0.2 | 0.5 | 0.5 |
| DC_21-28-42_n79 | - | 0.2 | 0.5 | - |
| DC_21_n28-n77-n79 | - | 0.2 | 0.5 | - |
| DC_21_n28-n78-n79 | - | 0.2 | 0.5 | - |
| DC_21-42_n1-n77 | - | 0.5 | 0.2 | 0.5 |
| DC_21-42_n1-n78 | - | 0.5 | - | 0.5 |
| DC_21-42_n1-n79 | - | 0.5 | - | - |
| DC_21-42_n77-n79 | - | 0.5 | 0.5 | - |
| DC_21-42_n78-n79 | - | 0.5 | 0.5 | - |
| DC_28-32-38_n1 | 0.2 | - | - | - |
| DC_28-41-42_n78 | 0.2 | 0.4 | 0.5 | 0.5 |
| DC_29-30-66_n2 DC_29-30-66-66_n2 | - | 0.5 | 0.4 | 0.4 |
| DC_29-30-66_n66 | - | 0.5 | 0.3 | 0.3 |
| DC_29-30-66_n77 | 0.5 | 0.5 | 0.5 | 0.5 |
| DC_30-66-(n)5 | 0.5 | - | 0.4 | 0.5 |
| DC_42_n1-n77-n79 | 0.5 | 0.2 | 0.5 | - |
| DC_42_n1-n78-n79 | 0.5 | 0.2 | 0.5 | - |
| DC_42_n3-n28-n77 | 0.5 | 0.2 | 0.5 | 0.5 |
| DC_46-66_n25-n41 | - | 0.3 | 0.3 | 0.51 / 12 |
| DC_46-66_n41-n71 | - | 0.3 | 0.51 / 12 | 0.2 |
| DC_48-66_n25-n48 | 0.4 | 0.3 | 0.3 | 0.4 |
| DC_66-71_n2-n78 | 0.5 | - | 0.3 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 6: Void. NOTE 7: Void. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 9: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.3B.3.3.4	ΔRIB,c for EN-DC five bands

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |  |  |
| DC_1-3-5-7_n77 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-5-7_n78 DC_1-3-5-7-7_n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-5-41_n79 | - | - | - |  | 03 / 0.54 | - |  |
| DC_1-3-7_n3-n78 | 0.3 | 0.3 | 0.3 |  | 0.3 | 0.5 |  |
| DC_1-3-7_n7-n78 | 0.3 | 0.3 | 0.3 |  | 0.3 | 0.5 |  |
| DC_1-3-7-8_n28 | - | - | - |  | 0.2 | 0.2 |  |
| DC_1-3-7-8_n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-7_n8-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-7-20_n28 | - | - | - |  | 0.2 | 0.2 |  |
| DC_1-3-7-20_n38 | - | - | - |  | - | 0.2 |  |
| DC_1-3-7-20_n78 | 0.2 | 0.2 | 0.2 |  | - | 0.5 |  |
| DC_1-3-7_n26-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-7-28_n3 | - | - | - |  | 0.2 | - |  |
| DC_1-3-7-28_n5 | - | - | - |  | 0.2 | 0.2 |  |
| DC_1-3-7-28_n7 | - | - | - |  | 0.2 | - |  |
| DC_1-3-7_n28-n38 | - | - | - |  | 0.2 | - |  |
| DC_1-3-7-28_n40 | - | - | 0.3 |  | 0.2 | 0.8 |  |
| DC_1-3-7-28_n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-7_n28-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-7-32_n28 | - | 0.5 | - |  | - | 0.5 |  |
| DC_1-3-7-32_n78 | 0.3 | 0.3 | 0.3 |  | - | 0.5 |  |
| DC_1-3-7-38_n28 | - | - | - |  | 0.2 | 0.2 |  |
| DC_1-3-7-40_n78 | 0.2 | 0.2 | - |  | 0.45 | 0.55 |  |
| DC_1-3-7_n40-n78 | - | - | 0.3 |  | 0.8 | 0.5 |  |
| DC_1-3-7_n75-n78 | 0.3 | 0.3 | 0.3 |  | - | 0.5 |  |
| DC_1-3-8-11_n28 | - | 0.3 | 0.2 |  | 0.5 | 0.2 |  |
| DC_1-3-8-11_n77 | 0.2 | 0.3 | 0.2 |  | 0.5 | 0.5 |  |
| DC_1-3-8-20_n78 | 0.2 | 0.2 | 0.2 |  | - | 0.5 |  |
| DC_1-3-8_n28-n77 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-8-28_n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-8_n28-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-8-32_n78 | 0.2 | 0.2 | 0.2 |  | - | 0.5 |  |
| DC_1-3-8-40_n78 | 0.2 | 0.2 | 0.2 |  | 0.45 | 0.55 |  |
| DC_1-3-8-42_n77 | 0.2 | 0.2 | 0.2 |  | 0.5 | 0.5 |  |
| DC_1-3-8_n77-n79 | 0.2 | 0.3 | 0.3 |  | 0.5 | - |  |
| DC_1-3-11_n28-n77 | 0.2 | 0.3 | 0.5 |  | 0.2 | 0.5 |  |
| DC_1-3-18_n3-n41 | - | 0.5 | - |  | 0.5 | 03 / 0.54 |  |
| DC_1-3-18_n3-n77 | 0.2 | 0.2 | - |  | 0.2 | 0.5 |  |
| DC_1-3-18_n3-n78 | 0.2 | 0.2 | - |  | 0.2 | 0.5 |  |
| DC_1-3-18_n28-n41 | - | 0.5 | - |  | 0.2 | 03 / 0.54 |  |
| DC_1-3-18_n28-n77 | - | - | - |  | 0.2 | 0.5 |  |
| DC_1-3-18_n28-n78 | - | - | - |  | 0.2 | 0.5 |  |
| DC_1-3-18_n41-n77 | - | 0.5 | - |  | 03 / 0.54 | 0.5 |  |
| DC_1-3-18_n41-n78 | - | 0.5 | - |  | 03 / 0.54 | 0.5 |  |
| DC_1-3-18-42_n77 | 0.2 | 0.2 | - |  | 0.5 | 0.5 |  |
| DC_1-3-18-42_n78 | 0.2 | 0.2 | - |  | 0.5 | 0.5 |  |
| DC_1-3-18-42_n79 | 0.2 | 0.2 | - |  | 0.5 | - |  |
| DC_1-3-19-21_n77 | 0.2 | 0.3 | - |  | 0.5 | 0.5 |  |
| DC_1-3-19-21_n78 | 0.2 | 0.3 | - |  | 0.5 | 0.5 |  |
| DC_1-3-19-21_n79 | - | 0.3 | - |  | 0.5 | - |  |
| DC_1-3-19-42_n77 | 0.2 | 0.2 | - |  | 0.5 | 0.5 |  |
| DC_1-3-19-42_n78 | 0.2 | 0.2 | - |  | 0.5 | 0.5 |  |
| DC_1-3-19-42_n79 | 0.2 | 0.2 | - |  | 0.5 | - |  |
| DC_1-3-20_n7-n78 | 0.2 | 0.2 | - |  | - | 0.5 |  |
| DC_1-3-20_n8-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-20_n28-n75 | 0.2 | 0.5 | 0.2 |  | 0.5 | - |  |
| DC_1-3-20_n28-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-20-32_n28 | - | 0.5 | 0.2 |  | - | 0.5 |  |
| DC_1-3-20-32_n78 | 0.2 | 0.2 | - |  | - | 0.5 |  |
| DC_1-3-20-38_n78 | - | 0.2 | 0.2 |  | 0.4 | 0.5 |  |
| DC_1-3-20_n38-n78 | - | 0.2 | 0.2 |  | 0.4 | 0.5 |  |
| DC_1-3-20-40_n78 | - | - | - |  | 05 | 0.55 |  |
| DC_1-3-20_n41-n78 | - | - | - |  | - | 0.5 |  |
| DC_1-3-21-42_n77 | 0.2 | 0.3 | 0.5 |  | 0.5 | 0.2 |  |
| DC_1-3-21-42_n78 | 0.2 | 0.3 | 0.5 |  | 0.5 | 0.2 |  |
| DC_1-3-21-42_n79 | 0.2 | 0.3 | 0.5 |  | 0.5 | - |  |
| DC_1-3-21_n77-n79 | 0.2 | 0.3 | 0.5 |  | 0.5 | - |  |
| DC_1-3-21_n78-n79 | 0.2 | 0.3 | 0.5 |  | 0.5 | - |  |
| DC_1-3-28_n3-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-28_n7-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-3-28-40_n78 | - | - | 0.2 |  | - | 0.5 |  |
| DC_1-3-28_n40-n78 | - | 0.2 | 0.2 |  | 0.45 | 0.55 |  |
| DC_1-3-28-42_n77 | 0.2 | 0.2 | 0.2 |  | 0.5 | 0.5 |  |
| DC_1-3-28-42_n78 | 0.2 | 0.2 | 0.2 |  | 0.5 | 0.5 |  |
| DC_1-3-28-42_n79 | 0.2 | 0.2 | 0.2 |  | 0.5 | - |  |
| DC_1-3_n28-n77-n79 | 0.2 | 0.2 | 0.2 |  | 0.5 | - |  |
| DC_1_n3-n28-n77-n79 | 0.3 | 0.2 | 0.5 |  | 0.5 | 0.5 |  |
| DC_1-3_n28-n78-n79 | 0.2 | 0.2 | 0.2 |  | 0.5 | - |  |
| DC_1-3-38_n28-n78 | - | 0.2 | - |  | 0.2 | 0.5 |  |
| DC_1-3-41_n3-n41 | - | - | 03 / 0.54 |  | - | 03 / 0.54 |  |
| DC_1-3-41_n3-n77 | 0.2 | 0.2 | - |  | 0.2 | 0.5 |  |
| DC_1-3-41_n3-n78 | 0.2 | 0.2 | - |  | 0.2 | 0.5 |  |
| DC_1-3-41_n28-n41 | - | - | 03 / 0.54 |  | 0.2 | 03 / 0.54 |  |
| DC_1-3-41_n28-n77 | 0.2 | 0.2 | 03 / 0.54 |  | 0.2 | 0.5 |  |
| DC_1-3-41_n28-n78 | - | 0.2 | 03 / 0.54 |  | 0.2 | 0.5 |  |
| DC_1-3-41_n41-n77 | 0.2 | 0.2 | - |  | - | 0.5 |  |
| DC_1-3-41_n41-n78 | 0.2 | 0.2 | - |  | - | 0.5 |  |
| DC_1-3-41-42_n77 | 0.2 | 0.2 | - |  | 0.5 | 0.5 |  |
| DC_1-3-41-42_n78 | 0.2 | 0.2 | - |  | 0.5 | 0.5 |  |
| DC_1-3-41-42_n79 | 0.2 | 0.2 | - |  | 0.5 | - |  |
| DC_1-3-42_n28-n77 | 0.2 | 0.2 | 0.5 |  | 0.5 | 0.5 |  |
| DC_1-7-8-20 _n3 | - | - | 0.2 |  | 0.2 | - |  |
| DC_1-7-8-20 _n28 | - | - | 0.2 |  | 0.2 | 0.2 |  |
| DC_1-7-8-20_n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-7-8_n28-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-7-8-32_n78 | 0.2 | 0.2 | 0.2 |  | - | 0.5 |  |
| DC_1-7-8-40_n78 | 0.2 | - | 0.2 |  | 0.45 | 0.55 |  |
| DC_1-7-20_n3-n38 | - | - | 0.2 |  | - | 0.2 |  |
| DC_1-7-20_n3-n78 | 0.2 | 0.2 | 0.2 |  | - | 0.5 |  |
| DC_1-7-20_n8-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-7-20-28_n3 | - | - | 0.2 |  | 0.2 | - |  |
| DC_1-7-20_n28-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-7-20-32_n8 | - | - | 0.2 |  | - | 0.2 |  |
| DC_1-7-20-32_n28 | - | - | 0.2 |  | - | 0.2 |  |
| DC_1-7-20-32_n78 | 0.2 | 0.2 | 0.2 |  | - | 0.5 |  |
| DC_1-7-20-38_n3 | - | - | - |  | 0.2 | - |  |
| DC_1-7-28_n3-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-7-28_n7-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-7-28-32_n3 | - | - | 0.2 |  | - | - |  |
| DC_1-7-28_n40-n78 | 0.2 | - | 0.2 |  | 0.4 | 0.5 |  |
| DC_1-7-38_n3-n78 | 0.6 | 0.6 | - |  | - | 0.8 |  |
| DC_1-8_n3-n28-n77 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-8_n3-n28-n79 | 0.3 | 0.3 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-8_n3-n77-n79 | 0.2 | 0.2 | 0.2 |  | 0.5 | 0.5 |  |
| DC_1-8-11_n3-n28 | - | 0.2 | 0.3 |  | 0.5 | 0.2 |  |
| DC_1-8-11_n3-n77 | 0.2 | 0.2 | 0.3 |  | 0.5 | 0.5 |  |
| DC_1-8-11_n28-n77 | 0.2 | 0.2 | - |  | 0.2 | 0.5 |  |
| DC_1-8-11_n3-n79 | - | - | 0.3 |  | 0.5 | 0.5 |  |
| DC_1-8-11_n77-n79 | 0.2 | 0.2 | - |  | 0.5 | - |  |
| DC_1-8-20-28_n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_1-8_n28-n77-n79 | 0.3 | 0.3 | 0.3 |  | 0.5 | 0.5 |  |
| DC_1-8-42_n28-n77 | 0.2 | 0.2 | 0.5 |  | 0.5 | 0.5 |  |
| DC_1-11_n3-n28-n77 | 0.2 | 0.3 | 0.5 |  | 0.2 | 0.3 |  |
| DC_1-18-41_n3-n77 | 0.2 | - | 03 / 0.54 |  | 0.2 | 0.5 |  |
| DC_1-18-41_n3-n78 | 0.2 | - | 03 / 0.54 |  | 0.2 | 0.5 |  |
| DC_1-8-42_n3-n28 | - | 0.2 | 0.5 |  | 0.2 | 0.5 |  |
| DC_1-8-42_n3-n77 | 0.2 | 0.2 | 0.5 |  | 0.2 | 0.5 |  |
| DC_1-11_n3-n77-n79 | 0.2 | 0.3 | 0.5 |  | 0.5 | 0.5 |  |
| DC_1-19-21-42_n77 | 0.2 | - | - |  | 0.5 | 0.5 |  |
| DC_1-19-21-42_n78 | - | - | - |  | 0.5 | 0.5 |  |
| DC_1-19-21-42_n79 | - | - | - |  | 0.5 | - |  |
| DC_1-19-42_n77-n79 | 0.2 | - | 0.5 |  | 0.5 | - |  |
| DC_1-19-42_n78-n79 | - | - | 0.5 |  | 0.5 | - |  |
| DC_1-20-28-32_n3 | - | 0.2 | 0.2 |  | - | - |  |
| DC_1-20-38_n3-n78 | - | - | - |  | 0.2 | 0.5 |  |
| DC_1-21-28-42_n77 | 0.2 | - | 0.2 |  | 0.5 | 0.5 |  |
| DC_1-21-28-42_n78 | - | - | 0.2 |  | 0.5 | 0.5 |  |
| DC_1-21-28-42_n79 | - | - | 0.2 |  | 0.5 | - |  |
| DC_1-21_n28-n77-n79 | 0.3 | - | 0.3 |  | 0.5 | - |  |
| DC_1-21_n28-n78-n79 | 0.3 | - | 0.3 |  | 0.5 | - |  |
| DC_1-21-42_n77-n79 | 0.2 | 0.2 | 0.5 |  | 0.5 | - |  |
| DC_1-21-42_n78-n79 | - | 0.2 | 0.5 |  | 0.5 | - |  |
| DC_1-42_n3-n28-n77 | 0.2 | 0.5 | 0.2 |  | 0.5 | 0.5 |  |
| DC_2-5-7-66_n2 | 0.3 | - | 0.5 |  | 0.5 | 0.3 |  |
| DC_2-5-7-66_n7 DC_2-5-7-66-66_n7 | 0.3 | 0.2 | 0.5 |  | 0.5 | 0.5 |  |
| DC_2-5-7-66_n66 | 0.3 | - | 0.5 |  | 0.5 | 0.5 |  |
| DC_2-5-7-66_n78 | 0.2 | - | 0.2 |  | 0.2 | 0.5 |  |
| DC_2-5-30-66_n2 | 0.4 | - | 0.5 |  | 0.4 | 0.4 |  |
| DC_2-5-30-66_n66 | 0.4 | - | 0.5 |  | 0.4 | 0.4 |  |
| DC_2-5-30-66_n77 | 0.3 | 0.2 | 0.5 |  | 0.4 | 0.5 |  |
| DC_2-5-66_n2-n77 DC_2-5-66-66_n2-n77 | 0.2 | 0.2 | 0.3 |  | 0.3 | 0.5 |  |
| DC_2-5-66_n5-n77 DC_2-5-66-66_n5-n77 | 0.3 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_2-5-66_n66-n77 | 0.3 | - | 0.3 |  | 0.3 | 0.5 |  |
| DC_2-7-12-66_n2 | 0.3 | 0.3 | 0.5 |  | 0.5 | 0.3 |  |
| DC_2-7-12-66_n78 | 0.2 | 0.2 | - |  | 0.2 | 0.5 |  |
| DC_2-7-13_n25-n66 | 0.3 | 0.5 | - |  | 0.3 | 0.5 |  |
| DC_2-7-13-66_n66 | 0.3 | 0.5 | - |  | 0.5 | 0.5 |  |
| DC_2-7-28-66_n7 | 0.3 | 0.5 | 0.2 |  | 0.5 | 0.5 |  |
| DC_2-7-28-66_n66 | 0.3 | 0.5 | 0.2 |  | 0.5 | 0.5 |  |
| DC_2-7-29-66_n78 DC_2-7-7-29-66_n78 | 0.2 | 0.5 | 0.2 |  | 0.5 | 0.5 |  |
| DC_2-7-66_n25-n66 | 0.3 | 0.5 | 0.5 |  | 0.3 | 0.5 |  |
| DC_2-7-66_n66-n77 | 0.3 | 0.5 | 0.5 |  | 0.5 | 0.5 |  |
| DC_2-7-66_n66-n78 DC_2-7-7-66_n66-n78 | 0.3 | 0.5 | 0.5 |  | 0.5 | 0.5 |  |
| DC_2-7-66-71_n2 | 0.3 | 0.5 | 0.5 |  | - | 0.3 |  |
| DC_2-7-66-71_n78 | 0.2 | 0.2 | 0.2 |  | - | 0.5 |  |
| DC_2-12-30-66_n2 | 0.4 | 0.5 | 0.5 |  | 0.4 | 0.4 |  |
| DC_2-12-30-66_n66 | 0.4 | 0.5 | 0.5 |  | 0.4 | 0.4 |  |
| DC_2-12-30-66_n77 | 0.2 | 0.5 | 0.5 |  | 0.5 | 0.5 |  |
| DC_2-13-66_n2-n77 DC_2-13-66-66_n2-n77 | 0.2 | - | 0.3 |  | 0.3 | 0.5 |  |
| DC_2-13-66_n5-n77 DC_2-2-13-66_n5-n77 DC_2-13-66-66_n5-n77 | 0.3 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_2-13-66_n66-n77 DC_2-2-13-66_n66-n77 | 0.3 | - | 0.3 |  | 0.3 | 0.5 |  |
| DC_2-14-30-66_n2 | 0.4 | - | 0.5 |  | 0.4 | 0.4 |  |
| DC_2-14-30-66_n66 | 0.4 | - | 0.5 |  | 0.4 | 0.4 |  |
| DC_2-14-30-66_n77 | 0.2 | 0.2 | 0.5 |  | 0.5 | 0.5 |  |
| DC_2-29-30-66_n2 | 0.4 | - | 0.5 |  | 0.4 | 0.4 |  |
| DC_2-29-30-66_n66 | 0.4 | - | 0.5 |  | 0.4 | 0.4 |  |
| DC_2-29-30-66_n77 | 0.2 | 0.5 | 0.5 |  | 0.5 | 0.5 |  |
| DC_2-30-66-(n)5 | 0.4 | - | 0.5 |  | 0.4 | - |  |
| DC_2-46-66_n41-n71 | 0.3 | - | 0.3 |  | 0.51 / 12 | 0.5 |  |
| DC_3-7-8_n1-n40 | - | 0.3 | 0.2 |  | 0.1 | 0.8 |  |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78 DC_3-7-7-8_n1-n78 DC_3-3-7-7-8_n1-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_3-7-8-20_n1 | - | - | 0.2 |  | 0.2 | - |  |
| DC_3-7-8_n28-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_3-7-8-40_n78 | 0.2 | - | 0.2 |  | 0.45 | 0.55 |  |
| DC_3-7-20_n1-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_3-7-20_n8-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_3-7-20-28_n1 | - | - | 0.2 |  | 0.2 | - |  |
| DC_3-7-20_n28-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | - |  |
| DC_3-7-28_n1-n40 | - | 0.3 | 0.2 |  | - | 0.8 |  |
| DC_3-7-28_n1-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_3-7-28_n3-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_3-7-28_n7-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_3-7-28_n40-n78 | 0.2 | - | 0.2 |  | 0.4 | 0.5 |  |
| DC_3-7-32_n1-n78 | 0.3 | 0.3 | - |  | 0.3 | 0.5 |  |
| DC_3-7-40_n1-n78 | 0.2 | - | 0.45 |  | 0.2 | 0.55 |  |
| DC_3-8-11_n28-n77 | 0.3 | 0.2 | 0.5 |  | 0.2 | 0.5 |  |
| DC_3-8-40_n1-n78 | 0.2 | 0.2 | 0.45 |  | 0.2 | 0.55 |  |
| DC_3-8-41_n1-n78 DC_3-3-8-41_n1-n78 | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.5 |  |
| DC_3-19-21-42_n77 | 0.3 | - | 0.5 |  | 0.5 | 0.5 |  |
| DC_3-19-21-42_n78 | 0.3 | - | 0.5 |  | 0.5 | 0.5 |  |
| DC_3-19-21-42_n79 | 0.3 | - | 0.5 |  | 0.5 | - |  |
| DC_3-19-42_n1-n77 | 0.2 | - | 0.5 |  | 0.2 | 0.5 |  |
| DC_3-19-42_n1-n78 | 0.2 | - | 0.5 |  | 0.2 | 0.5 |  |
| DC_3-19-42_n1-n79 | 0.2 | - | 0.5 |  | 0.2 | - |  |
| DC_3-20-32_n1-n28 | 0.5 | 0.2 | - |  | 0.2 | 0.5 |  |
| DC_3-20-41_n1-n78 DC_3-3-20-41_n1-n78 | - | - | - |  | - | 0.5 |  |
| DC_3-21_n1-n77-n79 | 0.3 | 0.5 | 0.2 |  | 0.5 | - |  |
| DC_3-21_n1-n78-n79 | 0.3 | 0.5 | 0.2 |  | 0.5 | - |  |
| DC_3-21_n28-n77-n79 | 0.3 | 0.5 | 0.2 |  | 0.5 | - |  |
| DC_3-21-42_n1-n77 | 0.3 | 0.5 | 0.5 |  | 0.2 | 0.2 |  |
| DC_3-21-42_n1-n78 | 0.3 | 0.5 | 0.5 |  | 0.2 | 0.2 |  |
| DC_3-21-42_n1-n79 | 0.3 | 0.5 | 0.5 |  | 0.2 | - |  |
| DC_3-28-41-42_n78 | 0.5 | 0.2 | 0.43 / 0.54 |  | 0.5 | 0.5 |  |
| DC_7-8-20-32_n1 | - | 0.2 | 0.2 |  | - | - |  |
| DC_7-8-20-38_n1 | - | 0.2 | 0.2 |  | 0.2 | - |  |
| DC_7-8-32-38_n1 | - | 0.2 | - |  | 0.2 | - |  |
| DC_7-8-40_n1-n78 | - | 0.2 | 0.45 |  | 0.2 | 0.55 |  |
| DC_7-20-28-32_n3 | - | 0.2 | 0.1 |  | - | - |  |
| DC_7-20-28-38_n1 | - | 0.2 | - |  | - | - |  |
| DC_7-20-32-38_n1 | - | - | - |  | 0.2 | - |  |
| DC_7-28-32-38_n1 | - | 0.2 | - |  | 0.2 | - |  |
| DC_7-8-40_n1-n78 | - | 0.2 | 0.45 |  | 0.2 | 0.55 |  |
| DC_7-20-32-38_n1 | - | - | - |  | 0.2 | - |  |
| DC_7-20-38_n3-n78 | - | - | 0.4 |  | - | 0.6 |  |
| DC_8_n3-n28-n77-n79 | 0.2 | 0.2 | 0.2 |  | 0.5 | 0.5 |  |
| DC_8-11_n3-n28-n77 | 0.2 | 0.3 | 0.5 |  | 0.2 | 0.5 |  |
| DC_8-11_n3-n77-n79 | 0.2 | 0.3 | 0.5 |  | 0.5 | 0.5 |  |
| DC_8-42_n3-n28-n77 | 0.2 | 0.5 | 0.2 |  | 0.5 | 0.5 |  |
| DC_19-21-42_n1-n77 | - | - | 0.5 |  | 0.2 | 0.5 |  |
| DC_19-21-42_n1-n78 | - | - | 0.5 |  | - | 0.5 |  |
| DC_19-21-42_n1-n79 | - | - | 0.5 |  | - | - |  |
| DC_19-21-42_n77-n79 | - | - | 0.5 |  | 0.5 | - |  |
| DC_19-21-42_n78-n79 | - | - | 0.5 |  | 0.5 | - |  |
| DC_19-42_n1-n77-n79 | 0.3 | 0.5 | 0.3 |  | 0.5 | - |  |
| DC_19-42_n1-n78-n79 | 0.3 | 0.5 | 0.3 |  | 0.5 | - |  |
| DC_20-28-32-38_n1 | 0.2 | 0.2 | - |  | - | - |  |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |  |  |

7.3B.3.3.5	ΔRIB,c for EN-DC six bands

Table 7.3B.3.3.5-1: ΔRIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration4 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-3-7-8_n28-n78 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.5 |  |  |
| DC_1-3-7-8-32_n78 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | - |  | 0.5 |  |  |
| DC_1-3-7-8-40_n78 |  | 0.2 |  | 0.2 |  | - |  | 0.2 |  | 0.41 |  | 0.51 |  |  |
| DC_1-3-7-20_n8-n78 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.5 |  |  |
| DC_1-3-7-20_n28-n78 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.5 |  |  |
| DC_1-3-7-20-32_n78 |  | 0.2 |  | - |  | 0.2 |  | 0.2 |  | - |  | 0.5 |  |  |
| DC_1-3-7-20_n38-n78 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.5 |  |  |
| DC_1-3-7-28_n3-n78 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.5 |  |  |
| DC_1-3-7-28_n7-n78 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.5 |  |  |
| DC_1-3-7-28_n38-n78 |  | 0.2 | 0.2 | 0.2 |  | 0.2 | 0.2 | 0.5 |  |
| DC_1-3-7-28_n40-n78 |  | - |  | - |  | 0.3 |  | 0.2 |  | 0.8 |  | 0.5 |  |  |
| DC_1-3-8-11_n28-n77 |  | 0.2 |  | 0.3 |  | 0.2 |  | 0.5 |  | 0.2 |  | 0.5 |  |  |
| DC_1-3-8-20-28_n78 |  | - |  | - |  | 0.2 |  | 0.2 |  | 0.2 |  | 0.5 |  |  |
| DC_1-7-20-28-32_n3 |  | - |  | - |  | 0.2 |  | 0.2 |  | - |  | - |  |  |
| DC_1-7-20-38_n3-n78 |  | 0.6 |  | 0.6 |  | 0.2 |  | 0.4 |  | - |  | 0.8 |  |  |
| DC_1-8_n3-n28-n77-n79 |  | 0.3 |  | 0.3 |  | 0.2 |  | 0.5 |  | 0.5 |  | 0.5 |  |  |
| DC_1-8-11_n3-n28-n77 |  | 0.2 |  | 0.2 |  | 0.3 |  | 0.5 |  | 0.2 |  | 0.5 |  |  |
| DC_1-8-42_n3-n28-n77 |  | 0.2 | 0.2 | 0.5 |  | 0.2 | 0.5 | 0.5 |  |
| DC_3-7-8-40_n1-n78 |  | 0.2 | - | 0.2 |  | 0.42 | 0.2 | 0.52 |  |
| DC_7-8-20-32-38_n1 |  | - | 0.2 | 0.2 |  | - | - | - |  |
| DC_7-20-28-32-38_n1 |  | - | 0.2 | 0.2 |  | - | 0.2 | - |  |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 3: “-” denotes ΔRIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |  |  |  |  |  |  |

<<End of Change>>