3GPP TSG RAN Meeting #99	RP-230130

Rotterdam, Netherlands, March 20 - 23, 2023		            (Revision of RP‑222840)

# Source:	LG Electronics

# Title:	Revised WID on Rel-18 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL))

# Document for:	Approval

Agenda Item:	9.4.2.4

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Information about Work Items can be found at http://www.3gpp.org/Work-Items

Title: Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)

Acronym: DC_R18_xBLTE_2BNR_yDL2UL

Unique identifier: 961003

NOTE:	For new WIs/SIs leave the Unique identifier empty or you can make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| DC_R18_xBLTE_2BNR_yDL2UL | RAN4 | 961003 | Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure, such as preceding SI or a preceding WI (e.g. if you further enhance a topic).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 961103 | Core part: New WI on Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | Child WID |
| 961203 | Perf. part: New WI on Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | Child WID |

NOTE:	Also related or dependent WIs in other TSGs should be indicated.

3	Justification

All new DC configurations including NR 2bands inter band CA will be defined under this WI. New configurations still emerge from exiting bands and whenever new band is specified, it will create a potential for several new DC of LTE CA for up to 4 different bands DL with 1 band UL + NR CA for 2 different bands DL with 1 band UL.

The DC configurations including NR CA will be introduced in a release independent manner based on TS38.307, which will be updated depending on newly introduced DC configurations including NR CA.

The preconditions to propose DC of LTE CA for up to 4 different bands DL with 1 band UL + NR CA for 2 different bands DL with 1 band UL in Rel-18 are as follows.

- Constituent NR inter band CA including intra band CA for 2 different bands DL with 1 band UL shall be completed and specified in advance.

- Each of the paired DC configurations of 4 different LTE band including intra band CA + 1 NR band used in a certain DC of LTE CA for up to 4 different bands DL with 1 band UL + NR CA for 2 bands DL with 1 band UL shall be completed and specified in advance.

Example 1: If the following configuration is proposed,

| DC Downlink configuration | DC Uplink configuration |
| --- | --- |
| DC_1A-2A_n3A-n4A | DC_1A_n3A, DC_1A_n4A, DC_2A_n3A, DC_2A_n4A |

- NR CA of 2 different bands DL with 1 band UL of CA_n3A-n4A shall be completed and specified in advance.

- DC of DC_1A-2A_n3 and DC_1A-2A_n4A shall be completed and specified in advance.

Example 2: If the following configuration is proposed,

| DC Downlink configuration | DC Uplink configuration |
| --- | --- |
| DC_1C-2A_n3C-n4A | DC_1C_n3C |

- NR CA of 2 different bands DL with 1 band UL of CA_n3C-n4A shall be completed and specified in advance.

- DC of DC_1C-2A_n3C and DC_1C-2A_n4A shall be completed and specified in advance.

When requesting additional band combinations, proponent should follow the protocols listed below.

- Request for additions of band combinations to this WI shall be provided using an agreed template and sent to the 3GPP_TSG_RAN_WG4_NR_BANDS email reflector before a RAN4 Tdoc submission deadline and no new band combinations are allowed to be requested after the deadline except to correct the missing fallback and add more supporting companies for the proposed band combinations.

- When a proponent requests a new band combination, all the next level fallback configurations shall be listed and recorded in the request template and the status (“New”, “Ongoing”, “Completed”) of all the fallback configurations shall be declared accurately and clearly. For “New” fallback configurations, the proponent shall ensure these fallback configurations are also requested together with the higher order band combination in the same meeting.

- A band combination configuration can only be considered as completed when all of the fallback configurations are completed and specified in advance or at the same meeting. It is the responsibility of the proponent to ensure the status of all of the fallback mode configurations. Rapporteurs and other companies are encouraged to check the status of all of the fallback configurations once the higher order band combinations are declared as completed.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

- Specify the band-combination specific RF requirements for all listed DC of LTE CA for up to 4 different bands DL with 1 band UL + NR CA for 2 different bands DL with 1 band UL including at least

  - Applicable frequencies if necessary

  - Applicable bandwidths and bandwidth sets if necessary

- Analyse combinations that have self-desensitization due to following reasons:

  - TX Harmonic and/or intermodulation overlap of receive band

  - TX signal overlap of receiver harmonic frequency

  - TX frequency being in close proximity of one of the receive bands

  - Any other identified reasons such that insufficient cross band isolation, harmonic mixing

- For the combination where self-desensitization exists, specify at least needed

  - ∆TIB, c and ∆RIB, c

  - Reference sensitivity exceptions including MSD test cases

- Add conformance testing in RAN5 specifications (to follow at a later stage)

of all Rel-18 DC of LTE CA for up to 4 different bands DL with 1 band UL + NR CA for 2 different bands DL with 1 band UL that fall into the category is defined by the WI title.

An overview table of these DC configurations is provided in the appended Excel sheet which is updated to add new DC band combinations and revised the status per each DC band combination.

NOTE:	The scope of this work is limited to power class 3.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

This Perf. Part WI has to standardize the Perf. Part requirements:

- Required changes to be added to release independence TS 38.307.

of all Rel-18 DC  combinations that fall into the category is defined by the WI title. See overview table of these DC configurations is provided in the appended Excel sheet.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR Number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 37.718-11-21 | Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) |  | TSG#102 | Sang-Wook Lee, LG Electronics Inc., sangwook1.lee@lge.com |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2: The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-3 | Add new DC band combinations and related RF core requirements | TSG#102 | Core part |
| 38.307 | Release independent manner will be applied to all new DC band combinations according to each DC band combination | TSG#102 | Perf. part |

6	Work item Rapporteur(s)

Sang-Wook Lee,

Company: LG Electronics Inc.

Email: sangwook1.lee@lge.com

7	Work item leadership

R4

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

LG Electronics

Ericsson

Huawei

KT Corp.

LG Uplus

META

Nokia Shanghai Bell

Nokia

NTT DOCOMO, INC.

Samsung

SK Telecom

ZTE