3GPP TSG RAN meeting #99		RP-230139

Rotterdam, Netherlands, March 20 – 23, 2023

Source:	OPPO

Title:	On the scope of Rel-18 NR sidelink evolution WI

Agenda Item:	9.3.1.5

Document for:	Discussion and Decision

# Introduction

During discussions in the last RAN#98-e meeting, the scope of R18 SL-Evo work item was tentatively concluded without the CA work (but continuing the study on FR2 beam management) according to [1] and that the CA objective would be revisit in March 2023 (RAN#99 this meeting), including the possibility of including it in Rel-18 with minimal WG efforts, or dropping it from Rel-18 but re-consider it in Rel-19. The affected WID objectives were updated as followed.

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#98-e RAN#99) - Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, handling the limited capability, power control for simultaneous sidelink TX, packet duplication) - The work is limited to FR1 licensed spectrum and ITS band in FR1. - No specific enhancements of Rel-17 sidelink features with sidelink CA support. - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI) - Study and specify enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2, RAN4] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Work Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

Besides the CA objective, the current work on co-channel coexistence (Co-Ex) between LTE/NR V2X deserves also some attention / discussion in this RAN in order to provide some guidance around the need to include the case of 30kHz SCS in NR V2X to coexist with the LTE counterpart.

Furthermore, another leftover / late discussion topic for this WI in the last RAN#98-e was on documenting agreements and outcomes of study on NR SL beam management in R18 for which can be later used as the basis for a future normative work (e.g., in R19).

Since any decision for the above 3 topics will have impact on the scope of this WI and the workload in the WGs, in this contribution, we will firstly examine the current progress for the started 3 objectives, provide a detailed list of scope retractions to minimize the CA work (if to be supported), provide some recommendations on how to handle higher SCSs in the co-channel coexistence work, and finally suggestion of  options that could be down-selected in this RAN on handling the CA and Co-Ex objectives in this work item.

# Discussion

# Current scope and work progress of R18 SL-Evo WI

Currently 3 out of 4 objectives of the latest WID [1] have started in the WGs (RAN1/2/4), namely sidelink operation in unlicensed spectrum (SL-U), sidelink beam management in FR2 (FR2) and LTE/NR V2X co-channel coexistence (CO-Ex). According to the WI status report submitted to this RAN [2], the completion level (core part) is declared to be 55% and marked with yellow text, suggesting the overall progress is behind the schedule.

During 1Q of 2023, a substantial amount of online session time was spent on the WI in RAN1. Although some progress was made, but it was insufficient to keep up with the progress that is expected by this stage of the WI with only 3 RAN1 meetings remaining in R18. Judging from the remaining open issues listed in the SR (at least for RAN1), the list of topics to be finalized by RAN#101 (Sep’ 2023) is still quite considerable. It becomes questionable, even with the current scope of work (keeping the same workload) and without increasing the allocated TUs (currently 2 TUs), is it feasible to complete the remaining work within only 3 WG meetings. It is also observed that the FR2 beam management study still at the beginning stage (so far we have had only one meeting discussing technical solutions). Therefore, it seems infeasible to further increase the scope of this WI and workload in RAN1, and still expecting the work to complete in time by RAN#101.

Observation 1: The current work progress is behind the expected schedule and the list of remaining open issues is still substantial for the already started objectives. Without increasing the allocated TUs significantly, it is infeasible to further increase the scope of this WI and workload in RAN1, and still expecting the work to complete in time by RAN#101.

# Scope of the SL CA objective in Rel-18

According to the WID [1] (copied in the Introduction section), it is clear the target functionality for this objective is to support NR sidelink CA operation based on the existing LTE sidelink CA operation. In working towards this goal while minimizing the specification effort to support the CA feature for NR sidelink, a list of possible work areas for restriction was suggested and discussed in the last RAN#98-e meeting [3]. However, it was decided that the scope of CA objective should be revisited in this RAN plenary meeting with the possibility of including it in R18 with minimal WG efforts. The original list of restrictions from [3] are:

- Only Mode 2 operation in FR1 ITS band (intra-band CA only)

- For a CA capable UE, the UE supports transmitting SL HARQ feedback on the same carrier on which it receives the associated PSSCH (i.e., no PSFCH enhancement compared to R16)

- No enhancement related to RSRP feedback and CSI feedback compared to Rel-16.

- In Mode 1, no enhancement to DCI and PUCCH

As can be seen, the main mechanism to reduce the work scope for CA in R18 (besides limiting the supported resource allocation schemes) is based on a per-carrier operation. In the last meeting, SL HARQ feedback over PSFCH transmissions was identified as one big topic for work restriction (since the CA operation should be backward compatible to R16/17 UEs too). Following this “per-carrier operation” principle, some other existing SL procedures and feedback schemes should be also restricted for the CA work. Additionally, when considering the impact of AGC and resource allocation due to potential different SCS between carriers (which was not an issue in LTE CA), further assumptions / restrictions could be also applied to minimize the WG effort. These additional restrictions are identified as followed.

- Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues

  - Slots for PSFCH are aligned among the carriers for CA

- No enhancement related to congestion control compared to Rel-16.

  - Remain to be per-carrier operation

- No enhancement related to SL synchronization procedure compared to Rel-16.

  - Sidelink synchronization resources are aligned across CA carriers, same as LTE sidelink CA

- No enhancement related to SCI transmissions on PSCCH/PSSCH compared to Rel-16

  - SL resource indication remain to be per-resource pool and per-carrier basis

When all of the above restrictions are included for NR sidelink, the CA operation will be very comparable to the LTE sidelink CA operation. As such, the specification effort in the WG could then be really minimized. However, it should be noted that still significant amount of work needs to be done in RAN2 (leading WG) in order to support CA in R18, especially for the case of SL unicast communication due to complicated CA configuration over the PC5-RRC connection.

Observation 2: A full list of possible restrictions in order to minimize WG efforts to support a basic version of CA in R18 (just as LTE sidelink CA), but still sufficient for V2X operation in the ITS band (n47) could be as followed.

- Only Mode 2 operation

- Intra-band CA only in FR1 ITS band (i.e., Band n47)

- Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues

  - Slots for PSFCH are aligned among the carriers for CA

- No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, SL synchronization procedure, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation)

  - SL resource indication remain to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI)

  - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH

  - Synchronization resources are aligned across CA carriers, same as LTE sidelink CA

# Handling of NR V2X with 30kHz SCS in Co-Ex objective

One main contentious issue that has been discussed over the past few RAN1 meetings and in the last RAN#98-e meeting is related to the supporting higher SCSs in the dynamic resource pool sharing scheme for co-channel coexistence. According to the existing RAN1 agreement relevant to this issue, copied in the following, only the 15kHz SCS is included for Co-Ex while higher SCSs such as 30kHz and 60kHz are FFS.

Agreement in RAN1#110 For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints: - NR SL resource pool is configured with 15 kHz SCS. - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools - For NR PSFCH (if configured), at least the following alternatives are studied: - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions. - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH. - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots. - FFS: periodicities of the set.

From V2X deployment perspective, if legacy UEs are expected to operate in the same channel (where the support of 30kHz is mandatory and other SCSs are optional in R16/17) and a NR sidelink UE can support only one carrier, it may be necessary to consider higher SCS (i.e., 30kHz) in the Co-Ex objective scope. In this case, only 30kHz SCS should be consider (i.e., 60kHz becomes irrelevant).

On the other hand, if we could agree that at least two NR sidelink carriers should be supported by a Co-Ex capable UE in R18, and that one carrier (with 15kHz SCS) could be used for co-channel coexistence with LTE V2X and the other carrier for coexistence with legacy R16/17 UEs. In this case, RAN1 could continue to work on 15kHz SCS case only for Co-Ex.

Alternatively, since 30kHz SCS is more relevant to the V2X deployment, RAN could provide guidance to RAN1 to consider the 30kHz SCS only in dynamic resource pool sharing, instead of specifying two separate Co-Ex solutions in R18 (one for 15kHz and another one for 30kHz).

Finally, if both 15kHz and 30kHz SCSs should be supported in R18, we should target to specify simple solutions with minimal WG effort. In this case, we could consider the following simplifications in the Co-Ex work. For example, in order to avoid the issue of AGC due to different SCSs used between LTE and NR V2X operation (1 LTE subframe = 2 NR slots), instead of using a complicated scheme such as MCSt / dual-slot transmission (which would also involve further complicated specification work on resource (re)selection, filing of the GP symbol between the dual-slots and handling of the last PSSCH symbol at the end of the dual-slots), one could always avoids NR transmission in slots that time overlap with LTE reservations. This could be easily implemented in the MAC specification.

For handling of PSFCH transmission(s) in NR and causing an AGC issue within a LTE subframe, which is a common problem in both NR 15kHz and 30kHz SCSs, the solution may be the same or different for the two SCS cases. So far, RAN1 has had some difficult discussions for the 15kHz SCS case and some of the remaining options may involve very complicated design / changes to the NR slot structure. It is undesirable to have such changes at this late stage. Therefore, if RAN decides to additionally support 30kHz SCS in the Co-Ex objective scope, one possibility is to not handle the case of PSFCH transmissions in NR V2X. The technical reason (which has been repeated in RAN1) is that the interference coming from PFSCH transmission affects only 2 symbols in 15kHz and 1.5 symbols in 30kHz in an LTE subframe. Since PSFCH transmission occurs only for groupcast and unicast transmissions, the occurrences may not be frequent. On the other hand, dropping PSFCH transmissions will also impact to LTE V2X performance. Hence there is no solution that completely resolve the AGC issue. In this sense, whether or not to transmit PSFCH could be left to UE implementation decision (e.g., based on LTE reservations).

Overall, to summarize the above discussions, it is observed that:

Observation 3:

- Since 30kHz SCS is a basic feature to be supported by a NR sidelink UE in R16/17, whereas 15kHz and 60kHz SCSs are optional, the support of higher SCS in the Co-Ex objective should be limited to the case of 30kHz only.

- For the handling of higher SCSs in the Co-Ex objective, one of the following options could be considered:

  - Alternative A: Agree that at least two NR sidelink carriers should be supported by a Co-Ex capable UE in R18, and RAN1 focus only on the 15kHz SCS in dynamic resource pool sharing for Co-Ex.

  - Alternative B: RAN1 focus only on the 30kHz SCS in dynamic resource pool sharing for Co-Ex, since 30kHz SCS is more relevant to the V2X deployment and supported by legacy R16/17 UEs. That is, dropping the 15kHz SCS case.

  - Alternative C: Support both 15kHz and 30kHz SCSs in dynamic resource pool sharing and provide the following guidance to the WGs or make following changes in the WID

    - For NR 30kHz SCS, always avoid resources in slots that time overlap with LTE reservation(s) in the MAC layer.

    - For the AGC issue cause by PSFCH transmissions, no specific solution is specified for both 15kHz and 30kHz SCS. It can be up to UE implementation to resolve this issue.

# Overall WI scope for the remaining timeframe in R18

As mentioned earlier, also in Observation 1, the work progress is behind the schedule and it is infeasible to further increase the scope and workload for the R18 SL-Evo WI. However, in order to accommodate either the higher SCS 30kHz work in the Co-Ex objective or completing the CA objective in R18, the following options could be selected.

Proposal 1:

- Option 1:

  - Continue the Co-Ex objective in R18 with adoption of one of the alternatives (A, B or C) from Observation 3.

  - Re-consider the support of CA operation for NR sidelink in Rel-19.

- Option 2:

  - Close/stop the Co-Ex objective in RAN#99.

  - Start the CA objective with restricted CA operation (e.g., those from Observation 2).

- Option 3:

  - Continue the Co-Ex objective in R18 with adoption of one of the alternatives (A, B or C) from Observation 3.

  - Study CA operation for NR sidelink in RAN2 only (with restrictions if there is a concern on RAN2 TU and workload, e.g., those from Observation 2).

# Placeholder for FR2 study agreements

In the last RAN#98-e meeting, there was a late discussion / suggestion on documenting agreements and outcomes of WG’s study on NR SL beam management in Rel-18 for the purpose of using it as a reference/baseline when we want to develop a specification support for this feature in a future release (e.g., in Rel-19/20). During the discussion, concerns were raised on whether the study outcomes and agreements captured in RAN1 Chair’s notes and WI status reports were sufficient enough or useable since there is no version control. On the other hand, however, if a full/conventional 3GPP technical report is to be produced, additional significant workload will be placed on RAN1 and RAN2 delegates to translate agreements, reviewing the contents and spending considerable / precious online time discussing TR conclusion. In our view, it is desirable to avoid this kind of approach as the allocated RAN1 TUs for this WI are already depleted in handling the normal work.

Alternatively, if the goal should aim to minimize any additional workload in the WGs to translate and review text proposals and avoid possibly attempting to reach a study conclusion / recommendation for the future normative work, one could set up a new 3GPP internal TR capturing directly agreements and outcomes in different sections of the report without translation (e.g., one section for initial beam paring, one section for beam maintenance, another section for beam failure detection and recover). Since the FR2 SL beam management work in Rel-18 involves just a study, it is not necessary to draw a conclusion on which of the beam management solution(s) should be adopted and specified. It would be sufficient to capture all different/feasible solutions as the study outcome and selection of a final scheme to be specified could be down-selected during the normative phase.

Proposal 2: If there is a need to capture all study outcomes and agreements for the study on NR sidelink beam management in FR2 licensed spectrum, a new 3GPP internal TR could be created as a placeholder to capture all WGs’ study agreements/working assumptions and even conclusions from RAN1 and RAN2.

# Conclusion

in this contribution, the current completion rate, WI progress and remaining open issues for the 3 started objectives in the WGs were examined in Section 2.1. Based on the current progress and the fact there is only 3 WG meetings left until the target completion date for R18 in RAN1, a list of possible restrictions on the potential CA work is provided in Section 2.2. In Section 2.3, we provided some alternative ways forward on how to manage higher SCSs in the co-channel coexistence work. And finally, several options are given on how to jointly handle the Co-Ex and CA work in the remainder time of Rel-18. Overall, our observations and proposals are listed in the following.

Observation 1: The current work progress is behind the expected schedule and the list of remaining open issues is still substantial for the already started objectives. Without increasing the allocated TUs significantly, it is infeasible to further increase the scope of this WI and workload in RAN1, and still expecting the work to complete in time by RAN#101.

Observation 2: A full list of possible restrictions in order to minimize WG efforts to support a basic version of CA in R18 (just as LTE sidelink CA), but still sufficient for V2X operation in the ITS band (n47) could be as followed.

- Only Mode 2 operation

- Intra-band CA only in FR1 ITS band (i.e., Band n47)

- Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues

  - Slots for PSFCH are aligned among the carriers for CA

- No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, SL synchronization procedure, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation)

  - SL resource indication remain to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI)

  - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH

  - Synchronization resources are aligned across CA carriers, same as LTE sidelink CA

Observation 3:

- Since 30kHz SCS is a basic feature to be supported by a NR sidelink UE in R16/17, whereas 15kHz and 60kHz SCSs are optional, the support of higher SCS in the Co-Ex objective should be limited to the case of 30kHz only.

- For the handling of higher SCSs in the Co-Ex objective, one of the following options could be considered:

  - Alternative A: Agree that at least two NR sidelink carriers should be supported by a Co-Ex capable UE in R18, and RAN1 focus only on the 15kHz SCS in dynamic resource pool sharing for Co-Ex.

  - Alternative B: RAN1 focus only on the 30kHz SCS in dynamic resource pool sharing for Co-Ex, since 30kHz SCS is more relevant to the V2X deployment and supported by legacy R16/17 UEs. That is, dropping the 15kHz SCS case.

  - Alternative C: Support both 15kHz and 30kHz SCSs in dynamic resource pool sharing and provide the following guidance to the WGs or make following changes in the WID

    - For NR 30kHz SCS, always avoid resources in slots that time overlap with LTE reservation(s) in the MAC layer.

    - For the AGC issue cause by PSFCH transmissions, no specific solution is specified for both 15kHz and 30kHz SCS. It can be up to UE implementation to resolve this issue.

Proposal 1:

- Option 1:

  - Continue the Co-Ex objective in R18 with adoption of one of the alternatives (A, B or C) from Observation 3.

  - Re-consider the support of CA operation for NR sidelink in Rel-19.

- Option 2:

  - Close/stop the Co-Ex objective in RAN#99.

  - Start the CA objective with restricted CA operation (e.g., those from Observation 2).

- Option 3:

  - Continue the Co-Ex objective in R18 with adoption of one of the alternatives (A, B or C) from Observation 3.

  - Study CA operation for NR sidelink in RAN2 only (with restrictions if there is a concern on RAN2 TU and workload, e.g., those from Observation 2).

Proposal 2: If there is a need to capture all study outcomes and agreements for the study on NR sidelink beam management in FR2 licensed spectrum, a new 3GPP internal TR could be created as a placeholder to capture all WGs’ study agreements/working assumptions and even conclusions from RAN1 and RAN2.

# References

[1]	RP-222806 “WID revision: NR sidelink evolution”, OPPO, RAN#98-e

[2]	RP-230075 “Status report for WI NR sidelink evolution; rapporteur: OPPO”, RAN#99

[3]	RP-223495 “Moderator’s summary of discussion [98e-17-R18-SLEvo]”, Moderator (OPPO), RAN98-e