3GPP TSG-RAN Meeting #99	RP-230184

Rotterdam, Netherlands, 20th – 23rd March, 2023

Agenda item:	9.3.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz

Document for:	Discussion and Decision

# Introduction

In [1], RAN1 sent an LS to RAN Plenary requesting information about the transmission bandwidths to be supported by the above work item.

We note first that it is already agreed to support 15 PRB transmission bandwidth within the 3MHz channel bandwidth.

As discussed in the work WID in [2], the Future Railway Mobile Communication System (FRMCS) is planned to be deployed from approximately 2025 onwards, such that it is initially operated in parallel with the currently deployed GSM-R system on the 900 MHz spectrum (2x5.6 MHz FDD). To facilitate efficient and gradual migration from GSM-R to 5G NR based FRMCS during the decade 2025-2035, there is a need to support a range of transmission bandwidths for the 3 and 5 MHz channel bandwidths.

At one end, it is necessary to support a 12 PRB transmission bandwidth within the 3 MHz channel bandwidth, in order to enable deployment of 14 GSM-R carriers alongside NR at the start of migration. As explained in [3], this requirement comes from many rail operators who wish to migrate their networks to 5G NR based technology.

Further, within the 5 MHz channel bandwidth, standardized support of at least 18 and 20 PRBs would provide optimal support for simultaneous deployment of smaller numbers of GSM-R carriers (e.g. 8 or 6) alongside NR. Other numbers of GSM-R carriers would then be able to be supported in deployment without explicit standards support.

# Conclusion

As explained above, RAN should reply to RAN1 that transmission bandwidths of 12, 15, 18 and 20 PRBs should be supported to facilitate the required FRMCS migration steps.

A draft reply LS is appended to this tdoc.

# References

[1]   RP-230033	LS on transmission bandwidths for NR on dedicated spectrum less than 5 MHz (R1- 2302186)

[2]   RP-222645	Revised WID: NR support for dedicated spectrum less than 5MHz for FR1	Nokia, Nokia Shanghai Bell

[3]   RP-230189	UIC response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz	Union Inter. Chemins de Fer

3GPP TSG RAN#99			RP- 23xxxx

Rotterdam, Netherlands, 20th – 23rd March, 2023

Title:	[DRAFT] LS to RAN on transmission bandwidths for NR on dedicated spectrum less than 5 MHz

Response to:	R1-2302186/RP-230033

Release:	Rel-18

Work Item:	          NR_FR1_lessthan_5MHz_BW-Core

Source:	Nokia, Nokia Shanghai Bell [RAN]

To:	RAN1

Cc:

Contact Person:

#### Name:	Matthew Baker

E-mail Address:	<Matthew.Baker@nokia.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN would like to thank RAN WG1 for their LS on transmission bandwidths for NR on dedicated spectrum less than 5 MHz.

RAN Plenary has discussed the possible transmission bandwidth options for 3 MHz and 5 MHz channel bandwidths for the spectrum allocations on the bands of interest in this work item, namely n100, n8, n26 and n28. In addition to the transmission bandwidth of 15 PRBs in a 3 MHz channel bandwidth that meets the demand in bands n8, n26 and n28, there is a need to support additional transmission bandwidths for band n100.

As discussed in the WID in [1], the Future Railway Mobile Communication System (FRMCS) is planned to be deployed from approximately 2025 onwards, such that it is initially operated in parallel with the currently deployed GSM-R system on the 900 MHz spectrum (2x5.6 MHz FDD). To facilitate efficient and gradual migration from GSM-R to 5G NR based FRMCS during the decade 2025-2035, there is a need to support (in addition to 15 PRBs) also transmission bandwidths of 12 PRBs for the 3 MHz channel bandwidth, and 18 and 20 PRBs for the 5 MHz channel bandwidth.

2. Actions: RAN respectfully requests RAN1 to take the above information into account for their further work.

3. Dates of Next RAN Meetings:

TSG RAN Meeting #100		June 12th – 14th, 2023	Taipei, Taiwan

TSG RAN Meeting #101		September 11th – 15th, 2023	TBC, India

References

[1] RP-222645, “Revised WID: NR support for dedicated spectrum less than 5MHz for FR1”, Nokia, Nokia Shanghai Bell, RAN#95e