3GPP TSG-RAN Meeting #99	RP-230189

Rotterdam, Netherlands, 20th – 23rd March, 2023

Agenda item:	9.3.4.1

Source:	Union International Chemins de Fer

Title:	UIC response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz

Document for:	Discussion

# Introduction

With this document, UIC as the worldwide professional association representing the railway sector wants to provide a response to RAN on the [1]  R1-2302186 LS on transmission bandwidths for NR on dedicated spectrum less than 5MHz.

With the 3GPP work on the [2] WID NR_RAIL_EU_900MHz, the band n100 has been introduced in the 900MHz frequency band for Europe for Rail Mobile Radio (RMR) in 3GPP R17.

With the [3] Revised WID NR support for dedicated spectrum less than 5MHz for FR1 the work has been initiated on bandwidths <5 MHz (e.g., from around 3 MHz upwards) to enable during the migration phase the parallel operation of Future Railway Mobile Communication System (FRMCS) and GSM-R and massive infrastructure reuse.

In Europe, railway infrastructure managers are legally obliged to operate the GSM-R networks for train-to-track communications at a prescribed quality level until some future date. While GSM-R remains operational, the 5G NR based FRMCS successor networks have to be introduced. This will result in a prolonged (possibly up to 10 years) migration period where both GSM-R and FRMCS networks will have to be in operation in parallel.

The current GSM-R networks in most EU countries have available a total of 2x 4MHz FDD, i.e. 19 GSM-R channels. A few countries may use the extended GSM-R band of up to 2x7MHz FDD. For high traffic density areas such as large railway stations, junctions and shunting areas and in some cases even for the entire national coverage, the 2x4MHz is fully utilized in order to achieve the high network quality required for railway operation. Current usage and further roll-out of the European Train Control System (ETCS) amplifies the need for spectrum, which in some countries has necessitated usage of the extended GSM-R band of up to 2x7MHz.

When introducing 5G NR for FRMCS in band n100 of 2x5.6MHz FDD, the railway infrastructure managers need to maintain the network quality level of GSM-R while reducing the available bandwidth or number of GSM-R channels. At the initial phase of this migration, only a limited traffic volume may be converted from GSM-R to FRMCS as only a limited number of rolling stock may have been provided with FRMCS capability due to the long period required to upgrade the rolling stock. GSM-R network design indicates that in high traffic density areas up to 14 GSM-R channels may be needed to allow the required GSM-R network quality level. The table below shows that with 14 GSM-R channels an NR transmission bandwidth of only 12 5G NR RBs would be possible.

# Conclusion

Based on the above, UIC, representing the European railway infrastructure managers, wants to highlight the value of support in 3GPP of a 12 transmission bandwidth in a 3MHz channel bandwidth. In case in the initial migration phase only a transmission bandwidth of 15 RBs in a 3MHz channel bandwidth would be possible, the GSM-R network redesign would be significantly complicated.

For the case “up to below 5MHz”, the ideal situation would be when any number of RBs between 15 and 20 could be selected depending on FRMCS traffic needs. However UIC recognizes the complexity of this and can accept that in the “up to below 5MHz” case only a limited number of transmission bandwidths in a 5MHz channel bandwidth would be supported in 3GPP.

# References

[1]   R1-2302186 LS on transmission bandwidths for NR on dedicated spectrum less than 5MHz

[2]   RP-221768 WID Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR)

[3]   RP-222645	Revised WID: NR support for dedicated spectrum less than 5MHz for FR1