3GPP TSG-RAN5 Meeting #98	R5-231957

# Athens, Greece, Feburary 27th - March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2190 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap RRM TC 16.5.4.1 - CBW change 1Rx with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add minimum requirements for FR1 RedCap UE-specific CBW change RRM test case. - To add RedCap RRM TC 16.5.4.1 – NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Minimum requirements for FR1 RedCap UE-specific CBW change RRM test case. - RedCap RRM TC 16.5.4.1 - NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5.4.0 (new), 16.5.4.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0244 TS 38.533 CR 2225,2230 TS 38.903 CR 0482 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-230479r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.5.4	UE specific CBW change for RedCap

16.5.4.0	Minimum conformance requirements

16.5.4.0.1	Minimum conformance requirements for UE specific CBW change

The requirements in this clause apply for a RedCap UE receives reconfiguration of offsetToCarrier or carrierBandwidth to change channel bandwidth.

After the UE receives RRC reconfiguration involving offsetToCarrier or carrierBandwidth change on the old CBW, UE shall be able to receive PDSCH/PDCCH on the DL BWP with BWP ID firstActiveDownlinkBWP-Id or transmit PUSCH on the UL BWP with BWP ID firstActiveUplinkBWP-Id  of the new CBW right after a time duration of TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length slots which begins from the beginning of DL slot n, where

-	DL slot n is the last slot overlapping with the PDSCH containing the RRC command, and

-	TRRCprocessingDelay is the length of the RRC procedure delay in millisecond as defined in TS 36.331 [29] clause 11.2 is the corresponding RRC message is embedded in E-UTRA RRC message, otherwise it is the length of the RRC procedure delay in millisecond as defined in TS 38.331 [13] clause 12, and

-	TCBWchangeDelayRRC=6 ms is the time used by the UE to perform CBW change.

The UE is not required to transmit UL signals or receive DL signals during the above defined time duration TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length on the cell where UE-specific CBW change occurs. When THARQ> TRRCprocessingDelay a longer switching delay is allowed. Where THARQ is the time between DL data transmission and acknowledgement as specified in TS 38.213 [8].

The normative reference for this requirement is TS 38.133 [6] clause 8.13A.1 and 8.13A.2.

16.5.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE

16.5.4.1.1	Test purpose

To verify the UE specific CBW change delay requirement defined in 38.133 [6] clause 8.13A.

16.5.4.1.2	Test applicability

This test applies to all types of NR RedCap UEs with 1 Rx from Release 17 onwards.

16.5.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.5.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.5.4.1.

16.5.4.1.4	Test description

Same as the test description given in clause 6.5.8.1.

16.5.4.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.5.8.1 with following exceptions:

-	Table 6.5.8.1.4.1-1 is replaced by Table 16.5.4.1.4.1-1.

-	Table 6.5.8.1.4.1-2 is replaced by Table 16.5.4.1.4.1-2.

-	Table 6.5.8.1.4.1-3 is replaced by Table 16.5.4.1.4.1-3.

Table 16.5.4.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.5.4.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.5.4.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.5.4.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.5.4.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.5.4.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.5.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 16.5.4.1.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

16.5.4.1.4.2	Test procedure

Same as the test procedure given in clause 6.5.8.1.4.2.

16.5.4.1.4.3	Message contents

Same as the message contents given in clause 6.5.8.1.4.2.

16.5.4.1.5	Test requirement

Same as the test requirements given in clause 6.5.8.1.5 with followings exceptions:

-	Table 6.5.8.1.5-1 is replaced by Table 16.5.4.1.5-1.

Table 16.5.4.1.5-1: NR Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| TDD configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  | Config 1,2,4 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  | 20 MHz: NRB,c = 51 |
| Active DL BWP ID |  |  |  | 0 |
| Initial DL BWP Configuration (BWP-1) |  |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration (BWP-1) |  |  |  | ULBWP.0.2 |
| Initial Condition | Active DL CBW Configuration (CBW-1) |  |  | DLCBW.1.1 |
|  | Active UL CBW Configuration (CBW-1) |  |  | ULCBW.1.1 |
| Final Condition | Active DL CBW Configuration (CBW-2) |  |  | DLCBW.1.2 |
|  | Active UL CBW Configuration (CBW-2) |  |  | ULCBW.1.2 |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1,4 |  | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  |  | SMTC.1 RedCap FR1 |
| TRS Configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  | 1x1 |
| Propagation Condition |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |
| NocNote 2 |  | Config 1,2,4 | dBm/SCS | -104 |
|  |  | Config 3 |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2,4 | dBm/SCS | -87 |
|  |  | Config 3 |  | -84 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | Config 1,2,4 | dBm/9.36MHz | -58.96 |
|  |  | Config 3 | dBm/18.36MHz | -56.04 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.1 is linked with ULBWP.0.1; DLBWP.1.1 is linked with ULBWP.1.1; as defined in clause 12 of TS 38.213 [3]. |  |  |  |  |

<End of modified section 1>