3GPP TSG-RAN5 Meeting #98	R5-231842

# Athens, Greece, Feb 27th - Mar 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0625 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to HST test case 5.2A.3.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Message content is missing in test case 5.2A.3.5.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Message content has been added in test case 5.2A.3.5.1 Editor’s note has been removed Revise dmrs-AdditionalPosition value in TC 5.2A.2.5.1 and TC 5.2A.3.5.1 Add Ericsson as co-source |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work item cannot be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.3.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

5.2A.2.5.1	2RX PDSCH Demodulation Performance for HST-DPS CA

5.2A.2.5.1.1	Test Purpose

To verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions in the HST-DPS scenario for multiple CA configurations and with different channel models, MCSs and SCS for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput per CC.

5.2A.2.5.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 2DL CA.

5.2A.2.5.1.3	Test description

5.2A.2.5.1.3.1	Initial conditions

Same initial conditions as specified in clause 5.2A.2.1.1.3.1 with the following exception

Channel BW to be tested: largest aggregated bandwidth combination as per Table 5.1.1.5.2-2.

CA capability to be tested: As per table 5.1.1.5.2-1

Table 5.2A.2.5.1.3.1-1: Test point selection table

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.1.0-1 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1.0-3 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1.0-1 and Table 5.2A.3.1.0-3 per CC |
| 4 (note 2) | FDD 15 kHz + TDD 15 kHz | As defined in Table 5.2A.3.1.0-1 and Table 5.2A.3.1.0-2 per CC |
| 5 (note 3) | TDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1.0-2 and Table 5.2A.3.1.0-3 per CC |
| Note 1: For each test point, select any one of the CA configurations which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate based on the equation DataRate=10−3j=1JTBSj2μj Note 2: Test point 4 can be skipped if test point 3 is verified. Note 3: Test point 5 can be skipped if test point 3 or 4 is verified. |  |  |

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1, Table 5.2A-1 to Table 5.2A-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.3.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2A.2.5.1.3.3.

5.2A.2.5.1.3.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 5.2A.2.5.1.3.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2A.2.5.0-2 as appropriate on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to 5.2A.2.5.1.3.4-1 to 5.2A.2.5.1.3.4-4 as appropriate on both PCC and SCCs.

6.	Measure the average throughput per each component carrier for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL and decide pass or fail according to Table G.1.5-1 in Annex G.1.5.

7.	Repeat steps from 1 to 6 for each test points in Table 5.2A.2.5.1.3.1-1 as appropriate.

5.2A.2.5.1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2 with the following exceptions:

Table 5.2A.2.5.1.3.3-1: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | Not present |  |  |
| } |  |  |  |

Table 5.2A.2.5.1.3.3-2: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n8 |  |  |
| } |  |  |  |

<<<UNCHANGED PARTS SKIPPED >>>

5.2A.3.5.1	4RX PDSCH Demodulation Performance for HST-DPS CA

5.2A.3.5.1.1	Test Purpose

To verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions in the HST-DPS scenario for multiple CA configurations and with different channel models, MCSs and SCS for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput per CC.

5.2A.3.5.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 2DL CA and 4Rx antenna ports.

5.2A.3.5.1.3	Test description

5.2A.3.5.1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

Channel BW to be tested: largest aggregated bandwidth combination as per Table 5.1.1.5.2-2.

CA capability to be tested: As per table 5.1.1.5.2-1

Table 5.2A.3.5.1.3.1-1: Test point selection table for multiple CA configurations for HST-DPS with 1 active TCI state

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1-1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.5.0-3 |
| 1-2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.5.0-5 |
| 1-3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.5.0-3 and Table 5.2A.3.5.0-5 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. |  |  |

Table 5.2A.3.5.1.3.1-2: Test point selection table for multiple CA configurations for HST-DPS with 2 active TCI states

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 2-1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.5.0-4 |
| 2-1 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.5.0-6 |
| 2-3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.5.0-4 and Table 5.2A.3.5.0-6 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. |  |  |

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.x for TE diagram and clause A.3.2.6 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1, Table 5.2A-1 to Table 5.2A-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.3.3.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2A.3.5.1.3.3.

5.2A.3.5.1.3.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 5.2A.3.5.1.3.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2A.3.5.0-2 to 5.2A.3.5.0-8 as appropriate on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 5.2A.3.5.1.3.4-1 to 5.2A.3.5.1.3.4-4 as appropriate on both PCC and SCC.

6.	Measure the average throughput per each component carrier for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL and decide pass or fail according to Table G.1.5-1 in Annex G.1.5.

7.	Repeat steps from 1 to 6 for each test points in Table 5.2A.3.5.1.3.1-1 and Table 5.2A.3.5.1.3.1-2 as appropriate.

5.2A.3.5.1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2 with the following exceptions:

Table 5.2A.3.5.1.3.3-1: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | Not present |  |  |
| } |  |  |  |

Table 5.2A.3.5.1.3.3-2: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n8 |  |  |
| } |  |  |  |

Table 5.2A.3.5.1.3.3-3: NZP-CSI-RS-Resource for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | i-1 for CSI-RS resource #i, i=1,2,3,4,5,6,7,8 |  |  |
| qcl-InfoPeriodicCSI-RS | 2 for CSI-RS resource #1, #2, #3, #4 3 for CSI-RS resource #5, #6, #7, #8 | TCI-StateId for TCI-State #2 for CSI-RS resource #1, #2, #3, #4 TCI-StateId for TCI-State #3 for CSI-RS resource #5, #6, #7, #8 |  |
| } |  |  |  |

Table 5.2A.3.5.1.3.3-4: CSI-RS-ResourceMapping for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-9 with condition TRS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 5 for CSI-RS resource #1 and #3 9 for CSI-RS resource #2 and #4 6 for CSI-RS resource #5 and #6 10 for CSI-RS resource #7 and #8 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |  |
| } |  |  |  |

Table 5.2A.3.5.1.3.3-5: CSI-ResourcePeriodicityAndOffset for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots10 | 1 for CSI-RS resource #1, #2, #5, #6 2 for CSI-RS resource #3 #4, #7, #8 | periodicity: 10 slots. offset: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 | SCS 15kHz |
| Slots20 | 1 for CSI-RS resource #1, #2, #5, #6 2 for CSI-RS resource #3 #4, #7, #8 | periodicity: 20 slots. offset: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 | SCS 30kHz |
| } |  |  |  |

Table 5.2A.3.5.1.3.3-6: NZP-CSI-RS-ResourceSet for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 0 for Resource set #1 1 for Resource set #2 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries |  | Resource set #1 |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1 CSI-RS resource #1 |  |
| NZP-CSI-RS-ResourceId[2] | 1 | entry 2 CSI-RS resource #2 |  |
| NZP-CSI-RS-ResourceId[3] | 2 | entry 3 CSI-RS resource #3 |  |
| NZP-CSI-RS-ResourceId[4] | 3 | entry 4 CSI-RS resource #4 |  |
| } |  |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 4 entries |  | Resource set #2 |
| NZP-CSI-RS-ResourceId[1] | 4 | entry 1 CSI-RS resource #5 |  |
| NZP-CSI-RS-ResourceId[2] | 5 | entry 2 CSI-RS resource #6 |  |
| NZP-CSI-RS-ResourceId[3] | 6 | entry 3 CSI-RS resource #7 |  |
| NZP-CSI-RS-ResourceId[4] | 7 | entry 4 CSI-RS resource #8 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2A.3.5.1.3.3-7: NZP-CSI-RS-Resource for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 8 for CSI-RS resource #9 9 for CSI-RS resource #10 |  |  |
| qcl-InfoPeriodicCSI-RS | 0 for CSI-RS resource #9 1 for CSI-RS resource #10 | TCI-State #0 for CSI-RS resource #9 TCI-State #1 for CSI-RS resource #10 |  |
| } |  |  |  |

Table 5.2A.3.5.1.3.3-8: CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 12 for CSI-RS resource #9 13 for CSI-RS resource #10 | l0=12 for CSI-RS resource #9 l0=13 for CSI-RS resource #10 |  |
| } |  |  |  |

Table 5.2A.3.5.1.3.3-9: CSI-ResourcePeriodicityAndOffset for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots20 | 0 | periodicity = 20slots. offset = 0 slots | SCS 15kHz |
| Slots40 | 0 | periodicity = 40 slots. offset = 0 slots | SCS 30kHz |
| } |  |  |  |

Table 5.2A.3.5.1.3.3-10: NZP-CSI-RS-ResourceSet for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 2 for Resource set #3 3 for Resource set #4 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry |  | Resource set #3 |
| NZP-CSI-RS-ResourceId[1] | 8 | entry 1 CSI-RS resource #9 |  |
| } |  |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry |  | Resource set #4 |
| NZP-CSI-RS-ResourceId[1] | 9 | entry 1 CSI-RS resource #10 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2A.3.5.1.3.3-11: TCI-State

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-190 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-State ::= SEQUENCE { |  |  |  |
| tci-StateId | 0 for TCI state #0 1 for TCI state #1 2 for TCI state #2 3 for TCI state #3 |  |  |
| qcl-Type1 SEQUENCE { |  |  |  |
| bwp-Id | BWP-Id of active BWP |  | TCI state #0, TCI state #1 |
|  | Not present |  | TCI state #2, TCI state #3 |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 0 | CSI-RS resource #1 | TCI state #0 |
|  | 4 | CSI-RS resource #5 | TCI state #1 |
| ssb | 0 | SSB #0 | TCI state #2 |
|  | 1 | SSB #1 | TCI state #3 |
| } |  |  |  |
| qcl-Type | typeA |  | TCI state #0, TCI state #1 |
|  | typeC |  | TCI state #2, TCI state #3 |
| } |  |  |  |
| } |  |  |  |

5.2A.3.5.1.3.4	Test Requirement

Tables 5.2A.3.5.1.3.4-1 to 5.2A.3.5.1.3.4-4 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 and A.3.2.2 for each component carrier for throughput test point combination shall meet or exceed the specified value in Tables 5.2A.3.5.1.3.4-1 to 5.2A.3.5.1.3.4-4 for the specified SNR including test tolerances for the test points listed in Table 5.2A.3.5.1.3.1-1 and Table 5.2A.3.5.1.3.1-2.

Table 5.2A.3.5.1.3.4-1: Test requirements for single carrier performance for FDD 15 kHz SCS for HST-DPS CA configurations with 1 active PDSCH TCI state

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-15.1 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.1 |
| 10 | R.PDSCH.1-8.4 FDD | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.4 |
| 15 | R.PDSCH.1-15.2 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.3 |
| 20 | R.PDSCH.1-15.3 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.1 |
| 25 | R.PDSCH.1-15.4 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.4 |
| 30 | R.PDSCH.1-15.5 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.5 |
| 35 | R.PDSCH.1-16.3 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.2 |
| 40 | R.PDSCH.1-16.1 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.3 |
| 45 | R.PDSCH.1-16.4 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.2 |
| 50 | R.PDSCH.1-16.2 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.6 |

Table 5.2A.3.5.1.3.4-2: Test requirements for single carrier performance for FDD 15 kHz SCS for HST-DPS CA configurations with 2 active PDSCH TCI states

| Bandwidth (MHz) | Reference channel | Modulation format and code rate |  | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-15.1 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 11.1 |
| 10 | R.PDSCH.1-8.4 FDD | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 11.4 |
| 15 | R.PDSCH.1-15.2 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 11.3 |
| 20 | R.PDSCH.1-15.3 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 11.1 |
| 25 | R.PDSCH.1-15.4 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 11.4 |
| 30 | R.PDSCH.1-15.5 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 11.5 |
| 35 | R.PDSCH.1-16.3 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 11.2 |
| 40 | R.PDSCH.1-16.1 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 11.3 |
| 45 | R.PDSCH.1-16.4 |  | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.2 |
| 50 | R.PDSCH.1-16.2 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 11.6 |

Table 5.2A.3.5.1.3.4-3: Test requirements for single carrier performance for TDD 30 kHz SCS for HST-DPS CA configurations with 1 active PDSCH TCI state

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-22.1 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.0 |
| 10 | R.PDSCH.2-22.2 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.1 |
| 15 | R.PDSCH.2-22.3 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.1 |
| 20 | R.PDSCH.2-22.4 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.1 |
| 25 | R.PDSCH.2-22.5 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.2 |
| 30 | R.PDSCH.2-23.1 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.1 |
| 40 | R.PDSCH.2-10.5 TDD | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.1 |
| 50 | R.PDSCH.2-23.2 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.3 |
| 60 | R.PDSCH.2-23.3 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.3 |
| 80 | R.PDSCH.2-23.4 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.1 |
| 90 | R.PDSCH.2-23.5 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.3 |
| 100 | R.PDSCH.2-24.1 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.3 |

Table 5.2A.3.5.1.3.4-4: Test requirements for single carrier performance for TDD 30 kHz SCS for HST-DPS CA configurations with 2 active PDSCH TCI states

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-22.1 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.0 |
| 10 | R.PDSCH.2-22.2 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.1 |
| 15 | R.PDSCH.2-22.3 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.1 |
| 20 | R.PDSCH.2-22.4 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.1 |
| 25 | R.PDSCH.2-22.5 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.2 |
| 30 | R.PDSCH.2-23.1 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.1 |
| 40 | R.PDSCH.2-10.5 TDD | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.1 |
| 50 | R.PDSCH.2-23.2 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.3 |
| 60 | R.PDSCH.2-23.3 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.3 |
| 80 | R.PDSCH.2-23.4 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.1 |
| 90 | R.PDSCH.2-23.5 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.3 |
| 100 | R.PDSCH.2-24.1 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 11.3 |

<< END OF CHANGES >>