3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230077

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2915 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EN-DC RRC test case 8.2.2.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TTCN implementation of Step 13 doesn’t Release/Restablish the DRB-2 in the SS Configuration. This will lead to SS sending the DRB Data at step 15 with incremented RLC SN, which is incorrect, since RLC is expected to Restablish at step 13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added call to f_EUTRA_SS_Release_SingleDRB (eutra_Cell1, tsc_DRB2); at step 13 to release and Reconfig DRB-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody () |
| --- | --- |
| Reason for change | TTCN implementation of Step 13 doesn’t Release/Restablish the DRB-2 in the SS Configuration. This will lead to SS sending the DRB Data at step 15 with incremented RLC SN, which is incorrect, since RLC is expected to Restablish at step 13. |
| Summary of change | Added call to f_EUTRA_SS_Release_SingleDRB (eutra_Cell1, tsc_DRB2); at step 13 to release and Reconfig DRB-2. |
| TTCN module | RRC_Reconfig_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { //@sic R5-198928 sic@ ... // @siclog "Steps 13 - 14" siclog@ // SN terminated MCG DRB with security key s-KgNB to MN terminated MCG DRB with security key change to keNB v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 sic@ // NR PDCP is on NR PTC, therefore DRB2 must be split here to configure the proxy. MCG so lower layers on EUTRA v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; // @sic R5s201477 sic@ //@sic R5s200171 change 4.2 sic@ ... }

After Change

function fl_TC_8_2_2_8_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { //@sic R5-198928 sic@ ... // @siclog "Steps 13 - 14" siclog@ // SN terminated MCG DRB with security key s-KgNB to MN terminated MCG DRB with security key change to keNB v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, tsc_EUTRA_DelayBeforeIntraCellHO); f_EUTRA_SS_Release_SingleDRB (eutra_Cell1, tsc_DRB2); //New Change v_ConfigDrbSS := {cs_OneDRB_ConfigAM(tsc_DRB8)}; //@sic R5s200171 change 4.2 sic@ // NR PDCP is on NR PTC, therefore DRB2 must be split here to configure the proxy. MCG so lower layers on EUTRA v_SrbDRBList := {cs_SRB1_ConfigDef, cs_SRB2_ConfigDef, cs_RadioBearer_Reconfig_SplitDRB2(nr_Cell1), cs_OneDRB_ConfigAM(tsc_DRB8)}; // @sic R5s201477 sic@ //@sic R5s200171 change 4.2 sic@ ... }