3GPP TSG RAN Meeting #99    	            RP-230305

Rotterdam, Netherlands, March 20-23, 2023

Source:	Qualcomm Incorporated

Title:	Support of BWP without restriction

Agenda Item:	9.12

Document for: 	Discussion and Decision

# Background

At the RAN#97 meeting, RAN asked RAN4 to do a high-level analysis of options for BWP without restriction. RAN has received a LS from RAN4 regarding support of BWP without restriction [1], which includes comprehensive high-level analyses for the following options (other options were deprioritized in RAN4). The analyses are copied in the Annex of this contribution from the LS for convenience.

- Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP - Option B) Perform BM/RLM/BFD based on SSB outside active BWP - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions - Option B-2) BM/RLM/BFD on SSB outside BWP within measurement gaps - Option B-2-2) Dedicated MG or NCSG for RLM/BFD/BM measurements - Option C) NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

At the RAN#98e meeting, it was observed that 18 companies supported Option B-1-1 and Option C, while 5 companies support Option B-1-1, Option B-2-2, and Option C. The RAN#98e conclusion was the following:

RAN#98e conclusion: RAN4 shall not discuss this further and RAN will come back at RAN #99

In this contribution, we provide our views on how RAN plenary should proceed with this issue.

# Discussion

The high-level analyses from RAN4 indicated that different options have different impacts on specification/workload, mobility performance, throughput, and UE power consumption/complexity. Nevertheless, it is very important to make sure that BWP without restriction using SSB for BM/RLM/BFD is supported. We propose RAN to task WGs to specify Option(s) listed by RAN4.

According to the RAN#98e discussion [2], it turned out that there were diverging views on Option B-1-1, Option B-2-2, and Option C. All these options have technical rationales with different pros/cons and hence it was very difficult to converge on a single option. Although our first preference is Option B-1-1, we are ok to move forward with any combination of options as long as it includes at least Option B-1-1.

Proposal:

- Task the relevant Working Groups (RAN1, 2, 4) to specify BWP without restriction using SSB for BM/RLM/BFD.

# Conclusion

In this contribution, we provided our views on BWP without restriction.  We propose to support BWP without restriction using SSB for BM/RLM/BFD using Option B-1-1, Option B-2-2, and Option C.

Proposal:

- Task the relevant Working Groups (RAN1, 2, 4) to specify BWP without restriction using SSB for BM/RLM/BFD.

# Reference

- RP-222725, LS on BWP operation without restriction (R4-2220437; to: RAN; cc: RAN2, RAN1; contact: vivo)

- 3GPP RAN#98e Chairman’s note.

# Annex: High-level analyses of options agreed in RAN4

Table 1: High-level analysis of candidate options on RRM requirements impact/workload in RAN4

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | - CSI-RS based RLM/BFD/BM requirements are already specified. - Further study is needed to decide on whether timing requirements may need to be updated | Low |
| Option B-1-1) | - Existing SSB based RLM/BFD/BM measurement requirements will apply. - The applicability rule of existing requirements is to be updated. - Depending on RF BW and BB BW (FFT BW) assumption, at least intra-frequency measurement with gap need to be revisited | Low |
| Option B-1-2) | - Same as Option B-1-1). Additionally, - Interruption requirements need to be developed additionally to allow UE for switching. | Low or Medium |
| Option B-2-2) | - Existing requirements need to be updated for SSB based RLM/BFD/BM with dedicated NCSG. - Existing requirements need to be updated for SSB based RLM/BFD/BM with dedicated MG. | NCSG: Low or Medium MG: Medium |
| Option C) | - Existing requirements for SSB based RLM/BFD/BM can be re-used. - Clarification on the requirement applicability might be needed. | Low |

Note: “Low”/ “Low to Medium”/ “Medium” correspond to the low/low to medium/medium RRM requirements impact.

Table 2: High-level analysis of candidate options on UE power consumption / UE complexity

| Options | Technical analysis | Summary |  |
| --- | --- | --- | --- |
|  |  | Power consumption | UE complexity |
| Option A) | - UE works in active BWP. - No additional implementation complexity is needed | Low | Low |
| Option B-1-1) | - Depending on RF BW and BB BW (FFT BW) assumption, UE can consume much higher power than other options - If BB BW is assumed adapted to BWP BW, it should be FFS how interruption can be avoided and its impact on UE complexity | High | Low to medium |
| Option B-1-2) | - UE works in larger RF BW than active BWP periodically and UE may consume larger power than other options except Option B-1-1. | Medium | Low to Medium |
| Option B-2-2) | - UE works in active BWP. | Low | Medium |
| Option C) | - UE works in active BWP | Low | Low |

Note: “Low”/ “Low to Medium”/ “Medium” correspond to the low/low to medium/medium negative impact on UE power consumption and UE complexity comparing to the legacy solution.

Table 3: High-level analysis of candidate options on Mobility performance impact

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | - Measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements with gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option B-1-1) | - No measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements without gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement - Can also help L3 intra-frequency measurement | Low |
| Option B-1-2) | - No measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements without gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option B-2-2) | - Measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements with gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option C) | - Measurement gap may or may not be needed for intra-frequency L3 measurement depending on whether or not NCD-SSBs from all neighbor cells are present within UE active BWP. - Measurement gap is needed for inter-frequency L3 measurement. | Low |

Note: “Low” - means a limited impact on mobility performance comparing to the legacy solution.

Table 4: High-level analysis of candidate options on Throughput impact (Data interruption)

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |
| Option B-1-1) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |
| Option B-1-2) | Interruption is needed for BM/RLM/BFD measurements | Low to Medium |
| Option B-2-2) | NCSG or MG is needed for BM/RLM/BFD measurements | NCSG: Low to Medium MG: Medium |
| Option C) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |

Note 1: Throughput impact due to L3 measurements is not considered.

Note 2: “Low”/ “Medium” correspond to the low/medium negative impact on throughput comparing to the legacy solution.