3GPP RAN TSG Meeting #99		RP-230342

Rotterdam, NL, March 20 - 23, 2023

Agenda item:	9.2.2

Source: 	Qualcomm Incorporated

Title: 	TP for TR 38.893: Conclusions of SI

Document for:	Approval

# 1	Introduction

This document provides a text proposal for chapter 7 of TR 38.893 with conclusions of the study item.

<<< Start of TP for TR 38.893 >>>

# 7	Conclusions and recommendations

During the course of this study item, using Band n77 for US and Canada as an example, root causes were identified to necessitate a 3GPP solution to differentiate UE’s supporting one or more regional subsets within a larger global band.  Expected behavior of UE’s certified for only one or multiple regional subsets of the band in their home country when roaming to other countries was also clarified.  The case of an inbound roaming UE not certified for additional regional requirements in a subset of the band is bound by each country’s laws on whether it is allowed to operate within the country.

With the root causes identified, two solutions and variations were then considered.  The first solution is to define a new band or a new band number to be able to distinguish the UE capability as well as a means of access control by the network.  Considerations to simplify specifications and signalling of the new band number by leveraging those of the larger band.  A second solution is to reuse the existing larger band and band number, but to introduce new signalling in the form or band subsets to more closely couple the subset to the original superset band.

The key difference between defining a new band or band number compared to an approach of reusing the existing global band or band number is the need to introduce new signalling.  In general, it is discouraged to introduce new signalling unless there is a demonstrated need.  Moreover, since the problem of band subsets may not be rare, but is also not expected to be very commonplace, the introduction of new signalling is probably not justified.  In that sense, the solutions with a new band (number) are preferable so long as the issues identified previously are addressed.  These include

- The new band number is formally identified as a subset of an existing larger band,

- The UE is required to fully support the existing larger band as a prerequisite to supporting the new subset,

- Band combinations defined for the larger band automatically apply to the subset without the need for further specification work unless exceptions are identified,

- Band combinations signaled by the UE for the larger band implicitly apply to the subset without additional signaling needed,

- UE out-of-band blocking for the subset refers to FDL_low and FDL_high of the larger band since it is not expected that the UE will implement a sub-band filter for the subset band.

<<< End of TP  >>>