3GPP TSG RAN meeting #99									       RP-230349

Rotterdam, Netherlands, March 20-23, 2023

## Status Report to TSG

Agenda item:			9.2.9

| WI / SI Name | Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No | Testing part: No |
| Acronym | FS_NR_FR2_OTA_enh |  |  |  |
| Unique ID | 950068 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-223536 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2023 | Core part: - | Performance part: - | Testing part: - |
| Overall Completion level | Study Item: 40% | Core part: - | Performance Part: - | Testing part: - |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Bin Han |
|  | Company | Qualcomm Incorporated |
|  | Email | binhan@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### Void

## 2.2	RAN2

#### Void

## 2.3	RAN3

#### Void

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#106:

Baseline method for UE RF testing

- Agreements:

  - IFF based multi-AoA test system to be considered as baseline for the UE RF testing.

  - DFF based multi-AoA test system is not precluded.

Maximum AoA angular separation for UE RF testing

- Agreements:

  - Consider 150 º as the maximum AoA angular separation as the baseline unless the UE RF session requires 180º separation to be tested and the testability of 180º is concluded in testing enhancements SI.

AoAs angular separations for UE RF testing

- Agreements:

  - Consider the set of relative angular AoA separations S as {30°, 60°, 90°, 120° and 150°} as the baseline unless the UE RF session requires 180º separation to be tested and the testability of 180º is concluded in testing enhancements SI.

  - FFS whether only a subset of angular relationships can be defined for RF testing.

  - If the UE declaration approach is adopted, UE should declare AoA separations within the set S

Additional sources (probes) location

- Agreements:

  - For an example positioner implemented as a roll motor (azimuth positioner) on a turn-table (elevation positioner), so called combined axes positioner, place the measurement probes in the plane containing the legacy source and perpendicular to the turn-table axis (elevation). For conventional OTA system nomenclature where the elevation axis corresponds to the y axis and the legacy source is in the z direction, place the probes in the xz axis

Turn-table and Roll motion

- Agreements:

  - Apply a full rotation in  and apply a half rotation in 

Grids for 3D scan

- Agreements:

  - Use constant step size grid

Blocking by Probes

- Agreements:

  - Consider the repositioning concept with the proposed test system setup for multi-AoA RX testing to reduce the effect of blocking with the set of relative angular AoA separations S of {30°, 60°, 90°, 120° and 150°}

  - From MU perspective, the baseline is UE repositioning is applied.

  - Encourage companies to analyze the MU for the cases with or without UE repositioning.

Comment on the directionality of the AoA1-AoA2 DL orientation vectors, e.g., whether reciprocity of the TRPs will tolerate the observed directionality

- Agreements:

  - Both +/- offset with same value need to be validated for complete verification of UE performance (Need to consider the directionality of the AoA1-AoA2 DL)

  - Complementary grid is feasible from testability PoV

Polarization combinations

- Agreements:

  - Limit the polarization combinations for the multi-AoA

Additional test function

- Agreement:

  - For DL part, no need to define the additional beam lock test function.

Pass/Fail Test Procedure for Multi-DCI & Single-DCI Schemes

- Agreement:

  - Option 1 in [1] considered as candidate for further discussion and which also need to be aligned with RF session conclusion.

Measurement setup for RRM testing

- Agreement:

  - Reuse the same test system design of UE RF testing for UE RRM testing as the starting point.

Measurement setup for demodulation testing

- Agreement:

  - Reuse the same test system design of UE RF testing for UE demodulation testing as the starting point. The two AoAs are fixed during the testing

#### Remaining Open issues

Test methodology for RF requirements

- Whether only a subset of angular relationships from {30°, 60°, 90°, 120°, 150°} can be defined for RF testing.

- Define the test procedure.

- How to limit the polarization combinations for the multi-AoA.

Test methodology for RRM requirements

- How to control the SINR for multi-DCI scheme including non-overlapping and overlapping cases.

- The number of probes which is depending on the RRM session progress.

- Test parameters and side condition

Test methodology for Demodulation requirements

- How to select the test directions

- Test parameters and side condition

Uncertainty assessments

- Develop the related preliminary uncertainty assessments for the test methodologies

## 2.5	RAN5

#### Void

## 2.6	RAN6

#### Void

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A.

## 4.	References

- R4-2302923, WF for FR2 OTA test enhancement SI, Qualcomm Incorporated

- R4-2300086, TP on TR 38.871 for editorial correction and general aspects, Qualcomm Incorporated

- R4-2302924, TP to introduce Multi-AoA UE RF Test Aspects, Keysight Technologies UK Ltd

- R4-2300087, On test method for FR2 multi-Rx UE, Qualcomm Incorporated

- R4-2300353, On multi-Rx DL test methodology, Apple

- R4-2300763, Enhancement of UE beamlock test function command for multi-TRP connection, Anritsu Corporation

- R4-2300988, Discussion on maximum AoA angular separation for Multi-RX UE RF testing, Samsung

- R4-2301458, Consideration on AoA angular separation for UE RF testing, OPPO

- R4-2301564, Discussion on FR2 OTA test methods for multi-Rx, vivo

- R4-2302149, Discussion on Test methods, Huawei,HiSilicon

- R4-2302513, Discussion on FR2 methods for UEs with multi-Rx, ROHDE & SCHWARZ

- R4-2302520, Testing Considerations for Multi AoA Rx Testing, Keysight Technologies UK Ltd

- R4-2302150, TP on TR 38.871 calibration measurement procedure, Huawei,HiSilicon

- R4-2302148, Discussion on Measurement uncertainty, Huawei,HiSilicon

- R4-2303776, Topic Topic summary for [106][331] FS_NR_FR2_OTA_enh, Qualcomm

- R4-2302880, Ad-hoc minutes for FR2 OTA Test, Qualcomm