3GPP TSG RAN meeting #99                                                                    RP-230350
Rotterdam, Netherlands, March 20-23, 2023

Agenda item:	9.2.9

Source:	Qualcomm Incorporated

Title:	Views on the scope for NR FR2 OTA testing enhancements

Document for:	Approval

# Introduction

In RAN#98e, the revised SID on study on NR FR2 OTA testing enhancements was approved in [1]. As scheduled in [1], there is a checking point to revisit whether to include transmitting simultaneously in the scope of this SI in RAN#99.

The objectives for FR2-1 OTA testing for UEs with multi-panel reception and 4DL layer are as follows. - Define a test methodology for RF/RRM/Demodulation requirements testing for devices that can receive simultaneously from multiple Angle of Arrival (AoA) - The multiple AoA test setup should enable testing of up to 2 DL Layers with dual polarization for each angle - For RRM, the target should be to allow testing of 4 AoAs with 2 simultaneously active AoAs. - Define a test methodology for up to 4 DL MIMO layer demodulation testing - Note: Revisit whether or not to include the case of transmitting simultaneously in RAN#99 - Smartphone form factor should be the first priority, other UE types should also be discussed as 2nd priority - Develop the related preliminary uncertainty assessments for the test methodologies - FR2 test methods defined in TR 38.810 and TR 38.884 should be used as the baseline. - The tests shall take the test system reuse, test system complexity and test time into account to keep the whole test costs within a reasonable level.

In this paper, we provide our views on the scope for FR2 OTA testing enhancements

# Discussion

In RAN#98e, the pros and cons on whether to consider the case of transmitting simultaneously, i.e., multi-Tx, in the scope of this SI were discussed. The proponents believe that specifying the test methodology enabling multi-Tx besides multi-Rx can avoid the increase in testing cost since multi-Tx has already been included in the WID of Rel-18 NR MIMO Evo [2]. While the main concern from opponents is Rel-18 multi-Tx discussion is still at the early stage, the study on the test methodology without a clear definition on the core requirements for multi-Tx might lead to some uncertainty and high workload.

According to the TU budget of [2], RAN4 has already started the discussion on NR MIMO evolution for downlink and uplink. The WF [3] was approved with the following agreements related to multi-Tx:

<Way forward>: Proposed UE RF agenda from next meeting, RAN4#106-bis-e - General and work plan - UE RF on simultaneous transmission with multi-panel (STxMP) - Per-panel power limitation - Per-UE power limitation … <Agreement> - Current defined power classes shall be considered further as reference for any power limitation discussions while defining the new requirements for STxMP case, if needed. …

From test system perspective, it is recommended to consider the forward compatibility when studying the test method for multi-Rx. It is not realistic that the test system for multi-Rx could not be used for multi-Tx testing.

Observation 1: RAN4 has already started to discuss the requirements for multi-Tx and hence the test method should be considered in the SI.

RAN4 made good progress on the UE RF test method for multi-Rx in 2023Q1. Therefore, we expect it would not bring much additional workload to study feasibility of reusing the test system of multi-Rx for multi-Tx testing. To solve the concerns of extending the scope of SI, we could take the study on the test method of multi-Tx as the 2nd priority.

Proposal 1: Study the test method for multi-Tx in this SI as the 2nd priority.

In addition, RAN4 has got the consensus on using multi-Rx chain reception as the terminology when discussing and specifying the UE RF requirements. With that, we propose to revise the wording from “multi-Rx panel” to “multi-Rx chain” to align with RF requirements discussion (The revised SID is in [4]).

Proposal 2: Revise the wording from “multi-Rx panel” to “multi-Rx chain” to align with RF requirements discussion. The revised SID is in [4].

# Conclusion

In this paper, we provide our views on the scope for FR2 OTA testing enhancements. The following proposals are made:

Proposal 1: Study the test method for multi-Tx in this SI as the 2nd priority.

Proposal 2: Revise the wording from “multi-Rx panel” to “multi-Rx chain” to align with RF requirements discussion. The revised SID is in [4].

# References

- RP-223536, “Revised SID for Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements”, Qualcomm Incorporated, 3GPP RAN#98e

- RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, Samsung

- R4-2303495, “WF on NR MIMO evolution for downlink and uplink”, Samsung

- RP-230351, “Revised SID: Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements”, Qualcomm Incorporated