3GPP TSG RAN Meeting #99		RP-230409

Rotterdam, Netherlands, March 20th – 23rd, 2023

Agenda item:	9.4.5.6

Source:	Apple

Title:	Scoping of PC1.5 inter-band UL CA/DC with 2Tx and 3Tx

WI/SI:	4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

NR inter-band EN-DC and UL CA have been introduced since Rel-15. However, up till Rel-17, the HPUE feature for inter-band EN-DC and UL CA has only been specified up to PC2, despite PC1.5 has already been introduced for single UL TDD band since Rel-16. In RAN #98-e meeting, a new work item with an objective to introduce inter-band UL CA/DC with simultaneous 3Tx feature was approved [1]. The intended UL configurations include 1Tx PC3 in one band and 2Tx PC1.5 in the other band. Since this UL configuration already supports PC1.5 in one of the constituent bands, introducing PC1.5 for inter-band UL CA/DC would be inevitable as there is no good reason to constrain the power class for such band combinations to PC2 or PC3. As PC1.5 for inter-band UL CA/DC is a new UE feature where certain general requirements may need to be specified, also the simultaneous 3Tx is considered as a more advanced feature than 2Tx, it seems necessary to first introduce PC1.5 with PC2 1Tx in one band and PC2 1Tx in the other band in order to set the foundation for PC1.5 with 3Tx, or at least, PC1.5 with 2Tx needs to be developed in parallel with PC1.5 with 3Tx. However, PC1.5 for inter-band UL CA/DC with 2Tx has not been scoped in any of the existing work items. As a result, whether PC1.5 for inter-band UL CA/DC with 3Tx feature can be directly introduced before 2Tx would need further discussions. In this contribution, we share our views on the scoping of PC1.5 inter-band UL CA/DC feature with 2Tx and 3Tx and provide a few options on feature development procedure for consideration.

# Discussion

The objective on introducing inter-band UL CA/DC with simultaneous 3Tx feature in a new work item [1] approved in RAN #98-e meeting has called for the development of new PC1.5 feature for inter-band UL CA/DC in order to support the intended UL configuration which is composed of 1Tx PC3 in one band and 2Tx PC1.5 in the other band. Since PC1.5 for inter-band UL CA/DC is a new UE feature, certain general requirement may need to be specified. On the other hand, the simultaneous 3Tx operation is considered as a more advanced feature than 2Tx, therefore, it seems necessary to first introduce PC1.5 with PC2 1Tx in one band and PC2 1Tx in the other band in order to set the foundation for PC1.5 with 3Tx, or at least, PC1.5 with 2Tx needs to be developed in parallel with PC1.5 with 3Tx [2].

Observation 1: For PC1.5 inter-band UL CA/DC, it seems necessary to introduce PC1.5 with PC2 1Tx in one band and PC2 1Tx in the other band in order to set the foundation for PC1.5 with 3Tx, or at least, PC1.5 with 2Tx needs to be developed in parallel with PC1.5 with 3Tx.

However, PC1.5 for inter-band UL CA/DC with 2Tx has not been scoped in any of the existing work items. As a result, whether PC1.5 for inter-band UL CA/DC with 3Tx feature can be directly introduced before 2Tx would need further discussions.

If RAN/RAN4 decide it is necessary to introduce PC1.5 for inter-band UL CA/DC with 2Tx before 3Tx, the following options on feature development procedure can be considered:

Proposal 1: RAN/RAN4 to consider the following options on the feature development procedure for PC1.5 inter-band UL CA/DC with 2Tx and 3Tx:

Option 1: Revise the objectives of the current WID for “Low NR band 4Rx for handheld UE and 3Tx for inter-band UL CA and EN-DC” to include PC1.5 inter-band UL CA/DC with 2Tx configuration.

Option 2: Instigate a new RAN4 work item to specifically handle the PC1.5 inter-band UL CA/DC with 2Tx feature.

Option 3: Postpone the PC1.5 for inter-band UL CA/DC feature (including both 2Tx and 3Tx) to Rel-19.

In our view, the easiest path to introduce PC1.5 inter-band UL CA/DC is to start with PC2+PC2 in TDD+TDD band combinations, such as PC1.5 for UL CA_n41A-n77A. As the maximum total output power for PC2+PC2 is exactly PC1.5 where each UL can independently control its output power without the concern of exceeding the combined PCMAX, the rather complicated UL power sharing would not be needed. On the other hand, for TDD+TDD combination under simultaneous 2Tx operation, there would not be DL reception in both bands, therefore, MSD issue does not need to be considered.

If RAN/RAN4 would decide to introduce PC1.5 for inter-band UL CA/DC with 2Tx in Rel-18 timeframe, from RAN4 workload concern perspective, we think the work scope should be limited to (PC2 TDD + PC2 TDD) configuration only.

Proposal 2: If PC1.5 inter-band UL CA/DC with 2Tx would be introduced in Rel-18 timeframe, the work scope should be limited to (PC2 TDD + PC2 TDD) configuration only.

One further consideration for this WID is the “increasing UE power high limit” feature [3] for the inter-band UL CA/DC with 3Tx objective which was brought up during the WID discussions in last plenary meeting. Though the decision was to descope the “increasing UE power high limit” feature for this WID as some companies suggested that it can be handled in the Rel-18 coverage enhancement WID [4]. In our view, if the “increasing UE power high limit” feature would be revived for the intended inter-band UL CA/DC with 3Tx configurations, it is better to jointly develop the two features in the same WID, especially for the PC1.5 3Tx configurations.

Proposal 3: The “increasing UE power high limit” feature for the intended inter-band UL CA/DC with 3Tx configurations should be jointly developed in the same WID instead of being handled in separate WIDs.

# 3	Conclusion

In this contribution, we share our views on the scoping of PC1.5 inter-band UL CA/DC feature with 2Tx and 3Tx and provide a few options on feature development procedure for consideration.

Proposal 1: RAN/RAN4 to consider the following options on the feature development procedure for PC1.5 inter-band UL CA/DC with 2Tx and 3Tx:

Option 1: Revise the objectives of the current WID for “Low NR band 4Rx for handheld UE and 3Tx for inter-band UL CA and EN-DC” to include PC1.5 inter-band UL CA/DC with 2Tx configuration.

Option 2: Instigate a new RAN4 work item to specifically handle the PC1.5 inter-band UL CA/DC with 2Tx feature.

Option 3: Postpone the PC1.5 for inter-band UL CA/DC feature (including both 2Tx and 3Tx) to Rel-19.

Proposal 2: If PC1.5 inter-band UL CA/DC with 2Tx would be introduced in Rel-18 timeframe, the work scope should be limited to (PC2 TDD + PC2 TDD) configuration only.

Proposal 3: The “increasing UE power high limit” feature for the intended inter-band UL CA/DC with 3Tx configurations should be jointly developed in the same WID instead of being handled in separate WIDs.

# 4	References

RP-223517 “Low NR band 4Rx for handheld UE and 3Tx for inter-band UL CA and EN-DC”, OPPO et. al., 3GPP TSG-RAN Meeting #98-e, Online, December 12th – 16th, 2022

R4-2300361 “PC1.5 for inter-band UL CA/DC and MSD framework”, Apple, 3GPP RAN WG4 Meeting #106, Athens, Greece, February 27th – March 3rd, 2023

RP-212163 “New WID: Increasing UE power high limit for CA and DC”, China Telecom, Qualcomm, 3GPP TSG-RAN Meeting #93-e, September 13th – 17th, 2021

RP-221858 “Revised WID on Further NR coverage enhancements”, China Telecom, 3GPP TSG-RAN Meeting #96, June 6th – 9th, 2022