3GPP TSG-RAN Meeting #99		RP-230410

Rotterdam, Netherlands, March 20-23, 2023

Agenda item:	9.3.2.9

Source:	Apple

Title:	Motivation for Further Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

The recent achievement by RAN4 in defining Rel-17 TRP and TRS requirements for handheld UEs [1], [2] and the steady progress in the working group on Rel-18 TRP and TRS enhancements [3], [4] offer a unique opportunity for 3GPP RAN to resolve the currently ongoing discussion in relation to the potentially new XR UE device type [5].  Based on the proposed way forward (not yet approved) in [6] and the related discussion [7], it appears that one of the key concerns raised by operators is the network performance impact due to potential coverage reduction of XR devices which support 2 Rx antennas instead of 4 (in bands where 4 Rx are required).  There exists the potential to leverage OTA requirements for XR devices to address operator concerns and, coupled with the confidence we have in RAN4’s ability to conclude OTA requirements, this contribution provides the motivation to include XR scope into the Rel-19 further enhancements of TRP TRS work scope.

Considering the outcome of the RAN4 discussion to prioritize the performance phase of the Rel-18 TRP/TRS work item [8] and our own proposals to rescope it [9], and the progress of the Rel-18 work in general will help to scope the rest of the Rel-19 TRP/TRS work, as RAN Plenary discusses the scope of the Rel-19 package in the coming meetings.

# 2	Discussion

The proposed way forward on handling the new XR device type, discussed in the RAN #98 meeting, was as follows [6]:

- Proposed Way Forward: For 3GPP Rel-18 XR Work Item, include the following objective - Specify mechanism to support new device type (XR) that is only mandated to support 2Rx antenna ports in the NR bands in which 4Rx antenna ports is mandated in legacy [RAN4, RAN2]

The discussion of this proposal was captured in the RAN #98 report [7]:

RP-223208 Way forward on the support of 2Rx for XR device for Rel-18 XR WI Type: discussion For: Decision Source: Apple, ABS, APT, ASR, CATT, FGI, Google, HONOR, Lenovo, Meta, New H3C, Qualcomm, Samsung, Tencent, Verizon, vivo, Xiaomi Discussion: handled in discussion [98e-07-R18-XR]; Apple: KT Corp., Microsoft and Spark New Zealand are supporting this Tdoc RP-223208 handled in Mon GTW Proposed Way Forward: For 3GPP Rel-18 XR Work Item, include the following objective Specify mechanism to support new device type (XR) that is only mandated to support 2Rx antenna ports in the NR bands in which 4Rx antenna ports is mandated in legacy [RAN4, RAN2] Qualcomm: we support this, topic is controversial, decision should come from TSG RAN avoiding to spend more time in WGs Huawei: overall scope of the WI needs to be discussed before going in details, no all form factors have limitations; at first we should define which devices have problems with 4Rx CMCC: 2RX with 100MHz on smart phone will reduce network performance, defining a new redcap device may be considered Vodafone: we do not support 2Rx without further explanation, glasses have enough space for 4Rx so it is not about form factor China Unicom: same view as Vodafone Telecom Italia: strong concern about the proposal and would not like to see redcap used for this DT: similar view as other operators and start of an XR WI shall be shifted Orange: XR needs high capacity and going for 2RX is contradicting so too early to agree on anything there AT&T: we are fine to extend the SI by 1 quarter but scope shall not increase; form factors and use cases need to be clarified before abandoning 4Rx can be discussed Verizon: we can discuss the form factor; we were accomodating 1RX redcap so we are open for discussions T-Mobile: no industrial design criteria is given; difference between XR and redcap is that redcap is not supposed to be data heavy; reducing perf. criteria for XR is not appropriate without more detailed explanations/discussions Decision: The document was noted.

Observation 1:	Operators voiced concerns with the coverage of a 2Rx XR device potentially impacting network performance.

Observation 2:	In field operation, it is the radiated performance of devices that impacts network RF planning and performance.

Given Observations 1 and 2, the 2Rx XR discussion provides a clear motivation for 3GPP to define radiated requirements for XR devices and to study whether it is feasible to unify the OTA requirements for all XR devices, regardless of whether such devices support 2Rx or 4Rx.

Proposal 1:	To resolve operator concerns with the coverage of 2Rx XR devices, 3GPP should introduce OTA requirements for XR devices and study whether it is feasible to unify the OTA requirements for all XR devices, regardless of whether such devices support 2Rx or 4Rx.

With industry-wide recognition of the importance of OTA requirements in terms of conformance of devices to common radiated limits, device certification, and network planning, it is useful to review the status of the concluded Rel-17 OTA requirements, the scope of Rel-18 work (based on our proposal in [9]), and to project the potential scope of Rel-19 enhancements.  Table 1 below captures this in a color-coded format:  completed Rel-17 test methods and requirements in green, proposed rescoping of Rel-18 work in orange, and proposed scope of Rel-19 work in red.  This table is provided for information just to visualize how the XR related objectives fit into the potential OTA work scope overall.

Table 1: Status of TRP/TRS objectives by release

| Aspect / Feature |  | TRS |  |  |  |  |  |  | 1Tx TRP PC3 |  |  |  |  |  |  |  | 1Tx TRP PC2 |  |  |  | 2Tx Rad Power |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Browse | Talk |  | RedCap |  | XR |  | Browse |  | Talk |  | RedCap |  | XR |  | Browse |  | Talk |  | Browse |  | Talk |  |
| Test method and prelim MU |  | R17 | R17 |  | R18 |  | R19 |  | R17 |  | R17 |  | R18 |  | R19 |  | R17 |  | R17 |  | R18 |  | R18 |  |
| n1 | 72 < w ≤ 92 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n3 | 72 < w ≤ 92 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n5 | 72 < w ≤ 92 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 72 < w ≤ 92 mm | R19 |  | R19 |  |  |  |  |  | R19 |  | R19 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n8 | 72 < w ≤ 92 mm | R19 |  | R19 |  |  |  |  |  | R19 |  | R19 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 72 < w ≤ 92 mm | R18 |  | R18 |  |  |  |  |  | R18 |  | R18 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n41 | 72 < w ≤ 92 mm | R17 |  | R18 |  | R19 |  | R19 |  | R18 |  | R18 |  | R19 |  | R19 |  | R17 |  | R18 |  | R18 2p |  | R18 2p |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 72 < w ≤ 92 mm | R18 |  | R18 |  | R19 |  | R19 |  | R18 |  | R18 |  | R19 |  | R19 |  | R18 |  | R18 |  | R18 2p |  | R18 2p |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n78 | 72 < w ≤ 92 mm | R17 |  | R18 |  | R19 |  | R19 |  | R18 |  | R18 |  | R19 |  | R19 |  | R17 |  | R18 |  | R18 2p |  | R18 2p |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n79 | 72 < w ≤ 92 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The potential OTA work can be classified into three categories:  XR related aspects (test method and requirements), more bands for handheld TRP/TRS (no test methodology impact), and the first set of bands for RedCap devices.

Observation 3:	The general direction for Rel-19 TRP/TRS work should be to define test method and requirements for XR devices, improve band coverage for handheld TRP/TRS, and introduce the first set of TRP/TRS requirements for RedCap devices.

Considerations of the Rel-18 work item progress and band prioritization need to be taken as RAN Plenary considers the scope of further OTA work during the Rel-19 package discussions.

# Conclusions

In this contribution we have proposed an approach to resolve operator concerns associated with the 2Rx relaxation for the new XR device type via OTA requirements and described how this work can fit into the overall OTA scope of work in RAN4 in Rel-19.  The following observations and proposals have been made:

Observation 1:	Operators voiced concerns with the coverage of a 2Rx XR device potentially impacting network performance.

Observation 2:	In field operation, it is the radiated performance of devices that impacts network RF planning and performance.

Observation 3:	The general direction for Rel-19 TRP/TRS work should be to define test method and requirements for XR devices, improve band coverage for handheld TRP/TRS, and introduce the first set of TRP/TRS requirements for RedCap devices.

Proposal 1:	To resolve operator concerns with the coverage of 2Rx XR devices, 3GPP should introduce OTA requirements for XR devices and study whether it is feasible to unify the OTA requirements for all XR devices, regardless of whether such devices support 2Rx or 4Rx.

# 4	References

- RP-211158, "Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)," vivo, OPPO, CMCC, 3GPP RAN #92, June 2021

- RP-222152, "Status report for WI: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC); rapporteur: vivo," 3GPP RAN #97, September 2022

- RP-222669, “New WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies,” vivo, 3GPP RAN #97, September 2022

- RP-223112, “Revised WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC),” vivo, 3GPP RAN #98, December 2022

- RP-223502, “New WID on XR Enhancements for NR,” Nokia, Qualcomm, 3GPP RAN# 98, December 2022

- RP-223208, “Way forward on the support of 2Rx for XR device for Rel-18 XR WI,” Apple, ABS, APT, ASR, CATT, FGI, Google, HONOR, Lenovo, Meta, New H3C, Qualcomm, Samsung, Tencent, Verizon, vivo, Xiaomi, KT Corp., Microsoft and Spark New Zealand, 3GPP RAN #98, December 2022

- RP-230052, “Meeting Report for electronic meeting TSG RAN meeting #98-e,” 3GPP RAN

- R4-2302920, “Working procedure for Rel-18 TRP TRS requirements,” vivo, 3GPP RAN4 #106, February 2023

- RP-230158, “On rescoping the Rel-18 TRP TRS work item,” Apple, 3GPP RAN #99, March 2023