3GPP TSG-RAN4 Meeting #106	R4-2300358

# Athens, Greece, Feb. 27th – Mar. 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5910 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 36.101: P-Max definition correction for Band 14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2023-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In E-UTRA RAN4 specifications, the P-Max definition has been specified differently between Band 41 and other frequency bands since Rel-15. In RAN4 #104-e meeting, it was clarified that Band 14 should follow the same P-Max definition as with Band 41 rather than with other frequency bands, meaning that for Band 14, if P-Max is not indicated by the network, the UE would apply the maximum power according to its capability as specified in TS 36.101, clause 6.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Revise the context in clause 6.2.2, 6.2.3, 6.2.3D, 6.6.2.3 to align the Band 14 P-Max definition with Band 41. - Add a paragraph in clause 6.2.5 to clarify ΔPPowerClass relation with P-max for Band 14. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The P-Max definition for Band 14 is incorrectly specified and Band 14 HPUE cannot be realized since the network does not indicate P-Max. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.2.3, 6.2.3D, 6.2.5, 6.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

### 6.2.2	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non CA configuration unless otherwise stated. The period of measurement shall be at least as defined in Table 6.2.2-0.

Table 6.2.2-0: Measurement period for UE maximum output power

| TTI pattern | Minimum measurement period |
| --- | --- |
| Subframe | 1ms |
| Slot | 7OS |
| Subslot | 2OS, 3OS |

Table 6.2.2-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2 |  |  |
| 2 |  |  |  |  | 23 | ±22 |  |  |
| 3 | 31 | +2/-3 |  |  | 23 | ±22 |  |  |
| 4 |  |  |  |  | 23 | ±2 |  |  |
| 5 |  |  |  |  | 23 | ±2 |  |  |
| 6 |  |  |  |  | 23 | ±2 |  |  |
| 7 |  |  |  |  | 23 | ±22 |  |  |
| 8 |  |  |  |  | 23 | ±22 |  |  |
| 9 |  |  |  |  | 23 | ±2 |  |  |
| 10 |  |  |  |  | 23 | ±2 |  |  |
| 11 |  |  |  |  | 23 | ±2 |  |  |
| 12 |  |  |  |  | 23 | ±22 |  |  |
| 13 |  |  |  |  | 23 | ±2 |  |  |
| 14 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| 17 |  |  |  |  | 23 | ±2 |  |  |
| 18 |  |  |  |  | 23 | ±25 |  |  |
| 19 |  |  |  |  | 23 | ±2 |  |  |
| 20 | 31 | +2/-3 |  |  | 23 | ±22 |  |  |
| 21 |  |  |  |  | 23 | ±2 |  |  |
| 22 |  |  |  |  | 23 | +2/-3.52 |  |  |
| 23 |  |  |  |  | 236 | ±26 |  |  |
| 24 |  |  |  |  | 23 | +2/-32 |  |  |
| 25 |  |  |  |  | 23 | ±22 |  |  |
| 26 |  |  |  |  | 23 | ±22 |  |  |
| 27 |  |  |  |  | 23 | ±2 |  |  |
| 28 | 31 | +2/-3 |  |  | 23 | +2/-2.5 |  |  |
| 30 |  |  |  |  | 23 | ±2 |  |  |
| 31 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | ±2 |  |  |
| 34 |  |  |  |  | 23 | ±2 |  |  |
| 35 |  |  |  |  | 23 | ±2 |  |  |
| 36 |  |  |  |  | 23 | ±2 |  |  |
| 37 |  |  |  |  | 23 | ±2 |  |  |
| 38 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 39 |  |  |  |  | 23 | ±2 |  |  |
| 40 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 41 |  |  | 26 | ±22 | 23 | ±22 |  |  |
| 42 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| 43 |  |  |  |  | 23 | +2/-3 |  |  |
| 44 |  |  |  |  | 23 | +2/[-3] |  |  |
| 45 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 47 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 48 |  |  |  |  | 23 | +2/-3 |  |  |
| 50 |  |  |  |  | 23 | ±2 |  |  |
| 51 |  |  |  |  | 23 | ±2 |  |  |
| 52 |  |  |  |  | 23 | +2/-3 |  |  |
| 53 |  |  |  |  | 23 | ±2 |  |  |
| 65 |  |  |  |  | 23 | ±2 |  |  |
| 66 |  |  |  |  | 23 | ±2 |  |  |
| 68 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | ±2 |  |  |
| 71 |  |  |  |  | 23 | +2/-2.5 |  |  |
| 72 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| 73 |  |  |  |  | 23 | ±2 |  |  |
| 74 |  |  |  |  | 23 | ±2 |  |  |
| 85 |  |  |  |  | 23 | ±22 |  |  |
| 87 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| 88 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. NOTE 4: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. NOTE 6: When NS_20 is signalled, the total output power within 2000-2005 MHz shall be limited to 7 dBm. NOTE 7: Void. NOTE 8: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 9: Void. |  |  |  |  |  |  |  |  |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in TS 36.331 [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [7] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5;

<<< Unchanged sections omitted >>>

### 6.2.3	UE maximum output power for modulation / channel bandwidth

For UE Power Class 1, 2 and 3, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2-1due to higher order modulation and transmit bandwidth configuration (resource blocks) is specified in Table 6.2.3-1.

Table 6.2.3-1: Maximum Power Reduction (MPR) for Power Class 1, 2 and 3

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 1 |
| 16 QAM | ≤ 5 | ≤ 4 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 2 |
| 64 QAM | ≤ 5 | ≤ 4 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 2 |
| 64 QAM | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 3 |
| 256 QAM | ≥ 1 |  |  |  |  |  | ≤ 5 |

For PRACH, PUCCH and SRS transmissions, the allowed MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For each TTI pattern, the MPR shall be evaluated per Teval period as specified in table 6.2.3-2 and given by the maximum value taken over the transmission(s) within that period; the maximum MPR over TREF is then applied for TREF.

Table 6.2.3-2: MPR evaluation period

| TTI pattern | TREF | Teval |
| --- | --- | --- |
| Subframe | 1 subframe | 1 slot |
| Slot | 7 OS | Min(Tno_hopping, 7OS) |
| Sublot | 2 OS, 3OS | Min(Tno_hopping, 2OS/3OS) |

For UE Power Class 1 and 3 transmissions with non-contiguous resource allocation in single component carrier, the allowed Maximum Power Reduction (MPR) for the maximum output power in table 6.2.2-1, is specified as follows

MPR = CEIL {MA, 0.5}

Where MA is defined as follows for QPSK, 16 QAM and 64 QAM

MA =	8.00-10.12A		; 0.00< A ≤ 0.33

5.67 - 3.07A		; 0.33< A ≤0.77

3.31				; 0.77< A ≤1.00

Where MA is defined as follows for 256 QAM

MA = 8.00-10.12A		; 0.00< A ≤ 0.25

5.50				; 0.25< A < 1.00

Where

A = NRB_alloc / NRB.

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB, i.e. MPR  [3.0, 3.5 4.0 4.5 5.0 5.5 6.0 6.5 7.0 7.5 8.0]

The allowed MPR for transmission on an Scell in Band 46 or Band 49 within a component carrier of a nominal channel bandwidth of 10 MHz or 20 MHz is in accordance with 6.2.3-1 for RIV = ‘11111’ (10 MHz) and L = 10 (20 MHz) with L defined in Clause 8.1.4 of [6]. For all other possible values of the RIV defined in Clause 8.1.4 of [6] the allowed MPR is 2.5 dB for QPSK modulation, 3 dB for 16QAM modulation and 4 dB for 64QAM modulation (256QAM is FFS).

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [7] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5.

For UE Power Class 2 transmissions with non-contiguous resource allocation in single component carrier, the allowed Maximum Power Reduction (MPR) for the maximum output power is not specified in this version of the specification.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.5 apply.

<<< Unchanged sections omitted >>>

### 6.2.3D	UE maximum output power for modulation / channel bandwidth for ProSe

When UE is configured for E-UTRA ProSe sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA ProSe operating bands specified in Table 5.5D-1, this subclause specifies the allowed Maximum Power Reduction (MPR) power for ProSe physical channels and signals due to higher order modulation and transmit bandwidth configuration (resource blocks).

The allowed MPR for the maximum output power for ProSe physical channels PSDCH, PSCCH, PSSCH, and PSBCH shall be as specified in subclause 6.2.3 for  PUSCH for the corresponding modulation and transmission bandwidth.

The allowed MPR for the maximum output power for ProSe physical signal PSSS shall be as be as specified in subclause 6.2.3 for PUSCH QPSK modulation for the corresponding transmission bandwidth.

The allowed MPR for the maximum output power for ProSe physical signal SSSS is specified in Table 6.2.3D-1.

For a power class 2 capable UE operating on Band 41, the corresponding requirements for a power class 3 UE apply when an IE P-max as defined in [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [7] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5.

Table 6.2.3D-1: Maximum Power Reduction (MPR) for SSSS for Power Class 1, 2 and 3

| Channel bandwidth | MPR for SSSS (dB) |
| --- | --- |
| 1.4 MHz |  |
| 3.0 MHz |  |
| 5.0 MHz | ≤ 4 |
| 10 MHz | ≤ 4 |
| 15 MHz | ≤ 4 |
| 20 MHz | ≤ 4 |

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the requirements in subclause 6.2.3D apply for ProSe transmission and the requirements in subclause 6.2.3 apply for uplink transmission.

<<< Unchanged sections omitted >>>

### 6.2.5	Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,c for serving cell c. The configured maximum output power PCMAX,c is set within the following bounds:

PCMAX_L,c ≤  PCMAX,c  ≤  PCMAX_H,c with

PCMAX_L,c = MIN {PEMAX,c – TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc + ΔTIB,c + TC,c + TProSe, P-MPRc)}

PCMAX_H,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass}

where

-	PEMAX,c is the value given by IE P-Max for serving cell c, defined in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2.2-1 without taking into account the tolerance specified in the Table 6.2.2-1;

-	ΔPPowerClass = 3 dB for a power class 2 capable UE operating in Band 41, when P-max of 23 dBm or lower is indicated or if the uplink/downlink configuration is 0 or 6 in the cell; otherwise, ΔPPowerClass = 0 dB

-	ΔPPowerClass = PPowerClass – PPowerClass_Default dB for UE operating in Band 14, when P-max of 23 dBm or lower is indicated in the cell; otherwise, ΔPPowerClass = 0 dB.

-	MPRc and A-MPRc for serving cell c are specified in subclause 6.2.3 and subclause 6.2.4, respectively;

-	TIB,c is the additional tolerance for serving cell c as specified in Table 6.2.5-2; TIB,c = 0 dB otherwise;

-	TC,c = 1.5 dB when NOTE 2 in Table 6.2.2-1 applies;

-	TC,c = 0 dB when NOTE 2 in Table 6.2.2-1 does not apply;

-	TProSe = 0.1 dB when the UE supports ProSe Direct Discovery and/or ProSe Direct Communication on the corresponding E-UTRA ProSe band; TProSe = 0 dB otherwise.

-	For a power class higher than default UE power class capable UE except for operating in Band 14 and Band 41, ΔPPowerClass = PPowerClass – PPowerClass_Default dB, when the band is a TDD band whose frame configuration is 0 or 6; or P-max is not indicated in the cell; or P-Max is provided and set to the maximum output power of the default power class or lower, otherwise, ΔPPowerClass = 0 dB.

P-MPRc is the allowed maximum output power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPR c for serving cell c only for the above cases. For UE conducted conformance testing P-MPR shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2: P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.5-0 for different TTI patterns. For each TREF, the PCMAX_L,c for serving cell c is evaluated perTeval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_ L,c over the one or more Teval is then applied for the entire TREF. PPowerClass shall not be exceeded by the UE during any period of time.

<<< Unchanged sections omitted >>>

#### 6.6.2.3	Adjacent Channel Leakage Ratio

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. ACLR requirements for one E-UTRA carrier are specified for two scenarios for an adjacent E-UTRA and /or UTRA channel as shown in Figure 6.6.2.3-1.

Figure 6.6.2.3-1: Adjacent Channel Leakage requirements for one E-UTRA carrier

##### 6.6.2.3.1	Minimum requirement E-UTRA

E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned E-UTRA channel power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3.1-1, Table 6.6.2.3.1-2, and Table 6.6.2.3.1-3. If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the value specified in Table 6.6.2.3.1-1, Table 6.6.2.3.1-2, and Table 6.6.2.3.1-3.

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable,  and the corresponding requirements for a power class 3 UE shall apply.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [7] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5.

Table 6.6.2.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

Table 6.6.2.3.1-2: Additional E-UTRAACLR requirements for Power Class 1

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 37 dB | 37 dB | 37 dB | 37 dB | 37 dB | 37 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |
| NOTE 1: E-UTRAACLR1 shall be applicable for >23dBm |  |  |  |  |  |  |

Table 6.6.2.3.1-3: Additional E-UTRAACLR requirements for Power Class 2

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | N/A | N/A | 31 dB | 31 dB | 31 dB | 31 dB |
| E-UTRA channel Measurement bandwidth | N/A | N/A | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | N/A | N/A | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

<<< End of changed sections >>>