3GPP TSG-RAN4 Meeting #106	R4-2300504

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.852 | CR | 0001 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TR 38.852 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Union Inter. Chemins de Fer |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RAIL_EU_1900MHz_TDD |  |  |  |  |  | Date: |  |  | 23-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 38.852 Table 7.1.2.2.2-1: Additional Tx spurious emissions requirement derivation for n101 the spurious emission limit is incorrectly based on - 43 - GantRMR1900 = -61. This should be corrected to - 43 - GmaxRMR1900 = -57 dBm using GmaxRMR1900 = 14dBi as per the Note in that table. This makes it in line with the TS 38.104 Table 6.6.5.2.3-12 V17.8.0 (2022-12) Also in section 7.1.2.2.2 correct reference to band n100 to n101 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In TR 38.852 Table 7.1.2.2.2-1 correct - 43 - GantRMR1900 = -61 dBm to - 43 - GmaxRMR1900 = -57 dBm In 7.1.2.2.2 correct band n100 to n101 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential confusion between the TR 38.852 and the TS 38.104 |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial version |  |  |  |  |  |  |  |  |

# <Start of Change>

7.1.2.2.2	Tx spurious emissions

Additional Tx spurious emission requirements are considered for band n101 in 3GPP TS 38.104 [4], based on conversion of the EIRP limits from ECC Decision (20)02 [1], as summarized in table 7.1.2.2.2-1. The requirement is derived based on table 4.3.1-5, with the assumption of maximum gain value of the BS GmaxRMR1900:

Table 7.1.2.2.2-1: Additional Tx spurious emissions requirement derivation for n101

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 1920 MHz – 1980 MHz | - 43 - GmaxRMR1900 = -57 dBm (NOTE) | 5 MHz |
| NOTE: Assuming a 14dBi maximum gain (antenna gain and losses) for GmaxRMR1900. |  |  |

# <End of Change>