3GPP RAN4 Meeting #106	R4-2303683

# Athens, Greece,  27 Feb – 3 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1469 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-1, Correction of minor errors in suffix E (NR V2X/Sidelink) requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2023-03-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some editorial errors have been detected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5.4E.2.3: Duplicate sentence removed and reference corrected 6.2E.1.2: Extra number 4 removed from power tolerance ±24 >> ±2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors that degrade the quality of the specitication and increase risk for misunderstanding remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4E.2.3, 6.2E.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | - |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | - |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------- < Start of changes 1> ----------

5.4E.2	Channel raster

5.4E.2.1	NR-ARFCN and channel raster

For NR V2X, the NR-ARFCN and channel raster requirements in clause 5.4.2.1 apply for each operating band.

For NR V2X UE, the reference frequency can be shifted by configuration.

FREF_V2X = FREF + Δshift + N * 5 kHz

where

Δshift = 0 kHz or 7.5 kHz indicated in IE (frequencyShift7p5khz), and

N can be set as one of following values {-1, 0, 1}, which are signalled by the network in higher layer parameters or configured by pre-configuration parameters.

5.4E.2.2	Channel raster to resource element mapping

For NR V2X, the channel raster to resource element mapping requirements in clause 5.4.2.2 apply for each operating band.

5.4E.2.3	Channel raster entries for each operating band

For NR V2X, the channel raster entries requirements in clause 5.4.2.3 apply for each operating band.

The RF channel positions on the channel raster in each NR V2X operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

For NR V2X operating band n47, ΔFRaster = I × ΔFGlobal, where I ϵ {1}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <I>.

5.4E.3	Synchronization raster for V2X

There is no synchronization raster definition for NR V2X for both licensed bands and unlicensed bands.

---------- < End of changes 1 > ----------

---------- < Start of changes 2 > ----------

6.2E.1.2	UE maximum output power for V2X con-current operation

For the inter-band NR V2X con-current operation, the maximum output power is specified in Table 6.2E.1.2-1 for each operating band. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1.2-1: Power Class for NR V2X inter-band con-current combination (two bands)

| NR V2X con-current operating band Configuration | NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n1A-n47A | n1 |  |  |  |  | 23 | ±2 |  |  |
|  | n47 |  |  |  |  | 23 | +2/-3 |  |  |
| V2X_n5A-n47A | n5 |  |  |  |  | 23 | ±2 |  |  |
|  | n47 |  |  |  |  | 23 | +2/-3 |  |  |
| V2X_n8A-n47A | n8 |  |  |  |  | 23 | ±2 |  |  |
|  | n47 |  |  |  |  | 23 | +2/-3 |  |  |
| V2X_n39A-n47A | n39 |  |  |  |  | 23 | +2/-3 |  |  |
|  | n47 |  |  |  |  | 23 | +2/-3 |  |  |
| V2X_n40A-n47A | n40 |  |  |  |  | 23 | +2/-3 |  |  |
|  | n47 |  |  |  |  | 23 | +2/-3 |  |  |
| V2X_n41A-n47A | n41 |  |  |  |  | 23 | +2/-3 |  |  |
|  | n47 |  |  |  |  | 23 | +2/-3 |  |  |
| V2X_n71A-n47A | n71 |  |  |  |  | 23 | +2/-34 |  |  |
|  | n47 |  |  |  |  | 23 | +2/-3 |  |  |
| V2X_n78A-n47A | n78 |  |  |  |  | 23 | +2/-3 |  |  |
|  | n47 |  |  |  |  | 23 | +2/-3 |  |  |
| V2X_n79A-n47A | n79 |  |  |  |  | 23 | +2/-3 |  |  |
|  | n47 |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: For the con-current band combinations, the simultaneous transmission and reception of sidelink and Uu interfaces can be supported while operation is agnostic of the service used on each interface. NOTE 2: PPowerClass is the maximum output power specified without taking into account the tolerance for each operating band. NOTE 3: For inter-band con-current operation, the aggregation power apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |  |

For the intra-band con-current NR V2X operation, the maximum output power is specified in Table 6.2E.1.2-2. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1.2-2: NR V2X UE Power Class for intra-band con-current combination

| NR V2X con-current operating band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n79B |  |  | 26 | +2/-32 | 23 | +2/-32 |  |  |
| NOTE 1: Void. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For intra-band con-current aggregation the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

---------- < End of changes 2 > ----------