3GPP TSG RAN meeting #99									RP-230577

Rotterdam, March 20-23, 2023

Agenda Item:	7

Source:	Ericsson

Title:	Discussions on timing resiliency and URLLC enhancements in RAN

Document for:	Discussion, Decision

# 1	Introduction

SA2 has concluded the SI on FS_TRS_URLLC which studies solutions for using the 5G system (5GS) for time distribution. The work has triggered normative stage 2 work under the TRS_URLLC work item (see SP-221341), that is currently ongoing and will continue until the summer. The overarching objective of this work is to enable the 5GS to act as a backup for GNSS based time distribution.

At RAN#99 proposals to start the corresponding work in 3GPP RAN will be discussed. In this paper we present our view on the NG-RAN impacts to serve as a time distribution network. In particular we outline the requirements on NG-RAN for providing an accurate end-to-end (e2e) time distribution, which is key for 3GPP to be able to deliver a solution that is commercially attractive and scalable.

# 2	Discussion

# 2.1	KI#1	5GS network timing synchronization status and reporting

## 2.1.1	Time distribution to devices in RRC Connected mode

The SA2 work has been focused on various key issues (KI) related to the enabling of time distribution. For KI#1 that concerns 5GS network timing synchronization status and reporting we note that SA2 sent an LS to RAN3 in [2] and RAN3 replied in [3] at the last RAN3#119 meeting.

Part of the solution outlined by SA2 is that RRC Inactive and RRC Idle UEs are receiving the time synchronization status change via SIB 9 broadcast signaling. But to receive the accurate time status, the UE needs to enter RRC Connected mode for which time information is sent by the gNB to the UE using unicast signaling. In RRC Connected state it is possible for the network to compensate for over the air propagation delay and direct the UE to cells that offer the best possible time accuracy. Therefore, we believe that a RAN WI on time distribution should be focused on time distribution to UEs in RRC Connected state. RAN2 is expected to lead this work.

Observation 1:	Accurate time distribution demands propagation delay compensation and NW control of the UE mobility, which are functions only available for RRC Connected UEs.

Proposal 1:	Potential RAN work on 5GS time distribution should be focused on time distribution to UEs in RRC Connected state (RAN2)

## 2.1.2	UE capability reporting

In the SA2 outlined solution, only the NW side time synchronization status is taken into consideration. The UE internal time accuracy, which is one essential component in the end-to-end time delivery chain appears to have been overlooked. For the 5GS to understand the end-to-end time budget splits this needs to be addressed by TSG RAN. We therefore propose that RAN should include among its objectives the specification of UE capability signaling of the UE internal time distribution accuracy. This proposal aligns with methods for distributing time in wired packet-based networks e.g., through Precision Time Protocol (PTP), that includes defined classes indicating a nodes capability for distributing time.

Observation 2:	The UE internal time distribution accuracy is an important component in the 5GS end-to-end time distribution accuracy.

A UE capability indication of its internal time accuracy between the UE receiving antenna port and related application boundary would allow proper selection and mapping of UE classes towards certain required service levels. For a certain end to end service level a certain UE capability will be required. It will allow comparison and proper selection of devices used in client network for balanced 5GS budget, which is a pre-requisite for cost and spectrum efficient implementation of time as a service (TaaS).

Earlier RAN work for time Propagation Delay Compensation (PDC) assumed 50-100ns time (in)accuracy between UE antenna and its application boundary. Such UE accuracies were required for 5GS to provide an end-to-end time accuracy in the range of ~1us.

Stricter end to end accuracies and 5GS synchronicity budgets smaller than 1 us could be expected for future applications and flexible support for that will be needed.

Some services would not require 1 us as assumed in RAN PDC work. TS 22.104 includes services with 5GS synchronicity budgets of 10, 20 and 50 us (and even 100 us sometimes mentioned sufficient) i.e., far lower accuracy than 50-100 ns would be sufficient for those services.

A UE capability indication will also be needed to differentiate between UE implementations, this to avoid pushing tight timing requirements and associated cost/complexity to regular UEs or allow flexibility towards UEs that does not need to support strictest 5GS synchronicity budget service levels. This emphasizes the importance of the mentioned UE capability. We therefore propose that RAN should include among its objectives the specification of a UE capability related to the UE internal time accuracy. Again, we believe that RAN2 is most suitable to lead this work.

Observation 3:	The 5GS time distribution accuracy is dependent on the targeted use case. UE capability signaling of the UE internal time distribution accuracy is vital to facilitate end-to-end service level agreements on 5GS time distribution accuracy.

Proposal 2:	Potential RAN work on 5GS time distribution should include an objective for defining UE capability signaling that enables a UE to indicate its internal time distribution accuracy (RAN2)

The discussed UE time accuracy should reflect the ability of a device to over time maintain an internal accurate and stable clock. Observability of the performance associated with such a capability is obviously important. We believe a simple yet attractive method to achieve such observability is to complement the signaling that sets the time in a UE with signaling that allows the NW or a RAN5 test function to ask the UE to expose its internal time over an external interface that is accessible by the NW or by a test function. This would support observability of the UE internal time and by comparing the UE internal time with a true external clock an observer would be able to determine the UE clock accuracy and compare it with the signaled UE capability.

## 2.2	KI#5	Interworking with TSN network deployed in the transport network

The outlined solution is that gNB RAN and UPF may support the functionality of Listener/Talker following:

-	such as parameters in a Transparent Container;

-	NG-RAN and UPF may support u-plane LLDP functionality.

It is unclear related to how NG-RAN supports the u-plane LLDP functionality. In our understanding, AN-TL (Access Network Talk Listener) is out of the 3GPP scope, and how the gNB interact with the AN-TL is by implementation.

Proposal 3: RAN to discuss and clarify that the impact in RAN3 is to support the set and get containers related to AN-TL. gNB supporting of u-plane LLDP functionality is outside 3GPP in the WID.

# 3	Conclusion

In this paper we have outlined the requirements on NG-RAN to support end-to-end time distribution with high accuracy. We observed that

Observation 1:	Accurate time distribution demands propagation delay compensation and NW control of the UE mobility, which are functions only available for RRC Connected UEs.

Observation 2:	The UE internal time distribution accuracy is an important component in the 5GS end-to-end time distribution accuracy.

Observation 3:	The 5GS time distribution accuracy is dependent on the targeted use case. UE capability signaling of the UE internal time distribution accuracy is vital to facilitate end-to-end service level agreements on 5GS time distribution accuracy.

Based on these observations we presented the following proposals:

Proposal 1:	Potential RAN work on 5GS time distribution should be focused on time distribution to UEs in RRC Connected state (RAN2)

Proposal 2:	Potential RAN work on 5GS time distribution should include an objective for defining UE capability signaling that enables a UE to indicate its internal time distribution accuracy (RAN2)

Finally, with regards to interworking with TSN network deployed in the transport network we proposed that:

Proposal 3: RAN to discuss and clarify that the impact in RAN3 is to support the set and get containers related to AN-TL. gNB supporting of u-plane LLDP functionality is outside 3GPP in the WID.

# References

S2-2301421, Support for network timing synchronization status and reporting KI1 - description, (SA2)

S2-2301463, Proposed method for Time Synchronization status reporting to UE(s), (SA2)

R3-230811, Reply LS on proposed method for Time Synchronization status reporting to UE(s), (RAN3)