3GPP WG RAN3 Meeting #119	R3-230135

# Athens, Greece, WG RAN3, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.821 | CR | 0002 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to abbreviations, figures and table header |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_NTN_solutions |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. VSAT abbreviation used in various places without being defined 2. Fig. 4.1-1 and fig. 4.1-2 can not be edited and source file was not provided (TR 21.801 H.5) 3. Table 7.2.1.1.1.2-5 header is corrupted |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. VSAT abbreviation is added in clause 3.3 2. Fig. 4.1-1 and fig. 4.1-2: bitmaps replaced by editable Visio figures 3. 7.2.1.1.1.2-5 header corrected and aligned with pCR R2-1914069 of RAN2 #107bis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description remains ambiguous creating misunderstanding issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.1, 7.2.1.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TR 38.821 was created by REL-16 SI FS_NR_NTN_solutions and exists therefore only in REL-16, so no cat.A CR(s) needed. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ECEF	Earth-Centered, Earth-Fixed

EIRP	Equivalent Isotropic Radiated Power

FAR	False Alarm Rate

FRF	Frequency Reuse Factor

FSS	Fixed Satellite Services

GEO	Geostationary Earth Orbiting

gNB	next Generation Node B

GW	Gateway

HAPS	High Altitude Platform Station

HEO	Highly Elliptical Orbiting

ISL	Inter-Satellite Links

LEO	Low Earth Orbiting

Mbps	Mega bit per second

MEO	Medium Earth Orbiting

MS	Mobile Services

MSS	Mobile Satellite Services

NAS-MM	Non-Access Stratum Mobility Management

NAS-SM	Non-Access Stratum Session Management

NGEO	Non-Geostationary Earth Orbiting

NTN	Non-Terrestrial Network

RAN	Radio Access Network

RTD	Round Trip Delay

SNR	Signal-to-Noise Ratio

SRI	Satellite Radio Interface

TLE	Two-Line Element

Rx	Receiver

UAS	Unmanned Aircraft System

UE	User Equipment

VSAT	Very Small Aperture Terminal

Next Modified Subclause

## 4.1	Non-Terrestrial Networks overview

A non-terrestrial network refers to a network, or segment of networks using RF resources on board a satellite (or UAS platform).

The typical scenario of a non-terrestrial network providing access to user equipment is depicted below:

Figure 4.1-1: Non-terrestrial network typical scenario based on transparent payload

Figure 4.1-2: Non-terrestrial network typical scenario based on regenerative payload

Next Modified Subclause

###### 7.2.1.1.1.2	4-step RACH enhancements for Non-Terrestrial Networks

…

Enhancement to timing advance

Problem Statement

Timing Advance (TA) is used to adjust the uplink frame timing relative to the downlink frame timing. As shown in Figure 7.2.1.1.1.2-4 (b), the DL and UL timing is aligned at gNB with timing advance. The timing advance is twice the value of the propagation delay. Different UEs usually have different timing advance.

Figure 7.2.1.1.1.2-4: Timing alignment at gNB side

The timing advance is derived from the UL received timing and sent by the gNB to the UE. UE uses the timing advance to advance/delay its timings of transmissions to the gNB so as to compensate for propagation delay and thus time align the transmissions from different UEs with the receiver window of the gNB. There are two possible ways for gNB to provide timing advance to UE:

(1) Initial timing advance during random access procedure: gNB derives the timing advance by measuring the received random access preamble and sends the value to UE via the Timing Advance Command field in MAC RAR [75].

Figure 7.2.1.1.1.2-5: MAC RAR

In NR, Uplink frame number for transmission from the UE shall start  before the start of the corresponding downlink frame at the UE [TS 38.213].

Figure 7.2.1.1.1.2-6: Uplink-downlink timing relation

is provided in SIB1 with the following possible values:

n-TimingAdvanceOffset         ENUMERATED { n0, n25600, n39936 }        OPTIONAL, -- Need S

In case of random access response, a timing advance command,  for a TAG indicates  values by index values of  = 0, 1, 2, ... , 3846, where an amount of the time alignment for the TAG with SCS of kHz is .  is defined in [TS 38.211] and is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response.

, where Hz and.

The maximum timing advance in NR which can be compensated during initial access is calculated in the following Table.

Table 7.2.1.1.1.2-4: Maximum timing advance compensated during initial access for different SCS

|  | SCS = kHz | Maximum timing advance compensated during initial access |
| --- | --- | --- |
| 0 | 15 | 2ms |
| 1 | 30 | 1ms |
| 2 | 60 | 0.5ms |
| 3 | 120 | 0.27ms |
| 4 | 240 | 0.15ms |

(2) Timing advance refinement in RRC_CONNECTED: gNB derives the timing advance by measuring the UL transmission and refines the timing advance via the Timing Advance Command MAC CE.

Figure 7.2.1.1.1.2-7: Timing Advance Command MAC CE

The timing advance command, , for a TAG indicates adjustment of a current  value, , to the new  value, , by index values of  = 0, 1, 2,..., 63, where for a SCS of  kHz, [ TS 38.213].

The maximum timing advance which can be adjusted via Timing Advance Command is calculated in the following table:

Table 7.2.1.1.1.2-5: Maximum timing advance adjusted via Timing Advance Command

|  | SCS = kHz | Maximum timing advance compensated via Timing Advance Command |
| --- | --- | --- |
| 0 | 15 | 0.017ms |
| 1 | 30 | 0.008ms |
| 2 | 60 | 0.004ms |
| 3 | 120 | 0.002ms |
| 4 | 240 | 0.001ms |

As mentioned above, the timing advance is twice the propagation delay. In NTN, the maximum round trip delay is 541.46ms for GEO and 25.77ms for LEO. The timing advance in NR as calculated in Table 7.2.1.1.1.2-1 and Table 7.2.1.1.1.2-2 is far from sufficient. Solutions for both UE with and without GNSS-capabilities should be considered.

END of MODIFICATIONS