3GPP TSG RAN meeting #99	 RP-230633

Rotterdam, Netherlands, March 20-23, 2023

## Status Report to TSG

Agenda item: 10.2.1 IoT (Internet of Things) NTN (non-terrestrial network) enhancements

| WI / SI Name | IoT (Internet of Things) NTN (non-terrestrial network) enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: - |
| Acronym | IoT_NTN_enh |  |  |  |  |
| Unique ID | 941004 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-223519 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: - |  |
| Overall Completion level | Study Item: | Core part: Overall: 50% For information RAN1: 60% RAN2: 45% RAN3: 60% RAN4: 15% | Performance Part: 0% | Testing part: - |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Abhishek Roy |
|  | Company | MediaTek |
|  | Email | abhishek.roy@mediatek.com |
| Secondary WG |  | RAN2 |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

###### RAN1#112, February 27th – March 3rd, 2023, Athens, Greece

Agreements on “9.11.3 Disabling of HARQ feedback for IoT NTN”

Conclusion

For eMTC HD-FDD single TB scheduled by single DCI, UE is not expected to receive a DCI with “HARQ-ACK bundling flag” field set to 1 in case the corresponding HARQ process is configured with HARQ feedback disabled by RRC signaling.

Agreement

For a DL HARQ process with disabled HARQ feedback in eMTC, UE is not expected to receive another MPDCCH carrying a DCI scheduling a PDSCH for a given HARQ process or to receive another PDSCH without corresponding MPDCCH for the given HARQ process that starts at a BL/CE DL subframe until X=3 (ms) have passed after the end of the reception of the last PDSCH for that HARQ process.

Agreement

For HARQ feedback for eMTC SPS PDSCH, at least the following is supported: UE follows the per-process HARQ feedback enabled/disabled configuration for the associated HARQ process except for the first SPS PDSCH after activation

- for the first SPS PDSCH after activation,

  - Option 1: If HARQ feedback for SPS activation is additionally enabled, ACK/NACK is reported by UE for the first SPS PDSCH after activation regardless of network configuration of enabled/disabled for this HARQ process, and follow per-process HARQ feedback enabled/disabled configuration otherwise.

Conclusion

For DCI indicating SPS PDSCH release, HARQ-ACK report is performed as legacy in eMTC, regardless of HARQ feedback enabled/disabled configuration.

Agreement

For DCI-based overridden mechanism/indication in single TB scheduled by DCI, down select one of the following alternatives based on the criteria DCI overhead, PDCCH monitoring/power consumption, HARQ timer, impact on scheduling flexibility, UE implementation complexity

- Alternative 1: applies to both semi-statically HARQ enabled and disabled processes

- Alternative 2: only applied to semi-statically HARQ disabled processes

- Alternative 3: only applied to semi-statically HARQ enabled processes

Agreement

Confirm the following working assumption with the following update:

Working assumption

For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission:

- Support Option 1 in case only per-HARQ process bitmap signaling is configured

- Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured

- Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured

  - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes.

  - FFS #2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI

  - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration.

RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”.

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

Agreement

For DCI-based overridden/direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc

- Option 1: Indication by adding one field in DCI

- Option 2: Indication by reusing/reinterpreting existing field in DCI

Agreements on “9.11.4	Improved GNSS operations for IoT NTN”

Agreement

The following alternatives can be considered to inform eNB the success of GNSS measurement at UE side after GNSS measurement in RRC connected.

- Alt-1: The UE will report the new GNSS validity duration

- Alt-2: The reception of any UL transmission from the UE at eNB after the GNSS measurement

Agreement

On the length of GNSS measurement gap, which is aperiodically triggered by eNB, the gap duration should be equal to or larger than the latest UE reported GNSS position fix time duration.

FFS: whether the gap duration is configured by eNB, or the gap duration is equal to the latest reported GNSS position fix time duration.

Agreement

- On when the GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, RAN1 can down select one of the following alternatives:

- Alt 1: the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot

  - FFS: details of X, e.g. predefined value or configured value

- Alt 2: the start time should be based on the current GNSS validity duration with delay or without delay

Agreement

UE reports only one GNSS position fix time duration for GNSS measurement at least when moving to RRC connected state.

Agreement

At least for the case when frequency error is within frequency error requirements, study the mechanisms and conditions to allow UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration.

- FFS: with legacy closed loop time correction or enhanced closed loop time correction

- This mechanism is enabled/configured by eNB

- FFS: whether such mechanism will be specified depends on the outcome of this study

#### Remaining Open issues

Objective 1 (IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17):

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates.

-	Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption.

## 2.2	RAN2

#### 2.2.1	Agreements

###### RAN2#121, February 27th – March 3rd, 2023, Athens, Greece

Agreements on “8.6.2 Performance Enhancements”

Agreements:

- For UE to report GNSS position fix time duration for measurement during the initial access, at least the following Msg5 message can be used:

  - RRCConnectionSetupComplete, RRCConnectionSetupComplete-NB,

  - RRCConnectionResumeComplete, RRCConnectionResumeComplete-NB,

  - FFS for RRCreestablishmentComplete and RRCConnectionReconfigurationComplete.

  - FFS for Msg3

- FFS whether the UE can stay in RRC_CONNECTED state when current GNSS position becoming out-of-date if the UE has initiated a new measurement

- The value range {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity} introduced in R17 is reused for connected UE GNSS validation duration report, unless modified by RAN1.

- UE reports GNSS validity duration after GNSS measurement. FFS whether the UE reports every time or only if the validity duration changes. FFS if the duration is the remaining validity duration or the whole duration.

Agreements on “8.6.3 Mobility Enhancements”

Agreements:

- Location-based connected mode measurement initiation is supported in quasi-Earth-fixed cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for quasi-Earth-fixed cell. FFS on whether the R17 IEs are reused or not. FFS if the same mechanism can also be used in idle (like in NR-NTN)

- Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris). FFS if the same mechanism can also be used in idle (like in NR-NTN).

- We don’t introduce any new low mobility criterion for enhanced mobility in Rel-18.

Agreements on “8.6.4 Enhancements to discontinuous coverage”

Agreements:

- RAN2 can continue to check whether dedicated RRC signalling can be used for providing satellite information corresponding to discontinuous coverage.

- RAN2 will support enhancements in paging and eDRX, in alignment with the work in SA2 and CT1. FFS on the details

- RAN2 may consider enhancements for connected UE upon detecting discontinuous coverage (e.g., suspend RLM, RLF detection, and RRC re-establishment process)

- Companies supporting the store and forward approach can bring a proposal to the plenary for TEI18 or for updating the WID.

#### 2.2.2	Remaining Open issues

Objective 1 (IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17):

- Possible updates on RAN2 aspects for Disabling of HARQ feedback in NB-IoT NTN.

- Any further details on disabling HARQ feedback in DL for both NB-IoT and eMTC based NTN.

- Any further details on HARQ mode A and HARQ mode B for UL HARQ operation in IoT NTN.

- Further details on RAN2 aspects on GNSS enhancements.

Objective 2 (Mobility enhancements):

- Possible enhancements for neighbour cell measurements in connected mode for eMTC-based NTN.

- Any further details on enhancements for neighbour cell measurements in connected mode for NB-IoT NTN.

- Further remaining details on CHO enhancements for eMTC-based NTN.

Objective 3 (Further enhancement to discontinuous coverage):

- RAN2 aspects on possible details of enhancements in discontinuous coverage (considering SA2 Study).

## 2.2	RAN3

#### 2.3.1	Agreements

###### RAN3#119, February 27th – March 3rd, 2023, Athens

Agreements on “18.2 Support discontinuous coverage”

- There is no need to enhance the existing S1AP procedure to support the tracking area reported by RAN before AN release.

- The time-based CHO over X2 should be supported.

#### 2.3.2	Remaining Open issues

- Align with the progress of NR NTN, if any.

- The potential issues of discontinuous coverage captured by the final TR from SA2.

## 2.4	RAN4

#### 2.4.1	Agreements:

RAN4#106, Athens, Greece, 27th Feb. – 3rd March, 2023

Agreements on “2 Requirements which not applicable due to lack of neighbor cell ephemeris”

- Conclusion: Postpone discussion till the related open issues for NB-IoT/eMTC core & perf. requirements for NTN WI are resolved

Agreements on “3-1: Neighbour cell measurements in connected mode”

Agreement

- The mechanism for defining the neighbour cell measurements in CONNECTED mode for NB-IoT from Rel-17 WI can be reused as baseline for NB-IoT NTN while NTN specific impact needs investigation.

Agreements on “3-2: Neighbour cell measurements in connected mode and triggering before RLF, for quasi-earth fixed cell”

Agreement

- It is up to RAN2 to decide the mechanism of neighbour cell measurements and corresponding measurement triggering before RLF for IoT NTN

Agreements on “3-3: Neighbour cell measurements in connected mode and triggering before RLF, for earth-moving cell”

Agreement

- RAN4 to wait for more RAN2 progress on neighbour cell measurement triggered before losing coverage.

Agreements on “4 For eMTC, CHO requirements”

Agreement

- Define RRM requirements for time and [location based] CHO based on further RAN2 progress.

- Define RRM requirements for time and [location based] CHO for both normal and enhanced overage for eMTC.

Agreement on “5 GNSS re-acquisition”

Agreement

- RAN4 to postpone the discussions on GNSS re-acquisition until more progress has been reached on this objective in other WGs.

#### 2.4.2	Remaining Open issues:

Core Part

- Define UE RRM CHO requirements for eMTC

- Define UE RRM core requirements for the introduced mobility enhancement features, if identified.

Performance Part

The work on Performance part has not started yet.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

RAN1#112, February 27th – March 3rd, 2023

Submitted TDocs to AI 9.11.3

- R1-2301804	FLS#2 on disabling of HARQ feedback for IoT NTN	Moderator(Lenovo)

- R1-2300119	Discussion on disabling of HARQ feedback for IoT NTN	Huawei, HiSilicon

- R1-2300147	Considerations for Disablement of HARQ in IoT NTN	Lockheed Martin

- R1-2300237	Discussion on disabling of HARQ feedback for IoT NTN	Spreadtrum Communications

- R1-2300268	Discussion on disabling of HARQ feedback for IoT NTN	OPPO

- R1-2300555	Discussion on the HARQ operation for IoT NTN	xiaomi

- R1-2300660	Discussion on remaining issues of disabling of HARQ feedback for IoT NTN	CATT

- R1-2300706	Discussion on disabling of HARQ feedback for IoT-NTN	ZTE

- R1-2300825	Disabling of HARQ feedback for IoT NTN	NEC

- R1-2300890	Discussion on disabling of HARQ feedback for IoT-NTN	Sony

- R1-2300922	Disabling of HARQ feedback for IoT NTN	Lenovo

- R1-2301022	Discussion on disabling of HARQ feedback for IoT NTN	CMCC

- R1-2301057	Disabling of HARQ for IoT NTN	MediaTek Inc.

- R1-2301151	On disabling HARQ feedback for IoT NTN	Ericsson

- R1-2301158	Disabling of HARQ feedback for IoT NTN	InterDigital, Inc.

- R1-2301209	On disabling HARQ feedback for IOT-NTN	Mavenir

- R1-2301285	Disabling of HARQ feedback for IoT NTN	Samsung

- R1-2301367	Discussion on HARQ Feedback Disabling for IoT NTN	Apple

- R1-2301434	Disabling HARQ Feedback for IoT-NTN	Qualcomm Incorporated

- R1-2301549	Views on Disabling of HARQ feedback for IoT NTN	Sharp

- R1-2301566	Disabling of HARQ feedback for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

- R1-2301623	Disabling of HARQ feedback for IoT NTN	Nordic Semiconductor ASA

Submitted TDocs to AI 9.11.4

- R1-2302120	Feature lead summary#3 of AI 9.11.4 on improved GNSS operations	Moderator (MediaTek)

- R1-2300120	Discussion on improved GNSS operations for IoT NTN	Huawei, HiSilicon

- R1-2300238	Discussion on improved GNSS operations for IoT NTN	Spreadtrum Communications

- R1-2300269	Discussion on improved GNSS operations for IoT NTN	OPPO

- R1-2300556	Discussion on the improved GNSS operation for IoT NTN	xiaomi

- R1-2300661	Considerations on improved GNSS operations for IoT NTN	CATT

- R1-2300707	Discussion on improved GNSS operation for IoT-NTN	ZTE

- R1-2300766	On Improved GNSS Operations for IoT NTN	NEC

- R1-2300923	Improved GNSS operations for IoT NTN	Lenovo

- R1-2301023	Discussion on improved GNSS operations for IoT NTN	CMCC

- R1-2301058	Improved GNSS operations for IoT NTN	MediaTek Inc.

- R1-2301159	Improved GNSS operations for IoT NTN	InterDigital, Inc.

- R1-2301286	Improved GNSS operations for IoT NTN	Samsung

- R1-2301303	On improved GNSS operation in IoT NTN	Ericsson Limited

- R1-2301368	Discussion on improved GNSS operations for IoT NTN	Apple

- R1-2301435	Improved GNSS Operations for IoT-NTN	Qualcomm Incorporated

- R1-2301567	Enhancements for long connections in NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

- R1-2301624	Improved GNSS operations for IoT NTN	Nordic Semiconductor ASA

## 4.2	RAN2

RAN2#121, February 27th – March 3rd, 2023, Athens, Greece

Submitted TDocs to AI 8.6.1

- R2-2300273	List of RAN2 Agreements in IoT-NTN	MediaTek Inc.

Submitted TDocs to AI 8.6.2

- R2-2301880	R18 IoT NTN performance enhancement	Ericsson

Submitted TDocs to AI 8.6.2.1

- R2-2300161	Discussion on HARQ enhancement for IoT NTN	OPPO

- R2-2300203	Discussion on the HARQ handling in IoT NTN	CATT

- R2-2300262	On Disabling HARQ Feedback in IoT-NTN	MediaTek Inc.

- R2-2300579	Disabling HARQ feedback for IoT-NTN	Interdigital, Inc.

- R2-2300889	Enhancement for UL and DL HARQ processes	Qualcomm Incorporated

- R2-2301043	Discussion on HARQ enhancement	Xiaomi

- R2-2301046	LS on NPDCCH monitoring for HARQ mode B	Xiaomi

- R2-2301251	Discussion on the HARQ enhancement for IoT-NTN	CMCC

- R2-2301659	Discussion on Timing Advance Report MAC CE transmission in eMTC NTN	Nokia, Nokia Shanghai Bell, Huawei, HiSilicon

Submitted TDocs to AI 8.6.2.2

- R2-2300175	Discussion on GNSS operation in connected mode	OPPO

- R2-2300204	Discussion on GNSS operation in connected mode	CATT

- R2-2300263	Enhancements on GNSS operation	MediaTek Inc.

- R2-2300580	GNSS acquisition and reporting for IoT NTN	Interdigital, Inc.

- R2-2300739	Improved GNSS Operation	Apple

- R2-2300892	GNSS fix in RRC_CONNECTED	Qualcomm Incorporated

- R2-2300979	Considerations on long GNSS operation in CONNECTED state	Lenovo

- R2-2301041	Discussion on GNSS operation enhancement	Xiaomi

- R2-2301053	Further discussion on GNSS enhancements	ZTE Corporation, Sanechips

- R2-2301209	Discussion on the enhancement of GNSS operation	Huawei, Turkcell, HiSilicon

- R2-2301252	Discussion on the GNSS enhancement for IoT-NTN	CMCC

- R2-2301493	On improved GNSS operation for IoT NTN	Samsung Electronics Benelux BV

- R2-2301660	On GNSS operation enhancements for IoT NTN	Nokia, Nokia Shanghai Bell

Submitted TDocs to AI 8.6.3

- R2-2300925	Analysis on mobility enhancements for IoT-NTN	Nokia, Nokia Shanghai Bell

Submitted TDocs to AI 8.6.3.1

- R2-2300144	Enhancements for neighbour cell measurements	Huawei, HiSilicon, Turkcell

- R2-2300162	Discussion on measurement enhancement for IoT NTN	OPPO

- R2-2300205	Enhancements for Neighbor Cell Measurements	CATT

- R2-2300264	Enhancements on neighbour cell measurement	MediaTek Inc.

- R2-2300366	Discussion on neighbour cell measurements in IoT NTN	Intel Corporation

- R2-2300581	Neighbour cell measurements before RLF	Interdigital, Inc.

- R2-2300750	Neighbour cell measurements before RLF for NB-IoT	Apple

- R2-2300891	Neighbour satellite and coverage information signalling	Qualcomm Incorporated

- R2-2300980	CONNECTED neighbour cell measurement for NB-IoT in NTN	Lenovo

- R2-2301012	Enhancements for neighbor cell measurements	NEC

- R2-2301054	Further discussion on neighbor cell measurement	ZTE Corporation, Sanechips

- R2-2301187	Consideration on enhancements for the neighbour cell measurement	Xiaomi

- R2-2301253	Discussion on enhancements for neighbour cell measurements	CMCC

- R2-2301494	On enhancements for neighbour cell measurements	Samsung Electronics Benelux BV

- R2-2301602	Discussion on Enhancements for neighbour cell measurements	Transsion Holdings

- R2-2301624	Discussion on Enhancements for neighbour cell measurements	Transsion Holdings

- R2-2301693	Discussion on neighbour cell measurements before RLF	Ericsson

Submitted TDocs to AI 8.6.3.2

- R2-2300145	Discussion on CHO enhancements	Huawei, HiSilicon, Turkcell

- R2-2300163	Discussion on mobility enhancements for eMTC NTN	OPPO

- R2-2300265	On Mobility Enhancements in IoT-NTN	MediaTek Inc.

- R2-2300749	Mobility enhancement in IoT NTN	Apple

- R2-2300981	IDLE mobility for moving cells in IoT NTN	Lenovo

- R2-2301056	Further discussion on other mobility enhancements	ZTE Corporation, Sanechips

- R2-2301495	Other mobility enhancements	Samsung Electronics Benelux BV

- R2-2301871	Conditional Handover in IoT NTN	Ericsson

Submitted TDocs to AI 8.6.4

- R2-2300206	Discussion on enhancements to discontinuous coverage	CATT

- R2-2300266	On Enhancements to discontinuous coverage	MediaTek Inc.

- R2-2300501	Impact of the UE Unreachability Periods on UE AS	Google Inc.

- R2-2300582	IoT-NTN discontinuous coverage enhancements	Interdigital, Inc.

- R2-2300654	Discussion on power saving enhancements for supporting discontinuous coverage	Spreadtrum Communications

- R2-2300751	Support on discontinuous coverage in IoT NTN	Apple

- R2-2300878	Considerations on Supporting Discontinuous Coverage	NEC Europe Ltd

- R2-2300890	RRC release procedure in discontinuous coverage	Qualcomm Incorporated

- R2-2300926	On IoT-NTN enhancements for discontinuous coverage	Nokia, Nokia Shanghai Bell

- R2-2300982	On mobility and power saving issues for discontinuous coverage	Lenovo

- R2-2301057	Discussion on discontinuous coverage enhancements	ZTE Corporation, Sanechips

- R2-2301106	Discontinuous coverage enhancements	Samsung Electronics Nordic AB

- R2-2301188	Discussion on enhancements to discontinuous coverage	Xiaomi

- R2-2301210	Discussion on the discontinuous coverage	Huawei, Turkcell, HiSilicon

- R2-2301254	Discussion on the discontinuous coverage for IoT-NTN	CMCC

- R2-2301603	Discussion on enhancement to discontinuous coverage for IoT NTN	Transsion Holdings

- R2-2301625	Discussion on enhancement to discontinuous coverage for IoT NTN	Transsion Holdings

- R2-2301862	Complementing discontinuous coverage with minimum support for discontinuous feeder link operation	Sateliot, GateHouse, Novamint, Intelsat, Airbus

- R2-2301870	IoT NTN Enhancements to discontinuous coverage	Ericsson

- R2-2301886	Complementing discontinuous coverage with minimum support for discontinuous feeder link operation	Sateliot, GateHouse, Novamint, Intelsat, Airbus, Hispasat, ESA, TNO

## 4.3	RAN3

RAN3#118, November 14th – 18th, 2022, e-meeting

- R3-226317	Discussion on the support for discontinuous coverage	Nokia, Nokia Shanghai Bell

- R3-226318	Support for IOT NTN enhancements	Nokia, Nokia Shanghai Bell, CATT, Thales, Ericsson, Huawei

- R3-226444	Discussion of SA2 SI Conclusions and Potential RAN3 impacts	 Ericsson, Thales, Huawei

- R3-226445	Time-Based CHO and IoT NTN   Ericsson LM

- R3-226462	Time-Based HO for IoT NTN – S1AP Impacts	Ericsson, Huawei

- R3-226477	Further discussion on discontinuous coverage issue for IoT NTN	ZTE

- R3-226478	Support of discontinuous coverage for IoT NTN	ZTE

- R3-226594	Discussion on the discontinuous coverage issues	Samsung

- R3-226627	Further discussion on the support of discontinuous coverage	Huawei

- R3-226628	Support for IoT NTN enhancements 	Huawei, Nokia, Nokia Shanghai Bell, CATT, Ericsson

- R3-226629	X2AP CR on IoT NTN Functionality 	Huawei, CATT, Nokia, Nokia Shanghai Bell, Qualcomm, ZTE, CMCC, Ericsson

- R3-226630	S1AP impact to support discontinuous coverage	Huawei

- R3-226634	Discussion on the support of discontinuous coverage	CATT

RAN3#117bis-e, October 10th – 18th, 2022, e-meeting

- R3-225461	Discussion on the support for discontinuous coverage	Nokia, Nokia Shanghai Bell

- R3-225462	Support for IOT NTN enhancements	Nokia, Nokia Shanghai Bell

- R3-225613	Further discussion on discontinuous coverage issue for IoT NTN	ZTE

- R3-225614	Discussion on discontinuous coverage(draft LS included)	   CATT

- R3-225691	Further discussion on the support of discontinuous coverage	Huawei

- R3-225932	CB: # IoTNTN_Coverage - Summary of email discussion	ZTE - moderator

## 4.4	RAN4

RAN4#106, Athens, Greece, 27th Feb. – 3rd March, 2023

Submitted TDocs to AI 10.6.1, 10.6.4, 10.6.5 (RRM aspects)

- R4-2300394, Work Plan for Core part RRM requirements for IoT NTN enhancements, MediaTek Inc.

- R4-2303237, WF on RRM requirements for R18 IOT NTN enhancements, MediaTek Inc.

- R4-2302791, Topic summary for [106][233] IoT_NTN_enh, Moderator (MediaTek Inc.)

- R4-2300395, Discussion on RRM requirements for IoT NTN enhancement, MediaTek Inc.

- R4-2301807, Discussion on RRM requirements for IoT NTN enhancement, Huawei, HiSilicon.

- R4-2302397, Discussions on RRM requirements for IoT NTN enhancements, Ericsson.

Submitted TDocs to AI 10.6.2 (RF aspects)

- R4-2302426, RF spec impact analysis, Ericsson.

# 5	Others

END