3GPP TSG RAN meeting #99										RP-230661

Rotterdam, Netherlands, March 20-23, 2023

## Status Report to TSG

Agenda item:			9.2.6

| WI / SI Name | Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_AIML_Air |  |  |  |  |
| Unique ID | 940084 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221348 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2023 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 30 % | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Juan Montojo (RAN1); Mattias Frenne (RAN2); Xiaofeng Liu (RAN4) |
|  | Company | Qualcomm; Ericsson; CAICT |
|  | Email | juanm@qti.qualcomm.com; mattias.frenne@ericsson.com; liuxiaofeng1@caict.ac.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

##### 2.1.1.1	RAN1#112

General aspects of AI/ML framework

Agreement

To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.

Note: The Case definition is NOT intended to introduce sub-levels of Level z.

Note: Other cases may be included further upon interest from companies.

FFS: Z4 and Z5 boundary

Agreement

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

    - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

    - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

Agreement

- AI/ML-enabled Feature refers to a Feature where AI/ML may be used.

Agreement

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

Agreement

For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.

Evaluation on AI/ML for CSI feedback enhancement

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, except for Method 3 which has been supported, There is no consensus on whether to adopt an additional method.

Agreement

Confirm the following working assumption of RAN1#110bis-e:

Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors - Note: Eventual KPI can still be used to compare the performance

Conclusion

For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which have been agreed as the baseline metrics, for whether/how to introduce an additional intermediate KPI, NO additional intermediate KPI is adopted as mandatory.

- It is up to companies to optionally report other intermediate KPIs, e.g., Relative achievable rate (RAR)

Agreement

For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison

Agreement

Confirm the following WA on the benchmark for CSI prediction achieved in RAN1#111:

Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

Agreement

The CSI prediction-specific generalization scenario of various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h, etc.) is added to the list of scenarios for performing the generalization verification.

- FFS various frequency PRBs (e.g., trained based on one set of PRBs, inference on the same/different set of PRBs)

Agreement

For how to separate the templates for different training types/cases for AI/ML-based CSI compression without generalization/scalability verification, the following is considered:

- The determined template in the RAN1#111 working assumption is entitled with “1-on-1 joint training”

- A second separate template is introduced to capture the evaluation results for “multi-vendor joint training”

    - Note: this table captures the results for the joint training cases of 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, or N>1 NW part models to M>1 UE part models. An example is multi-vendor Type 2 training.

- A third separate template is introduced to capture the evaluation results for “separate training”

- FFS: additional KPIs for each template, e.g., overhead, latency, ect.

Agreement

For the evaluation of training Type 3 under CSI compression, besides the 3 cases considered for multi-vendors, add one new Case (1-on-1 training with joint training) as benchmark/upper bound for performance comparison.

- FFS the relationship between the pair(s) of models for Type 3 and the pair(s) of models for new Case

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers.

- Option 1-1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference, any specific model operates on multi-layers jointly.

  - FFS on the reported complexity and storage

  - FFS: input/output type

- Option 1-2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference, the model operates on multi-layers jointly.

  - FFS: input/output type

- Option 2 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report the setting is

    - Option 2-1: layer specific and rank common (different models applied for different layers; for a specific layer, the same model is applied for all rank values), or

    - Option 2-2: layer specific and rank specific (different models applied for different layers; for a specific layer, different models are applied for different rank values)

- Option 3 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report whether the setting is

    - Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference), or

    - Option 3-2: layer common and rank specific (different models applied for different rank values; for a specific rank, the same model is applied for all layers)

- Other options not precluded.

Agreement

The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE.

- For AI/ML based solutions: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank.

- For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB

  - Option 2a: The above-mentioned “CSI feedback overhead” is calculated as each CSI reported payload with a given rank

  - Option 2b: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank

Working Assumption

For the initial template for AI/ML-based CSI compression without generalization/scalability verification achieved in the working assumption in the RAN1#111 meeting, X, Y and Z are determined as:

- X is <=80bits

- Y is 100bits-140bits

- Z is  >=230bits

X, Y and Z are applicable for per layer

Working assumption

The following initial template is considered to replace the template achieved in the working assumption in the RAN1#111 meeting, for companies to report the evaluation results of AI/ML-based CSI compression of 1-on-1 joint training without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS the description and results for generalization/scalability may need a separate table

- Note: the values of CSI feedback overhead for the mean UPT and 5% UPT may need to be revisited in the 112bis-e meeting

- FFS: training related overhead

- FFS: how to capture CSI overhead reduction to the template

  - Note: It is to be captured to the template after a way is found on how to derive the CSI overhead reduction.

Table X. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, [traffic type], [Max rank value], [RU]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AI/ML model backbone |  |  |  |
|  | Pre-processing |  |  |  |
|  | Post-processing |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| CSI reconstruction part | AI/ML model backbone |  |  |  |
|  | [Pre-processing] |  |  |  |
|  | [Post-processing] |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| Common description | Input type |  |  |  |
|  | Output type |  |  |  |
|  | Quantization /dequantization method |  |  |  |
|  | Rank/layer adaptation settings for rank>1 |  |  |  |
| Dataset description | Train/k |  |  |  |
|  | Test/k |  |  |  |
|  | Ground-truth CSI quantization method (including scalar/codebook based quantization, and the parameters) |  |  |  |
|  | Overhead reduction compared to Float32 if high resolution quantization of ground-truth CSI is applied |  |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Benchmark |  |  |  |  |
| Benchmark assumptions, e.g., CSI overhead calculation method (Optional) |  |  |  |  |
| SGCS of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| SGCS of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for SGCS, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for SGCS, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … (other layers) |  |  |  |  |
| NMSE of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| NMSE of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for NMSE, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for NMSE, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … (other layers) |  |  |  |  |
| Other intermediate KPI (description/value) (optional) |  |  |  |  |
| Gain for other intermediate KPI (description/value) (optional) |  |  |  |  |
| Gain for Mean UPT (for a specific CSI feedback overhead) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for 5% UPT | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for upper bound without CSI compression over Benchmark –Mean UPT (Optional) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for upper bound without CSI compression over Benchmark –5% UPT (Optional) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| [CSI feedback reduction (%)] |  |  |  |  |
| … |  |  |  |  |
| FFS others |  |  |  |  |

- Note: “Benchmark” means the type of Legacy CB used for comparison.

- Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc.

- Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

Working assumption

A separate table to capture the evaluation results of generalization/scalability verification for AI/ML-based CSI compression is given in the following initial template

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI compression with model generalization/scalability, [Max rank value], [Scenario/configuration]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |
|  | Pre-processing |  |  |
|  | Post-processing |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
|  | Generalization/Scalability method description if applicable, e.g., truncation, adaptation layer, etc. |  |  |
|  | Input/output scalability dimension if applicable, e.g., N>=1 NW part model(s) to M>=1 UE part model(s) |  |  |
| Dataset description | Ground-truth CSI quantization method |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |
| Generalization Case 1 | Train (setting#A, size/k) |  |  |
|  | Test (setting#A, size/k) |  |  |
| SGCS, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| SGCS, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| NMSE, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| NMSE, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (other settings for Case 1) |  |  |  |
| … |  |  |  |
| Generalization Case 2 | Train (setting#A, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
| … (results for Case 2) |  |  |  |
| … (other settings for Case 2) |  |  |  |
| Generalization Case 3 | Train (setting#A+#B, size/k) |  |  |
|  | Test (setting#A/#B, size/k) |  |  |
| … (results for Case 3) |  |  |  |
| … (other settings for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting#A, size/k) |  |  |
|  | Fine-tune (setting#B, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
| … (results for Fine-tuning) |  |  |  |
| … (other settings for Fine-tuning) |  |  |  |
| FFS others |  |  |  |

- Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc.

- Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction with generalization verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI prediction with model generalization, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model description (e.g., backbone, structure) |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Generalization Case 1 | Train (setting#A, size/k) |  |  |
|  | Test (setting#A, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 1) |  |  |  |
| Generalization Case 2 | Train (setting#A, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 2) |  |  |  |
| Generalization Case 3 | Train (setting#A+#B, size/k) |  |  |
|  | Test (setting#A/#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting#A, size/k) |  |  |
|  | Fine-tune (setting#B, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Fine-tuning) |  |  |  |
| FFS others |  |  |  |

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for multi-vendor joint training and without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

- FFS case of multiple layers

Table X. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | [Training method] |  |  |
|  | Quantization /dequantization method |  |  |
| Dataset description | Train/k |  |  |
|  | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| Case 1 (baseline): NW#1-UE#1 | UE part AI/ML model backbone/structure |  |  |
|  | Network part AI/ML model backbone/structure |  |  |
| ... (other NW-UE combinations for Case 1) |  |  |  |
| Case 2 (1 NW part to M>1 UE parts) | NW part model backbone/structure |  |  |
|  | UE#1 part model backbone/structure |  |  |
|  | UE#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE#M part training dataset description and size |  |  |
| Case 3 (N>1 NW parts to 1 UE part) | UE part model backbone/structure |  |  |
|  | NW#1 part model backbone/structure |  |  |
|  | NW#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | NW#N part training dataset description and size |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| Case 1: NW#1-UE#1: Intermediate KPI | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (results for other NW-UE combinations for Case 1) |  |  |  |
| Case 2: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 3: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

- Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc.

- Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction par

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for sequentially separate training and without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

- FFS case of multiple layers

Table X. Evaluation results for CSI compression of separate training without model generalization/scalability, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
|  | Shared output of CSI generation part/input of reconstruction part is before or after quantization |  |  |
| Dataset description | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| [Benchmark: NW#1-UE#1 joint training] | UE part AI/ML model backbone/structure |  |  |
|  | Network part AI/ML model backbone/structure |  |  |
|  | Training dataset size |  |  |
| ... (other NW-UE combinations for benchmark) |  |  |  |
| Case 1-NW first training | NW part AI/ML model backbone/structure |  |  |
|  | UE#1 part model backbone/structure |  |  |
|  | UE#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE#M part training dataset description and size |  |  |
|  | [air-interface overhead of information (e.g., dataset) sharing] |  |  |
| Case 1-UE first training | NW#1 part model backbone/structure |  |  |
|  | NW#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | NW#N part training dataset description and size |  |  |
|  | UE part model backbone/structure |  |  |
|  | [air-interface overhead of information (e.g., dataset) sharing] |  |  |
| Case 2-UE first training | UE#1 part model backbone/structure |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE part AI/ML model backbone/structure |  |  |
|  | NW part training dataset description and size (e.g., description/size of dataset from M UEs and how to merge) |  |  |
| Case 3-NW first training | NW#1 part model backbone/structure |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | UE part model backbone/structure |  |  |
|  | UE part training dataset description and size (e.g., description/size of dataset from N NWs and how to merge) |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| NW#1-UE#1 joint training: Intermediate KPI | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (results for other 1-on-1 NW-UE joint training combinations) |  |  |  |
| Case 1-NW first training: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 1-UE first training: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 2-NW first training: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 3-NW first training: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

- Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantizaion (SQ/VQ), etc.

- Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

Other aspects on AI/ML for CSI feedback enhancement

Agreement

In CSI compression using two-sided model use case, further study potential specification impact of the following output-CSI-UE and input-CSI-NW at least for Option 1:

- Option 1: Precoding matrix

  - 1a: The precoding matrix in spatial-frequency domain

  - 1b: The precoding matrix represented using angular-delay domain projection

- Option 2: Explicit channel matrix (i.e., full Tx * Rx MIMO channel)

  - 2a: raw channel is in spatial-frequency domain

  - 2b: raw channel is in angular-delay domain

- Note: Whether Option 2 is also studied depends on the performance evaluations in 9.2.2.1.

- Note: RI and CQI will be discussed separately

Agreement

In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured.

- Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including

  - Option 1a: CQI is calculated based on target CSI with realistic channel measurement

  - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

  - Option 1c: CQI is calculated based on legacy codebook

- Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including

  - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment

    - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW.

  - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder.

- Other options are not precluded

- Note1: feasibility of different options should be evaluated

- Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated

- Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

Conclusion

In CSI compression using two-sided model use case, further discuss the pros/cons of different offline training collaboration types including at least the following aspects:

- Whether model can be kept proprietary

- Requirements on privacy-sensitive dataset sharing

- Flexibility to support cell/site/scenario/configuration specific model

- gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware

- Model update flexibility after deployment

- feasibility of allowing UE side and NW side to develop/update models separately

- Model performance based on evaluation in 9.2.2.1

- Whether gNB can maintain/store a single/unified model

- Whether UE device can maintain/store a single/unified model

- Extendability: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use

- Whether training data distribution can be matched to the device that will use the model for inference

- Whether device capability can be considered for model development

- Other aspects are not precluded

- Note: training data collection and dataset/model delivery will be discussed separately

Agreement

- In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least

- Enhancement of CSI-RS configuration to enable higher accuracy measurement.

- Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc.

  - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side.

- Signaling for triggering the data collection

- In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least:

- Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement.

- Contents of the ground-truth CSI including:

  - Data sample type, e.g., precoding matrix, channel matrix etc.

  - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like).

  - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator)

- Latency requirement for data collection

- Signaling for triggering the data collection

Agreement

In CSI compression using two-sided model use case, further study the following aspects for CSI configuration and report:

- NW configuration to determine CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration.

- How UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network.

Agreement

In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least:

- The priority rule regarding CSI collision handling and CSI omission

- Codebook subset restriction

- CSI processing Unit

In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least:

- NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side.

- UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side.

  - Network may configure a threshold criterion to facilitate UE to perform model monitoring.

- UE-side monitoring based on the output of the CSI reconstruction model at the UE-side

  - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side.

  - Network may configure a threshold criterion to facilitate UE to perform model monitoring.

- FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE.

Note: Monitoring approaches not based on intermediate KPI are not precluded

Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

Evaluation on AI/ML for beam management

Agreement

- Further study the impact of quantization error of inputed L1-RSRP (for training and inference)  for AI/ML model for beam management.

  - Existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) is the starting point for evaluation at least for network-sided model.

Agreement

- Further study on whether/how to evaluate the performance impact with L1-RSRP measurement accuracy.

Agreement

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as

  - Option A (baseline): the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B(optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Other options are not precluded and can be reported by companies.

- Note: This is only for evaluation discussion

Agreement

- For AI/ML models, which provide L1-RSRP as the model output, to evaluate the accuracy of predicted L1-RSRP, companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as:

  - The difference between the predicted L1-RSRP of Top-1[/K] predicted beam and the ideal L1-RSRP of the same beam.

Agreement

- For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), further study the following options as AI/ML model inputs

  - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID

    - E.g., measurements of Set B of beams together with default values (e.g. 0) for the beams not in Set B are used as AI inputs in a certain order/ matrix/ vector.

    - Detailed assumption can be reported by companies.

  - Alt 3: Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly

  - Note: Specification impact can be discussed separately.

Agreement

- Additionally study the following option on the selection of Set B of beams (pairs) (for Option 2: Set B is variable)

  - Opt D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C

- Companies report the number of pre-configured patterns used in the evaluation for Option 2: Set B is variable if applicable (e.g. Opt A and Opt B)

Other aspects on AI/ML for beam management

Conclusion

For the sub use case BM-Case1 and BM-Case2, “Alt.2: DL Rx beam prediction” is deprioritized.

Agreement

Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, study the following alternatives (including feasibility/necessity) with potential down-selection:

- Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy

- Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER

- Alt.3: Performance metric based on input/output data distribution of AI/ML

- Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP

- Other alternatives are not precluded

- Note: At least the performance and spec impact should be considered

Conclusion

Regarding the explicit assistance information from UE to network for NW-side AI/ML model, RAN1 has no consensus to support the following information

- UE location

- UE moving direction

- UE Rx beam shape/direction

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity, feasibility and the potential specification impact (if needed) of the following information reported from UE to network:

- Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s)

  - Whether/how to differentiate predicted L1-RSRP and measured L1-RSRP

- Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams)

  - FFS: Definition/content of confidence/probability information

- Note: At least the performance and spec impact should be considered

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements:

- Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable

- Beam indication from network for UE reception

- Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused).

Conclusion

Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information

- NW-side beam shape information

  - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc.

- Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion

  - e.g., some information following the same principle of Rel-17 positioning agreement

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding NW-side performance monitoring, study the following aspects as a starting point including the study of necessity:

- Configuration/Signaling from gNB to UE for measurement and/or reporting

- UE reporting to NW (e.g., for the calculation of performance metric)

- Indication from NW for UE to do LCM operations

- Other aspect(s) is not precluded

- Note1: At least the performance and reporting overhead of model monitoring mechanism should be considered

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following aspects as a starting point including the study of necessity and feasibility:

- Indication/request/report from UE to gNB for performance monitoring

  - Note: The indictation/request/report may be not needed in some case(s)

- Configuration/Signaling from gNB to UE for performance monitoring

- Other aspect(s) is not precluded

Evaluation on AI/ML for positioning accuracy enhancement

Agreement

For both direct AI/ML positioning and AI/ML assisted positioning, companies include the evaluation area in their reporting template, assuming the same evaluation area is used for training dataset and test dataset.

Note:

- Baseline evaluation area for InF-DH = 120x60 m.

- if different evaluation areas are used for training dataset and test dataset, they are marked out separately under “Train” and “Test” instead.

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [with or without] model generalization, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m]

| Model input | Model output | Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Test | Model complexity | Computation complexity | AI/ML |

Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m]

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | Test | Model complexity | Computation complexity | AI/ML |

Agreement

The agreement made in RAN1#110 AI 9.2.4.1 is updated by adding additional note:

Note: if complex value is used in modelling process, the number of the model parameters is doubled, which is also applicable for other AIs of AI/ML

Agreement

For both the direct AI/ML positioning and AI/ML assisted positioning, study the model input, considering the tradeoff among model performance, model complexity and computational complexity.

- The type of information to use as model input. The candidates include at least: time-domain CIR, PDP.

- The dimension of model input in terms of NTRP, Nt, and Nt’.

- Note: For the direct AI/ML positioning, model input size has impact to signaling overhead for model inference.

Agreement

For direct AI/ML positioning, study the performance of model monitoring methods, including:

- Label based methods, where ground truth label (or its approximation) is provided for monitoring the accuracy of model output.

- Label-free methods, where model monitoring does not require ground truth label (or its approximation).

Agreement

For AI/ML assisted approach, study the performance of label-free model monitoring methods, which do not require ground truth label (or its approximation) for model monitoring.

Conclusion

- No dedicated evaluation is needed for the positioning accuracy performance of model switching

- It does not preclude future discussion on model switching related performance

Agreement

For direct AI/ML positioning, study the impact of labelling error to positioning accuracy

- The ground truth label error in each dimension of x-axis and y-axis can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range.

  - Value L is up to sources.

- Other models are not precluded

- [Whether/how to study the impact of labelling error to label-based model monitoring methods]

- [Whether/how to study the impact of labelling error for AI/ML assisted positioning.]

Observation

Evaluation of the following generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario.

- The generalization aspects include:

  - Different drops

  - Different clutter parameters

  - Different InF scenarios

  - Network synchronization error

- Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training.

  - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset.

  - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset.

Note: ideal model training and switching may provide the upper bound of achievable performance when the AI/ML model needs to handle different deployment scenarios.

Other aspects on AI/ML for positioning accuracy enhancement

Agreement

Regarding training data generation for AI/ML based positioning,

- The following options of entity and mechanisms to generate ground truth label are identified

  - At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a)

  - At least LMF with known PRU location is identified to generate ground truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b)

  - At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a)

  - FFS whether and if so, applicable conditions and potential specification impact for the following options to generate ground truth label

    - UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods

    - Network entity generates ground truth label based on positioning methods

- The following options of entity to generate other training data (at least measurement corresponding to model input) are identified

  - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b)

    - PRU

    - UE

  - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b)

    - TRP

- Note: transfer of training data from the entity generating training data to a different entity is not precluded and associated potential specification impact is for further study

Agreement

Regarding training data collection for AI/ML based positioning, study benefit(s) and potential specification impact (including necessity) at least for the following aspects

- Associated information of training data

  - Quality indicator at least for ground truth label (if needed)

  - Other information associated with training data is not precluded. E.g., information related training dataset/samples, information related to scenario, resource configuration & mapping, timing for training data, information on implementation imperfections, etc.

- Assistance signaling and procedure to facilitate generating/collecting training data

  - Potential determination of the UE/PRU/TRP which can provide the training data

  - Configuration of reference signal (for measurement and/or label)

  - Signaling other than above 2 for data collection

    - E.g., requested quality of training data

Agreement

Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects

- Entity to derive monitoring metric

  - UE at least for Case 1 and 2a (with UE-side model)

    - FFS PRU for Case 1 and 2a

  - gNB at least for Case 3a (with gNB-side model)

    - FFS gNB for Case 3b (with LMF-side model)

  - LMF at least for Case 2b and 3b (with LMF-side model)

  - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b)

- If model monitoring does not require ground truth label (or its approximation).

  - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc.

  - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc.

  - report of the calculated metric and/or model monitoring decision

- If model monitoring requires and is provided ground truth label (or its approximation)

  - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc.

  - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc.

  - report of the calculated metric and/or model monitoring decision

- Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded

Agreement

Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement

- For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead

  - Potential new measurement: CIR/PDP

  - existing measurement: e.g., RSRP/RSRPP/RSTD

  - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied

  - Note2: study the impact of model input for other cases are not precluded

- For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF

  - new measurement report: e.g., ToA, path phase

  - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP

  - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD

- Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model

  - RS configurations

  - Other assistance information is not precluded

Note: Companies are encouraged to report their assumption of functionality and their assumption of information element(s) of AI/ML functionality identification for AI/ML based positioning with UE-side model (Case 1 and 2a).

#### 2.1.2	Remaining Open issues

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

- Complete AI/ML model, terminology and description to identify common and specific characteristics for framework investigations:

  - Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

  - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

    - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

    - Various levels of UE/gNB collaboration targeting at separate or joint ML operation.

  - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating

  - Dataset(s) for training, validation, testing, and inference

  - Identify common notation and terminology for AI/ML related functions, procedures and interfaces

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

## 2.2	RAN2

#### 2.2.1	Agreements

##### 2.2.2.1	RAN2#121

Data Collection

Proposal 1	RAN2 to simultaneously focus on studying data collection solutions for both NW- and UE-sided AIML models, including assistance signalling and (dataset) reporting from the concerning entity.

Proposal 2	Study RAN2 implications of data collection for all concerning LCM purpose, e.g., model training/monitoring/selection/update/inference/etc.

Proposal 3	RAN2 to separately analyse the data collection requirements and solutions for the different LCM purposes. FFS if general frameworks/solutions could be adopted.

Proposal 4	Wait for RAN1 requirements before discussing specific data collection solutions for use cases and for the related (LCM) procedures. In the meantime, RAN2 can summarize the implementation of existing frameworks while focusing on different performance metrics.

Proposal 5	When summarizing the different data collection frameworks, RAN2 can start by considering the following metrics: a) the content of the data, b) the data size, c) latency and periodicity, d) signalling, entities involved, and configuration aspects. FFS on how to handle security/privacy.

Proposal 6	Consider the following existing frameworks as starting points to be considered for data collection: SON & MDT, UE assistance information, RRM measurement reports, CSI reporting framework, LPP Provide location information. FFS whether other frameworks should be discussed.

Proposal 7	Upon receiving specific (RAN1) requirements, RAN2 to decide whether the existing frameworks can be reused/extended, or whether a new framework is required.

Proposal 8	For data collection, RAN2 will simply keep progressing and will inform of concerning agreements to RAN1 when necessary.

P1-P8 are loosely endorsed with the understanding that we can also go beyond, e.g. analyse other methods.

Endorse the table (below) as a starting point (e.g. can add more columns if needed later, modify, add rows etc). Content shall be interpreted as current content.

Chair: There is significant support to aim for evaluating the data collection methods per LCM purpose

R2 may consider including the existing EVEX framework for this SI, FFS exactly what this means, can discuss next meeting.

|  | Involved Network entity | RRC state to generate data | Max payload size per reporting* | Contents to be collected | End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Logged MDT | TCE/OAM (It can be utilized by gNB) | RRC_IDLE/RRRC_INACTIVE | <9kbyte | L3 cell/beam measurements, location info, sensor info, timing info | Procedure latency***: Latency to enter CONNECTED state Latency to receive gNB request signaling (~20ms) Air interface signaling latency****: ~20ms (RRC) Other latency: Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message, Privacy via user consent |
| Immediate MDT | TCE/OAM (It can be utilized by gNB) | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements, location info, sensor info | Procedure latency: Report interval: l20ms~30min for periodic report TTT for event triggered report Air interface signaling latency: ~20ms (RRC) Other latency: Forwarding latency between gNB and TCE | Event triggered report, Periodic reporting | AS security via RRC message, Privacy via user consent |
| L3 measurements | gNB | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements | Procedure latency: Report interval: l20ms~30min for periodic report TTT for event triggered report Air interface signaling latency: 20ms (RRC) | Event triggered report, Periodic reporting | AS security via RRC message. |
| L1 measurement (CSI reporting) | gNB | RRC_CONNECTED | <1706bit in PUCCH, <3840bit in PUSCH | L1 CSI measurement | Procedure latency: Report interval: 4-320 slot for periodic report and semi-persistent report 0-32 slot after reception of DCI for aperiodic report Air interface signaling latency: 1 TTI (PUCCH) | Aperiodic report, Semi-persistent report, Periodic report | No AS security |
| UAI | gNB | RRC_CONNECTED | <9kbyte | Assistance information to show UE preference | Procedure latency: Upon generation of UE's preference Air interface signaling latency: ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Early measurements | gNB | RRC_IDLE/RRC_INACTIVE | <9kbyte | L3 cell/beam measurements | Procedure latency: Latency to enter CONNECTED state Latency to receive gNB request signaling (~20ms) Air interface signaling latency: ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| LPP | LMF | RRC_CONNECTED | <9kbyte | Location info | Procedure latency: Latency to get upper layer trigger (for UE triggered) Or latency to receive NW request message (~20ms) Air interface signaling latency: ~20ms (RRC) Other latency: Forwarding latency between gNB and LMF | UE-triggered, NW-triggered | AS security via RRC message |

Note:
* The payload size doesn't consider signaling overhead.
** The End-to-End report latency is the latency from availability of the measurement report at the UE side to the availability of the measurement report at the terminated network entity. The time to generate data or perform measurements depends on RAN1/RAN4 specification.
*** Procedure latency is the latency caused by procedures, including procedure to ready for reporting (e.g. entering CONNECTED state, report interval)
****Air interface signaling latency is the latency to transmit one report, e.g. RRC signaling latency or PUCCH signaling latency.

Model Transfer

We Use the wording “model transfer/delivery”

model delivery that serves the use cases in the SI is within RAN2 scope, regardless other aspects.

Agreed:

- Aim to at least analyze the feasibility and benefits of model/transfer solutions based on the following:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

- Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP).

Table: relations between solutions and use cases

| Solutions | Applicable use cases |
| --- | --- |
| Solution 1a, 1b | CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 1a and 1b. |
| Solution 2a, 2b | CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 2a and 2b. |
| Solution 3a, 3b | Positioning accuracy enhancement |
| Solution 4 | CSI feedback enhancement Beam management Positioning accuracy enhancement |

- Note: the solutions use case relation is preliminary (work in progress), and the purpose is to have better understanding on what to further analyse

The table (below) can serve as starting point for continued discussion (but contains some parts that seems non consensus, e.g. delta configuration).

|  | Pros | Cons |
| --- | --- | --- |
| Solution 1a | 6. The existing RRC signaling solutions can be reused as baseline, at least including delta signaling and segementation 9. Additional security and verification may not be necessary as the UE already established security before the transfer is initiated 11. gNB can take the control of the AIML model transfer itself, which can not be achieved by traditional UP based solution | 1. Face challenges to convey large size or “no upper limit size” AI model by RRC message (e.g. >45kBytes) 2. Maybe high control plane overhead, as a large model size may need segmentation/transmission/acknowledgment. This consumes critical configuration time for model transfer/delivery 3. An incomplete control plane model transfer has to be restarted upon mobility, as there are no current procedures to resume transmission across gNBs. Some companies wonder whether it is critical or not as it depends on how frequent the gNB to send new/updated AI/ML to the UE |
| Solution 2a and 3a | 5. Service continuity on model transfer/delivery is easy to achieve compared with Solution 1a 6. Impacts on RAN2 may be limited (some companies think that LPP signalling is in RAN2 scope) | 1. Face challenges to convey large size or “no upper limit size” AI model by RRC message (e.g. >45kBytes) 3. If NAS does the segmentation, it may introduce some overhead 4. (only valid for Solution 2a) CN is not a good option for later on model monitoring/activation/deactivation/fallback/update that requires less latency. The model transfer/delivery is transparent to gNB, it could be tricky to get gNB involved in the AI model LCM. It could be problematic when the network needs to be in control of what happening at the UE side and especially in two-sided models where one side of the model is intended to be located at the network side |
| Solution 1b | 1. The network can provide different 5QIs for model transfer/delivery with different QoS requirements (e.g. can support large model size) 2. Compared with CP-based solutions, this Solution 1b can reduces control plane overhead, reduces overhead at gNB for model delivery/transfer 5. Compared with CP-based solutions, it may not need to consider CP message segmentation, CP message blocking issue | 5. Not compatible with current mobility procedure. Supporting model transfer during mobility is not so straightforward |
| Solution 2b and 3b | 1. The network can provide different 5QIs for model transfer/delivery with different QoS requirements (e.g. can support large model size) 5. Compared with CP-based solutions, it may not need to consider CP message segmentation, CP message blocking issue | 2. CP signalling is needed to configure and initiate the model transfer from the CN 4. May be unable to support delta-model transfer/delivery based on current user plane framework |
| Solution 4 | 2. If 3GPP network can be aware of AI/ML model in this Solution 4, the network can provide different 5QIs for model transfer/delivery with different QoS requirements (e.g. can support large model size). How to synchronize 3GPP and server so that the network can take appropriate actions is not clear, and it may not be fully under 3GPP control | 2. There may be inter-operability issues, such as: a) Different implementations may lead to different model performances and a huge burden of model management (e.g., frequent model activation/deactivation) b) Massive offline coordination is needed or requires lots of coordinations among vendors, especially for the CSI compression use case 4. When network cannot control the model transfer/delivery, the transfer of large model may impact important and delay sensitive user data traffic |

Model ID and UE cap

RAN2 assumes that Model ID is unique “globally”, e.g. in order to manage test certification each retrained version need to be identified.

#### 2.2.2	Remaining Open issues

And while it was not agreed which issues will be addressed at the next meeting, based on what was discussed during RAN2#121 the following do appear to be relevant:

    - Architecture; functionality-to-entity mapping,

    - Life Cycle Management implications from a RAN2 point of view,

    - Progress with data collection and model transfer tables (e.g., decide how to use them, whether there is a need to restructure them, analyse/modify content, etc)

      - Understand EVEX and other potential frameworks for data collection

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

## 4.1	RAN1

#### 4.1.1	RAN1#112

// TR

R1-2301402	Technical report for Rel-18 SI on AI and ML for NR air interface	Qualcomm Incorporated

// General Aspects of AI/ML framework

R1-2301868	Final Summary of General Aspects of AI/ML Framework	Moderator (Qualcomm)

R1-2301865	Summary#3 of General Aspects of AI/ML Framework	Moderator (Qualcomm)

R1-2301864	Summary#2 of General Aspects of AI/ML Framework	Moderator (Qualcomm)

R1-2301863	Summary#1 of General Aspects of AI/ML Framework	Moderator (Qualcomm)

R1-2300043	Discussion on common AI/ML characteristics and operations	FUTUREWEI

R1-2300107	Discussion on general aspects of AI/ML framework	Huawei, HiSilicon

R1-2300170	Discussion on general aspects of common AI PHY framework	ZTE

R1-2300178	Discussion on general aspects of AIML framework	Ericsson

R1-2300210	Discussion on general aspects of AI/ML framework	Spreadtrum Communications

R1-2300279	On general aspects of AI/ML framework	OPPO

R1-2300396	On General Aspects of AI/ML Framework	Google

R1-2300443	Discussions on AI/ML framework	vivo

R1-2300529	General aspects on AI/ML framework	LG Electronics

R1-2300566	Views on the general aspects of AI/ML framework	xiaomi

R1-2300603	Further discussion on the general aspects of ML for Air-interface	Nokia, Nokia Shanghai Bell

R1-2300670	Discussion on general aspects of AI/ML framework	CATT

R1-2300743	Discussion on general aspects of AI/ML framework	Fujitsu

R1-2300823	Discussion on general aspects of AI ML framework	NEC

R1-2300840	Considerations on general aspects on AI-ML framework	CAICT

R1-2300868	Considerations on common AI/ML framework	Sony

R1-2300906	Discussion on general aspects of AI/ML framework	KDDI Corporation

R1-2300940	Discussion on general aspects of AI/ML framework	Intel Corporation

R1-2300989	Discussion on general aspects of AI/ML framework	CMCC

R1-2301040	Discussion on general aspects of AI/ML framework for NR air interface	ETRI

R1-2301139	General aspects of AI/ML framework	Fraunhofer IIS, Fraunhofer HHI

R1-2301147	Discussion on general aspects of AI/ML framework	Panasonic

R1-2301155	Discussion on general aspects of AI/ML framework	InterDigital, Inc.

R1-2301160	Discussion on AI/ML Framework	Rakuten Mobile, Inc

R1-2301177	General aspects of AI and ML framework for NR air interface	NVIDIA

R1-2301198	General aspects of AI/ML framework	Lenovo

R1-2301220	General aspects of AI/ML Framework	AT&T

R1-2301254	General aspects of AI ML framework and evaluation methodogy	Samsung

R1-2301336	Discussion on general aspect of AI/ML framework	Apple

R1-2301403	General aspects of AI/ML framework	Qualcomm Incorporated

R1-2301484	Discussion on general aspects of AI/ML framework	NTT DOCOMO, INC.

R1-2301586	Discussion on general aspects of AI/ML LCM	MediaTek Inc.

R1-2301663	Discussions on Common Aspects of AI/ML Framework	TCL Communication Ltd.

R1-2301664	Identifying Procedures for General Aspects of AI/ML Frameworks	Indian Institute of Tech (M), CEWiT, IIT Kanpur

// AI/ML for CSI feedback enhancement

R1-2301939	Summary#4 for CSI evaluation of [112-R18-AI/ML]	Moderator (Huawei)

R1-2301938	Summary#3 for CSI evaluation of [112-R18-AI/ML]	Moderator (Huawei)

R1-2301938	Summary#2 for CSI evaluation of [112-R18-AI/ML]	Moderator (Huawei)

R1-2301936	Summary#1 for CSI evaluation of [112-R18-AI/ML]	Moderator (Huawei)

R1-2300044	Discussion and evaluation of AI/ML for CSI feedback enhancement	FUTUREWEI

R1-2300108	Evaluation on AI/ML for CSI feedback enhancement	Huawei, HiSilicon

R1-2300154	Evaluations of AI-CSI	Ericsson

R1-2300171	Evaluation on AI CSI feedback enhancement	ZTE

R1-2300211	Discussion on evaluation on AI/ML for CSI feedback enhancement	Spreadtrum Communications, BUPT

R1-2300280	Evaluation methodology and results on AI/ML for CSI feedback enhancement	OPPO

R1-2300348	Evaluation on AI ML for CSI feedback enhancement	Mavenir

R1-2300397	On Evaluation of AI/ML based CSI	Google

R1-2300444	Evaluation on AI/ML for CSI feedback enhancement	vivo

R1-2300501	Evaluation of AI/ML based methods for CSI feedback enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2300530	Evaluation on AI/ML for CSI feedback enhancement	LG Electronics

R1-2300567	Discussion on evaluation on AI/ML for CSI feedback enhancement	xiaomi

R1-2300604	Evaluation of ML for CSI feedback enhancement	Nokia, Nokia Shanghai Bell

R1-2300671	Evaluation on AI/ML for CSI feedback enhancement	CATT

R1-2300716	Evaluation on AI/ML for CSI feedback enhancement	China Telecom

R1-2300744	Evaluation on AI/ML for CSI feedback enhancement	Fujitsu

R1-2300841	Some discussions on evaluation on AI-ML for CSI feedback	CAICT

R1-2300941	Evaluation for CSI feedback enhancements	Intel Corporation

R1-2300990	Discussion on evaluation on AI/ML for CSI feedback enhancement	CMCC

R1-2301031	Evaluation on AI/ML for CSI feedback enhancement	Indian Institute of Tech (H)

R1-2301041	Evaluation on AI/ML for CSI feedback enhancement	ETRI

R1-2301097	Evaluation of joint CSI estimation and compression with AI/ML	BJTU

R1-2301156	Evaluation on AI/ML for CSI feedback enhancement	InterDigital, Inc.

R1-2301178	Evaluation of AI and ML for CSI feedback enhancement	NVIDIA

R1-2301199	Evaluation on AI/ML for CSI feedback	Lenovo

R1-2301223	Discussion on AI/ML for CSI feedback enhancement	AT&T

R1-2301255	Evaluation on AI/ML for CSI feedback enhancement	Samsung

R1-2301337	Evaluation for AI/ML based CSI feedback enhancement	Apple

R1-2301404	Evaluation on AI/ML for CSI feedback enhancement	Qualcomm Incorporated

R1-2301466	Evaluation of AI/ML based methods for CSI feedback enhancement	SEU Late submission

R1-2301485	Discussion on evaluation on AI/ML for CSI feedback enhancement	NTT DOCOMO, INC.

R1-2301587	Evaluation on AI/ML for CSI feedback enhancement	MediaTek Inc.

R1-2301688	Evaluation of AI and ML for CSI feedback enhancement	CEWiT

R1-2301913	Summary #4 on other aspects of AI/ML for CSI enhancement	Moderator (Apple)

R1-2301912	Summary #3 on other aspects of AI/ML for CSI enhancement	Moderator (Apple)

R1-2301911	Summary #2 on other aspects of AI/ML for CSI enhancement	Moderator (Apple)

R1-2301910	Summary #1 on other aspects of AI/ML for CSI enhancement	Moderator (Apple)

R1-2300045	Discussion on other aspects of AI/ML for CSI feedback enhancement	FUTUREWEI

R1-2300071	Further discussions of AI/ML for CSI feedback enhancement	Keysight Technologies UK Ltd, Universidad de Málaga

R1-2300109	Discussion on AI/ML for CSI feedback enhancement	Huawei, HiSilicon

R1-2300153	Discussion on AI-CSI	Ericsson

R1-2300172	Discussion on other aspects for AI CSI feedback enhancement	ZTE

R1-2300212	Discussion on other aspects on AI/ML for CSI feedback	Spreadtrum Communications

R1-2300281	On sub use cases and other aspects of AI/ML for CSI feedback enhancement	OPPO

R1-2300398	On Enhancement of AI/ML based CSI	Google

R1-2300445	Other aspects on AI/ML for CSI feedback enhancement	vivo

R1-2300531	Other aspects on AI/ML for CSI feedback enhancement	LG Electronics

R1-2300568	Discussion on potential specification impact for CSI feedback based on AI/ML	xiaomi

R1-2300605	Other aspects on ML for CSI feedback enhancement	Nokia, Nokia Shanghai Bell

R1-2300672	Potential specification impact on AI/ML for CSI feedback enhancement	CATT

R1-2300717	Discussion on AI/ML for CSI feedback enhancement	China Telecom

R1-2300745	Views on specification impact for CSI feedback enhancement	Fujitsu

R1-2300767	Discussion on AI/ML for CSI feedback enhancement	NEC

R1-2300842	Discussions on AI-ML for CSI feedback	CAICT

R1-2300863	Discussion on AI/ML for CSI feedback enhancement	Panasonic

R1-2300869	Considerations on CSI measurement enhancements via AI/ML	Sony

R1-2300942	On other aspects on AI/ML for CSI feedback	Intel Corporation

R1-2300991	Discussion on other aspects on AI/ML for CSI feedback enhancement	CMCC

R1-2301042	Discussion on other aspects on AI/ML for CSI feedback enhancement	ETRI

R1-2301098	Joint CSI estimation and compression with AI/ML	BJTU

R1-2301157	Discussion on AI/ML for CSI feedback enhancement	InterDigital, Inc.

R1-2301179	AI and ML for CSI feedback enhancement	NVIDIA

R1-2301200	Further aspects of AI/ML for CSI feedback	Lenovo

R1-2301224	Discussion on AI/ML for CSI feedback enhancement	AT&T

R1-2301256	Representative sub use cases for CSI feedback enhancement	Samsung

R1-2301313	Discussion on AI/ML for CSI Feedback Enhancement	III

R1-2301338	Discussion on other aspects of AI/ML for CSI enhancement	Apple

R1-2301405	Other aspects on AI/ML for CSI feedback enhancement	Qualcomm Incorporated

R1-2301486	Discussion on other aspects on AI/ML for CSI feedback enhancement	NTT DOCOMO, INC.

R1-2301588	Other aspects on AI/ML for CSI feedback enhancement	MediaTek Inc.

R1-2301665	Discussions on Sub-Use Cases in AI/ML for CSI Feedback Enhancement	TCL

// AI/ML for beam management

R1-2301958	Feature lead summary #3 evaluation of AI/ML for beam management	Moderator (Samsung)

R1-2301957	Feature lead summary #2 evaluation of AI/ML for beam management	Moderator (Samsung)

R1-2301956	Feature lead summary #1 evaluation of AI/ML for beam management	Moderator (Samsung)

R1-2300046	Discussion and evaluation of AI/ML for beam management	FUTUREWEI

R1-2300110	Evaluation on AI/ML for beam management	Huawei, HiSilicon

R1-2300173	Evaluation on AI beam management	ZTE

R1-2300179	Evaluations of AIML for beam management	Ericsson

R1-2300213	Evaluation on AI/ML for beam management	Spreadtrum Communications

R1-2300282	Evaluation methodology and results on AI/ML for beam management	OPPO

R1-2300399	On Evaluation of AI/ML based Beam Management	Google

R1-2300446	Evaluation on AI/ML for beam management	vivo

R1-2300532	Evaluation on AI/ML for beam management	LG Electronics

R1-2300569	Evaluation on AI/ML for beam management	xiaomi

R1-2300593	Discussion for evaluation on AI/ML for beam management	InterDigital, Inc.

R1-2300606	Evaluation of ML for beam management	Nokia, Nokia Shanghai Bell

R1-2300673	Evaluation on AI/ML for beam management	CATT

R1-2300718	Evaluation on AI/ML for beam management	China Telecom

R1-2300746	Evaluation on AI/ML for beam management	Fujitsu

R1-2300843	Some discussions on evaluation on AI-ML for Beam management	CAICT

R1-2300943	Evaluations for AI/ML beam management	Intel Corporation

R1-2300992	Discussion on evaluation on AI/ML for beam management	CMCC

R1-2301180	Evaluation of AI and ML for beam management	NVIDIA

R1-2301201	Evaluation on AI/ML for beam management	Lenovo

R1-2301257	Evaluation on AI/ML for Beam management	Samsung

R1-2301339	Evaluation for AI/ML based beam management enhancements	Apple

R1-2301406	Evaluation on AI/ML for beam management	Qualcomm Incorporated

R1-2301487	Discussion on evaluation on AI/ML for beam management	NTT DOCOMO, INC.

R1-2301589	Evaluation on AI/ML for beam management	MediaTek Inc.

R1-2301689	Evaluation on AI/ML for beam management	CEWiT

R1-2301897	Summary#4 for other aspects on AI/ML for beam management	Moderator (OPPO)

R1-2301896	Summary#3 for other aspects on AI/ML for beam management	Moderator (OPPO)

R1-2301895	Summary#2 for other aspects on AI/ML for beam management	Moderator (OPPO)

R1-2301894	Summary#1 for other aspects on AI/ML for beam management	Moderator (OPPO)

R1-2300047	Discussion on other aspects of AI/ML for beam management	FUTUREWEI

R1-2300111	Discussion on AI/ML for beam management	Huawei, HiSilicon

R1-2300174	Discussion on other aspects for AI beam management	ZTE

R1-2300180	Discussion on AIML for beam management	Ericsson

R1-2300195	Discussion on other aspects of AI/ML beam management	New H3C Technologies Co., Ltd.

R1-2300214	Other aspects on AI/ML for beam management	Spreadtrum Communications

R1-2300283	Other aspects of AI/ML for beam management	OPPO

R1-2300400	On Enhancement of AI/ML based Beam Management	Google

R1-2300447	Other aspects on AI/ML for beam management	vivo

R1-2300533	Other aspects on AI/ML for beam management	LG Electronics

R1-2300570	Potential specification impact on AI/ML for beam management	xiaomi

R1-2300594	Discussion for other aspects on AI/ML for beam management	InterDigital, Inc.

R1-2300607	Other aspects on ML for beam management	Nokia, Nokia Shanghai Bell

R1-2300674	Potential specification impact on AI/ML for beam management	CATT

R1-2300747	Sub use cases and specification impact on AI/ML for beam management	Fujitsu

R1-2300824	Discussion on AI/ML for beam management	NEC

R1-2300844	Discussions on AI-ML for Beam management	CAICT

R1-2300870	Consideration on AI/ML for beam management	Sony

R1-2300944	Other aspects on AI/ML for beam management	Intel Corporation

R1-2300993	Discussion on other aspects on AI/ML for beam management	CMCC

R1-2301043	Discussion on other aspects on AI/ML for beam management	ETRI

R1-2301181	AI and ML for beam management	NVIDIA

R1-2301197	Discussion on AI/ML for beam management	Panasonic

R1-2301202	Further aspects of AI/ML for beam management	Lenovo

R1-2301258	Representative sub use cases for beam management	Samsung

R1-2301340	Discussion on other aspects of AI/ML for beam management	Apple

R1-2301407	Other aspects on AI/ML for beam management	Qualcomm Incorporated

R1-2301488	Discussion on other aspects on AI/ML for beam management	NTT DOCOMO, INC.

R1-2301539	Discussion on other aspects on AI/ML for beam management	KT Corp.

R1-2301590	Other aspects on AI/ML for beam management	MediaTek Inc.

R1-2301685	Discussions on Sub-Use Cases in AI/ML for Beam Management	TCL Communication Ltd.

// AI/ML for positioning accuracy enhancement

R1-2302170	Summary #6 of Evaluation on AI/ML for positioning accuracy enhancement	Moderator (Ericsson)

R1-2301969	Summary #4 of Evaluation on AI/ML for positioning accuracy enhancement	Moderator (Ericsson)

R1-2301949	Summary #3 of Evaluation on AI/ML for positioning accuracy enhancement	Moderator (Ericsson)

R1-2301947	Summary #2 of Evaluation on AI/ML for positioning accuracy enhancement	Moderator (Ericsson)

R1-2301946	Summary #1 of Evaluation on AI/ML for positioning accuracy enhancement	Moderator (Ericsson)

R1-2300112	Evaluation on AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2300141	Evaluation of AI/ML for Positioning Accuracy Enhancement	Ericsson Inc.

R1-2300175	Evaluation on AI positioning enhancement	ZTE

R1-2300284	Evaluation methodology and results on AI/ML for positioning accuracy enhancement	OPPO

R1-2300401	On Evaluation of AI/ML based Positioning	Google

R1-2300448	Evaluation on AI/ML for positioning accuracy enhancement	vivo

R1-2300534	Evaluation on AI/ML for positioning accuracy enhancement	LG Electronics

R1-2300571	Evaluation on AI/ML for positioning accuracy enhancement	xiaomi

R1-2300608	Evaluation of ML for positioning accuracy enhancement	Nokia, Nokia Shanghai Bell

R1-2300675	Evaluation on AI/ML for positioning enhancement	CATT

R1-2300719	Evaluation on AI/ML for positioning accuracy enhancement	China Telecom

R1-2300748	Discussions on evaluation results of AIML positioning accuracy enhancement	Fujitsu

R1-2300845	Some discussions on evaluation on AI-ML for positioning accuracy enhancement	CAICT

R1-2300994	Discussion on evaluation on AI/ML for positioning accuracy enhancement	CMCC

R1-2301101	Evaluation on AI/ML for positioning accuracy enhancement	InterDigital, Inc.

R1-2301182	Evaluation of AI and ML for positioning enhancement	NVIDIA

R1-2301203	Discussion on AI/ML Positioning Evaluations	Lenovo

R1-2301259	Evaluation on AI/ML for Positioning	Samsung

R1-2301341	Evaluation on AI/ML for positioning accuracy enhancement	Apple

R1-2301408	Evaluation on AI/ML for positioning accuracy enhancement	Qualcomm Incorporated

R1-2301591	Evaluation of AIML for Positioning Accuracy Enhancement	MediaTek Inc.

R1-2301690	Evaluation on AI/ML for Positioning Accuracy Enhancement	CEWiT

R1-2302019	FL summary #3 of other aspects on AI/ML for positioning accuracy enhancement	Moderator (vivo)

R1-2301996	FL summary #2 of other aspects on AI/ML for positioning accuracy enhancement	Moderator (vivo)

R1-2301847	FL summary #1 of other aspects on AI/ML for positioning accuracy enhancement	Moderator (vivo)

R1-2300113	Discussion on AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2300142	Other Aspects of AI/ML Based Positioning Enhancement	Ericsson Inc.

R1-2300176	Discussion on other aspects for AI positioning enhancement	ZTE

R1-2300215	Discussion on other aspects on AI/ML for positioning accuracy enhancement	Spreadtrum Communications

R1-2300285	On sub use cases and other aspects of AI/ML for positioning accuracy enhancement	OPPO

R1-2300402	On Enhancement of AI/ML based Positioning	Google

R1-2300449	Other aspects on AI/ML for positioning accuracy enhancement	vivo

R1-2300535	Other aspects on AI/ML for positioning accuracy enhancement	LG Electronics

R1-2300572	Views on the other aspects of AI/ML-based positioning accuracy enhancement	xiaomi

R1-2300602	Other aspects on AI-ML for positioning accuracy enhancement	Baicells

R1-2300609	Other aspects on ML for positioning accuracy enhancement	Nokia, Nokia Shanghai Bell

R1-2300676	Potential specification impact on AI/ML for positioning enhancement	CATT

R1-2300749	Discussions on specification impacts for AIML positioning accuracy enhancement	Fujitsu

R1-2300831	Discussion on AI/ML for positioning accuracy enhancement	NEC

R1-2300846	Discussions on AI-ML for positioning accuracy enhancement	CAICT

R1-2300871	On Other Aspects on AI/ML for Positioning Accuracy Enhancement	Sony

R1-2300995	Discussion on other aspects on AI/ML for positioning accuracy enhancement	CMCC

R1-2301115	Designs and potential specification impacts of AIML for positioning	InterDigital, Inc.

R1-2301140	On potential AI/ML solutions for positioning	Fraunhofer IIS, Fraunhofer  HHI

R1-2301183	AI and ML for positioning enhancement	NVIDIA

R1-2301204	AI/ML Positioning use cases and associated Impacts	Lenovo

R1-2301260	Representative sub use cases for Positioning	Samsung

R1-2301342	On Other aspects on AI/ML for positioning accuracy enhancement	Apple

R1-2301409	Other aspects on AI/ML for positioning accuracy enhancement	Qualcomm Incorporated

R1-2301489	Discussion on other aspects on AI/ML for positioning accuracy enhancement	NTT DOCOMO, INC.

R1-2301592	Other Aspects on AI ML Based Positioning Enhancement	MediaTek Inc.

## 4.2	RAN2

#### 4.2.1	RAN2#121

##### AIML methods

Data Collection

R2-2301440	Outcome of [Post120][054][AIML18] Data Collection	Ericsson, vivo	discussion	Rel-18	FS_NR_AIML_air

R2-2300708	Open issues on AI/ML model delivery and data collection in post-meeting email discussion	Apple	discussion	Rel-18	FS_NR_AIML_air

R2-2302286 	Summary of [AT121][025]: Progress table of analyzing data collection framework (Apple)	Apple

R2-2300418	Analysis on data collection framework for AI/ML air interface	Intel Corporation	discussion	Rel-18	FS_NR_AIML_air

R2-2300097	Initial Consideration on Data Collection for AIML	OPPO	discussion	Rel-18	FS_NR_AIML_air

R2-2301055	Data collection for AI/ML	InterDigital Inc.	discussion	Rel-18	FS_NR_AIML_air

R2-2301107	Data collection for AI/ML	Samsung Electronics Nordic AB	discussion

R2-2301769	Data Collection	LG Electronics	discussion	Rel-18

R2-2301787	Further Considerations on Data Collection	ZTE Corporation,Sanechips	discussion	Rel-18	FS_NR_AIML_air

Model Transfer

R2-2301576	Report of [Post120][053][AIML18] model transfer delivery (Huawei)	Huawei	report	Rel-18	FS_NR_AIML_air

R2-2302268	Report of Offline 027 model transfer delivery (Huawei)	Huawei

R2-2301770	Model Transfer Delivery	LG Electronics	discussion	Rel-18

R2-2301788	Further Considerations on AI Function Mapping and Model Transfer	ZTE Corporation,Sanechips	discussion	Rel-18	FS_NR_AIML_air

R2-2300253	AIML Model transfer Requirements during Handover	NEC	discussion	FS_NR_AIML_air

R2-2300257	On AI/ML model transfer over radio interface	Dell Technologies	discussion	Rel-18

R2-2300327	Discussion on AI/ML Model Transfer and Delivery	Futurewei Technologies	discussion

Model ID and UE cap

R2-2300709	Discussion on RAN2 aspects of AI/ML model identification, LCM and capability	Apple	discussion	Rel-18	FS_NR_AIML_air

R2-2300096	AIML Methods Discussion in General	OPPO	discussion	Rel-18	FS_NR_AIML_air

R2-2300374	Model ID and Model Identification	MediaTek Inc.	discussion

R2-2301101	Some considerations about CP/UP solution and model ID	Sony	discussion	Rel-18	FS_NR_AIML_air

R2-2301105	AI/ML Capability Reporting 	Samsung Electronics Nordic AB	discussion

General

R2-2301427	Discussion on AI/ML methods	Qualcomm Incorporated 	discussion	Rel-18

R2-2300249	AIML Architecture Assumptions	NEC	discussion	FS_NR_AIML_air

R2-2300289	Consideration on General Aspects of AIML for NR Air-interface	CATT	discussion	FS_NR_AIML_air

R2-2300393	Discussion on AIML for NR air interface	Xiaomi	discussion

R2-2300398	AIML methods 	Nokia UK	discussion	Rel-18	FS_NR_AIML_air

R2-2300417	Discussion on life cycle management: RAN2 impact	Intel Corporation	discussion	Rel-18	FS_NR_AIML_air

R2-2300660	Discussion on AIML methods	Spreadtrum Communications	discussion	Rel-18

R2-2300679	Further discussion on AIML methods	vivo	discussion	Rel-18	FS_NR_AIML_air

R2-2300806	Discussion on RAN2 aspects for LCM	MediaTek Inc.	discussion

R2-2300950	General issues on AI for air interface	Lenovo	discussion	Rel-18

R2-2301033	Discussion on AIML methods for general aspects of AIML via air interface	Fujitsu	discussion	Rel-18	FS_NR_AIML_air

R2-2301256	Discussion on AIML methods for NR air interface	CMCC	discussion	Rel-18	FS_NR_AIML_air

R2-2301441	General aspects for AIML for NR air interface	Ericsson	discussion	Rel-18	FS_NR_AIML_air

R2-2301577	Discussion on AIML methods	Huawei, HiSilicon	discussion	Rel-18	FS_NR_AIML_air

R2-2301634	Protocol aspects of AI/ML framework for NR air interface	AT&T	discussion

R2-2301835	Discussion on AIML methods	TCL Communication Ltd.	discussion

R2-2301841	AI_ML model life cycle management during RRC state transitions and mobility	Rakuten Symphony	discussion	Rel-18

##### Use case specific aspects

R2-2301578	Discussion on use case specific aspects	Huawei, HiSilicon	discussion	Rel-18	FS_NR_AIML_air

R2-2301442	Use cases aspect for AIML for NR air interface	Ericsson	discussion	Rel-18	FS_NR_AIML_air

R2-2300399	Use case specific aspects	Nokia UK	discussion	Rel-18	FS_NR_AIML_air

R2-2300290	Consideration on the Use Case Specific AIML for NR Air-interface	CATT	discussion

R2-2300661	Discussion on use case specific aspects	Spreadtrum Communications	discussion	Rel-18

R2-2300680	Consideration of procedure and signaling of CSI compression	vivo	discussion	Rel-18	FS_NR_AIML_air

R2-2300697	Use case specific aspects	InterDigital	discussion	Rel-18	FS_NR_AIML_air

R2-2300951	Discussion on AI for air interface use cases	Lenovo	discussion	Rel-18

R2-2301034	Discussion on use case specific aspects of AIML via air interface	Fujitsu	discussion	Rel-18	FS_NR_AIML_air

R2-2301191	Consideration on AI/ML for positioning accuracy enhancement	Xiaomi	discussion

R2-2301257	Discussion on use case specific aspects for AIML for NR air interface	CMCC	discussion	Rel-18	FS_NR_AIML_air

R2-2301429	Discussion on the use case specific aspects	Qualcomm Incorporated 	discussion	Rel-18

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template